Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	ADVANCED STRENGTH OF MATERIALS 			              Time: 3 hours

Subject Code:	ME353			         						              Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

For a plate loaded in tension and containing a centrally loaded hole, the stress concentration has to be studied.  Plot nom and max as a function of d, where 0  d  80 mm and report in the relative effects of the stresses concentration and area change on the stresses.  Take F = 12 kN, w = 100 mm, t = 10 mm.

2.	Relative to an XYZ coordinate system a state of stress at a point in a structure is given by

[] = 3020−202010−10−20−100 MPa.  Determine the principal stresses and maximum shear stress.

(OR)

30MPa

3.	A point in a state of plane stress is isolated by three surfaces as shown. Using the transformation equation, determine the value of  and .

40MPa

45

τ

20MPa

20MPa



4.	Determine the maximum Tresca and Von Mises stress for the given stress matrix.  Compare the Von Mises stress with two times the Tresca stress. [] = 10200200−1020−10210 MPa.

(OR)

5.	A long bar 4 cm wide and 0.75 cm thick is loaded in tension.  The material has the following properties: KIC = 70 MPa.m, yield strength 1200 MPa and ultimate strength 1350 MPa.  If the bar has a crack 0.625 cm long on one edge, determine the maximum possible axial load that can be applied.

6.	At a point the state of stress is given by the stress matrix [] = 30−1520−15−2510201040 MPa.  Determine the strain energy per unit volume if E = 70 GPa and  = 0.33.

(OR)

[P.T.O]

7.	For the wire form shown, use Castigliano’s method to determine the vertical deflection of point A.  Consider bending only and let the bending rigidity of the cross section be EI.



A

B

R

C

P

L



8.	The elliptical cross section shown is transmitting a torsional moment T

a.	Determine the stress function, the maximum shear stress and its location.

y

a

b

z

b.	Determine the maximum shear stress if a = 10 mm, b = 20 mm, T = 400 Nm, E = 200 GPa, 	 = 0.3

(OR)

9.	A thin walled steel tube 1 m long, with the cross section shown in transmitting a torque of        2 kN-m.  The 50 mm dimension is between the wall centre.  For the material let E = 210 GPa and  = 0.29.

Determine the average shear stress in the wall and the total angle of twist of the tube.

50 mm

3 mm

75 mm