Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	DIGITAL SIGNAL PROCESSING 						      Time: 3 hours

Subject Code:	MT241		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Write the expression for energy and power of a signal.

2.	When is a system said to be linear?

3.	Mention two applications of Fourier Transform.

4.	Write the properties of linear convolution.

5.	What is Gibb’s effect?

6.	Mention the advantage of FFT over DFT.

7.	Compare speed and precision in filters.

8.	Mention the various types of windows for digital filters.

9.	What is Narrow – Band filter?

10.	Write the formula for DTFT.

PART – B (5 x 3 = 15 MARKS)

11.	Explain linear and nonlinear system with an example.

12.	Explain analysis and synthesis in Fourier Transform.

13.	Explain multirate decomposition in FT.

14.	Discuss about the time domain and frequency domain parameters of analog filter.

15.	Compare the characteristics of chebyshev and Butterworth filters.

PART – C (5 x 15 = 75 MARKS)

16.	Explain the common decomposition methods in detail.

(OR)

17.	Discuss the requirements and properties of linearity.

18.	Perform the circular convolution of the two sequences given below using DFT and IDFT.

x1(n)=2, 1, 2, 1x2(n)=1, 2, 3, 4

(OR)

19.	Briefly explain the independent variables in Frequency domain.

20.	Write short notes on

a.	Gibbs Effect		b.	Delta function		c.	Chirp signal

(OR)

21.	Discuss the following properties of DFT.

a.	Periodicity	b.	Linearity		c.	Circular symmetry

d.	Circular Convolution	e.	Time reversal.

[P.T.O]

22.	Explain in detail how an FFT works. List its advantages and disadvantages.

(OR)

23.	Design a two-pole bandpass filter that has the center of its passband atω = π2, zero in its frequency response characteristics at ω=0 and ω=π, and its magnitude response is 12at ω= 4π9.

24.	Determine the order and poles of a type I low pass chebyshev filter that has a 1 dB ripple in the pass band, a cutoff frequency Ωp = 1000 π, a stopband frequency of 2000 π,  and an attenuation of 40 db or more for ΩΩs.

(OR)

25.	Explain the design procedure of Butterworth filter. Draw the frequency response also.