Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title : 	INTRODUCTION TO AEROSPACE ENGINEERING     	     Time : 3 hours

Subject Code:	09AE201										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	______________ uses the buoyancy effect for flying.

2.	Name the pilot who crossed the mach no one first time in the history?

3.	_____________ Drag is zero when lift force is zero.

4.	Define static stability.

5.	Why propeller thrust decreases with altitude?

6.	Stiffeners are used for _____________.

7.	Sounding rockets are used for _____________.

8.	What are the types of satellites?

9.	Draw the runway of an airport and mark the different components of it.

10.	Name the space research organization of few countries.

PART – B  (5 x 3 = 15 MARKS)

11.	How the flight vehicles are classified?

12.	How the aerofoil generated lift.

13.	Name the metallic and non-metallic materials used in aircraft contractions.

14.	Differentiate between aircraft thrust and rocket thrust.

15.	Classify the types of wind tunnels.

PART – C  (5 x 15 = 75 MARKS)

16.	With neat sketch, explain the principle, construction, operation, advantages and disadvantages of a Ramjet engine.

(OR)

17.	a.	Differentiate between conventional and powered flight controls.		       				(7)

b.	What are basic instruments required for flying? Explain about any one instrument briefly.	(8)

18.	Explain briefly about the longitudinal and lateral stability of an aircraft.

(OR)

19.	Explain the aerodynamics of a wing and types of drags.

20.	Explain the applications of a titanium, aluminum and composites in aircraft constructions.

(OR)

21.	Describe the working principle of an aircraft piston engine with neat sketches and compare the piston engines with turbojet engines

22.	What are the different types of spacecraft configurations? Explain briefly.

(OR)

23.	What are hybrid rockets? Explain its construction, advantages and disadvantages briefly.

24.	Draw the layout of a subsonic wind tunnel and explain the functions of different components of a wind tunnel.

(OR)

25.	Explain the purpose, scope and different phases of a flight testing.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AIRCRAFT STRUCTURES								      Time : 3 hours

Subject Code:	09AE202		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write down the equation for shear stress.

2.	Write Bredtbatho formula.

3.	The critical load bearing structures of an aircraft are generally known as __________.

4.	The tension field beams are also known as __________.

5.	The fully constrained stationary structures designed to support loads are called __________.

6.	Draw shear force and diagram for simply supported beam with UDL load.

7.	The total strain energy stored in a body is known as __________.

8.	The energy stored in a body due to straining effect is called __________.

9.	Write the expression for Rankine’s constant.

10.	Write down the failure criteria for energy distortion theory.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Shear centre.

12.	Function of aircraft structures.

13.	Explain the type of supports with a neat sketch.

14.	State Maxwell’s reciprocal theorem.

15.	State maximum principle stress theory.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the horizontal and vertical components of the tip deflection of the cantilever as shown in figure. The second moments of area of its unsymmetrical section are Ixx, Iyy,Ixy.

(OR)

17.	Discuss in detail the various types of loads acting on the aircraft structure.

18.	Explain the factors influencing fuselage stress analysis in detail

(OR)

19.	Derive an expression for shear flow of an open tube of arbitrary cross section subjected to shear loads Sxand Sy without twist and modify this expression for a closed tube.

[P.T.O.]

20.	Derive Clapeyron’s three moment equation.

(OR)

21.	A thin cublar bar shown in the figure subjected to a torque T = 11,3000 Nm, has dimensions indicated in the figure.  Determine shear stress in the walls of the section.

a = 12.7 cm		t1 = 0.06 cm			t2 = 0.08 cm

t3 = 0.06 cm		t4 = 0.1 cm			t5 = 0.13 cm

22.	Prove Castigliano’s theorem for point or concentrated loads.

(OR)

23.	Derive quation for strain energy due to

a.	Axial loads.

b.	Bending loads.

c.	Torsional loads.

24.	At a section of a mild steel shaft the maximum torque is 8437.5Nm and maximum bending moment is 5062.5 Nm. The diameter  of the shaft is90mm and the stress at the elastic limit is simple tension for the material of the shaft is 220N/mm2. Determine whether the failure of the material will occur or not according to the maximum shear stress theory, if not, then find out the factor of safety.

(OR)

25.	Write the expression for crippling load when one end of the column is fixed and the other end is free.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AERODYNAMICS  										      Time : 3 hours

Subject Code:	09AE205									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name the forces and moments acting on an aircraft.

2.	__________ drag is more at the vicinity of fuselage and wing joints.

3.	Combination of __________ and __________ flows produces doublet.

4.	What is an irrorational flow?

5.	Induced drag of a wing with symmetrical aerofoil at zero degree angle of attack and airspeed of 10 m/s is __________.

6.	By-pass-ratio associated with __________ engine.

7.	What is service ceiling?

8.	Name the three phases of a take-off performance of an aircraft.

9.	__________ instrument used for measuring boundary layer thickness.

10.	What are internal balances?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw pressure distribution of symmetrical aerofoil for any three angle of attack.

12.	State the Kutta conditions.

13.	State the characteristics of a turbojet engines.

14.	List the conditions for obtaining maximum range for a jet engine aircraft.

15.	Name the subsonic and supersonic flow visualization methods.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain how an aerofoil generates lift.											    (8)

b.	What are the differences between center of pressure and aerodynamic center?		    (4)

c.	Draw CL vs. α plot for symmetrical and cambered aerofoil and define zero lift angle.    (3)

(OR)

17.	a.	Describe the characteristics of a swept wing.	                        						    (8)

b.	Explain induced drag and wave drag.											    (7)

18.	Obtain expression for a lift generate by an aerofoil using Kutta-Joukowski theorem.

(OR)

19.	Explain the flow combination to form the following.

a.	Rankine oval	.																    (7)

b.	Lifting flow over cylinder.														    (8)

20.	Explain the characteristics of turbo prop engines.

(OR)

21.	a.	Discuss classification of propellers. 					       						    (8)

b.	Analyze the methods used for drag reduction.   									    (7)

[P.T.O.]

22.	A Jet engine aircraft has the following data W = 50,000 N, T = 14,500N CD = 0.02+0.04CL2,        CL max=1.2 and S=30m2. Assuming the obstacle height of 15 m and µ=0.05. Calculate the total take-off distance and time at sea level.

(OR)

23.	A glider having W = 2000N, S = 8.0 m2, A = 16.0, e = 0.95 and CDO= 0.015 launched from a height of 300 m. Determine the maximum range, corresponding glide slope, forward velocity and lift coefficient at sea level.

24.	a.	Describe the instruments used turbulence measurements.							   (8)

b.	Explain any one subsonic flow visualization method.								   (7)

(OR)

25.	Describe the types of wind tunnel balance with neat sketches.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	GAS DYNAMICS										      Time : 3 hours

Subject Code:	09AE207									       	       Maximum Marks: 100

#### Use of Approved Gas tables is permitted.

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write down the continuity equation in integral form for compressible steady flow.

2.	At choked flow, ___________.

3.	The shock can be visualized as a thermodynamic device . ( True / False )

4.	In which surface condition (convex or concave corner), the oblique shock is created when the upstream is a supersonic flow?

5.	Mach angle, μ = __________.

6.	Define potential flow.

7.	The increase in angle of attack for the same profile, Mcr __________.

8.	What is the significance of area rule?

9.	Write down any two optical flow visualization methods.

10.	What do you mean by shock tube?

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly explain the compressibility effect of gases with pressure/volume relationship.

12.	A normal shock wave is standing in the test section of a  supersonic wind tunnel . Upstream of the wave , M1 = 3 , P1 = 0.5 atm and T1 =200K . Find downstream velocity u2 .

13.	At a given point on the surface of an airfoil, the pressure co-efficient is –0.3 at very low speeds. 	If the free stream Mach number is 0.6, find the Cp at this point.

14.	Distinguish between lower and upper critical Mach number.

15.	List out the advantages of helium and gun tunnels.

PART – C  (5 x 15 = 75 MARKS)

16.	Air flowing in a duct has a velocity of 300 m/s,  pressure 1 bar and temperature 290 K .

Taking ץ= 1.4 and R = 287 J/ kg K , determine:

a.	Stagnation pressure and temperature.

b.	Velocity of sound in the dynamic and stagnation conditions.

c.	Stagnation pressure assuming constant density.

(OR)

17.	Derive the expression for speed of sound.

18.	Derive the Hugoniot equation and draw the Hugoniot Curve with salient points.

(OR)

19.	Draw the T-S diagram for the flow processes when the pitot-static tube is place in a supersonic flow.

20.	Derive the relation for linearised small perturbation potential theory.

(OR)

21.	Derive and explain the Prandtl-Glauert transformation for Subsonic flow.

[P.T.O.]

22.	Write short notes on:

a.	Shock induced flow separation.

b.	Lift and Drag divergence.

(OR)

23.	Explain the characteristics of swept wing.

24.	Explain the Shock tunnel with sketches.

(OR)

25.	Discuss the design features of intermittent in-draft tunnels with sketches.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	SPACE DYNAMICS									      Time : 3 hours

Subject Code:	09AE208									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define specific impulse of a rocket engine. What is its unit?

2.	The specific impulse of solid propellants is _________ than that of liquid propellants.

3.	For a stable rocket, the center of pressure must be located _________ the centre of gravity. The  measurements are made from the tip of the rocket.

4.	The purpose of putting fins on a rocket is to provide _________ during flight.

5.	Asteroids are revolving about the sun between the orbits of _________ and _________.

6.	Define celestial sphere and celestial equator.

7.	Vis-viva Equation is V2 = _________.

8.	What is injection epoch?

9.	The energy of elliptic orbits is _________ , parabolic orbits is zero and hyperbolic orbits is

_________.

10.	Define Lambert Theorem in 2-body Problem.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the advantages of liquid fuel rockets.

12.	Explain static stability margin for a rocket.

13.	Explain geocentric-inertial coordinate system through a diagram. Show vernal equinox in it.

14.	What are in-plane injection parameters?

15.	Explain briefly patched-conic approach.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	From the first principles, derive the rocket equation

Vb = g0 Isp ln(Mi/Mf),

where Vb is the burnout velocity, g0 is the acceleration due to gravity at sea level, Isp is specific impulse and Mi/Mf is the mass ratio. If the Isp of a rocket is 390 s, and Vb is 7800 metres/s, calculate its mass ratio. (g0=9.8 m/s2).        								    (9)

b.    A two-stage rocket has the following design characteristics. First stage: propellant mass = 	20000 kg, structural mass = 4000 kg. Second stage: propellant mass = 10000 kg, structural 	mass = 1800 kg. The payload mass is 200 kg. The specific impulse of the both the stages is 	300 s. Calculate the final burnout velocity. (g0=9.8 m/s2).                                                 (6)

(OR)

17. 	Derive an expression for optimizing the total mass for a two stage rocket.

18.	a.  	What is static stability of a rocket? With the help of a neat diagram, explain the static 	stability regions in terms of the aerodynamic moment and angle of attack.                 	  (11)

b.    Explain static longitudinal stability of a rocket.                                                                  (4)

(OR)

[P.T.O.]

19.	a.    Explain directional stability. 			                                                            		    (4)

b.    Describe the dynamic stability of a rocket.			                                    			    (8)

c.    Explain lateral static stability of a rocket.                                                                           (3)

20.	a.   State Kepler’s laws of planetary motion. If the eccentric anomaly (E) and eccentricity (e) of  	a satellite are 80 degrees and 0.3, respectively, find the mean anomaly (M) of the satellite.

(5)

b. 	Sketch the classification of different atmospheric regions with respect to Earth and give 			short explanation of each region. Show the variation of temperature with altitude.      (10)

(OR)

21.	a.   	Explain the six orbital parameters of an Earth satellite through a diagram.                     (5)

b.  	If the position and velocity of a satellite are (0, –5888.9727, –3400) km and (8.3, 0, 0) 	km/s, respectively; find the semi-major axis, eccentricity and inclination of the satellite.

Earth’s gravitational constant = 398600 km3s-2.           			                      	  (10)

22.	a.  	Explain briefly three important perturbing forces acting on an artificial earth satellite.       																					    (6)

b.   	Explain Cowell’s method and Encke’s method.                                                                 (9)

(OR)

23.	a.   	Find the additional velocity required for a Hohmann transfer from a circular Earth satellite 		orbit of radius 8000 km to a circular earth satellite orbit of radius 10000 km.                 (10)

b.   Calculate the velocity change required to transfer a satellite from a circular orbit at an 	altitude of 700 km with an inclination of 40 to an orbit of the same size at an inclination of 	20. Earth’s gravitational constant = 398600 km3s-2.                                                  	    (5)

24.	a.   	Draw a neat sketch of hyperbolic trajectory. Show in the sketch, true anomaly of the  			asymptote, turn angle, periapsis, apoapsis and semi-major and semi-minor axes.             (7)

b.   	At a given point of a spacecraft’s geocentric trajectory, the radius is 16000 km, the speed is 	8.4 km/s, and the flight path angle is 45 degrees. Show that the path is a hyperbola and 	calculate the following: (i) C3, (ii) angular momentum (iii) true anomaly.          	    	    (8)

(OR)

25.	a. 	Estimate the trip time T from Earth to Mars along the Hohmann transfer orbit by                   	assuming the orbits of Earth and Mars around the Sun to be circular with radii of           	149.6 x 106 and 227.9 x 106 km, respectively. The value of the Sun’s gravitational constant 	= 1.32715 x 1011 km3s-2.                                                                                    		    (5)

b.  	Calculate the synodic period of Venus relative to the Earth. The orbital periods of Earth   	and Venus are 365.26 days and 224.7 days, respectively.                                                 (5)

c.  	Calculate the sphere of influence of Mars. The masses of Mars and Sun are                   	6.4169 x1023  kg and 1.989 x1030 kg, respectively. The radius of Mars’s orbit about Sun is 	227.9 x106 km.        															    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AIRCRAFT PROPULSION								      Time : 3 hours

Subject Code:	09AE209				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The thrust produced by jet engine per kg of airflow is called as __________.

2.	Pulsejet engine works based on __________ thermodynamic cycle.

3.	Define degree of reaction.

4.	__________ is used to increase angular momentum of the fluid without increasing its radius of rotation.

5.	What is called combustion intensity?

6.	The ratio of turbine rotor blade work to the energy supplied to the rotor blade is called as __________.

7.	The ratio of actual gross thrust to ideal gross thrust is called as__________.

8.	What is internal compression of a supersonic inlet?

9.	Define match point.

10.	What is called operating line?

PART – B  (5 x 3 = 15 MARKS)

11.	What is ISA? State its uses.

12.	What are the differences between centrifugal and axial compressor?

13.	List the factors affecting combustion chamber design.

14.	Classify the aircraft inlets.

15.	State the assumption made in the matching procedure of a single spool turbojet engine.

PART – C  (5 x 15 = 75 MARKS)

16.	A Brayton cycle works between 1 bar, 300K and 5 bar, 1250K. There are two stages of compression with perfect inter cooling and two stages of expansion. The work out of first expansion stage being used to drive the two compressors, where the interstate pressure is optimized for the compressor. The air from the first stage turbine is again heated to 1250K and expanded. Calculate the power output of free power turbine and cycle efficiency without and with a perfect heat exchanger and compare them. Also calculate the percentage improvement in the efficiency because of the addition of heat exchangers.

(OR)

17.	The ratio of net work to turbine work of an ideal gas turbine plane is 0.563.Take the inlet temperature to the compressor as 300 K. Calculate the temperature drop across the turbine if the thermal efficiency of the unit is 35%. Assume a mass flow rate of 10 kg/s,

Cp =1 kJ/ kg K and γ =1.4.

18.	A centrifugal compressor has a pressure ratio of 4:1 with an isentropic efficiency of 80% when running at 15000 rpm and inducing air at 293 K. Curved vanes at inlet give the air a prewhirl of 25˚ to the axial direction at all radii and the mean diameter of eye is 250 mm. The absolute air velocity at inlet is 150 m/s. Impeller tip diameter is 600 mm. Calculate the slip factor.

(OR)

[P.T.O.]

19.	An axial flow air compressor of 50 % reaction design had blades with inlet and outlet angles of 45˚and 10˚reapewctively.The compressor is to produce a pressure ratio of 6:1 with an overall isentropic efficiency of 0.85 when inlet static temperature is 37˚C.The blade speed and axial velocity are constant throughout the compressor. Assuming a value of 200 m/s for blade speed find the number of stages required if the work done factor is:

a.	unity 			b.	0.87 for all stages.

20.	What are the requirements of combustion chambers? Classify the combustion chamber and explain the operating principle of each type of combustion chambers.

(OR)

21.	Explain the working principles and components of an impulse and reaction turbines.

22.	a.	Describe the working of after burner with a neat sketch.   				      			  (10)

b.	List the functions of an aircraft engine nozzle.		                               			    (5)

(OR)

23.	Describe the working of supersonic inlet and it different operating conditions.

24.	Explain the procedure to find equilibrium point.

(OR)

25.	Describe the general matching procedure of a turbojet engine.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AIRCRAFT STABILITY AND CONTROL				      Time : 3 hours

Subject Code:	09AE215		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is fineness ratio?

2.	What is the Criterion for longitudinal control?

3.	What are the disadvantages of canard configuration?

4.	Define Hinge moment coefficients.

5.	What is stick fixed neutral point?

6.	What is meant by dihedral effect?

7.	What is adverse yaw?

8.	What is meant by stability quartic?

9.	Define degrees of freedom of a system.

10.	Define Dynamic stability.

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss about Upwash and Downwash due to wing.

12.	The dynamic pressure over tail is different from the free stream dynamic pressure. True/False. Justify your answer

13.	What is meant by Weathercocking effect?

14.	Define Routh’s discriminant.

15.	An airplane w/s=3000N/m2, V=450 kmph flying at 6 Km altitude has a characteristic equation in longitudinal mode of the form (λ2+λ+6) (3λ2+λ+1) = 0. Find period and time to damp it. (Density is 0.66 kg/m3 at 6 Km altitude).

PART – C  (5 x 15 = 75 MARKS)

16.	For an airplane the pitching moment co-efficient is given by the following expression when c.g. lies at 0.25 c. Cmcg = 0.05- 0.10CL0.01δe where δe is in degrees. Determine the following:

a.	Is the airplane statically stable? Justify.

b.	What is the static margin?

c.	What is the location of neutral point stick fixed?

d.	What is the value of CL for which equilibrium is achieved with zero elevator deflection?

e.	What is the value of elevator effectiveness (Cmδ)?

f.	If the elevator deflection is limited to ± 25º, locate the most forward c.g. location for which trim is obtained at CL=1.5 in free light.

(OR)

17.	Describe the criteria for longitudinal static stability. Explain the contributions of fuselage and nacelles to Cmcg and Cmα.

18.	a.	Write short notes on: stabilizer settings and CG location.					   		    (8)

b.	Briefly explain the stick force gradients and stick force per g load?			   		    (7)

(OR)

[P.T.O.]

19.	Discuss the power effects on longitudinal static stability for both jet and propeller driven airplanes

20.	Based on strip theory derive an expression for aileron control power.

(OR)

21.	a.	Explain the phenomenon of Rudder lock and methods to prevent rudder lock.		    (7)

b.	An airplane with the following characteristics is coming in to land at sea level at a speed of 1.2 times the stalling speed. What would be the amount of rudder deflection required if cross wind of 10 m/s is encountered? W/S= 1500 N/m2, Vv =0.05, CLαv =2.87 rad-1, τrudder=0.5, Cηß= 0.071 rad-1, CLmax=1.8, ηv=1.0.							    	    (8)

22  	An airplane of weight 300,000 N, wing area 100 m2 flies at 6 km altitude at a speed of 400 	kmph. The characteristic equation is given λ3+4λ2+12λ+16 = 0

a.	Write Routh discriminent for this.

b.	Find roots and sketch the nature with time.

c.	For oscillations, compute the period and period for halfening the amplitude.

d.	Comment on such oscillation.

(OR)

23.	Discuss the following:

a.	Phugoid motion.										    						    (7)

b.	Stability derivatives in longitudinal dynamics.						    			    (8)

24.	Describe the following.

a.	Wind axes system and stability axes system. 						    				    (5)

b.	Short period and long period modes of oscillation of longitudinal motion. 		     	    (5)

c.	Spiral mode and Dutch roll mode of lateral motion. 					    			    (5)

(OR)

25.	a.	Discuss briefly about the two control airplane. 						    			    (7)

b.	Discuss in detail the phenomena of autorotation and spin. 				    			    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AIRCRAFT PERFORMANCE							      Time : 3 hours

Subject Code:	09AE216									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Bluff bodies produce more drag than streamlined bodies (True/False). Justify.

2.	Reynolds’s number is a ratio of __________ and __________.

3.	__________ is used for controlling induced drag.

4.	Define Aspect ratio.

5.	State the condition for steady level flight.

6.	What are SFC and TSFC?

7.	Name the high lift devices used in an aircraft.

8.	Rate of climb at service ceiling is _________ m/s

9.	Define load factor.

10.	Classify the types of propellers.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the differences between streamlined bodies and bluff bodies?

12.	What is drag polar? Draw a typical drag polar for a civil aircraft.

13.	Why propeller thrust decreases with altitude?

14.	Define thrust augmentation. List the methods used for thrust augmentation.

15.	List the assumptions made in Froude momentum theory.

PART – C  (5 x 15 = 75 MARKS)

16.	What is Magnus effect? Obtain expression for lift generated by a rotating cylinder.

(OR)

17.	What are the types of drag acting on an aircraft? Describe them briefly.

18.	Explain the following:

a.	Drag reduction methods.														    (8)

b.	Pressure distribution on aircraft wing.											    (7)

(OR)

19.	Explain the plan form characteristics for the following wings:

a.	Elliptical wing.																    (7)

b.	Delta wing.																	    (8)

20.	An aircraft weighs 250 kN has a wing area of 80 m2 and its drag equation isCD=0.016+0.04CL2. Calculate theL

a.	Minimum thrust required 		b.	Minimum power required for straight and level flight and the corresponding true air speed at sea level at an altitude where σ=0.58 . Assume ρ∘=1.23Kgm3. Also derive the equation used.

(OR)

[P.T.O.]

21.	An aircraft having wing area of 255 m2 and weight of  10000 kg and a clear drag polar (flaps and gear up) of CD = 0.023 + 0.0735CL2 Calculate the following:

a.	(L/D) max

b.	VD min at sea level and at 40000 m

c.	Tmin for level flight

22.	a.	Derive the Brequette Range formula for Jet-Propelled airplanes.		       			    (8)

b.	For passenger aircraft, show that

TT0max=1+CLCD2

where

T0 =Thrust for level flight

T = Thrust for climbing flight at the same angle of incidence.	      					    (7)

(OR)

23.	What are the three phases of take-off an aircraft? Derive and obtain expression for estimating the take- off distance.

24.	Explain the following:

a.	Propeller coefficients.							 								    (7)

b.	V-n diagrams.																    (8)

(OR)

25.	Based on Froude’s momentum theory derive an expression for the ideal propulsive efficiency of an airscrew.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BASICS OF AEROSPACE ENGINEERING				      Time : 3 hours

Subject Code:	10AE202									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.    	Wings are the parts of airplanes which provide __________ and support the entire weight of the 	aircraft and its contents while in flight.

2.	What is Mach number regime for subsonic flight?

3.	Flight Instruments enable the aircraft to fly through the _________ weather conditions and during _________.

4.	Flight Instruments help to visualize the attitude, location and _________ of the aircraft.

5.	When the force of lift is greater than the force of _________, the airplane is able to fly.

6.	The NACA 2418 airfoil has maximum thickness of _________.

7.	_______ includes the entire tail group, consisting of fixed surfaces such as the vertical stabilizer and the horizontal stabilizer.

8.	Landing Gear should locate near the _________ of the plane.

9.	Define specific impulse of a rocket.

10.	Rocket thrust is caused by _________ acting in the combustion chamber and nozzle.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the materials used for aircraft construction?

12.	Draw and specify the four forces acting on an aircraft.

13.	Draw an airfoil and explain its basic nomenclature.

14.	Name the five primary stresses acting on an aircraft.

15.	State the advantages of liquid propellants.

PART – C  (5 x 15 = 75 MARKS)

16.	   Explain the features of monoplane and biplane. Explain why a monoplane is advantageous

over a biplane.

(OR)

17.  a.	 Describe the contribution of Sir George Cayley in the area of aerodynamics.        	    (8)

b. 	 Enumerate the works of Otto Lilienthal- the Glider man.                            	    		    (7)

18.	Explain the important features of an aircraft and the functions of its components with a neat sketch.

(OR)

19.  	Why are flight instruments important? Explain the six basic flight instruments.

20.	Sketch the classification of different atmospheric regions with respect to Earth and give     explanation of each region. Show the variation of temperature with altitude.

(OR)

21.	a.	Define the following: 								  							(4x2)

i. 	Dynamic  pressure     	ii. 	Aerodynamic centre	iii. 	Bernoulli’s equation

iv. 	Center of pressure

b.	Explain about NACA 5 digit numbering system.				     					    (7)

[P.T.O.]

22.	What are the metallic and non-metallic materials used in an aircraft?  Explain their advantages and disadvantages.

(OR)

23.	What is the purpose of landing gears?  What are its design considerations? What are three common types of landing gears? Explain the tri-cycle gear configuration with a neat sketch.

24.	a.	What is propulsion? Classify jet propulsion. Explain the principle of operation of a 				turboprop engine.		  						                						  (10)

b.	Write the advantages of a jet engine.              			                             		    (5)

(OR)

25. What is the principle of operation of rockets? Explain the operation of liquid propellant 	rockets. What are their uses?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INTRODUCTION TO AEROSPACE ENGINEERING	      Time : 3 hours

Subject Code:	11AE201				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Who is known as the father of aviation?

2.	What is the operating cycle of a jet engine?

3.	Define lift and drag.

4.	Define International standard atmosphere.

5.	List the components of a turbofan engine.

6.	Define Special Fuel Consumption.

7.	What are the various wing configurations of an aircraft?

8.	What are the uses of titanium alloys?

9.	Name few satellites launched by India Space Research Organization.

10.	Define Kepler’s second law.

PART – B  (5 x 3 = 15 MARKS)

11. 	What are the advantages of jet propulsion over rocket propulsion?

12.	What are the various aerodynamic forces acting on an aircraft?

13. 	Enumerate the advantages and disadvantages of turboprop engines.

14. 	Write short notes on monocoque and semi-monocoque structures construction.

15. 	What are the various spacecraft configurations?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the thrust equation for a simple turbojet engine.

(OR)

17.	Write short notes on the early developments of airplanes, biplanes and monoplanes.

18.	Explain the various components of an airplane and their functions with neat sketches.

(OR)

19.	Explain the various types of drag acting on an aircraft and the methods to minimize them.

20.	Explain the various types of aircraft power plants with neat sketches.

(OR)

21.	Define specific impulse of rocket engines and derive the rocket thrust equation.

22.	Explain in detail the various metallic and non metallic materials used in aircrafts. Also list out 	the materials used for turbine blades.

(OR)

23.	What is a flight envelope? Draw and explain the V-n diagram.

24.	Derive the Orbit equation for a spacecraft.

(OR)

25.	Write down the principle of operation of rocket and discuss the various types of rockets and 	their applications.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AIRCRAFT STRUCTURES								      Time : 3 hours

Subject Code:	11AE202									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define truss.

2	Write the general form of theorem of three moment equation.

3.	The stress due to suddenly applied load is ___________ times that of gradually applied load.

4.	Proof resilience per unit volume is called as ___________.

5.	What you mean by crippling load?

6.	Define factor of safety.

7.	Shear flow can be defined for both thin and thick walled section (true/false).

8.	What you mean by shear centre?

9.	In a Wagner beam, the normal stresses are carried by___________.

10.	Define longeron.

PART – B  (5 x 3 = 15 MARKS)

11.	Define continuous beam and state its advantages.

12.	Prove Maxwell’s reciprocal theorem.

13.	Calculate the buckling load of a mild steel column of equivalent length is 2m for which Euler’s formula is valid. Assume EI = 7MN-m2.

14.	Write short notes on Bredt-Batho theory.

15.	How the loads are distributed in fuselage members?

PART – C  (5 x 15 = 75 MARKS)

16.	A continuous beam, 22 m long supported over spans AB=6m, BC=12m and CD= 4m, carries uniformly distributed load of 2kN/m, 1kN/m and 3kN/m run over span of AB, BC and CD respectively.  Using Clapeyron’s moments, find:		a.	Bending moments at the supports B and C 	b.	Reaction at the supports.  Draw bending moment and shear force diagrams also.

(OR)

17.	A truss of 12 m span is loaded as shown in the figure.  Determine the forces in the members of the truss by method of joints.

[P.T.O]

18.	Show that the strain energy U due to bending of a cantilever of length l with a concentrated load W at the free end is given by U=w2l6EI where EI is the flexural rigidity of the beam.  Hence show that the deflection of the free end of the beam is wl3EI .  Show further that for a rectangular beam cross-section the strain energy can be expressed as U=σ218Ex volume of the beam where σ is the maximum bending stress in the beam.

(OR)

19.	A cantilever of rectangular section breadth b, depth d and of length l carries uniformly distributed load spread from free end to the mid section of the cantilever.  Using Castigliano’s theorem find:

a.	Deflection due to shear at the free end.		b.	Deflection due to bending at the free end.

20.	In a material the principal stresses are 60Mpa, 48Mpa and –35Mpa.  Calculate:

a.	Total strain energy;															    (4)

b.	Volumetric strain energy;														    (4)

c.	Shear strain energy;															    (4)

d.	Factor of safety on the total strain energy criterion if the material yields at 120Mpa. 	    (3)

Take E=200Gpa and 1/m=0.3

(OR)

21.	Compare the crippling loads given by Rankine’s and Euler’s formulae for tubular strut 2.25m long having outer and inner diameters of 37.5 mm and 32.5mm loaded through pin-joint at both ends.  Take yield stress as 315MN/m2; Rankine constant a=1/7500 and E=200GN/m2.  If elastic limit for the material is taken as 200MN/m2; then for what length of the strut does the Euler formula cease to apply?

22.	Calculate the shear flow distribution in the channel section shown in the figure produced by a vertical shear load of 4.8kN acting through its shear center.

Assume A1= A2 = A3 = A4= 300mm2.

(OR)

23.	Find the shear flow distribution as shown in the figure.

Assume A1= A3 = A4 = A6=900mm2 and A2 = A5=1200mm2.

24. 	The fuselage of a light passenger-carrying aircraft has the circular cross section shown in the figure. The cross-sectional area of each stringer is 100mm2, and the vertical distances given in the figure are to the midline of the section wall at the corresponding stringer position.  If the fuselage is subjected to a bending moment of 200 kNm applied in the vertical plane of symmetry, at this section, calculate the direct stress distribution.

(OR)

25.	Explain in detail about Tension field web beam.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FLIGHT MECHANICS AND PERFORMANCE			      Time : 3 hours

Subject Code:	11AE204				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The position at which it is assumed that the total lift act on the airfoil is the __________.

2.	Excess power of the engine is equal to __________.

3.	What is endurance?

4.	The minimum turning radius is dictated by __________ and __________.

5.	What is horn balance?

6.	‘For the system to be dynamically stable, the system must be statically unstable’-True/False.

7.	Directional stability is also called as __________.

8.	What is polyhedral wing?

9.	What is phugoid mode?

10.	Dutch roll is controlled by__________.

PART – B  (5 x 3 = 15 MARKS)

11.	Calculate the pressure, density and temperature at 8 km in the ISA.

12.	Define absolute ceiling and service ceiling.

13.	Explain the effect of CG for longitudinal static stability.

14.	Define cross coupling effect. Give an example.

15.	What are the steps involved in spin recovery?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss the various types of drag acting on the airplane.    							  (10)

b. 	Explain the various drag reduction methods with a neat sketch.						    (5)

(OR)

17. 	a.	Derive an expression for drag polar and explain it with a neat plot.        				    (8)

b. 	Draw the power required and power available curve for a jet engine and piston engine and 	state your observations.      													    (7)

18.	a.	Derive the conditions for minimum drag and minimum power required in steady level 			flight.																		  (10)

b.	An aircraft weighing 2, 50,000 N has a wing area of 80 m2 and its drag equation is 
	CD= 0.016 + 0.04 CL2. Calculate the minimum thrust required for level flight.   		    (5)

(OR)

19.	a.	Derive the Breguet Range formula and discuss its implications.          				  (10)

b.	Explain-V-n diagram.															    (5)

20.	a.	Explain the basic characteristics of aircraft control surfaces.    						    (5)

b.	Derive an expression for wing contribution to static longitudinal stability and offer 
your comments on this expression.    											  (10)

(OR)

[P.T.O.]

21.	a.	Explain the power effects of propeller engine on longitudinal stability.				    (7)

b.	Derive an expression for stick fixed neutral point and static margin equation.   		    (8)

22.	a. 	What is aero elasticity?  Explain aileron reversal. 								    (8)

b. 	Write short notes on Dihedral Effect.	 										    (7)

(OR)

23.	a.	Discuss the basic requirements of rudder with neat sketch.   						  (10)

b.	Explain Rudder lock.  														    (5)

24. 	a.	Using Routh’s criteria determine the stability of the system represented by the following 	polynomial.

λ4 + 2 λ3+5 λ2+2 λ+2 =0.

λ4 + 3 λ3+2 λ2+4λ+1=0.		     												    (5)

b.	Derive the stability derivatives for longitudinal dynamics.  						  (10)

(OR)

25.	Explain the following:

a.	Directional divergence.														    (5)

b.	Autorotation.																	    (5)

c.	Spiral divergence.															    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AERODYNAMICS										      Time : 3 hours

Subject Code:	11AE205									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is streamline in a flow?

2.	Define irrotational flow.

3.	Which combination of basic flows generates lift for flow over a circular cylinder?

4.	State Kutta-Joukowskii theorem.

5.	Define Complex Potential for irrotational flow.

6.	What is the relationship between complex potential and flow velocity?

7.	In case of incompressible boundary layer theory, define Displacement Thickness.

8.	In case of incompressible boundary layer theory, define Energy Thickness.

9.	State Biot-Savart law and its application.

10.	Define Horse-shoe Vortex.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the relation between vorticity and circulation?

12.	State Kutta condition. What is its physical significance?

13.	What is Cauchy-Riemann relation?

14.	In incompressible laminar boundary layer, how does the boundary layer thickness change with local Reynolds number?

15.	Briefly explain the principle of Prandtl’s Lifting line theory.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive Bernoulli equation for incompressible steady flow using Euler’s equations.

(OR)

17.	For 2-D flow in x-y plane, velocity along x-axis is given by ux,y=−yx2+y2.  Use the condition at y-axis v(x = 0, y) = 0. Find out:

a.	Velocity along y-axis v(x, y).													    (5)

b.	Potential function φ(x, y).														    (5)

c.	Stream function Ψ(x, y).														    (5)

18.	Prove Kutta-Joukowskii theorem for flow over a thin airfoil.

(OR)

19.	Derive the lift per unit length of a cylinder (with radius R) rotating about its axis with angular velocity ω rad/s in a cross flow with velocity U and density ρ.

20.	Use Kutta-Joukowskii transformation ς=z+a2z  to transform the a circle of radius b with centre at (0, 0). The radius of circle b is greater than constant a in the transformation.

a.	Show that the resulting form is ellipse.											  (10)

b.	Find the length of semi-major and semi-minor axes of the ellipse.					    (5)

[P.T.O.]

(OR)

21.	The camber line of a circular arc airfoil is given by yc=4hxc1−xc.  Derive the expression for load distribution (pressure distribution between lower and upper surface) at an angle of attack α.

22.	The velocity profile in a laminar boundary layer flow at zero pressure gradient is approximated as uU=32yδ−12yδ3. Using Momentum Integral equation, obtain the expression for

a.	δxx			b.	Cf(x)													(8+7)

(OR)

23.	Consider an incompressible laminar boundary layer with constant free stream velocity U. Assume that velocity distribution in the boundary layer is given by

u=U2yδ−yδ2

Determine:

a.	boundary layer thickness δx  		b.	the wall skin friction coefficient Cf.

24.	For a finite wing with elliptic distribution of circulation along the span:

a.	Prove that downwash is constant along the span. 									    (8)

b.	Find the lift of the wing.														    (7)

(OR)

25.	In case of two wings with same span, one wing has elliptic  Γ=Γe1−y2s2 distribution of circulation and another has parabolic  Γ=Γp1−y2s2. Both the wings have same lift. Find:

a.	Relationship between Γeand Γp. 												    (8)

b.	Relationship between downwash velocity in plane symmetry we and wp..			       (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INTRODUCTION TO AEROSPACE ENGINEERING	      Time : 3 hours

Subject Code:	12AE201				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is Mach number?

2.	What are the various aerodynamic forces acting on an aircraft?

3. 	Define drag?

4. 	Classify the different types of flight vehicles.

5.	The use of trim tab is __________.

6.	What are the advantages of using composite materials in an aerospace industry?

7.	What is By-pass ratio of a turbofan engine?

8.	Define specific impulse?

9.	What is the purpose of turn indicator?

10.	Expand the following:

a. LPSC		b.	DRDO

PART – B  (5 x 3 = 15 MARKS)

11.	What is the difference between biplanes and monoplanes?

12.	What is the different wing configurations used in an aircraft?

13.	What are the advantages of having semi-monocoque type of fuselage structure?

14.	List the various types of satellite used and their applications.

15.	What is the basic engine instruments used in an aircraft?

PART – C  (5 x 15 = 75 MARKS)

16.	Write a note on the contribution of the following in the history of aviation.

a. 	Sir George Caley			b. 	Langley				c.	Wright Brothers

d. 	Otto Lilienthal

(OR)

17.	Explain the various developments that have taken place in the field of aerodynamics and structure over a period of time in the history of aviation.

18.	Describe the different components of an aircraft and their functions.

(OR)

19.	Explain in detail the major conventional controls and its purpose.

20. 	Explain about the various materials used for aircraft construction.

(OR)

21.	Write a short note on the following.

a. 	Monocoque structure.		b. 	Semi-Monocoque structure.	c. 	Geodesic structure.

22.	With a neat sketch explain the working principle of a turbofan engine.

(OR)

23.	a. 	Explain the operating principle of a liquid propellant rocket engine?                        	  (10)

b. 	State the advantage and disadvantages of liquid propellant over solid propellant.      	   (5)

[P.T.O.]

24. 	What are the major processes involved in flight testing and also explain their processes.

(OR)

25.	Explain in detail the basic instruments used for flying.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	THERMODYNAMICS									      Time : 3 hours

Subject Code:	12AE202									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by a closed system? Give an example.

2.	Work done in a free expansion process is _________.

3.	Define – PMM of second kind.

4.	What do you mean by “Clausius inequality”?

5.	Define sensible heat of water.

6.	What do you mean by reduced properties?

7.	What is Joule-Thomson coefficient?

8.	State Gibbs function.

9.	What is classical thermodynamics?

10.	What are the assumptions made in Maxwell-Boltzmann Statistics?

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by Point and Path function? Give an example.

12.	What are the corollaries of Carnot theorem?

13.	What do you understand the terms “Degree of super heat” and “Degree of sub-cooling”?

14.	State Tds Equations.

15.	Define the Uncertainty Principle.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	680 kg of fish at 5°C are to be frozen and stored at – 12°C. The specific heat of fish above 	freezing point is 3.182, and below freezing point is   1.717 kJ/kg K. The freezing point is     	– 2°C, and the latent heat of fusion is 234.5 kJ/kg. How much heat must be removed to 	cool the fish, and what per cent of this is latent heat?								    (8)

b.	A mass of 1.5 kg of air is compressed in a quasi-static process from 0.1 MPa to 0.7 MPa for which pv = constant. The initial density of air is 1.16 kg/m3. Find the work done by the piston to compress the air.														    (7)

(OR)

17.	In oil cooler, oil flows steadily through a bundle of metal tubes submerged in a steady stream of cooling water. Under steady flow conditions, the oil enters at 90°C and leaves at 30°C, while the water enters at 25°C and leaves at 70°C. The enthalpy of oil at t°C is given by                      h = 1.68 t + 10.5 × 104 t2 kJ/kg .What is the cooling water flow required for cooling 2.78 kg/s of oil?

18.	A heat pump working on the Carnot cycle takes in heat from a reservoir at 5°C and delivers heat to a reservoir at 60°C. The heat pump is driven by a reversible heat engine which takes in heat from a reservoir at 840°C and rejects heat to a reservoir at 60°C. The reversible heat engine also drives a machine that absorbs 30 kW. If the heat pump extracts 17 kJ/s from the 5°C reservoir, determine

a.	The rate of heat supply from the 840°C source.

b.	The rate of heat rejection to the 60°C sink.

[P.T.O.]

(OR)

19.	a.	Ten grams of water at 20°C is converted into ice at –10°C at constant atmospheric 	pressure. Assuming the specific heat of liquid water to remain constant at 4.2 J/gK  and 	that of ice to be half of this value, and taking the latent heat of fusion of ice at 0°C to be 	335 J/g,  calculate the total entropy change of the system.  						    (8)

b.	A heat engine receives reversibly 420 kJ/cycle of heat from a source at 327°C, and rejects 	heat reversibly to a sink at 27°C. There are no other heat transfers. For each of the three 	hypothetical amounts of heat rejected, in (a), (b), and (c) below, compute the cyclic 	integral of  dQ/T from these results show which case is irreversible, which reversible, and 	which impossible:

a.	210 kJ/cycle rejected.

b.	105 kJ/cycle rejected.

c.	315 kJ/cycle rejected.														    (7)

20.	A rigid vessel contains 1 kg of a mixture of saturated water and saturated steam at a pressure of 0.15 MPa. When the mixture is heated, the state passes through the critical point. Determine:

a.	The volume of the vessel.

b.	The mass of liquid and of vapour in the vessel initially.

c.	The temperature of the mixture when the pressure has risen to 3 MPa.

d.	The heat transfer required to produce the final state (c).

(OR)

21.	Two vessels, A and B, each of volume 3  m3  may be connected together by a tube of negligible volume. Vessel A contains air at 7 bar, 95°C while B contains air at 3.5 bar, 205°C. Find the change of entropy when A is connected to B. Assume the mixing to be complete and adiabatic.

22.	Derive  Clausius – Clapeyron equation.

(OR)

23.	Prove  that

24.	Explain the concept of Quantum principle and Wave and particle duality.

(OR)

25.	Write short notes on:

a.	Maxwell Boltzmann statistics.													    (8)

b.	Equipartition of energy.														    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	STRENGTH OF AEROSPACE MATERIALS			      Time : 3 hours

Subject Code:	12AE203									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Hooke’s law.

2.	Write the formula for hoop stress developed in a thin cylinder.

3.	What is point of contra flexure?

4.	List out the types of loads.

5.	Define composite shaft.

6.	Give the expression for torsional rigidity of a solid circular shaft.

7.	Write the formula for calculating deflection of a SSB carrying central point load.

8.	What is slenderness ratio?

9.	Define principal plane.

10.	List any two theories of failure.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw stress strain curve for a ductile material and mark the important points on it.

12.	Draw the SFD of a cantilever beam carrying UDL over its entire length.

13.	What are the assumptions made in theory of simple bending?

14.	Write the limitations of Macaulay’s method.

15.	Explain strain energy theory.

PART – C  (5 x 15 = 75 MARKS)

16.	A member ABCD is subjected to point loads P1, P2, P3and P4 as shown in figure given below.

Calculate the force P2 necessary for equilibrium if P1 = 45 kN, P3 = 450 kN and P4 = 130 kN. Determine the total elongation of the member assuming E = 2 X 105 N/mm2.

(OR)

17.	A cylindrical vessel 1.5 m diameter and 4 m long is closed at the ends by rigid plates. It is subjected to an internal fluid pressure of 3 N/mm2. If the maximum principal stress is not to exceed 150 N/mm2, find the thickness of the shell. Assume E = 2 X 105 N/mm2 and Poisson’s ratio = 0.25. Find the changes in diameter, length and volume of the shell.

18.	A cantilever beam 1.5 m long is loaded with a uniformly distributed load of 2 kN/m run over a length of 1.25 m from free end. It also carries a point load of 3 kN at a distance of 1.25 m from fixed end. Draw the SFD and BMD.

(OR)

[P.T.O.]

19.	A simply supported beam of span 6 m carries a point load of 1 kN at 1 m from the left support and another point load of 4 kN at 1 m from the right support. It carries a UDL of 2 kN/m run over a distance of 2 m at mid span of the beam. Find the SF and BM values. Draw SFD and BMD.

20.	A timber beam of rectangular cross section of length 8 m is simply supported. The beam carries a UDL of 2 kN/m run over the entire length and a point load of 10 kN at 3 m from the left support.  If the depth is twice the width and the bending stress in the timber is not to exceed 8 N/mm2, find the suitable dimensions of the section.

(OR)

21.	A solid cylindrical shaft is to transmit 300 kW at 100 rpm.

a.	If the shear stress is not to exceed 80 N/mm2, find its diameter.

b.	What percentage savings in weight would be obtained if this shaft is replaced by a hollow 	one whose internal diameter equals to 0.6 times the external diameter? The length, the 	material and the maximum shear stress being the same.

22.	A beam of length 6 m is simply supported at its ends and carries two point loads of 48kN and 40kN at a distance of 1 m and 3 m respectively from the left support. Find:

a.	the deflection under the each load 		b.	maximum deflection

c.	then point at which the maximum deflection occurs.

Take E = 2 X 105 N/mm2 and I = 85 X 106 mm4.

(OR)

23.	A hollow cylindrical cast iron column is 4 m long with both ends fixed. Determine the minimum diameter of the column if it has to carry a safe load of 250 kN with a factor of safety of 5. Take the internal diameter as 0.8 times the external diameter. Take c = 550 N/mm2 and    = (1/1600) in Rankine’s formula.

24.	At a certain point in a strained material, the intensities of stresses on two planes at right angles to each other are 20 N/mm2  and 10 N/mm2  both tensile. They are accompanied by a shear stress of magnitude 10 N/mm2 . Find graphically or otherwise, the location of principal planes and evaluate the principal stresses.

(OR)

25.	Determine the diameter of a bolt using maximum strain energy theory, which is subjected to an axial pull of 9 kN together with a transverse shear force of 4.5 kN. Elastic limit in tension is 225 N/mm2, factor of safety = 3 and Poisson’s ratio = 0.3.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AERODYNAMICS										      Time : 3 hours

Subject Code:	12AE204									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define uniform flow.

2.	The lift and drag force acting on an airfoil is _________ and_________ to the relative wind	 direction respectively.

3.	The motion in which vorticity is zero is said to be _________ (irrotational/rotational) flow.

4.	Define Stream line.

5.	What type of flow can be described by Laplace equation?

6.	Magnus effect can be explained from flow over a rotating circular cylinder. State true or false.

7.	Define geometric angle of attack.

8.	Draw the lift curve of cambered airfoil.

9.	Define the term Downwash.

10.	State Helmholtz theorem.

PART – B  (5 x 3 = 15 MARKS)

11.	Air at 300 km/h, 1atm and 30°C flows over a two-dimensional rectangular wing of chord 1.2m.  	If the zero lift angle of attack for the wing is –2° and Cl,α=0.10, calculate the lift acting on the 	wing per unit span when the angle of attack is 3°.

12.	Write the general form of energy equation for a two dimensional flow and explain it.

13.	Derive Kutta-Joukowski theorem.

14.	Describe Kutta condition.

15.	What is induced drag?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail about aerodynamic forces and moments.

(OR)

17. 	a.	Describe the physical meaning of  ∇⋅V.  										    (5)

b.	Explain the concept behind gradient of scalar field and divergence of vector field.  Assume two dimensional pressure field in Cartesian space for gradient of scalar field and velocity field for divergence of vector field.												  (10)

18.	a.	Consider a velocity field where the x and y components of velocity are given by 					u= Cxx2+y2 	v= Cyx2+y2  where C is constant.  Obtain the equations of streamlines.

(3)

b.	For velocity field given by u= Cx v=− Cy where C is constant.  Calculate the stream 			function and velocity potential.  Using your results, show that lines of constantφare 				perpendicular to lines of constantψ.  											  (12)

(OR)

19.	Derive the momentum equation in integral form using control volume approach in Cartesian coordinate system in (x, y, z) plane.

[P.T.O]

20.	Explain Rankine’s half-body with neat sketches.

(OR)

21.	Derive expressions for the velocity potential Φ and stream function ψ for a Doublet.

22.	Using the Biot-Savart’s law, derive the expression for induced velocity for straight line segment of vortex filament. Using this relation derive the velocity induced due to infinite line vortex.

(OR)

23.	Derive the fundamental equation for thin airfoil theory and give the assumptions that are made in thin airfoil theory.

24.	Derive the general form of Prandtl’s lifting line theory.

(OR)

25.	Explain the terms:

a.	Starting vortex.  																    (7)

b.	Bound vortex and horseshoe vortex.											    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	THERMAL ENGINEERING							      Time : 3 hours

Subject Code:	12AE207				 					       	       Maximum Marks: 100

#### Answer ALL questions

(Use of Mollier chart, refrigeration charts and steam tables is permitted.

Data can be assumed suitably if not provided)

PART – A  (10 x 1 = 10 MARKS)

1.	Define ideal gas.

2.	State law of corresponding states.

3.	The portion of a nozzle with minimum cross section is known as ___________.

4.	Name the two common types of compounding of steam turbines.

5.	For the same compression ratio and same heat input, the efficiency of Otto cycle is ___________ than that of a Diesel cycle.

6.	The Rankine efficiency of a good steam power plant is in the range of ___________.

7.	Define Stoichiometric air-fuel ratio.

8.	An analysis which includes the steam in the exhaust is called as ___________.

9.	One ton of refrigeration is equal to ___________kW.

10.	Name the different types of dust systems used in air-conditioning plants.

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by apparent molecular weight? Write the equation to estimate the same.

12.	Draw the parts of a steam injector and mention its use.

13.	Draw the p-V diagram of a Brayton cycle and indicate the various processes.

14.	State the method of conversion of volumetric analysis to weight analysis.

15.	A refrigerating system operates on reversed Carnot cycle. The higher temperature of the system is 35C and the lower is 15C. Determine the coefficient of performance of the system.

PART – C  (5 x 15 = 75 MARKS)

16.	A vessel contains at 1 bar and 20C a mixture of 1 mole of CO2 and 4 moles of air. Calculate for the mixture :

a.	The masses of CO2, O2 and N2 and the total mass.

b.	Percentage of Carbon content by mass.

c.	Apparent molecular weight and gas constant of the mixture.

d.	Specific volume of the mixture.

The volumetric analysis of air is taken as 21% Oxygen and 79% Nitrogen.

(OR)

17.	Air (N2 = 77%, O2 = 23% by weight) at 25C and 12 bar is contained in a vessel of capacity       0.6 m3. Some quantity of CO2 is forced into the vessel so that the temperature remains at 25C but the pressure rises to 18 bar. Find the masses of O2, N2 and CO2 in the cylinder.

18.	A convergent-divergent nozzle is to be designed in which steam initially at 14 bar and 80C of superheat is to be expanded down to a back pressure of 1.05 bar. Determine the necessary throat and exit diameters of the nozzle for a steam discharge of 500 kg/hour, assuming that the expansion is in thermal equilibrium throughout and friction reheat amounting to 12% of the total isentropic enthalpy drop to be effective in the divergent part of the nozzle.

[P.T.O.]

(OR)

19.	Explain the pressure compounding of steam turbines with a neat sketch. Also show the 	variation of pressure and velocity of steam moving through the turbine.

20.	An engine of 250 mm bore and 375 mm stroke works on Otto cycle. The clearance volume is 0.00263 m3.The initial pressure and temperature are 1 bar and 50C. If the maximum pressure is limited to 25 bar, find the following :

a.	Air standard efficiency 	b.	Mean effective pressure of the cycle.

Assume ideal conditions.

(OR)

21.	Explain the working of a power plant operating on reheat cycle with a diagrammatic layout and its T-S diagram.

22.	The following is the ultimate analysis of a sample of petrol by weight : Carbon = 85% ; Hydrogen = 15% . Calculate the ratio of air to petrol consumption by weight if the volumetric analysis of the dry exhaust gas is : CO2 = 11.5 % ; CO = 1.2 % ; O2 = 0.9 % and N2 = 86 %. Also find the percentage of excess air.

(OR)

23.	Explain the construction and means of analysis of combustion products in Orsat apparatus with a sketch.

24.	Explain the components and working of a practical vapour absorption system with a neat sketch.

(OR)

25.	Briefly explain the components and working of a central Air-conditioning system and list its advantages.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	WIND TUNNEL TECHNIQUES							      Time : 3 hours

Subject Code:	12AE209				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a wind tunnel?

2.	List the applications of spin tunnel.

3.	The speed range of smoke flow visualization is ________.

4.	Pitot tube measures the differential pressure of __________ and _________ pressure.

5.	What are the forces and moments measured using six component balance?

6.	What are external balances?

7.	Classify the supersonic wind tunnels.

8.	What are shock tubes?

9.	State the limitations of Schlieren techniques.

10.	What are the advantages of strain gauge balances?

PART – B  (5 x 3 = 15 MARKS)

11.	Classify the wind tunnels.

12.	Explain the tuft flow visualization technique.

13.	State the advantages of internal balances.

14.	What are disadvantages of an Induction-Type wind tunnel?

15.	What are differences between Schlieren and Shadowgraph methods.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the construction and operating principle of closed circuit subsonic wind tunnel with its advantages and disadvantages.

(OR)

17.	Explain the following special purpose tunnels.

a.	Smoke tunnel.																    (8)

b.	Water tunnel.																    (7)

18.	Describe the following flow visualization methods.

a.	Surface oil flow.																    (8)

b.	Smoke flow.																	    (7)

(OR)

19.	Explain the following:

a.	Pressure transducer. 															    (8)

b.	Pitot tube design. 															    (7)

20.	Explain the construction, calibration and working principle of a strain gauge balance.

(OR)

21.	Describe the following wind tunnel balances.

a.	Platform type.																    (5)

b.	Yoke-type.																	    (5)

c.	Pyramid-type. 																    (5)

[P.T.O.]

22.	Explain the construction and working principle of blow down type supersonic wind tunnel.

(OR)

23.	Describe the working of shock tube with its applications.

24.	Explain the Shadow graph flow visualization technique.

(OR)

25.	Describe the following:

a.	Pressure sensitive paints.														    (8)

b.	LDV.																		    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AIRCRAFT INSTRUMENTATION AND AVIONICS		      Time : 3 hours

Subject Code:	12AE211				 					              Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the use of night vision goggles?

2.	What is an autopilot?

3.	Mention the instruments working on Pitot-static system.

4.	What are the types of air speed?

5.	What is mean by electronic flight information system?

6.	Define Aircraft Heading.

7.    	Name the temperature measuring instruments.

8.   	Define Tachometer.

9.    	Name some ‘Illities’ of avionics.

10.  	A ___________ is a small computer on a single integrated circuit containing a processor core, memory, and a programmable input / output peripherals.

PART – B  (5 x 3 = 15 MARKS)

11.  Define HOTAS.

12.  	Compare TAS and EAS.

13.	Define precession and rigidity.

14.	Distinguish between engine instruments and navigation instruments.

15.	What is the need for two different speeds in ARINC 429 data bus?

PART – C  (5 x 15 = 75 MARKS)

16.   a.   Compare and contrast the display technologies LED, LCD, EL. 					    (5)

b.   What are the various types of CRTs used in civil and military aircraft and explain them in

detail.									 									  (10)

(OR)

17.  	a.   	Explain the basic principle of Head up display and what are its limitations? How are they

over come in Helmet mounted display?											  (10)

b.   	What are the special features of Direct voice input?								    (5)

18.   Explain with a neat sketch the working principle of Pitot-Static system.

(OR)

19.  	Write short notes on the following:

a.	Accelerometer.   																    (5)

b.   	Altimeter.       																    (5)

c.   	Vertical speed indicator.  							 							    (5)

20.  What is the basic working principle of gyroscope. How do they function? Give your answer 	with the aid of an illustration.

(OR)

[P.T.O.]

21.   Explain the following with neat sketches.

a.   Heading Indicator. 															    (5)

b.   Attitude Indicator.                                            									    (5)

c.   Turn & slip Indicator.															    (5)

22.  Write in detail about the various mechanical, magnetic and electrically operated Tachometers.

(OR)

23.  Write short notes on the following.

a. 	Pressure gauge.					  											    (5)

b. 	Fuel temperature gauge.				       										    (5)

c. 	Fuel flow meter.    					  										    (5)

24.  Explain the MIL STD 1553B data bus in detail bringing out clearly the bus architecture, 		protocol, word and message formats and coupling methods.

(OR)

25.  a.    Explain the interface of a seven segment LED with the microprocessor to display a binary

data.																		  (10)

b.   	List out a few advantages of digital systems over its analog counterpart.                    	    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AIRCRAFT STRUCTURES								      Time : 3 hours

Subject Code:	12AE212				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Sum of the distribution factor at the joint in moment distribution method is_________.

2.	The deflection of curved beams and spring can be found using _______ theorem.

3.	The basic structural elements in aircraft fuselage are________.

4.	The structural unit which transfers the concentrated loads to the shell of an airplane wing is 	________.

5.	When the loads act at the shear center they produce _________.

6.	The Bredt-Batho theory is given by_________.

7.	The buckling coefficient of the thin plate depends on__________.

8.	In Needham method, the structural member is divided into _________.

9.	FRP means ________.

10.	Fiber should have high _________ and __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between statically determinate and indeterminate structures.

12.	State the advantage of having a semi- cantilever type of wing construction.

13.	Write the expression for angle of twist per unit length for thin walled closed section subjected to torque T.

14.	List the possible failure modes of a thin stiffened sheet subject to compression.

15.	Define matrix.

PART – C  (5 x 15 = 75 MARKS)

16.	A continuous beam ABC is of length 10 m and is supported at A, B, and C. AB = 6 m and    BC = 4 m. A load of 3 kN is applied at 2 m from A and uniformly distributed load of  1 kN/m over the span BC. Assuming EI is constant; Find the reactions and moment at the support by using Clapeyron’s three moment equation method.

(OR)

17. 	A beam of length L and uniform EI is simply supported at its ends. It is subjected to a uniformly distributed load of intensity ‘q’ N/m. Compute the maximum deflection using Castigliano’s theorem.

18.	Explain monocoque and semi- monocoque type of fuselage construction.

(OR)

19.	What are the structural parts of an aircraft wing? List their functions.

20.	Find the shear flow distribution and locate the shear center for the section shown in  the figure subjected to vertical shear load of 60 kN at the shear center. The area of the each stringers are given by A1 = A2 = A3 = 2.5 cm2 and A4 = A5 = A6 = 5 cm2 .

[P.T.O.]

(OR)

21.	Determine the shear flow and the angle of twist per unit length of the two cell tube as shown in the figure. The section is subjected to a torque 10000 Ncm and t = 0.1 cm. Take G = 25x 105 N/cm2.

22.	Explain two methods to determine crippling strength.

(OR)

23.	a. 	Explain the concept of ‘effective sheet width’.									    (8)

b. 	Explain Inter Rivet Buckling.													    (7)

24.	Describe the Metal Matrix Composites (MMC) and its applications.

(OR)

25.	Write the properties of fiber and matrix and various applications of composite materials.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FLIGHT DYNAMICS									      Time : 3 hours

Subject Code:	12AE214									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define thrust and propulsive efficiency.

2.	Write the formula for power required and available.

3.	Define Range of an aircraft.

4.	Write down the conditions for unaccelerated, steady level flight.

5.	Define mean aerodynamic chord.

6.	What do you meant by inherently stable airplane?

7.	What is meant by weather cock effect?

8.	Define neutral point and static margin.

9.	Define stick fixed and stick free stability?

10.	What is autorotation?

PART – B  (5 x 3 = 15 MARKS)

11.	What causes induced drag?

12.	What are the conditions of maximum endurance and maximum range for a propeller driven 	aircraft in terms of lift and drag coefficients?

13.	Why the airplane is considered as a dynamic system in six degrees of freedom?

14.	Differentiate between yaw and sideslip angle.

15.	Distinguish between static and dynamic stability. Graphically represent a system which is statistically stable but dynamically unstable.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive the three equations of Motion which describe the translational motion of an 	airplane through three-dimensional space over a flat earth.						  	  (10)

b.	Discuss power available and power required curves for both jet and propeller driven airplanes.	  								   	                					    (5)

(OR)

17.	Explain the drag polar. Discuss in detail about the various types of drag in an airplane and methods of minimizing the drag.

18.	a.	An airplane weighing 180,000 N has a wing area of 45 m2 and drag polar given by

CD = 0.017 + 0.05 CL2

i.	Obtain the thrust required and power required for a rate of climb of 2,000 m/min at a 	speed of 540 kmph at 3 km altitude.

ii.	Obtain the rate of climb at 400 kmph at sea level if the trust available is 45,000 N.                                                           																			    (8)

b.	Explain V-n diagram and Load factor with a neat sketch.  				    			    (7)

(OR)

[P.T.O.]

19.	a.	A jet airplane has a weight of 922,140 N and wing area of 158 m2. The weight of the fuel 		and oil together is 294,300 N. The drag polar is given by CD = 0.017 +0.0663 CL2. Obtain 	the maximum range in constant CL flight at an altitude of 10 km. Assume TSFC to be

0.95 hr-1.												    	    				    (7)

b.	In straight and level flight show that the velocity corresponding to minimum power 	condition is 0.76 times the velocity corresponding to minimum thrust required condition. 																					    (8)

20.	Describe the criteria for longitudinal static stability. Explain the contributions of fuselage and wings to Cmcg and Cmα.

(OR)

21.	Describe the following:

a.	Aerodynamic balancing of a control surface.				    						    (5)

b.	Hinge Moment Coefficient.								    					    (5)

c.	Stick force gradients.						    			    						    (5)

22.	a.	Explain the effect of dihedral on static lateral stability with a neat sketch.		    	    (7)

b.	Discuss briefly the following: Aileron reversal and Adverse yaw.			   		    (8)

(OR)

23.	a.	Describe in detail basic requirements of the rudder. 					    			    (8)

b.	Explain the Rudder lock and what is the purpose of a dorsal fin and how does it achieve it?

(7)

24.	a.	Explain the various types of axes systems with neat sketches.				    		    (7)

b.	Discuss briefly the longitudinal stick free dynamic stability.				    		    (8)

(OR)

25.	a.	Write short notes on the following:  Dutch roll, Spiral divergence and Phugoid motion. (8)	b.	The lateral stability quartic for an airplane is λ4 + 16λ3 + 13.1 λ2 + 9.8λ + 0.73 = 0.

Extract the roots of this quartic. Obtain the time to double or halve the amplitude and the period of the oscillatory mode (s).    								    			    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	COMPUTATIONAL FLUID DYNAMICS				      Time : 3 hours

Subject Code:	12AE216		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define convergence.

2.	What is the need for staggered grid?

3.	What types of grids are used in FVM?

4.	Define discretisation and round off error.

5.	Explain cell centered method.

6.	Distinguish between vortex centered approach and cell centered approach in FVM.

7.	Define vortex panel method.

8.	What are the methods available for grid generation?

9.	What is convergence?

10.	What does it mean by semi implicit method?

PART – B  (5 x 3 = 15 MARKS)

11.	Mention the mathematical properties elliptical, parabolic and hyperbolic fluid dynamic equation.

12.	Elaborate the basic aspects of finite difference equations.

13.	Differentiate between structured and unstructured grid.

14.	Write down the conservative form of the Continuity equation and explain the terms involved.

15.	Write down an expression for substantial derivative in Cartesian coordinates.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the continuity equation for incompressible flow in differential form.

(OR)

17.	Derive the momentum equation for unsteady quasi one dimensional flow.

18.	Write down the procedure for the calculation of pressure coefficient distribution around a circular cylinder using the source panel technique.

(OR)

19.	Why is staggered grid adopted for incompressible flow for the MAC scheme? Outline the MAC method and show how the incompressible flow equation is obtained.

20.	Explain the grid generation technique based on PDE and summarize the advantages of elliptical grid generation methods.

(OR)

21.	State and explain the difference between explicit and implicit methods with suitable examples.

22.	How is the accuracy of the discretization process determined? What are the uses and difficulties of approximating the derivatives with higher order finite difference schemes?

(OR)

23.	Elaborate about quadrilateral Lagrange elements and Isoparametric elements in FEM.

[P.T.O]

24.	Outline the procedure for adaptive stepping with Runge-Kutta scheme and present the algorithm for Runge-Kutta method to simultaneous ordinary differential equation.

(OR)

25.	Explain the potential functional formulation of potential flow stating the boundary conditions using Galerkin approach. Find the velocity distribution along the vertical central line of the confined flow inside a cylinder.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AEROSPACE PROPULSION							      Time : 3 hours

Subject Code:	12AE218				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the purpose of dilution air in combustion chamber?

2	State the advantage of can type combustion chamber.

3.	How will you classify turbine blade cooling?

4.	Write down the difference between centrifugal and axial flow compressor.

5.	The ratio of useful power output to the total power output of the propulsion system is called as_________.

6.	Define uninstalled engine.

7.	What is the function of gear box in a turboprop engine?

8.	What are the factors affecting thrust?

9.	Define over expanded nozzle.

10.	What is choked nozzle?

PART – B  (5 x 3 = 15 MARKS)

11.	State the advantages and the disadvantages of annular combustor.

12.	Explain the difference between Impulse and reaction blade.

13.	What are the requirements of an aircraft intake?

14.	What are the factors to be considered while designing a supersonic inlet?

15. 	Define SFC. Write down its significance.

PART – C  (5 x 15 = 75 MARKS)

16. 	Illustrate the working principle of combustion chamber also explain the purpose of different

zones in the flame holder.

(OR)

17. 	a. 	What are the factors influencing the rate of reaction?								    (7)

b. 	Explain the effects of Chemical Variables on Flame Speed.						    (8)

18.	Explain the thermodynamic cycle of the centrifugal compressor. Also derive the work done of the centrifugal compressor?

(OR)

19.	Explain the following:																(3×5)

a.	Overall turbine performance.

b.	Losses in turbine blade.

c.	Limiting factor of gas turbine engine.

20.	Discuss the effect of shock waves in supersonic inlets.

(OR)

21.	a.	With the aid of a neat diagram, explain the working principle of ram jet engine.     		   (10)

b.	Draw the thermodynamic cycle of ram jet engine.                                                    		     (5)

22.	Derive the engine performance of real cycle for a turbofan engine.

[P.T.O.]

(OR)

23.	What is an afterburner and why is it used? Draw the T-S diagram of an ideal turbojet engine with an afterburner.

24. 	a.	Draw a neat sketch of a Turbo pump feed system employed in rockets.				     (7)

b.	An ideal rocket is designed to operate at sea level using a propellant whose products of combustion have a specific heat ratio of 1.3. Determine the required chamber pressure if the exit Mach number is 2.52. Also determine the nozzle area ratio between the throat and exit.

(8)

(OR)

25.	a.	How is regressive, neutral and progressive burning of the solid propellant grown achieved? 	Explain with the aid of a diagram.							 					   (10)

b.	State five advantages and five disadvantages of solid propellant rocket.   				    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	GAS DYNAMICS										      Time : 3 hours

Subject Code:	12AE220									       	       Maximum Marks: 100

#### Answer ALL questions

(Use of approved Gas tables permitted)

PART – A  (10 x 1 = 10 MARKS)

1.	For a calorically perfect gas , Cp ≠ Cp (T) and Cv = Cv (T) . (True / False).

2.	Pb > Pe for a ___________.

3.	When does Shock polar become circle?

4.	For a flow through a Prandtl – Meyer expansion wave, ___________ stays constant.

5.	Mach angle  = ________.

6.	Upper critical Mach number is the value of M∞ for which the entire flow around the body is  ___________.

7.	Write down correction factors involved in the pressure coefficient for an airfoil in the 	transonic flow.

8.	Define Mdrag divergence .

9.	Distinguish between camber and thickness of an airfoil.

10.	How will you control the wave drag effect on the rectangular wing placed in a supersonic flow?

PART – B  (5 x 3 = 15 MARKS)

11.	Air at stagnation condition has a temperature of 800 K. Find the stagnation velocity of sound and the maximum possible fluid velocity.

12.	Show that total pressure decreases across an oblique shock.

13.	Write the Prandtl – Glauret rule.

14.  Write the relation between Cl and Cd wave .

15.  What is the effect of thickness over the performance of wings?

PART – C  (5 x 15 = 75 MARKS)

16.  Derive the Prandtl equation for the normal shock.

(OR)

17.	The pressure and temperature and the Mach number at the entry of a flow passage are 2 bar, 275 K and 1.3 respectively. If exit Mach number is 2.4 determine;

a.	Velocity of sound at stagnation conditions.

b.	The maximum velocity.

c.	Temperature and pressure at exit.

d.	Critical Mach number at entrance and exit, Take  = 1.3 and R = 0.46 kJ/kgK.

18.	Derive the expression for change of speed with respect to Mach number for weak oblique  shock wave.

(OR)

19.	Derive Prandtl – Meyer function for a expansion fan created on a convex corner.

20.	Derive the small-perturbation equation, which is valid for subsonic and supersonic flows.

(OR)

21.	Discuss the similarity rule with a suitable example.

[P.T.O.]

22.	Derive the expression for pressure co-efficient of linearized supersonic flow.

(OR)

23.	Write short notes on:

a.	Lower and upper critical Mach numbers.					 					    (8)

b.	Shock induced separation.											 			    (7)

24.	Briefly explain the characteristics features of swept wings.

(OR)

25.	Write short notes on:

a.	Transonic area rule.															    (7)

b.	Tip effect.																	    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	HEAT AND MASS TRANSFER							      Time : 3 hours

Subject Code:	12AE221									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the driving force for heat transfer?

2.	Define overall heat transfer co-efficient.

3.	What is meant by transient heat conduction?

4.	Write the significance of Fourier number.

5.	Define Nusselt Number

6.	What is thermal boundary layer?

7.	The heat transfer by radiation takes place by means of _________.

8.	State Stefan-Boltzmann law.

9.	What is aerodynamic heating?

10.	Draw the heat transfer distribution of a thrust chamber.

PART – B  (5 x 3 = 15 MARKS)

11.	Calculate the rate of heat transfer per unit area through a copper plate 45 mm thick, whose one face is maintained at 350oC and the other face at 50oC. Take thermal conductivity of copper as 370 W/m2K.

12.	What is meant by lumped system analysis? When is it applicable?

13.	A hot plate 1 m x 1.5 m is maintained at 300oC. Air at 25oC blows over the plate. If the convective heat transfer coefficient is 20 W/m2K, calculate the rate of heat transfer.

14.	Assuming the Sun to be a black body emitting radiation with maximum intensity at λ = 0.49 μm. Calculate the surface temperature of the sun.

15.	What is mean by ablative heat transfer?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the general three-dimensional heat conduction equation in Cartesian coordinates.

(OR)

17.	A 150 mm steam pipe has inside diameter of 120 mm and outside diameter of 160 mm. It is insulated at the outside with asbestos. The steam temperature is 150oC and the air temperature is 20oC. Take the heat transfer coefficient at steam side and air sides as 100 W/m2oC and 30 W/m2oC respectively. Thermal conductivity for asbestos and steel are 0.8 W/moC and 42 W/moC. How thick should the asbestos be provided in order to limit the heat losses to 2.1 kW/m2?

18.	An egg with mean diameter of 40 mm and initially at 20oC is placed in a boiling water pan for 4 minutes and found to be boiled to the consumer’s taste. For how long should a similar egg for the same consumer be boiled when taken from a refrigerator at 5oC? Take the properties for egg k = 10 W/moC, ρ = 1200 kg/m3, Cp = 2 kJ/kgoC, and h = 100 W/m2oC.

(OR)

[P.T.O.]

19.	A 10 cm diameter apple approximately spherical in shape is taken from a 20oC environment and placed in  a refrigerator where temperature is 5oC and average heat transfer coefficient is 6 W/m2K. Calculate the temperature at the centre of the apple after a period of 1 hour. The physical properties of apple are ρ = 998 kg/m3 , Cp = 4180 J/kg K, k = 0.6 W/mK.

20.	Air at 20oC at atmospheric pressure flows over a flat plate at a velocity of 3 m/s. If the plate is   1 m wide and 80oC, calculate the following at 300 mm:

a.	Hydrodynamic boundary layer thickness.

b.	Thermal boundary layer thickness.

c.	Local friction coefficient.

d.	Average friction coefficient.

e.	Local heat transfer coefficient.

f.	Average heat transfer coefficient.

g.	Heat transfer.

(OR)

21.	A hot plate 20 cm in height and 60 cm wide is exposed to the ambient air at 30oC. Assuming the temperature of the plate is maintained at 110oC, find the heat loss from both surfaces of the plate. Assume a horizontal plate.

22.	Two large parallel plates are maintained at 600 K and 900 K and emissivities are 0.4 and 0.7 respectively. Determine heat transfer by radiation and also calculate percentage reduction in heat transfer and shield temperature when another plate of emissivity 0.05 introduced in between them.

(OR)

23.	Hydrogen gases at 3 bar and 1 bar are separated by a plastic membrane having thickness       0.25 mm. The binary diffusion coefficient of hydrogen in the plastic is 9.1 x 10-8 m2/s. The solubility of hydrogen in the membrane is 2.1 x 10-3  kg-mole/m3 bar. A uniform temperature condition of 20oC is assumed. Calculate:

a.	Molar concentration of hydrogen on both sides.

b.	Molar flux of hydrogen.

c.	Mass flux of hydrogen.

24.	Discuss in detail the factors influencing the injector behaviour.

(OR)

25.	Explain the cooling methods to reduce aerodynamic heating. Also explain the methods of cooling thrust chambers.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FINITE ELEMENT ANALYSIS							      Time : 3 hours

Subject Code:	12AE222		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the types of elements?

2.	__________are used for pin jointed structures in which only axial forces are transmitted.

3.	Name any two FEA software.

4.	Write down the expression of stiffness matrix for one dimensional bar element.

5.	__________ of the solution means that as FE mesh is refined, the computed solution approaches the exact solution.

6.	What is point load?

7.	What are the classifications of coordinates?

8.	State the properties of stiffness matrix.

9.	What is the characteristic matrix of 1D potential flow element?

10.	What is a truss?

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by discretization and assemblage?

12.	Distinguish between potential energy function and potential energy functional.

13.	Define shape function.

14.	What is essential boundary condition?

15.	What is the difference between explicit and implicit solution of assembled matrix?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail the various procedures for the formulation of finite element problem.

(OR)

17.	Find the unknown values (X, Y, Z) of the following equation using Gauss elimination method. 3X + 2Y + Z = 10, 2X + 3Y + 2Z = 14, X + 2Y + 3Y = 14.

18.	For the two bar truss shown in the figure determine the displacement of node 1 and stress in 	element 1-3.

(OR)

[P.T.O.]

19.	Determine the support reaction forces at the two ends of the bar shown in the figure. Take P = 60KN, E = 20KN/mm2, A = 250mm2, L = 150mm,   ∆ = 1.2mm.

20.	A thin plate of uniform thickness of 25mm is subjected to a point load of 25N at mid depth, the plate is also subjected to a self-weight. If E = 2*105 N/mm2, ρ=.8*10-4 N/mm3, calculate Nodal displacement.

(OR)

21.	Derive the stiffness matrix for a constant strain triangular element.

22.	A bridge is supported by several concrete piles and the geometry and loads of typical piles. The load 20 KN/m2 represents weight of the bridge and an assumed distribution of the traffic on the bridge. The concrete weight approximately 25 KN/m3 and its modulus f is 28 x106 KN/m2. Analyse the pile for stress and displacement by FEM having more than one element.

(OR)

23.	Determine the stiffness matrix for the CST shown in the figure. Assume plane stress conditions. Coordinates are in mm. Take E = 210 GPa , υ = 0.25, t = 10 mm.

24.	Explain the basic theory about bending behavior of plates and how it can be solved by FEA.

(OR)

25.	Solve the given problem for temperature distribution. Consider the end convection coefficient. Take T1=1700 C , h1= 0.0025 w/ cm2 0C, h2= 0.0625 w/ cm2 0C,  Tα = 250C, l = 10 cm, k = 0.17 w/ cm0C.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	GUIDANCE AND CONTROL OF AEROSPACE VEHICLES														      								    Time : 3 hours

Subject Code:	12AE223				 						      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the essential elements of inertial navigation?

2.	Vor is type of vfr technique. (True/False).

3.	What is guidance?

4.	What is adaptive control?

5.	Define control law.

6.	Control of the helicopter in flight is established by controlling the ________ angle with the means of __________.

7.	Define closed loop transfer function.

8.	State the principle of Optimality.

9.	What are the main parts of a launch vehicle?

10.	What are reaction wheels?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the types of terrain reference navigation?

12.	What is terminal guidance?

13.	Define fly by light with diagram.

14.	Draw a typical pitch control system for a rocket.

15.	Draw the launch sequences.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	What are the various types of radio navigation used? Explain in detail. 				    (7)

b. 	Write a short note on GPS. 													    (8)

(OR)

17.	a. 	What are accelerometers? Define its types.	 									    (8)

b. 	Explain the working of ring laser gyroscopes. 									    (7)

18.	Explain minimum energy orbital transfer.

(OR)

19.	Explain the strategy of optimal terminal guidance.

20.	How are the conventional hydro-mechanical controls replaced by fly by wire? Explain in detail.

(OR)

21.	What are autopilots? Give brief note on the different types of autopilots and their systems.

22.	How are guidance systems of missiles classified? Elaborate.

(OR)

23.	Define single axis closed loop model of attitude control.

24.	Give a detailed note on stabilization of spacecrafts.

(OR)

25.	Give a detailed note on control strategies.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CRYOGENIC PROPULSION  							      Time : 3 hours

Subject Code:	12AE228									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	__________ is the temperature below which the cryogenic range begins.

2.	Boiling point of Helium is __________.

3.	What are the devices that can be used to modify the ideal thermodynamic system into actual 	system?

4.	What is maximum inversion temperature?

5.	In an expanded foam insulation, the heat is transferred only by__________.

6.	The liquid boils off continuously due to __________.

7.	Name the two types of cryogenic valves.

8.	Cryogenic instrumentation should be capable of __________ operation from outside.

9.	__________ is the heat exchange medium in regenerative heat exchanger.

10.	Mechanical expander is used in __________ cycle.

PART – B  (5 x 3 = 15 MARKS)

11.	Define superconductivity.

12.	What is Joule-Thompson effect?

13.	What is multilayer insulation?

14.	Write the design principle of Dewar vessel.

15.	What is a Cryocooler? On what cycle it operates?

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the peculiar properties of Helium with neat sketches of T-S and Phase diagram.

(OR)

17.	With the help of neat sketch and T-S diagram explain thermodynamically ideal liquefaction system.

18.	Explain Linde-Hampson Liquefaction system with a neat sketch and T-S diagram. Derive the expressions for Liquid yield and work requirement.

(OR)

19.	Explain Claude system of Liquefaction with T-S diagram. Derive the expressions for Liquid yield and work requirement.

20.	Explain in detail about expanded-foam insulations and gas-filled powders and fibrous insulations used in cryogenic systems.

(OR)

21.	Compare the advantages and disadvantages of various insulations used in cryogenic systems.

22.	With a neat sketch explain the construction of a Dewar vessel for storing cryogens.

(OR)

23.	Discuss the types of cryogenic fluid transfer lines.

[P.T.O]

24.	With a neat schematic diagram explain the working process of magnetic refrigeration system and also state its advantages and disadvantages.

(OR)

25.	Discuss the applications of cryogenics in space.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INTRODUCTION TO NON DESTRUCTIVE TESTING	      Time : 3 hours

Subject Code:	12AE229				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	__________   is fracture that involves little or no plastic, or permanent, deformation.

2.	What is casting?

3.	Vapor degreasing is a ____________ process.

4.	ISO 23279, Non-destructive testing of welds belongs to _____________ NDT test.

5.	Define Radiography.

6.	What is the function of stationary unit?

7.	Define Wavelength.

8.	List out the couplants in Ultrasonic testing.

9.	What is cavity radiator?

10.	Define hit data set.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the reasons for a crack in a material?

12.	List out the linear measuring instruments in visual inspection method.

13.	What is intensifying screen?

14.	What are the variables considered in the Ultrasonic testing?

15.	Write the advantages of the acoustic emission testing.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss about the conditions for effective NDT (Non-destructive testing).

(OR)

17.	Explain briefly about the corrosion induced discontinuities and geometric discontinuities.

18.	Write in detail about the equipment and accessories in visual inspection testing.

(OR)

19.	Explain the procedure for Penetrant test.

20.	Write about the characteristics of radiation in the radiographic test.

(OR)

21.	Explain in detail about the variables in the radiographic test.

22.	Discuss about the evaluation of results from Ultrasonic testing.

(OR)

23.	Explain briefly about the characteristics of Eddy current flow.

24.	Discuss about the theory and principles behind Thermal infrared testing.

(OR)

25.	Briefly explain about the sensing and measurements in Acoustic Emission Testing.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	SPACE DYNAMICS									      Time : 3 hours

Subject Code:	12AE232									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The gradual transition from vertical to horizontal flight is caused by the force of gravity, and it is called ___________.

2. 	Define specific impulse for a rocket engine.

3. 	Write down the expression for time versus true anomaly.

4. 	The semi-minor axis b is related to the semi-major axis a by the relationship of _________.

5. 	Write down the expression for escape velocity.

6. 	The change in longitude in a geosynchronous circular orbit is given by the equation _________.

7. 	Define orbit perturbations.

8. 	For a typical communication satellite at geosynchronous altitude, the eccentricity may vary from ___________ to ____________ in six months as a result of solar radiation pressure effects.

9. 	The sphere of influence with respect to A1 is defined by the condition ________.

10. 	The mean distance between the Earth and the moon is _________ km, and the mass of the moon is __________ that of the Earth.

PART – B  (5 x 3 = 15 MARKS)

11. 	Write down the relation between escape velocity and circular velocity.

12. 	What are the basic assumptions made in the solution of circular restricted three body problem?

13. 	What are the different types of perturbing forces acting on artificial Earth satellites?

14. 	Write short notes on sun-synchronous orbits.

15. 	Distinguish between flyby mission and lander mission.

PART – C  (5 x 15 = 75 MARKS)

16. 	Derive the thrust equation of a rocket.

(OR)

17.	A sounding rocket of initial mass m0 and mass mf after all propellant is consumed is launched

vertically (γ= 90°). The propellant mass ﬂow rate me is constant.

a.	Neglecting drag and the variation of gravity with altitude, calculate the maximum height h 		attained by the rocket.

b.	For what ﬂow rate is the greatest altitude reached?

18.	State and prove the Kepler’s first law of planetary motion from the fundamental principles of motion.

(OR)

19.	Explain the concept of the celestial sphere with the neat sketch.

20.	Describe Encke's Method for the computation of perturbed satellite orbits.

(OR)

21.	Discuss the elementary aspects of perturbed orbits.

[P.T.O.]

22.	Describe the effects of Earth's Oblateness.

(OR)

23.	A satellite is to be launched into a sun-synchronous circular orbit with period of 100 minutes.              Determine the required altitude and inclination of its orbit.

24.	Explain the concept of Hohmann transfer for the coplanar and non-coplanar trajectories.

(OR)

25.	Explain the geometry of the two-dimensional interplanetary trajectories.