1Reg.No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BASIC INDUSTRIAL BIOTECHNOLOGY				      Time : 3 hours

Subject Code:	09BT201									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The equipment which provides controlled environment for the growth of microbes is called as ___________.

2.	The invading of a process by foreign microbes is called as ___________.

3.	__________ organisms is used for the fermentation of  L-glutamic acid.

4.	The molecular formula for ethanol is __________.

5.	Give two examples for secondary metabolites

6.	Name any two antifoaming agents

7.	What are bacterocin?

8.	List out some of the uses of Xanthan gum

9.	What are vaccines?

10.	What do you mean by Monoclonal antibodies?

PART – B  (5 x 3 = 15 MARKS)

11.	Illustrate the importance of block diagram.

12.	What are amino acids?

13.	Brief on secondary metabolites.

14.	Write a note on the applications of PHB.

15.	Write down the importance of recombinant proteins with examples.

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat sketch explain the process flow diagram of a fermentation process.

(OR)

17.	Write detailed notes on traditional and modern biotechnology with application.

18.	Explain the process flow chart for the production of citric acid.

(OR)

19.	Explain in detail about the production of L- Lysine with a neat flow diagram.

20.	What are the steps involved in the production of cephalosporin? Explain in detail.

(OR)

21.	Explain various steps involved in the production of vitamin B12.

22.	Write short notes on various types of proteases and their applications.

(OR)

23.	What is SCP? Explain in detail various methods of mass culturing of SCP.

24.	What are recombinant vaccines? Illustrate with a neat diagram the production of subunit vaccines.

(OR)

25.	Give an account on the application and products of plant cell culture and animal cell culture.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	PRINCIPLES OF CHEMICAL ENGINEERING			      Time : 3 hours

Subject Code:	09BT203				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Density is the ratio of __________.

2.	Define Molarity.

3.	___________is a stream bled off to remove an accumulation of inerts or unwanted material that might otherwise build up in the recycle stream.

4.	Define specific heat.

5.	Viscosity of a liquid tends to __________ with increase in ___________.

6.	State hydrostatic law.

7.	Which flow meter is known as variable area flow meter?

8.	What is the purpose of gaskets in the valves?

9.	__________ pump is used for transporting sludge.

10.	Vacuum is a region with a gaseous pressure much less than __________  pressure.

PART – B  (5 x 3 = 15 MARKS)

11.	Calculate average molecular weight of air as density 1 kg/ m3 at 273 k and 101.325 kPa.

12.	Distinguish between limiting and excess reactant.

13.	Prove that Reynolds number is a dimensionless number.

14.	What is meant by void fraction?

15.	Give a short note about priming.

PART – C  (5 x 15 = 75 MARKS)

16.	Two engineers calculated average molecular weight of a gaseous mixture containing N2, O2 and CO2. One engineer used correct values of molecular weight and found to be 28.98 g/mol. Other engineer used incorrect values for molecular weight of N2 and O2 as 14 and 16 g/mol respectively and found to be 14.55 g/mol. Calculate the composition of gas.

(OR)

17.	A solution is prepared from 50% v/v of 0.25N K2SO4. Express its concentration for the following:																		(5x3)

a.	Normality.		b.	Molarity.		c.	Morality.		d.	Mole fraction.

e.	Percentage w/v.

18.	A distillation column separates 10,000 kg/hr of a mixture containing equal mass of benzene and toluene. The product (D) recovered from the condenser at the top of the column contains 95% benzene and the bottom (W) from the column contains 96% toluene. The vapor (V) entering the condenser from the top of the column is 8000 kg/hr. A portion of the product from the condenser is returned to the column as reflux (R) and the rest is withdrawn as the final product (D). Find the ratio of the amount refluxed (R) to the product withdrawn (D).

(OR)

[P.T.O.]

19.	Calculate the amount of heat for a steam boiler with two input stream and one output stream with the following data.

| Stream | Enthalpy | Flow rate | Temperature |
| --- | --- | --- | --- |
| Input 1 | 125.7 kJ/kg | 120 kg/min. water | 30°C |
| Input 2 | 272 kJ/kg | 175 kg/min. | 65°C |
| Output | 2793.4 kJ/kg | 295 kg/min. saturated steam | 204 °C |

20.	a.	Show that the discharge (Q) consumed by an oil ring depends upon the internal diameter of 	the ring (D), rotational speed (N), density (ρ), viscosity (µ), surface tension (σ) and the 	specific weight of oil (w) by using Buckingham’s π-theorem method.				  (10)

b.	Summarize the various types of pressure measurements devices used in bioprocess 	industries.																	    (5)

(OR)

21.	a.	Starting from basic mass balance equation of fluids, derive continuity equation for 	compressible and incompressible fluids. 										  (10)

b.	Discuss in detail about various losses in a pipe with diagram.						    (5)

22.	Elaborately describe with a neat diagram about venturi meter, orifice meter and pitot tube with its applications.

(OR)

23.	a.	Derive an expression for Ergun equation starting from basic principles of fluidization	. (10)

b.	Explain the different types valves used in industries.								   (5)

24.	Discuss in detail about the working and construction of positive displacement and negative displacement pumps with a sketch.

(OR)

25.	Brief a note on the following:														(3x5)

a.	Compressors.

b.	Fans.

c.	Blowers.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	UNIT OPERATIONS									      Time : 3 hours

Subject Code:	09BT205									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The driving force for heat transfer is ____________.

2.	____________ law of heat conduction states that heat flux is proportional to temperature gradient.

3.	____________ Condensation occurs on contaminated surfaces and when impurities are present.

4.	Substances that have emissivity of less than 1.0 are called ____________.

5.	1-4 shell and tube heat exchanges have ____________ tube passes.

6.	In evaporation per kg vapor evaporated per kg steam used is called ____________.

7.	The separation of dilute slurry or suspension by gravity settling into a clear fluid and slurry of higher solids content is called ____________.

8.	In settling, if the fall of the particle is not affected by container wall and other particles then we call this as ____________.

9.	For vigorous agitation with vertical agitators, baffles are used to reduce ____________.

10.	Kinematic similarity can be defined in terms of ratios of ____________or of ____________.

PART – B  (5 x 3 = 15 MARKS)

11.	State Fourier’s law of heat conduction.

12.	Define Emissivity and Kirchhoff’s law.

13.	What is Logarithmic mean temperature difference?

14.	Write a short note of filter aids.

15.	Name various types of agitators with diagrams.

PART – C  (5 x 15 = 75 MARKS)

16.	A cold-storage room is constructed of an inner layer of 12.7 mm of pine, a middle layer of 101.6 mm of cork board, and an outer layer of 76.2 mm of concrete. The wall surface temperature is 255.4K inside the cold room and 297.1K at the outside surface of the concrete.  The conductivities for pine, 0.151; for cork board, 0.0433; and for concrete, 0.762 W/m. K. Calculate the heat loss in W for 1 m2 and the temperature at the interface between the wood and the cork board.

(OR)

17.	Derive the expressions for heat conduction through a hollow cylinder and a hollow sphere.

18.	The heat transfer coefficient or rate of heat transfer by forced convection to a fluid flowing in a pipe is found to be influenced by the velocity u, density ρ, specific heat C, thermal conductivity k , viscosity 𝜇 as well as the inner diameter of the pipe D. Establish the form of a dimensionless equation for the heat transfer coefficient.

(OR)

19. 	a.	Explain in detail the mechanism of boiling.                                                        		    (8)

b.	Explain in detail the concepts of black body and grey body.                             		    (7)

[P.T.O.]

20. 	a.	A heavy hydrocarbon oil which has a Cpm = 2.30kJ/kg.K is being cooled in a heat                             	exchanger from 371.9 K to 349.7 K and flows inside the tube at a rate of 3630kg/h. A  	flow of 1450kg water/h enters at 288.6 K for cooling and flows outside the tube.           	Cpm= 4.187KJ/kg.K for water.

i.	Calculate the water outlet temperature and heat-transfer 	area of the overall 				Ui=340W/m2.K and the streams are countercurrent.

ii.	Calculate for parallel 	flow.                                                                        		  (10)

b.	Write an elaborate note on Fouling factor.                                                        		    (5)

(OR)

21. 	a.	Explain in detail Heat and material balances for evaporators.                             		    (7)

b.	Discuss in detail the types of evaporators with diagrams.                                       		    (8)

22.	a.	Write briefly the theory of filtration.                                                                      		    (8)

b.	Write in detail about the separation of liquids in a centrifuge.                             		    (7)

(OR)

23. 	Explain in detail the construction and working of plate and frame filter press with neat diagram.

24. 	a.	Explain briefly about Agitator scale up.                                                               		    (8)

b.	Explain in detail mixing of powders viscous material and pastes.                            	    (7)

(OR)

25. 	a. 	Discuss in detail about agitation equipments with a neat diagram.                        		    (9)

b. 	Elaborate on power for agitation.                                                               			    (6)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MASS TRANSFER OPERATIONS						      Time : 3 hours

Subject Code:	09BT206				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How the JA and NA are related?

2.	Define diffusion coefficient.

3.	State Raoult’s law.

4.	Define q.

5.	What is the relation between mole fraction and molal ratio?

6.	What is meant by stripping?

7.	What is an adsorption hysteresis?

8.	List out any two applications of ion exchange in biological industries.

9.	Mention some advantages of extraction of a solid at an elevated temperature.

10.	What is meant by extraction factor (ε)?

PART – B  (5 x 3 = 15 MARKS)

11.	Define permeability.

12.	Distinguish between distillation and fractionation.

13.	What happens to the heat released during gas absorption?

14.	State Freundlich adsorption isotherm and its application.

15.	Represent the ternary system in a triangular diagram.

PART – C  (5 x 15 = 75 MARKS)

16.	A small diameter tube closed at one end was filled with acetone to within 18mm of the top and maintained at 290 K with a gentle stream of air blowing across the top. After 15000 s, the liquid level had fallen to 27.5 mm. The vapour pressure of acetone is 21.95 k N/m2 and total pressure is 99.75 k N/m2. Calculate the diffusivity of acetone in air. Density of acetone 790 kg/m3.

(OR)

17.	Derive an expression of flux for non-diffusing component at steady state.

18.	Write a short note on the following:												 	(5x3)

a.	Simple distillation.

b.	Steam distillation.

c.	Flash distillation.																    	d.	Azeotropic distillation.

e.	Extractive distillation.

(OR)

19.	A liquid containing 0.4 mole fraction methyl alcohol and 0.6 mole fraction water is fed to an intermediate point on a continuous distillation column containing eight actual sieve trays. The distillation composition is 0.93 mole fraction methyl alcohol and the bottom composition is 0.10 mole fraction methyl alcohol when the reflux ratio is 2.0. The column is operated at atmospheric pressure and the feed enters at its boiling temperature. If the Mc Cabe-Thiele simplifying assumptions may be made, what is the overall plate efficiency?

[P.T.O.]

20.	Acetone is to be recovered from an air mixture containing 0.015 mole fraction of acetone in an absorption tower. Water enters the tower counter-current to air mixture. The air mixture enters the absorber at a flow rate of 2 kg/m2sec and the rate of water is 3.2 kg/m2sec. Assuming

ye = 1.75x. Determine the number of overall transfer units, if the outlet air mixture has a mole fraction of 0.00015; where x and y are in the units of mole fraction.

(OR)

21.	a.	Spell out the comparison of packed and plate tower.								  (10)

b.	What is the physical significance of NTU?										    (5)

22.	Describe in detail about different types of adsorbent preparation, characterization and application in industries.

(OR)

23.	Briefly discuss about the ion exchange equipments used in biotech industries with advantages and disadvantages.

24.	Which type of extractor is used for nuclear industries? Explain the working principle and construction with a neat diagram.

(OR)

25.	Recall the definition of the following:												(5x3)

a.	Solid leaching.

b.	Lixiviation.

c.	Elutriation.

d.	Decoction.

e.	Shanks.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title : 	BIOORGANIC CHEMISTRY							      Time : 3 hours

Subject Code:	09BT209										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The (+) sign indicates the substances that are ____________.

2.	How many constitutional isomers can you draw for C2H6O?

3.	Draw the structure of a peptide bond.

4.	Write the significance of active site.

5.	Which of the following C-terminal peptide bonds would be more readily cleaved by carboxypeptidase A ?

a.	Ser-Ala-Phe         b.	Ser-Ala-Asp           	c.	Val-Leu-Ile          d.	Val-Leu-Asp

6.	Serine proteases do not catalyze hydrolysis if the amino acid at the hydrolytic site is a D-amino acid.     - True or False?

7.	FAD is obtained by an enzyme-catalyzed reaction that uses FMN and ATP as substrates.

- True or False?

8.	Which of the following group can be transferred by N5-Methyl-Tetrahydrofolate?

a.	-CH3			b.	-CH2				c.	-CH               	d.	-CH=O

9.	The association between polypeptides can be obtained by _________ structure of proteins.

10.	What are the bonds involved in protein folding?

PART – B  (5 x 3 = 15 MARKS)

11.	Define E and Z isomerisms with an example each.

12.	What is the importance of disulfide bond?

13.	Draw the structure of NADP+  and name its parts.

14.	Define Co-enzymes, citing a few examples.

15.	Discuss the role of enthalpy in protein folding.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the following with suitable examples.

a.	SN1 and SN2 mechanisms.    													    (8)

b.	E1 and E2 mechanisms.          													    (7)

(OR)

17.	Describe in detail the principle, components, and applications of polarimeter.

18.	Give a detailed account of Covalent bond, Coordinate Covalent bond and Ionic bond.

(OR)

19.	Comment on the following in some detail.

a.	Gibb’s free energy.            														    (8)

b.	Free radicals.                     														    (7)

20.	Discuss the evidence of formation of oxocarbonium ion in lysozyme-catalysed reaction and explain the molecular mechanism of lysozyme-catalyzed reaction.

(OR)

21.	Bring out the types of protease and explain the mechanism of action of zinc protease.

[P.T.O.]

22.	Discuss the methods of peptide synthesis in detail.

(OR)

23.	Explain the overall mechanism of acid-catalysed ester hydrolysis.

24.	Elaborate the basic methods and the associated kinetics of protein folding.

(OR)

25.	Give an account of the principle, instrumentation and applications of NMR spectroscopy.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title : 	CELL BIOLOGY									 	     Time : 3 hours

Subject Code:	09BT210										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Actin is present in all __________ cells.

2. 	__________ and __________ types of reproduction takes place in mitosis and meiosis.

3. 	What is Endocyosis?

4. 	An example for an Antiporter is __________.

5. 	An example for autocrine agent is the __________ in monocytes.

6. 	Nuclear receptors have molecular masses between __________ and __________.

7. 	Define cell signaling.

8. 	__________ is the process of addition of a phosphate group to a protein.

9. 	What is Explant?

10. 	Give any two common cell lines.

PART – B  (5 x 3 = 15 MARKS)

11.	Give any three differences and similarities between eukaryotic and prokaryotic cells.

12. 	Highlight the importance of sodium potassium pump in transport mechanism.

13. 	Explain about quantification and characterization of receptors.

14. 	Write about the importance of G proteins in signal transduction.

15. 	What are the techniques in cell culture? Explain.

PART – C  (5 x 15 = 75 MARKS)

16. 	Give  a detailed explanation on structure and functions of Cell and its Organelles.

(OR)

17. 	Explain about Cell cycle in detail.

18. 	Explain in detail about Active transport mechanism.

(OR)

19. 	Write about ion-selective gated channel in detail.

20. 	Explain about Receptors and its types with examples.

(OR)

21. 	Write a detailed account on Models for Cell signaling.

22. 	What is Signal amplification? Discuss about its different models in detail.

(OR)

23. 	Explain about Role of Calcium ion flux and protein kinases in cell signaling.

24. 	Explain about generation, maintenance and characterization of cell lines.

(OR)

25. 	Write about Immunochemistry in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MICROBIOLOGY										      Time : 3 hours

Subject Code:	09BT211									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the difference between aerobic and anaerobic bacteria.

2.	True/False: Multiplication of bacteria cell begins in lag phase.

3.	Mention four important characteristics of Mycoplasma.

4.	What is a fastidious organism? Give an example.

5.	Define sterilization.

6.	Define Lawn culture.

7.	List out the composition of peptidoglycan.

8.	Give examples on opportunistic infections.

9.	Give characteristic feature of Aspergillosis.

10.	What is germ tube?

PART – B  (5 x 3 = 15 MARKS)

11.    Explain the principle of Gram staining experiment.

12.    Draw a neat diagram of bacterial cell and label the parts.

13.    Mention the factors that influence the growth of the bacteria.

14.    List the antibiotic and their antibacterial targets.

15.   What is rhizosphere? Describe about the factors affecting microbial flora of the rhizosphere.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the working principle of microscope. 									    (7)

b.	Draw and label the parts of microscope. 										    (8)

OR)

17.	Elaborate in detail about staining techniques for microscopic identification of microorganisms.

18.	Discuss about candidiasis.

(OR)

19.  Write a note on mycoses in detail.

20.  Briefly explain about the bacterial growth curve.

(OR)

21.	Calculate the generation time: Bacillus cereus divides every 30 minutes. You inoculate a culture with exactly 100 bacterial cells. After 3 hours, how many bacteria are present?

22.	List the chemical agents and their uses in sterilization.

(OR)

23.	Explain the mechanism of resistance in bacteria.

24.	Describe the industrial production of ethanol.

(OR)

25.	Give an account on the role of biofertilizers in soil fertility and agriculture.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MOLECULAR BIOLOGY								      Time : 3 hours

Subject Code:	09BT213							       			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Which of the following crosses will not produce heterozygous individuals?

a.	RRxrr		b.	RrxRr		c.	rrxRR		d.	RRxRR

2.	If a culture contains both F+ and F- cells, what is expected to happen over time?

a.	No conjugation, because conjugation occurs only between cells that are F+

b.	No conjugation, because conjugation occurs only between cells that are F-

c.	All F+ cells will be converted to F-

d.	All F- cells will be converted to F+.

3.   Which of the following are helical structures of DNA?

a. 	H-DNA, F-DNA, Z-DNA.				b. 	F-DNA, B-DNA, H-DNA.

c. 	Z-DNA, B-DNA, H-DNA.				d. 	B-DNA, Z-DNA, F-DNA.

4.	The strands of double stranded DNA cannot separate and allow DNA polymerase to replicate them if

a.	A helicase is not present at the replication fork.

b.	The polymerase synthesizes DNA in a 5’ to 3’ direction.

c.	The bases of the parental DNA strand are covered with SSB proteins.

d.	No primase is present.

5.	What sort of genes are usually transcribed by RNA polymerase II in humans?

a.	Genes for 5S ribosomal RNA.

b.	Genes for large ribosomal RNA molecules.

c.	Genes for enzymes involved in cellular metabolism.

d.	Genes for transfer RNA.

6.	What sequence of DNA in the coding strand would result in a newly transcribed strand of RNA with the base pairs 3’-U A C G U C U-5’?

a.	5’-U A C G C U -3’					b.	5’ A T G C G A -3’

c.	3’ A T G C G A-5’					d.	5’-T A C G C T-5’

7.	The three prime untranslated region(3'-UTR) is the section of

a.	mRNA.								b.	Promoter region.

c.	Nucleotides.							d.	Basepairing.

8.	DNA methylation is associated with

a.	CpG islands.							b.	CAT box.

c.	TATA box.							d.	Increasing gene transcription.

9.	With respect to the LAC operon, if both glucose and lactose are present and glucose is low, which of the following is NOT true?

a.	High CAP.							b.	Increased uptake of lactose.

c.	Low cAMP.							d.	Increased transcription of the lac operon.

10.	In the absence of tryptophan, the trp repressor is

- Active and can bind to the operator

- Active and cannot bind to the operator

- Inactive and cannot bind to the promoter

- Inactive and cannot bind to the operator

[P.T.O.]

PART – B  (5 x 3 = 15 MARKS)

11.  Give the schematic representation for the mechanism of bacterial conjugation.

12. 	State the functions of Centromeres and Telomeres.

13. 	What is self – splicing? Mention their classification.

14. 	Distinguish between Intragenic vs. Intergenic Suppression.

15. 	Mention the Regulated stages of gene expression.

PART – C  (5 x 15 = 75 MARKS)

16.	Elucidate the method involved in Hershey and Chase experiment with diagram.

(OR)

17. 	Explain the three different ways of genetic recombination with suitable diagram.

18.	Elucidate the steps involved in D-loop and rolling circle mode of replication with suitable diagram.

(OR)

19. 	Elucidate the mechanism involved in repairing the damaged bases to maintain their integrity.

20.	Explicate the significance of post transcriptional modification and mention the steps involved in them.

(OR)

21. 	Illustrate the three main stages involved in processing of tRNA molecules during transcription with diagram.

22. 	Exemplify the process of translation in prokaryotes with diagram.

(OR)

23. 	Enlist the various inhibitors involved in protein synthesis and mention their mechanism with

a diagram.

24. 	How did eukaryotic organisms become so much more complex than prokaryotic without whole genes? Explain the role of transcription factors in gene regulation.

(OR)

25. 	Give an account on the control of gene expression by means of regulatory proteins.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BIOPROCESS PRINCIPLES							     Time : 3 hours

Subject Code:	09BT218									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is fermentation?

2.	Name some equipments used for measuring inlet gas.

3.	What is enumeration medium?

4.	What is simple media?

5.	Define sterilization.

6.	Classify sterilization techniques.

7.	What is Yx/s?

8.	Define degree of reduction.

9.	What are the different approaches to modeling of cell growth?

10.	What is substrate inhibition?

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly explain about the devices used for measuring biomass.

12.	Write short notes on carbon source.

13.	Give short notes on dry heat sterilization and moist heat sterilization.

14.	Find out degree of reduction of the following; CH4, C2H6, CH3COOH

15.	Write short notes on fed batch cultivation.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain about the production of ethanol with a neat flow sheet.

(OR)

17.	Give a detailed account on instruments which are used for measuring temperature in fermenter.

18.	Classify media and add a note on any four types of it.

(OR)

19.	Write in detail about energy sources for the growth of microorganisms.

20.	Derive expression for I order sterilization kinetics.

(OR)

21.	Explain about filter sterilization in detail.

22.	Write down the procedure for writing elemental balance with an example.

(OR)

23.	The chemical reaction for the conversion of ethanol to acetic acid is given below;

C2H5OH +O2  → CH3COOH +H2O

Acetic acid is produced from ethanol during the growth of acetobacter acetate which has the composition of CH1.8O0.5N0.2.The biomass yield from the substrate is 0.14 g/g. The product yield from the substrate is 0.92 g/g. NH3 is the Nitrogen source. Find out stoichiometric coefficient of oxygen.

[P.T.O]

24.	Give an account on Monod model.

(OR)

25.	Explain about batch and continuous operation and also add a note on its advantages and disadvantages.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ENZYME ENGINEERING AND TECHNOLOGY		     Time : 3 hours

Subject Code:	09BT220									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The first Enzyme Commission, gave its report in __________.

2.	Leather industry is one of the major market for __________.

3.	In 1958, __________ suggested a modification to the lock and key model.

4.	Positive allosteric modulation is also known as __________.

5.	Bacterial enzyme such as __________, induction with lactose a-priori will give better yield, since lactose turns on this enzyme in E. col.

6.	Fusion proteins can be constructed with __________ or maltose binding protein or other proteins to facilitate the purification of the protein.

7.	Enzyme molecules are __________ bonded to each other to create a matrix consisting of almost only enzyme.

8.	A __________ coupling agent is used to mediate between carrier and enzyme functional groups.

9.	A common example of a commercial biosensor is the __________.

10.	Biosensors can be classified by their __________ type.

PART – B  (5 x 3 = 15 MARKS)

11.	Write a note on constitutive enzymes.

12.	Give a note on Allosteric enzymes.

13.	Write a source of enzymes extracts.

14.	Mention any three advantages of immobilized enzymes.

15.	Write a note on thermal detective biosensors.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the applications of enzymes in pharmaceutical and food industries.

(OR)

17.	Explain the nomenclature and classification of enzymes in detail.

18.	Explain the Michelis Menten equation and its kinetics.

(OR)

19.	Give a detailed note on mechanism of enzyme action.

20.	Discuss the methods of characterization of enzymes.

(OR)

21.	Describe the steps involved in isolation and purification of enzymes from animal and microbial 	source.

22.	Elucidate the immobilization techniques.

(OR)

23.	Discuss the overview applications of immobilizations.

[P.T.O]

24.	Explain the principle and application of any three types of biosensors with neat diagrams.

(OR)

25.	Describe the industrial and healthcare applications of biosensors.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BIOPROCESS ENGINEERING							      Time : 3 hours

Subject Code:	09BT221									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the name of the first reactor designed by Lettinga?

2.	Baffles prevent ___________.

3.	What are the two gassing out techniques?

4.	What is scale up?

5.	Name any two methods of on-line and off-line biomass estimation.

6.	What is metabolite?

7.	What is metabolism?

8.	Expand PTM of a protein molecule.

9.	Give the mass balance equation of a Batch reactor.

10.	Give an example of a Bioprocess simulation software.

PART – B  (5 x 3 = 15 MARKS)

11.	Give a brief note on states of aggregation and late mixing.

12.	Discuss the criteria for scale of bioreactor.

13.	Give a brief note on Space time, Space velocity and enzyme electrodes.

14.	What are plasmids? Add a note on rDNA technology.

15.	Discuss on single cell models.

PART – C  (5 x 15 = 75 MARKS)

16.	What is fluidization? Explain the hydrodynamics of a Fluidized Bed Reactor.

(OR)

17.	Explain the RTD and stability analysis of Packed Bed Reactor.

18.	Write notes on:

a.	Mass transfer and continuity equation. 											    (5)

b.	Microbial oxygen demand.      													    (5)

c.	The three stages of OTR.										 				    (5)

(OR)

19.	Derive the expression for agitator scale up based on power consumption and impeller tip speed.

20.	Explain microbial calorimetry.

(OR)

21.	Give a detailed account on flow injection analysis.

22.	What are the guidelines needed for choosing host vector systems? Explain.

(OR)

23.	Give a detailed account on Bioreator stratergies for maximizing product formation.

24.	What is a structured model? Explain how a plasmid becomes structurally instable.

(OR)

[P.T.O]

25.	a.	Explain the dynamic simulation of batch reactor.									(8)

b.	Discuss the modes for cellular energetics and metabolisms.						(7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DOWNSTREAM PROCESSING							      Time : 3 hours

Subject Code:	09BT222									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Why cell disruption is necessary?

2.	Write short notes on High Value – Low Volume products.

3.	What are filter aids? Give examples.

4.	What are the various types of centrifugal separators?

5.	Specify few drawbacks of ultrafiltration systems.

6.	Mention two adsorption isotherms.

7.	Write the classification of chromatographic techniques.

8.	What is the principle of Ion Exchange Chromatography?

9.	Why is crystallization needed in a bioprocess industry?

10.	Define Bone dry substance.

PART – B  (5 x 3 = 15 MARKS)

11.	Write notes on morphology of cells.

12.	Explain the importance of bioseparations.

13.	Explain dialysis process.

14.	Explain the principle behind gas chromatography.

15.	Write short notes on Rotary dryer.

PART – C  (5 x 15 = 75 MARKS)

16.	State and explain various cell disruption methods.

(OR)

17.	Explain the role of a bioprocess engineer in a bioprocessing industry with case studies related to various types of industries.

18.	Write detailed notes on various filtration equipment used in bioprocessing industries.

(OR)

19.	Explain the Centrifugation process. Also, write detailed notes on various centrifuges used in industries.

20.	Write in detail about the various protein precipitation methods.

(OR)

21.	Write detailed notes on:

a.	Membrane Separation Techniques

b.	Liquid-Liquid Extraction

22.	Write short notes on:

a.	Retention Time.

b.	Reverse Phase Chromatography.

c.	Thin Layer Chromatography.

[P.T.O.]

(OR)

23.	Explain the principle and working mechanism of HPLC.

24.	Explain about the various industrial crystallizers with neat diagrams.

(OR)

25.	What is Lyophilization? Explain the process of lyophilization in detail. What are the applications of lyophilization?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CHEMICAL THERMODYNAMICS AND BIOTHERMODYNAMICS														      					           Time : 3 hours

Subject Code:	09BT229										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Entropy varies with temperature. True or False?

2.	What is the unit of enthalpy?

3.	____________ is a form of potential energy that can be absorbed or released during a chemical reaction.

4.	Quantifying the deviation from ideal solution property for real liquid solutions is called as ____________.

5.	The value of a reaction quotient when the reaction has reached equilibrium is known as ____________.

6.	Raoult's law states that the vapor pressure of a solvent above a solution is equal to the vapor pressure of the pure solvent at the different temperature scaled by the mole fraction of the solvent present. True or False?

7.	What is TS diagram in a Carnot cycle?

8.	What is Liquefaction?

9.	The two types of metabolism are ____________ and ____________.

10.	Maximum portion of a cell contains ____________.

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes on thermodynamic properties.

12.	Write down the differences between ideal and non ideal solutions.

13.	What are the criteria for Phase Equilibrium?

14.	Write short notes on Entropy.

15.	Write short notes on Metabolic Pathways.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive an expression for the Maxwell relations.

(OR)

17.	Derive an expression for the residual property for enthalpy and entropy.

18.	Derive Gibb’s Duhem equation.

(OR)

19.	Explain:		a.	Concept of Excess Properties 		b.	Fugacity Concepts

20.	Explain the chemical reaction equilibria in detail.

(OR)

21.	Describe in detail about the vapor liquid equilibria.

22.	Explain the power cycle in detail.

(OR)

23.	Explain the liquefaction of gases by Linde Process and Claude Process

[P.T.O]

24.	Discuss the metabolic pathways relating to energy in detail.

(OR)

25.	Describe the stoicheometry of cellular growth in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BIO REACTION ENGINEERING						      Time : 3 hours

Subject Code:	09BT230									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define rate constant.

2.	What is order of reaction?

3.	What is batch reactor?

4.	Define Damkohler number.

5.	Define RTD.

6.	Write down the equation which relates E & F curve.

7.	What is parallel reaction?

8.	Define  (Psi).

9.	Define degree of reduction.

10.	Explain about respiratory quotient (RQ).

PART – B  (5 x 3 = 15 MARKS)

11.	Explain about the factors affecting rate of the reaction.

12.	What are the advantages and disadvantages of CSTR?

13.	What are the characteristics of tracer?

14.	Briefly explain about the qualitative findings of parallel reaction.

15.	Find out the degree of reduction of methane and ethane.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive rate equation for irreversible, bimolecular type, second order reaction.

(OR)

17.	a. 	A certain reaction has a rate equation given by									    (7)   		–rA  = 0.005 CA2,      mol/(cm3.min)

If the concentration is expressed in mol/l and time in hours, what would be the value and units of the rate constant?

b. 	On doubling the concentration of reactant, the rate of reaction triples. Find the reaction order.																		    (8)

18.	Derive performance equation of MFR.

(OR)

19.	Derive performance equation of PFR.

20.	Elaborate on dispersion model in detail.

(OR)

21.	Explain about pulse input experiment for RTD.

22.	Elaborate quantitative treatment of MFR.

(OR)

23.	Explain about Denbigh reactions for PFR and MFR.

[P.T.O.]

24.	Elaborate the procedure for elemental balance with an example.

(OR)

25.	A chemical equation for respiration of glucose is given by;

C6H12O6+6O2→6CO2+6H2O

Candida utilis cells convert glucose to CO2 and H2O during growth. A cell composition is CH1.4O0.55N0.2 + 5% ash. Yield of biomass from substrate is 0.5. Ammonia is used as the nitrogen source.

What is the oxygen demand with growth? Candida utilis is also able to grow with ethanol as substrate producing cells of the same composition as above. On a mass basis how is the maximum possible yield from ethanol compared with maximum possible yield with glucose  achieved?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title : 	PROTEIN ENGINEERING								      Time : 3 hours

Subject Code:	09BT232										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is a covalent bond?

2. 	Give an example for ultra violet ray.

3. 	What is single letter code for aminoacid Lycine?

4. 	What is pKa?

5. 	What is Tim barrel structure?

6. 	Define domain.

7. 	Draw the structure of IgG.

8. 	What are abzymes?

9. 	Define Receptor.

10. 	Give any two applications of Molecular Docking.

PART – B  (5 x 3 = 15 MARKS)

11.	Describe about van der Waals interaction in protein structure.

12. 	Draw the structure of any 3 aminoacids.

13. 	Write about TIM barrel structures.

14. 	Describe about Leucine zippers.

15. 	Write a brief note on Immunotoxins.

PART – C  (5 x 15 = 75 MARKS)

16. 	Give a detailed explanation of different types of bonds in protein structure.

(OR)

17. 	Explain about interaction and elucidation of protein structure in detail.

18. 	Explain about different aminoacids and their molecular properties in detail.

(OR)

19. 	Write about characteristics of aminoacids in detail.

20. 	Explain about primary, secondary and super secondary structure of protein in detail.

(OR)

21. 	Give a detailed account on Tertiary and Quaternary structure of protein.

22. 	Write about prokaryotic and eukaryotic transcription factors in detail.

(OR)

23. 	Explain about Enzymes in detail.

24. 	Write about the methods of Protein engineering in detail.

(OR)

25.	Explain about Drug designing in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ANIMAL BIOTECHNOLOGY							      Time : 3 hours

Subject Code:	09BT234									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The ____________ system is an alternative to enzyme immobilization.

2.	Cell culture in ____________ has been touted as "Biology's New Dimension".

3.	Northern blot is a technique used in gene expression by detection of ____________.

4.	RFLP analysis may be subdivided into single and ____________ paradigms.

5.	Gene therapy may be classified into the ____________ types.

6.	Radioimmunotherapy  involves the use of radioactively ____________ against cellular antigens.

7.	____________ first saw clinical use in IVF for purposes of assisted fertilization in the treatment of male factor infertility

8.	There are two methods of ____________ IVF.

9.	____________ stem cells are the descendants of totipotent cells and can differentiate into nearly all cells.

10.	____________ are harvested from the inner cell mass of mouse blastocysts.

PART – B  (5 x 3 = 15 MARKS)

11.	Write a short note on serum free media.

12.	Give a note on in situ hybridization.

13.	Comment on recombinant cytokines.

14.	What is micromanipulation? Give examples.

15.	List the importance of transgenic animals.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the maintenance and preservation of animal cell cultures.

(OR)

17.	a.	Give an account on somatic cell fusion.											(10)

b.	Write a note on organ culture.													(5)

18.	Describe the principle, instrumentation and applications of PCR.

(OR)

19.	Explain the principle of southern blotting and RFLP with detailed applications.

20.	Describe the monoclonal antibodies in therapy of animal diseases.

(OR)

21.	Explain the gene therapy for animal diseases.

22.	Explain the process of IUF and embryo transfer with a neat diagram.

(OR)

23.	Give an account on breeding of farm animals.

24.	Explain the stem cell culture in the production of animal technology.

(OR)

25.	Elucidate the concept of transgenic animals’ technology and strategies for the production of 	transgenic animals.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INSTRUMENTAL METHODS OF ANALYSIS			      Time : 3 hours

Subject Code:	09BT251									      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention any two electrodes used to measure pH.

2.	Mention any two uses of buffer solution.

3.	Mention any two uses of colorimeter.

4.	A photon has no electric charge. True or False?

5.	Give the full form of HPLC.

6.	Write some of the chromatography techniques.

7.	The isotope of an element is defined by the _________ number.

8.	What is a GM counter?

9.	What is thermo gravimetric analysis?

10.	What is differential scanning analysis?

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes on pH meter.

12.	What is Spectrophotometer? Write briefly.

13.	Write short notes on electrophoresis.

14.	What is a radioactive isotope? Write short notes.

15.	Give a brief account on the thermal analysis.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive Henderson-Hasselbalch equation.

(OR)

17.	Explain in detail about the pH meter.

18.	Explain the working principle of colorimeter.

(OR)

19.	Describe the working principle of a UV spectrophotometer.

20.	Explain the construction and working principle of a HPLC system.

(OR)

21.	Discuss in detail of how a protein molecule is separated using an affinity chromatography.

22.	Explain the atomic structure and radioactive decay in detail. Write some of the uses of radioactive decay.

(OR)

23.	Explain with a schematic diagram of a GM counter in detail.

24.	Describe in detail about the Differential Thermal Analysis (DTA).

(OR)

25.	Explain in detail about the Differential Scanning Calorimeter (DSC).

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BASIC ENGINEERING BIOCHEMISTRY				      Time : 3 hours

Subject Code:	10BT201									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The fat soluble enzyme required for carboxylation reaction is ____________.

2. 	The vitamin containing isoalloxazine ring is ____________.

3. 	What are epimers?

4. 	Define Enantiomers?

5. 	Enlist any two aromatic amino acids.

6. 	Mention the three types of RNA.

7. 	Differentiate between glycogenesis and glycogenolysis.

8. 	What are ketone bodies?

9. 	The end product of purine metabolism in humans is ____________.

10. 	Hypouricemia is due to the deficiency of the enzyme ____________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Differentiate between molarity and molality.

12. 	What is meant by Saponification number?

13. 	Define Tautomerism? Give an example.

14. 	Briefly describe Redox Potential.

15. 	Differentiate between transamination and transmethylation.

PART – C  (5 x 15 = 75 MARKS)

16. 	Illustrate in detail the functions of Vitamin C. Add a note on its dietary sources.

(OR)

17. 	Discuss the biochemical functions of Vitamin A. Enlist any three Vitamin A deficiency diseases.

18. 	Elaborate in detail about the classification of lipids.

(OR)

19. 	Explain in detail about the classification of enzymes. Add a note on the effect of pH on enzyme activity.

20. 	Discuss in detail about physical and chemical properties of amino acids.

(OR)

21. 	Illustrate in detail about the structure of DNA.

22. 	Describe in detail the steps involved in glycolysis.

(OR)

23. 	Explain the steps involved in TCA cycle.

24. 	Elaborate in detail the reactions involved in urea cycle.

(OR)

25. 	Describe in detail the inborn errors of amino acid metabolism.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BASICS OF BIOCHEMISTRY FOR ENGINEERS		      Time : 3 hours

Subject Code:	12BT201										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What are Enantiomers?

2. 	What are glycoproteins?

3. 	Define Saponification.

4. 	Define Rancidity.

5. 	What are Zwitter ions?

6. 	What are simple proteins?

7. 	Define Chargaff’s rule.

8. 	Nucleotide is composed of  __________.

9. 	The metabolite excreted in urine during thiamine deficiency is __________.

10. 	The fat soluble vitamin required for carboxylation reaction is __________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Differentiate between monosaccharides and polysaccharides.

12. 	Define Tautomerism. Give an example.

13. 	Define Acid number. Mention its significance.

14. 	Enlist the types of bonds responsible for protein structure.

15. 	What are vitamins? Give two examples for fat soluble vitamins.

PART – C  (5 x 15 = 75 MARKS)

16. 	Describe the:		a.	Structure of glycogen 		b.	Structure of cellulose.		              (7+8)

(OR)

17. 	Discuss in detail about heteropolysaccharides with suitable examples.

18. 	Illustrate in detail about classification of lipids.

(OR)

19. 	Draw the structure of Cholesterol. Add a note on its properties and functions.

20. 	Discuss in detail about the secondary structure of proteins.

(OR)

21. 	Explain in detail the nomenclature and classification of enzymes.

22. 	Describe in detail about the structure of tRNA.

(OR)

23. 	Compare the structural features of B-DNA, A-DNA and Z-DNA.

24. 	Comment on the dietary source and functions of Vitamin C.

(OR)

25. 	List out the components of Vitamin B-Complex. Add a note on its deficiency diseases.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BIOSTATISTICS										      Time : 3 hours

Subject Code:	12BT202				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.   	True (or) False: Collection of any number of related observations is called Data.

2.   	Define Histogram.

3.   	True (or) False: Accepting Ho, when Ho is False is called Type – I error.

4.   	Test of significance between experimental values and theoretical values is called __________.

(a) Student’s ’t’ test  (b) Critical value  (c) Chi – square test  (d) Standard error

5.   	Relation between mean, median and mode is __________.

(a)   Mean = 3Median – 2Mode             (b)    Median = 3Mean + 2Mode

(c)   Mode = 3Median – 2Mean             (d)   Median = 3Mean – 2Mode

6.  	Coefficient of variance is __________.

(a)  100 x xσ     (b)   100 x σx__    (c)   100 x x2σ   (d)   100 x xσ

7.   	Angle between the two regression lines is __________.

8.   	Probability of an impossible event is __________.

(a)    1    (b)    ∞    (c)    0    (d)    -1

9.  	Basic principles of the design of experiments are

(a) __________ (b) __________ (c) __________.

10. Define Analysis of variance.

PART – B  (5 x 3 = 15 MARKS)

11. 	Write down the types of Bar diagram and explain.

12. 	The following table gives the number of aircraft accidents that occurred during the various

days of the week.  Test whether the accidents are uniformly distributed over the week.

| Days | Mon | Tue | Wed | Thu | Fri | Sat |
| --- | --- | --- | --- | --- | --- | --- |
| No. of accidents | 14 | 18 | 12 | 11 | 15 | 14 |

13. 	Write down the advantages of arithmetic mean.

14. 	If A and B are independent, then prove that A and B are independent.

15. 	In a trivariate distribution, if  r12 = 0.77, r13 = 0.72 and r23 = 0.52, find the multiple correlation

coefficient R1.23

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Write down the principles to be followed in constructing a Good questionnaire.		    (7)

b.	The regional rainfall indices during the year 2001 to 2003 are given below:			    (8)

| Year | West | North | East | South | Centre |
| --- | --- | --- | --- | --- | --- |
| 2001 | 78.4 | 88.9 | 83.7 | 89.9 | 86.5 |
| 2002 | 75.6 | 62.5 | 103.6 | 75.5 | 77.4 |
| 2003 | 121.2 | 116.5 | 107.6 | 123.9 | 90.3 |

Represent the data by a multiple bar diagram.

(OR)

[P.T.O.]

17. 	a.	Define Primary data. Write and explain the methods of collecting the Primary data.   	    (7)

b.	Draw a pie diagram for the following data of Sixth Five-Year Plan Public Sector outlays:																						    (8)

| Agriculture and Rural Development | 12.9 % |
| --- | --- |
| Irrigation | 12.5 % |
| Energy | 27.2 % |
| Industry and Minerals | 15.4 % |
| Transport and Communication | 15.9 % |
| Social Services | 16.1 % |

18. 	The nicotine contents in milligrams in two samples of tobacco were found to be as follows.

| Sample A | 28 | 30 | 32 | 33 | 33 | 29 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sample B | 29 | 30 | 30 | 24 | 27 | 27 | - |

Test whether the samples have come from the same normal population.

(OR)

19. 	a.	Samples of two types of electric bulbs were tested for length of life	and the following data 		were obtained:									              						    (7)

|  | Type I | Type II |
| --- | --- | --- |
| Sample size | 8 | 7 |
| Sample mean | 1234 hrs | 1036 hrs |
| Sample S.D | 36 hrs | 40 hrs |

Is the difference in the means sufficient to warrant that Type I is superior to Type II.

b.	Given the following contingency table for hair colour and eye colour. Find the value of

Chi-square. Is there good association between the two.                                                     (8)

|  |  | Hair Colour |  |  | Total |
| --- | --- | --- | --- | --- | --- |
|  |  | Fair | Brown | Black |  |
| Eye Colour | Blue | 15 | 5 | 20 | 40 |
|  | Grey | 20 | 10 | 20 | 50 |
|  | Brown | 25 | 15 | 20 | 60 |
|  | Total | 60 | 30 | 60 | 150 |

20.  Calculate the mean, median and mode for the data given below:

| Wages(Rs) | 50-54 | 54-58 | 58-62 | 62-66 | 66-70 | 70-74 | 74-78 | 78-82 | 82-86 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. of persons | 3 | 7 | 15 | 28 | 35 | 26 | 19 | 11 | 8 |

(OR)

21. 	a.	Find the standard deviation for the following data:								    (7)

| x | 100 - 109 | 110 - 119 | 120 - 129 | 130 - 139 | 140 - 149 | 150 - 159 | 160 - 169 | 170 -179 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| f | 15 | 44 | 133 | 150 | 125 | 82 | 35 | 16 |

b.	The scores of two players X and Y in 12 rounds are given below. Who is the best player

and who is the consistent player. 												    (8)

| X | 74 | 75 | 78 | 72 | 78 | 77 | 79 | 81 | 79 | 76 | 72 | 71 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | 87 | 84 | 80 | 88 | 89 | 85 | 86 | 82 | 82 | 79 | 86 | 80 |

22. 	The dress materials required for making clothes for 10 different students are given below:

| Length (in cm) X | 150 | 152 | 145 | 160 | 172 | 190 | 195 | 200 | 210 | 215 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Breadth (in cm) Y | 60 | 62 | 59 | 65 | 70 | 72 | 75 | 78 | 80 | 82 |

a.	Find the two regression equations

b.	Calculate the breadth of the material, when the length is 220 cm and also calculate the 			length of the material, when the breadth is 85 cm.

(OR)

23. 	a.	A husband and wife appear in an interview for two vacancies in the same post. The 	probability of husband to be selected is 17 and wife is 15. Find the probability that

(i) Both of them will be selected (ii) Only one of them will be selected.                            (5)

b.	The first bag contains 3 white balls, 2 red balls and 4 black balls. Second bag contains 2 white, 3 red and 5 black balls and third bag contains 3 white, 4 red and 2 black balls.  One bag is chosen at random and from it 3 balls are drawn. Out of three balls two balls are white and one is red.  What is the probability that it came from the third bag.               (10)

24. 	a.	Write down the advantages and disadvantages of Randomized block design.                   (5)       b.	Four different machines are used for a production. The outputs are given in the table 	below:

| Outputs |  |  |  |
| --- | --- | --- | --- |
| Machine I | Machine II | Machine III | Machine IV |
| 15 | 12 | 14 | 10 |
| 17 | 10 | 09 | 14 |
| 14 | 13 | 07 | 13 |
| 11 | 17 | 10 | 15 |
| - | 14 | 08 | 12 |
| - | - | 07 | - |

Use One-way ANOVA to test whether the machines are equally effective.			  (10)

(OR)

25. 	The following data represent the number of units of production per day turned out by 5 	different workers using 4 different machines.

| Workers | Machine Type |  |  |  |
| --- | --- | --- | --- | --- |
|  | A | B | C | D |
| 1 | 44 | 38 | 47 | 36 |
| 2 | 46 | 40 | 2 | 43 |
| 3 | 34 | 36 | 44 | 32 |
| 4 | 43 | 38 | 46 | 33 |
| 5 | 38 | 42 | 49 | 39 |

a.	Test whether the mean production is the same for the different machine types.

b.	Test whether the 5 men differ with mean productivity.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MASS TRANSFER OPERATIONS						      Time : 3 hours

Subject Code:	12BT203									       	       Maximum Marks: 100

(Allowed Extra   graph sheets, Assume if any Data missing )

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the driving force in mass transfer operations?

2.	State the diffusion laws.

3.	Write the equation of relative volatility.

4.	What are the types of distillation?

5.	What is the system of absorption?

6.	State the absorption factor.

7.	The two types of adsorptions are: _________ and _________.

8.	Give the different adsorbents used in adsorption.

9.	What is the basic rate equation for mass transfer operation?

10.	List out the different solvents used in extraction operations.

PART – B  (5 x 3 = 15 MARKS)

11. 	Differentiate between molecular diffusion and eddy diffusion.

12. 	What is the relationship between theoretical plates and reflux ratio?

13. 	What are the important considerations to be made for the selection of a solvent?

14. 	What are the principles involved in Ion exchange?

15. 	Give the various types of equipments used in leaching and extraction.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive from fundamentals the expression for steady state equimolar counter diffusion of 	gas A 	through another gas B.                                                                             		    			(7)

b.	Oxygen (A) is diffusing through carbon monoxide (B) under steady-state conditions, with the carbon mono oxide non diffusing. The total pressure is 1x105 N/m2 and the temperature 0ºC. The partial pressure of oxygen at two plane 2.0 mm apart is respectively is, 13000 and 6500 N/m2. The diffusivity for the mixture is 1.87x105m2 /sec. Calculate the rate of diffusion of oxygen in kmol/sec through each square meter of the two planes. (R=8314 N.m/kmol.K)				(8)

(OR)

17.	a.	Derive and express of the Film theories and Penetration theories with a neat diagram.   		(7)

b.	A cylindrical tank of 4 meters diameter was filled with acetone to a depth of 2.3 cm from the top for storage and is exposed to ambient temperature of 18.8ºC in a gentle current of air.

i.	Calculate the fall in level of acetone after 3 hours.

ii.	Calculate the loss in rupees if acetone is worth ` 2.0 per liter.

Data: Vapor Pressure of acetone = 170mm Hg, Barometric pressure = 765mmHg, Density of acetone = 0.79 gm/cc, DAB = 0.09cm2/ sec,. Mol. Wt of acetone = 58, Gas  constant value = 82.0 6cm3.atm/ g mol. K.						                                        	    			(8)

18. a.	5000 kg/hr  of mixture of benzene and toluene  containing 60% benzene by weight are to 	be 			distilled in a fractionating column to give an overhead product containing 0.98 mole fraction of 			benzene and a bottom product  containing 0.97 mole fraction toluene. Calculate the moles of 				distillate and residue. 															(7)

[P.T.O.]

b.	A liquid mixture containing 50 % mol heptanes (A) and 50 % mol octane (B) is to be continuously flash vaporized at  1 standard atmosphere pressure to vaporize 60 mol% of  the feed. What will be the composition of the vapor and liquid in the separator for an equilibrium stage? The VLE data are given.																		(8)

| T, ºC | 98.5 | 106 | 110 | 115 | 120 | 125.5 |
| --- | --- | --- | --- | --- | --- | --- |
| V.P of A mmHg | 760 | 940 | 1050 | 1200 | 1360 | 1540 |
| V.P of B mmHg | 333 | 417 | 484 | 561 | 650 | 760 |

(OR)

19.	It is desired to fractionate a feed mixture of ethanol and propanol containing 65mol% ethanol to a distillate containing 92 mole %  ethanol and a residue containing 93 mole% propanol. The  feed enters as a saturated vapour.

a.	Determine the minimum reflux ratio 		b.	For a reflux ratio of 4, determine the theoretical number of plates needed 		c.	  Locate the feed plate point.

| X | 0 | 0.1 | 0.2 | 0.3 | 0.5 | 0.7 | 0.8 | 0.9 | 1.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | 0 | 0.19 | 0.34 | 0.47 | 0.67 | 0.83 | 0.89 | 0.95 | 1.0 |

X, Y mole fraction of ethanol in liquid and vapor phase respectively.

20.  	Ammonia in an ammonia air mixture is to be absorbed in an absorption tower in water flowing down the 	absorption tower packed with suitable packing. The data for the tower is as follows:

Air flow rate =150 kg/hr.m2, liquid  phase concentration: at the top of packing = 0.0000135 Kg 	NH3/ 	kg, at the bottom  of packing = 0.0005260 Kg NH3 / kg, Gas  phase  concentration: at the 	top of the 	packing = 0.000568 Kg NH3 / kg, at the bottom of the packing = 0.00957 Kg NH3 / kg. Assuming liquid  	concentration to be in equilibrium with gas phase, calculate the mass flow rate of water.

(OR)

21.	a.	Discuss about the principle of the absorption column with a neat sketch.                         	 	(7)

b.	A coal gas is to be free of light oil by scrubbing with wash oil as an absorbent. Light oil is            			recovered by absorbing with steam. The gas enters at 0.25m3/sec and 26 ºC and 803mmHg.              		This contain 2% volume of light oils vapors. The light can be assumed to be benzene and 95% of 			removal is required. The wash oil enters at 26ºC containing 0.005 mole fraction benzene whose 			molecular weight is 260. Oil to gas circulation rate of 1.5 times minimum is to be used. Wash oil 			and benzene solution are ideal. The vapour pressure of benzene at 26 ºC is 100 mmHg. Calculate 			the equilibrium data of X and Y and calculate the number of stages required for this operation.                                                                                       																					(8)

22.	a.	Differentiate between Physical and Chemical adsorption.                                           	    	(5)

b.	Derive the material balances for single stage cross current operation and two stages of fluid- solid operation with block diagram.                                                                       				(10)

(OR)

23. 	A solid adsorbent is used to remove colored impurities from an aqueous solution. The original value of color is on an arbitrary scale. It is required to reduce this value to 10% of its original value using the following data. Find the quantity of fresh adsorbent used for 1000 kg of solution for:

a.	single stage 	b.	2 stage cross current operation operating with the intermediate value of 24.

| kg of adsorbent/kg of solute | 0 | 0.001 | 0.004 | 0.008 | 0.02 | 0.04 |
| --- | --- | --- | --- | --- | --- | --- |
| Equilibrium color(Y) | 48 | 43 | 31.5 | 21.5 | 8.5 | 3.5 |

24.  	Describe the principles and operations of Bollman and Dorr extractor with a neat diagram.

(OR)

25. 	Write short notes on: Heap leaching, Roto cell and Packed bed leaching.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FLUID MECHANICS									      Time : 3 hours

Subject Code:	12BT204									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the unit of density of petrol.

2.	Express 3 m of water head in cm of mercury and pressure in KPa.

3.	Name the three flow measuring instruments.

4.	The rate of flow through a V-notch varies as _________.

5.	Write any one use of packed beds.

6.	The superficial velocity of fluid at which the fluid bubbles start getting formed is called the ________ velocity.

7.	The specific speed of a pump is given by the expression_________.

8.	What is the use of air vessels in a reciprocating pump?

9.	Write the use of air compressor.

10.	Write any two units of pressure.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the properties of ideal fluids?

12.	Define weir.

13.	Define drag force.

14.	Write the various modes of accomplishing agitation.

15.	Define pump.

PART – C  (5 x 15 = 75 MARKS)

16.	The right limb of a simple U-tube manometer containing mercury is open to the atmosphere while the left limb is connected to a pipe in which a fluid of specific gravity 0.9 is flowing. The centre of the pipe is 12 cm below the level of mercury in the right limp. Find the pressure of fluid in the pipe if the difference of mercury level in the two limbs is 20 cm.

(OR)

17.	A simple U-tube manometer containing mercury is connected to a pipe in which a fluid of specific gravity 0.8 and having vacuum pressure is flowing. The other end of the manometer is open to atmosphere. Find the vacuum pressure in pipe, if the difference of mercury level in the two limbs is 40 cm and the height of fluid in the left from the centre of pipe is 15 cm below.

18.	A horizontal venturimeter with inlet and throat diameters 30 cm and 15 cm respectively is used to measure the flow of water. The reading of differential manometer connected to the inlet and the throat is 20 cm of mercury. Determine the rate of flow. Take Cd = 0.98.

(OR)

19.	An orifice meter with orifice diameter 10 cm is inserted in a pipe of 20 cm diameter. The pressure gauges fitted upstream and downstream of the orifice meter given reading of 19.62 N/cm2 and 9.81 N/cm2 respectively. Co-efficient of discharge for the meter is given as 0.6. Find the discharge of water through pipe.

20.	What is Fluidization? Explain with neat sketch fluidizer bed system.

[P.T.O.]

(OR)

21.	Explain with a neat sketch drag curves for regular and irregular solids.

22.	Explain with a neat sketch Agitation by mechanical impellers.

(OR)

23.	An aqueous slurry containing 20 percent of solids particle (specific gravity= 1.85, average size = 0.1 mm) in water is to be prepared. The mixing vessel is 2 m in diameter and 2 m deep. It is fitted with four longitudinal baffles (width of each baffle = 0.1 Dt). A 120 rpm six- bladed turbine impeller (impeller diameter = 0.3 Dt), positioned at one impeller diameter above the vessel floor, is employed for the purpose. Estimate the power consumption for lifting the solids particle to the liquid surface.

24.	A centrifugal pump is to discharge 0.118 m3/s at a speed of 1450 rpm against a head of 25 m. The impeller diameter is 250 mm, its width at outlet is 50 mm and manometric efficiency is       75 %. Determine the vane angle at the outer periphery of the impeller.

(OR)

25.	A double-acting reciprocating pump, running at 40 rpm is discharge 1.0 m3 of water per minute. The pump has a stroke of 400 mm. The diameter of the piston is 200 mm. The delivery and suction heads are 20 m and 5 m respectively. Find the slip of the pump and power required to drive the pump.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	METABOLISM AND BIOENERGETICS				      Time : 3 hours

Subject Code:	12BT205				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Glycogenolysis.

2.	What are the two distinct phases in the pentose phosphate pathway?

3.	Give any two examples of branched chain amino acids.

4.	Draw the neat structure of tyrosine.

5.	How much energy is generated when 1gram of fatty acid is oxidized?

6.	In which form the fatty acids are ingested by our body?

7.	What are the two substrates involved in salvage pathway?

8.	What is the end product of purine metabolism?

9.	Give any two examples of energy rich compounds.

10.	Name any two drugs which inhibit oxidative phosphorylation.

PART – B  (5 x 3 = 15 MARKS)

11.	Write a short note on gluconeogenesis.

12.	List out any six inborn errors of amino acid metabolism.

13.	Explain in brief about the regulation of cholesterol in our body.

14.	Differentiate between nucleotide and nucleoside.

15.	What do you understand by bioenergetics?

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the formation of pyruvate during glycolysis.

(OR)

17.	Sketch the reactions involved in Tricarboxylic acid (TCA) cycle.

18.	Explain the biosynthetic pathway of tyrosine and phenyalanine.

(OR)

19.	Discuss on degradation steps involved in Leucine and Asparagine.

20.	Explain in detail about the different stages involved in beta oxidation of fatty acids.

(OR)

21.	Write a detailed note on biosynthesis of triacylglycerol.

22.	Explain the biosynthetic process of pyrimidine.

(OR)

23.	Write the reaction steps involved in De novo synthesis of purine metabolism.

24.	Give detailed description of respiratory chain and oxidative phosphorylation.

(OR)

25.	Explain in detail about Urea cycle.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INSTRUMENTAL METHODS OF ANALYSIS			      Time : 3 hours

Subject Code:	12BT206				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write Henderson-Hasselbalch equation.

2.	List any two uses of centrifuge.

3. 	Write Beer - Lambert’s law.

4. 	Mention any two applications of flourimeter.

5. 	Size exclusion chromatography is also called ____________.

6. 	Name any two types of immunoelectrophoresis.

7. 	Mention any two commercially available radionuclides.

8. 	Define autoradiography.

9. 	____________ is one of the types of DSC instruments.

10. 	Measurements of changes in sample mass with temperature are made using ________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Give a short note on bicarbonate buffering system.

12. 	Concise on essential parts of a colorimeter.

13. 	Elaborate on the applications of TLC and HPLC.

14. 	Reveal the operation and applications of Scintillation counter.

15. 	Mention the features of an ideal microbalance.

PART – C  (5 x 15 = 75 MARKS)

16. 	Elaborate on various calibration methods used in analytical instrumentation.

(OR)

17. 	Illustrate the principle and applications of high and low speed centrifuges.

18. 	Exemplify the basic components and working principle of a flame photometer.

(OR)

19. 	Explain the principle of UV - visible spectrophotometer and a note on its applications.

20. 	Give a brief account on various types of electrophoresis.

(OR)

21. 	Describe the types of High-Performance Liquid Chromatography.

22. 	Illustrate the applications of radioactive isotopes in diagnostic and medical field.

(OR)

23. 	Explicate the types of radioactive decay processes.

24. 	Give a detailed account on the principle and instrumentation of thermogravimetry.

(OR)

25. 	Explain the basic theory, construction and working of Differential Scanning Calorimeter (DSC).

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	HEAT TRANSFER OPERATIONS						      Time : 3 hours

Subject Code:	12BT208				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Thermal conductivity unit is ___________.

2. 	Rate of heat transfer per unit area is called ___________.

3. 	List out few dimensionless numbers involved in convection heat transfer.

4. 	The two distinct methods of condensation are ___________.

5. 	What is the expansion of 1-2 Heat exchangers.

6. 	State the LMTD.

7.  	An Evaporator capacity is  expressed as ___________.

8. 	What is your suggestion about  forward feeding methods over the backward feed methods?

9. 	List out the boiling stages.

10.	Write the unit of heat transfer coefficient?

PART – B  (5 x 3 = 15 MARKS)

11. 	State and explain the Fourier’s law of heat conduction.

12. 	Give the various laws of radiations.

13. 	Distinguish between co-current and counter current heat exchanger.

14. 	Write down the mass balance equation for single effect evaporator at steady state conditions.

15. 	Differentiate between drop wise condensation and film wise condensations.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Derive the expression for heat transfer through furnace wall made of three different   	materials in series. Assume k1, k2and k3   be the thermal conductivities of materials and

x1, x2 and x3 be the  respective thickness. Assume hot face and cold face  temperatures be T1 and T2 respectively. 															    (8)

b.	A 50 mm diameter pipe of circular cross section and with walls 3mm thick is covered with two concentric layers of logging. The inner layer having a thickness of 25mm and thermal conductivity of 0.08 (W/m K) and outer layer having thickness of 40 mm and thermal conductivity of 0.04 (W/m K) .Estimate the rate of heat loss per metre length of pipe and the temperature inside the pipe is 500K and outside surface temperature is 330K for pipe is 45(W/m K).                         													    (7)

(OR)

17.	a.	Derive an expression for the hollow sphere with a neat sketch.                            		  (10)

b.	Estimate the heat loss per square meter of the surface through a brick wall 0.5 thick when the inner surface is at 400K and the outside surface is at 310 K..The thermal conductivity of the brick may be taken as 0.7 w/mk.											    (5)

18.	Describe the heat transfer by convection with neat sketch.

(OR)

19.	a.	Find  inside heat transfer coefficient  using sieader –Tate equation for laminar flow. Data, 	I.D of tube = 20mm, NRe=15745, µ at mean temperature =550x10-5 pascal. µ at wall 	temperature =900 x10-6 pas. Prandtle number =36, k = 0.25 W/m K.            			  (10)

[P.T.O.]

b.	Calculate the heat loss by radiation from an unlogged horizontal steam pipe 50mm OD, at 377Kto air at 283 K.ę =0.90, delta=5.87 x10-8 w/square meter k4                            	    (5)

20.	a.	Discuss about various flow  heat exchangers.                                                             	    (5)

b.	Water enters a heat exchanger at 55ºC  and leaves at 85ºC. Hot gases enter at 305 ºC and leaves at 160 ºC. If the  total heat transfer area is 500 square metre and the overall  heat transfer coefficient is 600 kcal/hrm2ºC, determine  the total heat transferred per hour for:

i.	Parallel flow		ii.	Counter flow.                                                		  		  (10)

(OR)

21. 	a.	Discuss about the design of shell and tube heat exchanger  (1-1) with neat sketch.   	    (7)

b.	Calculate the heat transfer area of 1-2 heat exchangers from the following data. Inlet and                                                 outlet temperature of hot fluid are 423K and 353K respectively. Inlet   and outlet temperature of cold fluid are 303K and 318K respectively. Overall heat transfer coefficient = 4100 W/m2 K .  Q=407 KW, LMTD correction factor=0.84.					    	    (8)

22.  Calculate the amount of steam required for concentrating the solution of caustic soda from       	28% by weight of solids to 40% by weight of solids in a single effect evaporator. The feed rate 	in 25000 kg / hr and its temperature is 60 ºC. .The absolute pressure in the evaporator is

0.2 kg/ cm2.Saturated steam at 1.4 kg/ cm2 is to be used as heating medium. The elevation in boiling points is 25 ºC. If the overall heat transfer coefficient is 670 kcal/ hr m2 ºC. Calculate the heating surface required .The enthalpy data for various streams are as follows. Vapour at  0.2 kg/ cm2  = 623 kcal/ kg.   20% NaOH at 60 ºC = 50 kcal / kg, 40% NaOH  at 85 ºC =90 kcal/ kg. Latent heat of steam at 1.4 kg/ cm2  =534 kcal / kg.

(OR)

23. 	a.	Discuss about a single effect evaporator energy balance with suitable sketch.   		    (5)

b.	A single effect evaporator is fed with 5000 kg/ hr of solution containing 1% solution by     weight. Feed temperature is 303K and is to be concentrated to a solution of 2% solute by weight. The evaporator is  at atmospheric pressure (101.325 k pa) and area of evaporator is  69m2 .Saturated steam is supplied at 143.3 k pa as a heating medium. Calculate steam economy and overall  heat transfer coefficient.Data.Enthalphy of feed at 303K = 125.79

k j/ kg, Enthalpy of vapour at 101.325 k pa= 2676.1 k J / kg, Enthalpy of saturated steam at 143.325 k pa= 2691.5 kJ/ kg, Enthalpy of product= 419.1 k J/kg, Enthalpy of  saturated water at 383K= 461.30  k J/ kg, Saturation temperature of steam= 383K Boiling point  of solution = boiling point water= 373K.											  (10)

24. 	Discuss in detail about the boiling mechanism of heat transfer with a neat diagram.

(OR)

25.	Explain in detailed the different types of condensation with a neat diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BIOREACTOR ENGINEERING							      Time : 3 hours

Subject Code:	12BT209									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	C6H12O6+0.9O2+0.98NH3→0.88C6H10NO3+1.32CO2+3.92H2O.

Calculate Respiratory Quotient.

2.	Find the available electron present in C6H12O6.

3.	How many grams of cell can be produced by 2 mole of ATP?

4.	Write the expression for competitive substrate inhibition.

5.	Rate of oxygen transfer __________ by decreasing the liquid film resistance around the gas bubbles.

6.	Write the general unit for volumetric mass transfer coefficient.

7.	Define autocatalytic reaction.

8.	Air lift reactor is mostly used for __________ microorganisms.

9.	Write the methods to increase the dissolved oxygen concentration in broth medium.

10.	___________ sensor which is an integral part of fermenter and measured value, cannot be used 	directly for process control.

PART – B  (5 x 3 = 15 MARKS)

11.	Xt=0hr=2gl; Xt=40hr = 50 gl; Determine doubling time.

12.	How to estimate the kinetic parameters of Monod model?

13.	List out the resistances in transfer of oxygen from interior of gas bubble to site of intracellular region.

14.	What are the disadvantages of batch reactor?

15.	Draw the flow chart for controlling foam formation in Fermenter?

PART – C  (5 x 15 = 75 MARKS)

16.	a.							  	(12)

Find the stoichiometric coefficients for the given biological reaction when RQ = 1.44.

b.	Find the theoretical growth and product yield coefficient for ethanol fermentation as

represented by the following reaction: C6H12O6→2C2H5OH+2CO2

Assume 2 moles of ATP formed for 1 mole of  glucose.							(3)

(OR)

17.	Assume that experimental measurement for a certain organism has shown that cells can convert 23substrate carbon to biomass.

a.	Calculate the stoichiometric coefficients for following biological reactions: 			(12)

i.	C6H12O6+aO2+bNH3→cC4.4H7.3N0.86O1.2+dH2O+eCO2

ii.	C6H34+aO2+bNH3→cC4.4H7.3N0.86O1.2+dH2O+eCO2

b.	Calculate the yield coefficients biomass with respect to substrate and oxygen supply for both the reactions. Also, comment on the differences.								  (3)

[P.T.O.]

18.	a.	Citric acid is manufactured using submerged culture of Aspergillus Niger in a batch reactor 	operated @ 30°C. Over a period of two days, 2500 kg of glucose  and 860 kg of oxygen 	are 	consumed to produce 1500 kg of citric acid, 500 kg of biomass and other products. 	Ammonia is used as nitrogen source. Power input to the system by mechanical agitation of 	the broth is about 15kW and 100kg of water is evaporated over the culture period. Estimate 	the cooling requirements.														  (10)

b.	Explain the growth pattern of microorganism.									    (5)

(OR)

19.	a.	Derive the expression for non-competitive product inhibition on cell growth. 		    (8)

b.	Derive the expression for un-competitive toxic compound inhibition. 				    (7)

20. 	a.	Derive the expression for OTR.												  (12)

b.	Discuss about the foam formation in bioreactor and prevention method	.			    (3)

(OR)

21. 	Explain in detail about Gassing out techniques to determine the ka.

22. 	Derive the performance equation for chemostat with recycle.

(OR)

23. 	a.	Explain the Air lift reactor with neat sketch and its types. 							    (7)

b.	Explain in detail about packed bed bioreactor and its application, advantages and         			disadvantages of packed bed bioreactor.										    	    (8)

24. 	Write in detail about basic configuration of fermenter and its ancillaries.

(OR)

25. 	Write short notes on:

a.	Thermal mass flow meters.													    (4)

b.	Bourdon tube pressure gauge.													    (4)

c.	PI, PD and PID control system.													    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CHEMICAL AND ENZYME REACTION ENGINEERING

Time : 3 hours

Subject Code:	12BT210				 						      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the general unit of rate constant.

2.	The overall stoichiometric equation is: 2A+3B→4R+S; −rA=KCA0.5CB3.

Step 1: A+B→AB 				Time: 50 minutes

Step 2:AB+A→A2B			Time: 60 minutes

Step 3:A2B+B→2AB			Time: 20 minutes

Step 4:2AB+B→4R+S			Time: 40 minutes

Find the moleculartity of the reaction.

3.	What is CSTR?

4.	Calculate the first order rate constant for 83.3% conversion in 4 minutes.

5.	In 12 minutes experiment, the fraction of a material leaving the vessel between 5 and 12 minutes is 0.6. Find the fraction of material leaving the vessel between 0 and 5 minutes?

6.	Find the final concentration of A when CA0 = 2 mole/li, XA = 0.8 and εA = 0.6.

7.	Determine the value of 0∞δt−3t2dt.

8.	The rate of immobilized enzyme is __________ than free enzyme.

9.	What is the enzyme used to convert glucose to gluconic acid?

10.	Write the formulae to find KM and VM using Double - Reciprocal Plot.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the factors affecting rate of reaction.

12.	The activation energy of a bimolecular reaction is about 9150 cal/mole. How much faster does this reaction take place @ 227 °C and 127°C ?

13.	Write the characteristics of tracer.

14.	The reaction kinetics and stoichiometry are: A → R;  –rA = 0.5CA2. Find the average concentration of A remaining when CA0 = 2 mole/li and E = 0.5  for 1 <  t  <  3.

15.	Give short notes on calorimetric sensor.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the rate constant expression using:

a.	Collision theory for bimolecular collisions of unlike molecules.   					    (8)

b.	Transition state theory	.														    (7)

(OR)

17.	Derive the performance equation for:

a.	Zero order reaction.															    (5)

b.	nth order reaction.																    (5)

c.	Half life method.																    (5)

[P.T.O.]

18.	Derive the performance equation of equimolar bimolecular irreversible second order reaction in terms of XA.

(OR)

19.	a.	Derive the performance equation of PFR.										  (12)

b.	Find the empirical equation for first order reaction using half-life method. 			    (3)

20.	Derive the performance equation of Tank-In-Series Model.

(OR)

21.	The data given below represent a continuous response to a pulse input experiment. Calculate the mean residence time of fluid in the vessel, construct E – curve and fraction of material leaving the vessel between 3 and 6 minutes.

| Time (min) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 12 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cpulse (g/m3) | 0 | 1 | 5 | 8 | 10 | 8 | 6 | 4 | 3 | 2.2 | 1.5 | 0.6 | 0 |

22.	Derive the Michaelis – Menton equation and explain the methods to estimate MM parameters.

(OR)

23.	Explain different types of inhibition in detail.

24.	Explain in detail about immobilization techniques.

(OR)

25.	Write short notes on:

a.	Electrochemical sensors. 														    (5)

b.	Thermal sensors. 																    (4)

c.	Optical sensors.																    (3)

d.	Piezoelectric sensors. 															    (3)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BASIC INDUSTRIAL BIOTECHNOLOGY				      Time : 3 hours

Subject Code:	12BT211									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define fermentation.

2.	What is meant by a block diagram?

3.    __________ refers to ethanol with a low water content.

4.	__________ is a protease used for its anti-inflammatory effects.

5.	__________ are a group of compounds with large lactone rings.

6.    What are steroids?

7.	Define SCP.

8.	__________ is  used for debridement of wounds and burns without damaging healthy tissue.

9.	__________ are monospecific antibodies that are made by identical immune cells that are all clones of a unique parent cell.

10.	__________ is used to treat diabetes produced by recombinant technology.

PART – B  (5 x 3 = 15 MARKS)

11.	Brief on pictorial representation.

12.	Mention the uses of phenylalanine.

13.	Write the uses of streptomycin.

14.	Outline the steps involved in alpha amylase.

15.	Write short notes on biopolymers.

PART – C  (5 x 15 = 75 MARKS)

16.	What are the traditional and modern applications of biotechnology? Explain.

(OR)

17.	Explain in detail about the industrial fermentation process.

18.	Describe the production of  lactic  acid and its uses with a neat flow diagram.

(OR)

19.	Explain the production of  butanol with a neat flow diagram.

20.	a. 	Discuss the various types of  secondary metabolites.								(5)

b. 	Explain in detail the production of  Penicillin with flow diagrams.					(10)

(OR)

21.	Enumerate on the recovery and purification process of  Vitamin B12.

22.	Describe in detail about the production of Xanthan gum.

(OR)

23.	Write briefly about biopesticides and biofertilizers.

24.	Discuss the therapeutic and diagnostic application of insulin and human growth hormone.

(OR)

25.	Define Vaccines and add a note on bacterial vaccines.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BIOORGANIC CHEMISTRY							      Time : 3 hours

Subject Code:	12BT212									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the symbol given for a molecule which rotates the plane polarized light towards right hand side?

2.	Name the component used to produce plane polarized light?

3.	What type of reaction mechanism is followed during the hydrolysis of tert-butyl bromide?

4.	Define carbocation.

5.	The formation of di-sulphide bridge is an example for __________ bond.

6.	What are the atoms responsible for hydrogen bond formation?

7.	The ribonuclease catalysed reaction is an example for __________ catalysis.

8.	For what kind of isomerism can the catalytic activity of fumerase be an example?

9. 	The enzyme that hydrolyze the NAM-NAG bond in a peptidoglycan layer is __________.

10.	What is the vitamin source for NAD coenzymes?

PART – B  (5 x 3 = 15 MARKS)

11.	All amino acids are optically active except glycine. Justify.

12.	Differentiate the hemolytic and heterolytic fission.

13.	Write a note on hydrophobic interactions.

14.	What is electrostatic catalysis?

15.	Explain the role of metal ion involved in the reaction sequence of bovine erythrocyte superoxide dismutase.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the ‘R’ and ‘S’ configuration in detail with examples.

(OR)

17.	Illustrate the principle and instrumentation of polarimeter.

18.	Describe the mechanism of unimolecular and bimolecular nucelophilic substitution reactions.

(OR)

19.	Discuss in detail about the mechanisms of elimination reactions.

20.	Explain the various types of chemical bonds.

(OR)

21.	How can the structure of proteins be stabilized by chemical bonds and forces? Explain.

22.	Discuss the mechanism of covalent catalysis of enzymes.

(OR)

23.	Explain the stereo specificity of enzyme action.

24.	Describe the mechanism of action of lysozyme enzyme.

(OR)

25.	Explain the structure and mechanism of NAD, Dependent oxidation and reduction reactions.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CELL BIOLOGY										      Time : 3 hours

Subject Code:	12BT213									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The overall shape of a bacterial cell is determined by _________.

2.	The chromosomes of a eukaryotic cell are located in which organelle?

3.	Microtubules are found in _________.

4. 	What are fibrous proteins?

5. 	Define endocytosis.

6. 	_________ facilitates the diffusion of a specific molecule in or out of the cell by passive transport.

7. 	_________ are signaling molecules of the nervous system.

8. 	Define endocrine signaling.

9. 	The calcium binding protein is _________.

10. 	How does cAMP synthesis take place and act as a second messenger?

PART – B  (5 x 3 = 15 MARKS)

11. 	Give an account on structure and function of Mitochondria.

12. 	What are intermediate filaments? Mention their types.

13. 	Differentiate between active transport and passive transport.

14. 	What are G protein coupled receptors?

15. 	Write about calcium ion flux and its role in cell signaling.

PART – C  (5 x 15 = 75 MARKS)

16. 	What are the different phases of cell cycle and the molecules that control cell cycle? Explain.

(OR)

17. 	Give an account on plasma membrane and its models.

18. 	Explain about cytoskeleton protein and give a detailed note on microtubules for cell structure and cell movement.

(OR)

19. 	Explain in detail about extracellular matrix and basal lamina in the cell.

20. 	Explain the entry of virus into a cell and its multiplication.

(OR)

21. 	Write about ligand gated and voltage gated channels against neuronal cell membrane.

22.	Give a brief account on endocrine molecule and its action in cell signaling.

(OR)

23.	Describe in detail about membrane bound receptor for cell signaling.

24.	Explain the role of inoisitol triphosphate (IP3) in signal transduction.

(OR)

25. 	How does cAMP and cGMP act as a second messenger in signal transduction? Explain.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MICROBIOLOGY										      Time : 3 hours

Subject Code:	12BT214									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define resolving power of microscope.

2.	What are basic stains? Give examples.

3.	Define binary fission.

4.	What is Protista?

5.	Define phagemid.

6.	Define YAC.

7.	What is MDR?

8.	What is facilitated diffusion?

9.	Define bioremediation.

10.	Write the role of Bacillus thuringiensis in agriculture.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the principle of Gram staining.

12.	Comment on fed – batch culture.

13.	Describe the life cycle of Euglena.

14.	What is an anti-fungal and an anti-viral agent?

15.	What is bioleaching?

PART – C  (5 x 15 = 75 MARKS)

16.	Write the principle and applications of scanning electron microscope with a neat diagram.

(OR)

17.	Discuss in detail on nomenclature and classification of microorganisms.

18.	Give a detailed account on antifungal agents.

(OR)

19.	Define growth. Write briefly different methods of measurement of microbial growth.

20.	Write in detail on structure and life cycle of Chlamydomonas species.

(OR)

21.	Describe the mode of reproduction in fungi.

22.	Discuss in detail on the structural organization and multiplication of bacteria.

(OR)

23.	Explain in detail on animal virus replication.

24.	Describe the mechanism of nitrogen fixation.

(OR)

25.	Discuss about the principle and operation of different types of biosensors with neat diagrams.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MOLECULAR BIOLOGY								      Time : 3 hours

Subject Code:	12BT215									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The protein around which DNA in Eukaryotic chromosomes is wound is called _________.

2.	The process of naked DNA uptake by competent bacteria is called _________.

3.	Which is the replicative polymerase of E.coli?

4.	The 3’ to 5’exonuclease activity of DNA polymerase III is also known as _________.

5.	The consensus sequence of _________ is called Pribnow box and that of _________ is called TATA box.

6.	The RNA polymerase core plus the _________ factor make up the Holoenzyme.

7.	List the tRNA binding sites on ribosomes and expand the abbreviations.

8.	The ribosome binding site of mRNA of prokaryotes is _________ and that of eukaryotes is _________.

9.	Does the Lac operon switch on when both glucose and lactose are present? Why?

10.	Lactose is both a substrate as well as an _________ of the lac operon.

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss the process of horizontal gene transfer involving cell-to-cell contact in E.coli discovered by Joshua Lederberg.

12.	What is rolling circle replication?

13.	Discuss the various subunits in an RNA polymerase in prokaryotes.

14.	State Wobble Hypothesis.

15.	Differentiate positive and negatively regulated operons.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe how eukaryotic nuclear DNA is organized into chromosome.

(OR)

17.	Describe:

a.	The experiments of Oswald Avery et. al. that indicated DNA is the genetic material. 	   (7)

b.	The experiment of Hersey and Chase that finally proved that DNA indeed is the genetic 	material. 																	    (8)

18.	Explain the process of DNA replication in prokaryotes and the enzymes involved.

(OR)

19.	Explain the agents that cause DNA Damage and the types of DNA damage and repair.

20.	Explain the steps and processes in RNA synthesis and explain prokaryotic transcription in detail.

(OR)

21.	Describe in detail RNA splicing as a part of the post transcriptional modification in eukaryotic mRNA synthesis.

[P.T.O.]

22.	Describe prokaryotic protein synthesis in detail.

(OR)

23.	Explain Eukaryotic translation in detail.

24.	How is gene expression regulated in prokaryotes? Explain with an example of a negatively regulated operon (lac operon).

(OR)

25.	Explain trp Operon in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	GENETIC ENGINEERING AND BIOETHICS			      Time : 3 hours

Subject Code:	12BT216									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define unique site.

2.	The enzyme ___________ seals breaks in the phosphodiester bond of DNA.

3.	Expand pBR322 and pUC19.

4.	The Ribosome Binding Site (RBS) for prokaryotes are called as __________.

5.	RFLP stands for ___________.

6.	How many primers are used in a PCR tube during RAPD?

7.	The process or method of transfer naked DNA to bacteria is called ___________.

8.	The enzyme that converts mRNA to cDNA is ___________.

9.	Production of biomolecules from GMO beyond ___________ liters is considered Large scale.

10.	List the three elements of containment.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between Northern, Southern and Western hybridization.

12.	What kind of vectors can be transferred between different species? Elaborate.

13.	Write a short note on Random Amplified Polymorphic DNA.

14.	Tabulate the differences between genomic library and cDNA library.

15.	What is the scope and functions of Recombinant DNA Advisory Committee (RDAC)?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Define restriction enzyme and describe in detail their classification. 				  (10)

b.	What is Star activity and chromosome walking?									    (5)

(OR)

17.	a.	Explain in detail how you would analyze proteins using blotting technique. 			  (10)

b.	What is DNA fingerprinting and autoradiography?								    (5)

18.	a.	Write the features an ideal cloning vector.  										    (5)

b.	You have to clone a DNA fragment of about 3Kb and you have a choice of two cloning vectors pBR322 and pUC19 .What will be your choice? Substantiate.  				  (10)

(OR)

19.	Explain the cloning vectors derived from bacteriophages.

20.	Explain the mechanism of Polymerase Chain Reaction (PCR).

(OR)

21.	Describe Taqman RT-PCR and RAPD in detail.

22.	A researcher in a molecular virology lab is asked to clone the genes expressed in the cell during Swine Flu virus infection. Which technique will he use? Explain the experiment in detail.

(OR)

23.	Describe the various methods by which a scientist can introduce DNA into cells.

[P.T.O]

24.	Elaborate on the stipulated guidelines on rDNA research activities.

(OR)

25.	Define bio-safety level and explain in detail the types of bio-safety levels.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	IMMUNOLOGY										      Time : 3 hours

Subject Code:	12BT217		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the features of innate immunity.

2.	Where is Bursa of Fabricius located?

3.	What are Lymph nodes?

4.	What is a hapten and an antigen?

5.	What is autoimmunity?

6.	Define immunological tolerance.

7.	What is the role of Interleukin I (IL 1) in hypersensitivity?

8.	What are sequestered antigens?

9.	What is Radioimmunoassay?

10.	Define Polyclonal serum.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the clonal selection theory.

12.	Define Null cells .Which type of null cells function as killer cells?

13.	State the laws of transplantation.

14.	Write short notes on immunoelectrophoresis.

15.	List out the applications of monoclonal antibodies.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the structure and function of the thymus and spleenwith neat illustrations.

(OR)

17.	Describe the different types of  monocytes and granulocytes occurring in the body

18.	Write short notes on:

a.	Major Histo compatibility 														    (3)

b.	Macrophages																	    (3)

c.	Neutrophils																	    (3)

d.	Dentritic cells																    (3)

e.	Mast cells																	    (3)

(OR)

19.	How many classes of antibodies are recognized mention briefly their properties? Describe the structure, characteristics and functions of IgG.

20.	What is complement system? What are its functions? Describe the process of alternative pathway.

(OR)

21.	What is hypersensitivity? Define Type I hypersensitivity and describe its general mechanism with a suitable diagram.

[P.T.O.]

22.	Explain the organization and  the role of MHC in Transplantation and explain their significance in transplantation immunology

(OR)

23.	Write short notes on:

a.	Tumour associate antigens 													    (3)

b.	Tumour specific antigens														    (3)

c.	Atopy																		    (3)

d.	Xenograft																	    (3)

e.	Isograft/syngraft																    (3)

24.	What is ELISA? Give a concise account of ELISA used for the detection and measurement of antibody.

(OR)

25.	Write short notes on

a.	Opsonization						 											    (3)

b.	Immunofluorescence 															    (3)

c.	Preciptin test																	    (3)

d.	Complement fixation 															    (3)

e.	Antibody Dependent Cell – mediated Cytotoxicity							 	    (3)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	UPSTREAM BIOPROCESSING							      Time : 3 hours

Subject Code:	12BT218				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Fermentation is defined as __________.

2.	Pathogenicity means__________.

3.	__________ is the method used to primarily screen the microbes which produce antibiotics.

4.	Lyophilizaton is otherwise called as __________.

5.	The organism which utilizes energy from carbon source is called as __________.

6.	__________ is used as the precursor for Penicillin V.

7.	For 2 variables response surface plot will  be ___________.

8.	The initial number of microorganism present in 2 liters of medium before sterilization is       1017 org/ml. Calculate Nabla factor.

9.	What is pitching?

10.	__________% of inoculum is added of culture volume.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the general requirements of fermentation process?

12.	Discuss the method to preserve isolated microbe under reduced temperature.

13.	What are chelators? Give example.

14.	2 litres of medium is taken for sterilization. The Del factor for heating and cooling is 1.12 and 1.3 respectively. The initial number of microbes before sterilization is 2.7x1016 org/ml. Calculate Holding time if  k is 2.5 min-1.

15.	Draw the process flow diagram of inoculum development.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the basic configuration of a fermentor with a neat diagram.

(OR)

17.	a.	What are the five groups of commercially important fermentation process available?	(7)

b.	Explain the various size reduction equipments used in industry.					(8)

18.  	Elaborate the primary screening methods followed in industries to isolate industrially important 	microbes.

(OR)

19.  	Discuss various methods of quality control procedures followed to maintain the quality of industrially important microbes in detail.

20. 	Discuss in detail about the basic requirement for the production of industrial fermentation medium.

(OR)

[P.T.O.]

21. 	For the following data calculate the difference, average difference, mean square, experimental error and factors showing larger effect.

| Factors | Carbon | Nitrogen | Vitamins | Minerals | Dummy-1 | Precurssor | Dummy-2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Σ(H) | 3.0 | 14.1 | 9.5 | 9.1 | 9.4 | 13 | 9.1 |
| Σ(L) | 14.0 | 4.3 | 9.3 | 9.7 | 9.4 | 4.8 | 9.6 |

22. 	Derive the expression for thermal death kinetics of microbes and arrive at the expression for del factor.

(OR)

23. 	Air is sterilized through a depth filter and is sent at a flow rate of 14 m3/min for fermentation process for 45 min with a linear velocity of 0.17m/min and the value of the rate constant is 1.55 m-1. Calculate:

a.	Initial number of microorganism present in air.

b.	Radius of the filter.

c.	Length of the filter.

d.	Cross sectional area of filter.

24. 	Write a detailed note on the development of inoculum for brewing process.

(OR)

25. 	What are the methods involved in the development of inoculum for Pseudomonas denitrificans for production of vitamin B12? Explain.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BIOPROCESS CALCULATIONS						      Time : 3 hours

Subject Code:	12BT219									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	1 torr = __________ Pa,

a.	13.32244         	b. 	133.3244           	c.	1.33            		d. 	1333244

2.	Molecular weight is defined as:

a.	mass/mole        	b.	mole/mass          	c.	mole x mass       	d.	massmole

3.	In distillation process, __________ = (individual compound in waste/feed) x 100

4.	__________ = (purge/recycle feed)

5.	__________ the reactant that would disappear first if a reaction goes to completion.

6.	__________ the ratio of stoichiometric coefficients of two molecular components in the balanced reaction.

7.	__________ is the energy in transmit between a hot source and a cold receiver.

8.	__________ is the energy which a substance/body possesses by virtue of its motion relative to some reference plane.

9.	__________ is defined as the quantity of heat required to raise the temperature of one gram of water by 1°C in the vicinity of 15°C.

10.	__________ is the heat required to vaporize a unit amount of solid at constant temperature and pressure.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the average molecular weight of air and its composition by weight?

12.	It is desired to make up 1,000 kg of a solution containing 35% by weight of a substance “A”. Two solutions are available, one containing 10 weight % of “A” and other containing 50 weight % of “A”. How many kilograms of each solution will be required?

13.	Ammonia is produced by the following reaction:

N2 + 3H2    →     2NH3

Calculate:

a. 	The molal flow rate of hydrogen corresponding to nitrogen feed rate of 25 kmol/hr, if they 	are fed in the stoichiometric proportion.

b. 	The kg of ammonia produced per hour, if percent conversion is 25 and nitrogen feed rate is 		25 kmol/hr.

14.	A stream of nitrogen flowing at a rate of 100 kmol/hr is heated from 303K to 373K. Calculate the heat that must be transferred.

Data: Cp° for nitrogen = 29.5909 5.141x103T + 11.1829x106T2    4.968x109T3

15.	Calculate the heat of reaction at 298.15K of the following reaction:

C2H6   →    C2H4 + H2

| Component | C2H6 | C2H4 | H2 |
| --- | --- | --- | --- |
| ΔH °C kJ/mol | –1560.69 | –1411.2 | –285.83 |

PART – C  (5 x 15 = 75 MARKS)

16.	Cracked gas from a petroleum refinery contains 45% methane, 10% ethane, 25% ethylene, 7% 	propane, 8% propylene and 5% butane by volume.

[P.T.O.]

Calculate:

a.	The average molecular weight of gas mixture

b.	The composition by weight 					c. 	Specific gravity of the gas mixture

(OR)

17.	A solution containing 55% benzene, 28% toluene and 17% xylene by weight is in contact with its vapour at 373K. Calculate the total pressure and molar composition of the liquid and vapour. Data: Vapour pressure data at 373K, Benzene = 178.60kPa, Toluene = 74.60kPa and Xylene = 28kPa.

18.	A multiple effect evaporator system has a capacity of processing 10,000 kg per day of solid caustic soda when it concentrates weak liquor from 4 % to 25% by weight caustic soda. When the same plant is fed with 10% weak liquor and if it is concentrated to 50% (both on weight basis), find the capacity of the plant in terms of solid caustic soda. Assume that the water evaporating capacity to be same in both cases.

(OR)

19.	A Dryer system handles 10,000 kg/day of wet solids. Wet solids containing 50% solids and 50% water are fed to the first dryer. From the first dryer the product that comes out has 20% moisture. This is admitted to the second dryer from which the product coming out has 2% moisture. Calculate the percentage of original water that is removed in each dryer and final weight of the product.

20.	In production of chlorine gas by oxidation of hydrochloric acid gas, air is used 30% in excess of that theoretically required. Based on 4 kmol HCl, calculate:

a. 	The weight ratio of air to hydrochloric acid gas in feed.

b. 	If oxidation is 80% complete, find the composition of product stream on mole basis.

(OR)

21.	Hydrogen is to be produced from methane by partial oxidation in the presence of steam. A charge to the reactor consists of 100kg methane at 698K, 100kg oxygen at 698K and 100 kg of steam at 1253K. The product gases are assumed to leave the reactor in chemical equilibrium at 1198K. The equilibrium constant value is 0.7 at 1198K.

Kp=yCO2.yH2yCO.yH2O=0.7

where y stands for the mole fraction of the component, calculate the moles in SI units of various components present in product gas.

22.	A natural gas has the following composition on mole basis

CH4 = 84%, C2H6 =13% and N2 =3%

Calculate the heat to be added to heat 10 kmol of natural gas from 298K to 523K using heat capacity data given below:

Cp = a+bT+cT2+dT3, kJ/(kmol.K)

| Gas | a | b x103 | c x106 | d x109 |
| --- | --- | --- | --- | --- |
| CH4 | 19.2494 | 52.1135 | 11.973 | 11.3173 |
| C2H6 | 5.4129 | 178.0872 |  67.3749 | 8.7147 |
| N2 | 29.5909 | 5.141 | 13.1829 | 4.968 |

(OR)

23.	Flue gas leaving the boiler stack at 523K have the following composition:

CO2=11.31%, H2O=13.04%, O2=2.17% and N2=73.48% (by volume). Calculate the heat lost in    1 kmol of gas mixture above 298K, using the heat capacity data given below:

Cp = a+bT+cT2+dT3 , kJ/(kmol.K)

| Gas | a | b x103 | c x106 | d x109 |
| --- | --- | --- | --- | --- |
| CO2 | 21.3655 | 64.2841 | 41.0506 | 9.7999 |
| H2O | 32.4921 | 0.0796 | 13.2107 | 4.5474 |
| O2 | 26.0257 | 11.7551 | 2.3426 |  0.5623 |
| N2 | 29.5909 | 5.141 | 13.1829 | 4.968 |

24.	Obtain an empirical expression relating the heat of reaction and the temperature of the reaction for the following reaction.

SO2 + ½ O2    →  SO3

Using the same expression, calculate the heat of reaction at 773K

Cp° = a+bT+cT2+dT3 , kJ/(kmol.K)

Data:

| Component | ΔHf o kJ/kmol |
| --- | --- |
| SO3 | 3,95,720 |
| SO2 | 2,96,810 |

| Component | a | b x103 | c x106 | d x109 |
| --- | --- | --- | --- | --- |
| SO3 | 22.036 | 121.624 |  91.867 | 24.369 |
| SO2 | 24.771 | 62.948 |  44.258 | 11.122 |
| O2 | 26.026 | 11.755 | 2.343 |  0.562 |

(OR)

25.	Calculate the heat of reaction at 773K for the following reaction.

CO2 + 4H2   →  CH4 + 2H2O

ΔHR o  =  165.1 kJ/mol

Heat capacity data:                Cp° = a+bT+cT2+dT3 , kJ/(kmol.K)

| Component | a | b x103 | c x106 | d x109 |
| --- | --- | --- | --- | --- |
| CO2 | 21.3655 | 64.2841 |  41.0506 | 9.7999 |
| H2 | 28.6105 | 1.0194 | 0.1476 | 0.769 |
| CH4 | 19.2494 | 52.1135 | 11.973 |  11.3173 |
| H2O | 32.4921 | 0.0796 | 13.2107 |  4.5474 |

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DOWNSTREAM PROCESSING							      Time : 3 hours

Subject Code:	12BT220									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Bioseparations engineering refers to __________.

2.	Reactive bioprocessing is defined as __________.

3.	Filter aid is defined as __________.

4.	In centrifugation, instantaneous velocity is defined as __________.

5.	Physical process extraction is __________.

6.	The types of two phase extraction are __________, __________.

7.	An example for column chromatographic techniques is __________.

8.	Formulae of ΔG° = __________.

9.	Percentage of moisture content in lyophilized product is __________.

10.	Sublimation is __________.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the unit operations involved in an intracellular product?

12.	Define Filtration.

13.	Give short notes on Adsorbent and Adsorbate.

14.	Draw the flow diagram of chromatography.

15.	Write short notes on Crystallization.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss the methods of pretreatment and stabilization of bioproduct.				(8)

b.	Explain any one mechanical method of cell disruption.							(7)

(OR)

17.	Give an account of the action of enzymes in cell disruption. How is the method useful in sequential release of products?

18.	A 30 ml sample of broth from penicillin fermentation is filtered in the laboratory on a 3 cm2 filter at a pressure drop of 5 psi. The filtration time is 4.5 minutes. The filter cake of Penicillium chrysogenum is significantly compressible with s = 0.5. If 500 liters broth from a pilot-scale fermenter must be filtered in 1 hour, what size filter is required if the pressure drop is:

a.	10 psi		b.	5 psi

Resistance due to the filter medium is negligible.

(OR)

19. 	A continuous disc-stack centrifuge is operated at 5000 rpm for separation of bakers' yeast. At a feed rate of 60 L/ min, 50% of the cells are recovered. At constant centrifuge speed, solids recovery is inversely proportional to flow rate.

a.	What flow rate is required to achieve 90% cell recovery if the centrifuge speed is 	maintained at 5000 rpm?

b.	What operating speed is required to achieve 90% recovery at a feed rate of 60 L/min?

[P.T.O.]

20. 	Aqueous two-phase extraction is used to recover -amylase from solution. A polyethylene glycol-dextran mixture is added and the solution separates into two phases. The partition coefficient is 4.2. Calculate the maximum possible enzyme recovery when:

a.	the volume ratio of upper to lower phases is 5.0.

b.	the volume ratio of upper to lower phases is 0.5.

(OR)

21. 	Explain different methods of precipitation of proteins.

22. 	Explain the principle, practice and application of Gas Chromatography and High Performance Liquid Chromatography with neat diagrams.

(OR)

23. 	A pilot scale gel-chromatography column packed with Sephacryl resin is used to separate two hormones A and B. The column is 5 cm in diameter and 0.3 m high; the void volume is           1.9 x10 4 m 3. The water regain value of the gel is 3 x10 3 m 3/kg dry Sephacryl; the density of wet gel is 1.25 x103 kg/m3. The partition coefficient for hormone A is 0.38; the partition coefficient for hormone B is 0.15. If the eluant flow rate is 0.7 L/ h, what is the retention time for each hormone?

24. 	Explain the principle and opearation of spray dryer, rotary dryer and tray dryer with diagram.

(OR)

25. 	Explain the different types of crystallizers with a neat diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CHEMICAL THERMODYNAMICS AND BIOENERGETICS														      								      Time : 3 hours

Subject Code:	12BT221										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a “System” in thermodynamics? Give an example.

2.	What does the Second law of thermodynamics signify?

3.	What is an isothermal process?

4.	Change in entrophy is notated as _________.

5.	Define phase rule.

6.	What do you mean by thermal equilibrium?

7.	What is an endothermic process?

8.	What is entrophy?

9.	Expand  NADH.

10.	Define “Bioenergetics”.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the scope of thermodynamics? State what is an adiabatic process?

12.	Define activity coefficient.

13.	Brief on azeotropes.

14.	What do you understand about IC engines?

15.	Write a short note on liquefaction.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the expression for Maxwell’s relation.

(OR)

17.	Describe the thermodynamic property of fluids and derive an expression for the same

18.	Discuss partial molar property. Add a note on chemical potential and fugacity.

(OR)

19.	 What does Gibbs-Duhem equation signify? Derive the Gibbs Duhem Equation.

20.	Explain in detail about effect of temperature and pressure on equilibrium constant.

(OR)

21.	a.	Discuss the criteria for chemical reaction equilibrium.								(7)

b.	Explain and draw the VLE diagram for multi-component system.					(8)

22.	Give a detailed account on the “Steam Power Plant”.

(OR)

23.	Discuss on Internal combustion engines.

24.	Explain the metabolic pathways energy coupling ATP and NADH.

(OR)

25.	Describe in detail about biological oxidation and reduction reactions.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BIOPHARMACEUTICAL TECHNOLOGY				      Time : 3 hours

Subject Code:	12BT222									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Pharmacodynamics is defined as

a.	The effect of our body has on drugs.

b.	The action of the liver on drug molecules.

c.	The effect of a drug has on our body.

d.	The movement of a drug around the body.

2.	Write any four routes of administration.

3.	____________ are solid dosage forms in which medicinal agents and/or inert substances are enclosed in a small shell of gelatin.

4.	_________ can be conducted on a tablet press using slugging tooling or on a roll press called a roller compactor.

a.	Wet Granulation.

b.	Dry Granulation.

c.	Tableting.

d.	Coating.

5.	Primary emulsion contains ____________ is an internal phase.

6.	How is ointment prepared?

7.	Which drugs promote the evacuation of bowel in milder action, elimination of soft but formed stools?

8.	It is a mixture of toxoids of tetanus and diphtheria and pertussis vaccine.

a.	Triple antigen.

b.	Double antigen.

c.	Single antigen.

d.	Double vaccine.

9.	What is meant by a double blind clinical trial?

10.	Which of the following areas of study is part of preclinical trials?

a.	Drug design.

b.	Lead discovery.

c.	Formulation.

d.	Structure-activity relationships.

PART – B  (5 x 3 = 15 MARKS)

11.	Define the following:

a.	Pharmacognosy			b.	Biopharmaceutical

12.	What are two essential properties of powders intended for compression into tablets?

13.	List out the advantages of parenteral drug solutions.

14.	Define the following:

a.	antacids					b.	antiseptics

15.	What is placebo effect?

[P.T.O.]

PART – C  (5 x 15 = 75 MARKS)

16.	Describe about drug review process and explain the various phases of drug development process.

(OR)

17.	Explain in detail about the two phases of drug metabolism with any three suitable examples for each phase.

18.	Enumerate the stages of tablet formation and tableting process, dry and wet granulation methods.

(OR)

19.	Discuss in detail about the quality control tests for tablets and capsules.

20.	Give an account of parental solutions and ointment preparation methods.

(OR)

21.	Write short notes on the following:

a.	injections		b.	oral liquids		c.	packaging of semisolid dosage forms.	(3x5)

22.	Elucidate the basics of following:													(5x3)

a.	cold remedies	b.	laxatives		c.	analgesics		d.	antiseptics

e.	antibiotics

(OR)

23.	Explicate the recent advances in the manufacturing of drugs using rDNA technology.

24.	Write about the clinical trials and regulations of drug manufacturing and write any three major regulations of FDA and Indian drug regulations.

(OR)

25.	List out the good laboratory practices and good manufacturing practices to be followed in the pharmaceutical industry.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	PLANT BIOTECHNOLOGY AND TISSUE CULTURE	      Time : 3 hours

Subject Code:	12BT223										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define MS media.

2.	What is Microsatellites?

3.	The methods used in protoplasmic fusion are ___________, ___________ and ___________.

4.	Give any two examples of artificial seeds produced from transgenic plants.

5.	What are the advantages of direct DNA transfer method in plants?

6.	What is T-DNA?

7.	Give any two examples of nematode resistance plants.

8.	Define antifungal proteins.

9.	Name any two examples of edible vaccines.

10.	List out any two industrial enzyme.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish RAPD and SCAR markers with suitable examples.

12.	Comment on significance of somaclonal variation.

13.	Differentiate between Ti and Ri plasmids.

14.	Write short notes on vertical and horizontal resistance.

15.	Comment on therapeutic proteins applications with suitable examples.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss in detail about the mitochondrial and chloroplast genome structure.

(OR)

17.	Give a brief account of molecular marker-aided breeding with special reference to RFLP maps.

18.	Define the following:

a.	Protoplast 		b.	Viability test 	c.	Protoplast fusion 		d.	Cybrids

e.	Artificial seeds

(OR)

19.	Describe in detail the somatic embryogenesis and give a note on its applications.

20.	Explain the role of reporter genes and selectable markers in gene transfer in plants.

(OR)

21.	Write a note on different vectors used in Agrobacterium mediated gene transfer in plants.

22.	Discuss in detail the herbicide resistance and insect resistant plants with suitable illustrations.

(OR)

23.	Give an account on techniques used in plant breeding for diseases resistances.

24.	Write about the production of edible vaccines and transgenic plants using plant tissue cell         	culture along with its applications.

(OR)

25.	What are the techniques used for in-vitro production of secondary metabolites? Give the  	significance of bioactive compounds.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ANIMAL BIOTECHNOLOGY AND TISSUE CULTURE	      Time : 3 hours

Subject Code:	12BT224										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the concentration of Penicillin and Streptomycin in 1ml of the medium?

2.	Tripsin will be active at __________ temperature.

3.	Anthrax is an acute disease caused by the bacterium ____________.

4.	What is a Probe?

5.	What are cytokines?

6.	Name the  chemical used for attenuation of Rabies Virus for vaccination.

7.	What is Microinjection?

8.	Define Artificial insemination.

9.	Which cell stage of the embryo is much suitable to transfer in the Womb of the mother?

10.	Name any one viral vector used for gene transfer in animals.

PART – B  (5 x 3 = 15 MARKS)

11.	Write a note on the role of CO2 in cell culturing.

12.	What is the method of disaggregation of cells using collagenase?

13.	Write a note on the Rituximab.

14.	What is meant by Embryo transfer technology?

15.	Write a short note of HSV- TK in homologus recombination.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Draw the layout plan of a cell culture lab and label it properly. 					(5)

b.	Write a note on chemically defined medium with examples.     						(5)

c.	Write the procedure for the cell culture preservation.                						(5)

(OR)

17.	Write a note on the following:

a.	Primary cell culture with neat diagrams.                           							(7)

b.	Production of tissue Plasminogen Activator. 										(8)

18.	Explain in detail about Anthrax and Rabies infection in animals.

(OR)

19.	Explain the following:

a.	Translocation of gene loci caused due to Folicular Lymphoma and its detection. 		(8)

b.	How Alu-PCR is used to isolate HumanSpecific DNA sequence? Explain.			(7)

20.  Describe the following:

a. 	How cytokines are produced with a neat diagram.          							(7)

b. 	Write note on Rebif a recombinant interferon.             							(4)

c. 	Write the product case study of Viraferon Peg.           								(4)

(OR)

[P.T.O.]

21.	Explain in detail about the production of Rabies vaccine with a neat flow diagram.

22.	Describe in detail about Enrichment of X and Y bearing sperms from semen and its identification methods.

(OR)

23.	Explain the steps involved In Vitro fertilization and embryo transfer.

24.	Describe the following methods of gene transfer:

a.	Electroporation. 																    (5)

b.	Particle Bombardment. 														    (5)

c.	Ultrasound. 																	    (5)

(OR)

25.	Describe about the ethical issues involved when we develop transgenic animals.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BASICS OF BIOINFORMATICS						      Time : 3 hours

Subject Code:	12BT225		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Give an example for DNA databank.

2. 	Expand SCOP.

3. 	Give an example for multiple sequence alignment tool.

4. 	Define parametric sequence alignment.

5. 	What are substitution matrices?

6. 	What is HSP?

7. 	State any one difference between Horizontal gene transfer and Vertical gene transfer in  	phylogeny.

8. 	State any one tool to construct phylogenetic tree.

9. 	Name the scientist who developed chain termination method in DNA sequencing.

10. 	Give any one application of gene prediction in drug discovery.

PART – B  (5 x 3 = 15 MARKS)

11. 	Describe about ftp protocols.

12. 	What is string similarity? Explain.

13. 	Write about BLOSUM matrices in detail.

14. 	Explain about parsimony methods in phylogenetic tree construction.

15. 	Write a brief note on Gene prediction methods.

PART – C  (5 x 15 = 75 MARKS)

16. 	Write about Nucleotide sequence databanks in detail.

(OR)

17. 	Explain the importance of structural databank in the development of Biological          Databanks in detail.

18. 	Discuss on local alignment in detail.

(OR)

19. 	Explain about Parametric and Suboptimal methods in detail.

20. 	Write about BLAST in detail.

(OR)

21. 	Write about aminoacid substitution matrices in detail.

22. 	Explain about the common multiple sequence methods in detail.

(OR)

23. 	Give a detailed account on application of MSA in phylogenetic tree construction in detail.

24. 	Give a detailed explanation about map alignment and sequence assembly in detail.

(OR)

25. 	Elaborate on DNA sequencing methods.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ROLE OF  BIOTECHNOLOGY IN ENVIRONMENT	      Time : 3 hours

Subject Code:	12BT251										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	__________ refers to the complete or nearly complete eradication of human civilization by nuclear warfare.

2. 	What is meant by water pollution?

3. 	Mention the radioactive pollutants present in water.

4. 	What is meant by COD?

5. 	What is meant by biomass?

6. 	Give two examples of earthworm species used in vermicomposting.

7. 	Mention any two National Parks in India.

8. 	Define biodiversity.

9. 	__________ is defined as the process whereby organic wastes are biologically degraded under controlled conditions.

10. 	__________is a site for the disposal of solid waste in which refuse is buried between layers  so as to fill in or reclaim low-lying ground.

PART – B  (5 x 3 = 15 MARKS)

11.	Brief on two of the biggest nuclear disasters.

12.	Distinguish between aerobic and anaerobic waste water treatments.

13.	Write short notes on biogas.

14. 	State the Hotspots in Biodiversity.

15.	What is meant by genetically engineered microorganisms?

PART – C  (5 x 15 = 75 MARKS)

16. 	Describe the causes and effects of water pollution and global warming.

(OR)

17.	Outline on the causes and effects of acid rain and ozone depletion.

18.	Explain the steps in aerobic and anaerobic waste water treatment.

(OR)

19.	Explain the principle and operation of BOD and COD biosensors with a neat diagram.

20.	Describe the production of biogas with a neat diagram.

(OR)

21.	Discuss in detail about biomass as a source of renewable energy.

22. 	Describe about the in situ and ex situ methods of biodiversity conservation.

(OR)

23.	a. 	Sketch on the loss of biodiversity.

b. 	Write short notes on values of biodiversity.

24.  Give a note on in situ and ex situ methods of bioremediation.

(OR)

25.	Discuss on genetically engineered microbes employed for bioremediation.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	TISSUE ENGINEERING								Time : 3 hours

Subject Code: 	12BT319								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Write a detailed note on ethical and regulatory issues associated with tissue engineering.

2.	Elaborate structure and organization of epithelial cells.

(OR)

3.	Discuss various Phases of wound healing process.

4.	Explain various aspects of animal cell culture like cell expansion, cell transfer, cell storage and its characterization. 													   (5+5+5+5)

(OR)

5.	Elaborate cell-cell interactions in detail.

6.	What is role of second messengers in cell signaling and discuss various types of second messengers.

(OR)

7.	Explain differential cell adhesion hypothesis. How different cell populations are separated by FACS? 																		(10+10)

8.	What is the role of scaffolds and their essential properties (Biocompatibility, Porosity, Mechanical strength etc.) in bone tissue engineering?

(OR)

9.	a.	Give a brief note on various type of stem cells.

b.	How Red Blood Cells (RBCs) can be obtained from embryonic stem cells? 		(10+10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	ADVANCED ANIMAL BIOTECHNOLOGY				Time : 3 hours

Subject Code: 	12BT323									       	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Describe in detail the techniques, reagents and culturing process of animal cells with reference to the proper aseptic procedures.

(OR)

2.	Elucidate the preservation and cloning mechanism for the selection and production of cell lines. Brief out on the assays for characterization of cell lines.

3.	Detail the scale up mechanism from cell vials to large scale reactors and detail the role of aeration and mixing.

(OR)

4.	Briefly explain microencapsulation process along with the production process of vaccines and pharmaceutically important proteins.

5.	Define the molecular diagnostic methods in animal forensics and detail the process of GMO detection using biosensors.

(OR)

6.	Define development of transgenic animal; give a brief account on various animal models used in biopharming.

7.	Compulsory:

Xenotransplantation of organs and its implications, expression of recombinant proteins in cell lines: explain these briefly.

8.	Describe and detail the ELSI on stem cell therapy and elucidate the various biosafety levels in animal cell culturing.

(OR)

9.	Describe the patentable aspects and inventions in animal biotechnology and detail the effect 	of molecular methods for bioterrorism.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	INDUSTRIAL SAFETY ENGINEERING					Time : 3 hours

Subject Code: 	12BT327										  Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Explain the case study of Bhopal gas tragedy, Seveso disaster and Texas refinery        explosion.

2.	a.	Discuss the criteria to be followed in the design of site layout and planning in            	industry. 										      						      (10)

b.	What are the precautionary measures to be taken while handling dangerous and toxic 	chemicals in industry? Explain.							      				      (10)

(OR)

3.	a.	Describe the various high pressure and high temperature operations carried out in              	industries.									      	      						      (10)

b.	Explain the different types of equipments used for handling materials and machineries in 	industry.										      							      (10)

4.	a.	Describe the different types of ventilation and exhaust system used in industries.       (10)

b.	Discuss the various sources of noise that affects the workers in industry.	      	      (10)

(OR)

5.	a.	Explain the different types of instruments used for measuring the noise levels in        	industry.										      							      (10)

b.	Discuss the various light sources used in industry.				      			      (10)

6.	a.	Explain the different types of Non – respiratory protective equipments used in          	industry.										      							      (10)

b.	What are the steps to be followed for accident prevention in industry? Explain.  	      (10)

(OR)

7.	a.	How will you identify and analyze the causes of injury to men and machineries?      	Explain.									                 						      (10)

b.	Explain the different types of fire prevention and protection equipments used in industry with neat diagram.								  	     					      (10)

8.	a.	Discuss the role of government and trade union in promoting safety and health training 		in industry.										      						      (10)

b.	Write note on “on – site” and “off – site” safety provisions in industry.	      		      (10)

(OR)

9.	a.	Explain the various occupational health hazards encountered in industry.	            (15)

b.	Discuss the different types of major industrial hazards.	     	       	       				(5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	BIOPHARMACEUTICAL TECHNOLOGY				Time : 3 hours

Subject Code: 	12BT330									       	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Explain the importance of microbes in Pharmaceutical industry.		      		      (10)

b.	Discuss the origin, scope and applications of Pharmaceutical Biotechnology.             (10)

2.	a.	Explain the mechanism involved in distribution of drug and metabolism of drug in 	human system.										      					      (15)

b.	Differentiate between Pharmacokinetics and Pharmacodynamics.		       			(5)

(OR)

3.	a.	Explain the mechanism of action of drug absorption from the gastro intestinal tract.   (15)

b.	Write note on ADME properties of drug.					       					 (5)

4.	a.	Describe the various forms of oral solid dosages.				      				      (10)

b.	Discuss the role of preservatives in drug formulation.				      			      (10)

(OR)

5.	a.	Explain the process involved in manufacturing of tablets with a neat flow diagram.   (10)

b.	Write notes on capsules, oral liquids and ointments.				      			      (10)

6.	a.	What is the role of protein in drug delivery system? Explain.			      		      (10)

b.	Explain the production of any two secondary metabolites. 			      			      (10)

(OR)

7.	a.	Discuss the mechanism involved in controlled drug delivery system.	                     (10)

b.	Explain the mechanism of action of liposome mediated novel drug delivery system.  (10)

8.	a.	Describe the production of human growth hormone with a neat diagram.      		      (10)

b.	What is gene therapy? Explain its clinical importance.				      		      (10)

(OR)

9.	a.	Explain the importance of monoclonal antibody and recombinant human 	deoxyribonuclease in pharmaceutical industry.				  			 	     (15)

b.	Differentiate somatotropin from somatostatin.			     	       	      				(5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	RESEARCH METHODOLOGY							Time : 3 hours

Subject Code: 	12BT333								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Write an essay on:

a.	Laboratory safety. 															(5)

b.	Biosafety in biology laboratories. 												(7)

c.	Recombinant material safety. 													(8)

2.	Describe the four major classes of measurements.

(OR)

3.	Explain in detail about the care and use of animals in scientific research.

4. 	Describe the different types and methods of data collection.

(OR)

5. 	Explain probability and non probability sampling.

6. 	Describe Regression analysis in detail.

(OR)

7.	Write an essay on the graphical presentation of data.

8. 	a.	What are the different types of research studies? 								      (10)

b. 	Explain in detail the major components of a written research report. 			      (10)

(OR)

9. 	Explain the adopted methods of citing the following types of references:

a. 	referencing texts and quotes.

b. 	referencing internet sites.

c. 	referencing photos.

d. 	referencing figures or tables. 												    (4x5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	INSTRUMENTATION AND BIOTECHNIQUES			Time : 3 hours

Subject Code: 	12BT334										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	Explain and derive Beer Lamberts’ Law. 									      (10)

b.	Describe the principle and applications of UV-Vis spectrophotometer.			      (10)

(OR)

2.	Write an essay on the principle and applications of Fluorimetry.

3.	Write an essay on Ion Exchange Chromatography.

(OR)

4. 	Describe the principle and application of Polyacrylamide gel electrophoresis.

5. 	What are fluorochromes? Give examples. Explain the working principle of fluorescent   microscope with a neat diagram.

(OR)

6.	Describe the principle, types and application of ELISA.

7. 	Explain the application of radiation in sterilisation.

(OR)

8. 	Describe the different methods of enzyme immobilisation.

9.	Compulsory :

Write an essay on BLAST.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	MOLECULAR AND CELLULAR BASIS OF CANCER

Time : 3 hours

Subject Code: 	12BT341										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Summarize the following:

a.	Ras / MAPk pathway.

b.	JAK-STAT pathway.

2.	Illustrate the idea of tumor markers with a few examples.

(OR)

3.	Carcinogenesis is the process by which normal cells are transformed into cancer cells.  Justify the statement with a legible diagram.

4.	Infere the mechanism behind activation of oneogene with a neat diagram.

(OR)

5.	Elaborate the role of antioxidants in cancer progression and prevention.

6.	Briefly describe about GI cancer and its types.

(OR)

7.	Give an outline of epigenetic silencing of suppressor genes.

8.	Depict the role of immunotherapy in cancer.

(OR)

9.	Contrast the utility of surgery in cancer treatment.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	PHYTOCHEMICALS AND HERBAL MEDICINE		Time : 3 hours

Subject Code: 	12BT342										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Write an account on nutraceuticals and their significance.

2.	Bring out the classification of crude drugs.

(OR)

3.	Comment on Indian system of medicine.

4.	How do biotechnological tools influence the medical plant cultivation?

(OR)

5.	Write an account on cultivation of medical plants in India.

6.	Analyze the secondary metabolite production in plants through tissue culture method.

(OR)

7.	Describe biogenesis of phytopharmaceuticals.

8.	Critically comment on drug evaluation method.

(OR)

9.	Explain the extraction method of resins from plants.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	ADVANCED PLANT BIOTECHNOLOGY				Time : 3 hours

Subject Code: 	12BT343									       	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Analyze the Agrobacterium mediated gene transfer technique in plants.

2.	How will you isolate and purify DNA from plant tissues?

(OR)

3.	Discuss the types of blotting techniques.

4.	Explain the role of molecular scissors in genetic engineering.

(OR)

5.	Describe the concept of RNA interference.

6.	Elucidate the physical method of gene transfer in plants.

(OR)

7.	Comment on silicon carbide mediated gene transfer technique in plants.

8.	Write a detailed account on herbicide resistance plants.

(OR)

9.	Give a note on edible vaccines in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INSTRUMENTAL METHODS OF ANALYSIS			      Time : 3 hours

Subject Code:	12BT206				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name the principal cation of extracellular fluid.

2.	What is the ratio of bicarbonate to carbonic acid at a normal blood pH?

3. 	Write Beer - Lambert’s law.

4. 	Mention the use of flame photometer.

5. 	Mention the advantages of using agarose gel in electrophoresis.

6. 	Visual output of the chromatograph is said to be __________.

7. 	__________ is an atom with a specific number of protons and neutrons in the nucleus.

8. 	The total number of protons and neutrons in an atomic nucleus is called as __________.

9. 	Which technique is used as a tool for studying curing processes in polymer industries?

10. 	List any two applications of DSC.

PART – B  (5 x 3 = 15 MARKS)

11. 	Write the Henderson-Hasselbalch equation for bicarbonate buffer.

12. 	What is spectrophotometer? Distinguish spectrophotometer from colorimeter.

13. 	Differentiate between analytical and preparative chromatography.

14. 	Define half life (t1/2) and mean lifetime (τ).

15. 	Define thermogravimetry and add a note on microbalance.

PART – C  (5 x 15 = 75 MARKS)

16. 	Discuss the role of blood buffers in acid-base balance and respiratory mechanism for Ph regulation.

(OR)

17. 	Derive Henderson-Hasselbalch equation and summarize the errors affecting pH measurements with glass electrode.

18. 	Illustrate the basic components of a spectrophotometer.

(OR)

19. 	Explain the types of Raman spectroscopy and add a note on its applications.

20. 	Give a brief explanation on the types of detectors used in gas chromatography.

(OR)

21. 	Describe the types of electrophoresis.

22. 	Summarize on various applications of radioactive isotopes.

(OR)

23. 	Explicate the types of radioactive decay processes.

24. 	Explain the principle and instrumentation of thermogravimetry.

(OR)

25. 	Elucidate the construction and working principle of DTA and DSC with a neat diagram.