Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BASIC ELECTRICAL ENGINEERING					      Time : 3 hours

Subject Code:	09EE101									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Electric power = _____/______.

2.	When a current I flows into a network of two resistances R1 and R2 connected in series, then the formula for voltage (V1) across R1 is given by __________.

3.	The flux which does not follow the desired path in a magnetic circuit is __________.

4.	Define Magnetic field intensity.

5.	__________ connects the valve house and power house in a hydro power plant.

6.	The sinusoidal alternating voltage equation is given by, v =__________.

7.	The back emf equation of a d.c. motor is given by, Eb =__________.

8.	The transformer works on the principle of __________.

9.	The instrument used for measuring dc quantities is __________.

10.	Induction type energy meter is an indicating instrument.  State true or false.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Ohm's law and mention its limitations.

12.	A coil has 2000 turns and it carries a current of 10A. If the length of the magnetic path is 20	metre, find magnetic field intensity (H).

13.	List the advantages of 3-phase system.

14.	A 6-pole lap wound d.c. generator has 51 slots, each slot containing 18 conductors. The useful flux is 35 milli webers. Find the generated e.m.f. of the machine, if it is driven at a speed of 750 r.p.m.

15.	What are the different types of wiring?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Define the Kirchoff’s Laws and illustrate the use of Kirchoff’s Laws in a circuit.		    (5)

b.	Calculate:

a.	the equivalent resistances across the terminals of the supply.

b.	total current 	supplied by the source.										  (10)

(OR)

[P.T.O.]

17.	Determine the equivalent resistance Req of the circuit given below.

18.	a.	Compare electric and magnetic circuits.    										    (5)

b.	Briefly write about induced EMF and its types.									  (10)

(OR)

19.	a.	Write down the laws of electromagnetic induction and explain their significance.		    (7)

b.	Prove that the coupling co-efficient between two magnetic circuits is:				    (8)

k=ML1L2

20.	a.	With a neat sketch describe the operation of thermal power generating station.		  (10)

b.	Compare overhead and underground distribution systems.							    (5)

(OR)

21.	a.	An alternating sinusoidal voltage equation is given by V = 230 sin 314t. From the equation 		find Vm, Vavg, Vrms, Form factor, Peak factor, Frequency, Time period.		  		  (10)

b.	Draw the single – line diagram of a power system. 								    (5)

22.	Explain the construction and operating principle of a DC Generator in detail.

(OR)

23.	a.	Explain the principle of operation of a transformer. 								   (7)

b.	Explain the principle of operation of any one type of single phase induction motor with 			necessary diagrams.															   (8)

24.	Describe the working of:

a.	Induction type Energy meter		b. 	Moving coil instrument					(8+7)

(OR)

25.	a.	Describe the operation of staircase wiring with a neat diagram.	       					    (8)

b.	What are the different methods of earthing?  Explain them briefly.					    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRONIC CIRCUITS								      Time : 3 hours

Subject Code:	09EE201		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	If the output voltage of a bridge rectifier is 100V, the PIV of diode will be_____________.

2.	To have less ripples in the DC output, the value capacitor should be _____________.

3.	Which wave shaping circuit is used to trigger the electronic devices?

4.	If the capacitor in an RC integrator gets short circuited, the output voltage will be ___________

5.	The base current flowing through the base resistance of differential amplifier are Ib1=85µA and Ib2=750µA. Calculate the input offset current.

6.	Which electronic circuit is used to match the resistance between the load and amplifier by having a proper turns ratio in the coupling transformer?

7.	In class–A amplifier, the output current flows for the  ____________ cycle of the input signal.

8.	In an amplifier with negative feedback, the bandwidth is increased by a factor of ____________.

9.	A bistable multivibrator is a Triggered oscillator.  Say True or False.

10.	Wien bridge oscillator can typically generate frequencies in the range of_____________.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the limitations of Linear Voltage Regulator?

12.	What voltage will the capacitor charge up to in the given circuit for the single input pulse shown in the figure?

13.	Draw the frequency response of RC coupled amplifier. What are the advantages and disadvantages of RC coupled amplifier?

14.	What are the characteristics of Class A Amplifier?

15.	Compare RC phase shift oscillator and Wien Bridge Oscillator.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With relevant circuit diagram, explain the working principle of LC Filter. 			    (7)

b.	A full wave rectifier is fed with a voltage, 50 sin 100 πt. Its load resistance is 400Ω. The diodes used in the rectifier have an average forward resistance of 30Ω. Compute the          i.	average and rms values of load current		ii.	ripple factor 		iii.	efficiency of rectification.																	    (8)

(OR)

[P.T.O.]

17.	a.	With relevant circuit diagram, explain the working principle of transistor series voltage 	regulator.																	    (7)

b.	With neat circuit diagram, explain the protection circuits used for series voltage regulators. 																					    (8)

18.	a.	With relevant circuit diagram and waveforms, explain how sine wave and ramp signals are 	differentiated using differentiating circuit. 										    (8)

b.	With neat circuit diagram and wave forms, explain the biased positive and negative 	clampers.																	    (7)

(OR)

19.	a.	Obtain the output voltage of RC high pass circuit with the step input voltage applied at 	t=t1. Also obtain the expression for the output voltage.							    (7)

b.	Obtain the output voltage Vout  in the circuit given in the figure.					    (8)

20.	Design a single stage common emitter BJT amplifier for the circuit given in the figure. Given data: Required Gain 75, Output voltage 4V, hfe=330, hie=4500 Ω, hoe=30µA/V, hre=2×10-4 (Assume all other required data).

(OR)

21.	With relevant circuit diagram, explain the difference mode and common mode operations of differential amplifier. Also analyze the different configuration of differential amplifier with circuit diagrams.

22.	a.	The transistor in the feedback circuit given in figure 4 has β=200. Determine:

i.	feedback factor		ii.	feedback ratio		iii.	voltage gain without feedback

iv.	voltage gain with 	feedback in the circuit. In the transistor, under the conditions of 	operation, VBE may be assumed to be negligible. 			      		       	        		  (10)

b.	Explain the effects of negative feedback on BJT amplifiers.						    (5)

(OR)

23.	a.	A class B push-pull amplifier is supplied with Vcc=40V. The minimum voltage reached by 	the collector due to signal swing is Vmin = 8V. The dissipation in both the transistors totals 	30 W. What is the conversion efficiency of the amplifier?							    (7)

b.	With relevant circuit diagram and waveforms, explain the operation of complementary 	symmetry class-B push pull amplifier.											    (8)

24.	a.	With relevant circuit diagram, explain the working principle of Hartley Oscillator.	  (10)

b.	A Hartley Oscillator is designed with L1=20µH, L2=2mH and variable capacitance. 	Determine the range of capacitance value, if the frequency is varied between 950 kHz and 	2050 kHz.	    																    (5)

(OR)

25.	a.	With relevant circuit diagram and waveforms, explain the working principle of collector 	coupled Astable Multivibrator.					           							  (12)

b.	A monostable multivibrator is used as a voltage-to-time converter. Find the time period and frequency if R = 10kΩ, C = 0.01µF, VBBVCC=0.5 .								    (3)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DIGITAL ELECTRONICS								      Time : 3 hours

Subject Code:	09EE202									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	To convert an octal number to binary, each octal digit is replaced by its _______Bit Binary equivalent.

2.	Convert (7421)8 to base 16.

3.	A logic circuit with an output X=ABC+AC

4.	The output expression for an AND-OR-INVERT circuit having one AND gate with inputs A, B, C and D one AND gate with inputs E and F is.

5.	Like the latch the flip-flop belongs to a category of logic circuit known as.

a.		Astable Multivibrator				c.		Bistable Multivibrator

b.		Monostable Multivibrator			d.		Shift Register

6.	The terminal count of a module 13 binary counter is _______.

7.	Module 10 ring counter requires a minimum of _________ Flip-Flops.

8.	With a 1 MHZ clock frequency eight bits can be entered in parallel into a shift register in _________ microseconds.

9.	A typical OLMC consists of ________, ________, and _________.

10.	The three states of a tristate output buffers are ________.

PART – B  (5 x 3 = 15 MARKS)

11.	Convert the decimal number 3.248 x 104 to a single precision floating point binary number.

12.	Develop a logic circuit with four input variables that will only produce a 1 output when exactly three input variables are 1.

13.	Design an SR Flip-Flop and draw its truth table.

14.	Write the guidelines for obtaining state assignment.

15.	Implement the following Boolean expression into PAL  X=AB+AB+AB.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Simplify the following Boolean expression using the K-Map method.

Y(A, B, C, D) = ∑(1, 3, 4, 5, 6, 7, 9, 11, 12, 13, 14, 15).							    (8)

b.	Find the complement of Y=AB+AB+AB+AB.							 	    (7)

(OR)

17.	Using the Quine - McClusky method simplify the SOP expression for the Boolean expression

Y(A, B, C, D) = ∑(1, 3, 4, 5, 7, 9, 12, 13).

18.	With a neat sketch explain the 4 to 3 priority encoder.

(OR)

19.	With a neat sketch explain about 4 bit magnitude comparator.

20.	Design and explain the operation of BCD ripple counter.

(OR)

21.	Explain in detail about sequence detector with necessary diagram.

[P.T.O.]

22.	Derive the state table and diagram for the following sequential circuit.

(OR)

23.	Design a 3 bit counter using D FF and derive its excitation table.

24.	Draw the architecture Xilinx XC4000 Configurable Logic Block (CLB).

(OR)

25.	With a neat sketch explain the functions of Emitter coupled logic.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	COMMUNICATION ENGINEERING					      Time : 3 hours

Subject Code:	09EE203							       			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The 75W unmodulated carrier has 100W of power when modulated. What is m?

2.	What is the main advantage of super heterodyne receiver over TRF?

3.	Define quantization noise.

4.	Give the merit and demerit of PWM compared with PAM.

5.	Define Probability of error.

6.	What is maximum Nyquist frequency?

7.	What is multiplexing?

8.	Mention the applications of optical fiber system.

9.	Define the term luminance.

10.	Mention three different color TV systems.

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss about low level and high level modulation systems.

12.	Explain the difference between delta modulation and pulse code modulation.

13.	For the coherent single sideband BFO receiver, an RF reduced carrier frequency of 30 MHz with an upper side band that extends from just above 30 MHz to 30.005 MHz, an RF local oscillator frequency of 20 MHz, an IF center frequency of 10 MHZ and a BFO output frequency of 10MHz. Determine:

a.	Demodulated first IF frequency band.

b.	Demodulated information frequency band.

14.	Describe the principle of slow speed TDM with basic circuit.

15.	Explain the principle of interlaced scanning.

PART – C  (5 x 15 = 75 MARKS)

16.	With the help of block diagram, explain various stages of three different types of FM transmitter. Discuss about the purpose of pre-emphasis and de-emphasis process.

(OR)

17.	a.	Explain the working of superheterodyne  receiver for AM broadcast reception with RF 	stages.						                                              		 				  (10)

b.	Mention briefly the salient features of satellite communication system.   	                   (5)

18.	With the help of block diagram and simple mathematical expression, explain PAM, PPM 	and PWM.

(OR)

19.	a.	Explain the principle of PCM with the help of block diagram. Explain how companding 	improves S/N ratio.                                                                                                           (10)

b.	Discuss the merits and demerits of PCM and DPCM.                                                        (5)

20.	Explain the principle behind the frequency shift keying technique. Explain the function of FSK modulator and demodulator with block diagram.

[P.T.O.]

(OR)

21.	Explain the operation of BPSK with block diagram of transmitter and receiver in detail.

22.	Explain the frequency division multiplexing link with block diagram and also explain the    	formation of different standard groups.

(OR)

23.	a.	Explain the effect of noise in FM systems and analyze the performance with mathematical 	approach.                                                                                                                           (10)

b.	The signal to noise of an AM system is 25 dbs. The highest audio frequency transmitted is 3 KHz. The transmitted carrier power is reduced by one tenth and FM with a deviation of +15 KHz is employed. Calculate signal-to-noise ratio.	                                               (5)

24.	a.	Explain the constructional details and working principles of image orthicon and vidicon 	camera tubes. 																  (10)

b.	Explain about the frequency spectrum and principle of color TV system. 			    (5)

(OR)

25.	a.	Draw the block diagram of a typical monochrome TV receiver and explain its salient 	features. 																	  (10)

b.	Describe the working of PAL color TV transmitter with the help of block diagram.       (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	NEURAL NETWORKS AND FUZZY SYSTEMS           	      Time : 3 hours

Subject Code:	09EE206										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are Dendrites?

2.	Which is the fundamental building block of the biological neural network?

3.	Define Back Propagation Network (BPN).

4.	What are the different types of training?

5.	What is the main purpose of Hopfield network?

6.	Give two examples of feedback networks.

7.	Give the expression for cardinality of power set.

8.	Differentiate fuzzification and defuzzification.

9.	Define core of a membership function.

10.	What are the assumptions to be made in a fuzzy control system design?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the Information flow in nervous system.

12.	Write a short note on Boltzmann training.

13.	Give the relation between BAM and Hopfield Network.

14.	List the defuzzification methods.

15.	What are the basic elements of a fuzzy logic control system?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Give the comparisons between the Brain and the Computer.						    (7)

b.	Give the comparisons between ANN and BNN.									    (8)

(OR)

17. 	Explain in detail the ANN terminologies.

18. 	Draw the Architecture of BPN and derive the generalized Back Propagation Rule.

(OR)

19. 	Explain the FOUR stages of training algorithm of Back Propagation Network.

20. 	Explain with Architecture, Application Algorithm of Discrete Hopfield Network.

(OR)

21. 	Discuss in detail the TWO types of Bidirectional Associative Memory Network.

22. 	Explain the Operations, Properties and Mapping of Classical Set.

(OR)

23. 	Explain the Cardinality, Operations and Properties of Fuzzy Relations.

24. 	Discuss the different kinds of Fuzzy Decision Making Logic.

(OR)

25. 	Explain with General structure, Need, and steps involved in Fuzzy Image Processing.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRIC CIRCUITS AND NETWORKS				      Time : 3 hours

Subject Code:	10EE201		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The number of independent loops for a network with n nodes and b branches is __________.

2. 	The Three equal resistance of 5Ω are connected in star. What is the resistance in one of the arms in an equivalent delta circuit?

3.	Reciprocity Theorem is applicable for which type of network?

4. 	When the superposition theorem is applied to any circuit, the dependent voltage source is always ___________.

5.	Transient current in an RLC circuit is oscillatory under __________ condition.

6. 	Define driving point impedance.

7. 	The current in the neutral wire of a balanced three-phase, four-wire star connected load is given by ___________.

8. 	The value of the coefficient of coupling is more for air cored coupled circuits compared to the iron core coupled circuits. (True/False).

9.  	A two-port network is simply a network inside a black box, and the network has only 	__________.

10. 	The values of L and C for a low pass filter with cut-off frequency of 2.5 kHz to operate with a terminated load resistance of 450 ohms are given by ____________.

PART – B  (5 x 3 = 15 MARKS)

11.	Convert the given star into an equivalent delta connection

12.	State the superposition theorem.

13. 	A series RL circuit with R = 30Ω and L = 15 H has a constant voltage V = 60 V applied at

t = 0. Determine the current I, the voltage across the resistor and the voltage across the inductor.

14. For three coupled coils calculate the total inductance.

15.	The port currents of a two-port network are given by

I1 = 2.5 V1 – V2   and    I2 =  V1 + 5 V2

Find the equivalent π network.

[P.T.O.]

PART – C  (5 x 15 = 75 MARKS)

16. 	Find the current through each resistor of the circuit as shown in the figure using nodal analysis.

(OR)

17.	Determine the current through 10 Ω resistance in the network shown in the figure below by star-delta conversion.

18. 	In the circuit shown, calculate current through 1 Ω resistance connected between A – B, using Thevenin’s Theorem.

(OR)

19. 	With the help of suitable circuit, prove the Norton’s Theorem.

20. 	Obtain the transient response of a RL series circuit with DC input and explain the significance of time constant.

(OR)

21.  A parallel RLC circuit, with R = 1000 Ω, C = 0.16 µF and L=1.0H, has an initial voltage

Vo = 50.0 V on the capacitor. Obtain the voltage V (t) when the switch is closed at t = 0.

22. 	For the frequency domain circuit shown in the figure determine the value of V out (t) for

V in (t) = 10 cos (377t) and a coupling coefficient k = 0.8.

(OR)

23. 	Discuss the power measurement in three phase circuits by the Two Wattmeter method.

24.  The impedance parameters of a two port network are Z11 = 6 Ω, Z22 = 4 Ω, Z12 = Z21 = 3 Ω. 	Compute the Y parameters and ABCD parameters and write the describing equations.

(OR)

25. 	In a constant-K pass filter the value of L = 95.54 mH and C = 0.265 µF. Estimate the values of Ro and fc. Also determine the attenuation in dB and nepers offered by the filter at a frequency of 3337Hz.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DIGITAL SIGNAL PROCESSING						      Time : 3 hours

Subject Code:	10EE202									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Find the signal xn=sinn9−πis periodic or not?

2.	What is aliasing?

3.	The total numbers of complex additions in FFT are ___________.

4.	Draw a flow graph of DIT radix-2 FFT.

5.	In ___________ transformation, any strip of width 2πTin S-plane is mapped into the entire Z-plane.

6.	The distortion in frequency axis due to non linear relationship between analog and digital frequency is called ___________.

7.	The Oscillations developed due to truncation of impulse response is called ___________.

8.	The ___________ window spectrum has the highest attenuation for side –lobes.

9.	___________ concept improving the speed of the DSP Processor.

10.	Modern CPU design includes ___________ instructions in order to improve the performance of multimedia use.

PART – B  (5 x 3 = 15 MARKS)

11.	Determine the response of the LTI system whose input x(n) and impulse response h(n) are given by x(n) = {1, 2, 0.5, 1} and h(n) = { 1, 2, 1, –1}.

12. 	Prove that n=0N−1x1nx2n=1Nk=0N−1X1kX2k by Parseval’s relation.

13.	For the analog transfer function, Hs=2S2+3S+2. Determine H(Z) using bilinear transformation if T= 1 second.

14.  	Compare the Hanning and Hamming window.

15. 	Difference between Van- Neumann and Harvard architecture.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain ADC Process with a block diagram.

(OR)

17.	An interconnection of LTI systems is shown in the figure. The impulse responses are h1(n)=12nun−un−3 ,   h2n=δnand h3n=un−1. Let the impulse response of the overall system from x(n) to y(n) be denoted as h(n).

a.	Express h(n) in terms of h1(n), h2(n) and h3(n).

b.	Evaluate h(n).

[P.T.O.]

18. 	Compute IDFT of the sequence,

X (K) =    {7, – 0.707 – j 0.707, – j, 0.707, – j 0.707, 1, 0.707 + j 0.707, j, – 0.707 + j 0.707}

using DIF algorithm.

(OR)

19. 	Evaluate the 8 – point DFT of the sequence x (n) =   {0.5, 0.5, 0.5, 0.5, 0, 0, 0, 0} using the 	radix-2 DIT algorithm.

20. 	Design a Chebyshev filter for the following specification using Bilinear transformation

0.8 	≤  	|H (ejɷ) | ≤ 1   ;        0  ≤ ɷ ≤ 0.2 π

|H (ejɷ) | ≤ 0.2 ;        0.6 π ≤ ɷ ≤ π

(OR)

21. 	Design a Butterworth filter using the impulse variance method for the following specifications.

0.8 ≤ |H (ejɷ) | ≤ 1   ;         0  ≤ ɷ ≤ 0.2 π

|H(ejɷ) | ≤ 0.2 ;        0.6 π ≤ ɷ ≤ π

22. 	Design a bandpass filter to pass frequencies in the range 1 to 2 rad/sec using Hanning window, with N = 5.

(OR)

23. 	Design an FIR lowpass filter satisfying the following specifications.

αp ≤ 0.1 dB ; 		αs  ≥ 44.0 dB

ɷP = 20 rad/sec    ɷs  = 30 rad/sec       	ɷSf = 100 rad/sec.

24. 	Explain VLIW architecture in detail.

(OR)

25. 	Discuss the factors that influence the selection of a DSP processor.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	RENEWABLE ENERGY SOURCES						      Time : 3 hours

Subject Code:	10EE203				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Solar radiation has not been absorbed or scattered and reaches the ground directly from the sun is ____________.

2. 	What is Photovoltaic effect?

3. 	Define power coefficient in wind mill.

4. 	What is meant by yaw control?

5. 	Mention the classification of biogas plants.

6.  	What is fermentation?

7. 	Give few utilization of hydrogen gas.

8.  	How many joules are in 1 kWh?

9. 	What is a spring tide?

10. 	Mention the name of the lowest level of tidal water.

PART – B  (5 x 3 = 15 MARKS)

11. 	Mention the different standards used to measure sun’s radiation.

12. 	What is meant by cut in speed in a wind mill?

13. 	What are the factors affecting biomass generation?

14. 	List the various methods of production of hydrogen for use as energy carrier.

15.  What are the prospects of ocean thermal energy conservation (OTEC) in India?

PART – C  (5 x 15 = 75 MARKS)

16.  	Discuss the energy resources available in world and the environmental aspects to be considered on utilization.

(OR)

17. 	Explain with a neat sketch the operation of solar PV system and discuss their applications.

18. 	Explain the wind energy conversion system with a suitable sketch.

(OR)

19. 	Briefly explain the factors influencing wind and site selection in wind energy system.

20.  a.	Discuss the factors affecting biodigestion in a biogas plant. 						  (10)

b.	Explain the process of photosynthesis. 											   (5)

(OR)

21. 	With a neat sketch explain the biogas power generation system.

22. 	Explain any one method to produce hydrogen gas with a neat sketch.

(OR)

23. 	Discuss with a neat diagram the working of fuel cell.

24. 	With a neat sketch, explain the working of open cycle ocean thermal energy conversion system.

(OR)

25.	Draw the basic elements of geo thermal energy plant and explain its working.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRONIC DEVICES								      Time : 3 hours

Subject Code:	11EE203				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	If lower the Q-point of operation what will be dynamic resistance high or low?

2.	Give the other three names of transition capacitance.

3.	What is early effect?

4.	Write the method of construction of dc load line.

5.	Name the four regions in JFET operation.

6.	Draw the structure of P-channel junction field effect transistor.

7.	How the complementary MOS device will be formed?

8.	Draw the symbol of Enhancement MOSFET.

9.	Define photolithography.

10.	What is oxidation?

PART – B  (5 x 3 = 15 MARKS)

11. 	Compare forward recovery time with reverse recovery time.

12.	Explain the difference between the structure of an NPN and a PNP transistor.

13.	Write about the operation of JFET.

14.	Compare PMOS with NMOS device.

15.	List the basic planar processor used to fabricate IC using silicon  planar technology.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain large signal and small signal diode model.

(OR)

17.	Describe in detail clippers and clampers with diagrams.

18.	With equations explain Ebers Moll model.

(OR)

19.	Summarize common emitter and common collector mode of transistor operation with relevant diagrams.

20.	Outline the transfer characteristics of JFET in detail with diagrams.

(OR)

21.	With diagrams explain the self bias configuration of JFET.

22.	Summarize the construction, basic operation and characteristics of depletion type MOSFET.

(OR)

23.	a.	Explain in detail the different ways of biasing the MOSFET in detail.  				  (12)

b.	Write about CMOS device.                                                                     				    (3)

24.	a.	Write short notes on ion implantation.                   			                                        (8)

b.	Write short notes on Metallization.				                                                              (7)

(OR)

25.	a.	Interpret monolithic diode in detail.                                                           			  (10)

b.	Explain epitaxial growth in IC fabrication.                                                			    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTROMAGNETIC FIELDS							      Time : 3 hours

Subject Code:	11EE206									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The differential volume in cylindrical co-ordinate system is __________.

2.	The divergence of the curl of any vector field is __________.

3.	The relation between potential V and electric field is __________.

4.	The capacitance of a parallel plate capacitor having stored energy of 5µJ with a voltage

between the plates of 4V is __________.

5.   	State Ampere’s Circuital law.

6. 	The boundary condition between perfect dielectrics is that the tangential component of electric flux  density is __________ across the boundary.

7.   	Maxwell’s equation from Gauss law for good conductor is given by __________.

8.	Jc / JD  =  __________.

9.	Define Skin Depth.

10. 	Write down the expressions for Instantaneous, complex and average Poynting vectors.

PART – B  (5 x 3 = 15 MARKS)

11. 	Convert the point P (2, 4, –2) from Cartesian to Cylindrical and Spherical co-ordinate system.

12. 	State and derive Poisson’s and Laplace equations.

13. 	Define Inductance. Calculate the inductance of a solenoid of 300 turns wound tightly on a cylindrical tube of 5 cm diameter. The length of the tube is 50 cm and the solenoid is in air.

14. 	For a lossy dielectric  = 5 s/m and εr =1. The electric field intensity E = 100 sin1010 t. Find JC and JD.

15. 	Obtain the attenuation and phase shift constant for a wave in lossy dielectrics.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Transform the following vector to spherical coordinates at the point specified.

A= 4 ax – 2 ay – 4 az  at P(–2, –3, 4)											    (8)

b.	Determine the divergence of the vector									   		    (7)

P = sin a + 2z a +z cos az

(OR)

17.	a.	Determine the curl of the vector field A = z sina + 3z2 cosa at (5, /2, 1).		    (8)

b.	State and prove Stoke’s theorem.												    (7)

18. 	a.	State Coulomb’s law. Given Q1 = 2 x 109 C, Q2 = – 0.5 x 109 C, find the force on Q2 if

R12 = 4 x 102 m.																    (7)

b.	Derive the expression for Electric field due to charges distributed uniformly on an infinite 	line.  																		    (8)

(OR)

19.	a.	From Gauss law find the Dr and Er  for infinite sheet of charge having charge density s 	lying in XY plane.															    (7)

[P.T.O.]

b.	Derive the boundary conditions of the normal and tangential components of electric field at                   the interface of two media with different dielectrics.								    (8)

20. 	State Biot –Savart’s Law. Using Biot-savart’s law, find the magnetic field intensity H and 	magnetic flux density B due to infinite wire carrying a steady current I.

(OR)

21.	a.	Derive the magnetic field intensity H due to co-axial cable using Ampere’s circuital law. 																					    (8)

b.	Obtain the expression for inductance of toroid.						   			    (7)

22. 	From fundamental laws, derive Maxwell’s equations in integral and point form for a conducting and non-conducting media.

(OR)

23. 	a.	Discuss about transformer and motional emfs.									    (7)

b.	What is displacement current? Differentiate conduction current and displacement current?

Also derive the expression for Displacement current density.						    (8)

24. 	Using Maxwell’s equations, derive the electromagnetic wave equations for a conducting medium.

(OR)

25.  Define Poynting vector and prove that the electromagnetic power flow is the product of electric

and magnetic field intensities.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DC MACHINES AND TRANSFORMERS				      Time : 3 hours

Subject Code:	11EE207									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Lap winding is suitable for __________ current  D.C Generator.

2.	The critical resistance of the D.C generator is the resistance of __________.

3.	The most efficient method of increasing the speed of a D.C. shunt motor would be the __________ method.

4.	Ward –Leonard system of speed control is not recommended for constant speed operation

(True/False).

5.	Write down the EMF equation of the transformer at the primary side.

6.	What is the purpose of the magnetizing current in the transformer?

7.	The main purpose of performing opening circuit test on a transformer is to measure __________.

8.	Transformer cores are laminated in order to reduce __________ loss.

9.	The All day efficiency of the transformer depends primarily on amount and duration of load. (True/False)

10.	Which type of transformer connection is best suitable for 3-phase, 4 wire service?

PART – B  (5 x 3 = 15 MARKS)

11.	What is armature reaction in D.C. Generator?

12.	Give the voltage and torque equation of a D.C. Motor

13.	What are the losses occurring in a transformer?

14.	In a 25-KVA, 2000/200 V, single phase transformer, the iron and full load copper losses are 350 Wand 400 W. Calculate the efficiency at unity power factor at full load.

15.	Write short notes on Scott Connection in three phase transformer.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the neat diagram of D.C generator and illustrate its constructional details and the important parts.

(OR)

17.	a.	Draw the no load characteristics of various types of DC generators.	                   	    	    (8)

b.	Derive the EMF equation of D.C. Generator.             			                 			    (7)

18.	a.	Sketch the three point starter circuit and brief about its operation.	                   	           (8)

b.	Derive the Torque Equation of a D.C Motor.				                  			    	    (7)

(OR)

19.	A D.C shunt machine while running as a generator develops a voltage of 250 V at 1000 rpm on no load. It has armature resistance of 0.5 ohm and field resistance of 250 ohm. When the machine runs a motor, input to it at no load is 4 A at 250 V. Calculate the speed and efficiency of the machine when it runs as a motor taking 40 A at 250V. Armature reaction weakens the field by 4%.

[P.T.O.]

20. 	a.	Draw the equivalent circuit of a transformer and derive its parameters.                 		    (8)

b.	Draw and explain the no load phasor diagram of a single phase transformer   	    	    (7)

(OR)

21.	a.	Derive the EMF equation of single phase transformer. 			         	        	    (8)

b.	A 230/460 V transformer has a primary resistance of 0.2 ohm and reactance of 0.5 ohm and the corresponding values for the secondary are 0.75 ohm and 1.8 ohm respectively. Find the secondary terminal voltage when supplying 10 A  @ 0.8 p.f lagging.		    (7)

22. 	A 20 KVA, 2200/220 V, 50 Hz distribution transformer is tested for efficiency and  regulation as follows.

O.C test : 220 V 	4.2 A		148 W

S.C test  : 86 V 	10.5 A		360 W

Determine:

a.	Core loss 		b.	Equivalent resistance referred to primary and secondary

c.	Equivalent reactance referred to primary and secondary

d.	Regulation of the transformer at 0.8 p.f lagging current

e.	Efficiency at half load and full load at 0.8 p.f lagging current.

(OR)

23.	a.	Brief about Sumpner’s test on transformer.

b.	Write short notes on all day efficiency.

24.  Explain about the construction details of three phase transformer and describe about its various 	connection.

(OR)

25.	a. 	Illustrate about current transformer and potential transformer.

b. 	Mention the condition for parallel operation of three phase transformer.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INDUCTION AND SYNCHRONOUS MACHINES	            Time : 3 hours

Subject Code:	11EE209				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the use of circle diagram in an induction motor?

2.	Define the term slip of the induction motor.

3.	List the applications of shaded pole induction motor.

4.	Why a 1Φ induction motor is not self starting?

5.	Name the various types of alternators.

6.	Define voltage regulation.

7.	Why a synchronous motor is not self starting?

8.	Name the methods of starting synchronous motor.

9.	What is meant by slip test?

10.	What is meant by two reaction theory?

PART – B  (5 x 3 = 15 MARKS)

11.	What is the need for the equivalent circuit of a three phase induction motor?

12.	What is the function of centrifugal switch in a single phase induction motor?

13.	What is Potier reactance? How it is determined by Potier triangle?

14.	What are V and inverted V curves? How are they determined?

15.	Give the power angle characteristics of synchronous generator.

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain the torque equation of a three phase induction motor under starting and running condition.

(OR)

17.	With a neat diagram, explain the working of an auto transformer starter used for starting a three phase induction motor.

18.	Explain the working principle of single phase induction motor with the help of double revolving field theory.

(OR)

19.	Explain the construction and working principle of split phase and capacitor start induction motor with its torque-speed characteristics.

20.	The open circuit and short circuit test results for a 3Φ, star connected, 1000 KVA, 1905 V,

50 HZ alternator are:

| Open circuit terminal voltage (Voc)line V | 760 | 1500 | 1700 | 1905 | 2300 | 2600 |
| --- | --- | --- | --- | --- | --- | --- |
| Short circuit current Isc A | - | 220 | - | 335 | - | - |
| Field current If A | 10 | 20 | 25 | 30 | 40 | 50 |

The armature reactance per phase is 0.2Ω. Draw the open circuit and short circuit characteristics and find the regulation on full load 0.8 lagging p.f by ampere- turn method.

(OR)

[P.T.O.]

21.	Elucidate the terms synchronizing current, synchronizing power and synchronizing torque of synchronous machine.

22.	Explain the principle of operation of synchronous motor.

(OR)

23.	Describe in detail about hunting and methods of suppression in synchronous motor.

24.	Explain the Blondel’s two reaction theory of salient pole machine.

(OR)

25.	Explain the determination of direct(d) and quadrature(q) axis synchronous reactance using slip test.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title : 	ELECTRICAL MACHINE DESIGN						      Time : 3 hours

Subject Code:	11EE211										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A high value of flux density results in increased ___________.

2.	The operating life of a machine depends upon the type of ___________.

3.	What do you mean by leakage flux?

4.	Mention the two types of magnetic materials.

5.	Class ‘C’ type insulation material can have temperature limit of above ___________.

6.	Explain full pitch and short pitched conductors.

7.	List some advantages of having large number of poles.

8.	EMF generated in a conductor is given by ___________.

9.	What do you mean by tooth ripples?

10.	Define skin effect.

PART – B  (5 x 3 = 15 MARKS)

11.	Mention some advantages of utilization of hydrogen for the cooling of electrical machines.

12.	Define eddy current loss.

13.	State Faraday’s law of magnetic induction.

14.	List some advantages of high value of q in synchronous machines.

15.	Define stacking factor.

PART – C  (5 x 15 = 75 MARKS)

16.	A 350 Kw, 500 V, 450 rpm, 6 pole d.c generator is built with an armature diameter of 0.87 m and a core length of 0.32 m. The lap wound armature has 660 conductors. Calculate the specific electric and magnetic loadings.

(OR)

17.	A 5Kw, 250V, 4 pole, 1500 rpm shunt generator is designed to have a square pole face. The loading are average flux density in the gap = 0.42 wb/m2 and ampere conductors per meter = 15000. Find the main dimensions of the machine. Assume full load efficiency = 0.87 and ratio of pole arc to pole pitch = 0.66.

18.	Derive the expression for the output equation of a d.c machine which is to be related to its main dimensions.

(OR)

19.	Find the suitable number of poles and the diameter of the core of a 400Kw, 550V, 180 rpm, d.c generator having 92% efficiency. Assume an average flux density in the air gap of about 0.6 wb/m2 and ampere conductor per meter to be 35000.

[P.T.O.]

20.	Determine the dimension of the core and yoke for a 100 KVA, 50 Hz, single phase, core type transformer. A square core is used with distance between the adjacent limbs equal to 1.6 times the width of the laminations. Assume voltage per turn of 14 volts, maximum flux density 1.1 wb/m2 , window space factor 0.32, and the current density 3A/mm2 . Take stacking factor = 0.9, flux density in the yoke to be 80% of flux density in core.

(OR)

21.	Derive the expression for the eddy current loss per unit volume of a transformer.

22.	In the design of a 30 H.P, 3ø, 440V, 960 rpm, 50Hz delta connected induction motor assume the specific electric loading of 25000 ac/m, specific magnetic loading of 0.46 wb/m2 , full load efficiency of 86%, p.f = 0.87. Estimate the stator core dimension.

(OR)

23.	Calculate diameter, length, number of turns per phase, full load current, and cross section of conductors and total I2R loss of stator of 3 phase, 120 Kw, 2200 V, 50 Hz, 750 rpm star connected slip ring induction motor from the following particulars

Bav = 0.48 T, ac = 26000 Ac/m, ɳ = 92%, cos ø = 0.88, L= 1.25 Ʈ, Kw = 0.955, current density 	= 5A/mm2, mean length of stator conductor 75 cm, ƿ =0.021 Ώ/m.

24.	Explain the effect of short circuit ratio and its effect on the machine performance.

(OR)

25.	Find the main dimension of a 100 MVA, 11 KV, 50Hz, 40 pole, salient pole generator assuming air gap flux density as 0.65 wb/m2 and ampere conductor as 40,000 per meter. The peripheral speed should not exceed 60m/sec.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	POWER ELECTRONICS								      Time : 3 hours

Subject Code:	11EE212									              Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Draw the volt-ampere characteristics of a SCR.

2.	List the applications of IGBT.

3.	What are the two types of control normally used in AC voltage controller?

4.	Express the average output voltage of 3- pulse converter with RL load.

5.	List the limitations of variable frequency control.

6.	What is meant by time ratio (duty cycle) of a DC chopper?

7. 	How the output frequency is varied in the case of an inverter?

8.	What are the limitations of series inverter?

9. 	What is the need for tap changing in transformer?

10. 	List different types of control.

PART – B  (5 x 3 = 15 MARKS)

11. 	Compare the transistor and thyristor.

12. 	Mention functions of freewheeling diode.

13. 	What is the principle of step - up DC chopper?

14. 	What are the difference between VSI and CSI?

15.  	Draw the circuit diagram of single phase half wave AC voltage regulator.

PART – C  (5 x 15 = 75 MARKS)

16. 	With two transistor model explain the operation of TRIAC.

(OR)

17. 	Explain the switching characteristics of MOSFET.

18.	Explain the working with neat waveforms of 1Ф full converter having RLE load with discontinuous load current. Derive the expressions for Average voltages and RMS Voltage.

(OR)

19. 	Explain the operation of 3-phase semi converter with R load.

20. 	Explain the working of single phase full wave controller.

(OR)

21.	Discuss the time ratio control of Chopper.

22. 	Explain the operation of 3Ф VSI at 120 mode and write the expression for output phase and line voltage.

(OR)

23. 	Explain the working of basic series resonant inverter. 	Write its limitation.

24.  Draw the circuit diagram of single phase A.C voltage controller fed induction drive. Explain 	with neat wave forms.

(OR)

25.  	Describe the working of different types of UPS.

eg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	GENERATION, TRANSMISSION AND DISTRIBUTION	     Time : 3 hours

Subject Code:	11EE215		 								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The most simple and clean power plant is __________ plant.

2.	The connected load is generally __________ than the maximum demand.

3.	Define resistance of the transmission line.

4.	The choice of proper transmission voltage is essentially a question of __________.

5.	The most commonly used material for insulators of overhead lines is__________.

6.	If sag in an overhead line increases, tension in the line __________.

7.	Voltage drop in cable system is less than that of equivalent overhead line because of __________ of conductors in a cable.

8.	If the length of a cable increases, its insulation resistance  __________.

9.	The dc interconnector is used __________ the voltage drops in the various sections of the distributor.

10.	In a singly fed distributor, if fault occurs on any section, the supply to all consumers has to be 	_________.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the information conveyed by a load curve?

12.	What is Transposition? Write the advantage.

13.	Write the disadvantages of Corona?

14.	Why are single core cables not provided with armouring in AC transmission systems?

15. 	Compare Feeder with Distributor.

PART – C  (5 x 15 = 75 MARKS)

16. 	Compare the Hydro, Diesel and Nuclear power plants based on the following:

a.	site 		b.	initial cost		c.	running cost 		d.	source of power

e.	transportation 		f.	cleanliness.

(OR)

17.	Draw and explain the basic structure of the power system with relevant voltage levels.

18.	Calculate the capacitance of a 100 km long 3-phase, 50 Hz overhead  transmission line  consisting of 3 conductors, each of diameter 2 cm and spaced 2.5 m at the corners of an equilateral triangle.

(OR)

19.	Derive the equation to get Inductance of symmetrical 3 phase over head transmission line.

20.	List the methods to improve the string efficiency and explain.

(OR)

21.	A 3-phase, 220 kV, 50 Hz transmission line consists of 1.5 cm radius conductor spaced 2 metres apart in equilateral triangular formation. If the temperature is 40ºC and atmospheric pressure is 76 cm, calculate the corona loss per km of the line. Take mo = 0.85.

[P.T.O]

22.  	List the requirements of a good cable and draw the constructional diagram of underground cable and explain the various parts.

(OR)

23.	A single core cable for use on 11 kV, 50 Hz system has conductor area of 0·645 cm2 and internal diameter of sheath is 2.18 cm. The permittivity of the dielectric used in the cable is 3.5. Find:

a.	the maximum electrostatic stress in the cable

b.	minimum electrostatic stress in the cable

c.	capacitance of the cable per km length

d.	charging current.

24.	A single phase a.c. distributor AB 300 metres long is fed from end A and is loaded as under

a.	100 A at 0·707 p.f. lagging 200 m from point A

b.	200 A at 0·8 p.f. lagging 300 m from point A

The load resistance and reactance of the distributor is 0.2 Ω and 0.1 Ω per kilometre. Calculate the total voltage drop in the distributor. The load power factors refer to the voltage at the far end.

(OR)

25. 	What are the requirements of distribution system? Write the design considerations of the same.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	POWER SYSTEM ANALYSIS							      Time : 3 hours

Subject Code:	11EE216							       			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the expression to find the per unit value of any electrical quantity.

2.	List the symmetrical components.

3.	What is a primitive network?

4.	Number of branch in a tree is __________ than number of nodes.

5.	What is the need of load flow study?

6.	List the different types of bus.

7.	Write down the Power balance equations.

8.	Write the transmission loss equation of 3 unit system and also write the same in matrix form.

9.	How can we classify stability studies?

10.	Swing equation can be solved by__________ and __________ methods.

PART – B  (5 x 3 = 15 MARKS)

11.	A generator rated at 30 MVA, 11 kV has a reactance of 20%. Calculate its p.u. reactances for a base of 50 MVA and 10kV.

12.	List the various types of shunt and series faults.

13.	Compare the G-S and N-R methods of load flow solutions.

14.	What are the different types of constraints used in economic dispatch problem?

15.	List the methods of improving the transient limit of a power system.

PART – C  (5 x 15 = 75 MARKS)

16.	A 100 MVA, 33 KV, three phase generator has a reactance of 20 %. The generator is connected to motors through transmission lines and transformers. The motor has rated inputs of 30 MVA, 20 MVA and 50 MVA at 30 KV with 15 % sub transient reactance. The three phase transformers are rated at 110 MVA, 33KV Δ/110 KV Y with leakage reactance of 8 %. The line has a reactance of 45. Selecting the generator rating as base quantities in generator circuit, determine base quantities in other parts of system and evaluate the p.u. values.

(OR)

17.	Explain in detail the need for system analysis in planning and operation of power system.

18.	Obtain the ZBus for the 3-bus sample system shown in the below figure.

[P.T.O.]

(OR)

19.	a.	Two Generators G1 and G2 are rated 15MVA, 11KV  and 10MVA, 11KV respectively. 	The generators are connected through a transformer as shown in the figure. Calculate sub 	transient current in generator when a 3-phase fault occurs on HT side of the transformer.		  																			(10)

b.	Derive the expression for fault current in single Line to ground fault on an unloaded generator in terms of symmetrical components.									    (5)

20.	The system data for a load flow problem are given in table 1 and table 2.

a.	Compute Y bus

b.	Determine bus voltage at the end of 1st iteration by Gauss-Seidel method. Take acceleration factor as 1.

Table-1 Line data									Table-2 Bus data

| Bus Code | P | Q | V, p.u | Remarks |
| --- | --- | --- | --- | --- |
| 1 | - | - | ∟0 | Slack |
| 2 | 0.5 | 0.2 | - | PQ |
| 3 | 0.4 | 0.3 | - | PQ |

| Buscode of lines | Admittance(p.u) |
| --- | --- |
| 1-2 | 2-j8 |
| 1-3 | 1-j4 |
| 2-3 | 0.6-j2.6 |

(OR)

21.	With a flow chart and necessary equations, explain the NR Iterative method for solving load flow problem.

22.	a.	The Fuel cost equation for Two thermal plants in Rs/hr are given by

F1= 0.1P12 +25P1 +1.6 and

F2= 0.1P22 + 32P2 +2.1 where P1 and P2 are in MW. The total load is 250 MW. 	                Neglecting line losses and generating limits. Find the optimal dispatch and the fuel cost in Rs/hr. 																		    (8)

b.	Give the elementary idea of optimal load scheduling of Hydro - Thermal plants. 		    (7)

(OR)

23.	a.	Derive and draw the flowchart for coordination equation without losses and explain.	    (8)

b.	Explain the various constraints in unit commitment problem with suitable examples.	    (7)

24.	With the help of a neat flowchart, explain the Runge-Kutta Method of solving the swing equations.

(OR)

25.	Derive swing equation for a single machine connected to infinite bus system.  State the assumptions if any and state the usefulness of this equation.  Damping is not to be neglected.  State the reasons for non-linearity of this equation.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	POWER SYSTEM PROTECTION AND SWITCHGEARS

Time : 3 hours

Subject Code:	11EE218										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Current limiting reactors are used for __________.

2. 	List any two merits of static relay.

3. 	Mention two methods of preventing the operation of the differential relay due to harmonic inrush currents.

4. 	List two applications of Buchholz  relay.

5. 	Name the two arc extinction theories.

6. 	Define RRRV.

7. 	What are the different causes for overvoltage in transmission system?

8. 	What is arcing ground?

9. 	Write the necessity of earthing.

10. 	What are the types of grounding?

PART – B  (5 x 3 = 15 MARKS)

11. 	What is a protective relay? Mention the fundamental requirements of protective relays.

12. 	Draw the schematic diagram to show the differential protection provided for a bus section.

13. 	List any two advantages and disadvantages of oil circuit breaker.

14. 	List out the harmful effects of Lightning.

15. 	Write few important Indian Electricity rules for protection

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Explain fault current calculation using symmetrical components. 					    (7)

b. 	What is CT and PT and state its application in protection scheme.					    (8)

(OR)

17. 	a. 	Explain the nature and causes of faults.											  (10)

b. 	Discuss per unit representation.            											    (5)

18.	With the help of a neat diagram explain about the construction and working of Induction type directional power relay.

(OR)

19.  Describe with the help of neat sketches the graded time lag protection of a radial feeder.      		What are the disadvantages of such a protection in case of:

a. Parallel Feeder				       	b. 	Interconnected lines.

20.  Briefly explain about the following  phenomenon:

a. Current chopping   				b. Capacitive  current breaking.

(OR)

21. Explain briefly the following types of air blast circuit breakers:

a. 	Axial blast type   		b. 	Cross  blast type

[P.T.O.]

22. 	Discuss about the lightning phenomenon with a suitable diagram.

(OR)

23.  Explain the operation of any three types of lightning arresters with neat sketches.

24. 	Describe the concepts of resistance and reactance type earthing.

(OR)

25. 	What is insulation level?  Discuss in detail insulation levels of various substation equipments.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CONTROL SYSTEMS									      Time : 3 hours

Subject Code:	11EE219		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define transfer function.

2.	State Mason’s gain formula.

3.	Define damping ratio.

4.	Define steady state error.

5.	What are frequency domain specifications?

6.	Define phase cross over frequency.

7.	What is characteristic equation?

8.	What is centroid?

9.	What is the state model of nth order system?

10.	What are phase variables?

PART – B  (5 x 3 = 15 MARKS)

11.	Compare open loop and closed loop system.

12.	How the system is classified based on depending ratio?

13.	Write any three advantages of frequency response analysis.

14.	What is Nyquist stability criterion?

15.	List the advantages of modern control systems.

PART – C  (5 x 15 = 75 MARKS)

16. 	Write the differential equations governing the mechanical system shown in the figure and obtain its transfer function.

(OR)

17. 	Determine the overall transfer function C(s) / R(s) for the system shown below.

[P.T.O.]

18. 	The open loop transfer function of a unity feedback system is given by Gs=Kss+10. Determine the gain K, so that the system will have a damping ratio of 0.5. For this value of K, determine settling time, peak overshoot and time at peak overshoot for a unit step input.

(OR)

19. 	The open loop transfer function of a unity feedback servo system is  Gs=10s0.1s+1.  Evaluate the static error constants of the system. Obtain the steady state error of the system, when subjected to an input given by the polynomial, rt=a+at+a22t2.

20.  	Sketch the Bode plot for the following transfer function and determine the phase and gain cross over frequencies.

Gs=10s1+0.4s1+0.1s

(OR)

21. 	The open loop transfer function of a unity feedback system is  Gs=1s1+s1+2s. Sketch the Polar plot and determine the Gain margin and Phase margin.

22.  	Use Routh criterion to the location of roots on s-plane and hence the stability for the system represented by the characteristic equation, S6 + S5 + 3S4 + 3S3 + 3S2 + 2S + 1 = 0.

(OR)

23. 	Sketch the root locus of the system, whose open loop transfer function is

Gs=Kss+2s+4

Find the value of K so that the damping ratio of the closed loop system is 0.5

24. 	The state variable model of the system is given below. Determine the controllability and observability of the system.

X=010001−6−11−6 X+101 u

y=1051X

(OR)

25. 	Obtain the state model of the system which is given by the differential equation using using phase variable form,

d3ydt3+6d2ydt2+11dydt+6y=u

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	LINEAR INTEGRATED CIRCUITS						      Time : 3 hours

Subject Code:	11EE220									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The type of feedback used if external component is connected between output terminals and inverting input is __________.

2.	List the parameter  that is important for ac application.

3.	Show with circuit diagram an op amp used as a scale changer.

4.	The frequency of oscillation of a RC phase shift oscillator is __________.

5.	What is a precision diode?

6.	Name the circuit that is used to detect the peak value of non sinusoidal waveform.

7.	What is the other name of one shot?

8.	What are the modes of operation of timer 555?

9.	Which is greater “capture range’ or ‘lock range’?

10.	How many resistors are required in a 12-bit weighted resistor DAC?

PART – B  (5 x 3 = 15 MARKS)

11. 	Define CMRR.

12. 	Design a subtractor using op-amp.

13. 	What are the applications of peak detector?

14. 	In a astable multivibrator using 555 timer RA = 6.8 kΩ, RB = 3.3 kΩ, C = 0.1 μF. Calculate the free running frequency.

15. 	Define the terms settling time and conversion time related to DACs.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain in detail the DC characteristics of an Operational amplifier

(OR)

17. 	Explain the different frequency compensation techniques of Op-Amp.

18. 	With a neat circuit diagram explain the operation of a Op-Amp differentiator and derive an expression for the output of a practical differentiator.

(OR)

19.	Explain with neat circuit diagram any one sine wave oscillator using OpAmp and derive expression for frequency of oscillation and gain of Op-Amp.

20.	a.	What is a sample and hold circuit? Briefly explain its construction and application.   	  (8)

b.	Explain a positive clipper circuit using an Op-amp and a diode with neat diagrams. 	    (7)

(OR)

21. 	Explain the operation of Monostable multivibrator using operational amplifier and derive the expression for the pulse width.

22. 	Draw the functional diagram of IC 555 Timer in astable mode and explain its operation.   Derive an expression for frequency.

(OR)

[P.T.O.]

23. 	Draw and explain the function block diagram of a 723 IC regulator and make the necessary changes to make it as low voltage regulator.

24. 	a.	Explain the working principle of successive approximation type ADC.	        		  (11)

b.	Explain the any four specifications of data converters.                          	         		   (4)

(OR)

25. 	Explain with a schematic how a PLL can be used as

a.	frequency multiplier.                                           	                                            		    (8)

b.	frequency translator.                                                                                          		    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MICROPROCESSORS AND MICROCONTROLLERS	      Time : 3 hours

Subject Code:	11EE221										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Why address bus is unidirectional?

2.	Write down the control and status signals of 8085.

3.	The 8051 has __________ bytes of on chip RAM.

4.	List any four features of 8051 microcontroller.

5.	Specify the addressing mode followed in the following instructions:

MOV A, R0 and MOV A,@R0

6.	Enlist the various flags in the PSW register.

7.	List the modes of timer in 8051.

8.	What is the significance of C/T bit in TMOD register?

9.	How many register banks are placed inside internal RAM?

10.	The rate at which serial data transmitted is called ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Describe the function of the following pins of 8085:

a.	ALE		b.	SID			c.	SOD

12.	Discuss the advantages of micro controllers over microprocessors in control applications.

13.	Discuss about the various Bit handling instructions of 8051.

14.	List the interrupts of Microcontroller according to its priority.

15.	Write a program to generate saw tooth wave at the output of DAC.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Data byte 28H is stored in memory location 4500H and 10 H is stored in register B. Show 	the content of registers C and accumulator after the execution of the following instructions.

MOV A, M

ADD B

CMA

INR A

MOV C, A

RLC			     					   		       		   						    (5)

b.	Illustrate the timing of data flow when the instruction MVI B, 53H stored in memory 			location 2000H is being fetched. 							       	  				  (10)

(OR)

17.	With a neat functional block diagram, explain the architecture of 8085	Microprocessor.

18.	a.	Discuss the organization of internal RAM of 8051.								    (8)

b.	List all the registers used in 8051 microcontroller in brief.							    (7)

(OR)

[P.T.O.]

19.	With a neat sketch explain the architecture of an 8051 microcontroller.

20.	a.	Discuss the various addressing modes of 8051 with suitable examples.			  	    (7)

b.	Write short notes on:

i.	Assembler Directives.                                                          						    (8)

ii.	Signed number concepts.

(OR)

21.	What are the different groups of instructions supported by 8051? Explain them with suitable examples.

22.	Describe the serial communication feature of 8051 with various operating modes.

(OR)

23.	Draw the Format of timer mode control register TMOD and describe four modes of operations of Timer of 8051.

24.	Interface 8051 to external ROM and RAM and explain how 8051 access them.

(OR)

25.	Draw the circuit diagram to interface a stepper motor with 8051 microcontroller and write a program to rotate the motor in the clockwise direction. Assume step angle of 30o.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	C++ AND DATA STRUCTURES							      Time : 3 hours

Subject Code:	11EE223									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	An object is said to be _____________.

2.	The items in an array are called ____________ and items in a structure are called ___________.

3.	Variables that hold address values are _________.

4.	An object declared externally can only invoke ____________.

5.	Friend functions are used to _____________.

6.	Flow of data is  __________.

7.	Nodes with the same parent are known as _____________.

8.	In a doubly Linked list the first cell's previous pointer points to the last cell is known as _____________.

9.	The Time complexity of Heap sort is _______________.

10.	When can we use linear search?

PART – B  (5 x 3 = 15 MARKS)

11.	Write about constructors in C++.

12.	Write a short note on conversions from basic to user-defined data type.

13.	Write about this Pointer.

14.	What is Queue?

15.	Give a short note on merge sort.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss in detail about Arrays in C++ with suitable examples.

(OR)

17.	Explain the standard C++ string Class.

18.	Write about overloading Binary operator with necessary coding.

(OR)

19.	Explicate pointers and functions in C++.

20.	Write in detail about Virtual functions in C++.

(OR)

21.	Discuss about Exception handling in C++.

22.	Implement a stack in C++ using arrays.

(OR)

23.	Write a C++ program to implement insertion and deletion of a node in a single linked list.

24.	Explain Quick sort algorithm with suitable example.

(OR)

25.	Write a program in C++ to implement Binary search.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MEASUREMENT AND INSTRUMENTATION			      Time : 3 hours

Subject Code:	11EE225										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the two methods for measurement.

2.	Define calibration.

3.	How is the voltage connected in the induced type Energy meter?

4.     Name the any one error caused in dynamometer type wattmeter.

5.	What is the bridge circuit?

6.	The input to an AC bridges is fed from:

(a) An amplifier	(b) Regulated power supply	(c) Oscillator 	(d) DC Battery

7.	Give one examples of piezoelectric materials.

8.	Strain gauge is used:

a.	To convert sound energy into electrical energy

b.	To sense temperature

c.	To convert electrical current into a mechanical displacement

d.	To convert mechanical displacement into a change in resistance

9.	Define rise time of a pulse.

10.	What is spectrum analyzer?

PART – B  (5 x 3 = 15 MARKS)

11. 	What is a standard? What are types of standards?

12. 	Name the errors caused in dynamometer type wattmeter.

13. 	What is Kelvin’s double bridge?

14.  Define gauge factor and its expression.

15. 	State the need of Harmonics Distortion.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the various standards used for measurement and give its classification.

b.	Discuss in detail the about the errors in an instrument.  							(7+8)

(OR)

17.	Discuss the working principle of operation of Electrodynamometer type of instruments with its 	constructional diagram. Also derive the torque equation of the electrodynamometer.

18.	Explain the techniques available for adjustments to make the energy meter measure the energy 	consumed.

(OR)

19.	Discuss the construction and operation of dynamometer type wattmeter with torque equation.

20.	Describe how measurement of resistance can be done by using Wheatstone bridge. What 	types 	of resistance are suitable for measurement using this bridge?

(OR)

21.	Explain the construction of Anderson Bridge. Derive the unknown quantities at balance 	condition.

[P.T.O.]

22. 	Explain the principle, characteristics and advantages of LVDT with a neat diagram.

(OR)

23.	Explain about the various classifications of Transducers with an example of each type in detail.

24. 	Draw and explain the working principles and the block diagram of a Digital Voltmeter of ramp type.

(OR)

25.	With a neat block diagram explain how XY recorder can be measured using signal? Give the uses of XY recorder.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	EMBEDDED SYSTEM									      Time : 3 hours

Subject Code:	11EE231									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define an embedded system.

2.	Justify the use of physical and virtual device drivers in embedded system.		
3.	List out the classification of task classes in real time systems.

4.	Bring down the factors to estimate the program run times in embedded system.

5.	What are the states of a task?

6.     List down the top-down structure layers in real time operating systems.

7.	What is a cross-assembler?

8.	List down the software tools needed at the host in development process.

9.	Bring down the hardware components required for Automatic chocolate vending machine.

10.	Draw the architecture of smart card hardware.

PART – B  (5 x 3 = 15 MARKS)

11.	Real time video processing needs sophisticated embedded system with hard real time constraints. Why? Explain.

12.	What are properties of performance measures in a real time system? Explicate any one in brief.

13.	Briefly explicate any three memory management strategies for a system.

14.	List down the laboratory tools for Testing, Simulation and Debugging of an embedded system.

15.	List down the requirements for creating the SMS and send applications.

PART – C  (5 x 15 = 75 MARKS)

16.	Define design metrics in an embedded system and explain the abstraction of steps in the design process. What are the challenges in embedded system design: Optimizing Design Metrics?

(OR)

17.	a.	Exemplify the embedded hardware units and devices in a system.	    				  (12) 	b.	Briefly, elucidate the skills required for an embedded system designer. 				   (3)

18.	With a neat block diagram, explicate the structure of a real time system and Task Classes.

(OR)

19.	Exemplify the following terms in detail:

a.	Traditional performance measures.

b.	Perform ability.

c.	Cost Function and Hard Deadlines.

20.	a. 	Elucidate the timer and event functions in real time operating systems articulately.  	    (8)

b. 	Elucidate the memory management in real time operating systems with memory allocation 	and memory management after initial allocation. 			     						    (7)

(OR)

21.	Illustrate the Shared Data problem by Multiple Tasks and Solutions.

[P.T.O]

22. 	Describe the functions of complier, linker, locator, loader, interpreter, dissembler, cross-	assembler, and integrated development system in detail.

(OR)

23. 	With the help of a detailed design development process, explain the simulator, their features, and their software.

24. 	With a neat architecture of Hardware and Software, explain the case study of a digital camera.

(OR)

25. 	Elucidate in detail how embedded system is useful in automobiles and their functions of each discrete components which later integrated to central serving system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	VIRTUAL INSTRUMENTATION						      Time : 3 hours

Subject Code:	11EE232				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is virtual instrumentation?

2.	What is embedded controller?

3.	What are the three main components of a virtual instrument?

4.	What are the three palettes used in programming?

5.	What are the major components of a PC-based data acquisition system?

6.	What is the significance of codewidth in DAQ?

7.	Give the bandwidth supported by GPIB and RS232.

8.	What is meant by *IDN?

9.	Power spectrum VI returns the ___________ of periodic signals.

10.	What are the two components of Express VI?

PART – B  (5 x 3 = 15 MARKS)

11.	What is the significance of Active X programming?

12.	How can you create, modify and debug VIs using floating tools palette?

13.	What are the three methods that DAQ devices can be grounded?

14.	What is the difference between GPIB and serial link?

15.	What is meant by toolbox? List some of the toolboxes available.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Compare graphical programming with conventional programming.       				    (8)

b.	Explain the advantages of virtual instrumentation.                                 				    (7)

(OR)

17.	With a neat block diagram, explain graphical system design model.

18.	a. 	What is case structure? Describe the steps followed to create a case structure. 		    (8)

b.	What is sequence structure? Describe the types of sequence structure.    			    (7)

(OR)

19.	Draw and explain the structure of an instrument driver. Explain the virtual instruments in instrument driver.

20.	a. 	What is a plug-in DAQ device? Draw and explain the various functions in the DAQ 	device.                                            												    (9)

b.	Explain how acquired data can be stored and logged to disk.  						    (6)

(OR)

21.	Describe the software and hardware installation for DAQ.

22.	a. 	Draw and explain the different components of a motion control system. 			    (8)

b.	Explain the importance of image processing and analysis in IMAQ.         			    (7)

(OR)

[P.T.O.]

23.	a. 	Describe about the features of VISA and explain its programming terminology.     	    (8)

b.	Explain the basics of networking for office and industrial applications.    			    (7)

24. 	a. 	List the various signal processing and analysis tools and their applications.			    (6)

b. 	Draw the block diagram and explain the three control design tools.               			    (9)

(OR)

25.	Explain the application of virtual instrumentation in instrument control and remote testing of instruments with necessary diagrams.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AUTOMOTIVE ELECTRONICS						      Time : 3 hours

Subject Code:	11EE240				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What does a sensor do?

2.	What is an ECO sensor?

3.	List the types of starter motors.

4.	State two advantages of a permanent magnet starter.

5.	What is FBC?

6.	What is the timing of an injector pulse width, in milliseconds?

7.	In a conventional incandescent bulb the filament is made from __________.

8.	What is the operating frequency of electronic flasher?

9.	What is an open loop system?

10.	Where are 4-wheel steering systems required?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain throttle position sensors.

12.	Why are starter switches needed?

13.	What is multiport fuel injection?

14.	List the head light dazzling preventive methods.

15.	What is idle speed control?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain crank angle position and oxygen sensor with relevant sketches.

(OR)

17.	Describe altitude and throttle position sensors with block diagrams.

18.	Describe the operation of the pull-in and hold-on windings in a pre-engaged starter solenoid.

(OR)

19.	With a block diagram explain starter drive units.

20.	Explain what is meant by ignition timing and why certain conditions require it to be advanced 	or retarded.

(OR)

21.		Draw a block diagram of a fuel injection system. Describe briefly the purpose of each 	component.

22.	Draw a simplified circuit of a lighting system showing the side- and headlight bulbs, light 	switch, dip switch and main beam warning light.

(OR)

23.	Define the term ‘Expert or Intelligent lighting’. Draw a typical dim-dip circuit and state the reason why it is used.

24.	Elaborate on power steering and cruise control.

(OR)

25.	Describe distributor less ignition and integrated engine control system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ENERGY SYSTEMS									      Time : 3 hours

Subject Code:	11EE242									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Which device is used to convert sunlight to electricity?

2.	What type of generators are commonly used in wind turbine?

3.	The temperature difference between OTEC from __________ and ____________.

4.	The major factor in the tide formation is moon. True or false?

5.	Which type of gas is present inside the fluorescent tube?

6.	What is the function of choke in arc furnace?

7.	The primary requirement of a___________ is that it should maintain a continuous contact with the trolley-wire at all speeds.

8.	A Steam locomotive requires____________ repair and maintenance.

9.	A Sphere gap measure is linear and valid for gap spacing less than or equal to ____________.

10.	In an impulse current generator the capacitors are connected in ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Compare HAWT and VAWT.

12.	What are the sources of tidal energy? What is the minimum tidal range required for practical tidal plant?

13.	What is meant by ‘point to point method’ of lighting calculations?

14.	What are the requirements of an ideal traction system?

15.	What are the different errors to constitute the measurements of impulse voltage?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With the help of schematic diagram explain the working of high temperature solar thermal 	electric power plant.															  (10)

b.	Describe the principles of solar photo voltaic energy conversion.					    (5)

(OR)

17.	a.	With the help of a neat block diagram explain the various components of Wind Energy 	Conversion (WEC) system.													  (10)

b.	Discuss the various environmental and safety aspects of WEC system.				    (5)

18.	Describe the ‘closed cycle’ OTEC system, with its advantage over ‘open cycle’ system.

(OR)

19.	Explain with sketches the various methods of tidal power generation. What are the limitations of each method?

20.	What do you understand by polar curves? Explain Rousseau’s construction for calculating M.S.C.P of a lamp.

(OR)

21.	Explain the method of induction heating and describe coreless type of induction furnace.

[P.T.O.]

22.	With a neat sketch explain the following:

a.	Mechanism of train movement.

b.	Speed-Time curves for a passenger train.

(OR)

23.	What is the specific energy consumption? Enumerate the factors which affect the specific energy consumption of trains operating at a given schedule speed.

24.	Describe briefly, with the aid of suitable diagrams, equations and/or examples, where appropriate, the following:

a.	Why is a Cockcraft-Walton circuit preferred for voltage multiplier circuits?

b.	Describe the operation of Cockroft Walton dc generator with suitable diagrams.

c.	Give the expression for ripple and regulation in voltage multiplier circuits.

(OR)

25.	With the aid of suitable diagrams briefly describe the use of vertical and horizontal sphere gaps in the measurement of high ac voltages.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BASIC ELECTRICAL ENGINEERING					      Time : 3 hours

Subject Code:	12EE101									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State any one of the limitations of Ohm’s law.

2.	What will be the fuse setting of a 60 W, 250 V instrument?

3.	How many maxwell are considered as one webber?

4.	Give the expression for mutual inductance between two coils.

5.	What is self inductance in DC power supply?

6.	Give the Indian standard frequency.

7.	The principle of operation of a transformer is ___________.

8.	Write the expression of the speed of the three phase power supply in a three phase coil.

9.	Mention any two types of wiring.

10.	Name the instrument used for measuring insulation resistance.

PART – B  (5 x 3 = 15 MARKS)

11.	Define voltage division technique.

12.	Explain permeance of the magnetic flux.

13.	Draw the circuit diagram of long shunt DC generator and write down KCL and KVL                                                                                                        equation.

14.	What are the drawbacks of the nuclear power generation?

15.	Draw and explain the wiring layout for stair case wiring.

PART – C  (5 x 15 = 75 MARKS)

16.  a.	Two 50 Ω resistors are connected in series. When a resistor R is connected across one of 			them, the total circuit resistance is 60 Ω. Calculate the value of R.  					    (7)

b.	Derive voltage division and current division technique. 							    (8)

(OR)

17.	a.	Find the equivalent resistance between the terminals A and B of the figure shown below.																						  (10)

1Ω

1Ω

2Ω

A

B

2Ω

1Ω

b.	Explain source transformation.					                          					    (5)

[P.T.O.]

18.	a.	Define the following terms:

i.	magnetic flux.															    (2)

ii.	magnetic flux density.														    (2)

iii.	magneto motive force	.													    (2)

iv.	magnetic field intensity.													    (2)

b.	State and illustrate Faraday’s law.												    (7)

(OR)

19.	a.	Define self inductance and mutual inductance.									    (5)

b.	Derive the coupling coefficient.												  (10)

20.	Obtain the form factor and peak factor for the waveform shown in 	the figure below.

v

Vm

π

0

(OR)

21.	a. 	With suitable diagrams, explain a hydro power generating station.					  (12)

b. 	Mention the advantages of a thermal power plant.								    (3)

22.	Discuss the construction, principle and operation of a DC motor.

(OR)

23.	Describe the construction, principle and operation of a three phase induction motor.

24.	a.	Write a note on the different types of torques in measuring instruments.				    (5)

b.	Explain the operation of a Megger instrument.									  (10)

(OR)

25.	a.	Draw and explain about stair case wiring.										    (7)

b.	Elaborate the function of induction type energy meter.							    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRIC CIRCUITS AND NETWORKS				      Time : 3 hours

Subject Code:	12EE201									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The lamps in a set of Christmas tree are connected in series. If there are 20 lamps and each lamp has 25Ω resistance, calculate the current taken from the supply of 230V.

2.	In an electrical network, there are two resistances R1 and R2 connected in parallel. If any one of the resistance is disconnected, what will happen to the net equivalent resistance of the network?

3.	A super mesh requires the application of ____________.

4.	___________ theorem permits to transfer source from one position to another.

5.	What is the expression of time constant for RC series network?

6.	A network function is given by Z(s)=s+4s. Find the pole-zero plot.

7.	The maximum value of coefficient of coupling is ___________.

8.	In a balanced star connected system, line voltages are ___________ ahead of their respective phase voltages.

9.	Write the condition for symmetry in terms of Z parameters.

10.	Define cut off frequency.

PART – B  (5 x 3 = 15 MARKS)

11.	For the circuit given in below, determine the following quantities:

a. 		Total Resistance; 					b. 		Total current IT;

12.	State Thevenin’s Theorem.

13.	A function in s domain is given by I(s)=(s+1)s(s+2). Find the initial and final value of i(t).

14.	Two inductively coupled coils have self inductances L1 = 50 mH and L2 = 200 mH. If the coefficient of coupling is 0.5, find the value of mutual inductance between the coils.

15.	What is meant by impedance matching?

PART – C  (5 x 15 = 75 MARKS)

16.	In the network configuration of the figure, find the current in the  ½ ohm resistor using nodal 	analysis.

[P.T.O.]

(OR)

17.	Calculate the effective resistance between point A and B in the given circuit shown in figure.

18.	Find the current in the 6 Ω resistors using the principle of superposition for the circuit of figure.

(OR)

19.	Find the value of RL for maximum power transferred in the circuit shown in the figure and also find maximum power delivered to the load in the circuit.

20.	Derive an expression of Sinusoidal input of series RC circuits using Laplace Transform Analysis.

(OR)

21.	In the figure, the switch K is closed at position 1 at t = 0. Assuming no initial current through inductor, find current at t = 0.3sec. Find instant of time at which voltage across R equals voltage across L.

22.	a.	Calculate the total inductance of three coupled circuit shown in the figure.

b.	Derive an expression for output voltage, output current and amplification of single tuned circuit.

(OR)

[P.T.O]

23.	a.	The two wattmeter method is used to measure power in a three-phase load. The wattmeter

readings are 2000 W and 500 W respectively. Find the power factor of the circuit:

i.	When both readings are positive.

ii.	When second wattmeter current coil is reversed.

b.	A series combination of  3 ohm resistance and a 796.18μF capacitor in each branch forms a three phase star connected balanced load which is connected to a 415V, 3-phase, 50Hz AC supply. Calculate:

i.	the power consumed 		ii.	current drawn by the load.

If the same load is now connected as a delta, determine:

i.	the power consumed 		ii.	current drawn by the load

24.	Design m-derived T and π section low pass filter having design impedance of 600 Ω, cutoff frequency of 1 kHz and frequency of infinite attenuation 10.5 kHz.

(OR)

25.	Obtain impedance parameters of the network shown in the figure.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTROMAGNETIC FIELDS							      Time : 3 hours

Subject Code:	12EE202									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The intersection of cone and sphere in a spherical coordinate system is a ________.

2.	Divergence of curl of a vector is __________.

3.	A line charge is represented as _________.

4.	If the quantity changes with respect to time it is _________ field.

5.	Magnetostatic field is produced by __________ current.

6.	The point form of ohms law is ___________.

7.	The conduction current for electric field is __________.

8.	___________ is the unit of conduction current density.

9.	In wave equation for homogenous medium the static charges is __________.

10.	The distance the wave travels to complete one revolution is __________.

PART – B  (5 x 3 = 15 MARKS)

11.	 Locate the following points in the corresponding coordinate systems

P (2, 4, 2)  Q (3, 45º, 6)  R (1.5, 120º, 30º)

12. 	An electrostatic field is given by (x/2+2y) ax+2xay. Find the work done in moving a charge of –20 µ C from origin to (4, 0, 0)

13. 	A 200 turns rectangular coil 30 cm by 15 cm with a current of 5A is placed in a uniform magnetic field of 0.2 Tesla. Find the magnetic moment and the maximum torque.

14. 	A Faraday’s disc generator has a diameter of 0.7m and is rotated at a rate of 1500 rpm. The disc experiences a magnetic field of 0.02 milli Tesla. Calculate the e.m.f produced.

15. 	A copper sheet has σ = 5.8 × 107 Siemens/m and μr = 1. Calculate the skin depth at the frequency of 100 Hz.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Given vectors  A=2ax+3ay+4az   and  B=5ax+2ay−6az determine:

i.	A+B		ii.	The component of Aalong ay

iii.	A unit vector parallel to 3A+B.

b. 	Use the spherical co-ordinate system to find the volume of a hemisphere.

c. 	Draw the differential element in a cylindrical coordinate system. Write the expressions for differential length, surfaces and volume.

d. 	Derive the relationship between Cartesian and spherical coordinate.	      	      (3+4+4+4)

(OR)

17. 	a. 	Compute the divergence of the following vector field:

Fx,y,z=x3+y3, 3x2, 3zy2.

b. 	Compute the curl of the following vector field:

Fx, y, z=excosy, ex siny, 0.

c. 	Give the physical meaning of divergence of a vector field. 		           		   (5+5+5)

[P.T.O.]

18. 	a. 	State and explain Columb’s law in detail. 										    (5)

b. 	Three point charges of 3*103 µC are placed at 3 corners of a square. The side of  a square is 0.2m. Find electric field intensity at vacant corner. 							  	  (10)

(OR)

19. 	An infinitely long line charge of uniform density ρL C/m is placed along Z –axis. Find the expression for electric field intensity at a point in Y –axis, which is ‘a’ meters away from Z – axis.

20. 	a. 	Find the expression for magnetic field intensity due to a infinite long straight conductor 	carrying a current of I amperes at a point which has the shortest distance of ‘a’ from the

points.																		  (10)

b.	A circular current loop x2 + y2 = 9, is in the z = 0 plane and it carries a current of 3 Amps.

Find  the magnetic field at the points (0, 0, 4) and (0, 0, –4).			            		    (5)

(OR)

21. 	a. 	State and prove Amper’s Circuital law.

b. 	State and prove Biot Savarat law.          											(7+8)

22. 	a. 	Illustrate the nature of the conductivity and static charges in free space and derive the 	Maxwell equation for the same in integral and differential form.					    (9)

b. 	Elaborate on eddy currents mentioning the merits and demerits of eddy currents in 	practical applications.															    (6)

(OR)

23. 	a. 	Verify whether the fields vectors  E=Emsinxsint ay  and  H=Eμ0cosxcost a  satisfy

Maxwell’s equations.															    (8)

b. 	Obtain an expression for the electro motive force generated in Faraday’s disc generator.       																					    (7)

24. 	a. 	Derive the general wave equation for a uniform plane wave.   						    (7)

b. 	Discuss the propagation of electromagnetic wave in dielectric medium.              		    (8)

(OR)

25. 	Derive the expressions for attenuation, phase and intrinsic impedance offered by good conductors to electromagnetic waves.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRON DEVICES									      Time : 3 hours

Subject Code:	12EE203									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the dynamic resistance of a PN Junction diode.

2.	How does a LED work?

3.	Why BJT is called as a current controlled device?

4.	What is the importance of the Q point?

5.	Why the input impedance of JFET is high compared to BJT?

6.	JFET can be used as a resistor by varying __________.

7.	Write the expression of MOSFET drain current.

8.	List any two applications of MOSFET.

9.	What is the need for buried layer in BJT fabrication process?

10.	What is Self isolation?

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly explain the temperature dependence of PN Junction diode VI characteristics.

12.	Draw the Ebers Moll model of a BJT.

13.	How does a JFET work as a switch? Explain.

14.	Sketch the structure of an N channel depletion type MOSFET.

15.	Briefly explain the process of Photolithography.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the Diffusion and transition capacitances of PN Junction diode.                      	   (7)

b.	Derive the small signal model and large signal model of a PN Junction diode.            	    (8)

(OR)

17.	Explain the principle of operation of the following:                                                    	        (3x5)

a.	Zener diode            b.	Photo diode           c.	Schottky barrier diode

18.	a.	Explain the operation of a BJT. What should be done to increase the speed of a bipolar       	transistor?                                                                                                                        	    (8)

b.	Describe the Input /Output characteristics of CB transistor configuration.                   	   (7)

(OR)

19.	a.	Draw and Explain the operation of Transistor Self bias circuit.                                    	    (7)

b.	Explain the operation of a high frequency effects in the operation of a Transistor amplifier.                                                                                                                         
																				    (8)

20. 	Explain the operation of JFET with its drain and transfer characteristics.

(OR)

21.	a.	Describe the biasing methods used in JFET.                                                                      (8)

b.	Draw and Explain the small signal model of JFET.                                                           (7)

22.	With neat diagram, explain the operation of MOSFET in Depletion mode and derive its      	current equations.

[P.T.O.]

(OR)

23.	a.	Compare the characteristics of PMOS and NMOS devices.                                          	    (8)

b.	Explain the operation of a MOSFET amplifier.                                                             	    (7)

24.	a.	Write a short note on Monolothic IC technology.                                                              (7)

b.	Explain the fabrication process of Monolithic diode.                                                    	    (8)

(OR)

25. 	Explain the fabrication process of PNP multiple emitter transistor and Enhancement MOSFET.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRONIC CIRCUITS								      Time : 3 hours

Subject Code:	12EE205		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The ripple factor for a full wave rectifier is ___________.

2.	What is a low pass filter?

3.	A circuit that removes a portion of the input signal without distorting the remaining part of 	the input is called ___________.

4.	Clampers are also known as ___________.

5.	CMRR is ___________.

6.	The current gain of the Darlington amplifier is ___________.

7.	The maximum efficiency of a Class-B amplifier is ___________.

8.	Negative feedback results in decreased ___________ gain.

9.	The piezo electric material used in a crystal oscillator is ___________.

10.	___________ is a free running multivibrator.

PART – B  (5 x 3 = 15 MARKS)

11.	Define ripple factor. What is the value of ripple factor for half wave rectifier?

12. 	What is a clipper circuit? Mention its types.

13.	Draw the circuit of emitter follower configuration with a Darlington amplifier.

14.	What are the four different combinations of negative feedback connections?

15.	Define Barkhausen criterion.

PART – C  (5 x 15 = 75 MARKS)

16. 	Discuss in detail the working of a full wave rectifier with corresponding circuit diagram 	and waveform. Also derive the necessary equations.

(OR)

17.	Explain the working of a transistorized series and shunt voltage regulators.

18. 	With a neat diagram, explain the clamping operation.

(OR)

19.	What is a pulse transformer? Explain its equivalent circuit.

20.	Discuss about cascaded JFET amplifier and its equivalent circuit with a neat diagram.

(OR)

21.	Define CMRR. Derive the expression for CMRR for a differential amplifier.

22. 	Describe the working of a current series feedback circuit with necessary circuit and equations.

(OR)

23. 	Explain in detail about the class B push-pull amplifier with a neat diagram.

24.	Describe the working of a Wien Bridge oscillator with a neat diagram.

(OR)

25.	With a neat diagram explain the operation of a Schmitt trigger.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DC MACHINES AND TRANSFORMERS				      Time : 3 hours

Subject Code:	12EE207									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In rotating machines, the coupling field is the link between ___________ and ___________.

2.	The main aim of the principle of energy conversion is to estimate forces – True or False?

3.	The distance between starting ends of two consecutive coils is called ___________ pitch.

4.	Define self excitation.

5.	State the principle of dc motor.

6.	Give the condition for maximum efficiency of dc machines.

7.	In ___________ type of transformer the core is surrounded by the windings.

8.	Define voltage regulation of a transformer.

9.	What is Sumpner’s test?

10.	What is an auto transformer?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the block diagram of electromechanical energy conversion system and discuss.

12.	A 4-pole, 1200 rpm, dc generator with a lap connected armature has 65 slots and 12 conductors per slot. The flux per pole is 0.02 Wb. Calculate the emf induced in the armature.

13.	Calculate the torque in Nm developed by a 440 V DC motor having an armature resistance of 0.25 Ω and running at 750 rpm when taking a current of 60 A.

14.	A 1-phase transformer has 40 primary and 1100 secondary turns. The net cross-sectional area of the core is 500 cm2. If the primary winding be connected to 50 Hz supply at 400 V, calculate the value of maximum flux density in the core and the emf induced in the secondary.

15.	What is the purpose of performing short-circuit test on a transformer? Why is the short-circuit test performed on HV side?

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the schematic showing a single excited electromechanical energy conversion system and derive the expression for energy stored in the magnetic field.

(OR)

17.	Find an expression for the force per unit area between the plates of a parallel plate capacitor in terms of the electric field intensity. Use both the energy and co energy methods.

18.	Draw and explain the no-load and load characteristics of dc shunt generator.

(OR)

19.	Explain the process of commutation in a dc machine. Discuss also the methods to improve commutation.

20.	Draw a neat sketch of a 3-point starter for a dc shunt motor and explain how it works. How the motor is protected against overloads and loss of supply?

(OR)

[P.T.O.]

21.	Two shunt machines loaded for the Hopkinson’s test take 15 A at 220 V from the supply mains. The generator output current is 85 A and the shunt field circuit currents are 3 A and 2.5 A for the generator and motor respectively. The armature resistance is 0.05 Ω. Calculate the efficiency of each machine for this particular load condition.

22.	Draw and explain the phasor diagram of a single phase transformer supplying:

a.	a lagging load 		b.	a leading load 		c.	a UPF load.

(OR)

23.	The primary and secondary windings of a 3 kVA, 6.6 kV/240 V transformer have resistances of 10 Ω and 0.013 Ω respectively. The leakage reactances of the windings are 17 Ω and 0.022 Ω. Estimate the percentage voltage regulation of the transformer when it is delivering full load at 0.8 p.f lagging at the rated voltage.

24.	Two single phase transformers supply in parallel to a common load of 1000 A at 0.8 p.f lagging. For each transformer the secondary emf on open circuit is 3300 V and the total impedances in terms of secondary are (0.1 + j 0.3) Ω and (0.05 + j 0.15) Ω. Determine the current shared by the two transformers.

(OR)

25.	Derive an expression for the saving in copper effected by using an auto transformer instead of a two winding transformer.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INDUCTION AND SYNCHRONOUS MACHINES		      Time : 3 hours

Subject Code:	12EE209				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The direction of rotation of a three phase induction motor can be changed by changing 	_________.

2.	The slip of a three phase induction machine when operating in regenerative braking mode is

_________.

3.	In what ratio will the starting line current and starting torque be reduced in star delta starting?

4.   	Write the EMF equation of an Alternator.

5.   	What is an induction generator?

6.   	What are the different types of Repulsion motor?

7.    	List few applications of synchronous motor.

8.    What are the types of Alternator based on its construction?

9.	How a synchronous machine is different from an Induction machine.

10.	Define voltage regulation of an alternator.

PART – B  (5 x 3 = 15 MARKS)

11.	An induction motor cannot run at synchronous speed. Why?

12.	Why starters are necessary for starting induction motors?

13. 	Draw the phasor diagram of loaded alternator at lagging pf.

14.	What are the conditions to be satisfied for parallel operation of alternators?

15.	What is hunting in synchronous machines and how it is suppressed?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the phenomena of crawling and cogging in three phase induction motors.    	    (7)

b.	Starting from the first principles develop the equivalent circuit of a three phase induction motor.                                                                                           					    (8)

(OR)

17.	a.	An 8 pole, 3-phase, 50 Hz induction motor is running at a speed of 710 rpm with an input 	power of 35 kW. The stator copper loss at this operating condition is known to be 1200 W 	while the rotational losses are 600 W. Find:

i.	the rotor copper loss 		ii.	the gross torque developed

iii.	the gross mechanical power developed 		iv.	the mechanical power output.                                  													 							   (8)

b.	Draw and explain the complete torque speed characteristic of a three phase induction 	machine for all ranges of speed.                                                              				    (7)

18.  a.	Explain the double field revolving theory for single phase induction motors.                   (8)

b.	Describe the construction and working of Hystersis motor.                                               (7)

(OR)

[P.T.O.]

19.	a.	A 200W, 240 V 50 Hz, single phase induction motor runs on rated load with a slip of

0.05 pu. The parameters are: R1m=11.4Ω,  X1m=14.5Ω , R2’= 13.8Ω , X2’=14.4Ω, XM=270Ω. core and mechanical loss=32 W.  Calculate:

i.	power factor 		ii.	synchronous power  	iii.	shaft power 		iv.	input power

v.	efficiency.                                                        								    (7)

b.	Explain how a universal motor can operate from dc as well as ac supplies.                       (8)

20.	a.	Define voltage regulation of an alternator. Explain the effect of load power factor on 	voltage regulation.															  (10)

b.	Starting from fundamentals derive the emf equation of an alternator.                                (5)

(OR)

21.	a.	Explain the MMF method of determining the voltage regulation of alternator.       	  (10)

b.	A 3-phase 11kV star connected alternator supplies a load of 10 MW at pf of 0.85 lagging. Calculate the generated voltage if the armature resistance is 0.1Ω per phase and the synchronous reactance is 0.66 Ω per phase.                     				          		    (5)

22.	 Explain two reaction theory of salient pole synchronous alternator.

(OR)

23.  Describe the slip test method for the measurement of Xd and Xq of synchronous machines.

24.	a.	Explain with neat sketches the principle of operation of a three phase synchronous motor. 	Also explain why it will not run at other than synchronous speed?                                    (8)

b.	What is a synchronous condenser? Explain with the help of phasor diagram its operation. 																				    (7)

(OR)

25.	Explain the variation in excitation on the armature current and power factor of a synchronous motor.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRICAL MACHINE DESIGN						      Time : 3 hours

Subject Code:	12EE211									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the main dimensions of a rotating machine?

2.	Give an expression for the output coefficient of an AC machine.

3.	State the difference between the armature winding of dc machine and the stator winding of ac 	machine.

4.	What is the need for brushes in a dc machine?

5. 	How the heat dissipation in a transformer occurs?

6. 	Define window space factor.

7. 	Write the expression for length of mean turn of stator winding for the Induction motor.

8.	How the induction motor can be designed for best power factor?

9. 	The limiting factor for the diameter of synchronous machine is ___________.

10. 	What are the two types of poles used in salient pole machine?

PART – B  (5 x 3 = 15 MARKS)

11.	Define Specific Magnetic Loading.

12.	List out the advantages of large number of poles in dc machine.

13. 	Mention the different methods of cooling of transformers.

14.  	What are the factors to be considered for the choice of specific electric loading in Induction motor?

15.  What are the advantages of large air-gap in synchronous machine?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive the output equation of a DC Machine.   	                                  				    (8)

b.	A 350 kW, 500 V, 450 rpm, 6 pole d.c. generator is built with an armature diameter of 0.87m  and a core length of 0.32 m. The lap wound armature has 660 conductors. Calculate the specific electric and magnetic loadings.								    		    (7)

(OR)

17.	a.	Prove that in a DC machine, the Volume of Active parts is proportional to the torque of the

machine.																	    (8)

b.  	Determine the apparent flux density in the teeth of a dc machine when the real flux density is  2.15 wb/sm2 ; slot pitch = 28mm; slot width = 10mm and the gross core length of 0.35m.  The number of ventilating ducts is 4, each of 10mm wide.  The magnetizing fore for a flux density of 2.15 wb/m2  is 55000 A/m.  The iron stacking factor is 0.9.		    (7)

18.	Calculate the diameter and length of an armature for a 7.5kW, 4 pole, 1000 r.p.m, 220V, shunt motor. Given is the full load efficiency as 0.83.  Maximum gap flux density = 0.9 Wb/m2, Specific electric loading  = 30,000 AT/m, Field form factor = 0.7. Assume that the maximum efficiency occurs at full load and field current is 2.5% of the rated current.  The pole face is square.

[P.T.O]

(OR)

19.	a. 	Why a wave winding is preferred compared to lap winding?						    (5)

b.	A Shunt Field coil has to develop an MMF of 9000 AT.  The voltage drop in the coil is 40V and the resistivity of round wire is 0.021 Ω/m and mm2.  The depth of the winding is 35 mm and length of mean turn is 1.4 m.  Design a coil so that the power dissipated is      70 W/m2 of the total coil surfaces. Take the diameter of the insulated wire 0.2 mm greater than that of a bare wire.														  (10)

20.	a.	Derive the expression for output equation of a single phase transformer.				    (7)

b.	Calculate the core and window areas required for a 1000 kVA 6600/400 V, 50 Hz Single phase core type transformer. Assume a maximum flux density of 1.25 Wb/m2 and a current density of   2.5 A/mm2, Voltage per turn is 30 V. Window space factor is 0.32.		    (8)

(OR)

21.	The tank of 1250 kVA, natural oil cooled transformer has the dimensions of length, width and height as 1.55 × 0.65 × 1.85 m respectively. The full load loss = 13.1 KW, loss dissipation due to radiations = 6 W/m2 - 0C, loss dissipation due to convection = 6.5 W/m2 - 0C, Improvement in convection due to provision to tubes = 40%, Temperature rise = 400C, Length of each tube   = 1m, diameter of tube = 50 mm.  Find the number of tubes for this transformer. Neglect the top and bottom surface of the tank as regards the cooling.

22.	Find the main dimensions of a 15 kW, 3 phase, 400 V, 50  Hz, 2810  r.p.m, squirrel cage induction motor having an efficiency  of 0.88 and a full load power factor of 0.9. Assume specific magnetic loading = 0.5 Wb/m2, specific electric loading = 25000 A/m. Take the rotor peripheral speed as approximately 20 m/s at synchronous speed.

(OR)

23.	A 3 phase, 2 pole, 50 Hz squirrel cage induction motor has a rotor diameter 0.20 m and core length  0.12 m. The peak density in the air gap is 0.55 Wb/m2. The rotor has 33 bars, each of resistance 125 μΩ and a leakage inductance 2 μH. The slip is 6 %. Calculate:

a.	the peak value of current in each bar	b.	Rotor I2R loss		c.	Rotor output

d.	torque exerted. Neglect the resistance of end rings.

24.	a.	Write short notes on short circuit ratio.											    (5)

b.	Determine the main dimensions for a 1000 kVA, 50 Hz, 3 phase, 375 r.p.m, alternator. The average air gap flux density is 0.55 Wb / m2 and the ampere conductors per metre are 28,000.  Use rectangular poles and assume a suitable value for ratio of core length to pole pitch is order that bolted on pole construction is used for which the maximum permissible peripheral speed is 50 m/s. The runaway speed is 1.8 times the synchronous speed.       (10)

(OR)

25.	A 1250 kVA, 3 phase, 6600 V, salient pole alternator has the following data: Air gap diameter = 1.6 m, length of core = 0.45 m, number of poles = 20, armature ampere conductors per metre = 28000, ratio of pole arc to pole pitch is 0.68, stator slot pitch = 28 mm, current density in damper bars = 3 A/mm2. Design a suitable damper winding for the machine.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	POWER ELECTRONICS								      Time : 3 hours

Subject Code:	12EE212							       			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is holding current in SCR?

2.	IGBT is a voltage controlled device. Why?

3.	What are the different turn on methods of SCR?

4.	In a single phase fully controlled bridge converter, write the condition for the two modes of operation.

5.	Write the expression for the average output voltage of a step-up chopper.

6.	What is meant by PWM control in dc chopper?

7.	Why diodes should be connected in antiparallel with the thyristors in inverter circuits?

8.	Inverter circuit uses forced commutation operation. (True or False)

9.	What are the main components used for isolating the Power Circuits, Power Semiconductor from the low-power circuit?

10.	What are advantages of SMPS over linear power supply?

PART – B  (5 x 3 = 15 MARKS)

11.	What is the difference between power diode and signal diode?

12.	Give an expression for average voltage of single phase semi converters.

13. 	Write down the expression for the average output voltage for step down and step up chopper.

14. 	Compare CSI and VSI.

15.	What are the applications of cyclo-converter?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Draw and explain the reverse recovery characteristics of a power diode.				    (7)

b.	Give the basic structure, characteristics and explain the operation of a GTO.			    (8)

(OR)

17.	a.	Compare Power MOSFET and IGBT.											    (5)

b.	Discuss the static and switching operation of Thyristor.							  (10)

18.	Describe the working of 1Ф fully controlled bridge converter in the Rectifying mode and inversion mode. Derive the expressions for average output voltage and rms output voltage.

(OR)

19.	a. 	A single phase controlled rectifier bridge consists of one SCR and three diodes. Sketch 	output voltage waveform for a firing angle α for the SCR and hence obtain an expression 	for the average output voltage under the assumption of constant current. Show the 	conduction of various components as well.

b. 	Draw waveforms of current through SCR and three diodes assuming constant load current.

c. 	For an source voltage of 230V, 50Hz and firing angle of 45o, find the average output current and power delivered to battery in case load consists of R = 5Ω, L = 8mH and E = 100V.

[P.T.O.]

20.	Describe the operation of a single-phase to single-phase circuit-step-down cycloconverter with neat output voltage and current waveforms.

(OR)

21.	a. 	With neat diagrams, explain Four-Quadrant operation of Chopper in detail.               	    (8)

b. 	A type – A chopper has Vdc = 230V, R = 10 Ohms. Take a voltage drop of 2V across chopper when it is on. For a duty cycle is 0.4, calculate average output voltage Vavg, rms values of output voltage and chopper efficiency.                            				            (7)

22.	Explain the 180 mode operation of a three-phase bridge inverter with an appropriate circuit diagram, waveforms and equations.

(OR)

23.	A single-phase half-bridge inverter has load R = 2Ω and dc source voltage Vs /2 =115V.

a. 	Sketch the waveforms for output voltage, load current, currents through thyristor 1 and 	diode1 and voltage across thyristor. Harmonics other than fundamental component are 	neglected. Indicate the devices that conduct during different intervals of one cycle.

b. 	Find the power delivered to load due to fundamental current.

c. 	Check whether forced commutation is required.

24.	a. 	Explain control scheme for single pulse width modulation with proper block diagram. (8)

b.	Explain HVDC transmission with proper block diagram.							    (7)

(OR)

25.	Draw the block diagram of various types of UPS and explain the functions of each block in them.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRIC DRIVES AND CONTROL					      Time : 3 hours

Subject Code:	12EE214		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the fundamental torque equation for motor-load system.

2.	Load Torques which always oppose the motion and change their sign on the reversal of 	motion are called ___________torques.

3.	What types of drives have been used in rotary printing press which has several printing 	stations?

4.	In Induction motors with normal design, the ratio of breakdown to rated torque varies from 	___________ to ___________.

5.	Write the condition to achieve the regenerative braking in dc drives.

6.	Controlled rectifier fed dc drives are also known as static  ___________ drives.

7.	What type of starter is used in textile induction machine during starting?

8.	How to connect the induction motor windings for zero sequence braking?

9.	What are the modes of variable frequency control of synchronous motor drives?

10.	In wound field Synchronous motor, the important feature is that its ___________ can be 	controlled by varying field current.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the factors influencing the choice of electric drives?

12.	A motor has heating time constant of 70 min and a cooling time constant of 90 min. When 	run continuously on full load of 400 kW, final temperature rise is 50° C.

Determine the maximum load the motor can deliver when subjected to intermittent periodic 	load cycle consisting of a load period of 10 min followed by a no load period of 15 min.

13.	Explain Dynamic Braking operation of chopper control of separately excited dc motor.

14.	How do we change the speed of three phase Induction motor using pole changing method?

15.	Draw the phasor diagram of a salient pole motor and write the torque and power equations.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	State essential parts of electrical drives.											    (5)

b.	What are the functions of a power modulator?									    (4)

c.	Mention the components of load torques.										    (6)

(OR)

17.	a.	What are the reasons for using load equalization in an electric drive?				    (5)

b.	Explain the multiquadrant operation of an electric drive with an example. 			  (10)

18.	 What type of closed-loop speed control schemes are used for the following conditions?

a.	Multimotor drives.															    (8)

b.	To get excellent speed regulation.												    (7)

(OR)

[P.T.O.]

19.	How do you select the motor rating while that motor is operated for short time duty and 	intermittent duty cycle?

20.	Explain single phase half controlled rectifier control of dc separately excited motor in 	continuous conduction and discontinuous conduction mode.

(OR)

21.	How is the chopper used to operate the DC separately excited motor in motoring and 	regenerative braking operation?

22.	a.	Explain the operation of voltage source inverter fed induction motor drive with neat 				sketch. 																		  (10)

b.	Explain about the Dynamic braking operation of voltage source inverter fed Induction 		    	motor drive.																	    (5)

(OR)

23.	Discuss about the speed control of three phase Induction motor using static sherbius slip power recovery schemes.

24.	Explain in detail about the solar powered pump drives.

(OR)

25.	How is the Brushless dc motor drive used for servo applications?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	GENERATION, TRANSMISSION AND DISTRIBUTION	      Time : 3 hours

Subject Code:	12EE215				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention few nuclear plants in Tamil Nadu.

2.	What is load duration curve?

3.	What is transposition in transmission lines?

4.	State the Ferranti effect.

5.	List out the types of insulators used in EHV transmission lines.

6.	What is corona effect?

7.	State the method used to find the location of faults in underground cables.

8.	Draw the diagram of a 3 ½ core cable.

9.	Mention of the level of voltages in the ac distributions.

10.	Mention the advantage of a ring main distribution compared with radial distribution.

PART – B  (5 x 3 = 15 MARKS)

11.	State the function of a wind mill.

12.	Mention the advantages of transmission line with high level of voltage for transmission.

13.	What is the effect of wind and ice on the sag in the transmission lines?

14.	What is insulation resistance of a cable?

15.	Mention the merits of three phase distribution system.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the function of a thermal power plant with a relevant diagram.

(OR)

17.	Explain the internal elements of a solar power plant and how the power is synchronized with electricity grid.

18.	Determine the efficiency and regulation of a 3- phase, 100km, 50 Hz transmission line delivering 20 MW at a p.f. of a 0.8 lagging and 66 kV to a balanced load. The conductors are of copper, each having resistance 0.1 ohm per km, 1.5 cm outside diameter, spaced equilaterally 2 meters between centers. Neglect leakage. Use:

a.	nominal –T		b.	nominal – π method.

(OR)

19.	Write short notes on:		a.	rigorous solution for long line 		b.	ABCD constants.

20.	Explain any two type of insulation used in overhead lines.

(OR)

21.	Derive the expression for sag in transmission line with unequal supports.

22.	Explain the different types of cable with necessary diagrams.

(OR)

23.	Explain the two methods of grading of cables.

[P.T.O.]

24.	Draw the schematic and explain the practical AC distribution system adopted.

(OR)

25.	A conductor of 1 cm diameter passes centrally through a porcelain cylinder of internal diameter 2 cm and external diameter 7 cm. The cylinder is surrounded by a tightly fitting metal sheath. The permittivity of porcelain is 5 and the peak voltage gradient in air must not exceed 34kV/cm. Determine the maximum safe working voltage.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	POWER SYSTEM ANALYSIS							      Time : 3 hours

Subject Code:	12EE216									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The impedance diagram is used for ____________ analysis.

2.	The positive sequence components have same phase sequence as that of original vectors (True /False).

3.	____________ faults are due to short circuits.

4.	The synchronous capacitor will ____________ reactive power.

5.	The normal value of the acceleration factor is ____________

6.	In FDLF method, mention the pair of power system parameters.

7.	If the fuel cost of the power generating unit  F = 0.06P2 + 40P + 120, the incremental fuel cost at 40 MW is given by____________.

8.	Define Unit Commitment.

9.	If the transmitted power increased beyond ____________, then the synchronism will be lost.

10.	What is EAC in stability analysis?

PART – B  (5 x 3 = 15 MARKS)

11. 	A three phase, Δ-Y transformer with rating 10 MVA, 132 KV / 11KV has its primary and secondary leakage reactance as 12 Ω / phase and 0.05 Ω / phase respectively. Calculate the p.u. reactance of transformer.

12. 	Write down the properties of Ybus.

13. 	Classify different types of buses in a power system network.

14. 	Write the co-ordination equation including losses and explain the terms involved.

15. 	List the methods of improving the transient stability limit of a power system.

PART – C  (5 x 15 = 75 MARKS)

16. 	Draw the impedance diagram for the electric power system shown in the figure. All the  impedance in per unit on a 100 MVA base. Choose 20 KV as the voltage base for generator. The three-phase power and line-line ratings are given below.

Generator G1: 90 MVA, 20 KV , X = 9 %

3ΦTransformer T1: 80 MVA, 20 / 200 KV,  X = 16 %

3Φ Transformer T2: 80 MVA, 200 / 20 KV , X = 20 %

Generator G2: 90 MVA, 18 KV, X = 9 %

Line: 200 KV , X = 120

Load: 200 KV, S = 48 MW +  j64 MVAR.

(OR)

[P.T.O.]

17. 	Draw the reactance diagram for the system given in the figure. Select a base value of 50MVA,    138 kV in the 40 line. The ratings are as follows:

Generator 1: 20 MVA, 18 kV, X” = 20% 	Generator 2: 20 MVA, 18 kV, X” = 20%

Synchronous motor : 30 MVA, 13.8 kV, X” = 20%

3-Phase - Transformer (T1, T2, T3, T4):20 MVA, 138/20kV, X = 10%

3-Phase - Transformer (T5&T6):15MVA, 138/13.8kV, X = 10%

All the transformers are connected to step up the voltages of generators to the transmission line voltages.

18.	Obtain the ZBus for the sample system shown in the figure using Zbus building algorithm.

(OR)

19.  Brief about the following:

a.	Various Sequence network used in power system.								    (6)

b.	Line to Line Fault occurs in power system and derive the expression for fault current.   (9)

20.	With the help of a neat flow chart and expressions, explain the Newton-Raphson method of load flow solution when the system contains voltage controlled busses in addition to swing bus and load bus.

(OR)

21. 	The figure shows a three bus power system.

Bus 1 : Slack bus, V= 1.05 < 0 p.u.

Bus 2 : PV bus, V = 1.0 p.u. P = 3 p.u.

Bus 3 : PQ bus, P=4 p.u., Q=2 p.u.

Carry out one iteration of load flow solution by Gauss Seidel method. Neglect limits on 	reactive power generation.

22.	a.	Write short notes on Unit commitment and list out various constraints.	   			    (8)

b.	Derive the loss formula co efficient in economic load dispatch.				 		    (7)

(OR)

23.   a. 	Write short notes on Hydro Thermal Scheduling.			       						    (8)

b. 	A power plant has three units with the following fuel cost equations:				    (7)

Maximum and minimum loading on each unit is 150 MW and 40 MW. The demand is 275MW.  Find the optimal scheduling.

24.	a. 	Explain the Runge-Kutta method of analyzing multi machine power system for stability 	with a neat flow chart.															  (10)

b. 	A 50Hz, 4 pole turbo generator of rating 20 MVA, 13.2 kV has an inertia constant of

H = 9kW–sec/kVA. Find the kinetic energy stored in the rotor at synchronous speed.    (5)

(OR)

25.	a. 	Write short notes on importance of power quality and its international standard.		  (10)

b. 	Describe the equal area criterion for transient stability analysis of a system. 			    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	POWER SYSTEM PROTECTION AND SWITCHGEARS     Time : 3 hours

Subject Code:	12EE218										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Why do we use reactors in Power System?

2. 	What is meant by IDMT in overcurrent relay?

3.  	List any two drawbacks of Buchholz relay.

4.  	To protect the transformer with star delta connection against fault, the current transformers secondary will have_________ connection.

5. 	Define current chopping.

6. 	Write any merits of air blast circuit breaker.

7. 	What are the types of lightning stokes?

8. 	Surge absorbers are otherwise called as __________.

9. 	What is solid earthing?

10. 	Define insulation coordination.

PART – B  (5 x 3 = 15 MARKS)

11.  	Mention a few applications of CTs and PTs in protection schemes.

12.  	Draw the schematic diagram for the biased differential protection of an alternator.

13.  	List any two advantages and disadvantages of oil circuit breaker.

14. 	What are the drawbacks of valve type lightning arresters?

15. 	Write few important Indian Electricity rules for protection.

PART – C  (5 x 15 = 75 MARKS)

16. 	Describe the construction and principle of operation of an induction type directional over 	current relay. Derive the torque equation.

(OR)

17.	With a neat sketch, explain the construction, operating   principle and characteristics of a Mho type relay. Derive the torque equation.

18. 	What are the abnormal conditions in a large synchronous generator against which protection

is necessary? Draw neatly and explain the principle of operation of:

a. 	Merz  Price differential protection.

b. 	Stator inter-turn fault protection scheme.

(OR)

19. 	Identify and explain any two protection schemes necessary for the protection of a power  transformer.

20.  Briefly explain about the arc phenomenon and various methods of arc extinction in a circuit

breaker.

(OR)

21.   With the help of a neat diagram, explain the construction, operating principle, advantages and 	applications of SF6 circuit breaker.

[P.T.O.]

22.  a. 	Discuss the various causes of over voltages in transmission lines.  					    (8)

b. 	How do earthing screen and ground wires provide protection against direct lightning 	strokes?                                                                                             					    (7)

(OR)

23.  List out the different types of lightning arresters. Also explain the operation of any three 	types 	of lightning arresters.

24. 	Write notes on:

a.	Arc Suppression Coil   		b.	Earthing Transformer.

(OR)

25.	Briefly explain the insulation levels of substation equipments.  How is the co-ordination carried out among them?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CONTROL SYSTEMS									      Time : 3 hours

Subject Code:	12EE219									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the term Transfer function of a physical system.

2.	Draw the circuit diagram of a lag network.

3.	What is steady state error in time domain analysis?

4.	List out any two characteristic of negative feedback.

5.	What is resonant frequency?

6.	Define the term: Phase margin.

7.	A root of a characteristic equation lies on the imaginary axis. Comment on the stability of this system.

8.	What is characteristic equation?

9.	Write down the state model equation of a continuous time invariant system.

10.	List out any two properties of state transition matrix.

PART – B  (5 x 3 = 15 MARKS)

11.	Compare open loop system with closed loop system.

12.	Find the steady state error of  type ‘0” system for unit step input.

13.	Draw the step response of a second order system with its specifications.

14.	What is Nyquist stability criterion?

15.	State the criteria of controllability.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the transfer function of DC generator.

(OR)

17.	Obtain the transfer function for the signal flow graph shown in figure using Mason’s gain formula.

[P.T.O.]

18.	A system described by the differential equation is given by  d2ydt2+8dy(t)dt+25y(t)=50x(t).  Determine:

a.	response of the system 		b.	the maximum output of the system when a step input of magnitude 2.5 units is applied to the system.

(OR)

19.	The block diagram of a fire control system with unity feedback is shown in the figure. Determine the steady state error of the system when the system input is  rt=5+4t+3t22using generalized error series method.

20.	The Bode magnitude plot of a system is given in the figure. Determine the transfer function of the system.

(OR)

21.	The loop transfer function of a unity feedback system is given by GsHs=1s1+s2. Sketch the polar plot and determine the gain and phase margin.

22.	The loop transfer function of a unity feedback system is given by GsHs=Ks2+s10+s. Sketch the Nyquist plot and hence calculate the value of K for the stability.

(OR)

23.	The loop transfer function of a unity feedback system is given by GsHs=1ss2+2s+2. Sketch the root locus of the system.

24.	Find the state equation and output equation for the system given by

YsUs=s3+5s2+6s+1s3+4s2+3s+3

(OR)

25.	A system is given by the state equation: xt+010021u(t)	and the output equation  	yt=001110. Check whether the system is controllable and observable.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DIGITAL ELECTRONICS								      Time : 3 hours

Subject Code:	12EE220		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Binary equivalent for the decimal number (625.25)10  is   ____________.

2.	Covert the given expression in standard SOP form

a.	 Y = A + AB + CB		b. 	Y = PQ + R + PR

3.	What are universal gates?

4.	Draw the logic diagram for odd parity checker.

5.	Write the characteristic table and equation of SR flip-flop.

6.	A counter has modulo of 10. The numbers of flip flops required are ____________.

7.	Name the two sequential switching circuits.

8.	What are the advantages of asynchronous sequential circuits?

9.	The difference between a PLA and a PAL is  ____________.

10.	What is the main advantage of CMOS over TTL?

PART – B  (5 x 3 = 15 MARKS)

11.	Convert the following to other canonical form F(x, y, z) = π (0, 3, 6, 7).

12.	Compare de-multiplexor and decoder.

13.	Differentiate level and edge triggering.

14.	Differentiate between synchronous and asynchronous circuits.

15.	Draw and explain the CMOS inverter circuit.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Convert the following													               (4x2)

i.	(37.29)10 into equivalent binary number

ii.	(110010.1110)2  into equivalent decimal number

iii.	(268.75)10 into equivalent octal number

iv.    (615)8 into equivalent hexa decimal number

b. 	Simplify the following Boolean expression

F1 = XY + XYZ +  XYZ '	+ X'	YZ											           (4)

c.	Convert the following Binary number into equivalent Gray code (10111011)2		    (3)

(OR)

17.	Simplify the Boolean function using Karnaugh Map

F(W, X, Y, Z) = ∑(0, 1, 2, 4, 5, 6, 8, 9, 12, 13, 14).

18.	a.	Implement the following function using 3:8 decoder

F1(A,B,C) = ∑m(0, 1, 4, 5, 7)													    (7)

b.	Design a BCD to Gray code converter use don’t cares. 							    (8)

(OR)

19.	a.	Design a full adder using only NOR Gates.										    (8)

b.	Implement the following Boolean function with 8:1 multiplexer.

F(A, B, C, D) = ∑m(0, 2, 6, 10, 11, 12, 13) + d(3, 8, 14)  							    (7)

[P.T.O.]

20.	a.	Draw and explain the working of the following flip-flops.

i. 	JK	       ii. 	D											    				    (5)

b.	Design a 4-bit ring counter with a neat diagram and explain its timing sequence.	          (10)

(OR)

21.	Explain different types of shift registers with neat diagrams.

22.	A sequential circuit with two D flip flops A and B, two inputs X and Y and one output Z is specified by the following state equations.

A(t+1)   =  X'Y+XA.

B(t+1)   =  X'B+XA

Z  = B

a.	Draw the logic diagram.

b.	Derive the state table.

c.	Derive the state diagram.

(OR)

23.	Reduce the number of states in the following state table and give the reduced state table and state diagram.

| Present state | Next state |  | Output |  |
| --- | --- | --- | --- | --- |
|  | x=0 | x=1 | x=0 | x=1 |
| a | f | b | 0 | 0 |
| b | d | c | 0 | 0 |
| c | f | e | 0 | 0 |
| d | g | a | 1 | 0 |
| e | d | c | 0 | 0 |
| f | f | b | 1 | 1 |
| g | g | h | 0 | 1 |
| h | g | a | 1 | 0 |

24.	a. 	What are FPGA’s? Explain the general architecture of FPGA XC4000.				  (10)	b. 	Write short note on ROM, PROM, EPROM.     						    			    (5)

(OR)

25.	a. 	Simplify the following Boolean logic and implement using PAL:

i. 	F1 = Σ (1, 2, 4, 6)

ii. 	F2 = Σ (0, 1, 6, 7)												  	  		  (10)

b.	Write short notes on emitter coupled logic and integrated injection logic.			    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	LINEAR INTEGRATED CIRCUITS						      Time : 3 hours

Subject Code:	12EE221									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Draw the symbol of an OP-AMP.

2.	The bandwidth of an ideal OP-AMP is ____________.

3.	What is the gain of an inverting amplifier with RF=10kΩ andRi=5kΩ?

4.	Draw the circuit diagram of a practical Integrator.

5.	What is clipper?

6.	The Schmitt trigger is also called as ____________.

7.	What is a voltage regulator?

8.	Mention any two applications of IC 555.

9.	Define Lock-in-range.

10.	What is Resolution?

PART – B  (5 x 3 = 15 MARKS)

11.	List the ideal characteristics of an OP-AMP.

12.	Design an OP-AMP differentiator that will differentiate an input signal  with  fmax=100 Hz.

13.	Explain Zero crossing detector.

14.	Compare series and switching regulator.

15.	Discuss charge balancing ADC.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the following dc characteristics of an OP-AMP with the related derivation.

a.	Input bias current		                    b.	Input offset current

(OR)

17. 	Explain the following frequency compensation techniques in an op-amp circuit with the related derivation.

a.	Dominant Pole compensation		b.	Pole-zero compensation

18.	Draw the circuit of an Instrumentation amplifier and derive the expression for its output.

(OR)

19. 	a.	With neat circuit diagram, derive the transfer function of a first order low pass filter.	  (10)

b.	Design a low pass filter at a cutoff frequency of 1 kHz with a passband gain of 2.     	    (5)

20.	Draw the circuit of a triangular wave generator using op-amp and explain its working with waveforms. Also, derive an expression for its frequency of oscillation.

(OR)

21. 	a.     Explain the significance of precision diode.                                                                	    (5)

b.     Draw the circuit for precision Full wave rectifier and explain its operation.                   (10)

22.	Discuss the operation of an SMPS with a neat sketch and necessary waveforms.

(OR)

23. 	Explain with a neat sketch, the working of IC 555 as a Monostable multivibrator.

[P.T.O]

24.	a.	Draw the block diagram of a PLL and explain.			                                          	    (9)

b.	With necessary waveforms, explain digital phase detector.							    (6)

(OR)

25.  Differentiate R-2R ladder DAC and inverted R-2R ladder.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MICROPROCESSORS AND MICROCONTROLLERS	      Time : 3 hours

Subject Code:	12EE223										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the role of program counter in 8085 microprocessor?

2.	List the hardware interrupts of 8085 microprocessor.

3.	The 8051 has ________ bytes of on chip ROM.

4.	What is the role of overflow flag in 8051 microcontroller?

5.	List the assembler directives.

6.	What is the value of register A after the execution of the following instructions?

MOV A, #F6H

SWAP A

7.	Write a bit manipulation instruction that disables all the interrupts.

8.	Name any two special function registers related with timers of 8051.

9.	An alternate function of port pin P2.3 in the 8051 is   _________.

10.	What is the role of BSR mode in 8255?

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss the different addressing modes of 8085 with examples.

12.	Compare Microprocessor and Microcontroller.

13.	Write the format of PSW in 8051.

14.	Differentiate the timer and counter operation of 8051.

15.	Write a program to generate saw tooth wave using DAC with the microcontroller.

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat functional block diagram, explain the architecture of 8085 processor.

(OR)

17.	Sketch the timing diagram for the execution of STA 526AH Instruction.

18.	Write short notes on:

a.	Organization of internal RAM in 8051.											    (8)

b.	Special function registers.					                                                			    (7)

(OR)

19.	Explain the internal architecture of 8051 Microcontroller in detail.

20.	a.	The internal RAM location 30H holds 40H. The value stored in RAM location 40H is 10H. 	What will be the value in 30H and 40H after the following instructions are executed?

MOV R0, #30H

MOV A, @R0

MOV R1, A

MOV A, #7FH

MOV @R1, A

XCH A, @R0															  (10)

b.	Briefly discuss the signed number concepts in 8051.                                                       (5)

[P.T.O.]

(OR)

21.	Explain the function of 8051 instructions for performing arithmetic, logical and data transfer operations with suitable examples.

22.	Draw the format of timer mode control register TMOD and describe four modes of operations of Timer of 8051.

(OR)

23.	Briefly discuss the serial communication and its related SFR in 8051.

24.	Explain the external memory interfacing with 8051.

(OR)

25.	With a neat diagram, explain the keyboard interfacing with 8051 Microcontroller.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DIGITAL SIGNAL PROCESSING						      Time : 3 hours

Subject Code:	12EE225				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	If x(n) = x(–n) then the signal is called _____________.

2.	Write the convolution sum for FIR system.

3.	The number of complex additions and multiplications in radix-2 FFT are _____________ and _____________ respectively.

4.	Bit reversing is required for both DIT and DIF algorithms. State whether this statement is true or false.

5.	What is the relation between digital and analog frequency in bilinear transformation?

6.	The two popular techniques used to approximate the ideal frequency response are _____________ and _____________ approximation.

7.	What is the width of main lobe in Triangular window spectrum?

8.	List the well known design techniques for designing linear phase FIR filters.

9.	List any two digital signal processing application with TMS 320 family.

10.	What is interpolation?

PART – B  (5 x 3 = 15 MARKS)

11.	Test the linearity of the following systems given  as

a.	y (n) = B x(n) + C   				b.	y(n) = n x(n)

12.	List the differences between linear convolution and circular convolution.

13.	Sketch the magnitude response of type-1 Chebyshev filters.

14.	What is Gibb’s phenomenon? How can it be reduced?

15.	What is multirate DSP? Mention any two applications of it.

PART – C  (5 x 15 = 75 MARKS)

16.	Classify the various types  of discrete time signals; sketch and mathematically represent them with examples where ever possible.

(OR)

17.	a.	Determine the response of the LTI system whose input x(n) and impulse response h(n) are 	given by, x(n) = {1, 2, 0.5, 1} and h(n)={1, 2, 1, –1}.   							    (8)

↑                                        ↑

b.	Explain about analog to digital conversion with relevant diagrams.		 			    (7)

18.  	In an LTI system the input x (n) = {1, 2, 3} and the impulse response h(n) ={–1, –1}.     Determine the response of LTI system by radix-2 DIF method.

(OR)

19.	Compute eight point DFT of a sequence x (n) = {1, 2, 3, 4, 4, 3, 2, 1} using DIT algorithm.

20.	Design a digital Chebyshev IIR low pass filter with the following specification

0.8≤Hejω ≤1 , 0≤ω≤0.2π

Hejω≤0.2, 0.32π≤ω≤π      with T=1 sec. Apply bilinear transformation.

[P.T.O.]

(OR)

21.	Find the direct form I and direct form II realization of a discrete time system represented by

yn=−38yn−1+332yn−2+164yn−3+xn+3xn−1+2xn−2

22.	Design an Low Pass Filter using rectangular window if N = 9 whose desired frequency response is

a.	Determine the impulse response.			b.	Determine H(z).

(OR)

23.	a.	Compare the Hamming window and Kaiser window.   							    (5)

b.	Write the procedure for FIR filter design by frequency sampling method.     			    (5)                                 c.	Compare IIR and FIR filters.                                          							    (5)

24.	Explain in detail about the application of DSP techniques in any one real time application with relevant diagrams.

(OR)

25.	Draw and describe the features of Van Neumann and Harvard architecture.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	C++ AND DATA STRUCTURES							     Time : 3 hours

Subject Code:	12EE227									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are different types of Linked Lists?

2.	Mention applications of stack.

3.	List the main properties of a binary heap.

4.	What is sorting?

5.	Define class.

6.	What is an implicit constructor?

7.	Define function overloading.

8.	Define operator overloading.

9.	Define Virtual function.

10.	Define ifstream.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between Arrays and Linked List.

12.	What is divide and conquer strategy? Give an example.

13.	Give four examples for String Manipulation functions.

14.	What is member-dereferencing operator?

15.	Define the exception handling constructs.

PART – C  (5 x 15 = 75 MARKS)

16.	Write an algorithm to insert a node in doubly linked list in the beginning, middle and end of the list.

(OR)

17.	Write the algorithm for inserting and deleting the element in stack and queue.

18.	Explain the algorithm for quick sort. Sort the given values using quick sort:

65, 70, 75, 80, 85, 60, 55, 50, 45

(OR)

19.	Explain the algorithm for linear and binary search with example.

20.	a.	Write an interactive C++ program to perform matrix multiplication.

b.	Write a C++ program to find the given number is prime or not.

(OR)

21.	a.	Define Constructor overloading and illustrate its use with a suitable program.

b.	Write a program to demonstrate the use of Copy constructor.

22.	List the different type of Inheritance and explain with example.

(OR)

23.	Write a program to demonstrate the overloading of functions in base and derived classes.

24.	Explain the difference between a normal Virtual function and Pure Virtual function with example.

(OR)

25.	Discuss about the streams and files in C++ with example.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MEASUREMENTS AND INSTRUMENTATION			      Time : 3 hours

Subject Code:	12EE229										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The difference between measured value and true value of the quantity is called __________.

2.	__________ error adds variability to the data but does not affect average performance for the group.

3.	There is a controlling force present in a power factor meter (Y/N).

4.	The electrodynamic type wattmeter is generally referred as __________.

5.	Suggest the nominal measuring capability of commercial Maxwell’s bridges.

6.	_________ is used in the measurement of earth resistance.

7.	The quality factor of the inductance is calculated as __________.

8.	_________ are broadly used in applications of experimental stress analysis of machine and structures.

9.	The wattmeters are operated with __________ damping.

10.	In the free running sweep of the CRO the __________ waveform is repetitive.

PART – B  (5 x 3 = 15 MARKS)

11. 	Present the structure of the fundamental elements of an instrumentation system.

12. 	Define Phantom Loading.

13. 	Draw a bridge to measure medium resistance and derive its bridge balance equation.

14. 	The problem of reverse motion in the case tachometer encoder can be solved by encoders.

Validate this statement.

15. 	Configure a circuit for the range of 100 mV – 1 Volt using auto ranging.

PART – C  (5 x 15 = 75 MARKS)

16.  	a. 	Justify the statement - Moving iron instruments are un-polarized.					    (5)

b. 	Derive the force and torque equations of a D’Arsonal Galvanometer and explain the same

with a neat diagram. 								     							  (10)

(OR)

17.  a. 	The full scale deflection current of an ammeter is 1 mA and its internal resistance is     	100 Ω. If this meter is to have scale deflection at 5A, what is the value of shunt resistance 	to be used? 																	    (5)

b. 	Discuss on the extension of range in an ammeter using D’Arsonal movement.       	  (10)

18. 	Elaborate on the electro dynamometer type wattmeter with a neat sketch and derive the instantaneous torque and average deflecting torque.

(OR)

19. 	Discuss on the different adjustments to be made in a single phase energy meter instrument.

20. 	Elaborate on a bridge that will be best suited for the measurement of very low resistance measurement.

(OR)

[P.T.O]

21. 	a. 	Derive the balance equations for Anderson’s bridge supporting it with corresponding  	diagram.																		    (8)

b. 	How is the inductance measured by using Maxwell’s bridge? 						    (7)

22. 	Suggest a code pattern which is most convenient in explaining the motion representation in a familiar natural binary system. Provide relevant justification with an instrumental working.

(OR)

23. 	a. 	State the principal property utilized for measurement of temperature using Platinum 	resistance thermometers with a relevant diagram.								  	  (10)

b. 	With a graphical sketch express the characteristics of any three materials used in resistance

thermometers.																    (5)

24. 	Write short notes on:

a.	Auto ranging 				b.	Polarity detection

c.	Auto Zeroing 				d.	Latch in DVM		         			      (4+4+4+3)

(OR)

25.	Demonstrate two circuits for the measurement of current and measurement of resistance by digital multimeter.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ILLUMINATION ENGINEERING						      Time : 3 hours

Subject Code:	12EE238 									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Good efficient lighting obtained by combining ____________ and ____________.

2.	The visible range of light is ____________ to ____________.

3.	____________ Emission is employed in Hot cathode Lamps.

4.	List the commonly used gases in discharge lamps.

5.	What are isolux diagrams?

6.	The unit of luminous intensity is____________.

7.	Why is periodic check of the interior lamps required?

8.	Why are shadows important while designing interior lighting?

9.	Where do we use narrow beam flood lights?

10.	List the factors responsible for the lighting scheme for roads.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between rod cells and cone cells.

12.	What are the methods employed to tackle evaporation of tungsten filament in an Incandescent 	bulb?

13.	What is MSLI?

14.	Classify the types of industrial lighting.

15.	Define Glare and Glare index.

PART – C  (5 x 15 = 75 MARKS)

16. 	Draw the structure of human eye and discuss the working of human eye with Luminosity curves for Human Eye.

(OR)

17.	Define Photometry and discuss the types of measurement with photometric bench.

18.	What are luminaries? Explain the types of luminaries and how they control and direct light.

(OR)

19.	Draw and discuss the working of fluorescent lamp, and list the factors deciding the dimension of fluorescent lamp and lamp voltage.

20.	Discuss how the lighting calculations are done for a room.

(OR)

21.	Discuss in detail the laws of illumination.

22.	Discuss in detail the design procedure for interior lighting.

(OR)

23.	Discuss in detail the design procedure for following interior lighting:					(7+8)

a.	Hotels		   	b.	Hospitals

24.	Discuss in detail the design procedure for exterior flood lighting.

(OR)

25.	Explain in detail the lighting procedure for two way traffic roads and junctions.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	RENEWABLE ENERGY SOURCES						      Time : 3 hours

Subject Code:	12EE240				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define solar constant.

2. 	Define PV effect.

3. 	Define tip speed ratio.

4. 	What is called tethering?

5. 	What are bio-fuels?

6. 	Define fermentation.

7. 	The process of splitting water into hydrogen and oxygen by means of direct electric current is known as ____________.

8. 	Give the main components of a fuel cell.

9. 	Define spring tides.

10. 	What is magma?

PART – B  (5 x 3 = 15 MARKS)

11. 	Give the advantages and disadvantages of PV solar energy conversion.

12. 	Write and explain wind power equation.

13. 	Write down the applications of biogas.

14. 	Give the possible areas of use of hydrogen.

15. 	What are the important components of tidal power plants?

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the following:

a.	Solar pumping system 	b.	Solar cooker 		c.	Solar furnace.

(OR)

17. 	Explain the solar air conditioning and refrigeration system.

18. 	With a neat diagram, explain how wind energy can be converted into electrical energy.

(OR)

19. Explain briefly about the horizontal wind mills with a neat sketch.

20. Describe about the factors affecting biogas generation.

(OR)

21. Discuss about biomass conversion techniques.

22. Explain any two types of fuel cells with neat sketches.

(OR)

23. Elaborate about hydrogen storage systems.

24. Discuss about ocean thermal electric conversion.

(OR)

25. Explain about geothermal power plants.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	NEURAL NETWORKS  AND FUZZY SYSTEMS			     Time : 3 hours

Subject Code:	12EE244										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define artificial neural network.

2.	What are the advantages of neural networks over conventional computers?

3.	A perceptron is ___________.

a.	a single layer feed-forward neural network with preprocessing.

b.	an auto-associative neural network.

c.	a double layer auto-associative neural network.

4.	Distinguish between a feed-forward network and a recurrent network.

5.	Define Back Propagation Network.

6.	What is the main purpose of Hopfield network?

7.	What are classical sets?

8.	Define fuzzification.

9.	What is automating the control of depth of anesthesia?

10.	List some of the applications of fuzzy logic control system.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain difference between supervised and unsupervised learning.

12.	Write the Perceptron learning rule.

13.	What are the four main steps in back propagation algorithm?

14.	List the defuzzification methods.

15.	State the purpose of Process Identification.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the analogy between biological and artificial neural network. With the help of suitable diagram discuss functioning of a simple artificial neuron.

(OR)

17.	Explain with proper diagrams McCulloch Pitt’s neuron model.

18.	Discuss the working of single layer perceptron and multilayer perceptron with relevant algorithm and compare them.

(OR)

19.	a.   	Explain perceptron convergence theorem. 										    (7)

b.	With neat diagrams, discuss linearly separable classifications.     					    (8)

20	Show how the Hopfield net may be used to maximize an objective function.

(OR)

21.	Explain Back Propagation training method along with its limitations.

22.	What are fuzzy sets? In detail, explain fuzzy set operations and properties of fuzzy sets.

[P.T.O.]

(OR)

23.	a. 	Explain the different types of membership functions used in fuzzification process.	    (8)	b. 	Discuss fuzzy rule based system.								    			           (7)

24. 	Explain the technique “fuzzy logic blood pressure during anesthesia” in a brief manner.

(OR)

25.	Explain how fuzzy logic is being implemented for image processing.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BASICS OF MEDICAL ELECTRONICS					      Time : 3 hours

Subject Code:	12EE251				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give two components present in the cell.

2.	What are the major ions present in cell?

3.	Define Relative Refractory Period.

4.	What are electrodes used for ECG measurement?

5.	Suggest two pressure transducers.

6.	How strain Gauges are used in Medical measurements?

7.	What is patient monitoring?

8.	What is micro shock?

9.	What are safety standards?

10.	Expand EEG and EMG.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the action potential and explain its periods.

12.	What are non invasive electrodes? Give examples.

13.	Give the block diagram representation for EMG measurement.

14.	Explain photo voltaic transducer.

15.	How respiration is measured?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the circulatory system of heart with a neat diagram.

(OR)

17.	a.	What are the functions of central nervous system?								    (6)

b.	How body temperature is measured?  Explain the electronics associated with it. 		    (9)

18.	a.	What are the factors to be considered when fixing up the bio electrodes? 			    (5)

b.	Explain the measurement EMG and how many electrodes are used.					  (10)

(OR)

19.	a.	Explain ECG machine. 														    (8)

b.	Explain about needle electrode.						 						    (7)

20.	a.	Explain Displacement sensor with diagram. 										    (7)

b.	What are the sensors used in temperature measurement? 							    (8)

(OR)

21.	a.	Explain photo voltaic transducer with diagram.				 					    (8)

b.	Describe the LVDT along with its characteristics.								    (7)

22.	a.	Explain the procedure to measure heart rate.	 									    (6)

b.	What are the functional blocks of blood pressure measurement? 					    (9)

(OR)

[P.T.O.]

23.	a.	Explain the respiratory system.													    (9)

b.	What is patient monitoring system? Explain.										    (6)

24.	What are the methods of accident prevention in hospitals?

(OR)

25.	a.	Describe an isolation monitor circuit providing electrical circuit.		 			  (10)

b.	Write a note on Area monitoring.						 						    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BASICS OF ELECTRIC AND HYBRID VEHICLE		      Time : 3 hours

Subject Code:	12EE252		 								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How to maintain the battery in a vehicle?

2.	Write the use of flywheel.

3.	List the battery parameters.

4.	What is the function of the spark plug?

5.	How do you vary the output of the solar plant?

6.	What is supply rail?

7.	What is back emf?

8.	Define Slip.

9.	List any two types of tyres.

10.	What is the function of aerial?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the function of a battery.

12.	Describe the use of batteries in hybrid vehicle.

13.	Write the function of Super Capacitor.

14.	Describe the use of DC motor in hybrid vehicle.

15.	Write the properties of luggage racks.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With a neat block diagram, explain the functions of hybrid vehicle. 				    (8)

b.	Briefly explain the historical milestones in the development of electrical vehicle.		    (7)

(OR)

17. 	Write brief notes on :

a.	Fuelled electrical vehicle.                                                         						    (8)

b.	Solar powered Vehicle.                                                             						    (7)

18. 	Briefly explain the significance and characteristics of temperature, heating and cooling needs of batteries in the electrical vehicle.

(OR)

19. 	Describe the operation of:

a.	Lead Acid Battery.															    (8)

b.	Lithium Battery.																    (7)

20. 	a.	Explain the operating principles of solar photovoltaic cell.                         			    (7)

b.	Describe the impact of wind power in the operation of electrical vehicle. 			    (8)

(OR)

21. 	Briefly explain the basic principles of hydrogen fuel cell with schematic diagram and methods 	to store the hydrogen.

[P.T.O.]

22. 	Describe the operation of:

a.	Induction Motor.		            													    (7)

b.	Switched Reluctance Motor.													    (8)

(OR)

23. With the aid of a schematic diagram, explain the role and significance of converters for 	Brushless DC motor and Switched Reluctance Motor.

24. 	a.	Describe the necessity of ancillary systems in the electrical vehicle. 				    (7)

b.	Explain the significance of power steering in electrical vehicle.        				    (8)

(OR)

25.	 Explain the fundamentals of electrical vehicle recharging and refueling in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BUILDING AUTOMATION								      Time : 3 hours

Subject Code:	12EE253		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the different types of transducers?

2.	Reason out the purpose of earthing.

3.	Define reactive power.

4.	Define Total Harmonic Distortion.

5.	Write the use of fire sensor.

6.	List any two merits of cables.

7.	What is the need of biometrics?

8.	Write the advantage of TCP networking.

9.	List the components of a safety system.

10.	Write the necessity of video management.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the functions of HVAC System.

12.	Describe the effect of power quality on energy consumption.

13.	Enumerate the best installation practices of safety system.

14.	Write the concept of generic model of security system.

15.	List the various future prospects of integrated systems.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the features and characteristics of building automation system. 				    (8)

b.	Describe the functions of temperature sensors and flow transducer. 					    (7)

(OR)

17. 	a.	Write the properties of valve and actuator.										    (8)

b.	Briefly describe the concept of the controller IO.									    (7)

18.	a.	Describe the role of metering in energy management system.						    (8)

b.	Write the importance of instantaneous and active powers.							    (7)

(OR)

19. 	a.	Explain the significance of energy conservation.									    (8)

b.	Describe the importance of energy saving.										    (7)

20.	a.	Explain the procedure for the detector placement for safety system. 				    (8)

b.	Describe the principles of fire extinguisher.										    (7)

(OR)

21.	 a.	With a neat block diagram, explain the function of fire alarm system. 				    (8)

b.	Enumerate the factors influencing in the selection of Cables.   						    (7)

22. 	a.	Explain the concept of antipass back in security system.							    (7)

b.	Describe the fundamentals of intrusion detection system.							    (8)

(OR)

[P.T.O.]

23.  	Write brief notes on:

a.	Basics of CCTV Camera.														    (8)

b.	Digital vs. Analog Recording. 													    (7)

24. 	With a neat block diagram, explain the energy management of integrated system.

(OR)

25. 	a.	Explain the challenges and benefits of integrated system.							    (8)

b.	Describe the basics of security in integrated system.								    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	GENERALIZED THEORY OF ELECTRICAL MACHINES																					Time : 3 hours

Subject Code: 	12EE306										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	With relevant diagram, explain the basic two pole representation of machine and primitive machine diagram of induction and synchronous machine.					(6)

b.	Obtain an expression for the electrical power input into the electrical parts of the machine and electrical torque equation of the Kron’s primitive machine. Show that no torque is produced by interaction between the flux and current on the same axis.	      (14)

2.	a.	Deduce Park’s transformation relating the 3 phase currents of a synchronous machine to 	its corresponding d-q axes currents. Express 3 phase currents in terms of d-q axes 	currents and its inverse. Describe, from physical concepts, how these transformations can 	be used for a poly phase Induction Motor.      								      (14)

b.	A 2 pole synchronous machine has 3 phase armature currents as under:

ia=Im coswt, ib=Im cos(wt-120), ic=Im cos(wt-240)

At time t = 0, the rotor phase–a axis is aligned with the field winding axis. Find the direct and quadrature-axes current components.									(6)

(OR)

3.	a.	Derive the transformation for currents between a rotating balanced 2-phase (α,) 	winding and a pseudo-stationary two phase (d, q) winding. Assume equal turns on all 	coils. Show that the transpose of current transformation matrix is equal to its inverse. 																			      (10)

b.	A 3-phase, 4 pole 60Hz squirrel cage induction motor develops a torque of 300 Nm at rated phase voltage of 240V. The three phase stator winding is now replaced by 2-phase stator winding for which the numbers of poles, effective number of turns per phase and conductor size are not changed. Will the induction motor be able to operate? If so, compute:

i.	the rated phase voltage of the two phase induction motor

ii.	the torque, if the two phase induction motor is energized at rated phase voltage 	obtained in part (i), from a 2 phase 50Hz supply system. Ignore the losses.

iii.	the rating in kW for both 3-phase and 2-phase induction motors.			      (10)

4.	a.	A dc shunt motor is to be brought to rest by dynamic braking method, which involves 	disconnecting the armature terminals from the supply and connecting it to a resistor R, 	the field current remaining unaffected.

i.	Obtain an expression for motor load. Solve it for negligible armature inductance.

ii.	Find the value of R so that the speed reduces to zero in minimum possible time. (10)

b.	With relevant expressions, explain the dynamic behavior of separately excited dc motor after sudden application of Moment of Inertia with respect to speed, armature current and energy loss.			  													      (10)

(OR)

[P.T.O.]

5.	a.	A separately excited dc generator running at 4500/π rpm has the following parameters. 	Rf = 80Ω, Lf = 40H, Ra = 0.1Ω, La = 0.3mH, Motional inductance Md = 0.8H, generated 	emf constant Kg = Mdwr  = 120 volts/field amp.

i.	When the field is unexcited and armature is open, find the armature voltage as a function of time and sketch it, if a constant voltage of 160 volts is suddenly impressed across the field terminals.

ii.	Sketch the rise of armature current in part (i), if the armature terminals are initially short circuited.

iii.	Armature voltage has attained the steady state value in part (i). Now the armature is suddenly connected to a load of resistance 1.1 Ω in series with an inductance of 1.7mH. Determine the armature current and the armature terminal voltage as function of time.														      (14)

b.	Obtain the expression for the armature current of separately excited dc generator after sudden short circuit.					  										(6)

6.	a.	Draw the two phase equivalent circuit of a three phase induction machine.			(6)

b.	With relevant generalized mathematical model of a poly phase induction machine. Derive the expressions for the voltage and electromagnetic torque equations.	      (14)

(OR)

7.	An induction motor has following parameters:

5HP, 200V, 3-phase, 60Hz, 4-pole, star connected, Rs = 0.277Ω, Rr = 0.183Ω, Lm = 0.0538H, Ls = 0.0553H, Lr = 0.056H, effective stator to rotor turns ratio, a = 3.

The motor is supplied with its rated and balanced voltages. Find the q and d axes steady state voltages and currents and phase currents Iqrr, Idrr, Iα and Iβ when the rotor is blocked. Use the stator reference frames model of the induction machine.

8.	What are the various basic parameters of a synchronous machine? Derive expressions for armature to field mutual inductances and armature self mutual inductances for a salient pole synchronous machine. How are these inductance expressions modified for a cylindrical rotor synchronous machine? What is the advantage of resolving armature mmf into d-q axes components?

(OR)

9.	a.	A basic 3-phase, 2-pole synchronous machine of the salient pole type has the following 	inductances, where  is the angle measured from d-axis of phase A.

Phase A self inductance, La=0.9 + 0.2cos 2

Mutual between phase A and field winding F, Maf = 8 cos

Mutual between phases B and C, Mbc = 0.4 + 0.2 cos 2

i.	Write expressions for all the self and mutual inductances in terms of

ii.	Determine Ld, Lq, Lo.													      (12)

b.	With relevant expressions, obtain the voltage-current relationship in d-q reference frame 	applicable to both transient and steady state analysis.								(8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	BIOMASS ENERGY									Time : 3 hours

Subject Code: 	12EE321								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Discuss the classifications of biomass resources.									(6)

b.	Briefly explain the biomass conversion process and various types of biogas plants.    (14)

2.	Explain the power generation techniques from fermentation and gasification.

(OR)

3.	a.	Briefly discuss about anaerobic digestion.                              						(8)

b.	Explain the hydrogen generation process from algae. 							      (12)

4.	Discuss the biogas production from dry dung cakes with industrial applications.

(OR)

5.	a.	Explain the wood gasifier power generation system. 			            			      (10)

b.	How is biomass production done from various organic wastes?					      (10)

6.	Discuss the various environmental aspects and issues of bio energy conversion.

(OR)

7.	a.	Briefly explain the energy and cost effectiveness.                              		             (10)

b.	How to produce bio energy economically? Explain.							      (10)

8.	Discuss the design and selection process of biogas plants with construction.

(OR)

9.	Briefly explain:

a.	Optimal design of fixed dome digester.	 					      				      (10)

b.	Production of electricity from biomass.										      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	SIMULATION OF POWER ELECTRONICS SYSTEMS

Time : 3 hours

Subject Code: 	12EE325										  Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Write short notes on classification of simulation programs.					      (10)

b.	Discuss in detail the mathematical modeling of power electronic systems. 		      (10)

2.	a.	What do you mean by m –file. Explain the features of m-file.                          	      (10)

b.	Describe the AC voltage controllers in detail. 				                                   (10)

(OR)

3.	a.	Explain in detail the trigonometric functions in MATLAB programming.                   (10)

b.	Write short notes on Graphics -2D plots.                    					                     (10)

4.	a.	Discuss the effects of sources and sinks model analysis using SIMULINK.	              (10)

b.	Write short notes on power electronics library.      	                                                  (10)

(OR)

5.	Explain the operations of performing Harmonic analysis using the FFT tool.

6.	Describe in detail the simulation of PWM inverters.

(OR)

7.	Discuss briefly on BLDC and SRM.

8.	Discuss briefly on Resonant Pulse inverter.

(OR)

9.	Write shorts notes on following:

a.	Zero current switching inverter 											      (10)

b.	Zero voltage switching inverter.									        	      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	WIND RESOURCE ASSESSMENT AND FORECASTING METHODS																		Time : 3 hours

Subject Code: 	12EE345										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	Discuss in detail about monitoring station instrumentation and the various measurement 	parameters.																      (10)

b.	Give the format of a ‘Sample Site Visit Checklist’ to be used during operation and

maintenance of a wind monitoring station.  Discuss the significance of station operation and maintenance.															      (10)

(OR)

2. 	a.	What is data validation? Illustrate the sequence and roles of the data validation process in 	a wind monitoring station.													      (10)

b.	Elaborate on the guiding principles of wind resource assessment.				      (10)

3. 	a.	What are the different categories of forecasting methods?  List the basic steps in a 	forecasting task.															      (10)

b.	Compute the following univariate statistics for the observed wind speed data (OBS) in Table 1: i) Mean Absolute Deviation (MAD), ii) Mean Squared Deviation (MSD),

iii) Standard Deviation (SD), iv) Autocovariance lag 1 and v) Autocorrelation lag1.   (10)

Table 1

| Date | Time (in minutes) | Observed Wind Speed (OBS) in km/h | Forecast Wind Speed (FST) in km/h |
| --- | --- | --- | --- |
| 2006/01/01 | 00:10 | 14.031 | 14.885 |
| 2006/01/01 | 00:20 | 14.12 | 15.056 |
| 2006/01/01 | 00:30 | 14.001 | 14.943 |
| 2006/01/01 | 00:40 | 13.657 | 14.566 |
| 2006/01/01 | 00:50 | 13.605 | 14.488 |
| 2006/01/01 | 01:00 | 13.67 | 14.592 |
| 2006/01/01 | 01:10 | 13.833 | 14.779 |
| 2006/01/01 | 01:20 | 14.145 | 15.142 |
| 2006/01/01 | 01:30 | 14.379 | 15.407 |
| 2006/01/01 | 01:40 | 14.495 | 15.552 |
| 2006/01/01 | 01:50 | 14.627 | 15.703 |
| 2006/01/01 | 02:00 | 14.47 | 15.545 |

(OR)

4. 	a.	A forecasting method is employed to obtain the wind speed forecast (FST) values 		(column 4 of Table 1).  Measure the suitability of the forecasting method for the given 	wind speed data by computing the following statistical measures:i) Mean Error

Mean Squared Error   iii) Mean Percentage Error     iv) Mean Absolute Percentage Error  v) Theil’s U-statistic														      (10)

b.	Write short notes on: 	i) Transformations and Adjustments						(5)

ii) Graphical summaries. 														(5)

[P.T.O.]

5.  a.	The following data represents the average monthly wind speed data of a region.  Show that a 3x3 MA is equivalent to a 5-period weighted MA with weights of 0.111, 0.2222, 0.3333, 0.2222 and 0.1111 for the first, second, third, fourth and fifth terms respectively.																		      (10)

| Period | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wind speed (m/s) | 5.5 | 4.3 | 5.7 | 6.2 | 4.5 | 3.8 | 2.6 | 3.9 | 2.7 | 1.9 | 4.9 | 3.7 |

b.	Discuss the Census Bureau methods of decomposition.						      (10)

(OR)

6. 	a.	Compute the Durbin-Watson statistic for the residuals given in the table and comment on 	its significance:																(5)

| Time(t) | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Error(et) | -0.6 | -0.9 | -0.3 | 3.2 | -6.1 | -5.4 | 1.3 | -8.1 | -2.2 | 0.3 |

b.	Highlight the general aspect of smoothing methods. Elaborate on exponential smoothing 	methods.																      (10)

c.	Write a short note on forecasting and decomposition.								(5)

7. 	What is simple regression?  How are the values of the parameters obtained? Highlight on the different tests performed on the model and the individual coefficients. Discuss in detail how simple regression is used for forecasting.

(OR)

8. 	a.	Discuss in detail about the Box-Jenkins methodology for examining correlations and 	stationarity of time series data.												      (10)

b.	Elaborate on forecasting with ARIMA models.								      (10)

9.	Compulsory:

a.	“Complicated forecasting models generally outperform simpler models.” Is the statement true? Justify your answer.														(2)

b.	What are dynamic regression models? Discuss on the basic forms of the model, its modeling procedure and forecasting.											(8)

c.	Do you think non-linear models are required for time-series forecasting? Why? Discuss in detail about any one non-linear forecasting model.							      (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	SOLAR CELL AND MODULE TECHNOLOGY			Time : 3 hours

Subject Code: 	14EE3051										  Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	With arc furnace explain production of metallurgical grade Si.					      (10)

b.	Describe various losses in solar cells.										      (10)

2.	a.	State the requirements of silicon for crystalline solar cell.						      (10)

b.	Summarize the model of solar cell.											      (10)

(OR)

3.	a.	Explain energy band diagram and potential of P-N junction.					      (10)

b.	Describe production of semiconductor grade silicon.							      (10)

4.	a.	With a diagram explain low cost silicon feed stocks as a function of impurity level.  (10)

b.	Describe refinding processes for solar grade Si.								      (10)

(OR)

5.	a.	With a block diagram explain various routes of purification and usage of silicon in the 	form of wafers and thin films.												      (10)

b.	Describe silicon feed stock for solar cell industry.							      (10)

6.	a.	Explain wafer cleaning and texturing in commercial silicon cell technology.		      (10)

b.	Describe the detailed process in fabricating the commercial solar cell.			      (10)

(OR)

7.	a.	Discuss parasitic junction removal in process of commercial silicon cell.		      (10)

b.	Discuss diffusion process of P-N junction in commercial silicon cell technology.	      (10)

8.	Explain the operation of Dye-sensitized solar cell and list the materials used with its 	properties.

(OR)

9.	Describe organization and structure of organic solar cell and state its commonly used materials.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRIC CIRCUIT ANALYSIS						     Time : 3 hours

Subject Code:	EE201										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	If two 10Ω resistors are connected in parallel, then the effective resistance will be _________.

2. 	Power = _________ / Resistance.

3. 	In a three phase circuit, active power = _________.

4. 	Define series resonance.

5. 	Define mutual inductance.

6. 	Give the maximum value of coefficient of coupling (k).

7. 	Draw Norton’s equivalent circuit.

8. 	State reciprocity theorem.

9. 	Define transient period.

10. 	What is the time constant of RC circuit?

PART – B  (5 x 3 = 15 MARKS)

11. 	Two inductance values 100mH and 150mH are connected in series. What is the effective

inductance value?

12. 	Give the expressions for Bandwidth and Q factor.

13. 	State dot rule in magnetic circuit.

14. 	State superposition theorem.

15. 	List the applications of transient analysis.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	State Kirchhoff’s current law. Give an example. 									    (5)

b.	Find the equivalent resistance across AB for the network given. 					  (10)

(OR)

17.	a.	State Kirchhoff’s voltage law. Give an example. 									    (5)

b.	Calculate:	i.	current through resistors 		ii.	value of unknown resistance X

iii.	equivalent resistance across AB. 											  (10)

[P.T.O.]

18. 	Derive the impedance, active power, reactive power and power factor of pure resistance circuit.

(OR)

19. 	Prove that only two watt meters are enough to measure three phase power.

20. 	Obtain the current through 15 ohm and power delivered to it using mesh method.

(OR)

21. 	Find the voltage across 5Ω resistor for the coupled circuit given.

22. 	Obtain the value of resistor ‘R’ for maximum power transferred to it. Also find maximum 	power.

(OR)

23. 	Calculate current through 5Ω resistor using Thevenin’s theorem.

24. 	A capacitor of 2μF with an initial charge Qo=200×10-6 C, is connected across the terminals of a 500Ω resistor at t = 0. Calculate the time in which the transient voltage across the resistor drops from 60V to 20V.

(OR)

25.	 For the circuit given the switch ‘S’ is in position 1 till steady state conditions are reached and then moved to 2. Find the energy dissipated in the two resistors. Show that this is equal to the energy stored in the capacitor before moving the switch.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRIC CIRCUITS									      Time : 3 hours

Subject Code:	EE245										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Kirchoff’s current law.

2.	What is the equivalent resistance when two resistances R1 and R2 are connected in parallel?

3.	Define RMS Value.

4.	Define Power Factor.

5.	What is meant by dependent and independent sources?

6.	Loop or mesh analysis is based on ____________ law.

7.	Convert the following Current Source into Voltage Source.

10k

10A

8.	State the Thevenin’s theorem.

9.	Define Q factor.

10.	State the condition for Resonance in RLC Series Circuit.

PART – B  (5 x 3 = 15 MARKS)

11.	Convert the circuit to its equivalent star.	 Take all resistances in ohms.

12.	Find the form factor of the half wave rectified sine waveform shown below.

13. 	Write the mesh current equations in the circuit shown below and determine the currents.

10V

50V

5

10

2

+

+

-

-

[P.T.O]

14.	State Maximum Power Transfer Theorem and derive the condition at which the Maximum Power is transferred from source to load.

15.	Define selectivity.

PART – C  (5 x 15 = 75 MARKS)

16.	Define the following:

a.	Electric current.		b.	Electric potential.			c.	Potential difference.

d.	Resistance.			e.	conductance.

(OR)

17.	Calculate the effective resistance between points A and B in the given circuit.

10



j30

R

L

I

v(t)

18.	A voltage of v(t) = 50 sin(ωt+30o) is applied as shown in figure. Calculate the true power, reactive power and power factor. Also draw the phasor diagram.

(OR)

19.	Derive the expression for RMS value, average value, form factor and peak factor of sinusoidal voltage waveform.

20

15

5

5

30

100V

100V

-

+

+

-

20.	Determine the power dissipation in the 30 ohm resistor of the circuit shown below using mesh analysis.

(OR)

21.	Determine the current through each resistance using Nodal analysis for the circuit shown in figure.

22. 	Determine the current in 5ohm resistor, using superposition theorem.

(OR)

23.	Use Thevenin theorem to find the current in 3 ohm resistor in the figure shown below. Also draw the Thevenin’s equivalent circuit.

24.	Derive the expression for resonance condition in RLC parallel circuit.

(OR)

25.	A coil of inductance 10H and 10 ohm resistance is connected in parallel with 100pF capacitor. The combination is applied with a voltage of 100V.

Determine:

a.	the value of resonant frequency.

b.	the current at resonance frequency.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	NETWORK THEORY									      Time : 3 hours

Subject Code:	EE246										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the relation between phase voltage and line voltage in the star connected load?

2.	Three equal resistances of 5 Ω are connected in a delta circuit. Obtain the equivalent star 	circuit.

3. 	Find the coefficient of coupling of two coils having L1 = 2 H, L2 = 8 H, M = 3 H?

4.	Define coupled circuits.

5.	What is the time constant for an RC circuit?

6.	Give the expression for Laplace transform for a function f(t) which is to be continuous and defined for values of t≥0.

7.	Differentiate active port and passive port.

8.	Give the classification of filters.

9.	For a positive real function, the real parts of the driving point functions Z(s) and Y(s) are positive or zero. (True or False).

10.	What is a Hurwitz polynomial?

PART – B  (5 x 3 = 15 MARKS)

11.	A 440V, 3Φ, 50Hz balanced supply is applied to a balanced delta connected load of  2+j3Ω   per phase .The phase current is 10 A. Find the total active power.

12.	Two 200µH coils have a mutual inductance of 40µH.  Determine the total inductance when they are connected in series aiding connection.

13.	A series RL circuit with R = 100Ω and L = 20H has a DC voltage of 200 V applied through a switch at t = 0. Find the expression for current.

14.	Define decibel and Neper.

15.	What are the properties of positive real functions?

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	The phase voltage of a star connected three phase ac generator is 230V. Calculate the

i.	Line voltage

ii.	Active power output if the line current of the system is 15A at a power factor 0.7

iii.	Active and reactive components of the phase currents.						    (7)

b.	Discuss with a neat diagram, the three phase power measurement using three watt meters.																						    (8)

(OR)

17.	Derive the expressions for three phase reactive power and power factor using two watt meter method for star connected load.

18.	Derive an expression for equivalent inductance in an inductively coupled parallel aided circuit and parallel opposing circuits.

(OR)

19. 	Derive the expression for maximum output current and output voltage for a double tuned coupled coil.

[P.T.O.]

20.	Obtain the sinusoidal response of RC circuit.

(OR)

21. 	In how many seconds after t = 0 has the current i(t) become one half of its initial value in the given circuit shown.

22.	Find the z parameters of the circuit shown below.

(OR)

23.	Design a low pass filter (both π and T-sections) having a cut-off frequency of 2 kHz to operate with a terminated load resistance of 500Ω.

24.	The driving point impedance of a one port reactive network is given by

Zs=5s2+4s2+25ss2+16

Obtain the first and second Foster networks.

(OR)

25. 	Check the positive realness of the given functionZ(s) = s + 3s + 1.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRON DEVICES									      Time : 3 hours

Subject Code:	EE247		 								       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is a clipper?

2. 	How does the reverse saturation current of a p-n junction diode vary with temperature?

3. 	How must the two transistor junctions be biased for proper transistor amplifier operation?

4. 	What is the relationship between α and β in the transistor?

5. 	Mention any device which is having high input impedance.

6. 	A certain JFET has a gm = 4 ms.  With an external ac drain resistance of 1.5 kΩ, what is     	the ideal voltage gain?

7. 	Draw the schematic symbols for both the n-channel and the p-channel depletion MOSFETs.

8. 	If the gate-to-source voltage in an n-channel E-MOSFET is made more positive, does the drain current increase or decrease?

9. 	What is the typical dimension of a wafer diameter?

10. 	Mention any three IC packages.

PART – B  (5 x 3 = 15 MARKS)

11.	Determine vo for the network of the figure for the input indicated.

12.	Design a collector-to-base bias circuit in which VCE = 4V and IC =  22mA. The supply voltage is 18V and the transistor hfe is 90.

13.	Define the saturation current and pinch-off voltage for a JFET, and state typical values for        	each quantity.

14.	Draw a cross-section diagram for an enhancement-mode MOSFET. Label all parts of the 	device and explain its operation.

15.	Why buried layer is used in integrated circuits?

PART – C  (5 x 15 = 75 MARKS)

16.	Sketch the voltage /current characteristics for a PN junction under forward and reverse biasing.

Show how temperature change affects the characteristics.

(OR)

17. 	Discuss the different types of junction breakdown that can occur in a reverse-biased diode. Sketch the circuit symbol for Zener diode, and briefly explain Zener diode operation.

[P.T.O.]

18.	Explain the configuration of Common Base and Common Emitter of transistor operation.

(OR)

19.	A collector-to-base bias circuit as in the figure has RB = 270Kohm, RC = 2.2Kohm, VCC = 18V, and a transistor with hFE = 100. Analyze the circuit to determine IB, IC and VCE. Calculate the maximum and minimum levels of IC and VCE for the given bias circuit when hFE(min)=50 and hFE(max)=200. Sketck the circuit Q points. Discuss the effect of hFE(min) and hFE(max) .

20.	Explain the working operation of n-channel p-channel JFETs and its output characteristics.

(OR)

21.	With a neat sketch explain about the biasing of JFET.

22.	Sketch typical drain and transfer characteristics for an n-channel DMOSFET, n-channel        	EMOSFET and explain its operation.

(OR)

23.	Explain the biasing of MOSFET.

24.	Describe the various ways for making diodes in a Monolithic IC technology.

(OR)

25.	Discuss the various methods used for fabricating IC resistors and compare their performance.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTROMAGNETIC FIELDS						            Time : 3 hours

Subject Code:	EE248										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A vector quantity has both ____________ and ____________ in a space.

2.	The dot product of two vectors A and B is represented as __________.

3.	Write the expression for electric field intensity (E).

4.	Define Gauss’s law.

5.	Relate J and H.

6.	Write Lorentz force equation.

7.	Write point form of Maxwell’s equation.

8.	Write the expression for generated emf.

9.	What is meant by electromagnetic waves?

10.	For a perfect dielectric the value of σ is ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Given A= 2ax + 4ay – 3az and B=ax – ay, Find A.B and A X B

12.	Write Poisson’s and Laplace’s equation.

13.	An air –core solenoid of 300 turns and length 0.5m has a single layer of conductors at a radius of 0.02m. Find the value of inductance (L).

14.	State Displacement current and Displacement current density in a static field.

15.	What is meant by Poynting vector?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the differential elements in cylindrical co- ordinate system with relevant diagram.

(OR)

17.	Derive the differential elements in Spherical co- ordinate system with relevant diagram.

18.	Four concentrated charges are located at the vertices of a plane rectangle as shown in the figure. 	Find the magnitude and direction of resultant force on Q1 using Coulomb’s law.

Q1 = 0.2

Q2 = 0.1

Q3 = - 0.2

Q4 = 0.1

2 cm

5 cm

(OR)

19.	Explain the boundary conditions at the interface of two dielectrics.

[P.T.O.]

20.	a. 	Briefly explain about Biot-Savart’s law.					     						  (10)

b. 	Find the capacitance of a parallel-plate capacitor containing two dielectrics, ξr1= 1.5 and 			ξr2= 3.5 each comprising one-half the volume, as shown in figure. Here A = 2m2 and

d = 10-3m. 								      							    		    (5)

ξr1

ξr2

A

(OR)

21.	Derive the boundary relations for magnetic fields.

22.	State and derive the expression for Faraday’s law.

(OR)

23.	Express the Maxwell’s equation in integral and differential forms.

24.	Discuss the propagation of electromagnet waves in a conducting conductor, perfect di-electric and for good conductor.

(OR)

25.	State and prove Poynting’s theorem.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DC MACHINES AND TRANSFORMERS				      Time : 3 hours

Subject Code:	EE250										      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What do you mean by residual flux in DC generator?

2.	The emf induced in the armature conductors of a DC Generator is in the shape of __________.

3.	Distinguish between DC generator and DC motor.

4.	How will you change the direction of rotation of a DC motor?

5.	Mention two types of constructions used in a transformer.

6.	List two applications of a transformer.

7.	Can the voltage regulation of a transformer go to negative? If so, under what condition?

8.	What are the functions of no-load current in a transformer?

9.	Mention four principal applications of transformers.

10.	What are the various kinds of 3 phase transformer connections?

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by self excited and separately excited DC generator?

12.	State the advantages of Swinburne’s test. Is it possible to conduct this test on DC series motor? Justify.

13.	The efficiency of a transformer is always higher than that of rotating electrical machines. Why?

14.	What is the purpose of conducting Short-circuit test on a transformer?

15.	What is an instrument transformer?

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat schematic diagram, explain the construction and working of a DC generator.

(OR)

17.	a. 	A 4-pole, lap-wound, DC shunt generator has a useful flux per pole of 0.07Wb. The 	armature winding consists of 220 turns each of 0.004 ohm resistance. Calculate the 	terminal voltage when running at 900 rpm, if the armature current is 50A.                      (7)

b. 	What is armature reaction? Mention the effects of armature reaction.                      	    (8)

18.	a. 	Explain the principle and operation of DC motor.							 		    (9)

b. 	Derive the torque equation of a DC motor.										    (6)

(OR)

19.	Describe with the aid of a circuit diagram the Hopkinson’s test for a pair of DC shunt machines. State the advantages and disadvantages of this method.

20	a.	Derive the emf equation of a transformer.

b.	Draw the equivalent circuit of a transformer and write down its equations.

(OR)

21. 	a. 	A 25-kVA transformer has 500 turns on the primary and 50 turns on the secondary 	winding. The primary is connected to 3000V, 50 Hz supply. Find the full-load primary and 	secondary currents, the secondary emf and the maximum flux in the core. Neglect leakage 	drops and no-load primary current.   											    (7)

[P.T.O.]

b. 	Define all-day Efficiency of a transformer. What are the various losses of the transformers?																					    (8)

22.	With a neat diagram explain how back – to – back test is conducted on two identical single phase transformers. Mention the merits and demerits of this test.

(OR)

23.	a. 	Explain the principle of operation of an auto-transformer.							    (8)

b. 	Derive the expression for saving of copper in using auto-transformer.				    (7)

24. 	Explain the working principle and connection diagram of potential and current transformers.

(OR)

25.	With a neat diagram explain the construction and working of 3-phase transformer. Mention the different 3 – phase transformer connections.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INDUCTION AND SYNCHRONOUS MACHINES        Time : 3 hours

Subject Code:	EE251				 					           Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the condition for maximum torque under both running and standstill condition?

2.	What is the disadvantage of DOL starter?

3.	What are the types of single phase induction motor?

4.	How many windings are there in a split phase single phase induction motor?

5.	Write the emf equation of synchronous generator.

6.	List the methods for finding the regulation of synchronous generator.

7.	What is synchronous speed of synchronous motor?

8.	What are synchronous condensers?

9.	Which test is conducted to find the values of Xd and Xq of an alternator?

10.	Which machine is called salient pole machine?

PART – B  (5 x 3 = 15 MARKS)

11.	What is induction generator?

12.	What is the use of shading coil in shaded pole induction motor?

13.	Compare salient pole rotor and cylindrical rotor of an alternator.

14.	Define the term hunting. What is the function of damper windings in synchronous machine?

15.	Write down the equation for reluctance power developed in a salient pole synchronous machine.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the method of speed control of three phase induction motor from stator side.

(OR)

17.	Draw neatly the typical speed torque characteristics of a polyphase induction motor and explain with the performance equation of three phase induction motor. Also compare squirrel cage and slip ring Induction rotor.

18.	Explain the construction and working principle of operation of shaded pole induction motor with speed torque characteristics.

(OR)

19.	a.	Using double field revolving theory, draw the equivalent circuit of a single phase 		induction motor at rest.												        (5)

b.	A 115V, 60Hz, 4 pole single phase induction motor is rotating in the clockwise 			direction at a speed of 1710 rpm. Determine its per unit slip:

i.	in the direction of rotation

ii.	in the opposite direction.

If the rotor resistance at standstill is 12.5 ohm, determine 	the 	effective rotor 			resistance in each branch.								      				      (10)

[P.T.O.]

20.	A 3.5MVA star connected alternator rated at 4160 volts at 50Hz has the open circuit

characteristics given by the following data.

Field current (Amps): 50       100     150     200     250     300     350     400      450.

EMF (volts)              :1620    3150   4160   4750   5130   5370   5550   5650    5750

A field current of 200A is found necessary to circulate full load current on short circuit of the Alternator. Calculate by:

a.	synchronous impedance method

b.	ampere turn method the full load voltage regulation at 0.8 power factor lagging

(OR)

21.	A 9 KVA, 208V,1200 rpm, three phase ,60 Hz, star connected alternator has a field winding resistance of 4.5 ohm. The armature winding impedance is 0.3 + j5 ohm/phase. When the generator operates at its full load and 0.8 pf lagging, the field winding current is 5 A. The rotational loss is 500 W. Determine the:

a.	voltage regulation 					b.	the efficiency of the generator

c.	the torque applied by the prime mover.

22.	Explain about the starting methods of synchronous motor.

(OR)

23.	a.	Compare synchronous and induction motor.

b.	Draw and briefly explain about ‘V’ curves and inverted ‘V’ curves of synchronous 	motor.

24.	A 70 MVA, 13.8 kV, 60 Hz, 3Ф star connected salient pole synchronous generator has Xd = 1.83Ω / phase and Xq = 1.21Ω / phase respectively. It delivers the rated load at 0.8 power factor lagging. The armature resistance is negligible. Determine:

a.	the voltage regulation 			b.	the power developed by the generator.

(OR)

25.	Explain the operation of a salient pole synchronous machine and also derive the expression for the no load generated EMF.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRICAL MACHINE DESIGN						      Time : 3 hours

Subject Code:	EE252										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define specific magnetic loading.

2.	What is leakage flux?

3.	Name the parameters that are affected by number of poles?

4.	Define commutator pitch.

5.	What are the advantages of using higher flux density in the core?

6.	Why is the efficiency of a transformer high?

7.	How the induction motor can be designed for best power factor?

8.	What are the factors to be considered for the choice of specific magnetic loading?

9.	What is runaway speed?

10.	What is the use of damper winding?

PART – B  (5 x 3 = 15 MARKS)

11.	What is the difference between real and apparent magnetic flux densities in rotating machines?                                                                                                                                                12.	What is the effect of inter pole on main pole?

13.	What are the advantages of three phase transformers over single phase transformers?

14.	Explain the phenomena of Cogging.

15.	What is short circuit ratio (SCR)?

PART – C  (5 x 15 = 75 MARKS)

16.	A 20HP, 440V, 4 pole, 50Hz, three phase induction motor is built with a stator bore of 0.25m and core length of 0.16m.  The specific electric loading is 23000 ampere conductors per metre.  Find the specific magnetic loading of the machine.  Assume the full load of 84% and a power factor of 0.82.  Using the data of the above machine, determine the main dimensions for a 15HP, 460V, 6 poles, 50Hz Motor.

(OR)

17.	a.	A 15kW, 230V, 4 Poles DC Machine has the following data: armature diameter = 0.25m, 	armature core length = 0.125m, length of air gap at pole centre = 2.5mm, flux per pole = 	11.7mwb, Calculate the mmf required for air gap:	i.	if the armature surface is treated 	as 	smooth 		ii.	if the armature surface is slotted and the gap contraction factor is 	1.18.       																	    (7)

b.	Find the permeability at the root of the teeth of a DC machine armature from the following data: slot pitch = 2.1cm, tooth width at the root = 1.07cm, gross length = 32cm, stacking factor = 0.9, real flux density at the root of the teeth = 2.25 tesla, apparent flux density at the root = 2.36 tesla.   															    (8)

18.	Determine the main dimensions, number of poles and the length of air gap of a 600kW, 500V, 900rpm DC generator.  Assume average gap density as 0.6wb/m2 and ampere conductors per metre as 35000.  The ratio of pole arc to pole pitch is 0.75 and the efficiency is 91%.  The following are the design constraints:

[P.T.O.]

Peripheral speed should not exceed 40m/s, frequency of flux not higher than 50Hz, current per brush not greater than 400A and armature MMF per pole not greater than 7500A.  The MMF required for air gap is 50% of armature MMF and gap contraction factor is 1.15.

(OR)

19.	The following particulars refer to the shunt field coil for a 440V, 6 pole, DC generator. MMF per pole = 7000A, depth of winding = 50mm, length of inner turn = 1.1m, length of outer turn = 1.4m, loss radiated from outer surface excluding ends = 1400W/m2, space factor = 0.62, resistivity = 0.02Ω/m and mm2.  Calculate:

a.	diameter of wire 		b.	length of coil 	c.	number of turns 		d.	exciting current.  Assume a voltage drop of 20% of terminal voltage across the field regulator.

20.	Calculate approximate overall dimensions for a 200kVA, 6600/440V, 50Hz, three phase core type transformer.  The following data may be assumed: Voltage per turn = 10Volts, maximum flux density = 1.3wb/m2, Window space factor = 0.3, Current density = 2.5A/mm2, Sf = 0.9, overall height = overall diameter. Use a 3 stepped core.  For a three stepped core: width of largest stamping = 0.9d and Net iron area = 0.6d2 where d is the diameter of the circumscribing circle.

(OR)

21.	A 250kVA, 6600/400V, three phase core type transformer has a total loss of 4800W at full load.  The transformer is 1.25m in height and 1m X 0.5m in plan.  Design a suitable scheme for tubes if the average temperature rise is to be limited to 350C.  The diameter of tubes is 50mm and is spaced 75mm from each other.  The average height of tubes is 1.05m. The specific heat dissipation from the tank walls is 6W/m2-°C and 6.5W/m2-°C due to radiation and convection respectively.  Assume that the convection is improved by 35%.

22.	A 15kW, 440V, 4 pole, 50Hz, three phase induction motor is built with a stator bore of 0.25m and core length of 0.16.  The specific electric loading is 23000ampere conductors per meter.  Using the data of this machine, determine the core dimensions, number of stator slots and number of stator conductors for a 11kW, 460V, 6 pole, 50Hz motor.  Assume a full load efficiency of 84% and power factor of 0.82 for each machine.  The winding factor is 0.955.

(OR)

23.	A 90kW, 500V, 50Hz, 8 poles Induction motor has a star connected stator windings accommodated in 63 slots with 6 conductors per slot.  If the slip ring voltage on open circuit is to be about 400V, find a suitable rotor winding.

24.	The following is the design data available for a 1250kVA, three phase, 50Hz, 3300V, star connected, 300rpm, alternator of salient pole type: Stator bore D = 19m, Stator Core length L = 0.335m, pole arc/pole pitch = 0.66, turns per phase = 150, single layer concentric winding with 5 conductors per slot, SCR = 1.2. Assume that the distribution of gap flux is rectangular under the pole arc with zero values in the interpolar region.  Calculate:

a.	Specific magnetic loading 		b.	armature MMF per pole 		c.	gap density over pole arc 		d.	air gap length.  MMF for air gap is 0.88 of no load field MMF and the gap contraction factor is 1.15.

(OR)

25.	Estimate the diameter, core length, size and number of conductors, number of slots for stator of a 15MVA, 11kV, 50Hz, 2 pole star connected turbo alternator with 60° phase spread.  Assume: Bav = 0.55wb/m2, ac = 36000A/m, current density = 5A/mm2, peripheral speed = 160m/s.  The winding should be arranged to eliminate 5th harmonic.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	POWER ELECTRONICS								      Time : 3 hours

Subject Code:	EE254										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	_________ is a current controlled device.

2.	Cut in voltage is also known as _________.

3.	Define firing angle.

4.	Free wheeling diode is also called _________.

5. 	Define cycloconverter.

6. 	_________ is a static device that converts fixed alternating voltage directly to a variable alternating  voltage without change in frequency.

7. 	Define Self Commutation.

8.	The ratio of  Vr/Vc is called _________.

9. 	List out the basic types of HVDC transmission systems.

10. 	List out the two types of Static UPS Systems.

PART – B  (5 x 3 = 15 MARKS)

11.  	Draw the equivalent circuit of MCT.

12.  	Define uncontrolled rectifier.

13.  	List out the applications of ac voltage controller.

14.  	Enumerate the requirements of a good inverter.

15.  	Draw the block diagram of short break static UPS system configuration	.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the structure, operation, switching characteristics of the Power MOSFET.	  (10)

b.	Give a comparison between an IGBT and MOSFET.								    (5)

(OR)

17.	Draw and explain the operation and equivalent circuit of SCR.

18.	Draw and explain the operation of a single-phase half wave rectifier with R load with necessary waveforms. Also, derive the average and RMS output voltage of the rectifier.

(OR)

19.	Explain the operation of a dual converter with a neat circuit diagram and waveforms.

20.	Describe the working of type-E chopper with relevant circuit diagrams and its operation in all the four quadrants.

(OR)

21.	Describe the basic principle and working of single-phase to single-phase step-up cycloconverter for both continuous and discontinuous conductions for a bridge type cycloconverter with a sketch of the waveforms.

22.	Explain the operation of three phase VSI bridge inverter in 180 degree mode conduction with necessary circuit diagram and waveforms. Derive the expression for the RMS value of phase voltage and line voltage.

[P.T.O.]

(OR)

23.	Describe the working of a single phase basic series inverter with appropriate circuit and 	waveforms.

24.	a.	List the merits of HVDC transmission.											  (5)

b.	With a neat block diagram, explain the monopolar HVDC system.					(10)

(OR)

25.	a.	Describe in detail about the concept of no-break static UPS configurations.			  (9)

b.	Why are nickel-cadmium batteries preferred over lead-acid batteries in a UPS?		  (6)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	POWER SYSTEM ANALYSIS							      Time : 3 hours

Subject Code:	EE255		 								       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Number of branches in a tree is __________ than a number of __________ in a network graph.

2.	Define bus incidence matrix.

3.	Name the diagonal and off diagonal elements of bus admittance matrix.

4.	1 + a + a2 + a3 = ?

5.	NR method is based on ___________.

6.	What is the condition for economic load dispatch in a power system?

7.	Which load flow analysis method uses large number of buses?

8.	For a power system we can improve the steady state stability limit by __________.

9.	Define acceleration area.

10.	In transient stability analysis, as long as equal area criterion is satisfied, the maximum angle 	to which rotor angle can oscillate is less than 90 °. (True / False).

PART – B  (5 x 3 = 15 MARKS)

11.	Write down the property of Ybus.

12. 	Give short notes on types of faults occurring in power system.

13.	What are the constraints to be satisfied to solve ELD problem?

14.	What is critical clearing angle in power system stability?

15.	What is SMIB? Brief its significance.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Two generators rated at 10MVA, 11KV and 15MVA, 11KV respectively are connected in      		parallel to bus. The bus bar feed two motors rated 7.5MVA and 10 MVA respectively. The 		rated voltage of the motor is 9 KV. The reactance of each generator is 12% and that of 			each motor is 15% on their own ratings. Assume 50MVA, 10KV base and draw the 				reactance diagram.		 													  (10)

b.	Determine the symmetrical components of unbalanced three phase currents:  	  	    (5)          		Ia=100 A   Ib= 120 A and Ic= 10130A.

(OR)

17. 	Obtain the ZBus for the 3-bus sample system shown in below the figure.

[P.T.O.]

18.	The table shows a three bus power system with line impedance.

| From | To | Line Impedance |
| --- | --- | --- |
| 1 | 2 | 04.j |
| 1 | 3 | 03.j |
| 2 | 3 | 02.j |

Bus 1: Slack bus, 	V= 1.05 < 0 p.u

Bus 2: PV bus, 	V = 1.0 p.u PG = 3 p.u.

Bus 3: PQ bus, 	P1 = 4p.u, Q1 = 2 p.u. Neglect limits on reactive power 	generation. Carry out one iteration of load flow solution by Gauss Seidel Method.

(OR)

19.	Explain the step by step computational procedure for the Newton Raphson method in load 	flow studies with appropriate expression.

20. 	a.	For the power system shown in the figure by taking generator rating as base value, specify 		the base value of transmission line and motor.				  	       				  (10)

b.	Define per unit value of a quality. How will you change the base impedance from one set    		of values to another set? What are the advantages of per unit form of representation?    (5)

(OR)

21.	For the three bus network shown in the figure build Z-bus.

22. 	a.	Write short notes on need for system analysis in planning and operation of power system. 																					    (8)

b.	Derive the fault current expression for single line to ground fault occurs in a power system 		network.							   											    (7)

(OR)

23.	Draw the flowchart for load flow solution using Newton Raphson method and write down 	the relevant expression.

24. 	a.	Derive the swing equation for a single machine connected to an infinite bus system.  	  (8)

b.	State and explain the equal area criterion.						  				    (7)

(OR)

25. 	Brief about the following:

a.	Hydro thermal scheduling.														    (7)

b.	Unit Commitment.															    (4)

c.	Economic Load dispatch.														    (4)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	POWER SYSTEM PROTECTION AND SWITCHGEARS     Time : 3 hours

Subject Code:	EE256		 									       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Line to ground fault is a symmetrical fault. (True / False).

2.	What is a current limiting reactor?

3.	What is pick-up current?

4.	What is a feeder?

5.	What is a fuse?

6.	The methods used for arc extinction in circuit breaker are __________ and __________.

7.	Write the function of isolator.

8.	What is the purpose of Klydonograph?

9.	What is a circuit breaker?

10.	Define Surge impedance.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the merits of static relays?

12.	Describe lightning phenomenon.

13.	List out the essential qualities of protection relays.

14.	What are the types of circuit breakers?

15.	What is earthing?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Compare Current Transformer and Potential Transformer. 						    (7)

b.	With a neat sketch, explain the types of a current transformer. 						    (8)

(OR)

17.	Write short notes on types and effects of faults.

18.	Explain the principle and operation of an induction type directional over current relay with a 	neat diagram.

(OR)

19.	Describe with suitable sketches, the microprocessor based directional relay.

20.	Describe in detail, the construction and working of sulphur hexafluoride circuit breakers 	with a neat sketch. Also state its advantages.

(OR)

21.	Explain the testing of circuit breakers.

22.	Discuss the principle and working of a horn gap arrester.

(OR)

23. 	What is a surge absorber? Discuss in detail about Ferranti surge absorber.

[P.T.O]

24.	Write short notes on the following:

a.	Resistance earthing. 															    (8)

b.	Voltage transformers earthing.													    (7)

(OR)

25.	What is insulation co-ordination? Discuss in detail, how insulation co-ordination is achieved among various components and apparatus present in a sub-station.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	LINEAR INTEGRATED CIRCUITS						      Time : 3 hours

Subject Code:	EE258		 								       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The type of feedback used if external component is connected between output terminals and inverting input is __________.

2.	List the parameters  that is important for ac application.

3.	Show with circuit diagram an op amp used as a phase shifter.

4.	The frequency of oscillation of a Wein bridge oscillator is __________.

5.	What is a precision diode?

6.	Name the circuit that is used to detect the peak value of non sinusoidal waveform.

7.	What is the other name of one shot?

8.	What are the modes of operation of timer 555?

9.	Which is greater “capture range’ or ‘lock range’?

10.	How many resistors are required in a 12-bit weighted resistor DAC?

PART – B  (5 x 3 = 15 MARKS)

11. 	Why is frequency compensation required in operational amplifier?

12. 	What are the basic requirements of a good instrumentation amplifier?

13. 	Sketch the circuit of an op-amp employed as, a non-inverting zero crossing detector, along with input and output waveforms.

14. 	What is lock range and capture range of PLL? Write the equation to determine it
15. 	What is meant by resolution of an ADC? Which is the fastest ADC? Why?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain various DC and AC characteristics of an op-amp. Distinguish between ideal and practical characteristics.

(OR)

17. 	Draw and explain the schematic block diagram of the basic OPAMP. Explain the significance of virtual ground in basic inverting OPAMP. How would you explain its existence?

18.	Draw the circuit of instrumentation amplifier and derive the expression for output voltage for it. Also write the advantages of instrumentation amplifier.

(OR)

19. 	Draw the circuit of a second order Butterworth low pass filter and derive its transfer function.

20. 	Explain the applications of op amp.

a.	Schmitt trigger. 																    (8)

b.	Logarithmic amplifier.														    (7)

(OR)

21. 	a.	Derive the gain of inverting and non inverting amplifier.                                         	    (8)

b.	What is a sample and hold circuit? Briefly explain its construction and application.	    (7)

[P.T.O.]

22.	a.	With a neat diagram explain the variable transconductance technique in analog          	multiplier and give its output equation.                                                                   		    (8)

b.	What are important building blocks of phase locked loop (PLL)? Explain its working.(7)

(OR)

23.	With a neat circuit diagram and internal functional diagram explain the working of 555 timers in astable mode.

24. 	With neat diagram, explain the working principle of:

a.	R-2R ladder type DAC. 														    (8)

b.	Weighted resistor DAC.                                               							  	    (7)

(OR)

25.  Explain the working principle of dual slope type ADC and discuss its advantages.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CONTROL SYSTEMS									     Time : 3 hours

Subject Code:	EE259										       	      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define time invarying and time varying control system.

2.	When will be a closed system is unstable?

3.	Why derivative control is employed in isolation?

4.	Find the Transfer function of a system  if its impulse response is e−3tsin 2t.

5.	What is a Bode plot?

6.	Define gain crossover frequency.

7.	What is a Nyquist plot?

8.	Define lag compensator. In which filter it is essentially used?

9.	What are the properties of state transition matrix?

10.	Write the advantages of sampled data control system.

PART – B  (5 x 3 = 15 MARKS)

11.	Give the advantages  of block  diagram  representation.

12.	Draw the  signal flow diagram  for the equation

a.	y2=a1dy1dt				b.	y3=d2y2dt2+dt1dt−y1.

13.	What are Static velocity error coefficient kv constant and ka constant static acceleration error  coefficient?

14.	Determine the resistance  values  to  obtain  the  following  electronic PI controller  with  proportional  and integral gain of  2 and 0.04 respectively. Use 1µF capacitor.

15.	Write down the concepts of state and state variable.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw a block diagram of the signal flow graph shown in the figure. Find the overall  transfer  function  by block  diagram  reduction  technique. Verify the result by applying  Mason's gain formula.

(OR)

[P.T.O.]

17.	Obtain the transfer function of the mechanical system shown in figure and draw its analogous circuit.

18.	The overall transfer function of a unity feedback system is given by

CSRS=10s2+6s+10
Find:

a.	The position, velocity and acceleration error constants.

b.	The steady state error for the input r(t) = 1+ t + t2

(OR)

19.	A   feedback control system is shown in the figure. Determine the value of K so that the system will have a damping ratio of 6. Also for this value calculate maximum overshoot, setting  time  and steady state error  for a unit ramp input.

20.	Sketch the bode plot  and hence determine  the gain cross over  frequency  and phase cross over  frequency for the transfer  function of a system  represented by

GS=10S1+0.5S1+0.1S

(OR)

21.	Sketch  the polar plot of a system whose  transfer  function is given as

Gs=5ss+1s+2.

22.	a.	The  characteristic equation  of a feedback control  system  is found as:

S4 + 9S3 + 11S2 + 6S + k = 0. Determine the value of K for which the system is absolutely  	stable and marginally  stable. Also determine the frequency of sustained oscillation.

b.	What is the concept and conditions for stability?

(OR)

23.	Using the Routh-Hurwitz stability criterion, ascertain stability for each of the following cases,	a.	S6 + 2S5 + 8S4 + 12S3 + 20S2 + 16S = 0

b.	S6 + S5 + 2S4 – 3S3 – 7S2 – 4S – 4 = 0

24.	Consider the following transfer  function of  a system :YSUS=S+12S2+7S+12.

Obtain  the  state  space representation  of this  system in

a.	Controllable  phase variable  	 	b.	Observable phase variable form.

(OR)

25.	Consider the following system with differential equation given by

y+46+11 {y+6=6u

Obtain a state model of the given system in a diagonal canonical form.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	C++ AND DATA STRUCTURES							      Time : 3 hours

Subject Code:	EE261										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In a single linked list, the address field of the last node is ____________.

2.	True or False: Queue follows last in first out logic.

3.	List the basic searching techniques.

4.	Name a sorting technique that follows ‘Divide and Conquer’ rule.

5.	State the purpose of using a constructor.

6.	Write an if…else statement that displays ‘Yes’ if a variable ‘age’ is greater than 21, and displays ‘No’ otherwise.

7.	Name the conversion function used to convert the basic data type to user defined data type.

8.	Name the memory management operators.

9.	What is ‘This pointer’?

10.	List the C++ keywords for exceptions.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate Doubly linked list and circular linked list.

12.	Sort the following numbers using Bubble Sort technique {56, 25, 31, 42, 16, 14 }.

13.	Mention the difference between structures and classes.

14.	There are 3 base classes(A,B,C). A Derived class ‘D’ is derived from these three base classes. What is the type of Inheritance? Discuss.

15.	State the purpose of using Friend function.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Define a Node in linked list and draw the structure of a single linked list. 			    (3)

b. 	With a program explain the concept of adding 2 nodes, Appending 2 nodes and displaying  the same using Circular linked list.												  (12)

(OR)

17. 	a.	Write a C++ program to give an overview about the operation of stack.				  (10)

b.	Briefly discuss about Queue.													    (5)

18.  	a. 	Explain Selection Sort with an example program. 							          (10)

b. 	Give an account on simple Merge sort. 											    (5)

(OR)

19. 	a. 	Describe Linear search with an example program.								  (10)

b.  	Give an overview on Binary search.												    (5)

20.	a.	Write an object oriented program to maintain the student details with the following data 	members namely name, register number, age, sex, department, grade, comments, marks 	using the functions namely getdata and putdata and display all the data members.	  (10)

b.  	With a C++ code explain the concept of constructors.								   (5)

(OR)

[P.T.O.]

21.	a.	Write a program to explain the concept of returning objects from functions.			  (10)

b. 	Write a program to explain the concept of static class data.     						    (5)

22.  a.	Write an object oriented program to overload the “+” operator. 				  	    (8)

b. 	Write an object oriented program to maintain the hostel mess details with the following data members namely name, register number, age, sex, year of study, name of the hostel, type of food, using the functions namely getdata and putdata. Design a base class with the data member name, register number, age, sex, year of study and another base class with the data members name of the hostel, type of food. Design the derived class with the data member previous dues. Display the data members and the fees to be paid.			    (7)

(OR)

23.  	a. 	Explain in detail about the different types of Inheritance.        						    (8)

b. 	With an example program explain the concept of pointers.       						    (7)

24.  	a. 	Illustrate Static functions with an example program. 	           						    (7)

b. 	Differentiate the Normal member functions accessed with pointers and Virtual member

functions accessed with pointers with an example program.      					    (8)

(OR)

25.	a. 	Write short notes on Exceptions.												    (5)

b. 	With an example program explain function Templates.              					  (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MEASUREMENTS AND INSTRUMENTATION		            Time : 3 hours

Subject Code:	EE262				 							       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define static error.

2.	Write the formula for standard deviation.

3.	Why a voltmeter is connected in parallel?

4.   	What is the principle of an AC bridge that is used for measuring capacitance?

5.   	Wheatstone’s bridge is used to measure ___________.

6.    	What is meant by self balancing bridges?

7.    	What is LVDT?

8.    	Thermocouple is used to measure ___________.

9.	List two applications of multimeter.

10.	Define harmonic distortion.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between precision and accuracy for an instrument.

12.	What is meant by creping in energy meter?

13.	With a neat circuit diagram write the balanced equation of Wheatstone bridge.

14.	What are primary and secondary transducers?

15.	Mention the need for ADC and DAC in digital data acquisition systems.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	How the range of d.c ammeter and d.c voltmeter can be extended? Derive the expressions 	to calculate shunt resistance and multiplier resistance.                                                       (8)

b.	Develop the torque equation for a PMMC instrument and show its scale is linear.         (7)

(OR)

17.	a.	Explain the working principle of attraction type moving iron instrument.			    (8)

b.	Explain the different types of errors in measurements.		                                               (7)

18.  	With circuit and phasor diagram explain the working of a single phase energymeter.

(OR)

19.	a.	Describe how to obtain iron loss of a ring specimen using wattmeter.                               (8)

b.	How to measure the power using instrument transformers?                                               (7)

20.	a.	Define voltage. Calculate the unknown inductance and resistance measured by Hay’s 	bridge. The bridge elements at the balancing condition are R1=5.1kΩ, C1=2µF, 	R2=7.9kΩ, R3=790Ω and the supply angular frequency is 1200 rad/sec.				    (8)

b.	Draw a neat diagram of Kelvin Double bridge and explain how to measure low resistance.

(7)

(OR)

21.	a.	Discuss the measurement of capacitance using Schering bridge.                                  	    (8)

b.	Explain the measurement of earth resistance.                     				                          (7)

[P.T.O.]

22.	a.	Describe the construction and working of potentiometer type resistance transducer for 			measuring linear displacement.                                 								  (10)

b.	Explain the principle of strain gauge.                                                                            	    (5)

(OR)

23.	a.	Explain how to measure pressure using capacitive transducer.                                  	    (8)

b.	What are piezo-electric transducers? Explain its working with a diagram.                  	    (7)

24.	a.	List the various applications of  spectrum analyzer and describe its working.                 (8)

b.	Explain the construction and working principle of XY recorder.                                      (7)

(OR)

25.	Describe in detail about the digital voltmeter, multimeter and frequency counter.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MICROPROCESSORS AND MICROCONTROLLERS	     Time : 3 hours

Subject Code:	EE263											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In an 8085 based system, the crystal frequency is 6 MHz. What is the frequency of clock?

2.	Define instruction cycle.

3.	What is meant by synchronous data transfer?

4.	How many address lines in a 4096 x 8 EPROM chip?

5.	What is the use of DMA?

6.	What is the need of interrupt?

7.	List the features of ADC 0808.

8.	Differentiate multiplexed and non multiplexed display.

9.	Identify the addressing mode used by each of the following instructions.

a.  	MOV A, R4.

b.  	MOV A, # 30H.

10.	What is the feature of 8751?

PART – B  (5 x 3 = 15 MARKS)

11.	Write an 8085 assembly language program to subtract two 16 bit numbers without using DSUB instruction.

12.	Differentiate I/O mapped I/O and memory mapped I/O.

13.	Explain how to connect multiple interrupts and to determine the priorities.

14.	Explain about the measurement of frequency, phase angle and power factor.

15.	Draw format of SCON register. Explain different bits in it.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the classification of the instruction set of 8085 Microprocessor based on 	functions and based on addressing modes with suitable examples.

(OR)

17.	a.	With a suitable block diagram explain the architecture of 8085 microprocessor  and also 	explain each and every block in detail.	                                               				    (8)

b.	Draw the timing diagram of STA 8255H and explain each machine cycles.   	           (7)

18.	Design a memory interfacing system for the 8085 microprocessor such that it should 	contain 8KB of EPROM and 8 KB RAM.

(OR)

19.	Explain the internal block of 8255A and explain its operation with different modes. Explain how it is interfaced with 8085.

20.	Describe how the interrupt is performed in the processor and describe all the 8085 interrupts 	and its vector locations with a neat diagram.

(OR)

21.	Explain the DMA data transfer scheme and draw the architecture of 8237 DMA controller with explanation of features and interfacing with 8085 microprocessor.

[P.T.O.]

22.	a.	Interface 4-digit 7-segment LED display to 8085 and write an assembly language program 		to display 1234 on it.          													    (8)

b.	Draw and explain the typical interfacing circuit for DAC0800 with 8085 and write a 				program to generate sine waveform.											    (7)

(OR)

23.	a.	Draw and explain the typical interfacing circuit for ADC with 8085 Microprocessor. 	   (7)

b.	Draw and explain the stepper motor interface with the 8051 microcontroller and write an

assembly language program to rotate it 95o  with half step mode in clockwise direction.

(8)

24.	a.	With a neat diagram explain the timer / counter functions in 8051 Microcontroller.      (7)

b.	Explain all groups of instructions of 8051 in detail with minimum two examples in each

group.                                                                                                                                   (8)

(OR)

25.	a.	Draw the block diagram of 8051 Microcontroller and explain the functions of each 	block 	in detail.                                                                                                             	    	    (8)

b.	Draw and explain the memory structure in 8051.	                                                     	    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	COMPUTER COMMUNICATION						      Time : 3 hours

Subject Code:	EE269		 								       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List any two applications of computer networks.

2.	Point out the network components of existing networks.

3.	Define LAN.

4.	State the protocol used by Ethernet for access of communication links.

5.	Expand BISYNC.

6.	List the data communication techniques.

7.	Define IP address.

8.	Which protocol used for fetching e-mail from a remote mailbox?

9.	List out the limitations of ISDN protocol.

10.	Mention the size of ATM cell.

PART – B  (5 x 3 = 15 MARKS)

11.	Outline the concepts of a network protocol.

12.	List the various transmission media in LAN.

13.	Compare asynchronous and synchronous communication.

14.	IP addresses are hierarchial with net id and host id – Justify.

15.	Give a note on VSAT networks.

PART – C  (5 x 15 = 75 MARKS)

16.	Illustrate the OSI reference model and explain the function of various layers.

(OR)

17.	Analyze the basics of Queuing theory and also explain the Queuing models.

18.	Illustrate the function of various LAN topologies.

(OR)

19.	Explain the LAN access techniques and token ring and CSMA/CD  bus.

20.	With a neat sketch explain the concepts of asynchronous communication.

(OR)

21.	Analyze the concepts of error control coding and X 2.5 protocols.

22.	Illustrate the working of TCP/IP protocols.

(OR)

23.	Explain the following algorithms.

a.	Routing algorithms.

b.	Congestion Control algorithms.

24.	Outline the ATM concepts and principles.

(OR)

25.	Illustrate the transmission structure and operation of ISDN protocol.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	POWER SYSTEM CONTROL							      Time : 3 hours

Subject Code:	EE275				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What do you mean by real power?

2.	What do you mean by reactive power?

3.	Expand LFC.

4.	What is two area system modeling?

5.	What do you mean by tap changing transformer?

6.	Define synchronous condenser.

7.	Define SCADA.

8.	What is transition?

9.    Define base point.

10.	Define participation factor.

PART – B  (5 x 3 = 15 MARKS)

11.	Write a short note on system load characteristics.

12.	Explain the steady state instabilities.

13.	What is a typical excitation system?

14.	Write a short note on energy control center functions.

15.	Explain the incremental cost curve.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the real power and reactive power improvement methods in detail.

(OR)

17.	Define basic P-F and Q-V control loops in detail.

18.	Explain briefly the LFC control of single area.

(OR)

19.	Explain briefly the fundamentals of speed governing mechanisms.

20.	Discuss about stability compensation.

(OR)

21.	Explain briefly static VAR system.

22.	Explain the system hardware configuration SCADA system in detail.

(OR)

23.	Discuss the various controls for secure operation.

24.	Discuss the economic controller added to LFC control.

(OR)

25.	Explain the coordination equation with loss and without losses in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MATERIAL SCIENCE									      Time : 3 hours

Subject Code:	EE276										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a bond?

2.	Refractive index and reflectivity are the properties of material known as _________.

3.	What are the types of semiconductor?

4.	What is doping?

5.	Define dielectric break down.

6.	What do you mean by Ferro electricity?

7.	Define magnetic flux.

8.	What is reluctance?

9.	Mention some types of super conductors.

10.	What are nano materials?

PART – B  (5 x 3 = 15 MARKS)

11.	Define lattice energy of ionic crystal.

12.	What is meant by loosely packed crystal structure?

13.	Write short notes on photoelectric device and give some example.

14.	State some similarities between electrical and magnetic circuit.

15.	Write the expression for Ohm’s law of magnetism.

PART – C  (5 x 15 = 75 MARKS)

16.	State the general features of electrical conduction in metals. Develop the Drude Lorentz theory to get expression for conductivity σ .

(OR)

17.	Write the procedure for finding Miller Indices and state its important features.

18.	Discuss clearly the mechanism of intrinsic conduction in semi conductor. How will you determine the band gap in intrinsic semi conductor?

(OR)

19.	What is Hall Effect? Give some its applications.

20.	How will you determine the dielectric constant of a dielectric material?

(OR)

21.	What is piezo electricity? Give few applications of piezo electric materials.

22.	Explain in detail about the Helesenberg’s theory of ferro magnetism.

(OR)

23.	What are ferrites? Explain their structure, properties and its applications in detail.

24.	Discuss in detail about the preparation and characterization of high temperature ceramic super conductor.

(OR)

25.	What are the mechanical, electrical, chemical properties of metallic glasses? State the advantages of using metallic glasses as transformer core material.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AUTOMOTIVE ELECTRONICS						      Time : 3 hours

Subject Code:	EE294				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Sensors convert some __________ property in to an electrical signal.

2.	Sensors used in motor vehicles to register the angle of rotation on the throttle valve and the rate of change is  called as __________.

3.	Which type of electric motor is used as a starting motor?

4.	The starter motor is driven by __________.

a.	chain drive  	b.	gear drive 	c.	flat belt drive  	d.	V-belt drive

5.	In a fuel injection system, the electronic control module (ECM) calculates the optimum fuel-injection volume for the engine condition based on the data received from the sensors, and injects this volume of fuel into the intake manifold at the optimum timing.

Is the above statement is True (or) False?

6.	Mention any one advantage of electronic ignition system.

7.	Name any two types of head lights.

8.	What are the two types of electrically operated oil pressure indicators?

9.	The action of increasing the fuel/air mixture during acceleration in order to improve the vehicle's speed and its smooth response is called as __________.

10.	The central electronic elements of today's electric power steering systems are modern __________ bit Micro Controller Units designed for safety-critical applications.

PART – B  (5 x 3 = 15 MARKS)

11.	What is crank angle position sensor?

12.	What is the function of starting motor?

13.	Differentiate Throttle body Injection and Multi port fuel injection system.

14.	Give the advantages of earth return system of current wiring.

15.	Differentiate open loop and closed loop control systems.

PART – C  (5 x 15 = 75 MARKS)

16. 	With the aid of a neat sketch explain the construction and operation of a typical oxygen sensor used in vehicle.

(OR)

17.	Sketch and explain the working of solenoids and stepper motors as actuators in vehicle.

18.	Draw and explain the construction and principle of working of a starter motor.

(OR)

19.	a.	Briefly mention the care and maintenance aspects of starter motor. 					    (7)

b.	What is principle of D.C. series motor and draw its characteristics. 					    (8)

20.	Explain the contact less ignition system with suitable sketch.

(OR)

21.	Explain the working of the Fuel Injection system and also describe the injection system controls.

[P.T.O.]

22.	Explain the working of electric horn and wiper system.

(OR)

23.	a.	Explain the electrical fuel pump with neat diagram.								    (7)

b.	What is headlight dazzling? Discuss the various cases of dazzle prevention.			    (8)

24. 	Write notes on the following:

a.	Exhaust emission control 		b.	Distributor less ignition.						(8+7)

(OR)

25. 	Write notes on the following:

a.	Integrated engine control system 	b.	Electronically controlled brake system. 		(8+7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	NETWORK ANALYSIS AND SYNTHESIS				      Time : 3 hours

Subject Code:	EE295				 						      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What do you mean by S-domain network?

2. 	What do you mean by zeros in a network?

3. 	Define Magnitude.

4. 	Define phase.

5. 	Define Graph and Tree in a network.

6. 	Distinguish between a loop and a node.

7. 	What is a two port network?

8. 	What is an asymmetrical attenuator?

9. 	Give one property of RL network.

10. 	Give one property of RC network.

PART – B  (5 x 3 = 15 MARKS)

11. 	Give the importance of poles in a network.

12. 	Give the features of Amplitude in a network.

13. 	List the features of fundamental cut set.

14. 	What is a lattice network?

15. 	Give any 2 conditions for a polynomial to be Hutwitz.

PART – C  (5 x 15 = 75 MARKS)

16.   Explain the general techniques in detail for the s-domain circuit analysis.

(OR)

17. 	Explain the physical significance of poles and zeros in a Transfer function.

18.	Bring out the importance of amplitude and phase characteristics from pole zero plot.

(OR)

19.	Give the effects of Responses due to exponential and sinusoidal sources.

20.  	Explain the Graph, subgraph, degree of a vertex and loops with an example.

(OR)

21.	Give the fundamental cutset, matrix their rules for construction in detail with an example.

22.  	Explain the design procedure of the symmetrical attenuator.

(OR)

23.	Explain the m-derived and composite filters in detail.

24.  Give the functions and properties and synthesis of  LC one-port networks.

(OR)

25.  	Explain the Hurwitz polynomials and their properties in detail.