Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	SIGNALS AND SYSTEMS								      Time : 3 hours

Subject Code:	09EI218				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define power signal.

2.	State Parseval’s theorem.

3.	When is a system said to be non-linear?

4.	The CTFT represents _________ signal as a superposition of complex sinusoids.

5.	What is aliasing?

6.	What is the Nyquist sampling rate of the signal, x(t) = sin c2(100πt).

7.	Discrete time Fourier series have no convergence issues and no Gibbs phenomenon. True/False.

8.	Write the differentiation property of DTFT.

9.	ROC cannot be asymmetric in z-transform. True/False.

10.	What is the ROC of infinite duration causal sequence?

PART – B  (5 x 3 = 15 MARKS)

11.	Find the energy of the signal xt=Ae−αtu(t), α>0.

12.	State the Dirichlet’s condition of CTFS.

13.	Consider the analog signal x(t) = 3cos2πt. Plot its spectrum X(jω) and check whether the signal is band-limited.

14.	Find the DTFT of the signal  xn=18nun−2.

15.	State and prove the time-shifting property of z-transform.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Find the response y(t) when x(t)=e−α tu(t) , h(t)=e−β tu(t)for α ≠ β and α = β. Also

0 < α < 1 and 0 < β < 1.														    (8)

b.	Find the convolution output y[n] when the input is x[n] = anu[–n] and the impulse 	response is h[n] = bnu[n], 0 < |a| < 1, 0 < |b| < 1, |b| < |a|. 						    (7)

(OR)

17.	State and prove all the properties of discrete time systems.

18.	a.	Using the differentiation  property of CTFT, find the Fourier transform of  y(t) = tx(t), 			where xt=e−btu(t) .												    			    (8)

b.	Find the Fourier transform of the rectangular pulse whose magnitude is A and have the 	time period varying from –T to T.												    (7)

(OR)

19.	a.	Find the Fourier series coefficients for the signal

x(t) = 1 + sin6t  + cos4t.														    (8)

[P.T.O.]

b.	A causal LTI system has a frequency response, H(jΩ)=1jΩ+4for a particular input, x(t). This system is observed to produce an outputy(t)=(e−4t−e−5t)u(t). Determine the input x(t).																			    (7)

20.	a.	Explain impulse sampling method in detail.										  (10)

b.	Discuss the effect of under sampling.											    (5)

(OR)

21.	Consider the analog signal: x(t) = 5 sin(10πt).

a.	Find the DT signal obtained, if x(t) is sample at a sampling rate, Fs = 15 Hz.

b.	Also find the reconstructed output by using as ideal low-pass filter.

22.	a.	A DT-LTI system is represented by the difference equation:

y[n] – 0.5y[n–1] = (1/3)x[n].  Find the:

i.	unit impulse response 			ii.	unit step response.						    (9)

b.	Find the convolution of x[n] = {4, 3, 5, 1, 2} with h[n] = {3, 1,–2, 4}. 				    (6)

(OR)

23.	a.	Determine the Fourier series representation for the signal:

x[n]=1+sin(4π21n)+cos(10π21n). Plot its magnitude and phase spectrum.			  (10)

b.	State and prove Parseval’s theorem of DTFT.									    (5)

24.	a.	Find the inverse z – transform of the following function using partial fraction expansion 	method:

X(z) = z+22z2−7z+3, for ROC: |z| > 3 and |z| < 0.5. 								    (7)

b.	Determine the z-transform of the following:

xn=ansin⋅ω0nun														    (8)

(OR)

25.	A causal DT-LTI system is described by,

y[n] – (5/6)y[n–1] + (1/6)y[n–2] = x[n]

where x[n] and y[n] are the input and output of the system respectively. Determine the following:

a.	system function H(z) 				b.	impulse response, h(n)

c.	step response 					d.	response y[n] for the input x[n] = (0.25)n u[n]

e.	whether the system is stable.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRICAL MACHINES								      Time : 3 hours

Subject Code:	09EI220										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How will you change the direction of rotation of a d.c motor?

2.	What do you mean by residual voltage?

3.	What is the function of transformer oil in a transformer?

4.	How transformers are classified according to their construction?

5.	List out the method for speed control of 3 phase cage type induction motor.

6.	Write down the EMF equation of a synchronous motor.

7.	Define Step Angle.

8.	What is the function of capacitor in single phase induction motor?

9.	What are the types of electric heating available?

10.	Name the type of braking used for tractive motors.

PART – B  (5 x 3 = 15 MARKS)

11.	Under what circumstances does a dc shunt generator fail to generate?

12.	Why transformers are rated in kVA?

13.	What are the advantages of squirrel cage induction motor?

14. 	Differentiate between “capacitor start” and “capacitor start capacitor run” single phase induction motor.

15.	What are the disadvantages of AC traction?

PART – C  (5 x 15 = 75 MARKS)

16.  	a.	Derive the EMF equation of DC Generator.                                                                       (7)

b.	Discuss about the types of DC Generators with voltage equation and circuit diagram.  (8)

(OR)

17.	a.	Brief about the characteristics of a Shunt and Series Motor.                                              (9)

b.	With a neat diagram, explain the working of a three point starter.                                     (6)

18.	With a phasor diagram, explain the operation of a transformer on load condition for UPF, Lagging and leading PF.

(OR)

19.	A 5kVA, 200/1000V, 50Hz, single phase transformer gave the following test results:  OC test (LV side) : 2000V, 1.2A, 90W; SC Test (HV Side): 50V, 5A, 110W.  Calculate the                  a.	parameters of the equivalent circuit referred to the LV side 		b.	output secondary voltage when delivering 3kW at 0.8pf lagging, the input primary voltage being 200V.  Find the percentage regulation also.

20.	a.	Derive the torque equation of Induction Motor for both starting and maximum condition.     																				  (10)

b.	Explain the torque speed curve of an Induction Motor.                                                     (5)

(OR)

[P.T.O.]

21.	Explain the V and Inverted V curves of a Synchronous Motor.

22.	Explain the working of a Shaded Pole Induction motor with the speed torque curve.

(OR)

23.	Based on the double field revolving theory, explain how the Single Induction motor is made self starting.

24.	a.	Explain about the Induction Heating.                                                                                 (8)

b.	Brief about the working of a welding generator.                                                                (7)

(OR)

25.	Discuss in detail about:

a.	AC traction system 	b.	DC traction System 		c.	Multiple traction system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	LOGIC AND DISTRIBUTED CONTROL SYSTEMS		      Time : 3 hours

Subject Code:	09EI224				 							       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	__________ Unit is essentially a solid – state switch that takes the decoded address signal.

2.	Define Polling.

3.	Give the advantages of PLC.

4.	Draw the symbol of Motor Overload Relay Contact.

5.	The _________ the process by sending an output signal to the control valve.

6.	What are the program control instructions in PLC?

7.	Mention the basic elements of a microprocessor based controller in DCS.

8.	What is mean by HLCD and CID in DCS?

9.	Define Data Highways.

10.	Name any two Industrial communication protocols.

PART – B  (5 x 3 = 15 MARKS)

11. 	Write the types of Data loggers.

12. 	Give the coil format for counter instruction.

13. 	What is meant by skip function in PLC?

14. 	Mention the Evolution of DCS.

15. 	What are the tasks of supervisory computer?

PART – C  (5 x 15 = 75 MARKS)

16. 	Briefly explain the block of data acquisition system.

(OR)

17. 	Discuss the operation of SCADA.

18. 	Write short notes on overview of PLC Systems.

(OR)

19. 	Describe briefly about the timer and counter functions with suitable examples.

20. 	List out the sequences of Dishwasher, and develop the ladder logic. Mention clearly about the function Matrix and register pattern.

(OR)

21. 	Write short notes on PLC Arithmetic functions and Number comparison functions.

22. 	Draw the architecture of DCS and explain the function of each block.

(OR)

23.  Discuss in detail about the building blocks of DCS.

24.  Discuss in detail about DCS – supervisory computer tasks and configuration.

(OR)

25. 	Discuss the real time implementation of DCS in Cement Plant.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title : 	NETWORKS & PROTOCOLS FOR INSTRUMENTATION & CONTROL														      				      Time : 3 hours

Subject Code:	09EI226											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the physical standards.

2.	What is UART?

3.	List out the serial data communications interface standards.

4.	Define troubleshooting.

5.	Mention the OSI layers.

6.	Define protocol.

7.	What is HART?

8.	Define field bus.

9.	What is mean by network topologies?

10.	List out the LAN standards.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the concepts of transmission characteristics data coding.

12.	Compare the balanced and unbalanced transmission lines.

13.	Write short notes on flow control protocols.

14.	Briefly explain ASCII based protocols.

15.	Compare circuit and packet switching.

PART – C  (5 x 15 = 75 MARKS)

16.	Illustrate the concepts of synchronous and asynchronous systems.

(OR)

17.	Explicate the communication principles and communication modes.

18.	Illustrate the functions of RS232, 423 and 485 interface standards.

(OR)

19.	Analyze the following.

a.	Interface to printers.			b.	Serial interface converters.

20.	Illustrate the data communication for instrumentation and control techniques.

(OR)

21.	With suitable example explain the individual OSI layers and OSI analogy techniques.

22.	Illustrate the functions of Allen Bradley protocol.

(OR)

23.	Explain the working of Modbus protocols and field bus.

24.	Analyze the following:

a.	Network Architecture and Protocols			b.	Internet work connections.

c.	Token bus

[P.T.O.]

(OR)

25.	Analyze the following:

a.	LAN Standards		b.	Ethernet.		c.	NOS.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CIRCUIT ANALYSIS & NETWORKS				           Time : 3 hours

Subject Code:	10EI201		 									      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define active and passive elements with example.

2.	Relate the VI relationships between R, L and C elements.

3.	List any two applications of maximum power transfer theorem.

4.	Point out the applications of voltage and current source transformations.

5.	Differentiate between frequency response and transient response.

6.	State the concept of complex frequency.

7.	Define coupled circuits.

8.	Compare self and mutual inductances.

9.	List any two characteristics of Ideal filter.

10.	Define two port networks.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Mesh, Loop, Branch and Node.

12.	State Norton theorem and list its two applications.

13.	A DC voltage of 100 volts is applied to a series RL circuit with R = 50Ω. What will be the current in the circuit at twice the time constant?

14.	Define the dot rule for coupled circuits.

15.	Compare low pass and high pass filter.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Analyze the concepts of current division and voltage division techniques.			(10)

b.	Obtain the total current from the source in the circuit given in the figure.			(5)

(OR)

7.	a.	State Kirchoff’s laws.															(5)

b.	Find the equivalent resistance of the circuit shown in the figure.					(10)

[P.T.O]

18.	a.	Three resistors 25 Ω, 50 Ω and 75 Ω are connected in star. What are the equivalent delta 			resistors?																	(7)

b.	Verify the superposition theorem to find the current through 5 Ω resistance for the network 	shown below.																(8)

(OR)

19.	a.	Each of the 3 arms of a delta connected network has resistance of 3 Ω. Find the equivalent 	star connected network.														(5)

b.	Find current through 10Ω resistance by Norton’s theorem.							(10)

20.	Derive the transient response of RL series circuit for charging current.

(OR)

21.	Analyze the frequency response of RLC series circuits.

22.	a.	Find the inductance of a coil of 200 turns wound on a paper core tube of  25cm length and 		5cm radius. Also calculate energy stored in it if current rises from zero to 7A.		(8)

b.	Derive an expression for co-efficient of coupling.									(7)

(OR)

23.	a.	Explain the equivalent circuit of coupled circuits.									(7)

b.	Compare tuned coupled circuits and double tuned circuits.							(8)

24.	Analyze the impedance matching equivalent Pi and T networks.

(OR)

25.	a.	Outline the concept of passive filter as a two port network.						(8)

b.	Explain the voltage and current ratios of two port networks.						(7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRONIC CIRCUITS						         Time : 3 hours

Subject Code:	10EI202				 						    Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the basic purpose of a filter?

2.	What is the ripple factor of a full wave rectifier?

3.	Which of the four h parameters has the greatest value?

4.	How to calculate the gain of a cascaded amplifier?

5.	Which parameters determine the performance of power amplifiers?

6.	What is a power amplifier?

7.	On what factors the voltage gain of a common source JFET amplifier depends?

8.	Which of the bias method provides a solid Q point in JFET amplifiers?

9.	What is monostable multivibrator?

10.	What is the advantage of using negative feedback in an amplifier?

PART – B  (5 x 3 = 15 MARKS)

11.	Define transformer utilization factor?

12.	Draw the h – parameter model of CE configuration. Also define each term in it.

13.	What are the advantages of  ‘push pull techniques’ in the power amplifier?

14.	What are the different types of biasing of JFET amplifier?

15.	What is negative feedback and its effect?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the operation of a half wave rectifier with a neat circuit diagram and its waveforms. Also derive its average values of output voltage and load current.

(OR)

17.	What is clipping and its types? Explain briefly about positive clipper with circuit diagram with its waveforms.

18.	Draw and briefly explain the hybrid equivalent circuit of a transistor of all configurations. On what factors h – parameters of a transistor depends?

(OR)

19.	What is cascade and cascode amplifier? Derive the overall power gain of multistage power amplifier with a general block diagram?

20.	Write short notes on:

a.	RC coupled Amplifier.				b.	Darlington amplifier.

(OR)

21.	Draw and explain the operation of class B push pull amplifier.

22.	Draw and briefly explain the small signal FET models.

(OR)

23.	Briefly explain about gate bias and self bias of JFET.

[P.T.O]

24.	Explain the operation of a monostable multivibrator with circuit diagram and its waveforms.

(OR)

25.	What is meant by negative feedback?  Explain the effects of negative feedback.  Also explain the various types of negative feedback connections.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	SENSORS AND TRANSDUCERS						      Time : 3 hours

Subject Code:	10EI203				 						      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A measure of the repeatability of a measurement of some quantity is __________.

2.	When the system is used to measure a condition not varying with time is called __________.

3.	__________ is the degree of closeness with which a given value may be repeatedly measured.

4.	When a first order transducer is excited by a unit step input U(s) = __________.

5.	__________ is the fundamental Piezo electric coefficient; it is related to the charge generation.

6.	The principle of operation of LVDT is based on the variation of __________.

7.	The voltage VH, is known as the __________.

8.	Piezo-electric transducers are __________.

9.	__________ is revolutionizing the data communication field and it is known that light can travel only in a straight line.

10.	__________ encoder can be used for a total angular displacement 360°

PART – B  (5 x 3 = 15 MARKS)

11.	Define Traceability.

12.	What is frequency response of Zero Order Transducers?

13.	List the factors to be considered for bonded strain gauge?

14.	State the principle of capacitive transducer.

15.	Compare digital transducer with analog.

PART – C  (5 x 15 = 75 MARKS)

16.	One hundred temperature readings were taken at small intervals of time and recorded to the nearest 0.5° C. The frequency of occurrences of the readings is given below

| Temperature in °C | 98.5 | 99 | 99.5 | 100 | 100.5 | 101.0 | 101.5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | 4 | 13 | 19 | 35 | 17 | 10 | 2 |

Calculate:

a.	Arithmetic Mean  	b.	Average Deviation 		c.	Standard Deviation

d.	Probable Error.

(OR)

17.	Discuss the various sources of errors in a measurement system.

18.	Derive the transfer function of second order transducer and obtain response of Ramp and impulse inputs.

(OR)

19.	Distinguish the following static characteristics of transducers:

a.	Input Impedance vs Output Impedance.

b.	Resolution vs Accuracy.

c.	Accuracy vs Precision.

d.	Threshold vs Resolution.

20.	Explain the principle, operation and characteristics of LVDT and RVDT.

[P.T.O.]

(OR)

21.	Give the principle of thermocouple and also explain the temperature compensation of the same.

22.	Define magnetostrictive effect and describe the application of magnetostrictive transducers.

(OR)

23.	Explain with a neat sketch the construction details of Piezo electric transducer.

24.	Describe the working of fibre optic displacement transducer. Draw its input-output characteristics.

(OR)

25.	a.	Describe the working principle of vibration sensors.

b.	Write short notes on pH sensors.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	SIGNALS AND SYSTEMS								      Time : 3 hours

Subject Code:	10EI204										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Sketch the signal x(t) = u(t) – u(t–a).

2. 	Represent the sequence x(n) = {1, 2} in terms of impulses.

3. 	The length of the sequence x1(n) is N1 and length of another sequence x2(n) is N2. Let y(n) = x1(n)*x2(n) where * represents convolution. The length of y(n) is __________.

4. 	Write the convolution integral formula.

5. 	A function f(t) and its Fourier transform  is  represents _________ of the function.

6. 	State duality property of Fourier transform.

7. 	What are the necessary and sufficient conditions for the existence of DTFT?

8. 	What are the methods available to avoid aliasing?

9. 	Find the Z-transform of the function x(n) = u(n).

10. 	Write the difference equation of the system is described by transfer function:

PART – B  (5 x 3 = 15 MARKS)

11. 	Find the energy of the signal .

12. 	Convolve the sequence x(n) = (1, 1, 1) and h(n) = (1, 2, 3).

13. 	Find the Fourier transform of the signal .

14. 	Prove Parseval’s theorem for discrete time signals.

15. 	Determine the Laplace transform of x(t) = tu(t).

PART – C  (5 x 15 = 75 MARKS)

16. 	Consider the triangular waveform x(t) shown below.							     (3+4+4+4)

Sketch the following wave forms:

a.			b.			c.		d.

[P.T.O]

(OR)

17.	Determine whether the given systems are memoryless, time invariant, linear, causal and stable.

a.	y(t) = x(t) cos(3t).															    (5)

b.	y(t) = x(t2).																	    (5)

c.		.														    (5)

18.	a.	Convolve x(n) = an u(n) and h(n) = bn u(n), , .  Assume . 		    (8)

b.	Consider a continuous time system whose input x(t) and output y(t) are related by where a is a constant. Find y(t) with the condition y(0)=y0 and

xt=ke−btut.		     														    (7)

(OR)

19.	a.	Convolve x(t) = u(t) – u(t–4) and h(t) = u(t) –u(t–1).					  	   		    (9)

b.	Convolve x(t) = e2t u(t) and h(t) = u(t).					    	  	   				    (6)

20. 	Find the trigonometric Fourier series for the signal x(t).

(OR)

21.	Derive time shifting, time scaling, frequency shifting, differentiation properties of continuous time Fourier transform.													      (3+4+3+5)

22.	Find the discrete time Fourier transform of the following signals.

a.																    (8)

b.																   (7)

(OR)

23.	How is sampling done in continuous time signals? Explain in detail with necessary equations.

24.	a.	Find the inverse Z-transform of

b.	Prove convolution property of Z-transform.

(OR)

25.	Find the output of the system whose input-output is related by difference equation.

for the step input. Assume initial conditions to be zero.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CONTROL SYSTEMS									      Time : 3 hours

Subject Code:	10EI205				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the basic elements used for modeling an electrical system.

2.	Define open loop and closed loop control system.

3.	Why derivative control is not employed in isolation?

4.	What are the advantages of PID controller?

5.	Find the Transfer function of a system if its impulse response is e-3t sin 2t.

6.	Define phase and gain margin.

7.	How the roots of characteristic equation are related to stability?

8.	What is Routh criterion?

9.	Define state model.

10.	List the advantages of sampled data control system.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the parameters used in the Voltage analogy and its conversion in Current.

12.	The damping ratio of a system is 0.75 and the natural frequency of oscillation is 12 rad/sec. Determine the peak time.

13.	Derive the response for a first order system with ramp signal as input.

14.	Consider a fourth-order system with characteristic equation

s4  + 8s3 + 18s2 + 16s + 50 = 0.

Find the stability of the system.

15.	Define the Controllability and Observability of a system.

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain transfer function C(s)/R(s) and C(s)/D(s) of the system shown in figure.

(OR)

17.	Find the transfer function for the diagram given below by converting it in to signal flow graph making use of Mason’s gain formula.

[P.T.O.]

18.	a. 	Describe the components of an automatic control system.

b. 	Explain the operation of synchros.

(OR)

19.	a. 	Compare AC and DC servo motors.

b. 	Explain the operation of gyroscope.

20.	Derive the response of second order systems for the unit step input.

(OR)

21.	The open loop transfer function of a unity feedback system is given G(s) = K/s(sT+1) where k and T are positive constants. By what factor should the amplifier gain K be reduced, so that the peak overshoot of unit step response of the system is reduced from 75% to 25%?

22.	Sketch Bode plot for the given transfer function and determine the system gain K for the 	gain cross over frequency of 5 rad/sec.

G=ks21+0.2s1+0.2s

(OR)

23.	The open loop transfer function of a unit feedback system is given by

Gs=1ss+12s+1

Sketch the polar plot and determine the gain margin and phase margin.

24.	A system is given by s5 + 2s4  + 24s3 + 48s2 – 25s – 50 = 0. Check the stability of the system using Routh criterion and also find the frequency of oscillation.

(OR)

25.	The open loop transfer function of a unity feedback system is given by Gs=ks+3ss−1. Sketch 	the Nyquist plot and determine the stability of the system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	SIGNAL CONDITIONING CIRCUITS					      Time : 3 hours

Subject Code:	10EI206				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a compensated op-amp?

2.	Define Slew rate.

3.	What is the difference between a basic comparator and the Schmitt trigger?

4.	What is a monostable multivibrator?

5.	Give the applications of Instrumentation amplifier.

6.	In what principle a basic switching regulator works.

7.	What are the limitations of a 3-pin regulator?

8.	Why do switching regulators have better efficiency than the series regulators?

9.	What are the function blocks of a PLL?

10.	What is VCO?

PART – B  (5 x 3 = 15 MARKS)

11.	Give the electrical characteristics of an ideal op-amp.

12.	Draw the circuit diagram of peak detector.

13.	What are the basic requirements of an instrumentation amplifier?

14.	What are the various specifications of a three terminal IC regulator?

15.	What is the need for amplitude modulation and also define AM?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A differential amplifier is shown in figure.

i.	Find the output voltage.

ii.	Show that the output corresponding to common-mode voltage is zero if (R1/R) = 			(R2/R1)

iii.	Find the CMRR of the amplifier if (R1/R) ≠ (R2/R1). 							    (7)

b.	Write short notes on differential amplifier.										    (8)

(OR)

[P.T.O.]

17.	a.	Write a note on frequency response and explain unity gain bandwidth.				    (8)

b.	Explain briefly about the construction and working of a non-inverting amplifier.		    (7)

18.	a.	Draw the circuit diagram of the temperature compensated log amplifier and derive the 	expression for output voltage.				 									    (7)

b.	Explain any two application of comparator with neat sketch.						    (8)

(OR)

19.	a.	Explain how multiplier works. Also explain how the voltage division and squaring can be 	achieved using multipliers.													  (10)

b.	Explain the working of Astable operation of IC 555 timer.							    (5)

20.	a.	Design a second order Butterworth low pass filter at cut-off frequency of 1 kHz. 		    (8)

b.	Discuss the working of an instrumentation amplifier.				   				    (7)

(OR)

21.	a.	Determine the transfer function of the band pass filter using op-amp and obtain the 	expression for bandwidth and Q-factor for the same.				 	  			  (10)

b.	Explain how op-amp can be used with capacitive displacement transducer. 	  		    (5)

22.	a.	Explain the working of a 723 general purpose regulator.							    (8)

b.	Discuss the frequency doubling circuit and derive the expression by considering two sinusoidal signals.															    (7)

(OR)

23.	a.	Define IC voltage regulator. Discuss the performance parameters and characteristics of the 	voltage regulators. 															    (8)

b.	Discuss the working of series op-amp voltage regulator with neat diagram.			    (7)

24.	a.	Explain the construction and working of voltage controlled oscillator with the help of a 	neat circuit diagram and waveforms.											    (9)

b.	Calculate output frequency, lock range and capture range of a 565 PLL circuit if,

RT = 10 kohm, CT = 0.01 μF and C = 10μF.										    (6)

(OR)

25.	a.	Discuss any two digital phase detectors in detail.									    (8)

b.	Explain how PLL acts as a frequency translator.									    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MICROPROCESSORS AND MICROCONTROLLERS	      Time : 3 hours

Subject Code:	10EI207											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the use of parity flag in 8085 microprocessor?

2.	List the 16 bit registers of 8085 microprocessor.

3.	Which interrupt has the highest priority in 8085 microprocessor?

4.	What is the role HOLD signal in 8085?

5.	What is the use of Mode-I operation in 8255?

6.	Mention the role of EOC and SOC in interfacing of converters with 8085.

7.	Classify the 8051 instruction set based on the format.

8.	Name any two special function registers related with timers of 8051.

9.	An alternate function of port pin P3.3 in the 8051 is _________.

10.	What is the role of BSR mode in 8255?

PART – B  (5 x 3 = 15 MARKS)

11.	List the different machine cycles of 8085.

12.	Discuss the addressing modes of 8085 with suitable examples.

13.	Differentiate BSR mode and I/O mode in 8255 PPI.

14.	Write about the banks of 8051 microcontroller.

15.	Write a program to generate a saw tooth wave using DAC with the microcontroller.

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat functional block diagram, explain the architecture of 8085 processor.

(OR)

17.	Sketch the timing diagram for the execution of MVI B, 05H Instruction.

18.	Discuss the different software and hardware interrupts of 8085 processor.

(OR)

19.	Write a program for arranging the given set of numbers in an ascending order.

20.	With a neat sketch explain the architecture of 8255 PPI.

(OR)

21.	Discuss the interfacing of stepper motor with 8085 microprocessor.

22.	Explain the internal architecture of 8051 Microcontroller in detail.

(OR)

23.	Explain the function of 8051 instructions for performing arithmetic, logical and data transfer operations with suitable examples.

24.	Describe the interfacing of ADC with 8051.

(OR)

25.	With a neat diagram, explain the keyboard interfacing with 8051 Microcontroller.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	PROCESS DYNAMICS AND CONTROL				      Time : 3 hours

Subject Code:	10EI208		 								       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is self regulation?

2.	Input variables of a process can be classified as _____________.

3.	Write the limitations of ON/OFF Control Mode.

4.	The derivative mode of controller operation provides the controller output which depends on the _____________.

5.	Feed forward control acts as a _____________.

6.	What is meant by controller tuning?

7.	What is the need for Valve Positioner?

8.	The type of valve used to handle slurries is _____________.

9.	What do you mean by bottom product in a distillation column?

10.	What is a Heat Exchanger?

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between Continuous Process and Batch Process.

12.	Why Derivative Control Mode is not recommended alone for any process?

13.	Name the different methods of Controller Tuning.

14.	What is Control Valve Flashing?

15.	How do you control drum level in a steam boiler?

PART – C  (5 x 15 = 75 MARKS)

16.	What are the major elements of a Process Control System? Explain each element in detail with a neat sketch.

(OR)

17.	Derive the transfer function of interacting systems.

18.	Explain in detail about the characteristics of Proportional, Integral and Derivative control modes with necessary diagrams.

(OR)

19.	Explain in detail about the control scheme used for controlling the Temperature Process with a neat diagram.

20. 	a.	Discuss in detail about the Controller tuning by Ziegler-Nichol’s method.			  (10)

b.	Write a brief note on ¼ Decay ratio method of Controller Tuning.					    (5)

(OR)

21. 	a.	Discuss in detail about the various steps involved in the process reaction curve method of 	controller tuning. Also tabulate the expressions for the “best” controller settings developed 	by Cohen & Coon.															  (10)

b.	Discuss in detail the commonly used time integral performance criteria. 			    (5)

[P.T.O.]

22. 	a.	With a neat sketch explain the characteristics of a Control Valve.					    (8)

b.	Explain in detail about the Control Valve Positioner with a neat diagram.			    (7)

(OR)

23. 	a.	Explain in detail about Control valve sizing.										    (8)

b.	Write short note on Cavitation and Flashing in Control valve						    (7)

24. 	Describe the working of distillation column with appropriate sketches.

(OR)

25.  Explain in detail about the following:

a.	Ratio control.																    (7)

b.	Cascade Control.																    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INDUSTRIAL INSTRUMENTATION					      Time : 3 hours

Subject Code:	10EI209								      			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Pirani gauge is useful for measurement of pressure in the range ___________.

2.	State the merits and demerits of elastic pressure gauges.

3.	When is a venture tube preferable to orifice plate?

4.	Does the operation of the turbine flow meter depend on liquid characteristics?

5.	Why three lead RTD circuit is popular compare to two and four wire circuit.

6.	What is the difference between broad band and selective band pyrometer?

7.	Write the safety considerations with radiation level sensors.

8.	Suggest a method to detect the presence or absence of solids in a silo.

9.	Differentiate between Newtonian and Non-Newtonian fluid.

10.	Suggest any two methods for measuring gas density.

PART – B  (5 x 3 = 15 MARKS)

11.	With a neat sketch explain how a pressure gauge can be calibrated

12.	Explain the working principle of Electromagnetic flow meter.

13.	State Seebeck and Peltier effect of thermocouple.

14.	Write a short note on level probe sizing.

15.	Why viscosity measurement is important in industries? What are its challenges?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	How pressure is measured using diaphragms? Give the relation between displacement and 	pressure.																		    (7)

b.	Explain the function of any one thermal conductivity gauge for vacuum measurement with 		a neat diagram. 																    (8)

(OR)

17.	a.	Explain the working principle of Mcleod gage with a neat diagram also give the output 	equation for the applied low pressure. 											    (8)

b.	Explain Force balance type pneumatic transmitter with a neat sketch. 				    (7)

18. 	Derive the steady state relationship between output voltage and velocity for constant temperature anemometer (CTA) and also the overall transfer function for CTA system with block diagram.

(OR)

19.	a.	List the different types of positive displacement liquid meters and explain the working 	principle of any one with neat sketch.											    (8)

b.	List and explain different types of orifice tap connections.      						    (7)

20.	a.	What is meant by reference junction compensation in thermocouple? Briefly explain any 	two methods for the same. 					              	          					  (10)

b.	Draw a neat sketch of Disappearing Filament type Optical Pyrometers and explain. 	    (5)

(OR)

[P.T.O.]

21.	Discuss in detail about the principle, construction, signal conditioning circuits of RTD.

22.	a.	Explain the working of field effect and ultrasonic level switches with neat diagram.	  (10)

b.	Discuss the applications, advantages and limitations of float level switches.             	    (5)

(OR)

23.	a.	Describe the method of measurement of level using capacitance probe. Give the relation 	between level and total capacitance of a cylindrical tank with capacitance probe.	          (10)

b.	Explain the working principle of displacer type level switches.                                  	    (5)

24.	a.	Explain any one type of oscillating element and rotating element Industrial Viscometer.

(10)

b.	What are parameters that affect the density measurement in gas column balance sensor? 	How are they compensated?                                                                           			    (5)

(OR)

25. 	a.	Explain any one method of Gas Density measurement with a neat sketch. 			    (5)

b.	With a neat sketch explain Saybolt and Capillary tube viscometer.     	          		  (10)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	LOGIC AND DISTRIBUTED CONTROL SYSTEMS		      Time : 3 hours

Subject Code:	10EI210				 							       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What does the term Data loggers refer to?

2.	Write the full form of SCADA.

3.	Mention any four output devices that would be interfaced with the input module of PLC.

4.	List the types of Counters used in PLC.

5.	List out the alternative programming languages of Ladder Logic.

6.	Give few examples of Data comparison functions in PLC.

7.	Write the full form for LCU.

8.	List the communication facilities of DCS.

9.	What does the term HART refer to?

10.	Mention the industries in which DCS is the heart of the control plant.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw a simple block diagram of Data loggers.

12.	Analyze the types of programming languages in PLC.

13.	What is the purpose of inter-locks in PLC?

14.	What are the functions of redundancy?

15.	Write a short note on DCS communication standards.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the architecture of a SCADA system with a block diagram.

(OR)

17.	With a simple block diagram compare the Data Acquisition System (DAS) and Direct Digital Control (DDC).

18.	Construct the architecture of the PLC. Detail the classification of input and output module of PLC. Explain the Analog input and Analog output module of the PLC with a block diagram.

(OR)

19.	Analyse the implementation of ON-Timer and Retentive timer.

20.	List five Arithmetic Functions used in PLC programming and explain them in detail.

(OR)

21.	Implement a Pressure control process with a closed loop control using PID block of a PLC. Draw the closed loop block diagram and explain about the PID controller available in PLC.

22.	Draw the building blocks of Distributed Control System and explain the functionality of Local Control Unit.

(OR)

23.	a. 	Analyze the process interfacing issues that anyone would experience with DCS.		    (8)

b. 	How is hardware failure in DCS handled by the concept called “Redundancy”?		    (9)

[P.T.O]

24.	Reproduce the Topology of Field Bus and PROFIBUS Structure and explain in detail.

(OR)

25.	Analyze the evolution of HART protocol with its explanation. Draw the signal architecture of “super-imposed DC”. List the communication modes as well.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BIOMEDICAL INSTRUMENTATION					      Time : 3 hours

Subject Code:	10EI211				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the equation for cell potential.

2.	What are the major ions present in a cell?

3.	Define Absolute Refractory Period.

4.	What are sensory nerves?

5.	What is Diathermy?

6.	What is Diastole?

7.	What is the measure of Lung Capacity?

8.	Give two precautions for Patient safety?

9.	What is the normal value of blood PH?

10.	What is Angiography?

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the action potential and its time periods.

12.	What is need for instrument amplifiers in medical equipments?

13.	What is meant by sodium pump?

14.	Define evoked potential.

15.	Explain Blood gas Analyzer.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the 3 Lead configurations in ECG measurement.		 					    (7)

b.	What are the types of muscles? Explain. 										    (8)

(OR)

17.	a.	Sketch a Neuron and explain its parts.											    (8)

b.	What is EOG and how is it measured? 											    (7)

18.	a.	Explain the measurement of Lung volume. 										    (7)

b.	Explain the measurement of Blood pressure. 									    (8)

(OR)

19.	a.	Discuss any one type Blood flow meter. 										    (8)

b.	Explain one cardiac output cycle.						 						    (7)

20.	a.	Explain Surgical Diathermy in detail.											    (7)

b.	Describe the Heart Lung machine. 												    (8)

(OR)

21.	a.	Explain any one type of nerve stimulator with the help of a neat diagram. 			    (8)

b.	What are the Hazards which may occur in Hospitals? Explain. 						    (7)

22.	Explain any one spectrophotometer used in Hospitals for Research.

(OR)

[P.T.O]

23.	a.	Describe a method of measuring dissolved oxygen content in the blood.				    (8)

b.	Explain cell counter in detail.					 								    (7)

24.	a.	What is the basic principle of MRI? Discuss one Medical application using MRI. 	  (10)

b.	Explain the principle of ultrasound scanner.										    (5)

(OR)

25.	Explain Bio Telemetry with a neat diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	NEURAL NETWORKS AND FUZZY LOGIC CONTROL     Time : 3 hours

Subject Code:	10EI212											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	___________are used to receive signals from other neurons.

2. 	What is bias?

3. 	Give some examples of feedback networks.

4. 	Write the energy function for discrete Hopfield network.

5. 	What are Fuzzy sets?

6. 	What you mean by Universal set?

7. 	What is support of a membership function?

8. 	The ___________ decision method presumes that future states of nature can be characterized as 	probability events.

9. 	What are the gases inhaled during anesthesia?

10. 	What is the main assumption in fuzzy logic control system designing of a plant?

PART – B  (5 x 3 = 15 MARKS)

11. 	What are the different types of learning rules?

12. 	State the purpose of process identification and list the types of plant identification.

13. 	Differentiate fuzzification and defuzzification.

14. 	What is Mutation and Reproduction?

15. 	List out stages involved in fuzzy logic controller system.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Explain briefly the operation of biological neural network with a simple sketch.            (10)

b.	Compare Artificial Neural Network and Biological Neural Network.                               (5)

(OR)

17. 	Explain the back propagation net with a neat sketch.

18.	Explain how ANN can be used for neuro controller for inverted pendulum.

(OR)

19.	Explain how the ANN can be used for process identification with neat sketch.

20.	Describe fuzzy relation in detail.

(OR)

21. 	Calculate the complement, union, intersection and difference of the fuzzy sets

A = {(1/2) + (0.5/3) + (0.3/4) + (0.2/5)} and B = {(0.5/2) + (0.7/3) + (0.2/4) + (0.4/5)}.

22.	Write about genetic algorithm and its application.

(OR)

23. 	With a neat sketch discuss the major components of Adaptive fuzzy system.

24.	Explain how fuzzy logic is being implemented for image processing in Television sets.

(OR)

25. 	Discuss the home heating system with fuzzy logic control.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DIGITAL CONTROL SYSTEMS						      Time : 3 hours

Subject Code:	10EI213				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is quantization noise?

2.	Give the transfer function of Zero order Hold.

3.	__________  provides a method for the analysis discrete time system in frequency domain.

4.	List the standard input signals in the response.

5.	If all initial states are controllable, the system is said to be ___________.

6.	The transfer function of LDS system is also called ___________.

7.	The sampled data control system is stable if all the poles of the Z- transfer function of the system lie inside the ____________ in Z-plane.

8.	___________ is used to determine whether the roots of the characteristic polynomial lie within a unit circle or not.

9.	Write the non interacting PID Controller equation for Position PID Algorithm.

10.	Draw the control scheme of bumpless transfer in velocity algorithm.

PART – B  (5 x 3 = 15 MARKS)

11. 	State Shannon’s sampling theorem.

12. 	State and prove linearity property.

13. 	Write the features of observabilty.

14. 	What is mean by quantization level?

15. 	Define Kalman algorithm.

PART – C  (5 x 15 = 75 MARKS)

16. 	Describe in detail about all D/A converters.

(OR)

17. 	a.	Discuss the sample data control system with example. 							  (10)

b.	Write short notes on reconstruction.                               							    (5)

18. 	a.	Find the inverse Z transform of

X(z)=1(1+02z−1)(1−0.2z−1)2.											    	  (12)

b.	Find the Z transform of x (n) = {1, 2, 5, 7, 0, 1}                     		             			    (3)

(OR)

19. 	Describe in detail about the Modified Z transform.

20. 	Investigate the pulse transformation:

a.	Cascade realization.                    												    (7)

b.	Parallel realization.                      												    (8)

(OR)

21. 	a.	Discuss about the controllability and observability.      							    (8)

b.	Give the design of state feedback control.                                 						    (7)

[P.T.O.]

22. 	Obtain H(Z) using bilinear transformation when T=1 sec for:

H(s)=2ss2+0.2s+1.

(OR)

23. 	Check the stability of sample data control system using Jury’s stability test:

a.	6Z2 – 2Z + 1= 0  																    (7)

b.	Z3 – 1.3Z2 – 0.08Z + 0.24 = 0												 	    (8)

24. 	Design using Dahlin’s algorithm:

Gp(s)=e−0.8s0.6s+1 when T = 0.4 and N=2. Find D (Z).

(OR)

25. 	Explain in detail about the implementation of control algorithms using microcontroller.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	COMMUNICATION ENGINEERING					      Time : 3 hours

Subject Code:	10EI214				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the degrees of modulation?

2.	What are the advantages of VSB-AM?

3.	What are the types of External noise?

4.	How FM wave can be converted to PM wave?

5.	Define adaptive delta modulation.

6.	Define peak frequency deviation for FSK.

7.	Define non resonant transmission line.

8.	What is the purpose of data modem?

9.	What are the advantages of optical fiber communication?

10.	Define orbital satellite.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the need of modulation?

12.	The antenna current of an AM transmitter is 8 A when only carrier is sent.  It increases to 8.93A when the carrier is modulated by a single sine wave.  Find the percentage modulation.

13.	State merits and demerits of BPSK.

14.	Write short notes on different types of data communication codes.

15.	Explain in detail about Data modems.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With the help of a neat diagram, explain the generation of DSB-SC using Balanced 	modulator.																	    (8)

b.	Write about the coherent detection method in detail for DSB-SC and SSB-SC.  What 	happens when there is phase mismatch?											    (7)

(OR)

17.	An AM signal is generated by modulating the carrier fc = 800MHz by the signal m(t)=sin3000πt+0.5cos5000πtthe AM signal s(t)=1001+m(t)cos2πfctis fed to a 50 ohm load.

a.	Determine the average power in the carrier and in the sidebands.

b.	Find the modulation index and peak power delivered to the load.

18.	Describe in detail about the modulation and demodulation process of FM wave.

(OR)

19.	a.	Explain the following:		i.	Shot noise 	ii.	Noise temperature. 				    (8)

b.	How do you represent narrowband noise?										    (7)

20.	A PCM system has the following parameters : a maximum analog input frequency of 4 kHz, a maximum decoded voltage at the receiver of ±2.55 V, and a maximum dynamic range of 45dB.  Determine the following : Minimum sample rate, minimum number of bits used in the PCM code, resolution and quantization error.

[P.T.O.]

(OR)

21.	Explain the relationship between:

a.	the minimum bandwidth required for an FSK system and the bit rate.

b.	the mark and space frequencies.

22.	Explain in detail about some optical sources used in optical fiber communication.

(OR)

23.	Describe the operation of frequency division multiplexing with necessary diagrams.

24.	Explain in detail about satellite system parameters and write the satellite system link equation.

(OR)

25.	Describe in detail about television signals and its receivers with necessary diagrams.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MODERN CONTROL SYSTEMS						      Time : 3 hours

Subject Code:	10EI218				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In closed loop control system, with positive value of feedback gain the overall gain of the system will __________.

2.	The temperature under thermal and electrical system analogy, is considered analogous to __________.

3.	The type 1 system has __________ at the origin.

4.	The effect of error damping is to reduce __________.

5.	The damping ratio of a system is 0.5. The value of maximum resonant peak is __________.

6.	At the phase cross over, ω=5 rad/sec , |G(j ω )H(j ω )| = 10dB. The system is __________.

7.	The root locus is a __________ method.

8.	Break points can be __________.

9.	X(t)=AX(t)+BU(t) is called __________.

10.	An  n*n matrix is said to be non-singular if the rank of the matrix r is __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Write Masons Gain formula.

12.	For a system having G(s).  Hs=ks+4ss3+5s2+6s. Find

a.	Type of the system.			b.	Order of the system.

13.	In a second order system given below, determine the natural frequency of oscillations and damping factor,  CsRs=98596s2+62.8s+3142.

14.	What are asymptotes? How will you find the angle of asymptotes?

15.	Define state and state variable.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the F-V and F-I analogies of the figure shown below.

(OR)

[P.T.O.]

17.	Determine the overall transfer function of the system whose signal flow graph is shown in figure.

18.	A unity feedback system has the forward transfer function Gs=K2s+1s5s+11+s2. The input r(t)=1 + 6t is applied to the systems. Determine the minimum of K if the steady error is  to be less than 0.1.

(OR)

19.	A unity feedback system has Gs=20s2+5s+5. Find the following values of

a)  	ω n         b)  	ω d           c)  	ξ 	   d)  	td    	e) 	t r 	   f)  	t p           g)  	M p 		h)   t s

20.	Plot the bode diagram for the following transfer function and obtain the gain and phase cross over frequencies Gs=10s1+0.5s1+0.01s.

(OR)

21.	The open loop transfer function of a unity feedback system is given by Gs=1+0.2s1+0.25ss31+0.005s1+0.001s. Sketch the polar plot and determine the phase margin.

22.	Sketch the root locus of the system whose open loop transfer function is Gs=Kss+2s+4. Find the value of K.

(OR)

23.	Illustrate the root locus for the unity feedback system whose open loop transfer function is

Gs=ks2+6s+25ss+1s+2. Determine the K value.

24.	A feedback system has a closed-loop transfer function  YsUs=10s+4ss+1s+3 .  Construct any one of the state model for this system and give block diagram representation for each state model.

(OR)

25.	Convert the following system matrix to canonical form and hence calculate the state transition matrix  eAt. , where   A=41−21021−13  .

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRICAL AND ELECTRONIC INSTRUMENTATION													      									    Time : 3 hours

Subject Code:	11EI201		 									      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The high torque to weight ratio in an analog indicating instrument indicates _____________.

2. 	A PMMC meter rated at 50μA is used in a rectifier type of instrument which uses  Full wave

rectification. What is the sensitivity on sinusoidal a.c.?

3. 	An AC bridge with Z1 =200   ∠30° , Z2 =150  ∠0°   , Z3 =250  ∠40° . In order to balance the bridge the Z4  should be ____________.

4. 	The shunt type ohmmeter is particularly suited for the measurement of

a. 	Low value resistors			b. high value resistors

c.  	Laboratory measurements		d. practical applications

5.  	An instrument which separates an ac signal into its various frequency components and displays each component as a vertical line on a CRT screen is

a.	Frequency meter		b.	CRO	c.	Spectrum analyzer	d.	Wave analyzer

6. 	If the op-amp is connected as a voltage follower, its output impedance is ____________.

7. 	Which oscilloscopes are used for investigation of transient non period phenomena which occur at high voltages?

8. 	When two sinusoidal voltages of equal frequency which are in phase with each other are applied to the horizontal and vertical plates, the pattern appearing on the screen is a _____________.

9. 	What are the two types of LCDs?

10. 	Which recorder records one or more dependent variables with respect to an independent variable?

PART – B  (5 x 3 = 15 MARKS)

11. 	Why PMMC type instrument is not suitable for AC measurement circuits?

12. 	Write short notes on shunt type ohmmeter.

13. 	Write short notes on true RMS reading voltmeter.

14. 	List out the main parts of a CRT.

15. 	Describe the advantages of LEDs.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the construction and working of electro dynamometer type instrument. Derive the deflection expression when operated with DC.

(OR)

17.	Draw the construction of single phase watt hour meter and explain its working principle. Derive the expression for the driving torque in single phase induction type meters.

18. 	Draw the internal structure of a CRT and explain the operation of oscilloscope amplifiers.

(OR)

19. 	Explain in detail about dual trace oscilloscopes and sampling oscilloscopes.

[P.T.O.]

20. 	a.	In a Kelvin double bridge, there is error due to mismatch between the ratios of outer and 	inner arm resistances. The following data relate to this bridge: standard resistance =

100.03 μΩ; inner ratio arms = 100.31 Ω and 200 Ω; outer ratio arms = 100.24 Ω and

200 Ω; the resistance of the connecting leads from standard to unknown resistance is

700 μΩ. Calculate the unknown resistance.										    (5)

b.	A bakelite sheet of 5 mm thickness is tested at 50 Hz between the electrodes 12 cm in 	diameter. The Schering bridge used has an air capacitor C3 of 106 pF, a non-reactive 	resistance of R1 of 319 Ω in parallel with a variable capacitor C1 and a non-reactive 	variable resistance R2. Balance is obtained with C1 = 0.55µF and R2 = 270 Ω. Determine 	the following: 	i.	Capacitance  		ii.	Power factor 		iii.	Relative permittivity of 	the 	sheet. [Note: 	permittivity of free space is 8.854 x 10-12]						  (10)

(OR)

21. 	In an Anderson bridge for the measurement of inductance the arm AB consists of an unknown impedance, a known variable resistance in arm BC, fixed resistance of 600Ω each in arms CD and DA, a known variable resistance in arm DF, and capacitor with fixed capacitance of 1µF in the arm CF. The a.c. supply of 100Hz is connected across A and C, and the detector is connected between B and F. If the balance is obtained with a resistance of 400Ω in the arm DF and a resistance of 800Ω in the arm BC, calculate the value of unknown R and L. Derive the conditions for balance.

22.	Draw the block diagram of the swept super heterodyne spectrum analyzer and explain the method of analysis with a necessary sketch.

(OR)

23.	a.	Explain the working of distortion meters with necessary diagrams.    				    (5)

b.	Sketch the block diagram of vector impedance meter. Explain its working in detail. 	  (10)

24. 	Explain the working of a ultra-violet recorder with neat sketch. Describe its applications.

(OR)

25. 	Explain the theory and working of an LCD. Describe the advantages and disadvantages of LCDs.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DIGITAL SIGNAL PROCESSING						      Time : 3 hours

Subject Code:	11EI202		 								       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the advantages of digital signal processing over analog signal processing.

2.	What is the need for FFT?

3.	What is the relation between analog and digital frequency in Bilinear Transformation Method?

4.	Mention the methods for IIR filter design.

5.	The filter represented by the equation  y[n]=−x[n]+2x[n−1]+3x[n−2]

is a ___________ system. (IIR/FIR).

6.	A stable FIR filter has __________ phase and __________ delay.

7.	Mention the advantage of Harvard architecture.

8.	What is multiple access memory?

9.	Which addressing mode is convenient for FFT computation?

10.	What is the function of MAC instruction?

PART – B  (5 x 3 = 15 MARKS)

11.	Bring out the differences between DIT and DIF algorithms.

12.	What is pre-warping in the design of IIR digital filters using Bilinear Transformation method?

13.	Enumerate the steps involved in the design of FIR digital filters using windows.

14.	What is pipe lining with reference to a digital signal processor?

15.	Mention the functions of Parallel Logic Unit.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the 8-point DFT of the sequence x(n) = 1   0 ≤ n ≤ 7

0	Otherwise

using DIT FFT algorithm

(OR)

17.	a. 	State and prove the periodicity property of DFT

b. 	Find the 4 point DFT of x[n] = {1, 2, 1, 1}.

18.	The system function of the analog filter Ha(s) using bilinear transformation method. Assume

T = 0.1sec.

Ha(s)=2s2+3s+2.

(OR)

19.	Obtain the direct form I, II and cascade realizations of the LTI system governed by the equation

y(n)=−38y(n−1)+332y(n−2)+164y(n−3)+x(n)+3x(n−1)+2x(n−2)

20.   Design a stable Low Pass FIR Filter using Rectangular window with a cut-off frequency of

1.2 rad/sample, N=9.

(OR)

[P.T.O.]

21. 	a.	Obtain the structure for the system function

H(z) = 0.5 + 0.75z1 + z2 + 0.25z3 + z4 + 0.75z5 + 0.5z6 using Linear Phase Realization.

b.	Obtain cascade realization and Direct Form  structure for H(z) = (1+2z1–z2) (1+z1–z2).

22.	Write a note on Instruction Pipelining and explain how it improves the throughput efficiency of the processor.

(OR)

23. 	a. 	Bring out the differences between CISC and RISC architecture.

b. 	Discuss about the different memory blocks in TMS processor and their function.

24. 	With a neat diagram, explain each functional block found in the architecture of TMS320c5X processor.

(OR)

25. 	Write a note on the addressing modes in TMS processor with examples.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INSTRUMENTATION AND PROCESS CONTROL FOR FOOD ENGINEERS											      Time : 3 hours

Subject Code:	11EI215				 							       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define measurement.

2.	What is a Transient Response?

3.	What is Proportional band?

4.	What is the advantage of closed loop system?

5.	What is the region of stability in S plane?

6.	What is the use of Block diagram algebra?

7.	Thermistor has ___________ temperature coefficient.

8.	What is the use of free end in a Bourdon gauge?

9.	What are the principles used for level measurement with capacitive sensing element.

10.	pH of distilled water is __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the block diagram of an automatic Process control loop.

12.	What is a two way control? Give some practical applications.

13.	Explain the BIBO concept of stability.

14.	Explain the working of a diaphragm.

15.	Explain level measurement using float.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Explain controllers and feedback control system with suitable example.				(10)

b. 	Brief about damped and cyclic response.										(5)

(OR)

17.	Obtain the Laplace transform of standard input signals.

18.	Explain the construction and working of pneumatically operated control valve.

(OR)

19.	a. 	Explain the concept of servo mechanism.										(5)

b. 	Brief about open and closed loop control system with example.					(10)

20.	Using block diagram reduction obtain the Transfer function for the system shown:

[P.T.O.]

-

G1

G2

G3

G4

H1

H2

H4

H3

+

+

+

+

+

-

-

-

R(s)

C(s)

(OR)

21.	The open loop transfer function of a unity feedback control system is given by,

Gs=KS+2S+4S2+6S+25

Determine the value of K which will cause sustained oscillation in the closed loop system using stability criteria using Routh array.

22.	a. 	Explain the construction and working of Inductive pressure transducer.				  (10)

b. 	Write short notes on bourdon gauge	.											    (5)

(OR)

23.	a. 	Explain the working of Thermocouple in detail.									  (10)

b. 	State the laws of thermocouple.												    (5)

24.	Explain the working of gas chromatography in detail with a neat sketch.

(OR)

25.	Explain various types of density measurement.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	C++ AND DATA STRUCTURES							      Time : 3 hours

Subject Code:	11EI217											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Evaluate !(1 && !(0 || 1)).

2.	What is the output of given code fragment?

int f=1, i=2;

while(++i<5)

f*=i;

cout<<f;

3.	State the use of void in C++.

4.	If an array is declared as int a[4] = {3, 0, 1, 2}, then values assigned to a[0] and a[4] will be _____________.

5.	How many constructors can a class have?

6.	The process of building new classes from existing one is called _____________.

7.	_____________ is very useful in situation when data have to stored and then retrieved in reverse order.

8.	Each node in a linked list has two pairs of _____________ and _____________.

9.	The complexity of Bubble sort algorithm is _____________.

10.	ios::ate is used to _____________.

PART – B  (5 x 3 = 15 MARKS)

11.	What are symbolic constants?

12.	Consider the following statements

char *ptr;

ptr = “hello”;

cout << *ptr;

What will be printed?

13.	What is the ambiguity between default constructor and default argument constructor?

14.	Define Height of tree.

15.	What is insertion sort? How many passes are required for the elements to be sorted?

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Explain Decision making statements in C++ with suitable examples.

b. 	Illustrate the working of inline functions.

(OR)

17.	a. 	Rewrite the following program after removing the syntactical error(s) if any. Underline 	each correction.

#include<iostream.h>

void main( )

{ F = 10, S = 20;

test(F;S);

test(S);

}

[P.T.O.]

void test(int x, int y = 20)

{ x=x+y;

count<<x>>y;

}

b. 	Find the output of the following program:

#include<iostream.h>

void main( )

{ int U=10,V=20;

for(int I=1;I<=2;I++)

{ cout<<”[1]”<<U++<<”&”<<V 5 <<endl;

cout<<”[2]”<<++V<<”&”<<U + 2 <<endl; } }

18. 	Discuss in detail about the memory management operators in C++.

(OR)

19. 	Write a C++ program that copies the elements from one array into another and uses pointer notation throughout in place of subscripts.

20. 	Create two classes DM and DB which stores the value of distances. DM stores distances in metres and centimetres and DB in feet and inches. Write a program that can read values for the class objects and add one object DM with another object of DB. Use friend function.

(OR)

21. 	Define a class Publication which has a title. Derive two classes from it – a class Book which has an accession number and a class Magazine which has volume number. With these two as bases, derive the class Journal. Define a function print() in each of these classes. Ensure that the derived class function always invokes the base(s) class function. In main() create a Journal called IEEEOOP with an accession number 681.3 and a volume number 1. Invoke the print() function for this object.

22. 	Evaluate the following postfix expression showing stack status after every step

8, 2, +, 5, 3, , *, 4 /

(OR)

23. 	Write a C++ program to implement a Circular Queue.

24. 	Show the steps involved in sorting the following list using quick sort technique.

(38 81 22 48 13 69 93 14 45 58 79 72)

(OR)

25. 	Discuss in detail about classes in C++ that are used to perform output and input of characters to/from files.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CIRCUIT ANALYSIS AND NETWORKS				      Time : 3 hours

Subject Code:	12EI201		 								       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What happens to the equivalent resistance when a resistance is added in parallel to the existing circuit?

2. 	How many loop equations are needed to solve a circuit having 'n' nodes and 'b' branches?

3. 	When does maximum power occur across the load?

4. 	Draw the Thevenin's equivalent circuit.

5. 	What is the Phase relationship between voltage and current in a capacitor and an inductor?

6. 	Define time constant.

7. 	Define mutual inductance.

8. 	Draw the frequency response of single tuned circuit.

9. 	What are the dependent and independent variable in hybrid parameter?

10. 	What is passive filter?

PART – B  (5 x 3 = 15 MARKS)

11.	State voltage division rule and current division rule.

12. 	State Norton's equivalent circuit.

13. 	Draw the graph of an impedance for the element R, L and C as a function of frequency.

14. 	Derive the equation for coefficient of coupling.

15. 	How do you convert T network into π  network and vice versa?

PART – C  (5 x 15 = 75 MARKS)

16. 	For the network shown in the figure obtain the mesh currents.

(OR)

17.	a.	Find the Equivalent capacitance in the figure.									(5)

b.	What is required voltage to produce 7.5 mA current in 6 ohm resistor shown in the figure? 																					(10)

[P.T.O]

18.	Using superposition theorem determine the current in 8 ohm resistor in the figure.

(OR)

19.	Find the maximum power that can be delivered to the variable resistor R in the figure.

20. 	Derive the response of a step input across C in series RC circuit.

(OR)

21. 	a.	What is pole-zero plot? 														    (4)

b.	Find the  pole-zero plot of the input impedance of the circuit.

H(s)=(s+2)(s2+4s+3)														    	    (4)

c.	What is free response and forced response?										    (7)

22. 	a.	Write down the dot rules applied to coupled circuits.								    (3)

b.	Compute the voltage V for the coupled circuit shown in the figure. 					  (12)

(OR)

23. 	Derive the voltage gain of a single tuned circuit and draw the frequency response.

24.	Find the Y-parameter of the circuit shown in the figure.

(OR)

25.	Explain how the RC circuit acts as low pass filter and high pass filter with transfer function and its frequency response.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRICAL MACHINES								      Time : 3 hours

Subject Code:	12EI202										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write down the emf equation for a d.c. generator.

2.	Distinguish between shunt and series field coil construction.

3.	How transformers are classified according to their construction?

4.	What is the purpose of laminating the core in a transformer?

5. 	List out the two types of three phase induction motors.

6. 	What are the advantages of salient type pole construction used in synchronous machines?

7. 	What is the function of capacitor in single phase induction motor?

8.	Define Step angle.

9. 	What are the advantages of induction heating?

10. 	Define Electric traction.

PART – B  (5 x 3 = 15 MARKS)

11.  	Under what circumstances does a dc shunt generator fail to generate?

12.  	Define voltage regulation of a transformer.

13.  	State the difference between slip ring rotor and cage rotor of an induction motor.

14.  	Discuss in brief about the repulsion motor.

15.  	List out the advantages of electric heating.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A 4-pole dc motor is lap wound with 400  conductors. The pole  shoe is 20 cm long and 	average flux density over one-pole is 0.4 T, the armature diameter being 30 cm. Find  the 	torque and gross mechanical power developed when the motor is drawing 25 A and 	running at 1500 rpm.															  (10)

b.	Discuss the characteristics of DC Shunt Generator in detail.						   (5)

(OR)

17.	a. 	A 400 V dc shunt motor takes a current of 5.6 A on no-load and 68.3 A on full-load. 	Armature reaction weakens the field by 3%. Calculate the  ratio of full-load speed to no-	load speed. Given Ra = 0.18 ohm, brush voltage drop =  2 V, Rf = 200 ohm.			 (10)

b. 	Describe in detail about the applications of a DC motor.							  (5)

18.	Explain the construction, principle and operation of a Transformer.

(OR)

19.	Derive an expression of EMF equation of  a Transformer.

20.	Explain in detail about the Torque – Slip characteristics of an induction motor with a neat sketch.

(OR)

21.	Explain the method of starting of synchronous motor with its applications.

22.	Briefly describe about the principle and operation of stepper motor with its applications.

[P.T.O.]

(OR)

23.	Describe the working of a shaded pole motor with a neat sketch.

24.	Explain the principle of operation and constructional details of a welding generator.

(OR)

25.	a. 	Discuss the conventional and recent trends in electric traction in detail.				  (10)

b. 	List out the requirements of an ideal traction system.								   (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	THERMODYNAMICS AND FLUID MACHINERY		      Time : 3 hours

Subject Code:	12EI204											      Maximum Marks: 100

(Use of Steam tables, Heat and Mass Transfer Data Book is permitted)

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define state.

2.	What is meant by heat engine?

3.	Define convection.

4.	Define emissive power.

5.	What is meant by Regenerators?

6.	What is boiler draught?

7.	Define kinematic viscosity.

8.	Mention the use of piezometer.

9.	Define rotational flow.

10.	Give any one application of Pelton wheel turbine.

PART – B  (5 x 3 = 15 MARKS)

11.	State the Kelvin – Plank statement of second law of thermodynamics.

12.	List the significant factors that affect the thermal conductivity.

13.	Define boiler efficiency.

14.	State Newton’s law of viscosity.

15.	Differentiate between laminar and turbulent flow.

PART – C  (5 x 15 = 75 MARKS)

16.	 In an air compressor air flows steadily at the rate of 0.5 kg/s through an air compressor. It enters the compressor at 6 m/s with a pressure of 1 bar and a specific volume of  0.85 m3/kg and leaves at 5 m/s with a pressure of 7 bar and a specific volume of  0.16 m3/kg. The internal energy of the air leaving is 90 kJ/kg greater than that of the air entering and cooling water in a jacket surrounding the cylinder absorbs heat from the air at the rate of 60 kJ/s. Calculate :

a.	The power required to drive the compressor 		b.	The inlet and output pipe cross-sectional areas.

(OR)

17.	A heat pump working on a reversed Carnot cycle takes in energy from a reservoir maintained at 5°C and delivers it to another reservoir where temperature is 77°C. The heat pump derives power for its operation from a reversible engine operating within the higher and lower temperatures of 1077°C and 77°C. For 100 kJ/kg of energy supplied to reservoir at 77°C, estimate the energy taken from the reservoir at 1077°C.

18.	A furnace wall is of three layers, first layer of insulation brick of 12 cm thickness of conductivity 0.6 W/mK. The face is exposed to gases at 870°C with a convection coefficient of 110 W/m2K. This layer is backed by a 10 cm layer of firebrick of conductivity 0.8 W/mK. There is a contact resistance between the layers of 2.6 × 10–4 m2 °C/W. The third layer is the plate backing of 10 mm thickness of conductivity 49 W/mK. The contact resistance between the second and third layers is  1.5 × 10–4 m2 °C/W.

[P.T.O.]

The plate is exposed to air at 30°C with a convection coefficient of 15 W/m2K. Determine the heat flow, the surface temperatures and the overall heat transfer coefficient.

(OR)

19.	Engine oil at 60°C flows over a flat surface with a velocity of 2 m/s, the length of the surface being 0.4m. If the plate has a uniform heat flux of 10 kW/m2, determine the value of average convective heat transfer coefficient. Also find the temperature of the plate at the trailing edge.

20. 	During the boiler trial for 24 hours following observations were made.

Steam generation: 16 bar, dry saturated.

Coal consumed: 10,000 kg

Rate of steam generation: 2500 kg/hr

Feed water temperature: 27oC

Total heating surface area: 3000 m2

Total grate area: 4m2

Calorific value of coal: 28000 kJ/kg

Determine:

a.	the mass of coal burnt per m2 of grate per hour 		b.	the equivalent evaporation from and at 100oC per kg of coal 		c.	the equivalent evaporation from and at 100oC per m2 of total heating surface per hour 		d.	the boiler efficiency.

(OR)

21.  Water flows at a velocity of 1 m/s through a pipe of 25 mm inner diameter and 30mm outer 	diameter and 3 m length. The air at 30°C flows across the tube, with a velocity of 12 m/s. The 	inlet temperature of the water is 60°C. Determine the exit temperature. The thermal 	conductivity of the tube material is 47 W/mK.

22.  	Describe the working principle of U- tube manometer with schematic diagram.

(OR)

23. 	With a neat sketch, explain the working principle of centrifugal pump.

24. 	a.	Discuss the concept of uniform flow, reversible flow and three dimensional flow. 	  (10)

b.	Draw the velocity diagram for a Pelton turbine.			 						    (5)

(OR)

25. 	Elucidate the working principle of Kaplan turbine with suitable sketches.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRICAL MEASUREMENTS						      Time : 3 hours

Subject Code:	12EI208				 					       		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The deflection torque is equal to __________ torque  at balanced condition in meters.

2.	__________ error in a meter is most harmful.

3.	The deflection torque equation of PMMC instrument is __________.

4.	Mention the need for calibration of meters.

5.	What is creeping in energy meter?

6.	Define power factor.

7.	A Kelvin’s double bridge is used for measuring __________ range resistor.

8.	Mention the need for current transformer in measurements.

9.	The Galvanometer instrument is __________ deflection at balanced condition.

10.	List the need for potentiometer circuit in measurements.

PART – B  (5 x 3 = 15 MARKS)

11.	Name the different static characteristics of an instrument and explain any one type.

12.	Differentiate between MI instruments with MC.

13.	What is potential transformer? Mention its applications.

14.	Differentiate between Anderson’s and Schering bridges

15.	Differentiate between MOS and CMOS.

PART – C  (5 x 15 = 75 MARKS)

16.	Enumerate the static and dynamic characteristics of an instrument. Also mention how these characteristics useful for an instrument design.

(OR)

17.	Explain the functional block of measurements, also mention its needs.

18.	Derive Td and TC of a PMMC instrument. Also explain its construction.

(OR)

19. 	Explain the construction and working principle of any two types of digital voltmeters.

20.	Explain the construction and working of induction type energy meter. Also mention its merits and demerits.

(OR)

21.	Explain the construction details of moving iron instrument with neat diagram.

22.	Elaborate on the Anderson bridge to estimate the unknown capacitance.

(OR)

23.	Explain the Kelvin’s double bridge to find the unknown resistance and derive the bridge 	sensitivity.

24.	Explain the construction details of X-Y recorder with neat block diagram.

(OR)

25.	Explain the construction details of magnetic tape recorder with neat sketches.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	SIGNALS AND SYSTEMS								      Time : 3 hours

Subject Code:	12EI209				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define CT signals.

2.	What do you mean by a causal signal?

3.	Write the general difference equation relating input and output of a system.

4.	Define Transfer function of the DT system.

5.	What do the Fourier series coefficients represent?

6.	What is the Fourier series of an impulse function?

7.	What is the drawback in DTFT?

8.	State sampling theorem.

9.	Define z transform.

10.	Define poles.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the fundamental period of the signal xn=3ei3πn+12.

12.	What are the properties of convolution?

13.	State Dirichlet conditions for Fourier series.

14.	Define aliasing. What is the condition for avoid the aliasing effect?

15.	What are the properties of ROC?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain all classification of signals with examples for each category.

(OR)

17.	Check whether the following systems are linear or not:

a.	y(t)=ex(t)		b.	y(t)=x2(t)		c.	y(t)=t2 x(t)

18.	Using the classical method, solve the differential equation

if the initial conditions are y(0+) = 9/4; = 5  and if the input is e−3tu(t).

(OR)

19.	Find the convolution of the following signals

a.	x(n)={1, 0, –1, 2, 1}; 	h(n)={1, 2, –1, 2}

b.	x(n)= n/2     0 ≤ n ≤ 5;             h(n) = 1     –3 ≤ n ≤ 3

= 0        otherwise                     = 0       otherwise

20.	Find the cosine Fourier series of half wave rectified sine function.

(OR)

21.	State and prove the properties of Continuous time Fourier Series.

22.	State and prove the properties of DTFT.

[P.T.O]

(OR)

23.	Find the Fourier transform of  the following signals

a.	x(t)=e−2tu(t−1)			b.	Sgn(t)			c.	u(t)

24.	Using the Laplace transform, find the transfer function of the following.

a.	An ideal differentiator.			b.	An ideal integrator.

c.	An ideal delay of T seconds.

(OR)

25.	Find Z-Transform of the following:

a.	x(n)=n an u(n).			b.	x(n)=2nsin u(n).		c.	x(n)=n2 u(n).

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DIGITAL ELECTRONICS								      Time : 3 hours

Subject Code:	12EI211				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The ____________ gate has an output which is the inverse of the input.

2.	The acronym for the TTL is given as ____________.

3.	The mathematical technique used for logical analysis of systems containing large numbers of ON-OFF elements is called as ____________.

4.	A ____________ truth table makes use of the fact that, if we have Ntwo-state variables, then there are 2Ndifferent ways of combining them.

5.	A ____________ is a device taking a single input signal and selecting one of many data-output lines, which is connected to the single input.

6.	A ____________ bit is a bit added to the end of a string of binary code that indicates whether the number of bits in the string with the value one is even or odd.

7.	____________ data transfer means that all bits in a given group are transferred simultaneously on separate lines.

8.	Commercially available adders such as the TTL 7483 and the CMOS 1D4008 4-bit full adders have ____________ circuits included on the chip.

9.	____________ has become a generic term for any semiconductor memory that can be written to as well as read from also.

10.	The FPGA configuration is generally specified using a ____________ language.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the circuit diagram of CMOS inverter for a NOT gate.

12.	Give the significance of error detecting and correcting codes.

13.	What do you understand from Parity generator/Checker?

14.	Differentiate between Asynchronous and Synchronous Counters.

15.	How FPGA is useful as a programmable logic device?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Write short notes on Digital and Analog Quantities. 								    (7)

b.	Briefly explain about the basic electronic concepts of digital design. 				    (8)

(OR)

17.	Give a brief performance comparison about the various logic families.

18.	a.	Give a short review of Binary, Octal and Hexa-decimal number system.  			    (8)

b.	Describe the significance of Excess 3 codes and Gray codes. 						    (7)

(OR)

19.	In a certain corporation the board members own all the stock, which is distributed as follows:

A owns 45 percent

B owns 30 percent

C owns 15 percent

D owns 10 percent

[P.T.O.]

Each member has a percentage vote equal to his holdings, and a total vote greater than 50 percent is required to pass a motion. Now design an electronic voting system for the corporation. In the board room each member is to have a switch with which to indicate a YES or NO vote. A lamp is to light if the total vote cast is more than 50 percent, indicating that the measure being voted on is passed.

a.	Write the truth table.

b.	Convert the truth table into a Karnaugh map.

c.	Draw the logic circuit using NAND gate and NOR gate only.

20.	a.	Elucidate the implementation of encoders and decoders in combinational logic design. (10)

b.	What are code-converters? Explain its significance. 								    (5)

(OR)

21.	a.	How is the implementation of logical functions using multiplexers done? 			    (8)

b.	Write short notes on parity checker and generator. 								    (7)

22.	a.	Using four D type flip-flops, design a 4-bit register which can be used for parallel-in 	parallel-out data transfer. Show the four input and four output waveforms when the data 	word 1101 is transferred in at clock pulse 1 and out at clock pulse 5. 				  (10)

b.	Briefly explain about the concept of Shift registers. 								    (5)

(OR)

23.	With suitable diagrams and explanations, explain the concept of asynchronous and synchronous counters.

24.	a.	With a suitable diagram, explain the construction and working of a PAL. 		 	  (12)

b.	Write down the practical applications of PLA. 									   (3)

(OR)

25.	a.	How FPGA is used as Programmable Logic Device? 								  (10)

b.	Write short notes on RAM and ROM. 											    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	C++ AND DATA STRUCTURES							      Time : 3 hours

Subject Code:	12EI212										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define Data structures.

2. 	In Binary search tree left sub tree of a node contains only nodes with keys strictly _______ than the node's key.

3. 	Pivot element is selected in which sort?

4.	_________ search checks each element in sequence until the desired element is found or the list is exhausted.

5. 	List the types of constructors.

6. 	Member Functions can be defined outside the Class by using _________ operator.

7. 	What is the output of the following code?

#include <iostream>

using namespace std;

int main ()

{

char St[6] = {'H', 'e', 'l', 'l', 'o', '\0'};

cout << St << endl;

return 0;

}

8. 	_________ keyword is used for operator overloading

9. 	Static function is defined by using the keyword _________.

10. 	A pure virtual function is one with the expression _________ .

PART – B  (5 x 3 = 15 MARKS)

11. 	List some applications of Stack.

12. 	Write the result of the first pass for the following input sequence  "5 1 4 2 8" using Bubble sort.

13. 	Write notes on constructor with example.

14. 	What is Inheritance?

15. 	Give an example for abstract class.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain in detail about the operations of Queue.

(OR)

17. 	With suitable example explain deletion operation in Binary search Tree.

18. 	Describe the Merge sort with example.

(OR)

19. 	Write the Binary search procedure and explain its function in detail.

20.  	How will you pass objects as function arguments explain with example?

[P.T.O.]

(OR)

21. 	a.	Explain Two Dimensional array with example.									 (10)

b.	Write an example to show how to read strings with embedded blanks and multiline 				strings.																		    (5)

22. 	Write a C++ program to perform addition, subtraction and multiplication of complex numbers using operator overloading

(OR)

23.	Explain different types of Inheritance with example.

24.	a.	Write notes on Abstract Classes and Pure Virtual Functions.		 			  	    (8)

b.	Explain Friend function with example.										 	    (7)

(OR)

25. 	a.	Write notes on ‘This’ pointer.								  					    (8)

b.	Write in detail about Stream Errors.							  					    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	COMMUNICATION ENGINEERING					      Time : 3 hours

Subject Code:	12EI215										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define modulation index in amplitude modulation.

2.	Draw the block diagram for generating single sideband suppressed carrier modulation using 	filter method.

3.	What is the difference between low level and high level transistors?

4.	TRF stands for __________.

5.	PSK is another form of angle modulated constant amplitude digital modulation. True or False. 6.	Write the other name of amplitude shift keying.

7.	__________ is the most prevalent encoding technique used for TDM digital signals.

8.	What is error detection?

9.	Write the aspect ratio.

10.	Write the expression for maximum radar range.

PART – B  (5 x 3 = 15 MARKS)

11.	A transmitter radiates 9KW without modulation and 10.125 KW after modulation. Determine depth of modulation.

12.	Draw the block diagram for basic radio transmitter.

13.	Determine:

a.	the peak frequency deviation

b.	minimum bandwidth

c.	baud for a binary FSK signal with a mark frequency of 49kHz, a space frequency of 51 kHz and an input bit rate of 2kbps.

14.	Write the disadvantages of optical fiber cables.

15.	List the devices used by modern TV camera systems for conversion of picture signals into electrical current.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss the reasons for which modulation is needed in communication system.		    (5)

b.	Explain the generation of double side band suppressed carrier amplitude modulation using 	balanced modulator.															  (10)

(OR)

17.	With a neat block diagram explain independent side band modulator.

18.	With a neat block diagram explain super heterodyne receiver.

(OR)

19.	Discuss about the types of noise.

20.	Explain the transmitter and receiver of pulse code modulation system.

(OR)

21.	With a neat block diagram explain delta modulation transmitter and receiver.

[P.T.O]

22.	Explain the components of fiber optic communication link with a block diagram.

(OR)

23.	Discuss the error correction mechanisms in detail.

24.	Explain about satellite system link models with suitable diagrams.

(OR)

25.	Draw the block diagram for monochrome TV receiver and explain the components.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	SIGNAL CONDITIONING CIRCUITS					      Time : 3 hours

Subject Code:	12EI216		 								       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A mathematical operation for finding the area under the curve of a graph is called _________.

2.	An ideal operational amplifier has

a. 	infinite output impedance				b. 	zero input impedance

c. 	infinite bandwidth					d. 	All of the above

3.	What is the output waveform?

a. 	sine wave		b. 	square wave		c. 	triangular wave		d. 	saw tooth wave

4.	If the two inputs to a differential amplifier are exactly the same, then the output is the signal multiplied by two.

a. 	True			b. 	False

5.	If the input to a comparator is a sine wave, the output is a:

a. 	ramp voltage		b. 	sine wave	c. 	rectangular wave		d. 	sawtooth wave

6.	A Howland current source is one that is a:

a.	Voltage to current converter			b.	Current to voltage converter

c.	An isolation amplifier					d.	An ideal Op – Amp

7.	An astable 555 timer has the following number of stable state

a. 	0		b. 	2		c.	3		d.	1

8.	The important feature of instrumentation amplifier

a. 	high CMRR					b. 	high gain accuracy

c. 	high gain stability			d. 	all the above

9.	Capture range of a PLL is:

a. 	greater than its lock range		b. 	less than its lock range								c. 	equal to its lock range			d. 	none of these

10.	How many op-amps are required to implement this equation?

V0 = V1

a. 	2	b.	3	c.	1	d.	4

PART – B  (5 x 3 = 15 MARKS)

11.	For the op – amp configuration shown in figure, determine the Rf if the gain required is 61.

[P.T.O]

12.	Explain the function of op – amp as integrator, draw the waveforms

13.	Analyze the 1st order Butterworth filter with next diagram.

14.	Give the Advantages of 723 general IC regulator.

15. 	How PLL can act as FSK Demodulator.

PART – C  (5 x 15 = 75 MARKS)

16.	An Op – Amp has a differential gain of 80dB and CMRR 0f 95dB. If V1 = 2µV and

V2 = 1.6µV, then calculate the differential and common mode output values.

(OR)

17. 	Find the output voltage V0 in terms of V1 and V2 for the op – amp circuit shown in the figure. Assume ideal Op- amps.

18. 	Explain in detail about clipper and clamper circuit and its types with neat circuit diagram and its wave forms.

(OR)

19. 	Design a circuit to give an output V0  = 2V1 – 3V2 + 4V3 – 5V4 (assume Rf1 = 100KΩ)

20. 	Briefly write about band elimination filter and explain the types with neat sketch.

(OR)

21. 	a. 	For instrumentation amplifier shown in figure determine the value of RG if required gain is 		1000. 																		(10)

b. 	Design a low pass filter with a cut off frequency of 15.9 kHz with a pass band gain of 1.5.																						(5)

22.	Give any three applications of IC 723 regulator with necessary circuit diagrams.

(OR)

23. 	a. 	Explain the phase angle detection circuit.  										(8)

b. 	Give out the applications of multiplier circuit.									(7)

24. 	Explain the working principle of Astable multivibrator using IC 555 timer with derivation of Duty cycle.

(OR)

25. 	Explain the theory and working of Analog phase detector/comparator in PLL.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INDUSTRIAL INSTRUMENTATION					      Time : 3 hours

Subject Code:	12EI218				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the unit for pressure.

2.	Mention the use of calibration.

3.	Write the necessity for standards.

4.	Name one standards Institute in India.

5.	How do you measure the temperature of sun?

6.	What is Seebeck effect?

7.	How does a mason measure level during construction?

8.	Define eddy current.

9.	Give the unit of viscosity.

10.	Define range of an instrument.

PART – B  (5 x 3 = 15 MARKS)

11.	Describe dynamic testing of pressure measuring system.

12.	Describe flow visualization.

13.	Describe the principle of electrical resistance sensors.

14.	What is an air bellow and mention its use.

15.	Differentiate density and viscosity.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the following:

a.	High pressure measurement.

b.	Low pressure measurement.

(OR)

17.	Explain in detail about capacitance type and Piezoelectric pressure measurement.

18.	Describe in detail about:

a.	Laser Doppler Anemometer.

b.	Hot Film Anemometer.

(OR)

19.	Describe in detail about

a.	Drag force flow meter.

b.	Ultrasonic flow meter.

20.	With a neat diagram explain the automatic null balance radiation thermometer.

(OR)

21.	With a neat diagram explain the fiber optic radiation thermometer.

22.	Explain the hook level indicator direct method and pressure gauge indirect method for level measurement.

(OR)

[P.T.O.]

23.	Draw and explain the following:

a.	Microwave level switch.

b.	Optical level detectors.

24.	Draw and explain the following:

a.	Magnetic type density measurement.

b.	Weight method of density measurement.

(OR)

25.	Draw and explain the following:

a.	Efflux cup viscometers.

b.	Industrial viscometers.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	PROCESS DYNAMICS AND CONTROL				Time: 3 hours

Subject Code:	12EI219			       						        	Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What do you understand from self regulating process?

2.	Define Manipulated variable.

3.	Give any two applications where two position control mode is best suited.

4.	What do you mean by Integral wind up?

5.	Give the expression for ITAE.

6.	Process reaction curve method is otherwise referred as ___________ method.

7.	What is final control element?

8.	Pneumatic control valves have high speed of operation. True / False.

9.	What do you mean by ratio control?

10.	What are the load variables that are measured for feed forward control?

PART – B (5 x 3 = 15 MARKS)

11.	List the uses of mathematical modeling of processes.

12.	Explain reset action mode.

13.	What are the performance criteria to be considered to achieve an optimum response for a particular process?

14.	Explain how flashing occurred in control valves.

15.	Draw the boiler drum level control system and give the load variables.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the transfer function for non interacting two tank system. Assume necessary 	variables required.									       						(7)

b.	A tank system having a time constant of 0.5 min and a resistance of 0.25 min/m2 is 	operating at steady state with an inlet flow of 2 m3/min. The flow is suddenly increased to 	3 m3/min. Plot the response of the tank level. Assume cross section area of 2 m2.	       (8)

(OR)

17.	a.	Derive the transfer function for mercury thermometer. Assume necessary variables 	required.										       								(7)

b.	A Thermometer having a time constant of 0.5 min is placed in a temperature bath and after thermometer comes to equilibrium with the bath, the temperature of the bath is Ti which increases linearly at the rate of 10C/min. What is the difference between the indicated and bath temperatures if:

i.	0.25 min after the change in temperature begins. 		ii.	3 min after the change in temperature begins. 	iii.	What is the maximum deviation between the indicated and bath temperature and when does it occur?		iv.	How many minutes does the response lag after long enough time is elapsed.			       									(8)

18.	Explain in detail about the following control modes:								    (3x5)

a.	Multi position control.		b.	Floating control (Single Speed).						c.	Floating control (Multiple Speed).

[P.T.O]

(OR)

19.	a.	An Integral controller is used for speed control with a set point of 12 rpm within a range of 	10 to 15 rpm. The controller output is 22% initially. The constant KI = 0.15% controller 	output per second per percentage error. If the speed jumps to 13.5 rpm, calculate the 	controller output after two seconds for a constant ep.			       					(7)

b.	Explain the transient response characteristics of the continuous controllers if the input of 	unit step is applied to it.								       						(8)

20.	a.	In an application of the Ziegler-Nichols method, a process begins oscillation with a 30% 	proportional band in an 11.5-min period. Find:	i.	the nominal three-mode controller 	settings 		ii.	settings to give quarter amplitude response.				       		(8)

b.	Discuss in detail about the Time Integral performance criterions.		       			(7)

(OR)

21.	a.	Discuss the Cohen- Coon method of controller tuning in detail.		       	       		(7)

b.	A PI controller indicates an output of 12 mA when the error is zero. The set point is 				suddenly increased to 14mA and the controller output is recorded and is given below. Find 		KP and TI.

| Time t (sec) | 0 | 10 | 20 | 30 |
| --- | --- | --- | --- | --- |
| Output (mA) | 14 | 16 | 18 | 20 |

(8)

22.	Discuss the factors that are to be considered for selecting the control valves.

(OR)

23.	a.	The vapour pressure of the liquid flowing through a control valve is given as 0.6 Kg/cm2. 		If the stream pressure is 0.5 Kg/cm2, is there a possibility:		i.	for cavitation to occur 	ii.	for flashing to occur 		iii.	What should be the upstream pressure for making the 	condition to 	switch over from one state to another state if the pressure recovery ratio is 	0.8?																	 		(6)

b.	Discuss the inherent characteristics of control valves.				       				(9)

24.	Explain the split range control in detail and discuss how it can be applied to control the pressure of a chemical reactor.

(OR)

25.	With an appropriate example explain the cascade control scheme and discuss the selection and tuning of this control algorithm.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRONIC INSTRUMENTATION					      Time : 3 hours

Subject Code:	12EI223				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In which form, a vector impedance meter determines impedance?

2.	How is the sensitivity of a voltmeter defined?

3.	What is the power requirement of an LED?

4.	Mention the use of an aquadag in a CRO.

5.	What is frequency synthesizer?

6.	Define harmonic distortion.

7.	What is the technique used in potentiometric type digital voltmeter?

8.	In a digital frequency meter, what is the use of Schmitt trigger?

9.	What is created if virtual presentation is combined with real world objects?

10.	How is the graphical code translated into the executable machine code in LabVIEW?

PART – B  (5 x 3 = 15 MARKS)

11.	What is the need for determining the storage factor in a Q meter?

12.	What are the advantages of LCDs?

13.	What is spectrum analyzer? What are its types?

14.	How can capacitance be measured using a digital capacitance meter?

15.	List the advantages of virtual instrument over a traditional instrument.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the measurement of DC and AC voltages and resistance using an Electronic     Multimeter.

(OR)

17.	With the help of a neat block diagram, explain the working of a vector impedance meter.

18.	Draw the block diagram of a general purpose oscilloscope and explain the principle.

(OR)

19.	Describe FM recording with a diagram and waveform. Explain its advantages and disadvantages.

20.	Describe the working of a sweep-frequency generator. What are the sweep errors?

(OR)

21.	a. 	Draw the basic block diagram of a function generator and explain. 					  (10)

b.	What is a wave analyzer? State its applications. 									    (5)

22.	What are the different types of DVM? Explain any two.

(OR)

23.	Describe with a block diagram, the working of a digital counter.

24.	Define virtual instrumentation. Explain about the architecture of virtual instrumentation.

(OR)

25.	Explain the various steps involved in creating virtual instruments using LabVIEW.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DIGITAL SIGNAL PROCESSING						      Time : 3 hours

Subject Code:	12EI224		 								       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the functional blocks of a digital signal processing system?

2.	Convolution performed using DFT is called circular convolution. Why?

3.	The periodicity of an 8 point DFT is ____________.

4.	Differentiate DIT and DIF FFT algorithms.

5.	One to one mapping of frequency from analog to digital domain is attained in ____________ technique of design of IIR filters.

6.	Direct form II structure of IIR filter is Canonic. True/False. Give reason.

7.	The main lobe width of the window spectrum affects the ____________ in the filter’s frequency response.

8.	How is it possible to design stable FIR filters?

9.	Draw the schematic of Modified Harvard Architecture.

10.	Implementation of FFT algorithm in TMS processor uses ____________ addressing mode.

PART – B  (5 x 3 = 15 MARKS)

11.	Use the matrix method to perform periodic convolution of x1[n]= {2, 1, 3, 4} and

x2[n]={3, 2, 4, 1}.

12.	Find the IDFT of X[k]={3, (2 + j), 1, (2–j)} using the DFT matrix.

13.	Using bilinear transformation, obtain H(z) if H(s)= 1s+12. Assume T= 0.1 sec.

14.	Realize the FIR transfer function H(z) using minimum number of multipliers.

H(z) = 15+12z−1+14z−2+14z−3+12z−4+15z−5.

15.	What is the function of MAC unit in digital signal processor?

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	What are the advantages of digital signal processing over analog signal processing?	(7)

b. 	Find the response of an LTI system with impulse response h[n] = {1, 2, 4} to an input

x[n]={1, 2} using circular convolution and verify the result using linear convolution.	(8)

(OR)

17.	An FIR filter has the impulse response h[n]={2, 2, 1}. Determine its response to an input

x[n]={3, 0, –2, 0, 2, 1, 0, –2, –1} using overlap add method.

18.	Using radix 2 DIT FFT algorithm, compute the 8 point DFT of x[n]={1, 2, 3, 4, 4, 3, 2, 1}.

(OR)

19.  	a. 	Determine the 4 point DFT of the sequence x[n] = n+1.							(8)

b. 	State and prove the circular convolution property of DFT.							(7)

[P.T.O.]

20. 	Design a digital Butterworth filter satisfying the following requirements using bilinear transformation technique.

0.707≤ H(ejω)≤ 1; 0≤ω≤0.45π H(ejω) ≤ 0.2; 0.65π ≤ ω ≤ π

(OR)

21.	Obtain the direct form I, direct form II , Cascade and Parallel realization of the LTI system 	governed by the  equation

y[n]=− 34y[n−1]+12y[n−2]+14y[n−3]+x[n]+4x[n−1]+3x[n−2]

22.	Design a linear phase FIR band pass filter to pass frequencies in the range c1 = 0.35π radians/sample to  c2 = 0.48π radians/sample using rectangular window by taking 5 samples of the window sequence.

(OR)

23. 	Determine the coefficients of a linear phase FIR filter of length N = 11 which has a frequency response H(2k/11) = 1   ; for k = 0, 1, 2, 3

0    ; for k = 4, 5

24.	With a neat schematic, explain the architecture of TMS320C5x processor and the function of

each block.

(OR)

25. 	a.	With an example, explain the various addressing modes used in TMS320c5X processor.(7)

b.	What are the four phases of an instruction cycle? Illustrate how instruction pipelining 	increases the efficiency of a processor.											    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	LOGIC AND DISTRIBUTED CONTROL SYSTEMS	             Time : 3 hours

Subject Code:	12EI225											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Which control mode speeds up the corrective action?

2.	Digital proportional controller equation is given by __________.

3.	Define scan time.

4.	Draw the symbol for normally open and normally closed Level Switch.

5.	Write the different types of Program control instructions.

6.	Mention any two arithmetic functions used in PLC.

7.	What is shared communication in DCS?

8.	Write any two Local Control Unit languages.

9.	List any two functions of low level human interface in DCS system.

10.	Mention any four types of industries that use DCS in their plant.

PART – B  (5 x 3 = 15 MARKS)

11.	What are data loggers?

12.	A Machine, M, is to be turned on either when Count A goes up to 11 or when count B goes up    
       to 16. One stop button or switch resets the entire process. Draw the ladder diagram for the 	above task.

13.	Explain the function of sequencer in a PLC.

14.	What is redundancy in DCS?

15.	List the requirements of operator station.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the block diagram of SCADA and explain each of its components.

(OR)

17.	Explain digital controller modes with suitable flow charts and equations.

18.	a.	Draw the ladder logic diagram for AND, OR, NOR, NAND and EXOR Gates.  		    (7)

b.	Write a PLC program to generate an alternating 	signal so that an alarm light connected to output bit location O: 3/1 flashes. Select a time period of 0.5 seconds OFF and 0.5 seconds ON.																			    (8)

(OR)

19.	Write short notes on timer and counter functions with suitable examples.

20.	Explain the following topics:

a. 	PID Function in PLC.															    (8)

b. 	Skip and MCR Function.														    (7)

(OR)

21. 	Develop ladder logic using sequencer function to control a dish washer. List all the operational     
 	patterns and also provide register pattern for the same. Assume equal time interval for the 
 	operations.

[P.T.O.]

22.	Draw the block diagram of DCS and explain the building blocks in detail.

(OR)

23. 	a.	Briefly discuss about the architecture of LCU with diagram.						    (8)

b.	Write short notes on the different types of LCU configuration.						    (7)

24.	a.	What are the requirements for an Operator Interface?								    (8)

b.	Explain the detail about the Operator Display.									    (7)

(OR)

25.	Can general purpose computers be used in DCS? If yes what do you think will be issues that 
       can occur during interface?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INDUSTRIAL DATA COMMUNICATION  AND NETWORKS														      							     Time : 3 hours

Subject Code:	12EI226				 							      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A __________ system is designed for sending data in both directions.

2.	The number of signal changes per second is defined as __________ .

3.	OSI reference model is a __________.

a.	Protocol 	b.	Standard 	c.	Model for designing a flexible network architecture

d.	Organization

4.	RS232 is a serial interface standard for peer communication. True/False

5.	__________ Serial interface standard operates in tri-state.

6.	How many lines are required to serially transmit a data using balanced transmission in full duplex mode?

7.	STX in BSC protocol means __________.

8.	What is a router?

9.	An IP address is a numeric quantity that identifies -
	a. 	a network adapter to other devices on the network

b. 	the manufacturer of a computer
c. 	the physical location of a computer
d. 	none of the above

10.	Data can be exchanged in two modes in Modbus. They are __________ and __________ .

PART – B  (5 x 3 = 15 MARKS)

11.	Represent the bit sequence 1011010 using RS232 signaling with one start bit, 2 stop bits and no parity.

12.	What is ARQ approach in flow control mechanism? How is it different from simple stop and wait approach?

13.	With an example explain how is CDMA implemented.

14.	Draw the response frame of MODBUS protocol for reading a register of address 000B with content 11AB .

15.	What are the layers in TCP/IP architecture?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What is the need of UART in RS-232 serial interface?                					    (3)

b.	Describe how UART functions as a transmitter and receiver.     					  (12)

(OR)

17.	a.	Discuss how data coding is done using ASCII and Gray codes. 					    (5)

b.	Using Polar line coding techniques encode the data 10110010.						  (10)

18.	Explain in detail about the most commonly used parallel interface standard IEEE 488.

(OR)

19.	Explain the architecture of USB with the reference to OSI layers.

[P.T.O.]

20.	Describe the bit oriented protocol in Data-link layer that uses stop and wait ARQ approach for flow control.

(OR)

21.	Discuss the functions of individual layers of OSI reference model for communication between open systems	.

22.	a.	Draw the flow chart for software logic of Transmitter in Allan Bradley Data highway plus 		protocol.															       			    (5)

b.	Discuss the architecture of field bus communication protocol.                          		  (10)

(OR)

23.	Elaborate on the industrial protocol that superimposes digital information over the existing 4mA to 20mA current loop.

24.	Discuss in detail about the IEEE 802.3 LAN standard.

(OR)

25.	a.	Explain the different controlled media access control mechanisms.	           			  (10)

b.	Differentiate circuit switching and packet switching.  			                   			    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INSTRUMENTATION AND CONTROL IN PETROCHEMICAL INDUSTRIES											      Time : 3 hours

Subject Code:	12EI236											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	__________ is the product obtained during Petroleum refinery, which is used for preparing candles, boot polish and electrical insulators.

2.	Define Bubble Point.

3.	Draw the typical Drying Curve.

4.	List the process variables present in the Chemical Reactor.

5.	Draw the P&ID diagram of Heat Exchanger with its symbol descriptions.

6.	Find the degrees of freedom for a liquid to liquid heat exchanger.

7.	What is Cocurrent operation in evaporator?

8.	___________ is the type of measurement whose sensing elements are not in contact with the process and popular in food industry.

9.	Illustrate the oxidation reaction of cyanide into cyanate, with a chemical equation.

10.	Define Precipitation.

PART – B  (5 x 3 = 15 MARKS)

11.  Write short notes on P&ID diagram with examples.

12.  What is a Dryer? Classify the different types of Dryers.

13.  Explain about the control of Condensers with necessary diagrams.

14.  Describe about Multiple effect Evaporator.

15.  What is Crystal Seeding?

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Discuss about Construction of Distillation Equipment and the variables involved in it.(8)

b. 	Illustrate the working of Reflux Control by maintaining the Boil-up rate.		      	    (7)

(OR)

17. 	Describe the Various Pressure control methods of Distillation Column in detail.

18. 	Explain the fluid-bed dryer controls of batch and continuous dryers.

(OR)

19. 	Elucidate about the different schemes of Temperature control in Chemical reactors.

20. 	Elaborate the different types of control strategy used for reboilers and vaporizers.

(OR)

21.  	Discuss the instrumentation and control in Liquid–Liquid heat exchanger.

22.  	Illustrate the working of different types of Evaporators in detail with necessary diagrams.

(OR)

23.  	a.  	Demonstrate the measurement and control of absolute pressure in an evaporator.		    (8)

b.  	Explain the implementation of cascade control in evaporators.						    (7)

[P.T.O]

24.  	Explain the process of waste water treatment using Chemical Reduction method.

(OR)

25. 	a.	Describe the schematic control system of an acid waste neutralization system.		    (8)

b. 	Explain the use of Biological Control in waste-water treatment.					    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INSTRUMENTATION AND CONTROL IN PAPER INDUSTRIES														      						     Time : 3 hours

Subject Code:	12EI237				 							       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What are the types of wood used for making pulp?

2. 	What are the objectives of bleaching the papers?

3. 	What is the pH value for pulp?

4. 	List any two temperature sensors used in paper industry.

5. 	Write the application of control valve.

6. 	Name any two flow sensors used in paper industry.

7. 	Define condensate conductivity control.

8. 	List the various rotating shaft type control valves.

9. 	List any two smart sensors used with computer control in Paper industry.

10. 	What are the various standards in PC based control system?

PART – B  (5 x 3 = 15 MARKS)

11. 	Write four steps followed in pulp making process.

12. 	Define the term consistency as used in paper and pulp industry.

13. 	Write the working principle of an alarm.

14. 	What are the various process parameters associated with white liquor flow control?

15. 	Justify the need of Industrial Robot in paper industry.

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Discuss with flow diagram the steps followed in paper making process.				    (8)

b. 	Explain how pulp is made using chemical recovery process.						    (7)

(OR)

17. 	Discuss in detail the mechanical and chemical pulping process. Compare their merits and demerits.

18. 	Explain with hardware the measurement and control of pressure and temperature in the pulp making process.

(OR)

19. 	Enumerate the construction and working principle of any two types of flow and level sensors.

20. 	a. 	Explain digester liquor feed pump control.										    (8)

b. 	With the neat sketch explain Stock chest level control.							    (7)

(OR)

21. 	a. 	Elaborate Brown stock water level control with neat sketch.						    (8)

b. 	Explain blow tank control with neat sketch.										    (7)

22. 	Discuss with necessary diagrams the working of Digester liquor feed pump controller.

(OR)

23. 	Explain with necessary diagrams the working of Brown stock washer level controller.

[P.T.O]

24. 	Describe a robot based automatic guided vehicle to transport materials in the shop floor of paper manufacturing industries.

(OR)

25. 	Explain about the computer based measurement and control in paper industry.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INSTRUMENTATION AND CONTROL IN IRON AND STEEL INDUSTRIES											      Time : 3 hours

Subject Code:	12EI238				 							       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the colour and odour of blast furnace gas?

2.	The molded iron collects at the bottom of the furnace and is tapped periodically through the ___________.

3.	What is an ingot?

4.	Name one integrated steel plant in India.

5.	What is an annunciation window?

6.	What are the uses of pressure sensors?

7.	How is temperature measured in steel industry?

8.	A combination of extreme temperature and full sprays will cause an _________and _________ the process.

9.	The basic function of rolling mill computer is __________.

10.	Give the uses of computer control.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate iron and steel.

12.	Differentiate hot rolling and cold rolling.

13.	Describe the procedure for weight measurement in iron and steel industry.

14.	Briefly explain the strand casting mold level control.

15.	Discuss about data logging process in steel plant.

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat sketch explain the Basic Oxygen Furnace (BOF) in steelmaking process.

(OR)

17.	Explain in detail with a flow diagram the process of steel making in a steel plant.

18.	Describe the primary rolling process in steel.

(OR)

19.	Explain the cold rolling and finishing process in steel.

20.	With a neat diagram explain the function of pressure sensor and flow sensor in steel rolling mill.

(OR)

21.	With a neat diagram explain the measurement of density, thickness and shape in steel rolling mill.

22.	With a neat sketch explain the blast furnace stove combustion control systems.

(OR)

23.	Draw and explain the waste water treatment in iron and steel industry.

[P.T.O]

24.	Discuss about the central utilities dispatch computer in a steel plant.

(OR)

25.	With a neat sketch explain the computer controlled rolling mill in steel plant.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ULTRASONIC INSTRUMENTATION					      Time : 3 hours

Subject Code:	12EI239										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	___________ is a cyclic pressure with a frequency greater than the upper limit of human hearing.

2.	Define Attenuation.

3.	What is poling?

4.	Name any two techniques used to measure the wavelength of ultrasonic waves using Electrical Method.

5.	Give any one application for ultrasonic temperature measurement.

6.	List the different methods of Pressure measurements.

7.	What are the different types of welds?

8.	Doppler SONAR is also known as ___________.

9.	Define Holography.

10.	Name any two application of ultrasonics in medical field.

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes on intensity of sound beam.

12.	What are search unit types?

13.	Explain the experimental technique used for the measurement of viscosity of liquid mixtures.

14.	Discuss the steps involved in ultrasonic inspection of welds.

15.	List out the applications of ultrasonics in medical diagnosis.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss in detail the different modes of Sound Wave Propagation.

(OR)

17.	Derive the expression for Velocity and Energy of vibrating particle in Progressive Waves.

18.	What is Magnetostriction Effect? Explain the generation of ultrasonic waves using Magnetostrictive method with a neat diagram.

(OR)

19.	Explain the principle, construction and working of a Piezoelectric Oscillator.

20.	Describe the following testing methods in detail:

a.  	Pulse echo         	b.		Contact testing      	c.		Immersion type

(OR)

21.	Explain the Ultrasonic Flow and Level measurements in detail.

22.	Elaborate the Ultrasonic measurement of Thickness and Depth with necessary diagrams.

(OR)

23.	Explain the working of different types of SONAR with a neat diagram.

[P.T.O]

24.	Explain the following types of Holography in detail:

a.  	Transmission acoustic holography.

b.  	Pulse reflection acoustic holography.

c.  	Linear acoustic holography.

(OR)

25.	Discuss about the different types of scans used in medical imaging and their clinical application.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INSTRUMENTATION AND CONTROL FOR AIRCRAFT

Time : 3 hours

Subject Code:	12EI240				 							       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Dual indicators are used to measure ___________ and ___________.

2. 	What is the colour indication in the display when the operation is prohibited?

3.	The altitude controller maintains cabin altitude by modulation of the:

a.	Safety and outflow valves.		b.	Outflow valve.	c.	Safety valve.	
	d.	All the above

4. 	Name the instruments that work based on the air data given by pitot tube.

5. 	Define rigidity.

6. 	The rotor of the gyroscope is operated by ___________.

7.  	What are the two basic types of thermocouples employed in aircraft thermoelectric indicating system?

8. 	In a high-performance aircraft the measures of fuel quantity are in terms of ___________ fuel gauging.

9. 	Write the requirement of pressure switch.

10. 	The power developed by a turbine engine is dependent on ___________ and ___________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Draw a diagram of instrument panels and layout.

12. 	Write the principle of pitot static probe.

13. 	Mention the properties of a gyroscope.

14. 	Explain how the Wheatstone bridge circuit may be utilized for the measurement of temperature.

15. 	Brief the principle of engine supercharging.

PART – C  (5 x 15 = 75 MARKS)

16. 	Describe the classification of aircraft instrumentation systems with relevant diagrams.

(OR)

17. 	With a neat diagram explain “LED” display and “head- up” display.

18. 	Define pitot pressure. With relevant diagram, explain the principle of airspeed indicator.

(OR)

19. 	Describe the construction and operation of an altimeter. Explain any special features which improve its accuracy.

20. 	Explain briefly the principle of Turn and Bank indicator.

(OR)

21. With a neat diagram, describe the principle of directional gyro.

22. 	Explain the fundamental operating principle of Bourdon tube which measures the pressure in Aircraft.

(OR)

[P.T.O.]

23. 	Describe the construction and operation of a float type and capacitor type of Fuel quantity indicating system.

24. 	Describe the operation of an engine vibration indicator. With a neat sketch explain the construction of sensing device.

(OR)

25. 	Describe the construction of typical thermocouple probe used for turbine-engine exhaust gas temperature measurement.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	OPTO ELECTRONICS AND LASER BASED INSTRUMENTATION														      						     Time : 3 hours

Subject Code:	12EI241				 							       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	LASERs and LEDs are used as _________ sources.

2.	BER stands for _________.

3.	Broadening of the transmitted signal is called _________.

4.	Light can be transmitted through optical fibers by the phenomenon of _________.

5.	LASER has high _________.

6.	Techniques to obtain high power LASER pulses are _________ and _________.

7.	LIDAR stands for _________.

8.	CO2  LASER is used for _________.

9.	_________ can be used for identification of finger prints.

10.	_________ is an optical instrument used in hospitals for diagnosis and treatment.

PART – B  (5 x 3 = 15 MARKS)

11.	Why do we prefer LASER diodes over LEDs for communication applications?

12.	Distinguish between step index and graded index fibers.

13.	State the threshold condition for LASER oscillation.

14.	List out the industrial applications of LASERs.

15.	Mention the advantages of LASER in surgery.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the construction and working of an Avalanche Photodiode.

(OR)

17.	Discuss in detail about the different types of hetero-structure LEDs with suitable diagrams.

18.	Explain the principle of fiber optic temperature and pressure sensors with suitable sketches.

(OR)

19.	Discuss in detail about the different types of Optical fibers.

20.	Explain the principle of Q switching and mode locking.

(OR)

21.	Discuss in detail about the construction and working of a He-Ne LASER.

22.	Explain the principle of LASER for measuring distance and velocity with a neat sketch.

(OR)

23.	a.	Explain the principle of LASER for measurement of atmospheric effects.  			    (8)

b.	Discuss the applications of LASER in melting and trimming of materials.  			    (7)

24.	a.	Explain in detail the principles of holographic interferometry.		    				    (8)

b.	Discuss about the principle of holography for non destructive testing.       			    (7)

(OR)

[P.T.O]

25.	a.	Discuss the photochemical and photo-thermal applications of LASER.      			  (10)

b.	Write short notes on Endoscopy.						    						    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	POWER PLANT INSTRUMENTATION					      Time : 3 hours

Subject Code:	12EI242										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define surge tank.

2.	What is concentration ratio?

3.	Write the need for steam flow measurement.

4.	Define combustion.

5.	List any two commercially available dust monitors.

6.	Define chromatography.

7.	Give the two important controls in the air-fuel circuit.

8.	Give the reason for the fuel admission interlock.

9.	Mention the use of bearing temperature.

10.  What is velocity of vibration?

PART – B  (5 x 3 = 15 MARKS)

11. 	Discuss the principle of a solar cell.

12. 	List the basic factors to be considered for feed water flow.

13. 	Write short notes on fuel analyzer.

14. 	Discuss the function of deaerator.

15. 	How to control the vibration of turbine blades?

PART – C  (5 x 15 = 75 MARKS)

16. 	With a neat sketch, explain in detail the operation of thermal power plant.

(OR)

17. 	Draw and explain the piping and instrumentation of a boiler system in a power plant.

18. 	Discuss the smoke density measurement using the Ringlemann chart.

(OR)

19. 	Discuss the steam temperature measurement with suitable sketch.

20. 	Elucidate the working principle of dissolved oxygen analyzer with a neat sketch.

(OR)

21. 	Describe a method to measure pH with a neat sketch.

22. 	Explain the furnace draft control system with suitable sketch.

(OR)

23. 	Describe the interlocks in the boiler.

24. 	Explain the method of vibration measurement in steam turbine using a block diagram.

(OR)

25. 	Briefly discuss the lubricating oil temperature control system with a block diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	AUTOMOTIVE CONTROL SYSTEMS					      Time : 3 hours

Subject Code:	12EI243				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Variable reluctance type sensors are used in __________ systems.

2.	The starting system uses a powerful electric motor to drive the engine at about __________ rpm to allow the fuel and ignition systems to operate.

3.	__________ refers to technology assisting in prevention of crash.

4.	__________ decreases the stopping distances on dry and slippery surfaces.

5.	__________ uses a high speed bus.

6.	In the __________ system, a twisted pair of wires is used to provide two different voltage levels and minimize the electrical interference.

7.	__________ Sensors require a supply of current.

8.	__________ uses sensors to continuously detect the speed of each wheel.

9.	__________ is essential, because of miniaturization of parts and equipments.

10.	__________  provide a wide variety of applications covering comfort, safety, security, high performance computing and networking.

PART – B  (5 x 3 = 15 MARKS)

11.	List out the advantages of nickel based batteries over lead acid batteries.

12.	Enumerate the components of a Chassis subsystem.

13.	Define the term protocol.

14.	List out the merits of electronic transmission control.

15.	Mention the features supported by automotive navigation systems.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss in detail about the Electronic Fuel control unit with a neat sketch.

(OR)

17.	Discuss in detail about the automotive ignition systems.

18.	Explain the operation of an automotive power train system.

(OR)

19.	Discuss in detail about the various comfort and safety subsystems in an automobile.

20.	Compare the features of various vehicle communication protocols.

(OR)

21.	Explain in detail about I2C communication protocol.

22.	Explain in detail the principle behind electronic control of braking and traction.

(OR)

23.	Write short notes on Engine Management systems.

24.	a.	Explain in detail the features supported by vehicle infotainment systems. 			    (8)

b.	Discuss in detail about System Level Tests.       				    					    (7)

(OR)

25.	Describe the method of software calibration using engine and vehicle dynamometers.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FIBER OPTICS AND LASER INSTRUMENTATION	            Time : 3 hours

Subject Code:	12EI247				 							      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Show diagrammatically the multimode propagation of light in a step index fiber.

2.	What do you understand by acceptance angle of an optic fiber?

3.	How can you classify the fiber optic sensors?

4.	Name two parameters which can be measured by interferometric fiber optic sensors.

5.	Give two examples of gas lasers.

6.	Define population inversion in laser.

7.	Mention two advantages of laser welding.

8.	What is the advantage in using laser in vapourising materials for deposition in a substrate?

9.	State two medical applications of laser.

10.	Mention any one application of holography.

PART – B  (5 x 3 = 15 MARKS)

11.	Mention the needed characteristics of photodetectors used in fiber optic communication.

12.	Write a short note on fiber optic gyroscope.

13.	Brief about the resonator configurations in laser.

14.	Detail about velocity measurement using laser.

15.	Write short note on endoscopy.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the principle, structure and working of an APD.

(OR)

17.	Explain the structure and working of LED as source in fiber optic communication. Mention the advantages of LED as source.

18.	Detail about the measurement of temperature and pressure using fiber optic sensors.

(OR)

19.	Write short notes on:

a.	measurement of length by fiber optic sensor.

b.	measurement of liquid level using fiber optic sensor.

20.	Describe with a sketch the Ruby laser and its working.

(OR)

21.	Brief about the semiconductor lasers.

22.	Explain the use of laser for measurement of

a.	atmospheric effects   		b.	length      		c.	distance

(OR)

23.	Detail about the industrial applications of laser for welding, melting and trimming of materials.

24.	Explain the holography applications for nondestructive testing and enumerate the medical applications of laser.

(OR)

25.	Brief about the photo thermal and photo chemical applications of laser.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INSTRUMENTATION AND PROCESS CONTROL FOR FOOD ENGINEERS											    Time : 3 hours

Subject Code:	12EI252				 							      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Transfer function.

2.	What is a Process?

3.	What is a Neutral zone?

4.	What is the purpose of spring in a control valve?

5.	How do you define the stability of the system based on the coefficient of the characteristics equation?

6.	Give Mason’s gain formula.

7.	The output of Thermocouple is in the range of ___________.

8.	What is gauge factor in a strain gauge?

9.	The gas chromatography works on ___________ principle.

10.	Float is used for ___________ measurement.

PART – B  (5 x 3 = 15 MARKS)

11.	What is feedback control? Mention its advantages.

12.	Brief about servo mechanism in control system.

13.	What is Routh stability criterion?

14.	Explain the working of Bimetallic strip.

15.	Write short notes on Buoyancy meter.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Explain an automatic process control system with a neat diagram.					  (10)

b. 	Obtain Laplace transform of cos at.											    (5)

(OR)

17.	a. 	What is Transient response? Distinguish transient and steady state response.			  (10)

b. 	Obtain Laplace transform of te−at.												    (5)

18.	Explain Proportional, Integral and Derivative control modes in detail.

(OR)

19.	a.	Explain two way control with example.										    	   (5)

b. 	Explain Hydraulic and pneumatic control system.								  (10)

20.	Find the stability of the system using Routh array whose characteristics equation is given by,

S6+2S5+8S4+12S3+20S2+16S+16=0

(OR)

[P.T.O.]

21.	Using Signal flow graph obtain the Transfer function for the system shown.

22.	a. 	Explain the construction and working of capacitive pressure transducer.				  (10)

b. 	Write short notes on Bellows.													    (5)

(OR)

23.	a. 	Explain the construction and working of RTD.									    (8)

b. 	Discuss the working of Thermistor in detail.									  	    (7)

24.	Explain the working of Mass spectrometer with a suitable diagram.

(OR)

25.	Describe various types of level measurement in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	INSTRUMENTATION AND CONTROL SYSTEMS	             Time : 3 hours

Subject Code:	12EI254				 							       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Classify instruments based on their functions.

2.	What is random error?

3. 	___________ pressure gauge is used for measuring very high pressure.

4.	Which transducer converts heat energy into electrical energy?

5. 	Hot wire anemometer is used for the measurement of _________.

6. 	Electromagnetic flow meter is not suitable for which kind of fluids?

7. 	What are the basic elements used for modelling mechanical translational system.

8. 	What is a transfer function?

9. 	Define bandwidth.

10. 	Find the order and type for system having G(s) =1/s3 (s+1)

PART – B  (5 x 3 = 15 MARKS)

11. 	Give the applications of measurement systems.

12. 	What is Negative Temperature Coefficient?

13. 	Define strain.

14. 	Write the condition of stability?

15. 	Why is it necessary to choose controller settings that satisfy both gain margin and phase margin?

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the Fundamental Process of measurement and describe generalized measurement system with example.

(OR)

17. 	What is recorder? Explain the working of X Y recorder.

18. 	Explain briefly the construction and working of flattened tube pressure cell.

(OR)

19. 	Explain briefly the construction and working of RTD and thermocouple.

20. 	With relevant diagram explain mechanical torsion meter.

(OR)

21. 	With a neat diagram explain the construction and working of ultrasonic flow meter.

22.  	With suitable examples explain about open loop and closed loop control systems.

(OR)

23. 	Define servo mechanism. Explain position control system using servomotor.

24. 	Construct Routh array and determine the stability of the system represented by the characteristic equation,s6 + 2s5 + 8s4 + 12s3 + 20s2 + 16s + 16 = 0.

(OR)

25. 	Sketch Bode plot for the transfer function, G(s)=5(1+2s)(1+4s)(1+0.25s).

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	SOFT COMPUTING									Time : 3 hours

Subject Code: 	12EI308									              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	Describe the architecture of neural network using biological neurons.			      (10)

b.	Explain the learning and training procedure of Hopfield network.				      (10)

(OR)

2.	a.	Draw and explain the architecture of back propagation algorithm. Also describe the           	updation of hidden layer and output layer weights.    							      (14)

b.	Using the Correlation learning rule, find the weights for the following input vectors.             	 	X1 = [1  –2  1.5  0]t, d1=1; X2 =[–1 –0.5 –2 –1.5]t, d2= –1; and X3 =[–1 1 0.5 –1]t,

d3= –1. Use learning rate of 1 and starting weights of [1 –1 0 0.5]t.  Find the final weight  		vector.											 							(6)

3.	Compulsory:

a.	Explain the various schemes of neuro-control.								      (10)

b.	Implement Neural Controller for balancing the inverted pendulum.      			      (10)

4.	a.	Discuss the various fuzzy propositions with examples.                                         	      (10)

b.	Three fuzzy sets are given as follows:										      (10)

P=0.12+0.34+0.76+0.48+0.210;Q=0.10.1+0.30.2+0.30.3+0.40.4+0.50.5+0.20.6andT=0.10+0.70.5+0.31

Perform the following operations

(i)R=PXQ(ii)S=QXT(iii)M=R∘S using max-product composition.

(OR)

5.	Two key variables in the new invention are uniqueness of the product (A) denoted by the universe X={1, 2, 3, 4} and Market size of the product (B) denoted by the universe Y={1, 2, 3, 4, 5, 6}.  Suppose we represent each of these variables as specific fuzzy sets.

A = 0.62 + 13 + 0.24 ; B=  0.42 + 13 +0.84 + 0.35

a.	 Construct a relation for the compound proposition IF A THEN B                               (14)

b.	Let the Almost High Uniqueness be represented by0.51 + 12 + 0.33 + 04.

Using max-min composition, find the fuzzy market size associated with the Almost High Uniqueness.																	(6)

6.	a.	With a neat schematic diagram, explain the structure of fuzzy logic controller with its 	models.																	      (12)

b.	What are the different methods of defuzzification?				  				(8)

(OR)

[P.T.O.]

7.	Explain the operation of the following:

a.	Sliding mode fuzzy logic control.											      (10)

b.	Sugeno fuzzy logic control.												      (10)

8.	Explain various operators of genetic algorithm with necessary example.

(OR)

9.	a.	Explain the fundamentals of evolution of Genetic Algorithm and search space. Also, 	discuss how the Genetic Algorithm differs from other traditional search methods. 	 (8)

b.	Discuss about the types of encoding in Genetic Algorithm.			 		      (12)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CONTROL SYSTEMS									      Time : 3 hours

Subject Code:	EI203										       	       Maximum Marks: 100

#### (Provide Polar graph and Semi log graph)

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Which is the analogous quantity for the Mass in a force voltage analogy?

2.	What do you mean by transfer function?

3.	What is settling time?

4.	What is peak time?

5.	What is the necessary condition for stability according to Routh stability criterion?

6.	What do you mean by centroid?

7.	What is Gain Margin?

8.	What is Phase Margin?

9.	Write the equation for Proportional Control.

10.	What is integral windup?

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between open loop and closed loop system.

12.	How will you classify the second order system based on damping ratio?

13.	How will you find the angle of asymptotes?

14.	Write any three applications of Bode Plot.

15.	How to eliminate the offset in Proportional control?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the Mathematical model for the given process.

M1

M2

K1

K2

B

x1

x

f(t)

B1

B2

(OR)

17.	a.	What is the need for block diagram approach?			  						    (2)

b.	Why signal flow graph is required? Write the Mason’s gain formulae.  			    	    (3)

c.	Demonstrate any five rules used in the block diagram reduction method.			  (10)

[P.T.O.]

18.	a.	Find the response of first order system CsRs=1τ s+1  when it is subjected a unit step 	input.																		  (12)

b.	What do you mean by time constant?					  						    (3)

(OR)

19.	a.	Obtain the expression for rise time of second order underdamped system.			    (7)

b.	Obtain the expression for peak overshoot of second order underdamped system. 		    (8)

20.	A unity feedback control system has an open loop transfer function                       Gs=kss2+4s+13. Sketch the Root Locus.

(OR)

21.	Determine the range of k for the stability of unity feedback system whose open loop transfer function 	Gs=kss+1s+2.	Use Routh stability criterion.

22.	Plot the bode diagram for the following transfer function and obtain the gain and phase cross over frequencies.

Gs=10s1+0.4s1+0.1s.

(OR)

23.	The open loop transfer function of a unity feedback system is given by Gs=1s1+s1+2s	Sketch the polar plot and determine the gain margin and phase margin.

24.	Explain the working of fuzzy logic controller with a block diagram.

(OR)

25.	Explain in detail about PI, PD and PID controllers. Specify the importance and uniqueness of P, I and D control actions. Give suitable applications for P, PI and PID controllers.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	EMBEDDED INSTRUMENTATION SYSTEM			      Time : 3 hours

Subject Code:	EI206											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List any two applications of real time embedded systems.

2.	Mention any two issues to be considered while designing real time systems.

3.	Define round-robin system.

4.	Draw the operating system kernel hierarchy.

5.	What are the factors affecting polled loop response time.

6.	Define interrupt latency.

7.	Name any two techniques used in black box testing.

8.	What is fault tolerance in a system?

9.	Name any two real time operating environments that are widely used in literature.

10.	Draw the block diagram of a generalized multimedia architecture.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the benefits of RTOS?

12.	Distinguish between sporadic and fixed rate interrupt systems.

13.	Give the typical memory map in real time system.

14.	What do you mean by system-level testing?

15.	What are the factors required to design real-time databases?

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat sketch explain the architecture of 68HC 11 Microcontroller.

(OR)

17.	With a neat sketch explain how multiple interrupts are handled with an interrupt handler.

18.	Describe the role of kernel in the operating system.

(OR)

19.	Describe process stack management in a real time system.

20.	Explain various optimization techniques and their impact on real time system performance.

(OR)

21.	Explain how memory loading analysis is done in a real-time system.

22.	Discuss the unit level and system level testing with examples.

(OR)

23.	Explain the reliable methods for testing Random Access Memory (RAM) and Read Only memory (ROM).

24.	With a block diagram describe how real-time image processing differs from ordinary image processing.

(OR)

25.	With an example explain real time database system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title : 	ELECTRONIC CIRCUITS								      Time : 3 hours

Subject Code:	EI238											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the Ripple factor.

2.	What are the advantages of SMPS over conventional regulators?

3.	Differentiate Clipper and Clamper circuits.

4.	Define time constant.

5.	Define CMRR.

6.	Draw the small signal equivalent circuit of FET.

7.	What is conversion efficiency in a power amplifier?

8.	List out the advantages of using negative feedback.

9.	State Barkhausen criteria.

10.	Draw the equivalent circuit of crystal oscillator.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the necessity of bleeder resistor in inductor filter?

12.	How does an oscillator get starting voltage?

13.	Draw the small signal equivalent circuit of BJT and MOSFET.

14.	Which is the commonly used feedback arrangement in cascaded amplifiers? Why?

15.	If R1=10KΩ and R2=5KΩ and C1=C2=0.1µF, find the frequency and duty cycle of the astable output.

PART – C  (5 x 15 = 75 MARKS)

16.	With necessary diagrams explain the operation of Full wave rectifier and derive the expressions for IDC, VDC, IRMS, VRMS, rectification efficiency, ripple factor and transformer utilization factor.

(OR)

17.	a.	Draw the circuit diagram of a HWR with capacitor input filter with suitable waveform and 		explain its working. Derive the expression for ripple factor. 						    (8)

b.	Draw and explain the working principle of a SMPS circuit.						    (7)

18.	Explain its operation of a clipper and clamper with necessary circuit diagram, input and output waveforms.

(OR)

19.	Write a short note on integrator and differentiator.

20.	Discuss the operation of the emitter coupled differential amplifier. Derive the expression for Ad, Ac and CMRR of a differential amplifier.

(OR)

21.	What is meant by cascaded amplifier? Explain the type of cascade amplifier that is used to improve the input impedance of emitter follower circuit.

[P.T.O.]

22.	A series fed Class A amplifier with RB = 1.5 KΩ and RC = 16 Ω operates from a DC source and applied sinusoidal input signal generates peak base current 9mA. Calculate quiescent current ICQ, quiescent voltage VCEQ,DC input power, AC output power & efficiency.

(OR)

23.	Explain the operation of the Class-B push pull power amplifier with a neat diagram and derive the theoretical max conversion efficiency of class B power amplifier.

24.	Explain RC phase shift oscillator with neat circuit diagram. Derive its frequency of oscillation.

(OR)

25.	a.	In Colpitts oscillator, frequency of oscillation is observed to be 2.5 MHz Oscillator uses

L = 10µH, C1 = 0.02µF. Find:

i.	value of C2 		ii. if L is doubled the new value of frequency of oscillations.	  (11)

b.	List the applications of Schmitt trigger.											    (4)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	SENSORS AND TRANSDUCERS						      Time : 3 hours

Subject Code:	EI247										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	A Wheatstone bridge requires a change of 7Ω in the unknown arm of the bridge to produce a  change in deflection of 3 mm of the galvanometer. Determine the sensitivity.

2. 	List out the different types of Errors.

3.	Differentiate between active and passive transducers.

4. 	Give an example for Inverse transducer.

5. 	The commonly used metal for RTDs is ___________.

6. 	Define gauge factor.

7. 	Draw the equivalent circuit of Piezo-electric crystal.

8. 	What is the permeability value of free space?

9.	Mention the applications of Hall effect transducer.

10.	Enlist two salient features of smart sensors

PART – B  (5 x 3 = 15 MARKS)

11. 	Derive the transfer function of a first order system citing suitable example.

12. 	Differentiate between analog and digital transducer.

13. 	Discuss the principle and working of RTD. Sketch its typical characteristics.

14. 	Portray the different modes of operation of piezo-electric transducers.

15. 	Describe how digital shaft encoder is used for measurement of angular displacement.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What are the basic blocks of a generalized instrumentation system? Draw the various 			blocks and explain their functions. 							           				  (10)

b.	With suitable examples explain various methods of measurements.					    (5)

(OR)

17.  a.	Discuss in detail on the following static characteristics: Static sensitivity, Linearity, 	Hysteresis and Loading effects.							            				    (8)

b.	The following 10 observations were recorded when measuring a voltage:

| 41.7 | 42 | 41.8 | 42 | 42.1 | 41.9 | 42 | 41.9 | 42.5 | 41.8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Find the:		i.	Mean	ii.	Average deviation	iii.	Standard deviation

iv.	Probable error	    														    (7)

18.  a.	Describe the classification of transducers citing suitable examples for each.			  (10)

b.	Write notes on the electrical phenomena used in transducers with an example.		    (5)

(OR)

19. 	Explain in detail the various factors influencing the choice of a transducer.

[P.T.O.]

20. 	a.	A strain gauge is bonded to a beam of 0.1m long and has a cross sectional area of 4cm2.

The youngs modulus of steel is 297GN/m2. The strain gauge has an unstrained resistance of 240Ω and a gauge factor of 2.2. When a load is applied, the resistance of strain gauge

changes by 0.013Ω. Calculate the change in length of the steel beam and the amount of force applied to the beam.														    (5)

b.	Describe the construction and working principle of LVDT. Discuss its advantages and

disadvantages.																  (10)

(OR)

21. 	Describe the constructional details of a resistance potential divider and derive the expression

for its output voltage when connected across a meter of finite resistance.

22. 	a.	Describe the different principles of working of capacitive transducer used for the 	measurement of displacement. 													    (8)

b.	Explain the application of capacitive transducer in Level measurement.				    (7)

(OR)

23. 	a.	Describe the properties of piezo – electric crystals. Derive expressions for voltage and 			charge sensitivities.									           					  (10)

b.	A pressure measuring capacitive transducer is having a spacing of 4mm between its diaphragm. A pressure of 600KN/m2 produces an average deflection of 0.3mm of the diaphragm. The transducer which has a capacitance of 300pf before the application of pressure is connected in an oscillator circuit having an frequency of 100KHz. Determine the change in frequency of the oscillator, after the pressure is applied to the transducer.																				    (5)

24. 	a.	Explain three different applications of Hall Effect transducers with neat sketches.	    (8)

b.	Briefly describe about the measurement of sound	.	 							    (7)

(OR)

25. 	a.	Give short notes on the working of optical transducer and translational encoder.		  (10)

b.	Describe any one digital speed transducer.										   (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ELECTRIC CIRCUIT ANALYSIS						      Time : 3 hours

Subject Code:	EI250				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The relationship between voltage and current in a pure capacitor is given by equation __________.

2.	How a current source can be transformed to a voltage source?

3.	What is the condition for series resonance?

4.	Write the formula to calculate power factor by two wattmeter method.

5.	How the self- inductance of two coils is related to the mutual inductance between them?

6.	What is meant by ideal transformer?

7.	Where is maximum power transfer theorem applied?

8.	State the reciprocity theorem.

9.	What is the time constant of a RC circuit with forced response?

10.	At steady state what is the voltage across the Inductor in an RL circuit with DC excitation?

PART – B  (5 x 3 = 15 MARKS)

11.	Find the equivalent resistance for the circuit shown in the figure.

12.	A circuit consists of 2 ohms resistor, 2 Henry inductor and 0.083 farad capacitor connected in series along with voltage source of voltage V= 12√2(sin3t).  Calculate the current from the source.

13.	What is the maximum possible mutual inductance of two inductively coupled coils with self-inductances of 100mH and 150mH?  If the co-efficient of coupling is 50% then what is the mutual inductance.

14.	Use superposition theorem to find current through 4 ohms resistor shown in the figure.

15.	Derive an expression for current in the case of RC series circuit with DC excitation when the switch is closed at t = 0.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Determine all the branch currents in the network shown below.					(10)

[P.T.O.]

b.	Illustrate Kirchhoff’s Voltage law and Kirchhoff’s current law.         	   			    (5)

(OR)

17.	a.	Find the current and voltage in the figure shown by network reduction.				  (10)

b.	Illustrate Current division technique and Voltage division technique.                   		    (5)

18.	a.	Derive the expression for resonance frequency in case of parallel RLC circuit.  		  (10)

b.	Define real, reactive and apparent power with the power triangle.     				    (5)

(OR)

19.	a.	Find Z1 in the three branch network shown in the figure. I = 31.524˚ A for an applied 	voltage of V = 50 60˚ 					 		           						  (10)

b.	In the circuit shown, R = 2 ohms, L = 1 mH and C = 0.4 microfarad.  Find the resonance 			frequency and half power frequencies.	                                                     			    (5)

20.	a.    Write the node equations for nodes A, B, C with D as reference node and find the node 	voltages.									   								  (10)

b.	Derive the relationship between self inductances, mutual inductance and co-efficient 				of coupling.																  (5)

(OR)

21.	a.	By inspection write the mesh equations of the complex network shown in figure.  	(12)

b.	A 220/110V, 50Hz Ideal Transformer has 166 turns on its primary.  What is the peak value 	of flux. 																		    (3)

22.	a.  	Obtain the Thevenin’s equivalent circuit of the figure shown below.   				  (10)

b.	Use superposition theorem to find the current from the source.              				(5)

(OR)

23.	a.	Find the value of the adjustable resistor R which results in maximum power transfer across 	the terminals ab of the circuit shown below.                                          				  (12)

b.	State compensation theorem and mention its applications. 							    (3)

24.	a. 	Obtain the transient response of RLC network with DC excitation when the circuit is over 	damped.																		  (10)

b.	A series R – L circuit has 2 ohm resistor in series with 10 H inductor excited by 100 V battery.  What is the time constant and damping ratio of the circuit.  Find the value of current after 8 seconds of switching.											    (5)

(OR)

25.	a.	In the figure below, switch S is closed.  Find the current drawn from source after 6ms of 	switching and the time when the current drawn from source is 300mA. 				  (10)

b.	Derive an expression for decay current of RL series circuit after the source has been short 	circuited and initial current through inductor is V/R.				     				  (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	BIOMEDICAL INSTRUMENTATION					      Time : 3 hours

Subject Code:	EI252										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define ‘active transport’.

2. 	Mention the uses of ECG.

3. 	What is the reason of applying gel while using a transducer?

4. 	Mention the importance of pneumotachograph.

5. 	What is diathermy?

6. 	Write the need for heart lung machine.

7. 	Mention the uses of gas analyzers.

8. 	What is a sterilizer?

9. 	Define biotelemetry.

10. 	Write the principle of NMR.

PART – B  (5 x 3 = 15 MARKS)

11. 	Define Latency.

12. 	What are oximeters?

13.	Draw the block diagram of anesthesia machine.

14. 	Write the principle of Spectrophotometer.

15. 	What are the uses of bio telemetry?

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Describe the 10-20 electrode system used in EEG. 								(10)

b. 	Bring out the salient features of Phonocardiography. 								  (5)

(OR)

17. 	a. 	Write short notes on action potential. 											  (5)

b. 	Describe the muscular system. 													(10)

18. 	Explain the Fick’s method for the determination of cardiac output.

(OR)

19. 	Draw the block diagram of audiometer and explain the measurement procedure.

20. 	With a neat sketch, explain the working principle of hemo dialysis.

(OR)

21. 	Describe the working principle of AC defibrillator.

22. 	Write short notes on Electrical safety hazards in hospitals.

(OR)

23. 	With a neat sketch, explain flame photometer.

24. 	Compare the features of X-ray and Fluroscopic techniques.

(OR)

25. 	Draw the block diagram of a CT and explain the different blocks in it.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	SIGNAL CONDITIONING CIRCUITS					      Time : 3 hours

Subject Code:	EI258				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The output resistance of an ideal opamp is __________.

2.	Define Slew rate.

3.	What is the significance of CMRR in a communication system?

4.	An op amp has bias currents IB1 and IB2 as 400nA and 300nA. Calculate the input bias current of the op amp.

5.	Give the transfer function of 2nd order low pass filter.

6.	State any one application of isolation amplifier.

7.	What are the types of voltage regulators?

8.	The purpose of IC 7805 is to achieve __________regulation.

9.	Define Lock-in-range of PLL.

10.	On what parameters does the free running frequency of VCO depend on?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the circuit diagram of a Non-Inverting Amplifier and find the gain of a non-inverting amplifier with RF=10kΩ andRi=5kΩ.

12.	Draw an adder circuit using op-amp to get add three input voltages V1,V2 and V3.

13.	What are the basic requirements of an instrumentation amplifier?

14.	What are the various specifications of a three terminal IC regulator?

15.	Define modulation and demodulation.

PART – C  (5 x 15 = 75 MARKS)

16.	What is an op-amp? List and explain the function of all the basic building blocks of an op-amp.

(OR)

17.	a.	Discuss on the ideal characteristics of op-amp									  (12)

b.	Design a non-inverting amplifier of gain 100.									    (3)

18.	a.	Design an Astable multi-vibrator using 555 timer.								  (10)

b.	Draw a sample and hold circuit, and explain its operation.							    (5)

(OR)

19.	With the help of circuits and necessary equations, explain how a differentiator and integrator is designed using op-amp.

20.	a.	What are the advantages of active filters over passive filters?						    (5)

b.	Design a wide band pass filter with fL=400Hz and fH = 2 kHz and pass band gain=4. Also 		draw an approximate frequency response plot for the filter.						  (10)

(OR)

21.	What is instrumentation amplifier? Discuss the operation of instrumentation amplifier with the help of circuit diagram.

[P.T.O.]

22.	List the various protection circuits used in series voltage regulator and explain in detail.

(OR)

23.	Discuss in detail about the different type of voltage regulators.

24.	List out the different types of analog modulation methods and explain in detail about them.

(OR)

25.	a.	With block schematic explain the working principle of PLL.						    (8)

b.	How is PLL used as frequency multiplier?	                                                                     (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DIGITAL CONTROL SYSTEMS						      Time : 3 hours

Subject Code:	EI259										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	_________ type ADC gives fast response.

2.	The signal fk=1, 2, 3, ....., then f(k–2) is _________.

3.	In sampled data control system, _________ is used as controller.

4.	Final value theorem states that, f( k) is causal,  F=_________.

5.	Zak=_________.

6.	The transfer function of Linear discrete time system is called as _________.

7.	Transfer function of zero order hold is, ________.

8.	List out the advantages of state space analysis.

9.	What are the demerits of velocity form of PID control algorithm?

10.	In digital control, manual to auto switching without disturbance to process, is termed as _________.

PART – B  (5 x 3 = 15 MARKS)

11.	State Shannon’s sampling theorem.

12.	Obtain the Z transform of f(k) = u(k) , where u(k) is a discrete unit step sequence.

13.	Draw the block diagram representation of state model of discrete system.

14.	Specify the criterion for stability of sampled data control system.

15.	Mention the advantages of digital controller.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Give a detailed illustration on any one type of analog to digital conversion technique.(13)

b.	Mention the applications of it.						       							    (2)

(OR)

17.	Explain the following:

a.	Sampled data control system. 			      				    						    (6)

b.	Digital to analog convertor.				      			    						    (9)

18.  a.	Find the Z transform of the given function ft=cosωt. 			    				    (7)

b.	Find out the inverse Z transform of Fz=1−z−11−z−1+0.5z−2. 		    				    (8)

(OR)

19. 	a.	Derive the frequency response of zero order hold device.							    (7)

b.	Obtain the z domain transfer function for gs=ss2+ω2.							    (8)

20. 	a.	Obtain the state  model of given system in parallel form:

y(k+3) + 6 y(k+2) + 11y(k+1) +6 y(k) =2 u(k)

(OR)

[P.T.O.]

21. 	Test the controllability and observability of the given state model:

x(k+1) =175928763 x(k)   + 10−1u(k) ,   y(k)=  101x(k) +D u(k)

22. 	For the given discrete time unity feedback system having its open loop transfer function

Gz=K0.3679z+0.2642z−0.3679z−1, find the range of K, for the system to be stable.

(OR)

23. 	Obtain the state transition matrix of the discrete time system if the system matrix is given as A=−300−2.

24. 	a.	Explain the velocity form of PID algorithm with a flow chart. 						  (10)

b.	List any two demerits of position form algorithm.								    (2)

c.	Mention the merits of digital algorithm.											    (3)

(OR)

25. 	Elaborate the implementation of digital control system in temperature control system with a  block diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DIGITAL SIGNAL PROCESSING						      Time : 3 hours

Subject Code:	EI262		 								       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define DFT.

2. 	What is the output voltage of an LTI system for an input of A sin(100πt)?

3.	What is the main advantage of FIR filter?

4. 	Draw a third order FIR filter structure.

5.	Draw the frequency response of Elliptic low pass filter.

6.	Draw the direct form-II structure of second order IIR filter.

7. 	What does VLIW stand for?

8.	What is a RISC processor?

9. 	What is a floating point processor?

10. 	What are special features of sum of products in DSP?

PART – B  (5 x 3 = 15 MARKS)

11. 	What are the differences between linear and circular convolution?

12. 	What are the steps involved in FIR filter design using window technique?

13. 	What are equations used for converting analog filter into digital filter using impulse invariant method and bilinear transformation method?

14. 	What are the advantage of more address buses and data buses?

15.	What are the needs of digital signal processor compared to microprocessor/microcontroller?

PART – C  (5 x 15 = 75 MARKS)

16. 	Derive and draw 8 point DIT structure.

(OR)

17.	a.	How do you convolve two signals? Explain with an example.						    (8)

b.	How do you obtain linear convolution using circular convolution?					    (7)

18.	a.	Discuss the Four types of FIR filters.											    (6)

b.	Design a 7th order ideal low pass FIR filter using Hamming window with Hejω=e−j3ω   for –3π/4  < ω <  3π/4 and 0 outside.											    (9)

(OR)

19.	a.	Explain the method of FIR filter using frequency sampling method.					    (8)

b.	What is transversal filter? Explain.												    (7)

20.	a.	How do you specify the filter specification?										    (3)

b.	Derive the order of analog low-pass Butterworth filter.							    (6)

c.	Compare Butterworth, Chebyshev and Elliptic filters.							    	    (6)

(OR)

21.	Realize the transfer function using direct form-II and parallel form:

H(z)=[1+(14)z−1][1+(12)z−1+(14)z−2].

[P.T.O]

22.	a.	How do you design a multiplier unit?											    (7)

b.	What are special addressing modes for DSP?										    (8)

(OR)

23.	Explain the special features of RISC.

24. 	Draw the Architecture of TMS 320C5X and explain the function of each block.

(OR)

25. 	Explain the most useful instructions used in IIR filter design program.