Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FOOD ENGINEERING OPERATIONS					      Time : 3 hours

Subject Code:	09FP201									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define effectiveness of screen.

2.	What are cyclone separators?

3.	Define power number.

4.	What is meant by forming?

5.	Write the Bond’s law for power calculation in size reduction.

6.	Define Kick’s law.

7.	Define blanching.

8.	What is enrobing?

9.	What is meant by cryogenic freezer?

10.	What is freezing?

PART – B  (5 x 3 = 15 MARKS)

11.	Write a note on screen openings.

12.	Explain the working of planetary mixers.

13.	Write a note on pressure homogenizers.

14.	Differentiate deep fat frying and shallow frying.

15.	Write the mechanisms involved in ice crystal formation.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	For IS 50 mesh screen, the cumulative mass fractions of feed, overflow and underflow are 	0.16, 0.615 and 0.03 and for IS 30 mesh screen, the cumulative mass fractions of feed, 	overflow and underflow are 0.75, 0.955 and 0.610 respectively. Calculate the screen 	effectiveness of IS 50 mesh and IS 30 mesh.        									  (10)

b.	Discuss working of disk separator and magnetic separator.							    (5)

(OR)

17.	a. 	During evaluation of an air screen grain cleaner with 2 screens, 250 g samples were 	collected for analysis of clean seed fraction from different outlets. The data are presented 	in the following Table. Calculate the cleaning efficiency of the cleaner.				    (5)

b. 	Explain the construction and working of air screen cleaner and pneumatic & aspirator       	separator. 		      						 	      								  (10)

[P.T.O.]

18.	During preparation of dough, 700 g of sugar are mixed with 100 kg of flour. Ten 100 g samples are taken after 1, 5 and 10 minutes and analyzed for the percentage sugar. The results are as follows.

Percentage after 1 min   0.21  0.32  0.46  0.17  0.89  1.00  0.98  0.23  0.10  0.14

Percentage after 5 min   0.85  0.80  0.62  0.78  0.75  0.39  0.84  0.96  0.58  0.47

Percentage after 10 min 0.72  0.69  0.71  0.70  0.68  0.71  0.70  0.72  0.70  0.70

Calculate the mixing index for each mixing time and draw conclusions regarding the efficiency of mixing. Assume that for ‘perfect mixing’ there is a probability that 99.7% of samples will fall within three standard deviations of the mean composition (σ = 0.01%).

(OR)

19.	a. 	Olive oil and rape-seed oil are blended in a ratio of 1 to 5 by a propeller agitator 20 cm  in 	diameter operating at 750 rev min-1 in a cylindrical tank 1 m in diameter at 20ºC. Calculate 	the size of the motor required. (The viscosity of olive oil at 20ºC is 0.084 N s m2, the 	density of olive oil is 910 kg m3, the viscosity of rape-seed oil is 0.118 N s m2 and the 	density of rapeseed oil is 900 kg m3).											  (10)

b. 	Write a note on pie and biscuit formers.		                 		    					    (5)

20.	a. 	Explain in detail the different size reduction equipments used for dry foods. 			  (10)

b. 	Explain the effect of size reduction on foods.										    (5)

(OR)

21.	a. 	Food is milled from 6 mm to 0.0012 mm using a 10 hp motor. Would this motor be 	adequate to reduce the size of the particles to 0.0008 mm? Assume Rittinger’s equation 	and that 1 hp = 745.7 W.							 							    (5)

b. 	Explain the theory of size reduction in liquid foods.			 					  (10)

22.	Explain in detail the different peeling methods.

(OR)

23.	Discuss briefly about blanching.

24.	Five-centimeter  potato cubes are individually quick frozen (IQF) in a blast freezer operating at –40ºC and with a surface heat transfer coefficient of 30 W m2 K1. If the freezing point of the potato is measured as 1.0ºC and the density is 1180 kg m3, calculate the expected freezing time for each cube. If the cubes are then packed into a cardboard carton measuring 20 cm×10 cm×10 cm, calculate the freezing time. Also calculate the freezing time for IQF freezing of    2.5 cm cubes. (Additional data: the thickness of the card is 1.5 mm, the thermal conductivity of the card is 0.07 W m1 K1, the thermal conductivity of potato is 2.5 W m1 K1 and the latent heat of crystallization 2.74 ×105 J kg1.)

(OR)

25.	a. 	Explain the theory of freeze concentration and the equipment used for freeze 	concentration.																  (10)

b. 	Explain the working of cooled liquid freezer. 									    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DRYING TECHNOLOGY								      Time : 3 hours

Subject Code:	09FP202									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the three types of moistures present in a food material?

2.	Convert 850 % dry basis moisture content to wet basis content.

3.	The capacity of a drum dryer depends on _________ and _________.

4.	Give examples for osmotic solvents.

5.	Enumerate the applications of spray drying in food industries.

6.	Define efficiency of spray dryer.

7.	Write any two advantages of freeze drying.

8.	The three ways in which microwave and dielectric energy may be combined with conventional

drying methods are _________, _________ and _________.

9.	Fluidized bed dryer is used for drying of powders in the particle size range of _________ to _________.

10.	Give some examples of food products suitable for drying in Splash dryer.

PART – B  (5 x 3 = 15 MARKS)

11.	500 kg of paddy at 22% moisture content (wb) is dried to a final moisture content of 14% (wb) for milling. Calculate the amount of moisture removed in drying.

12.	Write a note on the process steps involved in foam mat drying.

13.	Give the significance of atomization process in spray drying.

14.	Explain ionic conduction.

15.	Write a short note on the effect of operating parameters on fluidized bed drying.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the values of ‘c’ and ‘n’ from the Henderson’s equation for the following data obtained from thin layer drying of paddy.

RH = 30%	t = 50 ℃		Me = 10.5%

RH = 55%	t = 50 ℃		Me = 15.5%

(OR)

17.	Explain the principle and working of cabinet and vacuum dryers with neat diagrams.

18.	Describe the operation of the atmospheric twin drum dryer with a neat sketch.

(OR)

19.	Discuss the factors that influence osmotic dehydration process.

20.	Explain the design considerations of spray dryer chamber and draw the diagram of spray dryer layout.

(OR)

21.	Write a detailed description about the powder separators used in spray drying process.

22.	Describe microwave freeze drying process.

(OR)

[P.T.O.]

23.	Explain in detail about the interaction of electromagnetic fields with food materials.

24.	Explain the working of the following:

a.	Vibrated fluidized bed dryers.	 												(8)

b.	Hybrid fluidized bed dryers.													(7)

(OR)

25.	Discuss about the design considerations in flash dryer.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title:	FOOD SAFETY REGULATIONS AND CONTROL 		       Time: 3 hours

Subject Code:	09FP205	       	               	       	  			              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is the role of the World Health Organisation in food regulations?

2.	State one current issue under consideration for maximum residue limit.

3.	State any one law relating to processing of fruits and vegetables.

4.	Who is authorized to license the food business?

5.	How are foods derived from modern biotechnology labeled?

6.	State two limitations of labeling.

7.	Who are the members of the HACCP team?

8.	Give two potential hazards in the processing of any one food.

9.	What are the advantages of chlorination?

10.	Name two common contaminants of water.

PART – B (5 x 3 = 15 MARKS)

11.	Give a brief history of codex alimentarius.

12.	Give two examples of commonly committed offences with respect to food quality.

13.	What are the safety issues relating to GM foods?

14.	How is a critical control point determined?

15.	Give the microbiological safety aspects of drinking water.

PART – C (5 x 15 = 75 MARKS)

16.	Explain the functioning and responsibilities of WTO.

(OR)

17.	Discuss the functions and responsibilities of ICGFI.

18.	Explain how the Food Safety Act is enforced.

(OR)

19.	Write an account of AGMARK standards for at least five food commodities.

20.	Elaborate on the EU rules on nutritional labeling.

(OR)

21.	Discuss the need for labeling, developing labeling standards and limitations of labeling.

22.	With one example explain the implementation of HACCP.

(OR)

23.	Explain the determination of critical points and the decision tree for CCPs.

24.	Elaborate on the microbiological and chemical quality regulations for bottled water.

(OR)

25.	Explain the consequences of drinking contaminated water.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	UNIT OPERATIONS IN FOOD PROCESSING II		      Time : 3 hours

Subject Code:	09FP206										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	For separation of sugar solution from settled out mud, we use a ____________ centrifuge.

2.	Separation of solids from gases is carried out by using ____________.

3.	What are Filter aids?

4.	Define Constant pressure filtration.

5.	Define the term membrane selectivity.

6.	Schmidt number is ____________.

7.	How is swirling prevented in agitators?

8.	Define Froude number.

9.	What is angle of nip in Roll Crushers?

10.	What are the two methods of particle size analysis?

PART – B  (5 x 3 = 15 MARKS)

11.	How centrifuges are classified? Give examples.

12.	Define compressibility coefficient of filter cake.

13.	Mention five uses of reverse osmosis in food industry.

14.	Write short notes on mixing viscous and plastic masses.

15.	Define Bonds law and work index.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain in detail about principle of centrifugal separation.				 			    (5)

b.	Beer with a specific gravity of 1.042 and a viscosity of 1.40 x 103 N s m2 contains 1.5% solids which have a density of 1160 kg m3. It is clarified at the rate of 240 l h1 in a bowl centrifuge which has an operating volume of 0.09 m3 and a speed of 10 000 min1. The bowl has a diameter of 5.5 cm and is fitted with a 4 cm outlet. Calculate the effect on feed rate of an increase in bowl speed to 15 000 rev min1 and the minimum rev particle size that can be removed at the higher speed.					 						  (10)

(OR)

17.	Write briefly about:

a.	Tubular centrifuge		b.	Disc Bowl centrifuge 		c.	Nozzle centrifuge.       (3x5)

18.	Obtain the expression for the constant rate filtration and constant pressure filtration.

(OR)

19.	Explain in detail about construction and working principle of plate and Frame filter press 	with a neat sketch.

20.	Explain in detail about various modules of membrane equipment with a neat sketch.

(OR)

21.	Describe the principle behind cross flow microfiltration and ultra filtration with its applications.

[P.T.O.]

22.	With a neat sketch explain the working principle of powder and particle mixture.

(OR)

23.	Obtain the expression for power requirement of mixing and agitation process.

24.	a.	Differentiate between Crushing and Grinding operations.							    (5)

b.	A certain crusher accepts a feed material having a volume – surface mean diameter of

19 mm and gives a product of volume – surface mean diameter of 5 mm. The power required to crush 15 tonnes per hour is 7.5 kW. What will be the power consumption if the capacity is reduced to 12 tonnes per hour?  										  (10)

(OR)

25.	Explain in detail about construction and working principles of Hammer mill, Roller mills and Cutters.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FOOD MICROBIOLOGY								      Time : 3 hours

Subject Code:	11FP201									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define water activity.

2.	Name any two yeasts important in food microbiology.

3.	What do you mean by radappertization?

4.	What are chemical preservatives? Give two examples.

5.	What are malt beverages?

6.	Differentiate between hetero and homo fermentative bacteria.

7.	What is an indicator organism? Give an example.

8.	Which are the sources of contamination of milk?

9.	What are the symptoms of E. coli food poisoning?

10.	What is food intoxication? Give an example.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the characteristics of any three molds useful in food industry.

12.	Why is food packaging crucial for safety and quality control?

13.	Write short notes on oriental fermented foods.

14.	Write notes on lactic starter culture with suitable examples.

15.	Name any three viruses associated with food poisoning.

PART – C  (5 x 15 = 75 MARKS)

16.	Give a detailed account on factors affecting the growth of microorganisms in food.

(OR)

17.	Discuss any three techniques employed for the isolation and enumeration of microorganisms from food samples.

18.	Discuss the types of spoilage occur in canned food.

(OR)

19.	What steps should be followed to develop a HACCP plan? Highlight the benefits of HACCP for the consumer and industry.

20.	What is sauerkraut? Describe the process involved in the production of sauerkraut.

(OR)

21.	Describe the microbiological aspects of manufacture of alcoholic beverages.\

22.	What is food poisoning? Discuss aflatoxins and their impact on human health.

(OR)

23.	Describe the preparation of cheese and yogurt.

24.	What are food-borne diseases? Explain the symptoms, prevention and treatment of botulinum.

(OR)

25.	Explain the symptoms, prevention and treatment of food poisonings caused by Staphylococcus aureus and Shigella. 															     (10+5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	PROCESS ENGINEERING CALCULATIONS			      Time : 3 hours

Subject Code:	11FP202				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write any three units of pressure.

2.	Define the term molarity.

3.	State ideal gas law.

4.	Define the term Relative humidity.

5.	State law of conservation of mass.

6.	What is meant by component mass balance?

7.	Differentiate between evaporation and drying.

8.	State the principle of membrane separations.

9.	Define the term heat capacity.

10.	Define the term heat of solution.

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss briefly about the physical properties of a solution.

12.	State Joule Thomson effect.

13.	Write the component balance and total balance for binary distillation.

14.	Calculate the amount of water evaporated for concentrating 20% of 5000 kg Caustic soda solution to  55% NaOH by weight.

15.	Define the term sensible heat and latent heat.

PART – C  (5 x 15 = 75 MARKS)

16.	A solution of caustic soda in water contains 20% NaOH by weight at 60◦C. The density of the solution is 1.196 kg/l. Find the molarity, molality and normality of the solution.

(OR)

17.	A flue gas contains CO2 , O2 and N2 and the mole fractions are 0.14, 0.06 and 0.8 respectively. If the total pressure is 101.325 KPa find the density and average molecular weight at 298K.

18.	Calculate the specific volume of superheated steam at 10 MPa and 623 K:

a.	Using ideal gas law  			b.	van der Waal’s  equation.

Data :  Pc = 22.07 MPa, Tc = 647.11 K 	for water.

(OR)

19.	The air in a room is at 26.7°C and pressure is 101.325KPa contains water vapour with a partial pressure of 2.76KPa. Calculate absolute humidity, relative humidity, percentage humidity.

20.	Soyabean seeds are extracted with hexane in batch extractors The flaked seeds contain 18.6% oil, 69% solids, 12.4% moisture at the end of the extraction  Deoiled cake (DOC) is separated from the hexane oil mixture.  DOC analysis yields 0.8% oil, 87.7% of solids and remaining moisture. Find out the percentage recovery of oil. All percentages are by weight.

(OR)

[P.T.O.]

21.	An absorbtion tower with packing is used to absorb CO2 in an aqueous monoethanol amine 	MEA solution. The volumetric flow rate of incoming dry gas mixture is 1000 m3/hr at 45◦C and 	101.3 KPa . The CO2  content of the gas is 10.4 mol % while the outgoing gas mixture contains 	4.5 mol%  CO2. A 3.2 M MEA solution is introduced at the top of the tower at the rate of 	0.625lit/s. Dissolved CO2  concentration  of the entering solution is 0.166 kmol/ kmol of MEA. 	Find the concentration of dissolved CO2 in the solution leaving the tower.

22.	In a process of producing Jam, crushed fruit containing 14wt% solids is mixed with 1.22 Kg of sugar per kg of crushed fruit and 0.0025 Kg of pectin per kg of crushed fruit. The resultant mixture is evaporated in a kettle to produce Jam containing 67% solids. For a feed of 1000 Kg of Crushed fruit find kg of water evaporated and amount of jam produced.

(OR)

23.	Explain briefly about material balances in drying operations and discuss about concentration by non thermal process using membrane separations.

24.	Calculate the heat required to raise the temperature of CO2 from 298 K to 1273 K.

Cp  of CO2 = 6.85 + 0.0083 T 0.00025 T2  where T is in K  and Cp in Kcal/kgmol K.

(OR)

25.	a.	Discuss about Clasius Clayperon equation for latent heat calculation

b.	Discuss bout effect of temperature on heat capacity.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	PRINCIPLES OF FLUID MECHANICS AND HEAT TRANSFER														      							      Time : 3 hours

Subject Code:	11FP204				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	For a solid the strain is a function of the ___________.

2.	Fluids in which the value of m is not constant are known as ___________.

3.	There are ___________ fundamental kinds of physical entities in thermodynamics.

4.	___________ is concerned with transport processes and with the rates of chemical reactions.

5.	The specific enthalpy of evaporation decrease with ___________.

6.	The enthalpy of vaporization is the energy required to turn water into the gaseous form when it ___________.

7.	Thermal contact conductance is the study of heat conduction between ___________.

8.	The law of heat conduction, also known as ___________.

9.	The process of transport by fluid streaming is known as ___________.

10.	Heat can be transmitted though empty space by thermal radiation often called ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the causes of viscosity in liquids.

12.	Discuss the Zeroth Law of thermodynamics.

13.	Write short notes on properties of steam.

14.	What is the difference between conduction and convection?

15.	Define Newton's law of cooling.

PART – C  (5 x 15 = 75 MARKS)

16.	Briefly explain the working principles of the Veturimeter with a neat sketch.

(OR)

17.	In detail explain the Bernoulli equation of work and energy.

18.	Discuss the working principle of Carnot cycle.

(OR)

19.	Explain the three laws of Thermodynamics in detail.

20.	Explain in detail about the Constant pressure and constant volume processes.

(OR)

21.	Discuss the Phase transformation at constant pressure.

22.	a.	What are the different modes of heat transfer? Explain. 							    (9)

b.	Define theory of insulation.													    (6)

(OR)

23.	Briefly explain the steady heat conduction in plane walls without heat generation.

24.	Explain the forced convection heat transfer performance of an internally finned tube.

(OR)

25.	a.	In detail discuss the heat transfer coefficient for laminar flow.						    (9)

b.	Explain the mixed convection.													    (6)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FOOD BIOCHEMISTRY AND NUTRITION				      Time : 3 hours

Subject Code:	11FP205				 					      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the site of ETC?

2. 	How many moles of ATP are produced on complete oxidation of one molecule of glucose?

3. 	Name the carrier protein involved in the oxidation of fatty acids.

4. 	Give an example of any two essential amino acids.

5. 	Define energy value.

6. 	What is balanced diet?

7. 	What are enzyme inhibitors?

8. 	Explain “phytates”.

9. 	What are the causes of hypertension?

10. 	List types of diabetic.

PART – B  (5 x 3 = 15 MARKS)

11.	Write a short note on high energy compounds.

12.	How fatty acids get activated during their oxidation?

13. 	Write a short note on Nutritional labeling.

14. 	Write a short note on the fat metabolism.

15. 	Give a brief account on functional foods.

PART – C  (5 x 15 = 75 MARKS)

16. 	TCA cycle is an amphibolic pathway. Explain how with cycle.

(OR)

17. 	Enumerate the process of synthesis of glucose from non carbohydrate sources.

18. 	Explain the process of cholesterol biosynthesis.

(OR)

19. 	Give an account of synthesis of proteins.

20. 	Explain in detail about malnutrition its implication and solution.

(OR)

21. 	Write about importance of diet and dietary intake.

22. 	Discuss nutrition disorder associated with liver and kidney.

(OR)

23. 	Discuss in detail about naturally occurring anti nutritional factors.

24. 	Explain in detail about nutrition for specialized purpose.

(OR)

25. 	Write a detailed note on nutrition and aging.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	UNIT OPERATIONS IN FOOD PROCESSING – I	            Time : 3 hours

Subject Code:	11FP208										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Energy is an ___________ property

2.	The dimension for viscosity is ___________.

3.	At boiling point ___________ will be equal to atmospheric pressure.

4.	Steam distillation is only applicable for system ___________ with water.

5.	Long tube vertical evaporator is suitable for concentrating ___________.

6.	There are ___________ different methods of feeding the multiple effect evaporators.

7.	The two phases at equilibrium in liquid-liquid extraction are extract phase and ___________ phase.

8.	In liquid-liquid extraction the mixture should essentially form ___________ phase.

9.	Solid-liquid extraction is called ___________.

10.	Carbon di oxide is used as the solvent in ___________ fluid extraction.

PART – B  (5 x 3 = 15 MARKS)

11.	What is excess and limiting reactant?

12.	What is relative volatility and what is its significance?

13.	What is the purpose of entrainment separator in Evaporator?

14.	Define distribution coefficient in liquid-liquid extraction.

15.	What is the advantage of supercritical fluid extraction over solvent extraction process?

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	A solution of sodium chloride in water contains 20% NaCl (by mass) at 333 K (60°C). The 	density of the solution is 1.127 Kg/L. Find the molarity, normality and molality of the 	solution.	 																	  (10)

b. 	What is ideal gas law and how to calculate Universal gas R from standard definition?	   (5)

(OR)

17.	a. 	How to calculate average molecular weight for the gaseous mixture? 				    (5)

b. 	A saturated solution of salicylic acid in methanol contains 64 kg salicylic acid per 100 kg methanol at 25°C. Find mass% and mole % composition of the solution. 			  (10)

18.	Starting from material balance explain the method of calculating the number of theoretical stages using Mc Cabe Thiele Method.

(OR)

19.	Explain with a neat sketch the construction and operation of packed tower distillation column.

20.	Why multiple effect evaporator is needed? Explain various methods of feeding the multiple effect evaporators.

(OR)

21.	Write a short note on:

a.	Boiling point elevation. 														    (7)

b.   	Falling film evaporator. 														    (8)

[P.T.O]

22.	Explain liquid extraction in detail with triangular diagram.

(OR)

23.	What are the stages of crystallization? Explain the solubility curve with an example.

24.	What are the properties of supercritical fluids? How it can be used to extract essential oils? Explain with suitable a example.

(OR)

25.	Write short note on:

a.	Screw press. 																	    (8)

b.	Filter press for the extraction of oil. 											    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FOOD CHEMISTRY									      Time : 3 hours

Subject Code:	11FP212									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define Monosaccharides.

2. 	What do you mean by Bound water?

3. 	What are polysaccharides?

4. 	Name the two types of polysaccharides present in Starch.

5. 	What is EFA?

6. 	Define Derived lipids.

7. 	Define enzyme Specificity.

8. 	Give one example for Essential amino acid.

9. 	Rickets is caused due to deficiency of ___________.

10. 	Mention any one deficiency diseases of Thiamin.

PART – B  (5 x 3 = 15 MARKS)

11. 	Explain water activity.

12. 	Write a short note on dietary fibre.

13. 	Explain Rancidity of fats.

14. 	What is isoelectric point?

15. 	Write a note on Riboflavin.

PART – C  (5 x 15 = 75 MARKS)

16. 	Discuss briefly about chemical reactions of Carbohydrates.

(OR)

17. 	Write a short note on:		a.	Sorption isotherms 	b.	Classification of Carbohydrates.

18. 	Discuss briefly about Starch and its hydrolytic products.

(OR)

19. 	Write a brief notes on:		a.	Gums		b.	Cyclodextrins  	c.	Pectins.

20. 	Describe in detail about the various steps in the production of refined edible oil with a neat 	flow diagram.

(OR)

21. 	Write a brief notes on:

a.	Chemistry of fatty acids and glycerides 		b.	Auto Oxidation and Hydrolysis.

22.  Discuss in detail about nomenclature, classification, structure and chemistry of amino acids, 	peptides and proteins.

(OR)

23. Discuss in detail about the importance of enzymes in food industry with suitable	 examples.

24.  Discuss in detail about Fat soluble Vitamins and its deficiency diseases.

(OR)

25. 	Explain in detail about Water soluble Vitamins and its deficiency diseases.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FRUIT AND VEGETABLE PROCESSING TECHNOLOGY

Time : 3 hours

Subject Code:	11FP214		 								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the composition of any one variety of mango fruit.

2.	List the factors in fresh cut processing of vegetables to be considered in minimizing risk in the production–consumption chain.

3.	Define the process of blanching.

4.	Name two preservatives used in squashes.

5.	Give two advantages of spray drying.

6.	Define aroma stripping.

7.	State the working principle of freeze drying.

8.	Name two intermediate moisture foods.

9.	What is a puree?

10.	Give two advantages of tetra packs to customers.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the main causes for the post-harvest loss in tomatoes?

12.	What are the methods used for fruit juice clarification?

13.	What is the advantage of cold press juicers?

14.	What is the principle of vaccum dryers?

15.	Give two applications of reverse osmosis.

PART – C  (5 x 15 = 75 MARKS)

16. 	Elaborate on five factors responsible for spoilage of fruits and vegetables.

(OR)

17. 	Explain three general methods of preservation of whole fruits and vegetables.

18. 	Describe the preparation of jam, jellies and marmalade. Give the FPO specification for the same.

(OR)

19. 	Explain the basic concepts of hurdle technology.

20. 	With a flow chart explain the preparation of fruit bars.  Write a note on the equipment required for the process.

(OR)

21. 	Explain the extraction of mango pulp with a flow diagram.  Describe three types of evaporators used for the purpose.

22. 	Explain the principles of dehydration and the equipment used for the purpose.

(OR)

23. 	Give an account of the different types of packages used for dehydrated fruit and vegetable products.

[P.T.O]

24. 	Explain the aseptic packaging methodology. Give the structure of an aseptic bag.  List the advantages of this technology.

(OR)

25. 	Elaborate on the different systems used for filling small quantities and bulk storage.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FOOD PACKAGING TECHNOLOGY					      Time : 3 hours

Subject Code:	11FP218									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define gas transmission rate.

2.	Define the term "shelf life".

3.	List the metallic can types.

4.	What is meant by aerosol cans?

5.	Write the composition of cellulose.

6.	What is meant by thermoforming?

7.	What is barcodes?

8.	What is meant by hot wire sealing?

9.	Define the term permeability of gases in packs.

10.	Write any two merits of packaging using sachets.

PART – B  (5 x 3 = 15 MARKS)

11.	Indicate any three types of testing on packaging.

12.	What are the basics of canning operation?

13.	Write the advantages of pouch packaging.

14.	Differentiate hot bar sealing and impulse sealing.

15.	How is moisture controlled?

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the environmental factors and biological factors on quality of the foods.

(OR)

17.	Explain in detail the analysis of storage requirement and accelerated storage studies.

18.	Explain briefly the specialty of open top sanitary cans, three piece cans and two piece cans.

(OR)

19.	Illustrate the design features and applications of metal cans as packaging.

20.	Enumerate briefly the plastics used and their specific applications.

(OR)

21.	Explain the process of blow moulding, injection moulding and extrusion of flexible films.

22.	Write a detailed note on seal types and advantages.

(OR)

23.	Discuss the necessity of nutrition labeling and legislative requirements.

24.	Explain the active packaging and factors to be considered.

(OR)

25.	Explain the principle and applications of modified atmosphere packaging and specialty packages.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	MILLING AND BAKERY TECHNOLOGY				      Time : 3 hours

Subject Code:	11FP219									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Enlist the types of wheat.

2. 	What is Farinograph?

3. 	What is the significance of conditioning in wheat milling?

4. 	What is macaroni?

5. 	Enlist shortening agents used in bakery.

6. 	What is the ratio of sugar to butter in butter icing?

7. 	Why the dough possess elastic behavior?

8. 	What is the proofing period for bread manufacture?

9.  	Name the enzyme present in flour.

10. 	Enlist the uses of milk in cake making.

PART – B  (5 x 3 = 15 MARKS)

11. 	Enumerate the byproducts utilization of wheat mill.

12. 	Explain the working principle of pneumatic conveyor.

13. 	Give an account on different types of leavening agents used in bakery.

14. 	Enumerate the equipments used for dough rheology.

15.  Explain various types of biscuit doughs.

PART – C  (5 x 15 = 75 MARKS)

16.  	Give a detailed note on various tests of wheat and wheat flour and their significance in bread and chapathi making.

(OR)

17. 	a. 	Explain the manufacturing process of atta and semolina with a process flow chart.	    (8)

b. 	Explain the construction and working of plan sifter with a neat diagram.			    (7)

18. 	Discuss about the various unit operations of wheat milling with a flow diagram

(OR)

19. 	Explain in detail about various rolls used in the manufacture of wheat products.

20. 	Explain various types of mixers used for dough with a suitable sketch.

(OR)

21. 	Give a detailed account on the production control, laboratory norms and quality control of 	bakery products.

22. 	Discuss the role of different ingredients in bread making. Mention the steps involved in         straight dough method.

(OR)

23. 	a. 	Write a note on spoilage in Bread.												    (7)

b. 	Discuss about dough retarding and freezing.                                                               	    (8)

[P.T.O]

24. 	Discuss the manufacturing process of biscuits and wafers with a flow diagram.

(OR)

25. 	Write the importance of cake decoration. Enumerate the common types of icings used for cake decoration

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	PLANTATION PRODUCTS AND SPICES TECHNOLOGY

Time : 3 hours

Subject Code:	11FP220										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The coffee plant belongs to the__________ family.

2.	__________ involves removal of the silver skin in the center cut of the beans.

3.	CTC stands for __________.

4.	__________ is done in a special dryer at a high temperature after fermentation of tea.

5.	__________  and __________ are the types of fermentation of cocoa beans.

6.	The critical moisture content of dried cocoa lies between __________ and __________ (w.b).

7.	__________ ginger is preserved in brine or sugar syrup.

8.	The turmeric oil contains about __________ and  __________ .

9.	__________ has been identified as major component in clove.

10.	__________ is responsible for odor and flavor of cumin seeds.

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly discuss the chemical composition of coffee.

12.	What are the chemical and biochemical changes occuring during withering?

13.	Name the microorganisms and their role in fermentation of cocoa bean.

14.	How is paprika produced?

15.	Give a brief note on curing process of vanilla.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail the wet method preparation of parchment coffee with a neat flow diagram.

(OR)

17.	Write short notes on brewing and quality grading coffee.

18.	Discuss about chemical composition and green tea production in detail.

(OR)

19.	With a neat process flow sheet, explain in detail the steps involved in manufacturing process of black tea.

20.	Describe the processing and changes during fermentation of cocoa bean in detail.

(OR)

21.	Illustrate the production of chocolate and add a note on quality control measures.

22.	Give a detailed note on oleoresins and essential oil production of pepper. Mention its functional properties.

(OR)

23.	Explain the chemical structure and post harvest technology of ginger in detail.

24.	Enumerate the processing method of cinnamon quills and their oil production.

(OR)

25.	Describe in detail the dehydration process of garlic with a neat flow chart.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ENGINEERING PROPERTIES OF FOOD MATERIALS	      Time : 3 hours

Subject Code:	11FP221										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Give one important application of Sphericity.

2.	Define Roundness.

3.	Define Rheological properties.

4.	What do you mean by gumminess?

5.	Write down the factors affecting thermal conductivity.

6.	In thermal conduction ___________ provides resistance.

7.	What do you mean by pressure drag?

8.	Define friction drag.

9.	Give the frequency range of microwaves.

10.	An example of dipole is ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Write a short note on angle of repose.

12.	Enumerate the reasons for study of Rheological properties of foods.

13.	Define thermal diffusivity.

14.	Write a note on lift.

15.	Write in brief about dielectric constant and loss factor.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the density method of estimation of porosity.								    (9)

b.	The porosity of dried apples was measured using an air comparison pycnometer with

two identical chambers. The dried apple sample was placed in chamber 2. Valves 2 and 3 were closed and air was supplied to chamber 1. Valve 1 was closed and pressure P 1 was read as 0.508 atm. Then, valve 2 was opened and the new equilibrium pressure, P 2, was read as 0.309 atm. Calculate the porosity of the dried apple.						    (6)

(OR)

17.	Define the following:

a.	Bulk density.			b.	Static friction.		c.	Equilibrium moisture content.

d.	Water activity.		e.	Sorption isotherm.

18.	Describe in detail about the Texture Profile Analysis in food engineering.

(OR)

19.	a.  	Two parallel plates are 0.1 m apart. The bottom plate is stationary while the upper one

is moving with a velocity V. The fluid between the plates is water, which has a viscosity of 1 cp.

i.	Calculate the momentum ﬂux necessary to maintain the top plate in motion at a 	velocity of 0.30 m/s.

ii.	If water is replaced with a fluid of viscosity 100 cp, and momentum flux remains 			constant, find the new velocity of the top plate.								    (8)

[P.T.O.]

b.    Explain in brief about Newtonian and Non-Newtonian fluids.				        	    (7)

20.	Describe the methods of measuring thermal conductivities of powders and granular materials

(OR)

21.	a. 	A vacuum jacketed calorimeter of 86 g was used for the determination of speciﬁc heat of 	36 g of lean beef sample. The calorimeter cup and the sample were ﬁrst brought to 12 ◦C 	and then 68 g of water at 6 ◦C was poured into the cup. The system was then sealed and 	brought to equilibrium. The equilibrium temperature was recorded to be 7.8◦C. The 	specific heats of water and calorimeter cup are 4198 J/kg ◦C and 383 J/kg ◦C, respectively. 	The calorimeter was well insulated and the heat loss to the surrounding was negligible. 	Calculate the specific heat of the sample.	 									    (8)

b. 	How do you determine specific heat using method of comparison calorimeter?		    (7)

22.	Determine the viscosity of sunflower oil, using a falling ball viscometer.  Viscometer has a tube length of 10 cm and its ball has a diameter of 0.68 mm. Oil and the ball have densities of 921 kg/m3 and 2420 kg/m3, respectively. If it takes 44.5 seconds for the ball to fall from the top of the tube, calculate the viscosity of the oil.

(OR)

23.  Explain in detail about:

a.	drag 			b.	terminal velocity.

24.	a. 	Estimate the penetration depth of a chicken meat during processing in home type 				microwave oven. Chicken meat has a dielectric constant of 53.2 and dielectric loss factor 			of 18.1. Assume that dielectric properties are constant during heating. 				    (7)

b.  	Write a detailed note on dielectric property of sugars.								    (8)

(OR)

25.	Discuss in detail about effect of moisture content over dielectric properties.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FOOD REFRIGERATION AND COLD STORAGE CONSTRUCTION														      					      Time : 3 hours

Subject Code:	11FP229										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define ton of refrigeration.

2. 	Refrigerant used in domestic refrigerator is ___________.

3. 	Define COP.

4. 	The difference between dry bulb and wet bulb temperature is known as ___________.

5. 	During vapour compression refrigeration cycle, heat is rejected in___________.

6. 	During sensible cooling of air the specific humidity ___________.

7. 	For unsaturated air the dew point temperature is ___________ the wet bulb.

8. 	At saturated condition the dew point temperature is ___________ the wet bulb temperature

9. 	Define Dew point temperature.

10. 	Azeotropes are ___________.

PART – B  (5 x 3 = 15 MARKS)

11. 	What is evaporative condenser?

12. 	Explain the term ‘ozone depleting potential’.

13. 	Discuss pre cooling.

14. 	Explain ideal refrigerant.

15. 	Define evaporative cooling.

PART – C  (5 x 15 = 75 MARKS)

16. 	Discuss working of vapor compression cycle with a neat sketch.

(OR)

17. 	Explain types of compressor and evaporators.

18. 	Discuss cooling load estimation in cold storage plant.

(OR)

19. 	Explain optimum storage temperature of various food products.

20. 	Discuss controlled atmospheric storage and their applications.

(OR)

21. 	Discuss modified atmospheric storage and their applications.

22. 	Write a detailed note on status and application of cold storage.

(OR)

23. 	Write a note on food transport refrigeration in detail.

24. 	Explain different types of freezing equipments used for freezing foods.

(OR)

25.  	Discuss in detail about cryogenic freezing of foods.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	THERMODYNAMICS									      Time : 3 hours

Subject Code:	12FP202									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Distinguish between system and surroundings.

2.	Define thermodynamic equilibrium.

3.	Define heat capacity.

4.	Write the mathematical equation of first law of thermodynamics for flow and non-flow process.

5.	Entropy of universe is always increasing. Justify this statement.

6.	Carnot cycle contains ________ adiabatic and ________ isothermal process.

7.	Joule Thomson coefficient for ideal gas is ________.

8.	Define COP of refrigeration and write the relation between COP of refrigeration and COP of heat pump.

9.	Define Fugacity Coefficient.

10.	Define residual entropy and residual enthalpy.

PART – B  (5 x 3 = 15 MARKS)

11.	What are intensive and extensive properties? Give examples for each.

12.	Derive work expression for isothermal process of ideal gases.

13.	Explain in detail about ideal Rankine cycle and give its efficiency equation.

14.	Explain Clausius inequality in detail.

15.	What are the effects of temperature and pressure on fugacity?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Differentiate between path and point properties.									    (5)

b.	Explain in detail about working principle of Heat engine and Heat pump with a neat 	diagram.																		  (10)

(OR)

17.	One mole of a gas which obeys ideal gas equation is initially at 300 K and 0.1 MPa. The gas is heated at constant volume till the pressure rises to 0.5 MPa and then allowed to expand at constant temperature till the pressure reduces to 0.1 MPa. Finally the gas is returned to its original state by compressing at constant pressure. Calculate the work done by the gas in each of the processes and also estimate the net work done by the gas.

18.	Derive the first law of thermodynamics for non-flow and flow process.

(OR)

19.	a.	Air at 1bar and 25ºC enters a compressor at low velocity, discharges at 3 bar, and enters a 		nozzle in which it expands to final velocity of 600m/s at the initial condition of pressure 			and temperature. If work of compression is 240KJ/kg of air, how much heat must be 				removed during compression? 								 					    (5)

b.	One kilo mole of gas for which PV = nRT where R = 8.314 KJ/kmolK is originally at 300K and 1bar. It is then heated at constant pressure to a temperature of 400K and compressed 	isothermally to a 	volume equal to its initial volume. Assume that               Cp= 30KJ/kmol K. Find change in internal energy (ΔU), change in enthalpy (ΔH), Heat (Q) and work (W) interaction during the process.								  	  (10)

[P.T.O.]

20.	State and prove the Carnot theorem in detail with Carnot cycle.

(OR)

21.	a.	A refrigeration machine operating at a condenser temperature of 290 K needs 1 KW of 	 	power per ton of refrigeration. Determine the following:

i.	The coefficient of performance.

ii.	The heat rejected to the condenser.

iii.	The lowest temperature that can be maintained. 								   (5)

b.	Explain with neat diagram about vapour –compression refrigeration cycle.  			  (10)

22.	a.	Explain briefly about Clapeyron equation.						   				    (5)

b.	Draw and explain T-S diagram for a pure substance.								  (10)

(OR)

23.	a.	Show that Joule-Thomson coefficient for ideal gas is zero. 						    (5)

b.	Show that for van der-Waals gas, Joule-Thomson coefficient is given by

μJT=1Cp2aRT−b															  (10)

24.	Explain in detail about PVT behaviour of pure substances with neat diagrams.

(OR)

25.	Explain in detail about methods available for the determination of partial molar properties.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FOOD MICROBIOLOGY								      Time : 3 hours

Subject Code:	12FP205									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name any two beneficial molds in food microbiology.

2.	What are antimicrobial barriers?

3.	What do you mean by pasteurisation?

4.	Name any two natural food preservatives.

5.	What is sauerkraut?

6.	Define fermetnation.

7.	What are canned foods?

8.	What are coliforms? Give an example.

9.	What is food intoxication?

10.	What is botulinum toxin?

PART – B  (5 x 3 = 15 MARKS)

11.	Give a note on the growth curve of bacteria.

12.	Write a note on heat resistance of bacteria and their spores.

13.	Give a short note on traditional vegetable fermentation.

14.	Briefly explain about the microbiology of milk.

15.	Write the mechanism of action of Shigella toxin in human.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the factors affecting the growth of microorganisms in food.

(OR)

17.	Describe the methods of screening, isolation and enumeration of microorganisms in food samples.

18.	Explain the non-thermal methods of preservation in detail.

(OR)

19.	Illustrate the preservatives used in food products and mention its tolerance limit, mode of action in inhibition of microbial growth in foods.

20.	Describe the process involved in the large scale production of citric acid.

(OR)

21.	Explain the microbiological aspects of manufacture of alcoholic beverages.

22.	Illustrate the microbiology of water and the importance of processing of foods in industries.

(OR)

23.	Give a detailed account on the aflatoxin and their importance in canned foods.

24.	What is food poisoning? Discuss the symptoms, prevention and treatment of E. coli.

(OR)

25.	What are food-borne diseases? Explain the symptoms, prevention and treatment of food poisonings caused by Hepatitis.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	CEREALS AND PULSES TECHNOLOGY				      Time : 3 hours

Subject Code:	12FP208									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is puffed rice?

2. 	Split pulses after removal of the husk is called ___________.

3. 	Soaking of the pulse reduces ___________ factors.

4. 	List of any four traditional above ground storage methods.

5. 	Protein content of maize is ___________.

6. 	List any two disadvantages of bag storage.

7. 	List any two chemical methods employed to prevent insect infestation during storage.

8. 	What is a fumigant?

9. 	The relative humidity of ___________ is maintained in the grain storage.

10.  	___________ acid is used for degumming corn oil.

PART – B  (5 x 3 = 15 MARKS)

11. 	What is the composition of paddy and state its quality characteristics.

12. 	Write a note on the uses of rice bran oil.

13. 	What are the three major pulses grown in India? Give its uses.

14. 	Write a note on the process of semolina.

15. 	List out the merits and demerits of CAP storage.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the parboiling methods and the physiochemical changes during parboiling.

(OR)

17.	Describe the processing of paddy by products.

18. 	Illustrate on rice milling with a flowchart.

(OR)

19.	Enumerate the machineries used in rice milling with a diagram.

20. 	Discuss the traditional milling process of pulses.

(OR)

21.	Discuss dhals under the following heads:

a.	milling 			b.	advantage and disadvantage of milling        c.	uses

22.	Discuss on wet milling of corn and write a note on extraction and refining of corn oil in brief.

(OR)

23. 	Explain the use of starch for value added products.

24. 	Account on the types of bulk storage followed in India and discuss its merits.

(OR)

25. 	Enumerate on the quality changes of grain during storage and state its remedial measures.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	HEAT AND MASS TRANSFER							      Time : 3 hours

Subject Code:	12FP209				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Transfer of heat by conduction does not occur if the ___________.

2. 	Lumped heat transfer analysis of a solid subject suddenly exposed to a fluid medium at a different temperature is valid when ___________.

3. 	Ratio of heat transfer rate to the rate at which heat would be conducted within the fluid under a temperature gradient is known as  ___________.

4. 	Minimum value of Prandtl number is generally of ___________.

5. 	Air enters a counter flow heat exchanger at 700 C and leaves at 400 C. Water enters at 300 C and leaves at 500 C. The LMTD in deg C is ___________.

6. 	Baffles are provided in heat exchangers to ___________.

7. 	The amount of radiation mainly depends on ___________.

8. 	Absorptivity and reflectivity of a white body respectively are ___________.

9. 	Diffusion coefficient of water into air at 260C is nearly___________.

10.	Write the expression of Fick’s first law ___________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Discuss the basic difference between thermodynamics and heat transfer.

12. 	What do you understand by free and forced convection?

13. 	Write short notes on drop wise condensation?

14. 	What is meant by shape factor and mention its physical significance?

15. 	Define forced convective mass transfer with example.

PART – C  (5 x 15 = 75 MARKS)

16. 	Steam at T∞1 = 320°C flows in a cast iron pipe (k = 80 W/m °C) whose inner and outer diameter are D1 = 5 cm and D2 = 5.5 cm, respectively. The pipe is covered with 3 cm thick glass insulation with k = 0.05 W/m °C. Heat is lost to the surroundings at T∞2 = 5°C by natural convection and radiation, with a combined heat transfer coefficient of h2 = 18 W/m2 °C. Taking the heat transfer coefficient inside the pipe to be h1 = 60 W/m2 °C, determine the rate of heat loss from the steam per unit length of the pipe. Also determine the temperature drop across the pipe shell and the insulation.

(OR)

17. 	An aluminum plate (k = 160 W/mºc, ρ = 2790 kg/m3, Cp = 0.88 kJ /kg ºC) of thickness             L = 3 cm and at a uniform temperature of 225ºC is suddenly immersed at time t =0 in a well stirred fluid maintained at a constant temperature T∞ = 25ºC. Take h = 320 W/ m2 ºC. Determine the time required for the centre of the plate to reach 50ºC.

18.	In a surface condenser, water flows through staggered tubes while the air is passed in cross flow over the tubes. The temperature and velocity of air are 30°C and 8 m/s respectively. The longitudinal and transverse pitches are 22 mm and 20 mm respectively. The tube outside diameter is 18 mm and tube surface temperature is 90°C. Calculate the heat transfer coefficient.

(OR)

[P.T.O.]

19. 	Engine oil at 60°C flows over the upper surface of a 5 m long flat plate whose temperature is 20°C with a velocity of 2 m/s as shown in the figure. Determine the total drag force and the rate of heat transfer per unit width of the entire plate.

20. 	The outlet header of a high pressure steam super heater consists of pipe (Є = 0.8) of diameter 27.5 cm. Its surface temperature is 500 °C. Calculate the loss of heat per unit length by radiation if it is placed in an enclosure at 30 °C.  If the header is now enveloped in a steel screen of diameter 32.5 cm and emissivity of 0.7 and the temperature of the screen is 340°C, find the reduction in heat by radiation due to provision of this screen

(OR)

21.	A furnace of 25 m2 area and 12 m3 volume is maintained at a temperature of 925 °C over its entire volume. The total pressure of the combustion gases is 3 atm, the partial pressure of the water vapour is 0.1 atm and that of CO2 is 0.25 atm. Calculate the emissivity of the gaseous mixture.

22. 	A parallel flow heat exchanger has hot and cold water stream running through it, the flow rates are 10 and 25 kg / min respectively. Inlet temperatures are 75ºC and 25ºC on hot and cold side. The exit temperature on the hot side should not exceed 50ºC. Assume hi = h0 = 600 W/m2 K. Calculate the area of heat exchanger using € - NTU approach.

(OR)

23. 	Water at atmospheric pressure is boiled in a kettle of copper. The bottom of the kettle is flat,     30 cm in diameter and is maintained at a temperature of 118°C. Calculate the rate of heat required to boil water. Also estimate the rate of evaporation of water from the kettle.

24. 	Water has been spilled out through the accidental valve opening of a pipe line on the floor of an industrial plant. The water layer is 1 mm and at 25 °C. The air is also at 25 °C and 1 atm. The absolute humidity of air is 2 gms per kg of dry air. Determine the time required to evaporate the complete water if the evaporation takes place by molecular diffusion through an air film of 5 mm thickness.

(OR)

25.	Air with a velocity of 3 m/s is flowing over a tray full of water. Assuming temperature of air   20 °C and the temperature of the water on the surface 15 °C determine the amount of water evaporated per hour. Length of the tray along the air flow direction is 30 cm and its width is 50 cm.

Take the total pressure of air as 1.03 bar-ab and partial pressure of water vapour in it as 0.008 bar-ab.

Take the following properties of air.

ρ = 1.205 kg/m3 , Cp = 1000 J/kg-K.

K = 0.0223 W/m-K, v = 15 *10-6 m2/s.

Pr = 0.703, D = 0.15m2/hr.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FOOD BIOCHEMISTRY AND NUTRITION				      Time : 3 hours

Subject Code:	12FP210		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The deoxyribonucleic acid of the mitochondrial genome is present in the __________.

2.	Nucleic acids are built from subunits called __________.

3.	Before entering the Krebs cycle, pyruvate is converted to __________.

4.	The rate limiting enzyme in the first stage of cholesterol biosynthesis is __________.

5.	When foods are processed, the formation of __________ reaction products, leading to a loss of lysine availability, is the most common cause of poor protein quality of foods.

6.	The energy value of foods is expressed in __________ or __________.

7.	The anti-nutritional factor in legumes that has the ability to form foams is aqueous solution is __________.

8.	Accumulation of fluid in the abdominal cavity is called __________.

9.	Malnutrition in children, in addition to the general effects of impaired tissue function also affect their __________.

10.	The screening anthropometric tool to assess if a person has a healthy weight is __________.

PART – B  (5 x 3 = 15 MARKS)

11.	List three functions of mitochondria.

12.	Give a schematic representation of tyrosine synthesis.

13.	What is the difference between recommended dietary intake and acceptable dietary intake?

14.	Name two foods that contain cyanogens and their toxic effect.

15.	Define functional foods with two examples.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the structure and functions of the nucleus.

(OR)

17. 	Explain the process of oxidative phosphorylation and its regulation.

18. 	Explain the reactions of the urea cycle.

(OR)

19. 	Describe the reactions of the TCA cycle.

20. 	Give an account of the different carbohydrates in food.  Elaborate on the components of dietary fibre in food and their health benefits.

(OR)

21. 	Give the latest recommended dietary intakes for adults.  Explain the concept of balanced diet and how it can be accomplished.

22. 	Elaborate on three inborn errors of fat metabolism.

(OR)

23. 	Elaborate on three nutritional disorders associated with the liver.

24. 	“Diabetes is a life style disorder”.  How is it treated?

(OR)

25. 	Explain the importance of nutrition for sports persons.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FOOD REFRIGERATION AND COLD STORAGE CONSTRUCTION														      					      Time : 3 hours

Subject Code:	12FP214										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	One ton of refrigeration is __________ kcal/min

2.	The anti ozonic refrigerant is __________.

3.	The refrigerant component that dissipates heat energy from the refrigerant to the surroundings is __________.

4.	The thermodynamic law on which refrigerant works__________.

5.	COP of heat pump is __________.

6.	Work done by the refrigeration unit is __________.

7.	The boiling point of refrigerant should be __________.

8.	The advisable temperature for milk storage under refrigeration is __________.

9.	In cryogenic preservation, biological substances are stored below the boiling point of __________.

10.	The phenomenon of freezing point depression is used in technical applications to avoid __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Give a note on vapour compression cycle and vapour absorption cycle.

12.	Write about coolers and refrigerated container trucks.

13.	Give notes on refrigeration and freezing.

14.	What are psychrophilic and mesophilic microbes?

15.	Give a note on supply chain management and cold chain management.

PART – C  (5 x 15 = 75 MARKS)

16.	Enumerate about components of vapour compression refrigeration system.

(OR)

17.	Explain about sensible heating, sensible cooling, adiabatic heating process using psychrometry.

18.	Discuss the cold load estimation procedure in detail.

(OR)

19.	Write the optimum temperatures for storage of different food materials.

20.	Explain the theory of freezing food materials.

(OR)

21.	Discuss the freezer types in detail.

22.	Write a note on evaporative cooling and its applications.

(OR)

23.	Enumerate the basics of chilled food microbiology.

24.	Enumerate the factors to be considered for logistic supply.

(OR)

25.	Explain in detail about traceability and barcode systems in supply chain management.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	ENGINEERING PROPERTIES OF FOOD MATERIALS	     Time : 3 hours

Subject Code:	12FP215										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The three characteristic dimensions needed for determination of size are : ________, ________ and ________.

2.	Define Roundness.

3.	Define Rheology.

4.	What are Newtonian Fluids?

5.	Define thermal conductivity of a food material and write its unit in SI system.

6.	What is thermal diffusivity?

7.	What are the different types of forces acting in falling food materials?

8.	What is an ideal solution?

9.	Define dielectric constant.

10.	What is hysteresis?

PART – B  (5 x 3 = 15 MARKS)

11.	Calculate the sphericity of a cylindrical object of diameter 1.0 cm and height 1.7 cm.

12.	Write a note on shear thinning (Pseudoplastic) fluids.

13.	Derive the equation for Fourier’s law of heat conduction through foods.

14.	An emulsion of oil in water has oil droplets in the form of spheres of average diameter    9.5x106m. The specific gravity of oil is 0.85. Consider the viscosity of water as 1x103 Pa.s. Find the raised velocity of oil globules and give your comment.

15.	Write a short note on dipolar rotation.

PART – C  (5 x 15 = 75 MARKS)

16.  	Explain the Gas displacement method for the determination of volume of particles.

(OR)

17.	Cherry has a moisture content of 77.5% (wb). The apparent and bulk densities are 615 kg/m3 and 511 kg/m3 respectively. Assuming cherry contains only carbohydrate and water, calculate the total porosity of cherries when stacked as bulk. The densities of carbohydrate and water are 1586 kg/m3 and 997 kg/m3 respectively.

18.	Describe the working of capillary flow viscometer.

(OR)

19.	Describe in detail about Extensograph and Alveograph for determining the rheological behavior of dough.

20. 	Explain the comparison calorimeter method with necessary figure for the determination of      	specific heat of food materials.

(OR)

21.   Discuss the importance of Enthalpy and Latent heat properties of food materials	.

[P.T.O.]

22.	Derive the Stoke’s law equation for finding the terminal velocity of a spherical shaped food product.

(OR)

23.	Discuss the water activity determination by vapor pressure measurement method.

24.  	Explain the effect of temperature on dielectric properties of food materials.

(OR)

25.  Describe the transmission line method of measurement of dielectric properties.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	PLANTATION PRODUCTS AND SPICES TECHNOLOGY														      								     Time : 3 hours

Subject Code:	12FP217										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The tea plant belongs to __________ family.

2.	__________ aromatic compound is present in coffee.

3.	What is green tea?

4.	Cocoa butter contains fat between __________ and __________.

5.	The type of tea in which enzymatic browning takes place during production process.

6.	French coffee contains __________% of chicory.

7.	__________ is the principal alkaloid in pepper.

8.	__________ is the aromatic component of cardamom.

9.	Component present in vanilla is __________.

10.	Essential component in turmeric is __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain curing of vanilla.

12.	Differentiate between CTC and ORTHODOX process in TEA processing.

13.	Differentiate between Natural and Dutch process in manufacture of cocoa powder.

14.	Write about oleoresins and essential oils extracted from ginger.

15.	Mention the essential oil from garlic.

PART – C  (5 x 15 = 75 MARKS)

16. 	With flow chart, explain the wet and dry method of manufacture of parchment coffee.

(OR)

17. 	Explain the process of roasting, grinding and blending of coffee beans.

18. 	Discuss the method of manufacture of black tea under CTC process.

(OR)

19. 	Explain the types of tea. Also explain the process of making green tea.

20. 	With a diagram, explain the dryers used for processing of cocoa.

(OR)

21. 	Write a short note on processing of cocoa.

22. 	Explain the methods of manufacture of essential oil and oleoresin from pepper and ginger.

(OR)

23.	Discuss about the turmeric boiling, drying and polishing operations.

24. 	Explain the complete processing scenario of garlic.

(OR)

25. 	Describe the methods of manufacture of volatiles from minor spices.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	DAIRY  ENGINEERING  AND TECHNOLOGY			      Time : 3 hours

Subject Code:	12FP218										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Solid content of milk is ___________%

2. 	The size of the fat globules is reduced in ___________.

3. 	Safe storage temperature of milk is ___________0 C

4. 	The temperature of milk in UHT processing is___________0 C

5. 	The moisture content in milk powder is___________.

6. 	___________ reduces the size of the droplet in the spray dryer.

7. 	What is the function of vacuum evaporator?

8. 	The increase in volume of ice cream mix during freezing is known as ___________.

9. 	Whey is rich in ___________.

10. 	___________ is used as heating medium in dairy industry.

PART – B  (5 x 3 = 15 MARKS)

11.	Write notes on milk as a colloidal system.

12.  Explain the importance of the homogenization.

13. 	What is meant by cheese? List out different types.

14.	Explain the over run in the ice cream.

15.	Explain the types of the cream separator.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain various tests for milk.

(OR)

17.	Briefly discuss the application of enzymes in dairy industry.

18. 	Draw a neat sketch of HTST pasteurizer and explain the working principle.

(OR)

19. 	Discuss the working principle of cream separator.

20. 	Discuss about the manufacture of sweetened condensed milk.

(OR)

21. 	Explain the process of powder manufacturing using spray dryer.

22. 	Discuss the production of ice cream.

(OR)

23. 	Discuss the production of casein in detail.

24. 	Explain the production of yoghurt from milk.

(OR)

25. 	Discuss the utilization of energy in dairy industry.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	UNIT OPERATIONS IN FOOD PROCESS ENGINEERING – II			      																		      Time : 3 hours

Subject Code:	12FP222										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Constant boiling mixture is otherwise called __________.

2.	The two types of diffusion are molecular diffusion and __________ diffusion.

3.	Extraction of oil from oil seed using hexane is __________.

4.	Extract phase and __________ phase are the two phases in liquid-liquid extraction.

5.	The two types of packings used in absorption tower are random packing and __________ packing.

6.	Adsorption is __________ phenomenon.

7.	In crystallization the mass transfer will takes place from liquid to __________.

8.	The concentration difference between the saturated and __________ solution is the driving force in crystallization.

9.	__________ is used for purifying blood.

10.	The accumulation of solids on the membrane is called membrane __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define relative volatility. What is its significance?

12.	What is binodal curve and plait point?

13.	Define flooding velocity in the absorption towers.

14.	What are the stages of crystallization?

15.	Name some of the applications of ultra filtration.

PART – C  (5 x 15 = 75 MARKS)

16.	A mixture of 35 mole % A and 65 mole % B is to be separated in distillation column. The concentration of A in the distillate is 93mole % and 96 mole% of all A is in the distillate. The feed is half vapour and reflux ratio is 4:1. The relative volatility (αAB) is 2.5. How many equilibrium stages are required in each section of column?

(OR)

17. 	Derive the expression for Rectifying operating line and Stripping operating line starting from the material balance.

18.	a. 	Explain the working principle of leaching by percolation through stationary solid. 	    (9)  b. 	Give a note on Moving bed leaching. 											    (6)

(OR)

19. 	Under what circumstances distillation is not possible? How is liquid-liquid extraction carried out?

20. 	Write short notes on:

a. 	Fixed bed adsorber. 															    (8)

b. 	Pressure swing adsorber. 														    (7)

(OR)

[P.T.O.]

21. 	What are the types of packings used in absorption towers? Explain in detail the construction and operation of packed bed absorption towers.

22. 	What is solubility curve and equilibrium in crystallization? Explain in detail.

(OR)

23. 	What is the purpose of using vacuum crystallizer? Explain in detail the construction and operation of Vacuum crystallizer.

24. 	What is the difference between micro, ultra and nano filtration? Describe with suitable example and mention its application.

(OR)

25. 	Write short notes on:

a. 	Reverse Osmosis. 															    (8)

b. 	Pervaporation. 																    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FOOD SAFETY REGULATIONS						      Time : 3 hours

Subject Code:	12FP223									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The Sanitary and phytosanitary measures are a part of ________ agreements.

2.	________ and ________ are the parent organizations of the Codex Alimentarius Commission. 3.	________ acts as the ex-officio Commissioner of Food Safety of the Republic of India.

4.	Counterfeiting a grade designation mark with respect to AGMARK products, shall be punishable with ________ or ________.

5.	________ number is given for food additives approved by the European Union.

6.	Listing ingredients in order by ________ is a standard practice consumers recognize.

7.	________ recognized HACCP as the most effective means of controlling food borne disease.

8.	HACCP was developed from the ________ system that was used in mechanic systems.

9.	The TDS of potable water is to be restricted to _________ ppm.

10.	________ is used for disinfestation of water.

PART – B  (5 x 3 = 15 MARKS)

11.	What does the WHO do when it meets annually?

12.	What is the role of the “Central Government” when a vacancy comes in the Food Authority? 13.	What are the essential requirements of labeling of Irradiated Foods?

14.	List out the seven principles of HACCP.

15.	Enumerate the advantages of Chlorination of water.

PART – C  (5 x 15 = 75 MARKS)

16.	Write an exhaustive note on Codex Alimentarius.

(OR)

17.	What are the mandates of the WTO? Write a short note on the functions of WTO.

18.	Briefly explain the functions of the Scientific Committee, Scientific Panel and Central Advisory Committee of the Food Authority of India.

(OR)

19.	Write a note on FPO and its relevance to the new food safety system.

20.	What does the consumer look for in a food labeling?

(OR)

21.	What would the manufacturer have to keep in the back of his mind while designing a label while marketing it in the EU sector?

22.	What are the prerequisite for running a HACCP analysis?

(OR)

23.	How can you use the decision tree method to differentiate CPs from the CCPs?

24.	What are the sources of water? What are the possible microbiological hazards of water?

(OR)

25.	What is bottled drinking water? What are the EU regulations describing the same?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	PROCESS EQUIPMENT DESIGN  						      Time : 3 hours

Subject Code:	12FP224									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define fatigue.

2.  	What is meant by yield stress?

3. 	IS code of unfired pressure vessel is __________.

4. 	Write the application of drains in floating roof tank.

5. 	Give the common MOC of tanks.

6. 	Name the types of flow available in Heat exchangers.

7. 	Name two types circulations available in evaporators.

8. 	Define “Boiling point elevation”.

9. 	State the advantage of freeze dryer.

10. 	State on limitation of fluidized bed dryer

PART – B  (5 x 3 = 15 MARKS)

11. 	Write a note on Hoop stress.

12.  	Mention the uses of Copper and Copper alloys in food processing plants.

13.  	Write a note on Hortonspheres.

14.  	What are components in standard evaporators?

15. 	What do you mean by puff drying?

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Explain in brief about various types of heads (covers) available for shells of pressure 	vessels.                   															    (7)

b.	A thin cylindrical pressure vessel of 1.2 m diameter generates steam at a pressure of       1.75 N/mm2. Find the minimum wall thickness, if:	i.	the longitudinal stress does not exceed 28 MPa	ii.	the circumferential stress does not exceed 42 MPa.			    (8)

(OR)

17.	a.	Find the thickness for a tube of internal diameter 100 mm subjected to an internal pressure 	which is 5/8 of the value of the maximum permissible circumferential stress. Also find the 	increase in internal diameter of such a tube when the internal pressure is 90 N/mm2. Take 	E = 205 kN/mm2 and μ = 0.29. Neglect longitudinal strain.  				                   (8)

b.  	A spherical vessel 3 m diameter is subjected to an internal pressure of 1.5 N/mm2. Find the thickness of the vessel required if the maximum stress is not to exceed 90 MPa. Take efficiency of the joint as 75%. 				    									    (7)

18.	a.   	Explain in brief about various losses associated with storage of volatile liquids.     	    (7)

b. 	Describe with a neat sketch the parts of a cylindrical tank.	    	                       		    (8)

(OR)

19.  	Design a roof tank with the following data:

Tank diamater  20 m approx., Tank height   12 m approx., Sp.  gr.  of liquid  1, Conical  roof  	slope-permissible  1  in  5 , Superimposed load-125 kg/cm2 , Material-carbon steel  (structural), 	Permissible stress- 1420 kg/cm2, Density- 7.7, Modulus of elasticity-2 x 10 6  kg/ cm2 .										   												   [P.T.O.]

20.  	a. 	Explain in brief about classification of Reaction vessels.			   				    (7)       b. 	Describe about importance of agitation in Reaction vessels.		    				    (8)

(OR)

21. 	Design the shell for a reaction vessel with the following data.

Vessel shell internal diameter						- 2130 mm

Jacket internal diameter							- 2260 mm

Jacket length										- 2500 mm

Diameter of half coil or width of channel jacket		- 100 mm

Internal Pressure (shell)							- 5.5 kg/cm2

Internal Pressure (jacket)							- 3.5 kg/cm2

Temperature 										- 150°C

Material 										- open hearth steel (IS-2002)

Allowable stress									- 9.8 kg/mm2

Modulus of elasticity (at 200°C) 					- 19.00 x103 kg/mm2.

Poisson’s Ratio									- 0.3

22.  	Explain in detail about types of Heat Exchangers.

(OR)

23. 	A liquid food (specific heat 4.0 kJ/kg °C) flows in the inner pipe of a double pipe heat exchanger. The liquid food enters the heat exchanger at 20°C and exits at 60°C. The flow rate of the liquid food is 0.5 kg/s. In the annular section, hot water at 90°C enters the heat exchanger and flows counter currently at a flow rate of 1 kg/s. The average specific heat of water is 4.18 kJ/(kg °C). Assume steady-state conditions.

a. 	Calculate the exit temperature of water.

b. 	Calculate log-mean temperature difference.

c. 	If the average overall heat transfer coefficient is 2000 W/(m2 °C) and the diameter of the inner pipe is 5 cm, calculate the length of the heat exchanger.

d. 	Repeat these calculations for parallel flow configuration.

24. 	Illustrate with a neat sketch the construction and working of following in food industries.

a.	Cabinet dryers.     															    (8)

b.	Tunnel dryers.	 															    (7)

(OR)

25.	A cabinet dryer is being used to dry a food product from 68% moisture content (wet basis) to 5.5% moisture content (wet basis). The drying air enters the system at 54°C and 10% RH and leaves at 30°C and 70% RH. The product temperature is 25°C throughout drying. Compute the quantity of air required for drying on the basis of 1 kg of product solids.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May – 2015

Subject Title :	PROCESS ECONOMICS AND PLANT LAYOUT DESIGN

Time : 3 hours

Subject Code:	12FP225				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In the acronym PIECES, C stands for __________.

2.	In stainless steel alloys, the term 18/8 stands for __________.

3.	PERT stands for __________.

4.	NPW stands for __________.

5.	A company manufacturing shrimps can be preferably constructed near __________.

6.	What is COD?

7.	Write the full form of CEPCI.

8.	What is cost index?

9.	The investment required for utilities is __________ of total product cost.

10.	For a plant operating at 50% of its capacity, the cost for maintenance and repair is __________ of fixed capital investment.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the basic requisites to be considered for using a material, under hygienic condition, for direct contact with a food?

12.	What are trickling filters?

13.	A steam kettle of 500 L capacity costed Rs. 50,000 in 1995. A wants to buy a kettle of 1000 L capacity now. How can that be done? Given – CEPCI index – 1995 – 381.1, and that at 2014 – 605

14.	Write briefly on the methods of estimating labour cost.

15.	What is ROI? What is its significance?

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss in detail on the various aspects of conducting a feasibility studies.

(OR)

17.	What is a process flow sheet? Draw a detailed process diagram for a company manufacturing  100 kg / h of Mango squash.

18.	What are the factors to be considered while selecting a site for a bakery industry? Discuss in detail on any four factors that you consider important.

(OR)

19.	Discuss in detail on the methods of treating solid and liquid wastes.

20.	Mr X  plans to set up a plant manufacturing Fruit powder. The cost of the evaporator (10 kg/h) his friend bought for his concern was Rs. 10 Lakhs in 2001. The cost of steam boiler was Rs. 3 Lakhs then for an appropriate capacity. Calculate the cost of setting up a plant now for a plant manufacturing 25kg/h, by the method of Miller. CEPC index for 2001 = 394.3 and for 2014= 685.2.

(OR)

[P.T.O.]

21.	Discuss briefly on the following:

a.	Module method of cost estimation.	 											  (10)

b.	Cost-capacity factor. 															    (5)

22.	What are the factors that contribute to the directly to the manufacturing cost of a product? Discuss briefly on the same.

(OR)

23.	What is depreciation? Discuss briefly on the methods of determining the same.

24.	Discuss briefly on the various methods of determining profitability of a venture.

(OR)

25.	a. 	Discuss briefly on the optimization techniques followed in Food Plants. 			  (10)

b. 	Hence calculate the optimal time required for a Company to start production. 		   (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FOOD ADDITIVES										      Time : 3 hours

Subject Code:	12FP226									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	LD50 means _________.

2.	The _________ defines an estimate of the amount of food additives that can be ingested over a lifetime without appreciable health risk.

3.	_________ is a major acidulant used in carbonated beverages.

4.	The most widely known natural antioxidants is _________.

5.	The insoluble protein portion of wheat flour is known as _________.

6.	DATEM is _________.

7.	Monosodium glutamate is the best example for _________.

8.	_________ replaces all the functions of a fat with essentially no energy contribution.

9.	Carotenoids serve as a precursor to provide _________.

10.	_________ is a sweetener made from ordinary sugar.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the flow chart for assessing the intakes of food components and food additives.

12.	Write short note on HLB concept.

13.	What are humectants? Give two examples.

14.	What are flavour enhancers? Give their classifications.

15.	Write note on chelating agents.

PART – C  (5 x 15 = 75 MARKS)

16.	What are food additives? Classify the food additives with its functions and its importance with an example.

(OR)

17.	Write short notes on:

a.	NOEL					b. 	ADI					c. 	GRAS

18.	a. 	Explain the types of emulsions with an example.									    (5)

b. 	Discuss briefly on the mechanism of stabilization by emulsifiers.					  (10)

(OR)

19.	Discuss briefly on preservatives and antioxidants.

20.	Explain in detail about dough conditioners and its functions.

(OR)

21.	Briefly discuss about different categories of flour improvers.

22.	What are food colourants? Give their classifications.

(OR)

23.	Explain in detail about fat substitutes and replacers.

24.	Briefly discuss about natural and artificial sweeteners.

(OR)

25.	What are nutritional additives? Explain them in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	PROCESSING OF FOOD COMMODITIES				      Time : 3 hours

Subject Code:	12FP242									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	_________ method of bread making is also known as the Chemical Method of Bread manufacture.

2.	_________ and _________  are the most commonly used packaging material for biscuits.

3.	Canning is also known as _________.

4.	The removal of excess moisture from fruits as a mode of preserving it is called _________.

5.	The true constituents of milk are _________, _________ and _________.

6.	HTST pasteurization of milk refers to a temperature of _________ and a time of _________.

7.	Emptying of the abdominal cavity of fish prior to its processing is called_________.

8.	_________ is the gas used for stunning of animal.

9.	_________ and _________ are the commercial species of coffee grown worldwide.

10.	_________ is the primary component that is derived from vanilla bean.

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by a bench process (or) proofing time?

12.	What is the role of the Blanching?

13.	Differentiate between toned milk, skimmed milk powder and double toned milk.

14.	What is rigor mortis?

15.	Enumerate the types of conventional and non-conventional teas marketed commercially.

PART – C  (5 x 15 = 75 MARKS)

16.	Write an exhaustive note on manufacturing of bread.

(OR)

17.	Briefly explain the manufacture of Pasta.

18.	Write a note on canning of fruits and vegetables.

(OR)

19.	Write a note on Hurdle Technology.

20.	What are the different types of cheese? Write a note on the processing and manufacture of cheese.

(OR)

21.	Write a note on the pasteurization of milk.

22.	Write a detailed note on poultry processing.

(OR)

23.	What are the stunning techniques used in the slaughter of animals?

24.	Write a note on the manufacture of Conventional Tea.

(OR)

25.	Elaborate on the post-harvest activities of cocoa.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	FOOD SAFETY REGULATIONS AND CONTROL   	       Time : 3 hours

Subject Code: 	12FP315										  Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Discuss on CCP Monitoring Records for HACCP with example.    				      (10)

b.   	Elaborate on the HACCP system followed in milk processing industry.  		      (10)

2.   	a. 	Discuss the functions of WTO. 											      (10)

b. 	Explain the operation procedure for Codex Alimentarius. 						      (10)

(OR)

3.   	a. 	Discuss the responsibilities of WHO in eradication of swine flu. 				      (10)

b. 	Give a brief note on Six Sigma. 											      (10)

4.	Explain the principles of FSSAI and describe its functionaries in implementation of the Food Safety and Standards Act, 2006.

(OR)

5.	a. 	Discuss the role of Food Safety Officer in executing the Food Safety and Standard Act. 																			      (10)

b. 	State the regulations of PFA in relation to Food processing Sector.				      (10)

6.	a. 	Explain the guidelines followed worldwide in developing labeling standards. 	      (15)

b. 	State the regulations to be adhered in labeling of genetically modified foods.		(5)

(OR)

7.	Elaborate on the EU and US regulations in Nutrition labelling.

8. 	Discuss on the microbiological safety of drinking water and give the regulations for         	Bottled water.

(OR)

9.    Explain the guidelines of HACCP and ISO 22000 for animal products.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title: 	FOOD PLANT LAYOUT AND DESIGN					Time : 3 hours

Subject Code: 	13FP301								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Mr X wants to start a company manufacturing Instant premixes. You are deputed by the company to conduct a feasibility study. Can you do the same and give your suggestions?

2.	Mr Y. wants to set up an aseptic processing unit. Based on the concepts of plant design, discuss in detail on the steps involved in designing the same, with proper justification

(OR)

3.	Discuss in detail on the possible hazards in Food Industry and ways to prevent the same.

4.	Discuss in detail on principles of Hygienic design of a Food Plant manufacturing fruit products.

(OR)

5.	Discuss on the following:

a.	Types of soil and methods of cleaning the same. 								      (12)

b.	Water treatment methods for carbonated beverage manufacture. 					(8)

6.	a.	Discuss in detail on the Richard Muther’s Simplified Systematic Plant Layout. 	      (12)

b.	Hence construct a layout for a large scale Jam processing plant and justify the same. 	(8)

(OR)

7.	It is proposed to set up a dairy powder manufacturing unit. Calculate the total investment required for the same. Given – Cost of evaporator of 500kg/h capacity = Rs. 7 Lakhs in 2005. Cost of steam boiler of similar capacity in 2005 – Rs. 3 Lakhs.  CEPC index for 2005 = 381.1 and for 2014 = 645. Cost-capacity factor =0.514

8.	Discuss in detail on the following:

a.	Factors contributing to the cost of a product. 								      (15)

b.	Depreciation.																(5)

(OR)

9.	Discuss in detail on the methods of determining profitability of a proposed venture.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	FOOD CHEMISTRY									      Time : 3 hours

Subject Code:	FP230										       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Differentiate between bound and free water.

2.	What is sorption phenomena and sorption isotherms?

3.	Give two examples for reducing sugar.

4.	Draw the structure of maltose.

5.	Give two examples for compound lipids.

6.	What is Hydrogenation?

7.	What do you mean by denaturation of proteins?

8.	What is meant by enzyme specificity?

9.	List two water soluble vitamins.

10.	List one function of vitamin E.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the structure of glucose and give its reaction with bromine water.

12.	What is modified cellulose?

13.	Define the term rancidity. What are the different types of rancidity?

14.	What are essential amino acids? Give three examples.

15.	What are choline and carnitine?

PART – C  (5 x 15 = 75 MARKS)

16.	What is the importance of water in food? Diagrammatically explain the structure of water and ice.

(OR)

17.	What are the physical and chemical properties of carbohydrates?

18.	Draw the structure of pectin. Explain its properties with reference to gel preparation.

(OR)

19.	Write a note on the preparation and properties of starch.

20.	Give a process flow chart for the manufacture of edible oils. Explain oil refining.

(OR)

21.	Classify lipids with a suitable example.

22.	Discuss proteins under the following headings:

a.	Classification.		b.	Denaturation.		c.	Peptides.

(OR)

23.	Classify enzymes. Enumerate its properties.

24.	What is meant by bioavailability? Explain vitamin toxicity and stability.

(OR)

25.	Enumerate the functions of vitamins A and C.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/ May - 2015

Subject Title :	PLANTATION PRODUCTS AND SPICES TECHNOLOGY

Time : 3 hours

Subject Code:	FP232											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Why roasting is done in coffee?

2. 	What is instant coffee?

3. 	Define CTC tea.

4. 	What is green tea?

5. 	Explain conching.

6. 	Give the chemistry of chocolate.

7. 	Explain the term piperine.

8. 	What is the principle component in cardamom?

9. 	Which method gives higher oil recover in spices?

10. 	Define oleoresin.

PART – B  (5 x 3 = 15 MARKS)

11. 	Explain changes that take place during fermentation of coffee bean.

12. 	What is white tea?

13. 	Explain fermentation of cocoa bean.

14. 	What are uses of dried ginger?

15. 	How vanillin is extracted from vanilla?

PART – C  (5 x 15 = 75 MARKS)

16.  	Discuss dry processing of coffee.

(OR)

17. 	Explain in detail about production of instant coffee powder.

18. 	Discuss briefly about different types of tea and their manufacturing process.

(OR)

19. 	Explain the unit operations involved in tea manufacture.

20. 	Discuss with flow chart about chocolate production technology.

(OR)

21. 	Discuss in detail production of cocoa powder.

22. 	Explain the process of production of black pepper and white pepper.

(OR)

23. 	Explain unit operation involved in turmeric processing.

24. 	Explain cinnamon processing and oil extraction method.

(OR)

25. 	Explain with help of flow chart about vanilla processing.