Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	INTRODUCTION TO AEROSPACE ENGINEERING     	     Time : 3 hours

Subject Code:	09AE201						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The aircrafts powered by human arm, leg or body movement is known as __________.

2.	A liquid that contains both oxidizing and fuel species is known as __________.

3.	Write down the equation to find out drag.

4.	The horizontal force opposing the forward motion is known as __________.

5.	Name the ratio between surface area for burning grain and the throat area of exhaust nozzle?

6.	The main purpose of turbine is to __________.

7.	The decrease in stress experienced over a period of time by a material having constant strain is known as __________.

8.	If the deformations completely disappear on the removal of load, the material is said to be __________.

9.	NASA’s interstellar mission is named as __________.

10.	The satellites appear above the same location on the earth at all times are called __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Write shorts notes on duct jet propulsion and its clarifications.

12.	Write short notes on aerofoil nomenclature.

13.	Write short notes on the clarification of jet propulsion systems.

14.	Enumerate the group of materials that is used in the construction of aircraft.

15.	What are recoverable launch vehicles? Give an example.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the applications of rocket propulsion.

(OR)

17.	Explain briefly about the contribution of Wright brothers for the aviation industry.

18.	Name and explain the basic components and control surfaces of an aircraft.

(OR)

19.	Compare the drag generation for a blunt body and a streamlined body with sketch.

20.	Write short notes on

a.	Turboprop Engine.

b.	Turbofan Engine.

c.	Turbojet Engine.

(OR)

21.	Explain the equations of motions to study the performance of aircraft with neat sketch.

22.	Explain in detail about the properties of materials used in aircraft construction.

(OR)

23.	What are sandwich structures and explain its failure modes in detail.

24.	Explain the clarifications of rockets on the basis of energy source employed.

(OR)

25.	Explain in detail about the clarification of satellites and its applications.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	AERODYNAMICS  									      	     Time : 3 hours

Subject Code:	09AE205					 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Sketch the various forces acting in an aircraft.

2.	What is called free stream dynamic pressure?

3.	Incompressible flow is indicated by constant.

a.	Pressure			b.	Temperature			c.	Density

4.	Incompressible flow in a pipe is an example of __________ flow.

5.	Define induced drag.

6.	Write the condition for steady level flight.

7.	Endurance or Range - which of these is dependent on the altitude?

8.	What is wing loading?

9.	Instruments used for measurements of flow velocity are __________ and __________ Doppler anemometer.

10.	For measurement of force on a model in a wind tunnel, one normally uses __________ balance.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Dynamic similarity.

12.	Write short notes on doublet.

13.	Explain the method for estimating the drag of a complex body having many components like aircraft.

14.	Write short notes on climbing flight.

15.	What is the effect of thickness and camber in an aerofoil?

PART – C  (5 x 15 = 75 MARKS)

16.	Define Drag. How can the drag of a vehicle be estimated from the known pressure and Shear stress distribution. Derive the expression for drag and drag coefficient of a 2D body at angle of attack 0o as function of pressure and shear stress distribution.

(OR)

17.	Using Pi-Buckingham theorem, derive the non-dimensional parameters which determine the force on a body in viscous flow.

18.	Briefly explain about the Kutta-Joukowski theorem and the conditions of lift.

(OR)

19.	Write short notes on the following:													(8+7)

a.	Circulation   			b.	Velocity potential

20.	Describe the various drag reduction methods adopted by aircraft in detail.

(OR)

21.	Write notes on performance of a turbo jet engine.

22.	Derive the equation for range and endurance of a Jet powered aircraft.

(OR)

23.	Write notes on the following: 														(7+8)

a.	Assisted take-off 				b.	High lift flaps.

24.	Explain in detail the principle of hot-wire anemometry.

(OR)

25.	What are the different steps involved in wind tunnel testing? Explain each step briefly.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	GAS DYNAMICS										      Time : 3 hours

Subject Code:	09AE207		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the steady flow energy equation for an adiabatic flow of air.

2.	Define compressibility of the fluid.

3.	The Mach number behind the normal shock is always __________ .

4.	After the flow becomes choked, the __________ remains constant.

5.	The Mach angle is simply determined by __________ .

6.	Mention any two important geometrical features of supersonic profiles.

7.	The critical Mach number can be increased by__________ .

8.	Define drag divergence Mach number.

9.	What are the disadvantages of blow down tunnels?

10.	What is a gun tunnel?

PART – B  (5 x 3 = 15 MARKS)

11.	Calculate the isothermal compressibility for air at a pressure of 0.5 atm.

12.	Consider a Mach 2.8 supersonic flow over a compression corner with a deflection angle of 15°. If the deflection angle is doubled to 30°, what is the increase in shock strength? Is it also doubled?

13.	Differentiate between Mach waves and Mach angles

14.	What is a supercritical airfoil?

15.	What are the problems in the operation of supersonic wind tunnels?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the general relation for speed of sound in a gas (in terms of ratio of specific heats and local temperature in air medium).

(OR)

17.	a.	Derive the integral form of momentum equation for inviscid flows using control volume 	approach											  						  (10)

b.	Consider a turbojet-powered airplane flying at a velocity of 300 m/s at an altitude of 10km, where the free-stream pressure and density are 2.65 x 104 N/m2 and 0.414 kg/m3, respectively. The turbojet engine has inlet and exit areas of 2m2 and 1m2, respectively. The velocity and pressure of the exhaust gas are 500 m/s and 2.3 x 104 N/m2 respectively. The fuel-to-air mass ratio is 0.05. Calculate the thrust of the engine.					    (5)

18.	a.	Derive the expression for Rankine-Hugonoit relation and explain its significance		  (10)

b.	A uniform supersonic stream with M1 = 3, p1 = 1 atm, and T1 = 288 K encounters a 	compression corner which deflects the stream by an angle θ = 20°. Calculate the shock 	wave angle, and P2, T2, M2, P02 and T02 behind the shock wave. 					    (5)

[P.T.O.]

(OR)

19.	a.	Explain the basic principle behind method of characteristics. Briefly explain the design of 	2D supersonic nozzle contours with method of characteristics				  		  (10)

b.	Air at M=2.8 passes through a 16º compression corner. What should be the angle of expansion corner required to bring back the flow to M=2.8?  		 				    (5)

20.	Briefly explain the small perturbation theory of air flows. Derive an expression for linearized perturbation velocity potential equation

(OR)

21.	Explain the linearized two dimensional supersonic flow theory with suitable examples

22	a.	Explain the role of area rule in the design of high speed aircraft				     	    (7)

b.	Explain the Airfoils in the design of high speed aircraft with examples			    	    (8)

(OR)

23.	Write short notes on the following:

a.	Shock induced separation 								    						    (5)

b.	Characteristics of swept wings							    						    (5)

c.	Lower and upper critical Mach numbers						    					    (5)

24.	a.	Explain the design features and peculiarities of Transonic and Hypersonic tunnels	    (8)

b.	Write a brief note on shock tubes 	  						                			    (7)

(OR)

25.	Discuss the flow visualization techniques and Explain in detail the optical Schlieren flow visualization system with neat sketch.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	SPACE DYNAMICS								      	     Time : 3 hours

Subject Code:	09AE208				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are two common ways of separating two serial rocket stages?

2.	Define: Specific impulse and thrust of a rocket.

3.	Define static margin.

4.	In Heliocentric coordinate system, the origin is___________.

5.	What is Sidereal time?

6.	What is Diurnal motion?

7.	What are some different kinds of orbits?

8.	What is the orbital plane?

9.	What defines the shape of a trajectory?

10.	What is ICBM?

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly explain the fundamental reasons for staging in rockets.

12.	Briefly discuss on Lateral stability of rockets.

13.	Write short notes on: Motion of the vernal equinox.

14.	An earth satellite is in an elliptical orbit of eccentricity 0.3 and angular momentum 60,000 km2/s. Find the delta-v required for a 90° change in inclination at apogee (no change in speed). (Earth’s gravitational parameter, µ = 398600.4418 km³/s²).

15.	Calculate the radius of the earth’s sphere of influence. Mass of Earth = 5.974 × 1024 kg, Mass of Sun = 1.989 × 1030 kg, and mean orbital radius of Earth is 149.6 × 106 km.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Differentiate between a sub rocket and a stage.						    			    (5)

b.	A single stage rocket is launched vertically. The lift off mass, specific impulse, burn time and propellant mass flow rate are 1000 kg, 280 sec, 10 sec and 50 kg/s respectively. Calculate the ideal velocity at burn out. Also calculate the gravity corrected velocity, peak altitude and time to reach peak altitude considering constant gravity of 9.8 m/s2 and ignoring aerodynamic forces. 												  	  (10)

(OR)

17.	a.	Describe any two stage separation techniques for a multistage launch vehicle in 	atmosphere with neat illustration.										 	 	  (10)

b.	A low-cost four stage rocket is to be used to launch small payload to orbit. The concept proposed for the system utilizes propellants that are safe and cheap but provide a specific impulse of only 200 seconds. All four stages are identical. What structural efficiency is required to reach orbit with a finite payload?						    	    			    (5)

18 	Differentiate between static stability and dynamic stability. Explain in detail the Static Longitudinal stability and control of rockets.

(OR)

19.	Write short notes on the following:

a.	Directional stability and control. 							    					    (5)

b.	Stability and Equilibrium.								    						    (5)

c.	Dynamic stability	.								    							    (5)

20.	a.	Describe the Reference frames and Coordinate Systems used in space applications.	  (10)

b.	Briefly explain about the celestial sphere.							       			    (5)

(OR)

21.	a.	Discuss the effects of space environment on the selection of materials for spacecraft 	systems. 											    						    (8)

b.	Write short notes on: The Earth’s atmosphere. 						    			    (7)

22.	a.	Explain the most important perturbing forces acting on artificial Earth satellites	    	    (7)

b.	At the end of a rocket launch of a space vehicle, the burnout velocity is 9 km/s in a direction due north and 3 o above the local horizontal. The altitude above sea level is 805 km. The burn out point is located at the 27th parallel (27o) above the equator. Calculate and plot the trajectory of the space vehicle.			    					    			    (8)

(OR)

23.	An earth’s spacecraft is in a 300 km circular parking orbit. It is desired to increase the altitude to 600 km and change the inclination by 20°. Find the total delta-v required if

a.	The plane change is made after insertion into the 600 km orbit (so that there are a total of 	three delta-v burns).

b.	The plane change and insertion into the 600 km orbit are accomplished simultaneously (so 	that the total number of delta-v burns is two).

(Earth’s gravitational parameter, µ = 398600.4418 km³/s², Radius of the Earth = 6378 km)

24.	a.	Discuss in detail the Two-dimensional Interplanetary Trajectories.			    		    (8)

b.	Briefly describes the Launch of Interplanetary Spacecraft.			                		    (7)

(OR)

25.	After a Hohmann transfer from earth, calculate the minimum delta-v required to place a spacecraft in Mars orbit with a period of seven hours. Also calculate the periapse radius, and the angle between periapse and Mars’ velocity vector.

(Mean distance from Earth to the Sun = 149.6×106 km, Mean distance from Mars to the         Sun = 227.9×106 km, Sun’s gravitational parameter = 1.327×1011 km3/s2, Mars’ gravitational parameter = 42828.3 km3/s2, Radius of Mars = 3397 km)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	AIRCRAFT PROPULSION							      	     Time : 3 hours

Subject Code:	09AE209					 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What do you mean by Lapse rate?

2.	Under which principle Aircraft gas turbine engines are working?

3.	List down the components on a single stage of Axial Compressor.

4.	What is the range of value for power input factor?

5.	What is the fuel used in the Aircraft Engine?

6.	State any one purpose of Nozzle Guide Vanes in the case of Axial Flow Turbine.

7.	How will you stabilize the flame in the Afterburners?

8.	In a Convergent and Divergent Nozzle, whether the throat area is maximum or minimum?

9.	Write down in the increasing order of propulsive efficiency for Turbojet, Turbofan and Turboprop engines.

10.	What do you mean by specific thrust?

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate Isentropic process from Non-isentropic process.

12.	What is the necessity of pre-whirl in the centrifugal compressor?

13.	Classify the combustor used in the Aircraft Gas turbine Engine. Draw simple schematic sketches of them.

14.	What do you mean by atomization? What is the necessity of that?

15.	Draw T-S diagram of the turbojet engine.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What do you mean by “Standard Atmosphere”? Also the draw the neat schematic diagram 	of International Atmosphere Variation.											    (5)

b.	Illustrate the different layers present in the International Standard Atmosphere” with their properties.																	  (10)

(OR)

17.	a.	Explain the working principle of Jet Propulsion with a schematic sketch. 			    (8)

b.	What is the operating thermodynamic cycle of Jet Propulsion? Draw T-S and P-v diagram 	and mark all process in the diagram.											    (7)

18.	a.	Draw the sketch of typical single sided centrifugal compressor. Explain its working 	principles.																	    (8)

b.	Derive the relation for pressure rise across the centrifugal compressor.				    (7)

(OR)

19.	a.	Define “Degree of reaction for an Axial Compressor.								    (3)

b.	Obtain the relation for Degree of Reaction.										  (12)

20.	a.	Draw a schematic sketch of the combustor and mark all important zones of combustor. (5)

b.	Illustrate in detail the process that is happening in the combustor.					  (10)

(OR)

[P.T.O.]

21.	a.	Differentiate impulse turbine from the reaction turbine.							    (5)

b.	Explain the method of choosing blade profile, pitch and chord for an axial turbine.	  (10)

22.	a.	Draw the schematic diagram of an afterburner and mark all components in that.		    (6)

b.	Explain the fuel injection and flame stabilization methods employed in the case of 	afterburner.																	    (9)

(OR)

23.	a.	What is the purpose of nozzle and intakes in the aircraft gas turbine?				    (4)

b.	Explain the starting problem of the supersonic intake.								  (11)

24.	a.	Draw a schematic sketch of the turbofan engine and mark all salient components of that.     (6)

b.	Explain the working principle of turbofan engine with T-s diagram.					    (9)

(OR)

25.	Illustrate the procedures involved in matching of the compressor and turbine of the gas turbine engine. Draw the flow chart of the matching process.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ADVANCED PROPULSION SYSTEMS			      	            Time : 3 hours

Subject Code:	09AE211						 			              Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In aircraft gas turbines, the axial flow compressor is preferred because _________.

2.	A ideal turbojet works on _________ cycle.

3.	In the critical operation of supersonic inlets the normal shock position is _________.

4.	Ramjet is a _________ combustion.

5.	The operational range of Mach Number for a Ramjet engine is between

2 and 5

0.3 and 0.8

0.1 and 0.3

1.2 and 2.0

6.	Which engines requires the longest runway?

7.	A lean mixture will have air fuel ratio of _________.

8.	Only rocket engines can be propelled to ‘space’ because _________.

9.	Propulsive efficiency of a rocket engine is given by _________.

10.	DLPS Refers _________.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the methods of  blade cooling?

12.	What are the burners used in ramjet engine?

13.	How the scramjet engine differs from ramjet engine?

14.	Define ionization potential.

15.	How the nuclear rockets are classified? Give the specific impulse range for nuclear rockets.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the working of a centrifugal compressor and draw the velocity triangles.

(OR)

17. 	A centrifugal compressor has an impeller tip speed of 366 m/s. Determine the absolute Mach number of the flow leaving the radial vanes of the impeller when the radial component of velocity at impeller exit is 30.5 m/s and the slip factor is 0.9. Given that the flow area at impeller exit is 0.1 m2 and the total-to-total efficiency of the impeller is 90%. Determine mass flow rate.

18. 	Explain in detail about Subcritical, Critical and Supercritical charging.

(OR)

19. 	A ramjet is traveling at Mach 3 at an altitude of 4572 m, the external  static temperature is 258.4K, and the external static pressure is 57.1 kPa. The heating value of the fuel is             46,520 kJ/kg. Air flows through the engine at 45.35 kg/s. The burner exit total temperature is 1944K.Find the thrust, fuel ratio, and TSFC. The specific heat ratio can be assumed to be 1.4.

20.  Classify the development involved in nozzles.

(OR)

21.  	Explain the operating principles of  a rocket.

22. 	Explain briefly about thrust vector control in liquid rockets.

(OR)

23. 	Explain the important components of solid rockets.

24. 	Explain in detail about electro static and electro dynamic rockets with neat sketch.

(OR)

25. 	Explain the working of nuclear thermal rocket with neat sketch.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	AIRCRAFT STABILITY AND CONTROL				      Time : 3 hours

Subject Code:	09AE215				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the purpose of aileron in the aircraft?

2.	For statically stable, dCm/dCL is __________ (negative or positive).

3.	Write the formulae for finding the “Static Margin”

4.	State one difference between stick fixed and stick free.

5.	What is Proverse Yaw?

6.	Define Snaking.

7.	Which control surface is very useful when one of the engines is failed in the twin engine aircraft?

8.	What do you mean damping?

9.	Define aileron reversal.

10.	What are the control surfaces are used to get out of the spiral instability?

PART – B  (5 x 3 = 15 MARKS)

11.	What do you mean by degree of freedom?

12.	Define neutral point

13.	What is meant by `Weather Cock Stability'?

14.	Define Auto-rotation.

15.	What is meant by phugoid oscillation?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the basic concepts of static and dynamic stability with example.

(OR)

17.	Derive the equation for the contribution of wing for the  aircraft pitching moment.

18.	Explain the power effects in jet and propeller engine.

(OR)

19.	What is aerodynamic balancing? Explain any three in detail.

20.	Illustrate the following in details.

a. 	One engine In-operative Conditions.

b. 	Rudder lock.

c. 	Adverse Yaw & Aileron Reversal.

(OR)

21.	a.	Write the equations of stability derivatives of lateral stability.

b. 	Dihedral effect.

c. 	Aileron Power.

22.	Explain the significance of Routh's discriminant.

(OR)

23.	Derive the equation of motion for Dynamic Longitudinal Stability.

[P.T.O.]

24.	Write short notes on

a. 	Stability derivatives in longitudinal dynamic stability.

b. 	Stability quartic .

c. 	Spoils in lateral control.

(OR)

25.	Explain the following Phenomenon.

a. 	Dutch Roll.

b. 	Spiral instability.

c. 	Spin.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	BASICS OF AEROSPACE ENGINEERING			      	     Time : 3 hours

Subject Code:	10AE202						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is monoplane?

2.	What are the types of engines used in aircraft?

3.	List down different parts of airplane.

4.	What are the basic instruments necessary for flying the aircraft?

5.	What are different segments of the atmosphere?

6.	Define Mach number.

7.	What are the non-metallic materials used in aircraft construction?

8.	What are various forms of wing configurations considered?

9.	Indicate difference between propeller engine and turboprop engine.

10.	List down the different types of rockets.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the loads acting on the various part of airplanes?

12.	Explain how airplanes are classified.

13.	Explain how the temperature and pressure vary with altitude.

14.	Explain the difference between monocoque and semi-monocoque structures.

15.	Explain the basic difference of operation between jet engine and rockets.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain how lift is produced to keep the weight of the aircraft heavier than air.		    (8)

b.	What are the different topics considered in aerodynamics and explain the major

difference between them?														    (7)

(OR)

17.	a.	Explain the role of each part of the aircraft structure.								    (8)

b.	Explain the difference between different types of propulsion system used in airplanes.   (7)

18.	a.	Explain briefly the use of spars, stringers and bulkheads used in aircraft structures.	    (8)

b.	Explain with neat sketches explain the different types of landing gear used in airplanes.  (7)

(OR)

19.	a.	Explain briefly the use of any two instruments in airplanes.						    (7)

b.	Explain the working any one powered control or conventional control.				    (8)

20.	a.	Define drag. List and explain various types of drag developed during flight.			    (8)

b.	Explain what each digit indicates in the 4 and 5 digit airfoil specification.			    (7)

(OR)

21.	a.	What are different maneuvers performed by various types of aircraft?				    (8)

b.	Explain how rolling moment, yawing moment and pitching moment are produced.	    (7)

22.	a.	Draw a neat sketch of wing and indicate various parts of the wing structures.		    (8)

b.	Draw a neat sketch of fuselage and indicate various parts of the same.				    (7)

(OR)

[P.T.O.]

23. 	a.	What are the materials used for wing and fuselage structures. Also list the materials used in 	engines and state the salient features of the materials.								    (8)

b.	Define composite materials. List the advantages of composite materials over the 	conventional materials.														    (7)

24.	List various types of piston engines used in airplanes and briefly describe the features of any two of them.

(OR)

25.	What are the various types of gas turbine engines? Briefly explain the working of any two

of them.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	INTRODUCTION TO AEROSPACE ENGINEERING      	     Time : 3 hours

Subject Code:	11AE201						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Who is the father of Aviation?

2.	State Newton’s third law.

3.	Name the aerodynamic forces acting in an aircraft.

4.	Define ISA.

5.	Draw a P-V, T-S diagram for Brayton cycle.

6.	Write the different types of jet engines.

7.	What are the advantages of V-n diagram?

8.	List out any four composite materials.

9.	What is trajectory?

10.	Name any four spacecraft configurations.

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by the degree of freedom and how much required for airplane?

12.	Distinguish between stratosphere and troposphere.

13.	List out the essential features of jet propulsion.

14.	Draw the semi-monocoque structure of a wing.

15.	Write the different types of rocket engines.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail about the classification of duct jet propulsion.

(OR)

17.	Discuss about the advantages and disadvantages of monoplanes compared to biplanes.

18.	Describe the major components of an airplane with a neat sketch and explain their functions.

(OR)

19.	Enumerate the six factors of an ideal airfoil and give the significance of L/D ratio.

20.	What is the principle behind aircraft propulsion and explain it with turbojet engine.

(OR)

21.	Explain how a propeller produces thrust for aircraft propulsion and what are the merits and limitations of turbofan engines.

22.	Discuss briefly the choice of material for aircraft engine components.

(OR)

23.	With neat sketch, explain the construction of a typical aircraft wing. Show all the structural components of wing construction and mention the types of loads/stresses experienced by the structural members.

24.	Write in detail about the subsystems used in various spacecraft configurations with their functions.

(OR)

25.	a.	What are the components in rocket engines and explain their functions? 			    (8)

b.	Write about the different types of satellites according to their applications.			    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	AIRCRAFT STRUCTURES								      Time : 3 hours

Subject Code:	11AE202				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A truss in which the number of members is more than (2j-3) is known as ___________.

2.	A beam of length ’l’ fixed at both the ends carries a uniformly distributed load ‘w’ per unit length throughout the span. The bending moment at the ends is ___________.

3.	Strain energy (U) caused by bending is given by relation ___________.

4.	The maximum strain energy which a body stores upto elastic limit per unit volume of piece is called as ___________.

5.	The ratio between buckling load and safe load is known as ___________.

6.	___________ theory is suitable for ductile materials.

7.	In an ‘I’ section, symmetrical about XX and YY axes, shear centre lies at ___________.

8.	The locus of the shear centre at various section is known as ___________.

9.	The structural unit which transfers the concentrated loads to the shell of an airplane wing is ___________.

10.	The tension field beams are used in aircraft as ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Carry over moment and Carry over factor.

12.	Write down the formula to calculate the strain energy, if the torsion ‘T’ is applied.

13.	Give Rankine’ s formula and its advantages.

14.	Indicate the shear center for channel section and angle section.

15.	Define Wagner beam.

PART – C  (5 x 15 = 75 MARKS)

16. 	Determine the axial forces in all the members of the pin-joined truss loaded as shown in Figure.

[P.T.O.]

(OR)

17. 	State and prove Clapeyron’s 3-moment equation.

18. 	Calculate the vertical deflection of the point B and the horizontal movement of D in the pin-joined frame work shown in figure. All members of the frame work are linearly elastic and have cross sectional areas of 1800mm2, E = 200 Gpa.

(OR)

19. 	Find the deflection at the mid-point of a simply supported beam of length L, subjected to a uniform load ‘q’ using unit load method.

20. 	Derive an expression for the Euler’s crippling load for a long column with following end conditions.

a.	Both ends are fixed.															(8)

b.	One end fixed other is hinged.													(7)

(OR)

21. 	Explain the following theories of failure.

a.	Maximum shear stress theory.													(5)

b.	Maximum strain energy theory.												(5)

c.	Maximum shear strain energy theory.											(5)

22. 	Find the shear flow distribution and locate the shear center for the section shown in figure. Each of the stringers has an area of 4 cm2 and the section is subjected to vertical shear load of 50 kN at the shear center.

(OR)

23. 	Determine the shear flow and the angle of twist per unit length of the two cell tube as shown in the figure.  The section is subjected to a torque 100 kNm. Take G = 40 GPa.

24.	a. 	Explain Wagner beam.														 (10)

b. 	Explain Shear resistant web beam.												  (5)

(OR)

25. 	What are the structural parts of an aircraft Fuselage? List their functions.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	FLIGHT MECHANICS AND PERFORMANCE			      Time : 3 hours

Subject Code:	11AE204				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	With respect to increasing altitude, at a constant IAS the Mach number increases. True/False

2.	What is aspect ratio?

3.	The practical upper limit of an aircraft is called as _________.

4.	What is load factor?

5.	Degrees of freedom of an aircrafts is_________.

6.	What is the position of CG for longitudinal static stability?

7.	Directional stability is also called as _________.

8.	What is anhedral wing?

9.	Directional divergence will occur due to insufficient _________ stability.

10.	Combination of "tail-wagging" and rocking from side to side is called _________.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the power required and power available curve for a jet engine and piston engine.

12.	What is gliding flight? State the condition for flattest gliding.

13.	Why is relaxed static stability permitted in some airplanes?

14.	Distinguish between yaw and sideslip angle.

15.	What is snaking mode? What factors govern it?

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss in detail various types of drag in an airplane and methods of minimizing the drag.

(OR)

17.	a. 	Calculate the pressure, density and temperature at 7 km in the ISA.					(5)

b. 	Derive an expression for drag polar & explain it for various Mach number.			(10)

18. 	Derive:   Vmin.power= 0.76 Vmin.drag

(OR)

19.	a.	Derive the Breguet Endurance formula and discuss its implications.          			(10)

b. 	Explain –Climb hodograph.		 											(5)

20.	Write short notes on: a.	Aerodynamic balancing.	b.	Static and dynamic stability. (8+7)											    (OR)

21.	a.	Explain the power effects of jet engine on longitudinal stability.	    	    			(7)

b.	Discuss the neutral point and static margin estimation for stick fixed stability. 		(8)

22.	Quantitatively explain the contribution of different components of aircraft towards static lateral stability.

(OR)

23.	a.	Explain the requirements of rudder in aircraft. 									(8)

b.	Discuss with suitable example the coupling between rolling and yawing moment. 	(7)

24.	a.  	An airplane of W/S=4000 N/m2 is flying at sea level at V=400Kmph. The characteristic 	equation of one of its mode is given by λ2+ 0.5 λ+3 =0. Estimate the period of oscillation 	and the time to half amplitude.													(8)

b.	Derive the stability derivatives for lateral dynamics.  								(7)

(OR)

25.	Explain the following:  a.	Spin and Spin recovery.    b.	Autorotation.	 				(8+7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	AERODYNAMICS										      Time : 3 hours

Subject Code:	11AE205									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In a Barotropic flow, density is a function of __________.

2.	Write down the Bernoulli equation for incompressible flow.

3.	‘Magnus effect is not employed for powered flight’ – True/False

4.	In an Inviscid, incompressible flow, circulation around an aerofoil is equal and opposite to the circulation around the __________.

5.	For a symmetric aerofoil, the aerodynamic center is located at __________.

6.	The center of pressure for a cambered aerofoil varies with __________.

7.	Define critical Reynolds number.

8.	At ‘No slip’ region, the velocity is maximum – True/False

9.	What is a Bound vortex?

10.	Elliptic lift distribution yields minimum induced drag – True/False.

PART – B  (5 x 3 = 15 MARKS)

11.	What is a streamline? Draw streamlines and Potential lines for a Source flow.

12.	With neat sketch explain Kutta condition.

13.	Using the transformation, ξ=z+b2z, transform a circle into a straight line.

14.	What is meant by Displacement thickness? Give its equation.

15.	Define Biot-Savart law.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive Energy equation for a steady, inviscid and adiabatic fluid flow without any body force.

(OR)

17.	What is a Doublet? Derive equation of stream function and potential function for a doublet with neat sketch.

18.	State and prove Kutta-Joukowski theorem.

(OR)

19.	Consider the lifting flow over a circular cylinder. The lift coefficient is 5. Calculate the location of the stagnation points on the cylinder when the pressure equals free stream static pressure.

20.	Transform a circle to a cambered aerofoil.

(OR)

21. 	Consider an NACA 23012 aerofoil. The mean camber line for this aerofoil is given by  for

and  for . Calculate,

a.	The angle of attack at zero lift,

b.	The lift coefficient when

c.	The moment coefficient about the quarter chord and

[P.T.O.]

The location of the center of pressure in terms of xcpc  when α=4c.

22.	Find the Displacement thickness, Momentum thickness and Energy thickness for the velocity distribution in the boundary layer given by uue=2y8−y82.

(OR)

23. 	Derive equation for the boundary layer growth over a flat plate.

24.  	Obtain an equation for the induced velocity at a point in the field of an elementary length δs of a vortex tube of strength K, which subtends an angle δθ.

At a point P located by the co-ordinates R,θ from the element.

(OR)

25. 	What is a lifting line? Derive the fundamental equation for Prandtl’s lifting line theory and extend its application to the elliptic lift distribution.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	THERMODYNAMICS									      Time : 3 hours

Subject Code:	12AE202		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the processes involved in cannot cycle?

2.	Define Specific heat.

3.	What are the causes of irreversibility?

4.	Define Entropy.

5.	Plot the diesel cycle on p-V and T-s diagram.

6.	Specific heat at constant volume (CV) of __________ gas is a function of temperature alone.

7.	State Tds Equations.

8.	Internal energy of an __________gas is a function of temperature alone.

9.	Give the importance of Maxwell relations.

10.	Why does the ice float on water?

PART – B  (5 x 3 = 15 MARKS)

11.	State Zeroth law of thermodynamics.

12.	State the Kelvin-Plank statement of second law of thermodynamics.

13.	Difference between heat pump and heat engine.

14.	Draw the PVT surface. Give its significance.

15.	What is Joule Thomsons coefficient?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the concept of thermodynamic equilibrium and quasi-static equilibrium. 		    (7)

b.	One Kg of gas is expanded at constant pressure from 0.085 m3 to 0.13m3. If the initial 	temperature of gas is 225C, Find the final temperature, net heat transfer; change in 	internal energy and pressure of the gas.											    (8)

` (OR)

17.	Calculate the change in internal energy, heat transferred and change in enthalpy for 0.5 kg of air expanding according to law pV1.2 = C from 1 MPa and 300C  to 100 kPa. What will be the work done by air during the expansion?

18.	A reversible heat engine operates between two reservoirs at temperatures 700C and 50C. The engine drives a reversible 	refrigerator which operators between reservoirs at temperatures off 50C and -25C, the heat transfer to the engine is 2500 kJ and the net work output of the combined engine refrigerator is 400 kJ. Determine the heat transfer to the refrigerator and the net heat transfer to the reservoir at 50C.

(OR)

19.	One kg of ice at -5ºC is exposed to the atmosphere which is at 20ºC. The ice melts and comes into thermal equilibrium with the atmosphere. Determine the entropy increase of the universe.  State Clausius theorem. Prove that cyclic integral of dQ/T for a reversible cycle is equal to zero.

[P.T.O]

20.	A vessel of volume 0.04m2 contains a mixture of saturated water and saturated steam at a temperature of 250C. The mass of the liquid present is 9 kg. Find the pressure, the mass, the specific volume, the enthalpy, the entropy, and the internal energy?

(OR)

21.	Steam at 7 bar and 0.9 dryness expands to 1.5 bar. Calculate the change in internal energy, entropy and enthalpy using steam table for the following cases.

a.	The expansion is isothermal							  					    (7)

b.	The expansion is adiabatic													    (8)

22. From T-ds relations prove the following for an ideal gas.

a.	Cp – CV = R																    (7)

b.	Joule Thomson coefficient is Zero.											    (8)

(OR)

23.	Write down the Clausius Clapeyron equation, and derive the expression for the difference in heat capacities, Cp and Cv. What does the expression signify?

24. Explain the following terms 	a.	Quantum principle  								    (7)

b.	Uncertainty principle								    (8)

(OR)

25.	a.	Derive Maxwell’s relations													    (7)

b.	Deduce Cp – CV = TV2/KT													     (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	STRENGTH OF AEROSPACE MATERIALS			      Time : 3 hours

Subject Code:	12AE203		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the relation between different elastic constants.

2.	Define Thermal stress.

3.	Write any two assumptions made in simple Bending.

4.	__________load is one which is considered to act at a point.

5.	What is pure bending?

6.	A beam of unit length, fixed at both ends carries a point load of 20kN at the Centre .If EI for the beam is 80MNm2, deflection at the center of beam is__________.

7.	What are the effects of torsion?

8.	Define: Torsional rigidity.

9.	Maximum principal stress theory was postulated by__________.

10.	What is Mohrs circle?

PART – B  (5 x 3 = 15 MARKS)

11.	Define the term modulus of rigidity. Also write the relationship between Young’s modulus and modulus of rigidity.

12.	Write the sign convention for shear force and bending moment.

13.	What is called section modulus? Write its expression for a hollow circular shaft.

14.	Explain the moment area method of calculation of slope and deflection?

15. 	State the condition for maximum shear stress when a member is subjected to two direct stresses in two mutually perpendicular directions accompanied by a single shear stress.

PART – C  (5 x 15 = 75 MARKS)

16.	The following data refer to a mild steel specimen tested in a laboratory. Diameter of the specimen = 25mm, length = 300mm, extension under a load of 15N = 0.045mm, load at yield point = 127.65kN, maximum load = 208.6 kN, length of the specimen after failure = 375mm, Neck diameter = 17.75mm. Determine (a) Young’s modulus, (b) Yield point, (c) ultimate stress, (d) percentage of elongation, (e) percentage reduction in area, (f) safe stress in a factor of safety of 2.	`

(OR)

17. Derive a relation between the modulus of elasticity and the modulus of rigidity.

18. Draw the S.F and B.M. diagrams for a cantilever loaded as shown in figure.

[P.T.O]

(OR)

19. 	A simply supported beam of length 12m carries a uniformly Distributed Load of 25KN/m. 	Draw the S.F and B.M diagram for the beam. Also calculate the position and magnitude of 	maximum bending moment.

20. 	A hollow shaft of external diameter 120 mm transmits 300 kW power at 200 rpm. Determine 	the maximum internal diameter if the maximum stress in the shaft is not to exceed 60 N/mm2.

(OR)

21.	A close coiled helical spring made of 14 mm round steel has 14 coils and mean diameter of the coils is 160 mm. The spring is subjected to an axial load of 160 N. Determine the elongation, intensity of torsional stress and strain energy per unit volume under the loaded condition. Take G=0.85x105 MPa.

22. 	A simply supported beam AB of span 5 meters is carrying a point load of 30 kN at a distance 	3.75 m left end A. Calculate the slopes at A and B, deflection under the load, maximum 	deflection and the distance of maximum deflection. Take EI = 26x1012 Nmm2

(OR)

23. 	A pin-pin column is subjected to a compressive load ‘P’ with eccentricity ‘C’. Determine the 	maximum deflection and bending stress in the column.

24.	The stresses at a point in a component are 100 MPa (Tensile) & 50 MPa (Compressive). Determine the magnitude of the normal and shear stresses on a plane inclined at an angle of 250 with the tensile stress. Also determine the direction of the resultant stress and the magnitude and maximum intensity of shear stress.

(OR)

25.	Derive and explain the following theories

a.	Maximum principal stress theory

b.	Maximum principal strain theory

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	AERODYNAMICS										      Time : 3 hours

Subject Code:	12AE204									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Differentiate between finite control volume and infinitesimal fluid element approach.

2.	What is the physical meaning of the divergence of velocity.

3.	List out the difference between velocity potential function and stream function?

4.	For the velocity field   u = y/(x2 + y2)  and v = - x/(x2 + y2). Write the equation of streamline passing through the point (0, 5).

5.	Define Vortex flow.

6.	What is meant by source strength.

7.	What is zero lift angle of attack.

8.	For a symmetric airfoil, the center of pressure and aerodynamic centre are both at the _______.

9.	What phenomena cause induced drag?

10.	Define Geometric twist and Aerodynamic twist.

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly discuss on the importance of center of pressure.

12.	Define Circulation. How are angular velocity and vorticity related?

13.	Consider an airfoil in a flow at standard sea level conditions with a free stream velocity of       50 m/s. At a given point on the airfoil, the pressure is 0.9 x 105 N/m2. Calculate the velocity at this point.

14.	Explain the nomenclature of NACA 65-218 Airfoil.

15.	For an elliptic lift distribution induced drag is maximum. True/False. Justify your answer.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain in detail about the types of flow with neat sketches.						  (10)

b.	Give the relationship between line, surface and volume integrals.			 		    (5)

(OR)

17.	a.	Explain in detail about the aerodynamic forces and moments.						  (10)

b.	In low-speed, incompressible flow, the following experimental data are obtained for an NACA 4412 airfoil section at an angle of attack of 4°: Cl = 0.85 and Cm,c/4 = – 0.09. Calculate the location of the center of pressure.								 	    (5)

18.	a.	Derive the energy equation for an unsteady three dimensional flows in integral form using 	a control volume approach. 													  (10)

b.	The stream function in a two-dimensional, incompressible flow field is given as ψ =3xy   Determine the velocity components and check whether the flow is irrotational or rotational. 																	    (5)

(OR)

19.	List out the body forces and surface forces and derive the general x-momentum equation 	for an unsteady 3-D inviscid flow in partial differential form using a control volume approach.

[P.T.O.]

20.	a.	A source of strength 5 m2/s located at (–1,0) is combined with a sink of strength 7 m2/s 	located at (1,0). Find the stream function and the velocity potential function at point (2,1).							    (8)

b.	Derive the stream function and velocity potential function for a Doublet flow and Prove that the stream lines for a doublet are a family of circles with a diameter k/2πC.		    (7)

(OR)

21.	Describe in detail the Kutta – Joukowski theorem and generation of lift.

22.	a.	Define an Airfoil and explain its characteristics and nomenclature.			  		    (7)

b.	Write short notes on: Kutta condition and Kelvin’s circulation theorem.			  	    (8)

(OR)

23.	Derive the fundamental equation of thin airfoil theory. For flows over symmetrical aerofoil, show that the lift co-efficient is proportional to angle of attack.

24.	a.	Write short notes on: Downwash and Induced drag.								    (8)

b.	The measured lift slope for the NACA 23012 airfoil is 0.1080 degree-1, and αL=0= –1.3°. Consider a finite wing using this airfoil, with AR = 8 and taper ratio = 0.8. Assume that      δ= τ. Calculate the lift and induced drag coefficients for this wing at a geometric angle of attack = 7°.						  											    (7)

(OR)

25.	Briefly describe the nature of the flow past a wing of finite span. Also show that the downwash at any point along the span is proportional to the lift coefficient CL and that the induced drag is proportional to CL2.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	THERMAL ENGINEERING							      Time : 3 hours

Subject Code:	12AE207				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Which law is applicable for the behavior of a perfect gas __________.

2.	According to Dalton’s Law ,the total pressure of the mixture of gases is equal to __________.

3.	At very low temperature ,the melting and boiling temperatures become equal. This temperature is __________.

4.	On Mollier chart, flow through turbine is represented by __________.

5.	In Rankine cycle, the work output from the turbine is given by change of 	__________between inlet and outlet.

6.	Efficiency of cycle will increase due to Reheating if  mean temperature of heat addition in boiler is __________ than in reheat process.

7.	A coal containing very high percentage of durians is called __________ coal.

8.	During combustion of gaseous fuels, deficiency of air __________ the flame.

9.	The wet bulb temperature during sensible heating of air __________.

10.	In a refrigerating machine, heat rejected is __________ heat absorbed.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the difference between ideal and real gases?

12.	How will you increase the nozzle efficiency of the system?

13.	Define Brayton cycle.

14.	Write short notes on Gases fuels.

15.	Explain Refrigerants.

PART – C  (5 x 15 = 75 MARKS)

16. 	A certain quantity of air initially at a pressure of 8 bar and 280°C has a volume of 0.035 m3. It undergoes the following processes in the following sequence in a cycle : (a) Expands at constant pressure to 0.1 m3 (b) Follows polytropic process with n = 1.4, and (c) A constant temperature process (which completes the cycle).

(OR)

17. 	Explain briefly Dalton’s law and Gibbs-Dalton law.

18. 	In a nozzle steam expands from 12 bar and 3000C to 6 bar with flow rate of 5 kg/s. Determine throat and exit area if exit velocity is 500 m/s and velocity at inlet to nozzle is negligible. Also find coefficient of velocity at exit. Coefficient of velocity is the ratio of actual velocity of fluid at nozzle exit to the velocity at exit considering isentropic flow through nozzle

(OR)

19. 	Explain the following

a.	Velocity triangles.															(8)

b.	Steam turbine governing.														(7)

[P.T.O.]

20. 	In a constant volume ‘Otto cycle’, the pressure at the end of compression is 15 times that at the start, the temperature of air at the beginning of compression is 38°C and maximum temperature attained in the cycle is 1950°C. Determine (i) Compression ratio. (ii) Thermal efficiency of the cycle. (iii) Work done. Take γ for air = 1.4.

(OR)

21. 	Explain briefly Brayton cycle. Derive expression for optimum pressure ratio.

22. 	Briefly explain the classification of fuels.

(OR)

23. 	Find the stoichiometric air-fuel ratio for the combustion of ethyl alcohol (C2H6O), in a petrol engine. Calculate the air-fuel ratios for the extreme mixture strengths of 80 per cent and 130 per cent. Determine also the wet and dry analyses by volume of the exhaust gas for each mixture strength.

24. 	A Bell-Coleman refrigerator operates between pressure limits of 1 bar and 8 bar. Air is drawn from the cold chamber at 9°C, compressed and then it is cooled to 29°C before entering the expansion cylinder. Expansion and compression follow the law pv1.35 = constant. Calculate the theoretical C.O.P. of the system. For air take γ = 1.4, cp = 1.003 kJ/kg K.

(OR)

25. 	In detail explain the equipments and components of air –conditioning system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	AIRCRAFT STRUCTURES								      Time : 3 hours

Subject Code:	12AE212				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The number of equilibrium equations required for space truss analysis is _________.

2.	Continuous beam is a _________ type of beam (determinate/Indeterminate/)

3.	Name the structural components of a semi-monocoque wing.

4.	Name the materials used for aircraft constructions.

5.	What is shear flow? State its unit.

6.	The locus of a shear center is __________.

7.	Define effective width.

8.	What is local bucking?

9.	Define composite.

10.	List the properties of a carbon fiber.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the differences between determinate and indeterminate structures?

12.	List the function of a wing Rib and stiffener.

13.	Locate the shear center of the following sections

a.	L-Section

b.	C-Section

c.	T-Section

14.	What are the differences between inter rivet buckling and sheet wrinkling?

15.	List the advantages of a glass-fiber composite.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the forces in members of the truss shown in figure by using method of Joints.

(OR)

17.	Find the support moments and draw bending moment diagram for the indeterminate beam shown in figure.

18.	Explain the constructions details of a semi-monocoque wing and state the function of each component with its loading conditions.

(OR)

19.	Consider aircraft wing as cantilever beam and draw the shear force and bending moment diagram for a flight load.

20.	Find the shear flow distribution and locate the shear center for the section shown in fig.3.Each of the stringers has an area of 4 cm2 and the section subjected to vertical shear of 50 kN.

(OR)

21.	Locate the shear center for the section shown in fog.16. Plot the shear stress distribution when a vertical shear load of 1.2kNacts through the shear center

22.	Obtain expression for a buckling load of a thin plate subjected to compressive load at the two opposite edges.

(OR)

23.	 Explain the following methods to determine the crippling load:

a.	Needham’s methods.															(8)

b.	Gerard’s methods.															(7)

24. Explain the properties and applications of the following

a.	Polymer matrix composites.													(8)

b.	Aramid fiber reinforced composites. 											(7)

(OR)

25.	Explain any two composite manufacturing processes with neat sketch.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	FLIGHT DYNAMICS									      Time : 3 hours

Subject Code:	12AE214									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	On an aerofoil the forces of lift and drag are vertical and parallel respectively to the ________.

2.	The lowest total drag in level flight is when the parasite drag is equal to ________ drag.

3.	The manoeuvring load factor positive limit for a utility light aeroplane in the clean configuration is ________.

4.	Maximum horizontal speed occurs when maximum thrust is equal to ________ drag.

5.	The CG of a stable aeroplane is located at the ________.

6.	Static longitudinal stability is created by the CG being located ________ the netrual point.

7.	The lateral static stability is decresed by ________.

8.	A swept wing causes ________ effect.

9.	The short-period mode is an osscillation about the ________ axis.

10.	Dynamic longitudinal stability requires positive static ________ stability.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the velocity Vs power curves for propeller driven aircrafts and turbojet aircrafts.

12.	State the condition of minimum drag in level flight.

13.	Define degrees of freedom and how much required for an aircraft?

14.	What is the effect of dihedral on sweep back?

15.	Define Dutch roll.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the effect of Reynolds number on skin friction drag and pressure drag.

(OR)

17.	a.	Derive an expression for drag polar.											    (8)

b.	Explain the various types of drag.												    (7)

18.	An aircraft is flying steady level flight at η times its minimum power speed. Show that      P/Pmin = (3+ η4)/4η. Where P is the power required at flight speed and Pmin is the minimum power required.

(OR)

19. 	Obtain the expression for turn radius and turn rate for pull up and push over maneuver.

20. 	Derive the expression for wing contribution to static longitudinal stability and offer your comments on this expression.

(OR)

21. 	Derive an expression for stick fixed & stick free neutral point equation.

22. 	Write short notes on:

a.	One engine inoperative condition.												    (8)

b.	Rudder lock.																	    (7)

(OR)

23. 	Discuss the contribution of various components of airplane towards static directional stability.

24.  	Explain stability derivatives in lateral and directional dynamics.

(OR)

25. 	Write short notes on:

a.	Directional divergence.														(8)

b.	Spiral divergence.   															(7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	COMPUTATIONAL FLUID DYNAMICS			      	     Time : 3 hours

Subject Code:	12AE216						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the mathematical expression for steady and uniform flow.

2.	What is conservative form and differential form of general transport equation?

3.	Define the Prandtl mixing length.

4.	Express the mathematical form of Turbulent Kinetic Energy.

5.	What is discretization?

6.	What is consistence in numerical scheme?

7.	What is a closure problem?

8.	What is hybrid scheme?

9.	What are the two types of Boundary conditions?

10.	What is turbulent velocity?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the zones of dependence & influence?

12.	What is turbulence?

13.	What is convergence? Explain the condition for convergence in iterative method.

14.	Sketch the distribution of a property in the vicinity of a source at different Peclet numbers.

15.	Sketch and explain staggered grid for u – velocity.

PART – C  (5 x 15 = 75 MARKS)

16.	From the first principles derive the Energy equation.

(OR)

17.	a.	The following are the two components of velocity; determine the third component of 	velocity such that the incompressible continuity equation is satisfied.

u = x2 + yz +z2; v = xy2 – yz2 + xy. 												    (8)

b.	A fluid flow field is given by V = ix2y + jy2z – k ( 2xyz + yz2 ); calculate the velocity and 	acceleration at a point (2,1,3). 													    (7)

18.	Derive the time averaged Navier Stokes equations.

(OR)

19.	a.	What is the purpose of turbulent modeling and list the classical turbulent models. 	    (4)

b.	Explain the principle of Reynolds time averaging using u and u’. 					    (4)

c.	What are the advantages and dis-advantages of k – ε model. 						    (7)

20.	Derive an expression using FV method for a one dimensional unsteady diffusion and explain the explicit and implicit schemes.

(OR)

21.	An insulated rod whose ends are maintained at constant temperatures of 100o C and 500o C respectively.  Calculate the steady state temperature distribution in the rod of 500 mm length by using finite volume method.  And also compare the result with the analytical solution. Take thermal conductivity is 1000 W/m K, Cross-sectional area of rod is 10 x 10-3 m2.

22.	From the first principles derive and FV expression for a steady 1 dimensional convection – diffusion problem.

(OR)

23.	Derive the expressions for central and upwind differencing schemes and discuss the assessment of these two schemes.

24.	Draw a neat sketch of SIMPLE algorithm and derive and explain each step.

(OR)

25.	a. 	Explain the equation		A−1=1A CT .										    (3)

b. 	Explain: manipulation ofAI to obtainIA−1 .   							    (3)

c. 	Find the solution to the following simultaneous equations by one of the above methods

a + 2b + 3c = 14

4a + 5b – 6c = -4

7a -8b + 9c = 18															    (9)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	AEROSPACE PROPULSION							      Time : 3 hours

Subject Code:	12AE218				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Classify the aircraft engines.

2.	__________ Engine carries both Fuels and Oxidizers.

3.	Define degree of reaction.

4.	What is the function of impeller in a centrifugal compressor?

5.	___________ is method of thrust augmentation by adding additional fuel after the turbine section and burning at the inlet of a nozzle.

6.	__________ Engine have high SFC.

7.	State the phenomena of the stall in subsonic inlet.

8.	What is thrust reversal?

9.	Define specific impulse.

10.	Name the types of pumps used in liquid propellant rockets.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the differences between Air breathing propulsion and non-air breathing propulsion?

12.	State the principle of operation of axial flow compressor.

13.	What is propulsive efficiency, thermal efficiency and overall efficiency?

14.	What are functions of a nozzle?

15.	Compare solid propellant rocket and liquid propellant rocket.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the factor affecting the combustion chamber design.

(OR)

17.	Explain the combustion chamber operation.

18.	Explain the working of a centrifugal compressor and draw the velocity triangles.

(OR)

19.	A 10 stage axial flow compressor provides an overall pressure ratio of 5:1 with an overall isentropic efficiency of 87% when the temperature of air at the inlet is 15˚C. Work is equally distributed between the stages. A 50% reaction is used with a blade speed of 210 m/s and constant axial velocity of 170 m/s. Estimate the blade angles. Assume Ω = 1.

20.	Derive an expression for aircraft thrust and propulsive efficiency of an aircraft engine.

(OR)

21.	Explain the operation of turbojet engine with after burner.

22.	Explain successive steps in the acceleration and over speeding of a one dimensional supersonic inlet with sketches.

(OR)

23.	What are the types of nozzle? Explain various operating conditions of a C-D nozzle with suitable sketch.

24.	Explain function of liquid propellant rocket with neat sketch.

(OR)

25.	Explain the following:

a.	Grain shapes.																    (8)

b.	Cryogenic propulsion. 														    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	GAS DYNAMICS										      Time : 3 hours

Subject Code:	12AE220		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The flow in which the density of the fluid elements is always constant is

a.	Compressible flow    b.	Incompresssible flow  c.	isentropic flow

2.	Speed of sound value is __________.

3.	For a supersonic flow, velocity is __________.

4.	“Leading edge stall” is a characteristic of relatively __________airfoils.

5.	Continuity equation for an incompressible fluid is________.

6.	The general energy equation is applicable to __________.

7.	Define aerodynamic center?

8.	What is meant by lift?

9.	Flow on the upper surface of a finite wing deflected toward the __________.

10.	 The frequency measured in the wake of a cylinder is caused by __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain Mach number spectrum.

12.	What is expansion hodograph?

13.	Difference between mach waves and mach angles.

14.	Define Drag divergence Mach number.

15.	What are the effects of wing tip?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain why converging diverging configuration is required for the acceleration of flow from subsonic to supersonic conditions.

(OR)

17.	What are the flow losses that are suffered by a compressible flow in a variable area ducts? How does the back pressure affect the losses?

18.	An incident shock wave with wave angle=35 degree impinges on a straight wall. If the upstream flow properties are M1 = 3, P1 =1 atm, T1 =300 K, calculate the reflected shock wave angle with respect to the wall.

(OR)

19.	What is shock polar? What is use in supersonic aerodynamics and explain in details?

20.	Derive the general x-momentum equation for an unsteady 3-D inviscid flow in partial differential form using a control volume approach.

(OR)

21.	Derive the small perturbation potential theory.

22.	Using linearized potential equation show that Glauert’s compressibility factor can be 	obtained for coefficient of pressure, lift, moment and aspect ratio.

(OR)

23.	Define the terms critical Mach number and drag divergence Mach number with suitable 	sketch. Explain in detail about various methods to increase the critical Mach number.

24.	Explain the complete characteristics details of swept wing with suitable plots and sketches and give breif description about double delta wings.

(OR)

25.	Write in detail about transonic area rule with plots and sketches.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	HEAT & MASS TRANSFER								      Time : 3 hours

Subject Code:	12AE221				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Heat transfer takes place as per ___________.

2. 	Heat flows from one body to other when they have ___________.

3. 	The heat transfer from one surface (at a temperature T1) to the other surface (at T2) can be expressed as ___________.

4. 	State Biot number.

5. 	___________ is a mechanism, or type of transport in which fluid motion is generated by an external source.

6. 	Convection-cooling is sometimes loosely assumed to be described by ___________.

7. 	Heat energy received by the earth from the sun is due to ___________.

8. 	Which surface will radiate heat to large extent?

9. 	The combustion chamber in gas turbines and jet engines (including ramjets and scramjets) is called the ___________.

10. 	___________ is the reduction of heat transfer between objects in thermal contact or in range of radiative influence.

PART – B  (5 x 3 = 15 MARKS)

11. 	Define Fourier’s law of Conduction.

12. 	What is the use of Grober Charts? Explain with example.

13. 	Discuss the difference between free and forced convection.

14. 	Explain Mass Transfer Coefficient.

15. 	Write short notes on Aerodynamic Heating.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the significance of Biot number in transient heat conduction.		 		(4)

b.	A 50 × 50 cm2 aluminum slab of 6mm thick is at 400oC, initially and is suddenly immersed in water. So its surface temperature is lowered to 50°C. Determine the time required for the slab to reach 120°C by lumped heat analysis. Take heat transfer co-efficient h=100 w/m2K. 																			(10)

(OR)

17. 	a.	Derive an expression general heat conduction equation in Cartesian coordinates.      	  (7)

b.	An aluminum alloy fin of 5mm thick and 40 mm long and 1m width protrudes from a wall. The base temperature is 420°C and ambient air temperature is 25oC. The heat transfer co-efficient between aluminum rod and environment is 25 W/m2K. Calculate the heat loss from the fin material taking its thermal conductivity as 200 W/m-K. Also find efficiency of the fin.									       									(7)

18.	Briefly explain the Lumped Heat Capacity System method.

(OR)

19.	In detail explain the two dimensional steady state conduction.

[P.T.O.]

20.	Discuss the forced convection governing equations with suitable example.

(OR)

21.	a.	A pot of water, 20 cm in diameter and 15 cm tall, is kept at boil on a stove. The convection 	coefficient on top and side surfaces is 25 W/m2K, and the room temperature is 22 °C. 	Calculate the heat loss due to convection, neglecting the bottom surface.	  		    (8)

b.	Discuss the effect of heat transfer on inclined surfaces.			        	   		    (7)

22.	 A radiation shield is placed between two large parallel plates, one at 1000 K with emissivity 0.8 and other at 300 K with emissivity 0.6. The shield has emissivity as 0.1 on the side facing hot plate and 0.3 on the side facing cold plate. Calculate percentage reduction in radiation heat transfer as a result of radiation shield.

(OR_)

23.	 Emissivites of two large parallel plates maintained at T1K and T2K are 0.6 and 0.6 respectively. Heat transfer is reduced by 75 times when a polished aluminum radiation shields of emissivity 0.04 are placed in between them. Calculate the number of shields required.

24. 	a.	Discuss the application of heat transfer in aerospace engineering.		                   	    (7)

b.	Write brief notes on Heat Transfer Distribution.					      				    (8)

(OR)

25. 	Explain the following 	(a) Ablative Heat Transfer					      					    (8)

(b) Thrust Chambers						      					    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	FINITE ELEMENT ANALYSIS						      	      Time : 3 hours

Subject Code:	12AE222										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The functional is defined as _________.

2.	The finite element formulation based on calculus of variation is ___________.

3.	The residual is defined as _________.

4.	A shaft carrying a bevel gear should be modeled by 1D _________element.

5.	The plane stress deformation is characterized by _________.

6.	A quadratic spar element has __________ number of nodes.

7.	A bilinear quadrilateral element has _________ degrees of freedom.

8.	The elements with nodal points located only on the boundary are called as 	_________ elements.

9.	The strain matrix is analogous to ___________ matrix for a heat transfer problem.

10.	________ law relates the velocity of fluid flow to the hydraulic gradient.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the principle of subspace iteration method?

12.	Distinguish between variational and weighted residual methods.

13.	What are the limitations of CST element?

14.	Distinguish the characteristics of CST and quadrilateral elements.

15.	Mention two natural boundary conditions as applied to thermal problems.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the types of analysis that can be carried out using FEM with examples.

(OR)

17.	Determine the nodal displacements from the following set of equations using Choleski method:

12u1 + 6u2 - 3u3 = -30

6u1 + 10u2 - 5u3 + 2.5u4 = -140

-3u1 - 5u2 +8u3 + 4u4 = 175

2.5u2 + 4u3 + 4u4 = 17.5

18.	Derive the shape functions and element stiffness matrix for the quadratic spar element.

(OR)

19.	Using Galerkin weighted residual technique, solve for the displacement field for a simply supported beam subjected to a uniformly distributed load.

20.	A pin-jointed frame has vertices A(0,0), B(100,0) and C(170,-50). It is fixed at AC and subjected to a vertical upward load of 1500 N at B. The steel bars have a cross-sectional area of 1000 mm2 with Young’s Modulus 200 GPa.  Determine the nodal displacements and find the reaction forces.

(OR)

21.	Determine the natural frequencies and mode shapes of transverse vibration for a beam fixed at both ends. The beam may be modeled by two elements, each of length L and cross-sectional area A.

[P.T.O.]

22.	Derive the force vectors for the following two cases:

a.	body force acting on CST element.

b.	uniformly distributed load acting on one of the edges of the above element.

(OR)

23.	Consider a rectangular element defined by nodal coordinates (in ‘m’) as 1(0,0), 2(4,0), 3(4,2) and 4(0,2). The nodal displacement vector is given by q = [0.03, 0.04, 0, 0, 0,0,  0.05, 0.06]T. Evaluate the stress at the centroid of the element, assuming plane strain condition. Take Young’s modulus and Poison’s ratio as 30 x 106 N/m2 and 0.3 respectively.

24.	A hollow circular fin of length 0.4 m and cross-sectional area 0.1 m2 is insulated on the perimeter. The left end has a constant temperature of 100° C. A positive flux of 5000 W/m2 acts on the right end. Taking the thermal conductivity of the fin as 6 W/m° C, determine the temperature distribution using two elements.

(OR)

25.	Establish the finite element equations including force matrices for the analysis of one dimensional steady-state fluid flow through a porous medium.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	GUIDANCE AND CONTROL OF AEROSPACE VEHICLES

Time : 3 hours

Subject Code:	12AE223				 						      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The basic principle involved in accelerometers is __________.

2.	What is INS used for ?

3.	Name any two communication system.

4.	Name any two navigation systems.

5.	SELCAL refers __________.

6.	What is aileron differential control systems?

7.	Difference between SAT and TAT.

8.	How is gyro powered?

9.	What is TCAS?

10.	ACARS refers __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Inertial navigation.

12.	Difference between planar and non – planar.

13.	Write down the applications of UAV.

14.	What is meant by guided missiles?

15.	Define rocket thrusters.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the principle involved and construction of accelerometers and gyros.

(OR)

17. 	Define the following:

a. 	Attitude Heading Reference System.                 									(5)

b. 	Global positioning System.                              									(5)

c. 	Terrain Reference Navigation.                          									(5)

18.	Write in detailed about navigation guidance and control systems.

(OR)

19.	Explain the following.

a. 	Minimum Energy orbital transfer.               										(10)

b. 	Acceleration input of guidance system.        										(6)

20. 	Explain the merits (advantage) and demerits (disadvantages) of analog and digital fly by 	wire control systems.

(OR)

21.  What is an Autopilot? Briefly explain about longitudinal and lateral autopilots for an aircraft.

22. 	Derive the optimal control of dynamic systems.

(OR)

23. 	Explain the adaptive pitch control systems and yaw control of rockets.

24. 	Explain Missile control systems and Launch vehicle flight control systems.

(OR)

25. 	Derive the equation of Multi-axis rotational maneuvers of aircraft.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	CRYOGENIC PROPULSION							      Time : 3 hours

Subject Code:	12AE228				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the boiling temperature of liquid oxygen?

2.	Define isentropic process.

3.	State any one cryogenic propellant combination used in the rocket engine

4.	When the natural gas is condensed into a liquid at close to atmospheric pressure by cooling it to approximately ____________.

5.	Which phenomenon is eliminated while using filled Powders and fibrous materials as insulating materials?

6.	State any two highly porous materials used in Evacuated porous insulation

7.	State any one of the application of Cryostats.

8.	What is the medium employed in the Dewar vessels for storing cryogens?

9.	What is Magneto-caloric effect?

10.	Mention any one of the cryogenic applications in preservation.

PART – B  (5 x 3 = 15 MARKS)

11.	Sate the principle of Joule-Thomson effect.

12.	What do you mean by superconductivity?

13.	Classify different methods employed in insulation of cryogenic vessels and equipment.

14.	What do you mean by Geysering effect?

15.	What is Cryo pumping?

PART – C  (5 x 15 = 75 MARKS)

16.	Illustrate thermos-physical and fluid dynamic properties of liquid and gas hydrogen.

(OR)

17.	Explain the production of low temperatures using Joule-Thomson effect.

18.	Explain about the working of a precooled Linde-Hampson system with suitable diagram for neon and hydrogen.

(OR)

19.	What are the properties of the following cryogenic fluids?

(a) Hydrogen (b) Helium 3 (c) Helium 4

20.	What is the necessity of insulation in the cryogenic systems? Explain in detail about Vacuum insulation and Evacuated porous insulation.

(OR)

21.	What do you mean by Multilayer insulation? Explain in detail how the liquid – vapor shields act as insulation for the cryogenic storage tanks.

22.	Briefly explain about the basic design parameters of cryogenic fluid storage vessels.

(OR)

23.	Briefly explain about the cryogenic fluid transfer system.

24.	Explain the term cryogenics and its development in the field of space technology.

(OR)

25.	Illustrate in detail about the variables affecting heat exchanger and system performance of cryogenic equipment