Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	MECHANICS OF FLUIDS								      Time : 3 hours

Subject Code:	09CE202		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the term fluid mechanics.

2.	List some of the pressuring devices.

3.	Write the methods available for analyzing the fluids in motion.

4.	State how laminar and turbulent flow is differentiated.

5.	Draw the line sketch of venturimeter and mention its component parts.

6.	State the purpose of using pitot tube.

7.	Give example for major loss in pipes.

8.	Write the Darcy’s equation for friction.

9.	What is the dimension for discharge?

10.	Define Froude number.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between Meta centre and Metacentric height.

12.	Write the basic principles of fluid flow.

13.	Define Coefficient of Discharge.

14.	Write the Dupit’s equation to calculate the diameter of an equivalent pipe.

15.	List the forces influencing hydraulic phenomena.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Calculate the density, specific weight and weight of one litre of petrol of specific gravity 	0.7.

b.	Determine the specific gravity of oil having a viscosity of 0.07 poise and a kinematic 	viscosity of 0.042stokes.						            							(7+8)

(OR)

17.	A circular plate 1.5m diameter is submerged in water with its greatest and least depths below the surface being 2m and 0.75m respectively. Find the total pressure and centre of pressure.

18.	The velocity potential function for a two dimensional flow is Ø = xx(2y – 1). At a point p (4, 5) find the velocity and the value of stream function.

(OR)

19.	Derive the Bernoulli’s Equation applicable for steady, incompressible and irrotaional flow.

20.	An inclined venturimeter is installed for measuring the rate of flow with the diameter of inlet and throat as 40cm and 20cm respectively. The angle of inclination to the horizontal is 30°. The distance between the inlet and throat is 0.6m, the specific gravity of oil flowing in the meter is 0.7 and the differential mercury manometer shows a deflection of 50mm. Take the discharge coefficient as 0.98 and find the rate of flow.

(OR)

21.	Derive the Darcy – weisbach equation for friction.

[P.T.O.]

22.	Comment briefly on major and minor losses in pipes.

(OR)

23.	Three pipes of diameters 300mm, 200mm, and 400mm and length 450m, 255m, and 315m are connected in series. The difference in water surface level in two tanks is 18m. Determine the rate of flow of water if coefficients of friction for the pipes are 0.0075, 0.0078, and 0.0072. Neglect minor losses.

24.	Compare the Rayleigh’s method and Buckingham π method used in dimensional analysis.

(OR)

25.	Write a note on

a.	Limitations of dimensional analysis.

b.	Model analysis.

c.	Undistorted models.															(3x5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	MECHANICS OF DEFORMABLE BODIES – I		     	    Time : 3 hours

Subject Code:	09CE205							 			      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Draw the stress strain diagram of a brittle material and narrate the various points.

2.	Derive the relationship between elastic constants?

3.	Define slenderness ratio. Discuss its significance.

4.	Discuss strength and efficiency of riveted joints.

5.	Name the type of end conditions of beam.

6.	Define: Contra flexure.

7.	State the theory of simple bending.

8.	Write down the formula for calculating the load carrying capacity of beams.

9.	What is meant by flitched beams?

10.	What is the use of buffer springs?

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between Determinate and indeterminate systems.

12.	What are the assumptions of materials?

13.	Give the expression for SF and BM of a cantilever beam loaded at end with Load “W”.

14.	Write down the bending equation and narrate the components.

15.	What is meant by composite shaft? Where it is used?

PART – C  (5 x 15 = 75 MARKS)

16.  	Find the Poisson’s ratio and Bulk modulus of a material whose modulus of elasticity is         200 GPa and modulus of rigidity is 80 GPa. A 2 m long rod of 40 mm dia. Made with the same material is stretched by 2.5 mm under some axial load. Find the lateral contraction.

(OR)

17. 	Rails of 15 m length were laid on the track when the temperature was 200C. A gap of 1.8 mm was kept between two consecutive rails. At what max temperature the rails will remain stress free? If the temperature is raised further by 150C, what will be the magnitude and nature of stresses induced in the rails?

18. 	a.	Derive the expressions for the circumferential stress and longitudinal stress in thin 					cylinders.  																	(7)

b.	A curved beam of circular section of diameter 140 mm is subjected to pure bending moment of 34.5 KNm. The radius of curvature is 100 mm, Determine: (a) position of neutral axis and (b) maximum and minimum stresses.    							(8)

(OR)

19. 	A cantilever beam of length 3 m is carrying a uniformly distributed load of w KN/m. Assuming rectangular cross-section with depth(d) equal to twice the width(b), determine the dimensions of the beam, so that vertical deflection at the free end does not exceed 10 mm. Take maximum bending stress = 100 MPa and Modulus of elasticity E = 200 GPa.

20.	Sketch the bending stress distribution across the cross section of a rectangular beam section     330 × 400 m subjected to 80 KNm and locate the neutral axis of beam.

(OR)

[P.T.O.]

21.	A simply supported beam carries a uniformly distributed load of intensity w symmetrically distributed over part of its length. Determine the maximum deflection and check your results by assuming a = 0 and compare, when same beam with u.d.l on entire span.

22.	Explain theory of simple bending and derive the bending equation?

(OR)

23.	Explain how the deflection of beam under single point load can be calculated using strain energy method?

24.	Determine the diameter of solid steel shaft which will transmit 75 KW at 116 r.p.m. Also determine the length of the shaft if the twist must not exceed 2° over the entire length. The maximum shear stress is limited to 60 N/mm2. Take the value of modulus of rigidity = 8 x 104 N/mm4

(OR)

25. 	A shaft is to be fitted with a flanged coupling having 8 bolts on a circle of diameter 250 mm. The shaft may be subjected to either a direct tensile load of 440 KN or a twisting moment of 28 KNm. If the maximum direct and shearing stresses permissible in the bolt material are            125 N/mm2 and 55 N/mm2 respectively, find the minimum diameter of the bolt required. Assume that each bolt takes an equal share of the load or torque.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	MECHANICS OF DEFORMABLE BODIES II			      Time : 3 hours

Subject Code:	09CE206				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Mohr’s Theorem – I.

2.	What is a conjugate beam?

3.	What is a strut?

4.	Write the equivalent length for

A column with both ends fixed.

5.	Write any one assumption made in Lame’s theory.

6.	What is the expression for intensity of hoop stress induced in a thick cylinder?

7.	What is shear flow?

8.	What are the loads acting on a curved beam?

9.	State one reason for unsymmetrical bending.

10.	What is pure bending of a beam?

PART – B  (5 x 3 = 15 MARKS)

11.	A simply supported beam of 2 m span carries a point load of 20 kN at its mid-points. Determine the maximum slope and deflection of the beam. Take flexural rigidity of the beam as 500 x 109 N/mm2.

12.	Differentiate a short column and a long column.

13.	Differentiate a thin cylindrical shell and a thick cylindrical shell.

14.	Explain Winkler Bach theory.

15.	How will you calculate stress due to unsymmetrical bending?

PART – C  (5 x 15 = 75 MARKS)

16.	A simply supported beam AB of span 4 m, carrying a load of 100 kN at its mid-span C has cross-sectional moment of inertia 24 x 106 mm4 over the left half of the span and  48 x 106 mm4 over the right half. Find the slopes at the two supports and the deflection under the load by conjugate beam method. Take E = 200 GPa.

(OR)

17.	A horizontal beam AB is freely supported at A and B, 8 m apart and carries a uniformly distributed load of 15 kN/m run (including its own weight). A clockwise moment of 160 kN-m is applied to the beam at a point C, 3 m from the left hand support A. Calculate the slope of the beam at C by Macaulay’s method. Take EI = 40 x 103 kN-m2.

18.	List the end conditions for columns. Derive Euler’s crippling load for columns with any two end conditions.

(OR)

19.	Find the Euler’s crippling load for a hollow cylindrical steel column of 38 mm external diameter and 2.5 mm thick, take length of the column as 2.3 m  hinged at both ends. Take         E = 205 GPa. Also determine the crippling load by Rankine’s formula using constants as 335 MPa and 1/7500.

[P.T.O.]

20.	A cast iron pipe of 400 mm internal diameter and 100 mm thickness carries water under a pressure of 8 N/mm2. Determine the maximum and minimum intensities of hoop stress across the section. Also sketch the radial pressure distribution and hoop stress distribution across the section.

(OR)

21.	Explain the theories of elastic failure.

22.	Explain shear centre and shear flow for a symmetric  ‘I’ section.

(OR)

23.	A beam of rectangular section 20 mm x 40 mm has its centre line curved to a radius of 50 mm. The beam is subjected to a bending moment of 4 x 105 N-mm. Determine the intensity of maximum stresses in the beam. Also plot the bending stress across the section.

24.	a)	Compare symmetrical bending and unsymmetrical bending.						    (7)

b)	A rectangular beam 300 mm deep is simply supported over a span of 4 m. What uniformly distributed load the beam may carry, if the bending stress is not to exceed 120 MPa. Take    I = 225 x 106 mm4.															   (8)

(OR)

25.	Two wooden planks 150 mm x 50 mm each are connected to form a T – section of a beam. If a moment of 6.4 kN-m is applied around the horizontal neutral axis, inducing tension below the neutral axis, find the bending stresses at both the extreme fibres of the cross-section.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	SURVEYING											      Time : 3 hours

Subject Code:	09CE207		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by a bench mark?

2.	Name the adjustments to be made in a leveling instrument.

3.	What are the fundamental lines of a transit?

4.	What is the significance of Gale’s traverse table?

5.	Classify tacheometric methods.

6.	What are the observations required for complete location of a point with respect to the instrument station?

7.	What is the reason for introducing a transition curve between a tangent and circular curve?

8.	What is meant by shift of a curve?

9.	What is meant by  eccentricity of signals?

10.	Distinguish between single and reciprocal observations.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the types of adjustments of a level?

12.	List out sources of error in a theodolite work.

13.	Write a note on the methods of holding the staff.

14.	What are the difficulties in setting out a simple curve?

15.	State reasons for using a satellite station in a triangulation survey.

PART – C  (5 x 15 = 75 MARKS)

16.	What is meant by reciprocal leveling? Also discuss in detail about the methods adopted for calculation  of areas and volume by leveling.

(OR)

17.	a.	Describe the characteristics of contour with sketches.					 			    (8)

b.	Write notes on fly leveling.								 					    (7)

18.	Explain the various methods of traversing with the theodolite and discuss their merits and demerits.

(OR)

19.	The following are the lengths and bearings of the sides of the traverse ABCD.

| S.No. | LINE | LENGTH, m | BEARING |
| --- | --- | --- | --- |
| 1 | AB | 76.80 | 140º12’ |
| 2 | BC | 195.60 | 36º24’ |
| 3 | CD | 37.20 | 338º48’ |
| 4 | DA | ? | ? |

Compute the length and bearing of the line DA.

20.	Outline the tangential method of tacheometry and deduce expressions for horizontal and vertical distances.

(OR)

[P.T.O.]

21.	 A staff was held vertically at distances of 45 m and 120 m from the centre of a theodoloite fitted with stadia hairs and the staff intercepts with the telescope horizontal were 0.447 m and 1.193 m respectively. The instrument was then set over a station P of R.L 500.25 and the height of the instrument was 1.45 m. The hair readings on a staff held vertically at station Q were 1.20, 1.93 and 2.66 m while the vertical angle was -9º30’. Find the distance PQ and the R.L of Q.

22.	Explain the methods used for determining the length of transition curves.

(OR)

23.	Two straights AB and BC meet in an inaccessible point B and were joined by a circular curve of 480 m radius. Two points M and N were selected on AB and BC respectively, and the following data were obtained: Angle AMN = 159º48’, Angle CNM = 155º42’, MN = 127.5 m.

Make necessary calculations for setting out the curve, by the method of tangential angles given that the chainage of M was 1400.00 m

24.	The  triangulation stations A and B, 50 km apart, have elevations 243 m and 258 m respectively. The intervening ground may be assumed to have a uniform elevation of 216 m. Find the minimum height of the signal required at B so that the line of sight may not pass nearer the ground than 2.4 m.

(OR)

25.	A steel   tape 30m   long standardized at 10ºC with a pull of 100 N was used for measuring a base line. Find the  correction per tape length, if  the temperature at the time of measurement was 20 ºC, the pull   applied  was 150 N. Density of steel = 3000 kg/m3. Weight of  tape = 5.88 N.    E = 20X10-6 N/cm2. Co efficient of expansion of tape per 1ºC = 12 x 10-6.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	STRUCTURAL ANALYSIS -I							      Time : 3 hours

Subject Code:	09CE208		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the degree of statical indeterminancy for a propped cantilever?

2.	Define virtual work.

3.	State Muller Breslau’s principle

4.	Write the use of Influence line diagram.

5.	Write the expression for horizontal thrust of a two hinged arch considering bending 	deformation only.

6.	Define arch.

7.	Write the six equation of statics for a space structure.

8.	Where are suspension bridges used?

9.	Write the application of Clapeyron’s Theorem of Three Moments to fixed beam.

10.	Write the equation of the theorem of three moments for a continuous beam.

PART – B  (5 x 3 = 15 MARKS)

11.	State Castiglano’s first and second theorem.

12.	Construct the influence line for the reaction at A and B of the beam shown in figure

C

B m

A m

4m

4m

4 m

13.	Write the cause of rib shortening in the two hinged arch bridge.

14.	Differentiate the joints in plane frames and space frames with neat sketches

15.	State the Clapeyron’s theorem of three moments.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the horizontal displacement at joint B of the frame ABCD as shown in the figure by 	virtual work method. Assume EI to be constant for all members.

[P.T.O.]

(OR)

17.	The cantilever beam CD supports a uniformly distributed load ‘w’and a concentrated load ‘P’ as shown in the figure. Take L=5m, P =8KN, w=6KN/m & EI= 5MNm2. Determine the   deflection at D using Castiglano’s theorem

18.	Construct ILD for R in the beam shown below. Compute the ordinates for 1m interval 			from A to C.

(OR)

19.	Determine the maximum reaction at support B, maximum shear at point C and maximum 	positive moment that can be developed at point C on the beam shown due to

a.	a single concentrated live load of 8000N

b.	a uniform live load of 3000N/m

c.	a beam weight (dead load)of 1000N/m

20.	A semicircular two hinged arch of constant cross section is subjected to a concentrated load as 	shown in figure. Calculate the reactions of the arch and draw BMD.

(OR)

21.	The three hinged arch is subjected to loading as shown in the figure. Determine the components 	of reactions at A and C and the tension in the cable.

22. 	Compute the forces in all the members of the cantilever truss shown in figure and indicate the forces on a sketch of the truss along with their nature.

Analyse the pin jointed plane frame shown in the figure.  The axial stiffness for each member is 40KN/mm.

(OR)

23. 	A suspension cable of 3.6m span and 6m maximum dip at centre carries a dead load of 1KN/m throughout and a uniformly distributed load of 18m length of intensity of 1KN/m which covers left hand half span. The cable is stiffened by a three hinged girder with two hinges at the abutment and third at the centre (a) Draw the BMD and SFD for the girder giving values at salient points (b)	Calculate the maximum tension in the cable.

24. 	Use three moment equation to find MA and MB. Draw BMD and SFD.

(OR)

25.	Use three moment to determine the support moments for the continuous beam. Draw BMD and SFD.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	GEOTECHNICAL ENGINEERING						      Time : 3 hours

Subject Code:	09CE212		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define soil.

2.	Define specific gravity of soil.

3.	Define plastic limit.

4.	Define plasticity index.

5.	What is a flow net?

6.	Define critical hydraulic gradient.

7.	Define coefficient of consolidation.

8.	Define compression index.

9.	What is meant by shear strength of soil?

10.	Name a field test to determine shear strength of soil.

PART – B  (5 x 3 = 15 MARKS)

11.	The void ratio of a clay sample is 0.5 and the degree of saturation is 70%.Compute the water 	content.

12.	State the uses of particle size distribution curve.

13.	What is meant by quick sand condition?

14.	State the assumptions made in Terzaghi’s theory of consolidation.

15.	State the demerits of triaxial test.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail how soils are formed.

(OR)

17.	Write a brief note on scope of Geotechnical Engineering.

18.	Explain in detail how liquid limit, plastic limit and shrinkage limit can be determined.

(OR)

19.	What is the use of classification of soils? Discuss the various soil classification systems.

20.	A sand deposit is 12m thick and overlies a bed of soft clay. The ground water table is 5m below 	the ground surface. If the sand above the ground water table has a degree of saturation of 50%, 	plot the diagram showing the variation of total stress, pore water pressure and effective stress. 	The void ratio of sand is 0.7 and specific gravity = 2.7.

(OR)

21.	Explain constant and variable head permeability test.

22. 	A 3m thick clay layer beneath a building is overlaid by a permeable stratum and is underlain by 	an impervious rock. The coefficient of consolidation of the clay was found to be 0.025cm2/min. 	The final expected settlement for the layer is 8cm. How much time will it take for 80% of total 	settlement to take place? Also determine the time required for a settlement of 2.5cm to occur 	and compute the settlement that will occur in one year.

[P.T.O.]

(OR)

23.	Explain in detail the consolidation test.

24.	The following results were obtained from an undrained shear box test on a soil.

| Normal load (kg) | 25 | 50 | 75 |
| --- | --- | --- | --- |
| Failure load (kg) | 32 | 46 | 61 |

Determine the strength parameters in terms of total stresses. The cross sectional area of the 	shear box was 36 sqcm.

(OR)

25. Discuss the method of slices for the stability analysis of slope.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	DESIGN OF MASONRY AND TIMBER  STRUCTURES

Time : 3 hours

Subject Code:	09CE213				 						     Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the limit states that are considered in Limit State Method?

2.	What are the basic requirements of a structure?

3.	Define faced wall.

4.	Differentiate between load bearing and non load bearing wall.

5.	What is meant by bed block?

6.	State the middle third rule.

7.	Define shear wall.

8.	What are reinforced masonry walls?

9.	What is meant by fletched beam?

10.	Define built up column.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the various types of loads acting on a structure?

12.	Classify masonry wall.

13.	What are the conditions for stability of masonry structures?

14.	Mention the factors to be considered while planning the earthquake resistant design of masonry structures.

15.	List the factors that affect the strength of timber structures.

PART – C  (5 x 15 = 75 MARKS)

16.	Write a detailed note on various structural systems.

(OR)

17.	Discuss the working stress method and limit state method of design in detail.

18.    A masonry wall is subjected to an axial load of 150 kN and bending moment of   30kNm. The 	height of the wall is 4 m. Design the wall.

(OR)

19.	Determine the allowable axial load on column 300 mm x 60 mm constructed in first class brick work in CM 1: 6 using modular bricks 200 mm x 10 mm x 100 mm the height of pier between the footing and top of slab are 5.2 m. The strength of units may be assumed as 10.5 MPa.

20. 	What are the factors to be considered while designing the brick masonry with respect to stability and lateral supports on the structure? Explain in detail.

(OR)

21. 	Check the stability of a masonry retaining wall having a top width= 1.2m, bottom width= 3m, height of wall = 5.5m. Its backfill surface is vertical and it remains earth level with the top of the wall. The densities of masonry and soil are 24kN/m3 and that of soil is 18kN/m3. Angle of repose is 30o.  Coefficient of friction between soil and base is 0.5. Safe bearing capacity of soil is 20t/m3.

22. 	Write a brief note on special provisions for resisting earthquake for masonry walls.

(OR)

23.  Explain in detail the behaviour of unreinforced masonry and reinforced masonry   wall against 	earthquake.

24. 	A column is to carry a load of 500 kN. Its effective length is 3.5 m. Design a solid wood column using deodar.

(OR)

25.	Design a simply supported wooden beam for a span of 5m, subjected to a UDL of 12kN/m.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title : 	CONSTRUCTION MANAGEMENT						      Time : 3 hours

Subject Code:	09CE215										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define management.

2.	What is break even point?

3.	Name scheduling methods.

4.	What is progress report?

5.	What is cost control?

6.	What is network analysis?

7.	Name different types of contract.

8.	Define tender.

9.	State different types of bills.

10.	What is inventory?

PART – B  (5 x 3 = 15 MARKS)

11.	State the functions of management.

12.	How settlement of disputes will be done?

13.	Explain CPM.

14.	Describe legal implementation.

15.	What is imprest account and cash book?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the importance of management.											(8)

b.	Discuss the management of private contractors and contracting firms.				(7)

(OR)

17.	Describe in detail about break even analysis and chart pricing policies.

18.	Briefly explain the components of construction planning.

(OR)

19.	Briefly explain the components of resource planning.

20.	A project has the following schedule

| Activity | A | B | C | D | E | F | G | H | I | J | K | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Predecessor | None | None | A | B | B | C,D | E | E | G | H | I,J | F |
| Time in weeks | 4 | 1 | 1 | 1 | 6 | 5 | 4 | 8 | 1 | 2 | 5 | 7 |

Construct a PERT chart and compute an early start, a late start and slack time for each activity. Find the critical path.

(OR)

21.	Explain the principles of cost control and its control by graphical method.

[P.T.O.]

22.	Explain the quality control, inspection and duties of personnel.

(OR)

23.	Explain briefly about tender documents, submission and processing of tenders.

24.	Explain briefly about revised estimates, completion reports and certificates.

(OR)

25.	Explain in detail about the component of stores used by PWD and CPWD.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	APPLIED HYDRAULICS AND FLUID MACHINES		      Time : 3 hours

Subject Code:	09CE216				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write Chezys equation .

2.	Best section is one in which discharge is _________ for minimum wetted perimeter.

3.	What is laminar boundary layer.

4.	What are drag and lift coefficients.

5.	Write the equation for force exerted by jet of water on fixed vertical plate.

6.	What is the jet of water.

7. 	Name any two types of  turbines .

8.   	What is cavitation.

9. 	What are velocity triangles.

10.  	What is indicator diagram.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the draw down and back water curves.

12.	Explain displacement  thickness.

13.	State and explain the impulse  momentum equation.

14.	Discuss on similarity laws.

15.	Explain different types of efficiencies.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Draw a neat sketch of specific energy curve and explain the same.					    (6)

b.	Derive the conditions for the best trapezoidal section.							 	   (9)

(OR)

17.	a.	Determine the length of back water curve caused by an afflux of 2.0m in a rectangular 	channel of width 40m and depth 2.5m. The bed slope is 1/1000.

18.	a.	Explain boundary layer theory with a neat sketch and also the methods applied for the 			boundary layer separation.

(OR)

19.	 A circular plate of 3.0 m diameter is immersed in water such that its greatest and least depths are 4m and 1.5m resp. Determine the total pressure and centre of pressure  of the plate.

20.	a.	Derive the expression for the force exerted by the jet on stationary flat  plate.		    (6)

b.	A 25 mm diameter jet exerts a force of 450N in the direction of flow against a flat  plate which is held inclined at an angle of 300, to the axis of stream. Find the rate of flow.     (9)

(OR)

21.	 A nozzle of 5 cm diameter delivers a stream of water at 20m/s perpendicular to a plate that moves away from the plate at 5m/s. Find the force on the plate , work done and the efficiency of the jet.

22.	 Explain the various operating characteristic curves of turbines with neat graphs.

(OR)

[P.T.O.]

23.	A Pelton wheel is to be designed  for the following specifications. Shaft power = 11772 KW, head = 380m, speed = 750 rpm. Overall efficiency 86%, jet diameter is not to exceed 1/6 of wheel diameter, determine (a) wheel diameter  (b) diameter of the jet, (c) the number of jets.

24.	Draw a neat sketch of reciprocating pump and explain the working principle .

(OR)

25.	A centrifugal pump is to discharge  0.118 m3/s at a speed of 1450 rpm, against a head of 25m. The impeller diameter is 250mm, its width at outlet is 50mm and manometric efficiency is 75%. Determine the vane angle at the outer periphery of the impeller.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	REINFORCED CONCRETE STRUCTURES – II			      Time : 3 hours

Subject Code:	09CE217				 						       Maximum Marks: 100

#### Answer ALL questions

Use of IS 456, IS 3370, IS 1893 and IRC codes is permitted

PART – A  (10 x 1 = 10 MARKS)

1.	What is a riser?

2.	When counterfort retaining walls are adopted?

3.	What type of force will act in circular water tanks?

4.	When reinforcement is provided in two layers in water tanks?

5.	What is the load for a wheeled vehicle as per IRC class A loading?

6.	When T-beam bridge is recommended?

7.	Define ductility.

8.	What is the wind speed of Coimbatore city?

9.	A yield line ends at __________.

10.	Write the methods of determining the ultimate load capacity of slabs.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the stability requirements for a retaining wall?

12.	Explain about the staging and foundation used for water tanks.

13.	Explain Courbon’s method.

14.	Explain Cantilever method of analysis.

15.	List the characteristic features of yield lines.

PART – C  (5 x 15 = 75 MARKS)

16.	Design a doglegged staircase in a public building to be located in a staircase room 6 m long and 3 m wide.

Height between floors = 3.6 m

Live load = 4 kN/m2

The stairs are supported on beams over walls and sides of steps are built into the wall by 120 m. Adopt M20 grade concrete and Fe 415 reinforcements.

(OR)

17.	Design a counterfort type of retaining wall to suit the following data.

Height of the wall above ground level = 6 m

Safe bearing capacity of soil at site = 160 kN/m2

Angle of internal friction = 33 degrees

Density of soil = 16 kN/m3

Spacings of counterforts = 3 m c/c

Adopt M20 grade concrete and Fe 415 HYSD bars.  Sketch the reinforcement details.

18.	A rectangular RC water tank with an open top is required to store 85,000 litres of water. The inside dimensions of the tank may be taken as 6 m x 4 m. The tank rests on all the four sides. Design the side walls of the tank using M20 grade concrete and Fe 415 HYSD bars.

(OR)

[P.T.O.]

19.	Design a circular water tank of capacity 400 m3 (400 kl) resting on the ground and having a fixed base condition due to a rigid joint between the wall and the base slab. The materials used are M25 grade concrete and HYSD steel of Fe 415. Use the method recommended in IS 3370 Part (IV).

20.	Determine the maximum live load bending moment in the extrerior girder of the T beam and slab with the following data for the two trains of IRC Class A loading using Courbon’s method.

Clear width of roadway = 7.5 m

Span(centre to centre of bearings) = 16 m

Average thickness of wearing coat = 80 mm

Concrete mix = M20 grade

Steel = Fe415 HYSD bars

(OR)

21.	Design a solid slab bridge for class A loading for the following data.

Clear span = 4.5 m.

Clear width of roadways = 7 m.

Average thickness of wearing coat = 80 mm.

Use M20 mix. Take unit weight of concrete as 24000 N/m3.

22.	Explain Portal and Substitute frame methods of analysis.

(OR)

23.	a.	Explain the design procedure of multi-storeyed RC frames.					   	    (8)

b.	List the important provisions for earthquake effects as per IS 1893 (Part IV).		    (7)

24.	Design a simply supported square slab of 5m side length to support a service load of 4.5 kN/m2. Adopt M20 grade concrete and Fe 415 grade HYSD bars. Assume load factors according to the IS 456: 2000 standard code.

(OR)

25.	Design a rectangular slab 6.5 m x 4.5 m in size and simply supported along its edges is to be designed as an isotropically reinforced slab to support an uniformly distributed working live load of 4 kN/m2. Design the slab using yield line theory and adopting M20 grade concrete and Fe 415 HYSD bars.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	STRUCTURAL ANALYSIS-II							      Time : 3 hours

Subject Code:	09CE218				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the limitations of slope deflection method.

2.	Why is slope – deflection method called a ‘displacement method’?

3.	Define: Carry over moment and Carry over factor.

4.	Mention any four reasons due to which sway may occur in portal frames.

5.	Give the possible primary structures for the propped cantilever beam with explanation.

6.	A steel rod of length ‘l’, area of c/s ‘A’ and Young’s modulus ‘E’ is subjected to axial unit load. What is its axial flexibility?

7.	List the properties of stiffness matrix.

8.	For the portal frame fixed at the supports, which of these two methods is the best suit for the analysis - Flexibility method or Stiffness method? Why?

9.	What are the types of vibration?

10.	Define critical damping.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between Determinate Vs Indeterminate structures.

12.	Explain moment distribution theorem.

13.	Develop force transformation matrix for the structure shown in figure.

3m

3m

14.	Write about static condensation technique.

15.	A simply supported beam of span l with flexural rigidity EI is carrying a load W at the centre of the span. Compute the natural period and natural frequency.

[P.T.O.]

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the support moments for the continuous beam shown in figure by slope deflection method. Support B sinks by 10 mm. Take E = 200 GPa.  I= 16 X 107 mm4.

(OR)

17.	Analyze the frame shown in figure by slope deflection method. Draw BMD and SFD.

18.	Determine the support moments for the continuous beam shown in figure by moment distribution method and draw the BMD.

(OR)

19.	Analyze the frame shown in figure by Moment Distribution Method and draw the BMD.

20.	Analyze the continuous beam shown in figure by matrix flexibility method. Also draw the bending moment diagram.

(OR)

21.	Analyze the frame shown in figure by matrix flexibility method and draw BMD.

22.	Analyze the continuous beam ABCD shown in figure by Matrix stiffness method and then draw bending moment diagram. Take EI constant.

(OR)

23.	Analyze the portal frame shown in figure by matrix stiffness method and also draw bending moment and shear force diagram.

[P.T.O.]

24.	a.	A vertical cable 3 m long has a cross sectional area of 4 cm2 supports a weight of 50kN.

What will be the natural period and natural frequency of the system? E = 2.1 X 106 kg/cm2.

(10)

b.	A one kg mass is suspended by a spring having a stiffness of 1 N/mm. Determine the 			natural frequency. E = 2.1 X 106 kg/cm2 and I = 1300 cm4.                       			(5)

(OR)

25.	Derive the equation of motion for free vibration with viscus damping.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	FOUNDATION ENGINEERING							      Time : 3 hours

Subject Code:	09CE221				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is settlement?

2.	Define bearing capacity of a soil.

3.	Write any two types of boring.

4.	Name two Geophysical methods of testing.

5.	List any two types of footings.

6.	What are caissons?

7.	What are anchor piles?

8.	What is surcharge of soil?

9.	What is compaction?

10.	Define wet density.

PART – B  (5 x 3 = 15 MARKS)

11.  Explain Bulb of pressure.

12.	Discuss on wash boring.

13.	Write the classification of piles.

14.	Discuss on earth pressure on submergence of soils.

15.	Explain mechanical stabilization.

PART – C  (5 x 15 = 75 MARKS)

16.	Estimate the bearing capacity  of a 1.5m square footing in  a saturated clay deposit with SPT value of 20. The depth of foundation is 2m and unit weight of soil is 20KN/m3. Assume a suitable value of factor of safety

(OR)

17.	a.	Explain the various components of settlement.									  (10)

b.	Discuss the effects of water table on bearing capacity.								    (5)

18.	Explain in detail seismic refraction method of soil investigation with sketch.

(OR)

19.	Explain the standard penetration test with neat sketch.

20.	a.	A 300mm diameter pipe , 20m long pile  is driven by a 3.75ton hammer with a drop of 1 	meter. The total number of blows for 20m penetration is 550  and the test penetration 	reading  was  60 for 1m penetration. Estimate the capacity of the pile.

(OR)

21.	Explain the various factors to be considered while selecting a type of foundation.

22.	A Retaining  Wall 8m Height With A Smooth Vertical Back Retains. The Following Materials  Top 2m     clay, ٧s = 1.75KN/m3 Ǿ  = 300   c = 10KN/m2  Bottom 6m  saturated sand of           ٧s = 1.95KN/m3     Ǿ  = 300 .If the water table is on top of the sand layer , draw the diagram of lateral pressure on the wall , assuming that no tension crack develops on the top layer.

(OR)

[P.T.O.]

23.	A vertical wall with vertical back is 9m high.  The back fill has the following properties.          ٧m   = 16 KN/m3   Ǿ  = 300   c = 12KN/m2 ,  δ =  200.  Assuming a sliding plane 600 to the horizontal, find the thrust on the wall under the condition, neglecting cohesion along the wall and neglecting any tension cracks.

24.	 Enlist the various field compaction tests,  explain any two methods in detail.

(OR)

25.	What are the different methods of soil stabilization? Explain them with neat sketches.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	POLLUTION AND CONTROL ENGINEERING		      	     Time : 3 hours

Subject Code:	09CE237							 			      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the various sources of water pollution.

2.	State the effect of noise on human health.

3.	Enlist the types of industries that causes major air pollution.

4.	Explain photochemical air pollution.

5.	What is  recycling?

6.	Narrate the process of recovery from refuse.

7.	Define: Environmental quality.

8.	What are the parameters involved in EIA.

9.	Name any two vector borne diseases.

10.	What are the quality changes in milk due to infection?

PART – B  (5 x 3 = 15 MARKS)

11.	What is the use of Oxygen sag curve?

12.	What is the role of particulates in air pollution?

13.	Differentiate Incineration and pyrolysis.

14.	What are the impacts of urbanisation on environment?

15.	Narrate the principle behind food sanitation.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain in detail about the noise mapping.   										    (8)

b.	Classify the various water pollutants depending upon their sources.  				    (7)

(OR)

17.	What do you mean by agricultural pollution? How does it affect environment?

18.	Explain the salient features of the Kyoto treaty on Air pollution and prevention.

(OR)

19.	Discuss air pollution due to automobiles in metro cities with a case study of Chennai.

20.	Explain the principle behind sanitary land fill.

(OR)

21.	Discuss the various methods of hazardous waste management.

22.	Describe the various elements of Environmental Impact Analysis.

(OR)

23.	Explain the procedure involved in preparing EIA for thermal power plants.

24.	a.	Describe the rationale behind the methods of Vector control with a case study. 		    (7)

b.	Explain any two Engineering measures of vector control? 							    (8)

(OR)

25.	What is the significance of food sanitation in catering centres, kitchens and hotels?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title : 	CONSTRUCTION TECHNOLOGY						      Time : 3 hours

Subject Code:	09CE239										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define bearing capacity of soil.

2.	State the function of foundation.

3.	What is meant by stretcher bond?

4.	State the advantages of concrete hollow block masonry.

5.	State the functions of roof.

6.	State the use of gutter.

7.	State the need for weathering course.

8.	What are the causes for dampness?

9.	What is meant by underpinning?

10.	What is meant by shoring?

PART – B  (5 x 3 = 15 MARKS)

11.	Name the various methods of timbering of trenches.

12.	State the various factors that affect thickness of wall.

13.	Compare timber and concrete roof.

14.	What are the requirements of good stair?

15.	Compare framed and load bearing structures.

PART – C  (5 x 15 = 75 MARKS)

16. 	Write a detailed note on concepts of green building.

(OR)

17. 	Explain in detail various types of foundations and state under what circumstances each one is used.

18.  	With a neat sketch explain different types of stone masonry.

(OR)

19.	a. 	Compare stone and brick masonry.												    (6)

b. 	Write short note on appliances used for lifting stone.								    (9)

20. 	Write a detailed note on various roof covering materials commonly used.

(OR)

21. 	Write short notes on following:

a. 	Anti termite treatment.														    (7)

b. 	Any two types of floor.														    (8)

22. 	With neat sketch explain any two types of stairs.

(OR)

23.	Write short notes on following :

a. 	Types of pointing.					    	          								    (7)

b. 	Requirements of ideal material for damp proofing material.						    (8)

24. 	Explain in detail methods of construction of R.C.C. slabs, Beams and Columns.

(OR)

25. 	Write short notes on following:

a. 	Selection of equipment for earth work.		    	          							    (7)

b. 	Expansion joints for walls and roofs.			                     						    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	FLUID MECHANICS AND MACHINERY				      Time : 3 hours

Subject Code:	09CE242				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the capillarity rise?

2.	State Pascal’s law.

3.	What do you mean by ‘stream tube’?

4.	Write the continuity equation for three dimensional flow.

5.	What is Reynolds number for laminar flow through pipe?

6.	Name the minor losses of energy occurred in a pipe flow.

7.	Write the expression for force exerted by the jet of water on a stationary flat plate.

8.	What is the negative slip?

9.	What is the turbine?

10.	Write examples for reaction turbine.

PART – B  (5 x 3 = 15 MARKS)

11.	Define ‘viscosity’.

12.	What is the forced vortex?

13.	What do you mean by ‘pipes in series’?

14.	What is the cavitation?

15.	What is the specific speed of a turbine?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive the expression for surface tension in a water droplet.						    (7)

b.	Calculate the capillary effect in millimeters in a glass tube of 4mm diameter when immersed in (i) water and (ii) mercury. The temperature of the liquid is 20º C and the values of surface tension of water and mercury at 20º C in contact with air are 0.0735 N/m and 0.51 N/m respectively. The contact angle for water θ = 0º and for mercury θ = 130º. Take specific weight of water at 20º C as 9790 N/m3.								    (8)

(OR)

17.	Determine the total pressure on a circular plate of diameter 1m which is placed vertically in water in such a way that the centre of the plate is 2m below the free surface of water. Find the position of centre of pressure also.

18.	a.	Derive the continuity equation for a three dimensional flow.						  (10)

b.	A three dimensional flow field is given by V=2x2yi+3y2zj – (4xz+3yz2)k. Show that it is 	case of possible steady, incompressible fluid flow.								    (5)

(OR)

19.	Derive the Bernoulli’s equation for a stream line flow. Also, mention the assumptions made for its derivation.

[P.T.O.]

20.	A horizontal pipeline 40m long is connected to a water tank at one end and discharges freely 	into the atmosphere at the other end. For the first 25m of its length from the tank, the pipe is 	150mm diameter and its diameter is suddenly enlarged to 300mm. The height of water level in 	the tank is 8m above the centre of the pipe. Considering all losses of head which occur, 	determine the rate of flow. Take f = 0.01 for both sections of the pipe.

(OR)

21.	A horizontal venturimeter with inlet and throat diameters 30cm and 15cm respectively is used to measure the flow of water. The reading of differential manometer connected to the inlet and the throat is 20cm of mercury. Determine the rate of flow. Assume Cd as 0.98.

22.	Explain the working of Single acting reciprocating pump with neat sketch. Also obtain the expression for power required.

(OR)

23.	A centrifugal pump delivers water against a net head of 14.5metres and a design speed of  1000 rpm. The vanes are curved back to an angle of 30° with the periphery. The impeller diameter is 300mm and outlet width 50mm. Determine the discharge of the pump if manometric efficiency is 95%.

24. 	The following data is given for a Francis turbine. Net head H=60m; Speed N=700 rpm; Shaft power = 294.3 kW; overall efficiency = 84%; hydraulic efficiency = 93%; flow ratio = 0.20; breadth ratio n=0.1; Outer diameter of the runner = 2X inner diameter of runner. The thickness of vanes occupies 5% of circumferential area of the runner, velocity of flow is constant at inlet and outlet and discharge is radial at outlet. Determine: (a) Guide blade angle; (b) Runner vane angles at inlet and outlet; (c) Diameters of runner at inlet and outlet and (d)Width of wheel at inlet.

(OR)

25. 	A Pelton wheel is to be designed for the following specifications: Shaft Power = 11772 kW; Head = 380m; Speed = 750 rpm; Overall efficiency = 86%; Jet diameter is not to exceed one – sixth of the wheel diameter. Determine: (a) the wheel diameter, (b) the number of jets required and (c) diameter of the jet. Assume Kv1=0.985 and Ku1 = 0.45.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	MECHANICS OF SOLIDS								      Time : 3 hours

Subject Code:	09CE245				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Stress.

2.	Write the relationship between E and K.

3.	Define shear force.

4.	What is meant by sagging moment?

5.	Write down the simple bending equation.

6.	How springs are classified?

7.	What are methods used for analysis of stresses in two dimensions?

8.	Define Principal Plane.

9.	What do you mean by Effective Length?

10.	What is deflection of a beam?

PART – B  (5 x 3 = 15 MARKS)

11.	A rod 150 cm long and of diameter 2.0 cm is subjected to an axial pull of 20 kN.If the modulus of elasticity of the material of the rod is 2X105 N/mm2; determine: (a) the stress (b) the strain (c) elongation of the rod.

12.	Derive the relationship between bending moment and shear force.

13.	Write any four assumptions in the theory of simple bending.

14.	A bar is subjected to the uniform tensile stress of 125N/mm2. Find the intensity of normal, tangential and resultant stress on a plane normal to which is inclined 300 to the axis of the bar.

15.	State any three assumptions made in the Euler’s long column theory.

PART – C  (5 x 15 = 75 MARKS)

16.	A steel tube of 30mm external diameter and 20mm internal diameter encloses a copper rod of 15mm diameter to which it is rigidly joined at each end, if at temperature of 100C there is no longitudinal stress, calculate the stresses in the rod and tube when the temperature is raised to 2000C.Take E for steel and copper as 2.1X105 N/mm2 and 1X105 N/mm2. The value of co-efficient of linear expansion for steel and copper is given ass 11X10-6 per o C and 18X10-6 per0 C respectively.

(OR)

17.	A cylindrical shell of 500 mm diameter is required to withstand an internal pressure of 4 MPa. Find the minimum thickness of the shell, if maximum tensile strength for the plate material is 400 MPa and efficiency of the joints is 65%. Take factor of safety as 5.

18.	A cantilever beam of 2 m long carries a uniformly distributed load of 1.5 kN/m over a length of 1.6 m from the free end. Draw the shear force and bending moment diagrams for the beam.

(OR)

19.	A simply supported beam 6 m long is carrying a uniformly distributed load of 5 kN/m over a length of 3 m from the right end. Draw shear force and bending moment diagrams for the beam and also calculate the Maximum bending moment on the beam.

[P.T.O.]

20.	A close coiled helical spring has a stiffness of 5N/mm. its length when fully compressed with adjacent coils touching each other is 40 cm. the modulus of rigidity of the material of the spring is 8x104 N/mm2. Determine the wire diameter and mean coil diameter if their ratio is 1/10. What is the corresponding maximum shear stress in the spring?

(OR)

21.	A hollow shaft of diameter ratio 3/8 is required to transmit 600kW at 110 rpm, the maximum torque being 20% greater than the mean. The shear stress is not to exceed 63 MN/m2 and the twist in a length of 3m not to exceed 1.4 degrees. Calculate the maximum external diameter satisfying these conditions

22.	A rectangular block of material is subjected to a tensile stress of 110N/mm2 on one plane and a tensile stress of 47N/mm2 on the plane right angles to the former. Each of the above stresses is accompanied by a shear stress of 63N/mm2 and that associated with the former tensile stress tends to rotate the block anticlockwise. Find (i) the direction and magnitude of each principal stress and magnitude of greatest shear stress.

(OR)

23.	Using Mohr s circle method, determine the normal and tangential stresses across the oblique plane. Take major and minor principal stresses as 65 N/mm2  and 35 N/mm2,shear stress 25 N/mm2, oblique angle (θ=450). Check the answer analytically.

24.	A cantilever of length 2.5m is loaded with an udl of 10 kN/m over a length 1.5m from the fixed end. Determine the slope and deflection at the free end. Determine the slope and deflection at the free end of the cantilever L = 9500cm4, E = 210 GN / m2 Using Moment area method.

(OR)

25.	Derive an expression for critical load when both ends of the column are hinged.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June2015

Subject Title :	ENGINEERING MECHANICS							      Time : 3 hours

Subject Code:	09CE247				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the parallelogram law of forces

2.	What is free body diagram?

3.	State the Varignon’s theorem

4.	What is limiting friction?

5.	What is parallel axis theorem?

6.	Define: mass moment of inertia

7.	Give the mathematical definition of velocity and acceleration

8.	Define: angular velocity and angular acceleration

9.	What is inertia force?

10.	State the law conservation of energy

PART – B  (5 x 3 = 15 MARKS)

11.	The resultant of two forces is 14N when they act at 60. The same forces when they act at 90 produce a resultant of 12N.  Determine the magnitude of the two forces.

12.	State and explain  the laws of friction.

13.	State and explain the assumptions made in the analysis of perfect frames.

14.	The driver of a car travelling at 75kmph observes a traffic light 400m ahead turning red. The traffic light is timed to remain red for 30seconds before it turns green. If the motorist wishes to pass the signal without stopping find the required uniform retardation of the car and the speed with which the motorist should cross the signal.

15.	Distinguish between work , energy and moment.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	An electric light fixture weighing 18N hangs from a point C by two strings AC and BC  as 	shown in figure. AC is inclined with the vertical wall at 45o and BC is 	inclined with the 	horizontal ceiling at 60. Determine the forces in the strings.

[P.T.O.]

b.	A weight W = 1800N is suspended with chain AB 2m long. It is pulled by a horizontal force of 360N as shown in figure. Determine the force T in the chain.

(OR)

17.	A roller of radius 40cm weighing 4000N is to be pulled over a rectangular step of height 20cm by a horizontal force P applied at the centre of the roller. Find the value of P. What is the value of P and its direction in order to lift the roller over the step? P act at the same point.

18.	a.	Find the support reactions of a simply supported beam shown in figure.

b.	Four forces of magnitude and direction as shown in figure acts at the four corners of 	a square ABCD. Calculate the magnitude, direction and position of the resultant force.

(OR)

19.	A ladder of length 5m and weight 300N is placed against a vertical wall such that the inclination with vertical wall is 30o. A man weighing 750N climbs the ladder. At what position will the ladder slip? The coefficient of friction for both the contact surfaces is 0.2. If the bottom of the ladder is held by a horizontal string tied to the wall. What would be the tension in the string when the man is at the top of the ladder?

20.	Determine the forces in the members of the pin jointed truss shown in figure by method of joints.

(OR)

21.	Find the moment of inertia for an angle section shown in figure about its centroidal axes

22.	A burglar’s car had a start with an acceleration of 2m/s. A police in vigilant party came after 5seconds and continued to chase the burglar’s car with a uniform velocity of 20m/s. Find the time taken in which the police van will overtake the burglar’s car.

(OR)

23.	a.	A particle is projected with a velocity of 120m/s at an angle of 40o to the horizontal. Find 	the range, the maximum height attained and time of flight.

b.	An aero plane is flown horizontally at an altitude of 3km with uniform velocity of 750kmph. The pilot wishes to bomb a target in the ground. At what horizontal distance from the target should the bomb be released?

24.	A vehicle of weight 2500N is moving up a slope of 1 in 100 at a uniform speed of 40kmph. Find the power exerted by the engine if the frictional resistance is 5N per kN weight of vehicle

(OR)

25.	A ball of mass 60kg moving with a velocity of 5m/s collides directly with a stationery ball of mass 20kg. If the two balls get struck together after the impact, what is their common velocity? What is the loss of kinetic energy due to impact?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	SURVEY												      Time : 3 hours

Subject Code:	12CE201		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define leveling.

2.	Define Bench mark.

3.	Mention the Classification of Theodolites.

4.	What are the temporary adjustment of theodolite?

5.	List out the different methods of tacheometry.

6.	What is multiplying constant?

7.	What is GPS?

8.	Write the different types of curves.

9.	Write the use of Total station.

10.	What do you mean by triangulation?

PART – B  (5 x 3 = 15 MARKS)

11.	Write the Simpsons rule.

12.	What do you mean by closed traversing?

13.	Write the principle of stadia method.

14.	What are the instruments used for hydro graphic surveying?

15.	What are the uses of Total station?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the permanent adjustments of leveling

(OR)

17.	The following consecutive readings were taken along AB with a 4m leveling staff on 	continuously slowing ground at intervals of 20m:0.34m on A, 1.450, 2.630, 3.875, 0.655, 	1.745, 2.965, 3.945, 1.125, 2.475, 3.865 on B. The elevation A was 60.350. Enter the above 	readings in a level book form and work out RLs by rise and fall method. Also find the gradient 	of the line AB.

18.	Explain how horizontal angles are measured using repetition and reiteration method?

(OR)

19.	Explain with a neat sketch the components of a theodolite.

20. 	A theodolite has a tacheometric multiplying constant of 100 and an additive constant of zero. The centre reading on a vertical staff held at point B was 2.292 m when sighted from A. If the vertical angle was +25º and the horizontal distance AB 190.326 m. Calculate the other staff readings and show that the two intercept intervals are not equal. Using these values, calculate the level of B if A is 37.950 m angle of depression and the height of the instrument is 1.35 m.

(OR)

21. 	Explain the different between tangential and stadia tacheometry. How will you determine the stadia constants?

22. 	Explain the procedure of setting out simple curve by linear method.

(OR)

23.	Explain how GPS can be used for hydrographic surveying.

24.	Explain the various Baseline corrections.

(OR)

25.	Explain the usage of Total station in surveying.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	MECHANICS OF SOLIDS								      Time : 3 hours

Subject Code:	12CE202		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Poisson’s ratio.

2.	Define strain energy.

3.	What are principal planes?

4.	State the modes of failure of thin cylinders.

5.	Define Hogging bending moment.

6.	Identify a point of contra flexure.

7.	What is section modulus?

8.	Give the expression for shear stress in a beam section.

9.	What is Torsional rigidity?

10.	Sketch the shear stress distribution for a section subjected to Torque.

PART – B  (5 x 3 = 15 MARKS)

11.	Give the relationship between Youngs modulus and Rigidity modulus, Youngs modulus and Bulk modulus. Also give the relationship connecting all the three moduli.

12.	Explain the mohr’s circle method of calculation of principal stresses in a point subjected to like normal stresses in two perpendicular directions accompanied by a shear stress.

13.	Calculate the support reactions of a simply supported beam subjected to udl over the entire span along with a uniformly varying load over the entire span.

14.	Calculate the maximum bending stress in a beam section 200mm × 400mm subjected to a moment of 30KN-m.

15.	Find the moment of inertia and polar moment of inertia for a hollow circular section with inner diameter 0.8times outer diameter. Also find the polar modulus.

PART – C  (5 x 15 = 75 MARKS)

16.	A reinforced concrete column 450mm × 450mm has four steel rods 25 mm diameter embedded in it. Find the stresses in steel and concrete when the total load on the column is 1000 KN. Take Es = 205 KN/mm2 and Ec = 13.6 KN/mm2.

(OR)

17.	A bar of steel 20mm diameter was subjected to a tensile load of 25 KN and the measured extension on a 200mm gauge length was 7.8 × 10-3cm and the change in diameter was 2.32 × 10-4cm. Calculate the Poisson’s ratio and the values of the three moduli.

18.	At a point in an elastic material under strain, there are normal stresses of 50 N/mm2 and 30 N/mm2 respectively at right angles to each other with a shearing stress of 25 N/mm2. Find the principal stresses, position of principal planes and the maximum shear stress if,

a.	50 N/mm2 is tensile and 30 N/mm2 is also tensile.

b.	50 N/mm2 is tensile and 30 N/mm2 is compressive.

[P.T.O]

(OR)

19.	Calculate the increase in the volume enclosed by a boiler shell, 3 m long and 1 m in diameter, when it is subjected to an internal pressure of 1.5 N/mm2. The wall thickness is such that the maximum tensile stress in the shell is 29 N/mm2. Take E = 2.05 × 105 N/mm2 and (1/m) = 0.28.

20.	A beam ABC 11 m long is simply supported at A and B and has an overhang BC of 3m. It is loaded with udl of 4 KN/m over the whole length. A concentrated load of 30 KN is acting at 6m from A. Find the position and value of maximum bending moment and plot the B.M and S.F. diagrams.

(OR)

21.	A horizontal beam is simply supported at the ends and carries a distributed load of 30 KN/m between the supports placed 10 m apart. Counter clockwise moments of 300KN-m and   200KN-m are applied at the two ends of the beam. Draw the SF and BM diagrams showing the principal values.

22.	A timber joist of 4m span has to carry a total load of 80 KN uniformly distributed over the length. Calculate the dimensions of the joist section if the maximum permissible stress is limited to 8 N/mm2. The ratio of depth to width is to be 1.50.

(OR)

23.	A cross section of a joist is a Tee section 125 × 125 × 12 mm with 125cm side horizontal. Find the maximum intensity of shear stress and sketch the distribution of shear across the section if it has to resist a shear force of 90 KN.

24.	A shaft has to transmit 375 KW power at 80 rpm. The shaft must not be stressed beyond 75 N/mm2 and must not twist more than 1º in a length of 4m. Select a suitable diameter. Take rigidity modulus as 0.82 × 105 N/mm2.

(OR)

25.	Find the maximum permissible axial load for a closely coiled spring made out of 10mm diameter rod with 16 coils of 12cm mean diameter. If the maximum shear stress is limited to 100 N/mm2, calculate also the deflection under the load. Take N or C = 0.84 × 105 N/mm2.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ENGINEERING MECHANICS							      Time : 3 hours

Subject Code:	12CE203				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the law of transmissibility of force?

2.	What are the conditions of equilibrium of a system of coplanar concurrent forces?

3.	Differentiate between static and kinetic friction.

4.	Varignon’s theorem of moments states that the ____________.

5.	State perpendicular axis theorem.

6.	Define: Polar moment of inertia.

7.	Write the equation of motion of a projectile.

8.	What do you mean by curvilinear motion?

9.	State D’Alembert’s principle.

10.	Define: Coefficient of restitution.

PART – B  (5 x 3 = 15 MARKS)

11.	State and explain triangle law of forces.

12.	A body of weight 80N is placed on a rough floor. It requires a horizontal force of 45N to just cause sliding of the body on the floor. Determine the coefficient of friction.

13.	Determine the moment of inertia of a rectangular section about its base.

14.	A motor starts from rest and uniformly accelerated to speed of 20kmph over a distance of 200m. Calculate the acceleration and time taken.

15.	Distinguish between impulse and momentum. Are they related to each other?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	The resultant of two forces when they act at right angles is 10N. Whereas when they act at 	an angle of 50o  the resultant is 12N. Determine the magnitude of the two forces.

b.	Three forces of magnitude 50kN, 40kN and 25kN acting at a point, are inclined with x axis 	at 30o, 130o and 220o respectively. Determine the magnitude and direction of the resultant 	force.

(OR)

17.	Two spheres each of weight 800N and of radius 150mm rest in a horizontal channel of width 500mm as shown in figure. Find the reactions on the point of contact A, B  and C.

18.	a.	Determine the moment at point A of the force acting on the beam AB as shown in figure.

b.	A system of parallel forces acting on a bar ABCD as shown in figure. Find the resultant 	of the force system and reduce this system to a single force and couple at A.

(OR)

19.	A screw jack has square threads of mean diameter 8cm and a pitch of 0.8cm. The coefficient of friction is 0.1. A load of 5kN is to be lifted trough a height of 16cm.  Determine the torque required, work done and efficiency of the jack.

20.	Determine the moment of inertia of the tee section shown in figure about the centroidal axes.

(OR)

21.	Find the forces in all the members of the truss shown in figure by method of joints.

22.	a.	A particle is projected in air with a velocity of 160m/s and at an angle of 50o with 	horizontal. Find the time of flight, horizontal range and the maximum height reached by 	the particle.

b.	A shot is fired with a velocity of 50m/s from a point 20m in front of a vertical wall 6.5m 			high. Find the angle of projection to the horizontal to enable the shot just to clear the top of 		the wall.

(OR)

23.	a.	A car is driven at 70kmph on a curved road of radius 200m. Find the normal and tangential 	acceleration. The speed of the car is reduced to 40kmph in 10 seconds.  Find normal and 	tangential acceleration after breaking.

b.	A stone is dropped from the top of a tower. During the last one second of its travel the 			stone covers 3/5 of height of tower. What is the height of the tower?

24.	Block A of mass is 15kg and block B of mass 40kg are connected by a weightless string that passes over a frictionless pulley. Assuming coefficient of friction as 0.25, determine the velocity of the system two seconds after the system is set in motion. What is the string tension at this instant?

(OR)

25.	A ball of mass 40kg moving with a velocity of 8m/s strikes directly another ball of mass 30kg moving in the opposite direction with a velocity of 10m/s. if the coefficient  of restitution is 0.75, what is the velocity of each ball after impact.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	MECHANICS OF FLUIDS								      Time : 3 hours

Subject Code:	12CE205				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the relationship between mass density and weight density.

2.	What are the equipments used for pressure measurement?

3.	What do you mean by ‘path line’?

4.	Write the Euler’s equation.

5.	What is Reynolds number for turbulent flow through pipe?

6.	Name the minor losses of energy occurred in a pipe flow.

7.	What is the Pitot tube?

8.	What is the open channel flow?

9.	What are the types of similarity?

10.	Name any two dimensionless numbers.

PART – B  (5 x 3 = 15 MARKS)

11.	Define ‘Total pressure and centre of pressure’.

12.	What is the velocity potential function?

13.	Define Water hammer.

14.	Write the classification of weirs.

15. 	What is meant by scale effect?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Determine the specific gravity of a fluid having viscosity 0.05 Poise and kinematic 	viscosity 0.035 Stokes.														(5)

b.	A 400 mm diameter shaft is rotating at 200 rpm in a bearing of length 120mm. If the thickness of oil film is 1.5mm and the dynamic viscosity of the oil is 0.7 Ns/m2. Determine the torque required to overcome friction in bearing and power utilized in overcoming viscous resistance. Assume a linear velocity profile.								  (10)

(OR)

17.	Determine the total pressure on a circular plate of diameter 1.5m which is placed vertically in water in such a way that the centre of the plate is 3m below the free surface of water. Find the position of centre of pressure also.

a.	Compare Steady and Unsteady flows.											    (3)

b.	Prove that a two dimensional flow in x-y plane represented by equations u = 3x and v=3y is possible. Obtain the equation for stream function and velocity potential function.	  (12)

(OR)

18.	Derive the Bernoulli’s equation for a stream line flow. Also, mention the assumptions made for its derivation.

19.	Derive the Darcy - Weisbach equation for turbulent flow through pipe.

(OR)

[P.T.O.]

20.	Determine the rate of flow of water through a pipe of diameter 20cm and length 50m when one end of the pipe is connected to a tank and other end of the pipe is open to the atmosphere. The pipe is horizontal and the height of water in the tank is 4m above the centre of the pipe. Consider all minor losses and take f = 0.009 in Darcy-Weisbach equation.

21.	a.	An oil of specific gravity 0.8 is flowing through a venturimeter having inlet diameter 20cm 	and throat diameter 10cm. The oil-mercury differential manometer shows a reading of 	25cm. Calculate the discharge of oil through the horizontal venturimeter. Take coefficient 	of discharge Cd = 0.98.														    (8)

b.	A rectangular channel 2m wide has a discharge of 250litres per second, which is measured by a right angled V-notch weir. Find the position of the apex of the notch from the bed of the channel is maximum depth of water is not to exceed 1.3m. Take coefficient of discharge Cd = 0.62.															    (7)

22.	a.	A rectangular orifice of 2m width and 1.2m deep is fitted in one side of a large tank. The 	water level on one side of the orifice is 3m above the top edge of the orifice, while on the 	other side of the orifice, the water level is 0.5m below its top edge. Calculate the discharge 	through the orifice if coefficient of discharge Cd = 0.64.							    (9)

b.	A pitot tube is inserted in a pipe of 300mm diameter. The static pressure in pipe is 100mm of mercury (vacuum). The Stagnation pressure at the centre of the pipe, recorded by the Pitot tube is 0.981N/cm2. Calculate the rate of flow of water through pipe, if the mean velocity of flow is 0.85 times the central velocity. Take coefficient velocity Cv = 0.98.																				    (6)

(OR)

23.	A solid circular shaft transmits 75kW power at 200rpm. Calculate the shaft diameter, if the twist in the shaft is not to exceed 10 in 2m length of shaft, and shear stress is limited to 50MN/m2. Take C=100GN/m2.

24. 	The resistance ‘R’ experienced by a partially, submerged body depends upon the velocity ‘V’, length of the body ‘L’, mass density ρ, viscosity of fluid ‘µ’ and gravitational acceleration ‘g’. Obtain a dimensionless expression for R.

(OR)

25. 	Derive an expression showing the relationship between the torque and the variables of diameter, rotational speed, viscosity and density by Buckingham’s Pi Theorem.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ADVANCED STRENGTH OF MATERIALS			      	     Time : 3 hours

Subject Code:	12CE210						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	A simply supported beam of span l is carrying point load W at the mid span. What is

the deflection at the center of the beam?

2. 	The amount of deflection of a beam is subjected to some type of loading depends upon __________.

3. 	When the strut is vertical i.e. inclined at 90o to the horizontal is known as _________ .

4. 	What is Slenderness ratio (k)?

5. 	State Lame’s theory.

6. 	In case of a closed ends thick cylinder, the longitudinal stress is given by _________.

7. 	The theory of curved beam was postulated by __________.

8. 	State any one assumption made in the analysis of curved beam.

9. 	In the case of unsymmetrical bending, the direction of neutral axis is ________ .

10. 	In an  I section, symmetrical about XX and YY axes, shear center lies at _________.

PART – B  (5 x 3 = 15 MARKS)

11. 	A girder of uniform section and constant depth is freely supported over a span of 3 meters. If the point load at the mid span is 30 kN and IXX = 15.614 x 10-6 m4, calculate the central deflection. Take E = 200 GN/m2.

12. 	State the assumptions made while deriving the Euler’s formula.

13. 	Explain the Maximum shear stress theory.

14. 	State the values of h2 for rectangular, circular and triangular sections.

15. 	Give the reason for unsymmetrical bending.

PART – C  (5 x 15 = 75 MARKS)

16.	By double integration method, derive the expression Yc = 5Wl4/ 384EI for a simply supported beam with a uniform distributed load.

(OR)

17.	Using conjugate beam method find the mid span deflection of the beam shown in the figure.

E = 200 x 106 kN/m2 and I = 200 x 10-4 m4.

[P.T.O.]

18.	A column is made up of two rolled steel joints of I – section, 16 cm x 8 cm x 1 cm thick with plate 20 cm x 1 cm riveted with flanges one each on the top and on the bottom. The edges of the plates being flush with the outside edges of joists flanges. Determine, by Rankine’s formula the safe load the column of 4 m length, with both ends fixed, can carry with factor of safety 3. Take a 1/7500 and σc = 320 MN/m2.

(OR)

19.	A built –up beam shown in figure is simply supported at its ends. Compute its length, given that when it is subjected to a load of 40 kN per meter length, it deflects by 1 cm, find out the safe load, if this beam is used with both sides fixed. Assume a factor of safety of 4. Euler’s formula. E = 210 GN/m2.

20.	A solid shaft transmits 1000 kW at 300 r.p.m. Maximum torque is 2 times the mean. The shaft is subjected to a bending moment which is 1.5 times the mean torque. The shaft is made of ductile material for which the permissible tensile and shear stresses are 120 MPa and 60 MPa respectively. Determine the shaft diameter using a suitable theory of failure. Give justification for the theory used.

(OR)

21.	A thick cylinder of 150 mm outside radius and 100 mm inside radius is subjected to an external pressure of 30 M/Nm2 and internal pressure of 60 M/Nm2. Calculate the maximum shear stress in the material of the cylinder at the inner radius.

22.	A curved bar is formed of a tube of 120 mm outside diameter and 7.5 mm thickness. The center line of this beam is a circular arc of radius 225 mm. A bending moment of 3 kNm tending to increase curvature of the bar is applied. Calculate the maximum tensile and compressive stresses set up in the bar.

(OR)

23.	A circular horizontal section of hook is a symmetrical trapezium 60 mm deep, the inner width being 60 mm and the outer being 30 mm. Estimate the extreme intensities of stress when the hook carries a load of 30 kN, the load line passing 40 mm from the inside edge of the section and the center of curvature being in the load line. Also plot the stress distribution across the section.

24.	A 40 mm x 40 mm x 5 mm angle is used as a simply supported beam over a span of 2.4 meters. It carries a load of 200 N along the vertical axis passing through the centroid of the section. Determine the resulting bending stresses on the outer corners of the section, along the middle of the beam.

(OR)

25.	A cantilever of rectangular section 40 mm (width) x 60 mm (depth) is subjected to an inclined load P at the free end. The inclination of the load is 25o to the vertical. If the length of the cantilever is 2 meters and maximum stress due to bending is not to exceed 200 MN/m2, determine the value of P.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	SOIL MECHANICS										      Time : 3 hours

Subject Code:	12CE211									       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Specific Gravity.

2.	What are the methods by which water content of soil is determined?

3.	Define Liquid Limit.

4.	What do you mean by Consistency of soil?

5.	Define Darcy’s law.

6.	What is Pore Water?

7.	State the Components of Settlement.

8.	What is Contact Pressure?

9.	What are Shear Strength Parameters?

10.	Define Effective Stress.

PART – B  (5 x 3 = 15 MARKS)

11.	The natural water content of a saturated soil is 30%. If the specific gravity of solids of the soil is 2.7. Find its porosity.

12.	Give reasons why Classification of Soils is required?

13.	Write short notes on Quick Sand Condition.

14.	Differentiate between Compaction and Consolidation.

15.	Mention the important Characteristics of Mohr’s circle.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A wet sample of mass 1.9 Kg has volume of 945cm3 after over drying its mass reduced to 	1.7kg. The specific gravity of solids was found to be 2.7. Determine a)Moisture content        	(b) 	bulk density (c) dry density (d) void ratio (e) porosity (f) degree of saturation. 	    (8)

b.	Derive the relationship between γd = γ/1+w from fundamental principle.                          (9)

(OR)

17.	Explain with neat sketches core cutter and sand replacement methods in detail. 	                      18.	Explain in detail about hydrometer analysis for determining sizes of particle in a soil sample.

(OR)

19.	Explain in detail about various Soil Classification System.

20.	The discharge of water collected from a constant head permeameter in a period of 15 minutes is 500 ml. The internal diameter of the permeameter is 5 cm and the measured difference in head between two gauging points 15 cm vertically apart is 40 cm. Calculate the coefficient of permeability. If the dry weight of 15 cm long sample is 4.86N and specific gravity of the solids is 2.65, calculate the seepage velocity.

(OR)

21.	What is a flow net? Describe its properties and applications. Describe different methods used to construct the flow net.

[P.T.O.]

22.	A saturated soil has a compression index of 0.25.Its void ratio at a stress of 10 kN/m2 is 2.02 and its permeability is 3.4x10-7mm/s. Compute: (a) Change in void ratio if the stress is increased to 19 kN/m2 (b) settlement in if the soil stratum is 5m thick and (c) time required for 40% consolidation if drainage is one way.

(OR)

23.	In a laboratory the consolidation test was performed on a specimen of clay 3cm thick. The sample was drained at top and bottom. The time requires for 40% consolidation of the sample was observed to be 15minutes. Determine the co efficient of consolidation of clay.

24.	Triaxial compression test on a cohesive soil sample of cylindrical shape yielded the following results:

Minor Principal stress-120kN/m2

Major Principal stress-230kN/m2

If the angle of inclination of the rupture plane to the horizontal is 60o, determine the cohesion and angle of internal friction by drawing the Mohr circle or by calculation.

(OR)

25.	Explain in detail the different shear tests based on drainage conditions.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	STRUCTURAL ANALYSIS								      Time : 3 hours

Subject Code:	12CE214				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Degrees of freedom.

2.	Write Maxwell Reciprocal theorem.

3.	What is meant by influence lines?

4.	What are the uses of influence line diagram?

5.	Which of the two arches, viz. circular and parabolic is preferable to carry a uniformly distributed load? Why?

6.	Distinguish between two hinged and three hinged arches.

7.	Define a space frame and what is the nature of joint provided in the space trusses?

8.	What are the main functions of stiffening girders in suspension bridges?

9.	Compare force and displacement method.

10.	Write the application of consistent deformation method.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain Principle of superposition with example.

12.	Sketch the influence line for reaction at the right support of a simply supported beam of span    8 m and hence determine the value of the maximum reaction when an udl of 10 kN/m and of length 6m crosses the beam.

13.	A 3 hinged parabolic arch of span 30 m has a central rise of 5 m. Find the increase in the rise of the arch if the temperature rises through 30oC. Take coefficient of linear expansion of the arch material as 12 X 10-6 /0C.

14.	Define tension coefficient of a truss element. What is the tension coefficient for member of 3m length carrying 10 kN compressive axial force?

15.	A cantilever of uniform flexural stiffness is propped at the remote end. Find the load on the prop when a load W is applied at the centre of the cantilever.

PART – C  (5 x 15 = 75 MARKS)

16.	Using the principle of virtual work determine the Vertical and Horizontal deflection at ‘C’ of the truss shown in figure. Cross sectional area of all members = 100 mm2 E = 200 GPa.

[P.T.O.]

(OR)

17.	Find the horizontal deflection and rotation at joint C of the frame shown in figure. E = 200 GPa.  I= 40 X 106 mm4.

18.	Draw the influence lines for bending moment at B for the continuous beam shown in figure. Also find the value of shear force for the given loading system using influence lines.

(OR)

19.	Draw Influence Line Diagram (ILD) for shear force and bending moment for a section at 6m from the left hand support of a simply supported beam, 22m long. Hence, calculate the maximum bending moment and shear force at the section due to a uniformly distributed rolling load of length 7 m and intensity 15 kN/m run. Also calculate absolute BM.

20.	A two hinged parabolic arch of span 18m and rise of 3.6m carry a concentrated load of 25kN at the left quarter span section. Find

a.	The horizontal thrust at each support.

b.	The Max (+ve) and Max (-ve) BM and

c.	Draw BMD.

(OR)

21.	The three hinged parabolic arch has a span of 40m and a rise of 8m carries a uniformly distributed load of 30kN/m on the right half of its span. It also carries two concentrated loads of 100kN and 150kN at 8m and 16m from the left end. Determine the following

a.	The support reactions at the springing.

b.	Normal thrust and radial shear at 10m from the left support.

22.	A suspension cable is supported at two point "A" and "B”, "A" being one meter above "B". The distance AB being 20m. The cable is subjected to 4 loads of 2kN, 4kN, 5kN and 3kN at distances of 4m, 8m, 12m and 16m respectively from "A". Find the maximum tension in the cable, if the dip of the cable at point of application of first load is 1m with respect to level at A. Find also the length of the cable.

(OR)

23.	Analyze the space truss shown in figure by tension coefficient method.

24.	A beam AB of span 4m is fixed at A and B and carries a point load of 5kN at a distance of 1m from end A. Calculate the support moments by the method of consistent deformation.

(OR)

25.	A continuous beam ABCD 16m long is continuous over three spans: AB = 6m, BC = 5m, and CD = 5m, the supports being at the same level. There is an udl of 20kN/m over BC. On AB, there is a point load of 80kN at 2m from A. On CD, there is a point load of 60kN at 3m from D. Calculate the moments and reactions at the supports using three moment equation method.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	HIGHWAY ENGINEERING						      	     Time : 3 hours

Subject Code:	12CE215										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Select the correct statement

a.	Nagpur road plan formula take into account the towns with very large population

b.    Nagpur road plan has a target road length of 32 km per 100 square km.

c.	Second 20-year plan has provided 1600 km of expressways out of the proposed National 	highway.

d.	Second 20-year plan allowed deduction of length of railway track in the area while calculating the length of roads.

2.	The standardization and recommending specifications for roads and bridges is done by _________.

3.	Camber in the road is provided for

a.	effective drainage b. counteracting the centrifugal force c. having proper sight distance d. none of 	the above

4.	On a single lane road with two way traffic, the minimum stopping sight distance is equal to  _________ the stopping distance

5.	Flexible pavement distribute the wheel load

a.	directly to subgrade 						b.	through structural action

c.	through a set of layers to the subgrade 		d.	none of the above

6.	The critical combination of stresses for corner region in cement concrete roads is __________.

7.	Which of the following tests measures the toughness of road aggregates ?

a.	crushing strength test 		b.	abrasion test

c.	impact test 				d.	shape test

8.	Penetration test on bitumen is used for determining its __________.

9.	Scaling of cement concrete in rigid pavements is mainly due to __________.

10.	The overlay combinations are divided into __________ categories.

PART – B  (5 x 3 = 15 MARKS)

11.	List the functions of MoRTH.

12.	What are the factors governing the super elevation of a road surface?

13.	What is meant by Vehicle Damage Factor in IRC method of flexible pavement design?

14.	State the desirable properties of bitumen.

15.	Explain the remedies for mud pumping.

PART – C  (5 x 15 = 75 MARKS)

16.	Briefly highlight the third twenty year road development plan in India.

(OR)

17.	Explain the various stages of Transportation planning.

18.	a.	Brief the elements of highway cross section.	                                                                    (9)

b.	Explain the hairpin bends with neat sketch.                                                                       (6)

[P.T.O.]

(OR)

19.	a.	Explain why the width of pavements are to be increased on curves  and how it is 	determined	                                                                                                                        (7)

b.	Calculate the SSD for design speed of 70kmph for two and one way traffic road. Take  	reaction time 2.5 seconds  and  f=0.35.                         							    (8)

20.	a.	What are the components of rigid, flexible pavements? Explain their role.               	    (8)

b.	What are the factors to be considered in design of pavements?		                	  	    (7)

(OR)

21.  	Explain in detail about the IRC recommendations for design of flexible pavements.

22. 	Describe the procedure recommended by Bureau of Indian standards for carrying out the

following tests.

a.	Abrasion test  		b.	CBR  test

(OR)

23. 	a.	Explain the construction procedure for concrete roads.                                                	    (7)

b.	Explain the construction procedure of the Dense Bituminous Macadam road.          	    (8)

24.  Briefly explain the causes and remedies for various defects in flexible pavements.

(OR)

25. 	a.	Explain the typical rigid pavement failures.                                                                	    (8)

b.	Explain any one method for evaluating existing pavement.                                               (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	COMPUTATIONAL METHODS					       	   Time : 3 hours

Subject Code:	12CE216									             Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is the condition for applying iteration method to find the root of the equation f(x)=0?

2. 	What is the interval of the root of the equationex−3x=0 ?

3. 	The order of convergence of Newton Raphson method is ________.

4. 	Define Simpson’s three-eighth rule.

5.	The normal equations for fitting a straight line by the method of least squares are ________ and ________.

6.	Write down the Lagrange’s interpolation formula.

7.	Write down second order Runge Kutta method formula.

8.	Write down Milne’s predictor formula to predict the value of y at the given point of x.

9. 	Write down the standard five point formula.

10. 	Identify the equation fxx+2fxy+fyy=0.

PART – B  (5 x 3 = 15 MARKS)

11. 	Find the Eigen Value of the matrix 222−1 using Jacobi iterations.

12 . 	Using Newton-Raphson method establish the formula xn+1=xn2+N2xn to calculate the square

root of N.

13. 	Using Lagrange’s interpolation formula find y(x=2) from the table given below

X:	0		1		3		4

Y:	5		6		50		105

14. 	Compute y at x=0.25 by Modified Euler’s method given y'=2xy,y(0)=1 .

15.	Classify the partial differential equation (x+1)uxx−2(x+2)uxy+(x+3)uyy=0.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Solve the following equations3x+y−z=3;2x−8y+z=−5;x−2y+9z=8 by Gauss

Elimination Method.															  (10)

b.	Write down the working rule for finding the Eigen values of a 3x3 matrix using power

method.	     																	    (5)

(OR)

17.	Find the Eigen values and Eigen vectors for the given matrix using sweeping techniques161120003.

[P.T.O.]

18.	a.	Solve x3+3x2−4=0 by Graeffe’s root square method.							    (8)

b.	Solve x3=6x−4 by Newton’s Raphson method. 								    (7)

(OR)

19.	a.	Evaluate 06dx1+x2 by (a)Trapezoidal rule (b)Simpson’s one third rule (c)Simpson’s three

Eighth rule. (h=1).															  (10)

b.	Evaluate −22e−x2dx by Gauss two point formula.									   (5)

20.	From the following table find tan0.12 and tan0.26 using Newton’s interpolation formula.

X   		:   0.10		0.15			0.20			0.25			0.30

Tan x	: 0.1003		0.1511		0.2027		0.2553		0.3093

(OR)

21.	Use the method of least squares and fit a straight line to the following data and hence find y(x=25).

X:	0		5		10		15		20

Y:	7		11		16		20		26

22.	Compute y(0.3) given dydx+y+xy2=0,y(0)=1 by taking h=0.1 using fourth order Range

Kutta method.

(OR)

23.	Compute y(0.4)  using Adam’s Predictor and Corrector method for the differential equation y'=x2−y,y(0)=1.Use Taylor’s series to find y(0.1),y(0.2) and y(0.3).

24.	Solve ut=uxx subject to u(0,t)=0,u(1,t)=0 and u(x,0)=sinπx,0<x<1.

(OR)

25.	Using Crank-Nicholson’s scheme solve uxx=16ut,0<x<1,t>0 given

u(x,0)=0,u(0,t)=0,u(1,t)=100t.Compute u in t direction taking h=1/4.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	REINFORCED CONCRETE STRUCTURES				      Time : 3 hours

Subject Code:	12CE217				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What do you mean by equivalent concrete area of a reinforced concrete member?

2. 	What are the limit states that are considered in Limit State Method?

3. 	Distinguish between one way and two way slabs.

4. 	Name the structural components of staircase.

5. 	Find the depth of neutral axis in terms of  ‘d’ for a balanced section using Fe 415 steel,  in limit state method.

6. 	Under what circumstances are doubly reinforced beams used?

7. 	Compare the behavior of tied and spirally reinforced column.

8.	How do you classify one-way footing and two-way footing in foundation?

9.	What are the stability conditions that should be checked for retaining walls?

10. 	What are the components to design the small over head circular tanks?

PART – B  (5 x 3 = 15 MARKS)

11. 	State the assumptions made in working stress method.

12.	Draw the various types of Stair Cases? And explain its components.

13. 	When shear reinforcement is necessary in a beam? Explain.

14. 	Sketch the placement of steel in rectangular footing with a non-central load.

15. 	Define ring beams and mention its advantages.

PART – C  (5 x 15 = 75 MARKS)

16. 	A beam of reinforced concrete is 300mm wide and 450mm deep to the centre of tensile steel. It is reinforced with four bars of 16mm diameter as compressive steel and four bars of 25mm diameter as tensile steel. Determine the moment of resistance of the section. Cover to centre of compression steel is 50mm. Use  M20 concrete and Fe415 steel.

(OR)

17. 	A rectangular beam reinforced on both sides is 300mm wide and 500mm deep. The centres of steel are 50mm from the respective edges. If the limiting stresses in concrete and steel are 7N/mm2 and 230N/mm2 respectively. Determine the steel areas for a bending moment of 90kNm based on the revised elastic theory. Take m=13.33

18. 	Design a simply supported slab supported on masonry walls to the following requirements

(a) Clear span-3m

(b) Live load-2500N/m2

Use M20 concrete and Fe415 steel

(OR)

[P.T.O.]

19. 	Design one of the flights of a dog legged stairs between the landing beams using the following data:

Type of staircase: Dog legged staircase with waist slab, treads and risers

No. of steps in the flight: 10

Thread T = 300mm

Rise R = 150mm

Width of landing beams = 300mm

Grade of concrete = M20

Grade of steel = Fe415.

The general arrangement of stair case in an multistory housing complex: The risers are 150mm and treads are 250mm. The stair case is embedded into the wall by 200mm. The height between the floors is 3m. The service live load is 3 KN/m2. Adopt Grade of concrete = M20 & Grade of steel = Fe415.Design the staircase flight and draw a longitudinal section showing the details of reinforcements in the flight of the staircase.

20. 	A rectangular beam width b = 250mm and effective depth 500mm reinforced with 4 bars of 20mm diameter. Determine the shear reinforcement required to resist a shear force of 150kN. Use concreteM20 and steel Fe415.

(OR)

21. 	A rectangular RCC beam is 400x900mm in size. Assuming the use of grade M25 concrete and Fe415steel, determine the maximum ultimate torsional moment at the section can take it.(i) No torsion reinforcement is provided and(ii) Maximum torsion reinforcement is provided.

22. 	An axially loaded RCC column of unsupported length 2.75m has to carry an axial load of 2000kN.The effective lengths of the column corresponding to the long and short directions are 2.5m and 2.35m respectively. Design the column as a square section and as a rectangular section. Adopt M20 grade of concrete and Fe 415 HYSD bars.

(OR)

23. 	Design a reinforced concrete rectangular combined footing for two colums located 4m between centres. Each column is 400mm x400mm.Each column carries an axial load is 1200kN.The projections of the footing parallel to the length beyond the axis of each column is 1m. The limiting bearing capacity of the soil is 440kN/m2. Adopt M20 grade of concrete and Fe 415 HYSD bars.

24. 	A rectangular RC water tank with an open top is required to store 80,000 liters of water. The inside dimensions of tank may be taken as 6m * 4m. The tank rests on walls on all four sides. Design the side walls of the tank using M20 concrete and grade I steel.

(OR)

25. 	Design a cantilever retaining wall to retain earth embankment 4m high above G.L. The density of earth is 18KN / m3 and its angle of repose is 30 degrees. The embankment is horizontal at its top. The safe bearing capacity of the oil may be taken as 200KN / m2 and the co-efficient of friction between the soil and concrete is 0.5. Adopt M20 grade of concrete and Fe 415 HYSD bars.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	FOUNDATION ENGINEERING							      Time : 3 hours

Subject Code:	12CE219				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define bearing capacity.

2.	What is meant by differential settlement?

3.	State the advantages of percussion drilling.

4.	Classify geophysical methods.

5.	Define negative skin friction.

6.	Under what circumstances combined footings are used?

7.	State the functions of retaining wall.

8.	What is meant by sheet pile?

9.	What is meant by stability number?

10.	Define stabilization.

PART – B  (5 x 3 = 15 MARKS)

11.	A concentrated load of 200t is applied at the ground surface. Determine the vertical stress at a point P which is 6m directly below the ground.

12.	What are the factors that affect sample disturbance?

13.	Classify piles based on different criteria.

14.	Differentiate between active and passive earth pressure.

15.	State the functions of geotextiles.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the basis of construction of Newmarks influence chart. How it is used?

(OR)

17.	Explain in detail plate bearing test.

18.	Explain in detail standard penetration test.

(OR)

19.	Write a detailed note on augur boring and wash boring.

20.	Design a rectangular combined footing to support two adjacent columns of size 40cm X 40 cm at a distance of 5m and carrying loads of 300t and 400t. The lighter column is near the property line. The allowable soil pressure is 40t/m2.

(OR)

21. 	Write a detailed note on various types of foundation and choice of foundation.

22.	a.   	State the assumptions made in Rankines theory.     								    (5)

b.	 A retaining wall with a vertical back of height 7.32m supports a cohesionless soil of unit weight 17.3 kN/m3 and angle of shearing resistance = 30o . The surface of soil is horizontal. Determine the magnitude and direction of active thrust. 					  (10)

(OR)

23. 	Explain coulombs theory of earth pressure clearly stating the assumptions that are taken into account.

24. 	Explain in detail any three methods of soil stabilization.

(OR)

25.	Explain the friction circle method for the stability analysis of slopes.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	DESIGN OF STEEL STRUCTURES						      Time : 3 hours

Subject Code:	12CE223		 							       	       Maximum Marks: 100

#### Answer ALL questions

Use of IS 800, Steel Tables are permitted

PART – A  (10 x 1 = 10 MARKS)

1.	List the different channel sections.

2.	Define fasteners.

3.	Define tension splice.

4.	Define lacing.

5.	Define built up section.

6.	What is laterally unrestrained beam?

7.	What is a girder?

8.	What is a column base?

9.	Define wind load.

10.	Name the method of analysis of trusses.

PART – B  (5 x 3 = 15 MARKS)

11.	State the difference between working stress method and limit state method of design

12.	How the design of compression member will be carried out?

13.	Write the design steps of beam column.

14.	How the thickness of column base will be calculated?

15.	List the different types of roof trusses with neat sketches

PART – C  (5 x 15 = 75 MARKS)

16.	Calculate number of bolts required to transfer a factored axial tensile force of 200 kN through a bolted connection using 12 mm bolts in single shear with plate thickness of 8 mm.

(OR)

17.	Explain the design procedure of eccentric welded connection

18.	Design a tension member to carry a load of 300 kN. The two angles placed back to back with long leg outstanding are desirable. The length of the member is 2.9 m

(OR)

19.	Design a built up column 6m long to carry a factored axial load of 750 kN. The column is 	restrained in position but not in direction at both the ends.

20.	Design a laterally unsupported beam of effective span 6m to carry a UDL of 25kN/m. Assume all other relevant data.

(OR)

21.	Explain the design procedure of laterally restrained beam.

22.	Explain the design procedure of welded plate girder.

(OR)

23.	A Gantry crane exerts a load of 100 kN on each of its wheels, excluding impact and other 	loads, the wheel distance is 5 m. The span of the gantry is 16m. Design the girder assuming it 	has lateral support.

24.	Design a channel purlin section for the following data.

Span of the truss = 12m					Spacing of truss = 4m c/c

Spacing of purlin along slope = 1.8m		Pitch of roof = 30°

Assume all other relevant data.

(OR)

25.	Explain the design procedure of design of roof trusses.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ADVANCED STRUCTURAL ANALYSIS			       	     Time : 3 hours

Subject Code:	12CE224							 			      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How many slope deflection equations are available for a two span continuous beam?

2.	Write the fixed end moments for a beam carrying a central clockwise moment.

3.	What is relative stiffness?

4.	What are the moments induced in a beam member, when one end is given a unit rotation, the other end being fixed. What is the moment at the near end called?

5.	What is the indeterminacy for a propped cantilever beam?

6.	What are the conditions of equilibrium?

7.	Write the expression for displacement transformation matrix.

8.	What are the basic unknowns in stiffness method?

9.	What is a substitute frame?

10.	What is collapse load?

PART – B  (5 x 3 = 15 MARKS)

11.	Write down the general slope-deflection equations and state what each term represents.

12.	A beam is fixed at A and simply supported at B and C. AB = BC = l. Flexural rigidities of AB and BC are 2EI and EI respectively. Find the distribution factors at joint B if no moment is to be transferred to support C.

13.	Differentiate between determinate and indeterminate structures.

14.	Compare flexibility method and stiffness method of analysis.

15.	Define shape factor and write the formula.

PART – C  (5 x 15 = 75 MARKS)

16.	Analyze the continuous beam shown in figure by slope deflection method and draw bending moment diagram.

(OR)

[P.T.O.]

17.	Analyze the frame shown in figure by slope deflection method and draw bending moment diagram.

18.	Analyze the continuous beam shown in figure by moment distribution method and draw bending moment diagram.

(OR)

19.	Analyze the frame shown in figure by moment distribution method and draw BMD.

20.	Analyze the continuous beam shown in figure by matrix Flexibility method and draw BMD.

(OR)

21.	a.	Explain the characteristics of flexibility and stiffness method.						    (7)

b.	Explain the formulation of structure flexibility method.							    (7)

22.	a.	Explain Static condensation technique.											    (7)

b.	Explain the formulation of structure stiffness method.								    (7)

(OR)

23.	Analyze the frame shown in figure by matrix Stiffness method and draw BMD.

24.	a.	Explain the problems in Cantilever method and portal frame method.				    (7)

b.	Explain the step by step procedure for by substitute frame method of analysis.		    (7)

(OR)

25.	A continuous beam ABC is loaded as shown in figure. Determine the required MP if the load factor is 3.2.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	SANITARY ENGINEERING							      Time : 3 hours

Subject Code:	12CE226		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define sewage.

2.	Name any two sources of waste water.

3.	Define sewer.

4.	What is meant by sewer appurtances?

5.	State the use of skimming tank.

6.	State the use of grit chamber.

7.	What is meant by sewage farming?

8.	What is meant by self purification?

9.	State the use of soak pit.

10.	Name any two sanitary fittings.

PART – B  (5 x 3 = 15 MARKS)

11.	State the objectives of public sewerage system.

12.	Compare conservancy and water carriage system.

13.	State the advantages of oxidation pond.

14.	State the factors that affect sludge digestion.

15.	Draw a neat sketch of trap.

PART – C  (5 x 15 = 75 MARKS)

16.	Write a brief note on estimation of storm runoff.

(OR)

17.	Write a brief note on characteristics and composition of sewage.

18.	Explain in brief how testing of sewer lines are done.

(OR)

19.	Write a brief note on various materials used for sewer.

20.	With a neat sketch explain sedimentation tank.

(OR)

21.	With a neat sketch explain trickling filter.

22.	What is meant by sewage sickness? Explain the various preventive measures usually adopted to prevent sewage sickness.

(OR)

23.	Write a brief note on various methods of sewage disposal.

24.	With a neat sketch explain septic tank.

(OR)

25.	With a neat sketch explain any one system of sanitary plumbing.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	WATER RESOURCES ENGINEERING					      Time : 3 hours

Subject Code:	12CE227				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name any two methods used to find the mean rainfall on an area.

2.	Write the Dickens formula for estimating maximum flood.

3.	What is transmissibility?

4.	Mention the methods used for the artificial recharge of ground water.

5.	Write the relationship between duty and delta.

6.	Classify the crops based on crop seasons.

7.	Name any two sources of water for drinking purposes.

8.	What is per-capita demand?

9.	What is the function of surge tank?

10.	Mention any two measures for erosion control.

PART – B  (5 x 3 = 15 MARKS)

11.	Define transpiration.

12.	State the Darcy’s law of permeability.

13.	What is the necessity of irrigation?

14.	Draw the layout of conventional water treatment plant.

15.	What is the purpose of anchor blocks in pipe lines?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain the various types of precipitation. 										    (8)

b.	Discuss the factors affecting infiltration process.									    (7)

(OR)

17.	a.	Discuss the factors affecting runoff.												    (8)

b.	What is unit hydrograph? Discuss the assumptions involved in the unit hydrograph.	    (7)

18.	a.	What are the various types of aquifer? Explain with neat sketches.					    (8)

b.	Derive an expression for the discharge for steady radial flow into a well in unconfined 	aquifer. 																		    (7)

(OR)

19.	a.	During a recuperation test, the water in open well was depressed by pumping by 2 m and it 	recuperated 1.5 m in one hour. Estimate the yield from a well of 2 m diameter under a 	depression head of 2 m situated in the same area.									    (8)

b.	Discuss the benefits occurring from the conjunctive of waters.						    (7)

20.	a.	Discuss the factors affecting duty.												    (8)

b.	Explain the methods of measurement of consumptive use.							    (7)

(OR)

21.	a.	Discuss the various methods of irrigation.

b.	Design a trapezoidal concrete lined channel to carry a discharge of 100 m3/s  at a slope of 25 cm/km. The side slope of the channel is 1.5:1. The value of N is 0.016. Assume the limiting velocity as 1.5 m/s.

[P.T.O.]

22.	a.	Discuss the various methods of determining the wastage of water.

b.	What are the different methods of distribution system? Explain them.

(OR)

23.	Discuss the factors involved in the estimation of quantity of water for a region.

24.	Enumerate principle components of a hydro-electric scheme.

(OR)

25.	a.	Discuss the various methods of reservoir sediment control.						    (8)

b.	Discuss the mitigation measures for flood and drought.							    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	RAILWAYS, AIRPORTS AND HARBOURS			      	     Time : 3 hours

Subject Code:	12CE228						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the advantages of coning of wheels?

2.	Define ‘sleeper density’.

3.	List the components of a switch.

4.	State the principle of interlocking.

5.	State the location of fillets.

6.	List any four imaginary zones around airport.

7.	What is meant by mooring facilities?

8.	Mention the types of transit shed.

9.	What is mutation?

10.	Show an example of linguistic variable.

PART – B  (5 x 3 = 15 MARKS)

11.	What is creep? How it can be measured and prevented?

12.	Differentiate between ‘loop’ and ‘siding’. Also mention their types.

13.	Draw a typical airport layout and mark its components.

14.	What do you understand about ‘breasting dolphins’?

15.	Differentiate between ‘local minima’ and ‘global minima’

PART – C  (5 x 15 = 75 MARKS)

16.	Compare the advantages and disadvantages of various sleepers.

(OR)

17.	a.	A 6o curve diverges from a main curve of 4o in an opposite direction in the layout of a B.G. 	yard. If the speed on the branch line is restricted to 45 kmph, determine the speed 	restriction on the main line. Assume the permissible cant deficiency as 75 cm.		  (10)

b.	Determine the extent of gauge widening required for a B.G. track on a 5o curve, given the following data:

Rigid wheel base                      = 6 m

Diameter of wheel                    = 1.5  m

Depth of flange below rail top = 3.2 cm  										    (5)

18.	a.	Explain outer, home, starter, warner and advanced starter signals with a neat sketch.	    (8)

b.	What are the different systems of controlling the movement of trains? Explain the working 	principle of the system(s) which has been widely used on Indian Railway. 			    (7)

(OR)

19.	a.	Explain the tunnelling procedures in hard and soft rocks.							  (12)

b.	Explain the importance of ventilation in tunnels.									    (3)

20.	Sketch the typical layout of an airport showing the location of runways, taxiways, apron, terminal building etc. for the following runway configurations:

a.	Single Runway. 																    (3)

b.	Two Open Parallel Runways. 													    (4)

c.	Two Intersecting Runways at Farther End and 									    (4)

d.	Two Divergent Runways at Near End.											    (4)

[P.T.O.]

(OR)

21.	The following is the average of wind data for 10 years. An airport is to be designed for two runways. Determine the best runway orientations and calculate the total wind coverage.

| Directions | Percentage of time with wind velocity in KMPH |  |  |
| --- | --- | --- | --- |
|  | 6.4 - 25 | 25 - 50 | 50 - 75 |
| N | 0.3 | 1.3 | 0.1 |
| NNE | 2.0 | 4.0 | 0.1 |
| NE | 5.0 | 7.6 | 0.2 |
| ENE | 2.1 | 1.2 | 0.1 |
| E | 0.6 | 0.5 | 0.0 |
| ESE | 2.7 | 0.8 | 0.1 |
| SE | 3.9 | 3.0 | 0.2 |
| SSE | 2.9 | 2.5 | 0.1 |
| S | 1.6 | 1.2 | 0.0 |
| SSW | 2.0 | 2.0 | 0.1 |
| SW | 3.2 | 2.5 | 0.1 |
| WSW | 5.6 | 4.5 | 0.2 |
| W | 2.6 | 1.6 | 0.1 |
| WNW | 2.5 | 1.6 | 0.0 |
| NW | 3.5 | 2.5 | 0.1 |
| NNW | 1.5 | 0.5 | 0.0 |

22.	Explain the working principle of dry and wet docks with neat sketches.

(OR)

23.	What is ‘break water’? Illustrate the construction features of various types of break waters.

24.	Show a typical computational procedure of finding the weight matrix for predicting the outputs using a three layer (input, hidden and output) back propagation algorithm.

(OR)

25.	Bring out a typical computational procedure of optimization using Genetic Algorithm.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	INDUSTRIAL WASTE TREATMENT AND DISPOSAL	      Time : 3 hours

Subject Code:	12CE259		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are different water quality criteria?

2.	Define Recycle and reuse.

3.	What are important coagulants used in Industrial waste water treatment?

4.	What is residue?

5.	When is anaerobic decomposition adopted?

6.	What are the use of RO?

7.	What are the pollutants emitted from cement industry?

8.	Is it Electrodialysis a final method of treatment for industrial waste water?

9.	What is black water in pulp and paper industry?

10.	What is the importance of tanning?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the effects of waste water on land?

12.	How to remove dissolved impurities in waste water?

13.	When shall combined treatment of industrial and municipal waste water adopted?

14.	What are the waste water characteristics of Textile industries?

15.	What are the process involved in leather tanning?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What are process modification?

b.	Describe the process modification required.

c.	When shall an industry needs?

(OR)

17.	Give examples and explain volume reduction and strength reduction in a particular industrial waste water.

18.	a.	Explain the process of equalization?

b.	Why equalization is adopted?

(OR)

19.	Why sludge management is a problem in industries? What are the different management of sludge? Explain any one?

20.	What are the functions of stabilization ponds? What are the advantages and disadvantages of the ponds?

(OR)

21.	What are the aerobic and anaerobic treatments? What are the functions involved in Nitrogen and phosphorus removal?

22.	Give the flow chart of refineries and waste generated and suggest suitable treatment.

(OR)

23.	Give the flow chart for cement industry process and the waste generated treatment.

24.	Give the process involved in sugar industries, waste water generated and treatment involved.

(OR)

25.	What are the effects of waste water generated in pulp and paper industries? How to rectify the problems?

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	FLUID MECHANICS AND MACHINERY			      	     Time : 3 hours

Subject Code:	12CE263						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	State Newton’s Law of viscosity.

2. 	Find the pressure at a depth of 15m below the free surface of water in a reservoir.

3. 	Distinguish between path line and stream line.

4. 	The diameter of pipe at a section is 200 mm, if the velocity of the flowing fluid is 4m/s find the discharge through the pipe.

5. 	Write the relationship between stream function and velocity potential function.

6. 	List the application of Bernoulli’s equation.

7. 	Force exerted by a jet of water on the stationery vertical plate is in the direction of jet is given by _________.

8. 	How are pumps classified?

9. 	What is a draft tube?

10. 	Specific speed of the turbine is expressed as _________.

PART – B  (5 x 3 = 15 MARKS)

11. 	What are the advantages and limitations of Manometer?

12. 	List the various type of fluid flow.

13. 	State Bernoulli’s equation and list their assumptions.

14. 	Write short notes on selection of a pump.

15. 	How are turbines classified?

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	One litre of crude oil weighs 9.6 N. Calculate its specific weight, density and  specific    	gravity.

b.	The dynamic viscosity of oil, used for lubrication between a shaft and sleeve is                      6 poise, the shaft of diameter is 0.4 m and rotates at 190 rpm. Calculate the power lost in the bearing for a sleeve length of 90mm. The thickness of oil film is 1.5 mm.

(OR)

17. 	a.	U-tube manometer is used to measure the pressure of water in a pipe line , which is in 	excess of atmospheric pressure. The right limb of the manometer contains mercury and is 	open to atmosphere . The contact between water and mercury is in the left limb. Determine 	the pressure of water in the main line, if the difference in the level in the mercury in the 	limbs of u-tube is 10cm and the free surface of the mercury is in level with the centre of 	the pipe.

18. 	a.	Derive one dimensional continuity equation for fluid flow

b.	The velocity components in a two dimensional incompressible flow is                                        V= Y3+ 6x – 3x2y, and  U = 3y2x – 6y – x3.  a) Is the flow continuous , b) Is the flow Irrotational  c) In case the flow is irrotational , find potential function and stream function.

(OR)

[P.T.O.]

19.	a.	Water flows through a pipe AB is 1.2 m diameter at 3m/sec and passes through the pipe 	BC   of   1.5m diameter, At C, the pipe branches, Branch CD is 0.8 m in diameter and 	carries one third  of flow in AB. The flow velocity in branch CE is 2.5 m/sec.                            	Find a) Volume rate of flow in AB,  b)Velocity  in BC, c) Velocity in CD and d) Diameter 	of CE .

b.	The velocity components for a two dimensional incompressible flow are U=2x + y and

V= x–2y, find stream function.

20. 	a.	A 150 mm x 75 mm Venturimeter is placed vertically  in the upward direction. The  	difference between the throat and inlet is 225 mm above the inlet conveys oil of specific 	gravity of 0.78 at 29 liters/sec. Calculate the difference in pressure between the inlet and 	the throat. Take Cd = 0.96 .

b.	The difference in water surface levels in two tanks, which are connected by three pipes in series of length 300 m, 170 m, and 210 m and diameter 300 mm, 200 mm, and 400 mm respectively is 12 m. Determine the rate of flow if the coefficient of friction are 0.005, 0.0058 and 0.0048 respectively, considering the minor losses and neglecting the minor losses.

(OR)

21.	a.	Derive Darcy’s Weisbach Equation.

b.	Water is flowing through a pipe of length100 m having diameters 600 mm at the upper  end and 300 mm at lower end,andflows  at a rate of 50 litres/sec. The pipe as a slope of 1 in 30, Find the pressure at the lower end if the pressure at the higher level                            is 19.62 N/cm2.

22.	a.	Explain briefly the various components of a centrifugal pump with a neat sketch.

b.	A jet of water of diameter 50mm strikes a fixed plate in such a way that the angle between the plate and jet is 30 degrees, The force excerted in the direction of jet is 1471.5N Determine the rate of flow.

(OR)

23.  	a.	With  a neat sketch explain the working of an air vessel.

b.	A double acting reciprocating pump running at 40rpm discharging 1m3/minute. The pump has a stoke length of 400 mm, the diameter of the piston is 200 m, the delivery and suction head are 20m and 5m respectively. Find the slip of     the pump and power required to drive the pump.

24.  	Pelton wheel has a mean bucket diameter of 1m and is running at 1000rpm. The net head of the Pelton wheel is 700m. If the side clearance angle is 15º and discharge through the nozzle is 0.1m3/sec Find (a) Power available at the nozzle (b) Hydraulic efficiency of the turbine.

(OR)

25. 	An outward flow reaction turbine has internal and external diameters of the runner as                0.6 m and 1.2 m respectively. The guide blade angle is 15º and the velocity of flow through the runner is constant and is equal to 4m/s. If the speed of the turbine is 200rpm head on the turbine is 10m and the discharge at the outlet is radial find (a)Runner vane angle at inlet and outlet      (b) Work done by water on the runner /sec (c)Hydraulic efficiency.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	MECHANICS OF SOLIDS							      	      Time : 3 hours

Subject Code:	12CE264					 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define stress.

2.	Define modulus of rigidity.

3.	What do you mean by thin cylinders.

4.	When bending moment will be maximum?

5.	Write the types of beams.

6.	Define torsion.

7.	What are principal stresses?

8.	Write the types of springs.

9.	What is slope?

10.	Define deflection.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain Hooks law.

12.	Explain shear force and bending moment diagrams.

13.	Discuss on types of shafts.

14.	Explain maximum shear  stress theory.

15.	Write Euler’s formula for columns of different end conditions.

PART – C  (5 x 15 = 75 MARKS)

16.	In a tensile test on a hollow tube of external diameter 20mm and internal diameter 16mm, an axial load of 1500N produced an elongation of   0.0038mm in a length of 75mm while diameter is contracted by 0.0029mm. Calculate the Poisson’s ratio, Young’s modulus and bulk modulus.

(OR)

17.	A steel bar 900mm long,  its two ends are 40mm and 30mm in diameter and length of the rod is 200mm. The middle portion of the bar is 15mm in diameter and 500mm long. If the bar is subjected to an axial tensile load 15KN , find its total extension.

18.	Draw the shear force and bending moment diagram for the simply supported beam loaded as shown in figure.

(OR)

[P.T.O.]

19.	A simple supported beam of 6m span has a uniformly distributed load of 12 KN/m   over  the entire span and in addition a point load of 10KN at the center. Draw shear force and bending moment diagram.

20.	a.	Derive the relation σ /y =  E/R.

b.	Write the assumptions made in the theory of simple bending.

(OR)

21.	The shearing stress in a solid shaft is not to exceed 40 N/mm2 when the torque transmitted in 20000NM. Determine the minimum diameter of the shaft	.

22.	The principal stresses at a point across two perpendicular planes are 75MN/m2, ( tensile) and 35MN/m2 ( tensile) . Find the normal, tangential and the resultant stresses and its obliquity on a plane  at 200 with the major principal plane.

(OR)

23.	At a point in a strained material the permissible stresses are 100 N/mm2 (tensile) and 60N/mm2 (compression).Determine the normal stress , shear stress and resultant stress on a plane inclined at 500 to the axis of major principal stress.

24.	A cantilever beam   1.5m long  carries a  uniformly distributed load of W per m run,  over the entire length. Find the deflection at the free end, if the slope at the free end is 1.50 .

(OR)

25.	Derive the Euler’s formula when  both ends of the column are hinged.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination –  June 2015

Subject Title: 	REMOTE SENSING AND GIS							Time : 3 hours

Subject Code:	12CE342							              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

Explain the interaction of EMR with atmosphere , soil and water.

2.	Explain the digital image processing of satellite imageries for data interpretation.

(OR)

3.	Enumerate different types of image interpretation. Explain them with relative merits and

demerits.

4.	Write short notes on:

a.	Spatial & attribute data.

b.	Geographic scales.

c.	Requirements of GIS.

(OR)

5.	Compare the contrast raster and vector data representation with suitable examples.

6. 	Give a detailed account an overlay analysis in GIS.

(OR)

7.	Explain the four basic procedures for inputting spatial data into a GIS?

8.  	Explain the following

a.	Site selection for artificial recharge.

b.	Reservoir sedimentation.

(OR)

9. 	Explain the application of  GIS  in Drought monitoring and  Land use/land cover mapping .

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination –  June 2015

Subject Title: 	MATLAB												Time : 3 hours

Subject Code: 	12CE343								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	What is MATLAB and write down its applications? Also List down its features		(5)

b.	Explain how flowchart, pseudocode and algorithm are useful with an example

Write down the function of following MATLAB commands with an example		(5)

sum(A), ceil (A), img(A),size(A),trace(A),fix(A),rem(A),rand(n),sqrt(A), size(A))

c.	Write short notes on (with examples)						          			      (10)

i.	Strings.

ii.	Variables.

iii.	Arrays.

iv.	Special constants.

v.	Comment statements.

(OR)

2.	a.	Explain  the role of semicolon, comma and period in the Matlab statement/code	 with 	example.										 							(4)

b. 	What is expression and how it is written in Matlab programme.					(2)

c.	i) Use Matlab’s lengthcommand to find the length of each of the given vectors.            (8)

x=1:0.01:5

y=2:0.005:3

z=(100:0.5:200).’

x=(10:10:1000).’

(ii) Use Matlab’s linspace(a,b,n) command to generate n equally spaced numbers between

a and b for the given values of a, b, and n.

d. 	Explain basic mathematical operators, relational and Boolean operators in  Matlab with examples																	(6)

3. 	a. 	How Matlab selection statements (IF and SWITCH) are written in MATLAB. Explain 	with an example															(10)

b. 	Write a simple for loop to populate a column vector with the first 24 terms of the given sequence.															(10)

(i) an= (0.85)n

(ii) an= (1.25)n

(iii) an= sin(πn/8)

(iv) an= −3 cos(πn/12)

(OR)

4. 	a. 	What are the statements that can be used in Matlab for carrying out iterations? List down 	the conditions governing them.												      (10)

b. 	Write a Matlab program to find out the leap year.    							      (10)

5. 	a. 	Explain basic input and output statements, format command with examples.		     (15)

b. 	Write short notes on randomising and sorting a list.								(5)

(OR)

6. 	a. 	What are types of functions? Write a Matlab code with user-defined function for 	determining the factorial of a number.    									      (10)

b. 	Write Maltab commands to enter data to a text file and read them from a text file.	(5)

c. 	Explain fopen, fprintf and fscanf functions with an example.			 			(5)

7. 	a. 	Write down the MATLAB commands with an example to create all possible 3-D 	plots.

[P.T.O.]

(OR)

8.	a. 	Perform each of the following tasks for the problems given below. In each case, create a 	script file to draw the graph of the given function. Use linspace command to produce 	enough domain values to produce a “smooth curve.” Use the default plot command to 	produce a solid blue curve. Label the horizontal and vertical axes. Use title to provide a 	table containing the equation of the given function and the requested domain.	      (10)

(i) Sketch y = cosx on the interval[−2π, 2π].

(ii) Sketch y = lnx on the interval[0.1, 10].

(iii) Sketch y = ex on the interval[−2, 2].

(iv) Sketchy = |x| on the interval[−2, 2].

b.	 How the subplots can be created using MATLAB function. Explain with an example.(5)

c. 	Explain how histograms and pie charts can be prepared using Matlab commands.        (5)

9. 	a. 	What is a GUI in Matlab and explain about the different objects (edit, static text, axes, 	popup, list, check, push button) that can be created using GUI. How will you create them 	in GUI editor?      														      (16)

b. 	Write short notes on handles and call back functions in GUI.						(4)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	PRESTRESSED CONCRETE STRUCTURES			      Time : 3 hours

Subject Code:	14CE202		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the minimum grade of concrete for prestressing.

2.	List out the systems of prestressing.

3.	Define thrust line.

4.	List out the losses of prestressing systems.

5.	List out any two types of cable profile.

6.	Which force is predominant in end block?

7.	Which factor should be considered for long term deflection?

8.	Define secondary moment.

9.	Define linear transformation.

10.	List out the joints in-between the wall and floor slab of a tank.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the advantages of Hoyer’s long line system of prestressing?

12.	List out the assumptions made in strain compatibility method.

13.	List out the equations required for calculating short term and long term deflection.

14.	State the advantages and disadvantages of composite beams.

15.	Describe the design principles of pipes.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain briefly about the various system of prestressing involved in post tentioning members with neat sketch.

(OR)

17.	Explain briefly the advantages and disadvantages of prestressing members over R.C.C. with respect to safety, serviceability and economy.

18.	A prestressed concrete beam with a cross section 120mm wide and 300mm deep is used to support a UDL of 3KN/m over an effective span of 6m. The beam is pre-stressed by a straight cable carrying an effective pre-stressing force of 180KN at a constant eccentricity of 50mm. Given Ec=38KN/mm2, the modulus of rupture is 5N/mm2, area of the cable is 200mm2, and modular ratio is 6. Estimate the deflection of the beam at the following stages.

a.	Working load

b.	Cracking load

c.	1.5 times the cracking load

(OR)

19.	A pre-tensioned pre-stressed concrete beam having a rectangular section, 150mm wide and 350mm deep, has an effective cover of 50mm. If fck=40N/mm2, fp=1600N/mm2 and the area of pre-stressing steel Ap=461mm2. Calculate the ultimate flexural strength of the section using IS1343 code provisions.

[P.T.O]

20.	The end block of a post-tentioned member is 600mm wide and 600mm deep. Prestressing cables carrying a load force of 1200KN are anchored by plate anchorages 150mmx150mm locating with the centre coinciding the centre of the end block. The cable duct is 50mm diameter. 28days cube strength of concrete id 45N/mm2. The cube strength at transfer is 25N/mm2. Permissible bearing pressure behind anchorages is as per IS1343. Design suitable anchorages for the end-block.

(OR)

21.	A post tentioned prestressed concrete with rectangular cross section 300mmx400mm is prestressed with 1000mm2 of HTS wires. The steel is stressed to a level of 1200N/mm2. The beam is to span 9m. Calculate the deflection due to prestress, dead load and live load of 20KN/m. if a) the cable profile is straight with a constant eccentricity of 150mm and b) the cable profile is parabolic with a dip of 150mm at mid section and zero eccentricity at the ends. ES=2X105N/mm2; Ec=3.5x104N/mm2.

22.	A precast prestressed inverted T section is to be used in a composite slab of total depth 450mm and width 230mm. The inverted T consists of bottom flange of width 230mmx100mm thick and web of 300mm depth and 100mm width. The composite slab supports a live load of 12KN/m2 over a span of 9m. Prestressing force is 600KN, applied at an eccentricity of 70mm. M40 AND M20 concrete are used for precast and in situ concrete respectively. Determine the stresses at mid span section of the composite slab.

(OR)

23.	A two span continuous beam ABC, AB=BC=20mm is of rectangular section 500mm wide and 950mm deep. The parabolic cables has an eccentricity of 100mm below the centroid of mid span section , 200mm above the centroid of the central support and with zero eccentricity at A and C. Locate the pressure line when the beam carries the UDL of 18KN/m including the self weight. The prestressing force is 200KN.

24.	A prestressed concrete cylindrical water tank has to store 15lakhs litres of water with a storage depth of 8m. Assuming a flexible base, design the wall thickness and the spacing of 5mm dia HTS wires for prestressing. The design should satisfy the following conditions.

a.	Residual compressive stress of 0.7N/mm2 under working conditions.

b.	Cracking load factor of 1.2 and ultimate load factor of 2.

c.	M40 grade concrete with compressive stress of 13N/mm2 and tensile stress of 1.7N/mm2.

d.	Ultimate tensile stress of wires is 1500N/mm2 and initially stressed to 1200N/mm2

e.	Loss of prestress=20%

(OR)

25.	A series of rectangular precast beams 150mm wide and 300mm deep prestressed with stresses of 12N/mm2 at bottom and zero at top. They are simply supported over a span of 7.5m. The spacing between the beams is 600mmC/C. The propped and a 60mm thick slab is cast over the beam and allowed to harden. Assuming adequate shear transfer between the precast beam and cast-in situ slab, calculate the stresses in concrete at top and bottom of the mid span section when it carries a live load of 3KN/mm2.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	GIS APPLICATIONS IN CIVIL ENGINEERING			      Time : 3 hours

Subject Code:	14CE203		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Map.

2.	What are GIS?

3.	What are points, lines?

4.	What are overlay?

5.	What are raster data?

6.	What are digital image model?

7.	What is error elimination?

8.	What are output data?

9.	Define Remote Sensing.

10.	What is different output devices used?

PART – B  (5 x 3 = 15 MARKS)

11.	Describe different map analysis.

12.	Describe spatial and non spatial data with examples. Why are they called so?

13.	How overlays are prepared? What are their uses?

14.	What are different types of output devices?

15.	What are the applications of Remote Sensing?

PART – C  (5 x 15 = 75 MARKS)

16.	What are GIS components? How these components helpful in GIS analysis?

(OR)

17.	What are coordinate concepts? How the coordinates are prepared? How they are useful in Map preparation?

18.	a.	What are the purpose of digitizers and Scanners? How are these useful?

b.	Describe Data compression process.

(OR)

19.	a.	Differentiate between Vector and raster files.

b.	What is file organization?

20.	What are data retrieval processes available? How retrieval is done?

(OR)

21.	What is Artificial intelligence? What are its applications in Civil Engineering field?

22.	What are arc GIS? How they are applied in surveying field? Differentiate between arc GIS and QGIS.

(OR)

23.	In transportation Engineering How GIS tools are applied for solving the problems?

24.	Describe different types of errors in output. How these errors are rectified or eliminated?

(OR)

25.	What are the processes involved in output devices? When shall we use these different types?