Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	BASIC ELECTRICAL ENGINEERING				      	     Time : 3 hours

Subject Code:	09EE101						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List any two passive elements.

2.	How do you measure phase voltage?

3.	Define EMF.

4.	Write the significance of reluctance.

5.	Define RMS value.

6.	Write the importance of frequency in AC circuit.

7.	What is the input to the DC generator?

8.	What is meant by synchronous speed?

9.	What is meant by damping?

10.	Write the function of starter.

PART – B  (5 x 3 = 15 MARKS)

11.	Write kirchhoff’s voltage law.

12.	Write the significance of mutual inductance.

13.	Write the function of commutator.

14.	Describe the significance of back emf.

15.	Write the difference between MC and MI instruments.

PART – C  (5 x 15 = 75 MARKS)

16. 	Calculate a) the equivalent resistances across the terminals of the supply, b) total current supplied by the source and c) power delivered to 16 ohm resistor in the circuit shown in figure.

(OR)

17.	Find the value of R and the current flowing through it in the circuit shown when the current in the branch OA is zero.

[P.T.O.]

18.	Describe the significance of law of  electromagnetic induction with aid of a case study.

(OR)

19.	Obtain the expression for coupling coefficient and its impact on series and parallel magnetic circuits.

20.	a.	Write the essential features of indicating instruments. 							    (7)

b.	Explain the operation of induction type instrument. 								    (8)

(OR)

21.	a.	With a neat circuit diagram, explain the working of fluorescent light. 				    (7)

b.	Explain the working of residential building with neat residential building. 			    (8)

22.	With a neat block diagram, explain the construction and operating principles of transformer.

(OR)

23.	Describe the operating principles of single and three phase induction motors. 			(7+8)

24.	Explain the construction and operation of Nuclear power plant with neat block diagram.

(OR)

25.	Obtain the expressions for average, RMS, form and peak factors of pure sinusoidal waveform from its fundamentals.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	DIGITAL ELECTRONICS								      Time : 3 hours

Subject Code:	09EE202				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Decimal equivalent for the binary number (10001101)2 is _________.

2.	Simplify the expression Y=(A'+B)(A+B).

3.	What is the other name for Multiplexer?

4.	Draw the logic diagram for 2- bit parallel adder.

5.	Write the characteristic table and equation of T flip-flop.

6.	A counter has modulo of 9. The numbers of flip flops required are _________.

7.	What is an excitation table?

8.	What are the advantages of asynchronous sequential circuits?

9.	The programmable cells of __________ can be erased by using UV rays.

10.	Mention the important components in FPGA.

PART – B  (5 x 3 = 15 MARKS)

11.	Obtain the canonical sum of product form of the function Y(A,B,C)=A+BC.

12.	Implement the function Y = AB’ + A’B using only NAND gates.

13.	What is a synchronous circuit?

14.	Differentiate Mealey and Moore model.

15.	State the salient features of ECL family.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Convert the following													               (5x2)

i.	(268.75)10 into equivalent binary number

ii.	(11001011.01110)2  into equivalent decimal number

iii.	(35.45)10 into equivalent octal number

iv.    (725.25)8 into equivalent hexa decimal number

v. 	Convert the following Binary number into equivalent Gray code (10101101)2

b. 	Simplify the following Boolean expression

Y=AB+AC+ABC(AB+C)											                  (5)

(OR)

17.	Find the minimal sum of products for the boolean expression,

F(A, B, C, D) = ∑(1, 2, 3, 7, 8, 9, 11, 14, 15) using the Quine- Mcclausky method.

18.	a. 	Draw the logic circuit for following function using only NOR gates:

(A+B)(B+C)(A+C).    														    (7)

b.	Design a 8x1 MUX using two 4x1 MUX.									           (8)

(OR)

19.	a.	Implement a Binary to Gray code converter.										    (8)

b.	Implement the following Boolean function with 8:1 multiplexer.

F(A, B, C, D) = A’BD’+ACD+B’CD+A’C’D.	    							       	    (7)

[P.T.O.]

20.	Explain with the logic diagram and the truth table of J-K flip flop. Also find the characteristic equation using excitation table.

(OR)

21.	Define shift register and explain different types of shift registers with neat diagrams.

22.	What is a sequential machine? Illustrate the basic models of sequential machines.

(OR)

23.	Describe a state machine design process and design a 2 bit binary up counter.

24.	a. 	Explain the Xilinx XC4000 architecture with a neat diagram.						    (7)

b. 	Write notes on PLA and PAL.													    (8)

(OR)

25.	a.	Draw and explain CMOS inverter.												    (8)

b. 	Draw and explain the basic TTL circuit.         							             	    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTRONIC CIRCUITS								      Time : 3 hours

Subject Code:	09EE201				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The value of ripple factor of a full wave rectifier is __________.

2.	For a particular regulator the output voltage on no load is observed as 10 V while the full

load output voltage is observed as 9.8 V. Find its load regulation.

3.	Write the formula for calculating time constant in a RC series circuit.

4.	Mention some of the applications of pulse transformer.

5.	Why capacitive coupling is used to connect a signal to an amplifier?

6.	Why differential amplifiers are employing in signal conditioning circuits of sensors having mV output ?

7.	For the Class A operation the Q point is to be located at ___________.

8.	Which power amplifier gives minimum crossover distortion?

9.	Positive feedback increases the stability of the amplifier (Say True/False).

10.	Crystal oscillators are superior to tuned LC oscillators mainly because of their __________.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the difference between half wave and full wave rectifier?

12.	A square wave  is applied as input to a wave shaping circuit  . what will be shape of the output in  a) integrator b) differentiator. Justify the answer

13.	Draw and explain the circuit diagram of Darlington amplifier.

14.	What is crossover distortion in a power amplifier and how to eliminate it?

15.	A parallel LC circuit consists of a capacitor of 300pF and an inductance of 100μH. The Inductor has a

resistance of 5Ω. Find the value of resonant frequency.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the operation of Full Wave Rectifier with neat diagram. Derive the expression for efficiency and ripple factor.

(OR)

17.	Explain the working principle of Zener voltage regulator with neat diagram

18.	Draw and explain the response of low pass RC filter for a) step input b)exponential input.

(OR)

19.	Explain the operation of positive clipper and clamper with neat diagram.

20.	Explain the operation of single stage RC coupled  amplifier and derive the expression for Voltage gain, current gain, input and output impedance  using a low signal  frequency model.

(OR)

21.	Explain the working of emitter coupled differential amplifier with neat diagram. Analyse the circuit in differential mode.

22.	Explain in detail the operation of a class – B push pull power amplifier. Derive the expression for the power conversion efficiency of a class B power Amplifier

(OR)

23.	What are the advantages of negative feedback amplifiers? Explain the working of voltage series and current series feedback amplifiers.

24.	Explain the working principle of a RC phase shift oscillator with neat diagram.

(OR)

25.	Explain the working principle of colpitts oscillator with neat diagram   and write the formula for frequency of oscillations

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTROMAGNETIC FIELDS						      	      Time : 3 hours

Subject Code:	11EE206										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	An example for a scalar is _________.

2.	The value of  ar×aφ is given by _________.

3.	Electric flux is denoted by the symbol _________.

4.	Dielectric materials do not have _________.

5.	The magnetic field produced by a DC current is called _________ Magnetic field

6.	MMF is measured in _________.

7.	The intrinsic impedance of air is equal to _________.

8.	Motional emf is also called as _________.

9.	The propagation constant of a medium is given by _________.

10.	1 Neper is equal to _________.

PART – B  (5 x 3 = 15 MARKS)

11.	State divergence theorem. Give example for divergence field.

12.	State coulombs law.

13.	Define magnetization.

14.	Write the Maxwell’s equation in integral form derived from ampere’s circuital law..

15.	What is poynting vector?

PART – C  (5 x 15 = 75 MARKS)

16.	Evaluate both sides of divergence theorem for the field D→=2xyax→+x2ay→ C/m2 and the rectangular parallelepiped formed by the planes x = 0 and 1, y = 0 and 2, z = 0 and 3.

(OR)

Y

X

1

1

a

b

c

17.	Given A=2rcosφar+ raφ in cylindrical co-ordinates. For the contour shown in figure verify stoke’s theorem.

18.	State and prove gauss law and explain applications of gauss law.

[P.T.O.]

(OR)

19.	Use Laplace’s equation to find the capacitance per unit length of a co-axial cable of inner radius ‘a’ m and outer radius ‘b’ m. Assume V=V0 at r=a and V=0 at r=b.

20.	Derive an expression for the inductance of a co-axial cable.

(OR)

21.	Derive an expression for magnetic field (H) due to infinitely long straight conductor.

22.	Derive the expressions for transformer and motional emf.

(OR)

23.	Derive the maxwell’s equations in point form and integral form.

24.	State and explain the poynting vector and poynting theorem. Also derive the expression for poynting vector P.

(OR)

25.	Derive an expression for propagation constant and intrinsic impedance of a good conductor.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	INDUCTION AND SYNCHRONOUS MACHINES		      Time : 3 hours

Subject Code:	11EE209				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define crawling.

2.	What is an induction generator?

3.	The starting torque of single-phase induction motor is zero. True or False

4.	Mention any two applications of single-phase induction motor.

5.	Define voltage regulation of alternator.

6.	Draw the OCC of an alternator.

7.	Define hunting.

8.	Draw the V-curve of a synchronous motor.

9.	What is the difference between salient pole and non salient pole alternator?

10.	Which test is conducted to find Xd and Xq of a salient pole alternator?

PART – B  (5 x 3 = 15 MARKS)

11.	A 4-pole, three-phase induction motor runs at 1440rpm on 50Hz supply. Find the slip speed and slip.

12.	Write a note on linear induction motor.

13.	What are the conditions to be satisfied for the parallel operation of alternators?

14.	How the synchronous motor can be used as a synchronous condenser?

15.	Draw the phasor diagram of a salient pole alternator under UPF conditions.

PART – C  (5 x 15 = 75 MARKS)

16.	With the aid of necessary sketches, describe the features in the construction of cage and slip ring types of 3-phase induction motors.

(OR)

17.	a.	Obtain the expression for torque under running conditions of a 3-phase induction motor. 	Also obtain the condition for maximum torque.						        		    (8)

b.	The rotor resistance and standstill reactance per phase of 6-pole, 50 Hz, 3-phase induction 	motor are 0.001Ω and 0.005Ω respectively. Determine the speed at which maximum 	torque is obtained.								        						    (7)

18.	Using double revolving field theory, explain why a single-phase induction motor is not self starting.

(OR)

19.	Explain the operation and constructional features of a single-phase capacitor start induction motor.

20.	Derive expressions for pitch factor and distribution factor of an alternator and hence obtain the equation for emf induced.

(OR)

21.	Explain the synchronous impedance method of finding the voltage regulation of a non salient pole alternator.

22.	Explain any two starting methods of synchronous motor.

(OR)

23.	Draw the phasor diagram of synchronous motor when it takes power at (a) lagging, (b) unity and (c) leading p.f. conditions.

24.	Explain the terms direct-axis and quadrature-axis synchronous reactances of an alternator.

(OR)

25.	For a salient pole synchronous machine, neglecting the effect of resistance, derive an expression for power developed as a function of torque angle.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTRICAL MACHINE DESIGN						      Time : 3 hours

Subject Code:	11EE211		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the main dimensions of a rotating machine?

2.	How will you minimize the leakage flux?

3.	The number of segments in commutator is equal to ____________.

4.	What is square pole criterion?

5.	The transformer used in sub-stations and generating stations are called ____________.

6.	Why the efficiency of a transformer is so high?

7.	For good overall design of three phase induction machine’s L/τ ratio is ____________.

8.	What is rotating transformer?

9.	What is short circuit ratio?

10.	What is critical speed of alternator?

PART – B  (5 x 3 = 15 MARKS)

11.	What is real and apparent flux density?

12.	What is Interpoles and what is its purpose?

13.	List the different methods of cooling of transformers.

14.	List the factors which affect the length of air gap in induction machines.

15.	List the factors to be considered for the choice of specific electric loading in synchronous machines.

PART – C  (5 x 15 = 75 MARKS)

16.	Calculate the ampere-turns for the air gap of a dc machine given the following data.

Gross core length = 40cm, air gap length = = 1.0cm, slot pitch = 6.5cm, slot opening = 0.5cm, 	average value of flux density in the air gap =0.63T. Field form factor = 0.7, Carter’s coefficient 	= 0.72 for opening/gap Carter’s coefficient=0.82 for opening/gap= 1.0.

(OR)

17.	a.	Derive an equation for the slot leakage reactance.                                                    	    (7)

b.	Explain about the types of leakage flux.                                           					    (8)

18.	a.	Derive the output equation of a DC machine.                                         				  (10)

b.	Distinguish between lap winding and wave winding.                           				    (5)

(OR)

19.	A design is required for a 50kW,4pole, 600rpm, and 220V dc shunt generator. The average flux density in the air gap and specific electric loading are respectively 0.57T and 30000 ampere-conductors per meter. Calculate suitable dimensions of armature core to lead to a square pole face.

20.	a.	Derive the output equation of a single phase transformer.                					    (7)

b.	Discuss about the various methods of cooling of power transformer.                   		    (8)

[P.T.O.]

(OR)

21.	Determine the main dimensions of the core and window for a 500 kVA, 6600/400V, 50Hz, 	Single phase core type, oil immersed, self cooled transformer. Assume: Flux density = 1.2 T, 	Current density = 2.75 A/mm2, Window space factor = 0.32, Volt / turn = 16.8, type of core: 	Cruciform, height of the window = 3 times window width. Also calculate the number of turns 	and cross-sectional area of the conductors used for the primary and secondary windings.

22.	Obtain the following design information for the stator of a 30 kW, 440 V, 3 phase, 6 pole, 		50 Hz delta connected, squirrel cage induction motor, (i) Main dimension of the stator, (ii) 		No. of turns/phase (iii) No. of stator slots, (iv) No. of conductors per slot. If Specific Magnetic 	loading Bav = 0.48 Tesla, Specific Electric loading q = 26000 ac/m, Full load efficiency η= 	0.88, Full load power factor cosΦ = 0.86, Winding factor Kw = 0.955.

(OR)

23.	During the stator design of a 3 phase, 30 kW, 400volts, 6 pole, 50Hz, squirrel cage induction 	motor following data has been obtained. Gross length of the stator = 0.17 m, Internal diameter 	of stator = 0.33 m, Number of stator slots = 45, Number of conductors per slot = 12. Based on 	the above design data design a suitable rotor.

24.	Design the rotor of a 3 phase 20 MVA, 11 kV, 3000 rpm, 50 Hz, turbo alternator with the following design data. Diameter at the air gap = 0.8 m, Gross length = 2.4 m, stator turns       per phase = 18, Number of stator slots = 36, Exciter voltage = 220 volts, Estimate (a) Number of rotor slots, (b) area of the field conductor (c) Turns in the filed coil and (d) Field current.

(OR)

25.	The following information has been obtained during the preliminary design of a 3 phase 500 	kVA, 6.6 kV, 12 pole, 500 rpm, star connected salient pole alternator. Stator diameter = 1.3 m, 	gross length of stator = 0.21m, air gap flux per pole = 0.0404 wb. Based on the above 	information, design the field system of the alternator giving the following details.

a.	Length of the air gap.

b.	Diameter of the rotor at the air gap surface.

c.	Dimension of the pole.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	GENERATION, TRANSMISSION AND DISTRIBUTION

Time : 3 hours

Subject Code:	11EE215						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	One of the non-conventional energy sources is ______.

2.	India’s first nuclear power plant was built at ________.

3.	In short transmission lines the effects of _______ are neglected.

4.	In any trtansmission line, AD – BC = ________.

5.	The most commonly used material for insulators of overhead lines is ________.

6.	If the string efficiency is 100%, it means that ________.

7.	The metallic sheath is provided over the insulation to protect the cable from _______.

8.	If the length of a cable is doubled, its capacitance is _______.

9.	The service mains connect the _______    and the ______.

10.	The chances of faults in underground system are ______ compared to overhead system.

PART – B  (5 x 3 = 15 MARKS)

11.	Compare conventional and non-conventional power generation.

12.	What is proximity effect?

13.	What are the different factors affecting corona loss?

14.	What are the different types of underground cables?

15.	Define the terms: Feeder, Distributor and Service mains.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the schematic arrangement of hydro-electric power station. Also discuss about its advantages and disadvantages.

(OR)

17.	Explain with neat diagram the schematic arrangement of nuclear power station.

18.	Derive an expression for the inductance of single phase two-wire line.

(OR)

19.	Explain about surge impedance and surge impedance loading.

20.	Explain with neat diagrams the pin type and suspension type insulators.

(OR)

21.	Explain the different methods to increase the string efficiency.

22.	Compare overhead lines with underground cables.

(OR)

23.	Derive an expression for capacitance in three core cables.

24.	Explain the radial and ring main distribution systems with neat diagrams.

(OR)

25.	A 2-wires DC distributor cable AB is 2 Km long and supplies loads of 100A, 150A, 200A and 50A situated 500m, 1000m, 1600m and 2000m from the feeding point A. Each conductor has a resistance of 0.01 ohms per 1000m. Calculate the potential difference at each load point if a potential difference of 300 V is maintained at point A.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	CONTROL SYSTEMS								       	      Time : 3 hours

Subject Code:	11EE219				 					              Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Electrical time constant of an armature controlled dc servomotor is

a.	Equal to mechanical time constant			b.	Larger than mechanical time constant

c.	Smaller than  mechanical time constant		d.	None of the above.

2.	Signal flow graph is used to find

a.	stability of the system						b.	Controllability of the system

c.	Transfer function of the system				d.	Observability of the system

3. 	Velocity error constant is zero for

a.	Type 0 system							b.	Type 1 system

c.	type 2 system							d.	Type 3 system

4.	The addition of PD controller in cascade with the plant

a.	improves damping						b.	reduces steady state error

c.	increases undamped frequency				d.	increases the system order

5.	________ is the graph between Magnitude in decibel and phase angle in degrees.

a.	bode plot  	b.	polar plot    c.	Magnitude plot 	d.	both a and b

6. 	The frequency at which two asymptotes meet is called

a.	natural frequency							b.	cutoff frequency

c.	corner frequency							d.	damped frequency

7.	For a stable system roots should lie on ______ half of S plane.

a.	 Left half								b.	Right half

c.	Imaginary axis							d.	None

8.	The closed loop system is stable if the number of anti clockwise encirclement of 1+G(S)H(S) plane around the origin is_____ number of right half s-plane poles of open loop transfer function.

a.	less than  	b.	more than  		c.	equal  to  	d.	independent of

9.____ variable representation is not available for measurement and control purpose.

a.	physical  	b.	phase  			c.	both a and b  d.	none of the above

10.	In canonical form of state representation, matrix A is

a.	unit matrix    b.	diagonal matrix    c.	zero matrix   d.	any of the above

PART – B  (5 x 3 = 15 MARKS)

11. 	Derive the transfer function of single closed loop with negative feedback.

12. 	Enumerate the advantages of generalized error coefficients and static error coefficients method.

13. 	Define bandwidth and also write the effect of high bandwidth.

14.	How to find the system stability based on location of roots?

15.	Write the steps for state space representation of the system using canonical variables.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the transfer function X1(s)/F(s) and X2(s)/F(s) for the given mechanical system.

M1

M2

x1

f(t)

x2

K1

K2

B1

(OR)

17. 	a.	Find the overall transfer function using block diagram reduction technique.      		(5)

G1

G2

H1

R(S)

C(S)

b.	Find the overall transfer function of the given system using Mason’s gain formula.

(10)

18. 	The open loop transfer function of a unity feedback control system is

50

G(S)=  -----------

S (1+0.1S)

Find the steady state error using both static error and dynamic error coefficient                methods for r(t) =1+2t+t2  .

(OR)

19. 	Measurements are conducted on a servomechanism. Show the system response to be             c(t) = 1+0.2e-60t–1.2e-10t  when subjected to a unit step input (a)Obtain the expression for the closed loop transfer function. (b) Determine the type of the system based on damping.

20. 	Draw the bode plot of the system

K

G(S)=     ---------------------------- .Find the gain margin and phase

S (1+S) (1+0.1S)

margin and also find the value of K for phase margin =20.

(OR)

21. Draw the polar plot of the system

K

G(S)=     ----------------------------. Find the gain margin and phase

S (1+S) (1+0.1S)

margin. And also find the value of K for gain margin =20 db.

22. 	Sketch the root locus for the given open loop transfer function

G(S)H(S) = k / S(S+2)(S+4).

(OR)

23.	a.	The open loop transfer function of a unity feedback control system given by

K

G(S) =          ------------------------------------

S (ST1+1) (ST2+1)

Applying Routh –Hurwitz criterion determine the value of K in terms of  T1 and  T2 for the system to be stable.                                                                               				(10)

b.	Write the procedure to find stability of the system using Nyquist criterion.			(5)

24. 	Construct state space representation of the given system using

a.	Phase variable form  ii)canonical variable form

Y(s)U(s)=10(s+4)ss+1s+3

(OR)

25. Construct Jordon canonical form of the system   G(s) = 5 / (s+1)2(s+2).

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	C++ AND DATA STRUCTURES							      Time : 3 hours

Subject Code:	11EE223				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is derived data type in C++? Give one example.

2.	What is declaration in C++?

3.	List two types of operator overloading.

4.	What is the significance of Delete Operator?

5.	Write declaration statement for object array.

6.	Write declaration of static variable in C++ programming language.

7.	List two types of data structures.

8.	When a tree is called a binary tree.

9.	List two type of sorting algorithms.

10.	What is the time complexity of linear search?

PART – B  (5 x 3 = 15 MARKS)

11.	What is copy constructor? Give its significance.

12.	What is over riding? Give example.

13.	Compare errors and exceptions.

14.	Why list is called linear data structure.

15.	Write structure definition for creating node for linked list in C++ programming language.

PART – C  (5 x 15 = 75 MARKS)

16.	Write different control structures available in C++ programming language.

(OR)

17.	Write a C++ program to get a string from user and display its character count, and display its reverse.

18.	Explain different inheritance that are possible in C++ programming language with an example.

(OR)

19.	Write a C++ program to perform matrix addition using operator overloading.

20.	What is the significance of friend function? Give an example and explain the notion of friend function.

(OR)

21.	Write a C++ program to read contents from console and write it on a file.

22.	Write a C++ program to create a circular list and perform insert and delete operations.

(OR)

23.	Write a C++ program to create a binary search tree and perform insert and delete operations.

24.	Write a C++ program to implement binary search algorithm.

(OR)

25.	Write a C++ program to implement heap sort algorithm.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	EMBEDDED SYSTEM									      Time : 3 hours

Subject Code:	11EE231		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Which is the processor used for jpg compression and decompression?

2.	Which type of memory is used in storing application programs from where the processor 	fetches the instruction codes?

3.	Real time computing describes hardware and software systems subject to a __________.

4.	__________  is the probability that the system will not undergo failure over any part of a 	prescribed interval.

5.	What are the three important states of each task in an RTOS?

6.	Name any four methods used for inter process communication?

7.	The software development and debug tools for building programs that runs on the host is called 	__________.

8.	Which is the software tool used to simulate the behavior of CPU and memory?

9.	Give an example for SOC.

10.	Which language is popular for smart card applications?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the different methods of resetting CPU? Write their significance.

12.	List the main desirable properties of RT systems.

13.	Write the rules that decide any function to be reentrant.

14.	What is address resolution?

15.	Draw the state diagram for Automatic Chocolate Vending Machine.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Name the different program models for software designing.                                       	    (5)

b.	Tabulate the software tools used for designing an embedded system and their functional 			description. 						             										  (10)

(OR)

17.	a.	Brief about the processors involved in the design of complex embedded systems. 	    (5)

b.	What are the challenges faced in embedded system design? Also write in detail about the 			optimizing design metrics.														  (10)

18.	a.	What are the primary problems to get solved in RT systems?      					    (8)

b.	Classify and compare tasks in RT systems.										    (7)

(OR)

19.	a.	What are the issues in real time computing?										    (8)

b.	What are the properties of a good performance measure?  							    (7)

20.	What is shared data problem? How do semaphores overcome this problem?

[P.T.O]

(OR)

21.	Explain the timer functions of RTOS by considering the delaying of task with RTOS delay function as an example.

22.	How are embedded software getting into the target system? Describe any three methods in 	detail.

(OR)

23.	Describe the principle of operation of Logic analyzer and Instruction set simulators as the 	development and debug tools in embedded system design.

24.	For the design of digital camera, discuss the requirements, class diagrams, hardware and 	software architectures.

(OR)

25.	Design an embedded system for an adaptive cruise control system in a car. Explain the 	specifications, need and architectures in detail.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	AUTOMOTIVE ELECTRONICS						      Time : 3 hours

Subject Code:	11EE240				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is sensor?

2.	What type of input given to solenoids?

3.	Which motor is called cranking motor?

4.	What is the need of starter switch?

5.	What is FBC?

6.	What is the use of sparkplug?

7.	What is dazzling?

8.	Why we need horn?

9.	What is ABS?

10.	What is cranking?

PART – B  (5 x 3 = 15 MARKS)

11.	Why we need denotation sensor?

12.	What is the condition for starting?

13.	Brief multipoint fuel injection?

14.	List the Advantages of electronic ignition.

15.	Explain sidelight.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe Basic sensor arrangement.

(OR)

17.	Explain Fuel metering and  vehicle speed sensor.

18.	Write about the working of different starter drive Units.

(OR)

19.	Explain Series motor and its Characteristics.

20.	Describe Fuel injection systems.

(OR)

21.	Explain Contact less electronic ignition system.

22.	Brief about positive & negative earth systems.

(OR)

23.	Explain Fuel, Oil & Temperature gauges.

24.	Describe - Deceleration leaning and idle speed control.

(OR)

25.	Enumerate Current trends in modern Automobiles.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title : ENERGY SYSTEMS										      Time : 3 hours

Subject Code: 11EE242			 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Photovoltaic effect?

2. 	What is meant by pitch control?

3. 	Mention the name of the highest level of tidal water.

4. 	List the main components of a fuel cell.

5. 	High frequency induction heating is used for ___________.

6.  	List the types of lighting systems.

7. 	List out the demerits of electric traction system.

8.  	Define tractive effort.

9. 	A Van de graff generator has a belt speed of 2.5 m/s. charge density of 10 μc/m2 and a

belt width 2m. The maximum charging current is ___________.

10. 	Why we go for sphere gap measurement?

PART – B  (5 x 3 = 15 MARKS)

11.	Mention the use of charge controllers.

12.	Give the current scenario of renewable energy generation in India.

13.	List out the characteristics involved in good lighting scheme.

14.	What is schedule speed of a train and list out its affecting factors?

15.	How resistance shunt is usually designed?

PART – C  (5 x 15 = 75 MARKS)

16.	With neat sketch explain the WECS and classify its types.

[OR]

17.	Explain with neat sketch the operation of solar PV system and discuss their applications.

18. 	With a neat sketch, explain the working of ocean tidal energy conversion system.

[OR]

19. 	a.	Explain the principles of MHD power generation.								    (7)

b.	Briefly explain fuel cells. 														    (8)

20.	Describe the construction and principle of operation of (a) sodium vapour lamp and

(b) fluorescent lamp.

[OR]

21. 	Discuss the various electric heating methods.

22.	The distance between two stations is 1.6 km and the average speed of a train is 40 kmph. The acceleration, retardation during coasting and breaking are 2kmphps. 0.16kmphps and 3.2 kmphps respectively. Assume quadrilateral approximation of speed time curve, determine the duration of the accelerating, coasting and breaking periods and distance covered during these periods.

[OR]

23.	Derive the tractive effort during acceleration.

24. 	With a neat sketch explain the working principle of Van de Graff generator.

[OR]

25. 	Explain the various high voltage measuring techniques.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	AUTOMOTIVE ELECTRONICS						      Time : 3 hours

Subject Code:	11EE249		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the application of solenoids.

2.	List the purpose of flow sensors.

3.	Draw the speed Vs torque characteristics of DC series motor.

4.	__________type of starter used for DC series motor.

5.	List the types of ignition systems.

6.	List the advantages of electronic ignition system.

7.	__________type of sensor using in speedometer.

8.	What is electric braking system?

9.	__________type of motor is using in wiper mechanism.

10.	Write the expansion of ABS.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between solenoids with relay.

12.	Why DC series motors are started with loaded condition?

13.	Explain fuel injection system.

14.	Explain the positive earth system in automobile.

15.	Differentiate between open loop and closed loop control system.

PART – C  (5 x 15 = 75 MARKS)

16.	Write short note on

a.	Flow sensor

b.	Throttle sensor

c.	Speed sensor

(OR)

17.	Explain the basic arrangement of sensor and actuators in automobiles.

18.	Explain the any two types of DC motor starters.

(OR)

19.	a.	Draw and explain electrical characteristics of DC series motor.				   	  	    (8)

b.	Explain the methods of maintenance of starter.									    (7)

20.	a.	Write short note on fuel injection system.             								    (8)

b.	Design a Electronic spark timing control in automobile.							    (7)

(OR)

21.	Explain the feedback carburetor system in automobile with neat block diagram.

22.	Draw and explain the speedometer and fuel level measurement system in automobiles.

(OR)

23.	Explain various lighting system in automobiles.

24.	a.	Write short notes on integrated engine control system.                            				    (7)

b.	Explain electronically controlled electric power steering system with neat sketches.	    (8)

(OR)

25.	Explain the modern trends in automobiles.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	BASIC ELECTRICAL AND ELECTRONICS ENGINEERING													      	     							     Time : 3 hours

Subject Code:	12EE102						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Kirchhoff’s current law.

2.	Two resistances R1 and R2 of values 4  and 8  respectively are connected in parallel, what is the equivalent resistance of the combination?

3.	If a star connected network have 9  resistors is connected in each arms, find the equivalent delta connected network.

4.	Define peak factor.

5.	Write the EMF equation of DC generator.

6.	List any two types of single phase induction motor.

7.	The temperature coefficient of resistance of semiconductor is always _________.

8.	A table which lists all possible combinations of inputs and the corresponding outputs is called a ___________.

9.	Define instrument.

10.	Which are the materials used in piezo-electric transducers?

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate series and parallel circuits.

12.	Define the terms RMS and average value.

13.	Write down the principle of operation of a DC generator.

14.	Explain the function of AND logic gate.

15.	Explain the function of measurement system.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Find the current through each branch of the given circuit shown in figure using Kirchhoff’s 	laws.   	       								       									    (7)

[P.T.O.]

b.	A battery having an emf of 15V is connected across terminals AB of the circuit shown 
    		in figure. Find the current flowing in each resistance and total power absorbed by the 
    		circuit.							                                           						    (8)

(OR)

17.	Using source transformation techniques simplify the network shown in the figure.

18.	a.	An inductive coil takes 10A and dissipates 1000W when connected to a supply at 250V, 	50Hz. Calculate the impedance, effective resistance, reactance, inductance and power 	factor.									                 							    (7)

b.	Obtain the relationship between currents and the relationship between voltages in a delta 	connected system. Hence, derive the equation for power in such a system.               	    (8)

(OR)

19.	Determine the form factor and peak factor of a sinusoidal voltage wave form.

20.	Explain the working principle and constructional details of D.C Generator.

(OR)

21.	With a neat diagram, explain the construction of a transformer and derive the expression for the induced emf.

22.	a.	Draw the energy band diagrams for the following materials

i) Conductors ii) Insulators and iii) Semiconductors.					       			    (6)

b.	Describe the action of PN junction diode under forward bias and revere bias. 	           (9)

(OR)

23.	a.	Explain V-I characteristics of zener diode with neat sketch.			      			    (7)

b.	Show that 					      					    (8)

24.	Describe in detail the following terms:

a.	Temperature Transducers.								      						    (7)

b.	Piezoelectric Transducers.	                                                                                          	    (8)

(OR)

25.	Explain the working principle and operation of moving coil instruments with neat diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTRIC CIRCUITS AND NETWORKS				      Time : 3 hours

Subject Code:	12EE201				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What do you mean by source transformation?

2. 	What do you mean by Kirchhoff’s current law?

3.  	State Thevenins Theorem.

4. 	State Maximum power transfer theorem.

5. 	What is a RL circuit?

6. 	Define transient?

7. 	What is mutual inductance?

8. 	What is coefficient of coupling?

9. 	What is low pass filter?

10.	Define driving point.

PART – B  (5 x 3 = 15 MARKS)

11. 	Give the expression for delta to star transformation.

12. 	Define Reciprocity theorem.

13. 	What is final value theorem?

14. 	What is Self inductance?

15. 	What is impedance matching?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain with an example the loop analysis and the steps involved in detail.

(OR)

17.	Explain the various source transformations in detail with circuits.

18.  State Norton’s theorem and show the steps involved to justify the theorem.

(OR)

19.	State the Maximum power transfer theorem  and show the steps involved to justify the theorem.

20.	State and explain the initial value theorem.

(OR)

21. 	Explain the transient concepts in detail.

22.  Explain the double tuned circuits and give its importance.

(OR)

23.	What is magnetic circuits and derive the coefficient of coupling.

24.	Explain in detail the concepts of voltage and current ratios of two port networks.

(OR)

25.	Explain the characteristics of ideal filter.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTROMAGNETIC FIELDS							      Time : 3 hours

Subject Code:	12EE202				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The value of  ar×aφ is given by___________.

2.	The stokes theorem is applied to ___________ surface

3.	Force exerted per unit charge is called ___________.

4.	Electric field lines always start at ___________ charge.

5.  	Magnetic flux is denoted by___________.

6.   Gauss’s law for magnetic field is given by ___________.

7. 	The ratio of the magnitudes of conduction current density (JC) and displacement current density (JD) is___________.

8.	The intrinsic impedance of free space is ___________.

9.	Poynting vector gives the ___________ rate of energy flow per unit area at a point.

10.	All of the following can induce emf in a coil of wire except

a.	Moving a magnet through the coil.

b.	Placing a stationary coil of wire in an increasing magnetic field.

c.	Placing a stationary coil of wire in a stationary magnetic field.

d.	Placing a stationary coil of wire in a decreasing magnetic field.

PART – B  (5 x 3 = 15 MARKS)

11.  State divergence theorem.

12. 	Determine the force per metre length between two long parallel wires A and B separated by      5 cm in air and carrying currents of 40A in the opposite direction.

13. 	State the expression for H due to infinite sheet of current.

14. 	Write Maxwell’s equation in point and integral form for good conductor.

15. 	For a lossy dielectric material having r=1,  r=48 and  =20 s/m, calculate the propagation constant at a frequency of 16 GHz.

PART – C  (5 x 15 = 75 MARKS)

16.  In region ra  in a spherical co-ordinates, E=(rρ5ε)ar.Show that closed E.dS=(∇.E)dv.

(OR)

17.  a.	Write the expressions for differential length, surface and volume in all the three co-				ordinate 	systems.																(8)

b.	Find the divergence of the vector field A=ρsinφaφ+2ρcosφaφ+2z2az			(7)

18.  	Derive an expression for electric field intensity E due to an uniformly charged infinitely long 	straight line with constant charge density in C/m.

(OR)

19.  	A parallel plate capacitor with a separation of 1 cm has 29 KV applied when air was the dielectric used. Assume that the dielectric strength of air as 30 KV/cm. A thin piece of glass with r=6.5 with a dielectric strength of 290 kV/cm with thickness 0.2cm is inserted. Find whether glass will break or air?

[P.T.O.]

20.  State Biot-Savart’s law and use it to find the magnetic field strength due to an infinite current 	carrying conductor. Also find the magnetic flux density.

(OR)

21. 	Find the torque produced in a planar coil of ‘n’ turns which carries a current of I amperes.

22. 	Derive the expressions for transformer and motional emf.

(OR)

23. 	From fundamental laws, derive the generalized Maxwell’s equations.

24. 	Derive the expressions for wave parameters in perfect dielectric.

(OR)

25. 	Derive the mathematical form of Poynting’s theorem.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTRONIC CIRCUITS								      Time : 3 hours

Subject Code:	12EE205				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a Voltage Regulator?

2.	Define Rectifier Efficiency.

3.	What are the conditions for a series RC circuit to act as an integrator?

4.	Define Clamping.

5.	In a JFET, gate-source junction is always __________ biased and source-drain is __________ biased.

6.	Define CMRR.

7.	Which type of power amplifier exhibits crossover distortion in its output?

8.	Write the expression for gain of an amplifier with feedback.

9.	What is the use of commutating capacitor in bistable multivibrator?

10.	What is the main advantage of Crystal oscillator.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate Half Wave Rectifier and Full Wave Rectifier.

12.	List some applications of clamper circuits.

13.	Explain the operation of single stage RC coupled amplifier with a circuit diagram.

14.	Define (a) feedback (b) positive feedback and (c) negative feedback.

15.	How Schmitt trigger is different from multivibrator?  Also mention some applications of Schmitt trigger.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A 50Ω  load resistance is connected across a half wave rectifier. The input supply voltage 	is 230V (rms) at 50 Hz. Determine the DC output (average) voltage, peak-to-peak ripple in 	the output voltage (Vp-p), and the output ripple frequency (fr).	     If a 10000μF filter 	capacitor is added across the load resistor, determine the DC output voltage (i.e. average 	voltage), load current, peak-to-peak ripple in the output voltage, and the output ripple 	frequency.	                                                                                    					  (12)

b.	Why an LC filter is called Load independent?			           	   				    (3)

(OR)

17.	a.	Explain the working of Bridge rectifier.  Derive the expressions for RMS current, PIV, 	ripple factor and efficiency.								    					  (10)

b.	Explain the operation of Zener voltage regulator.			                 				    (5)

18.	a.	Explain how a high pass RC circuit act as differentiator.   		                			  (10)

b.	Brief the operation of Pulse Transformers.					     					    (5)

(OR)

[P.T.O.]

19.	a.	Explain the operation of clipper circuit whose output is as shown in following.		  (10)

b.	Explain the operation of integrator with a neat circuit.                                             	    (5)

20.	a.	Explain the small signal model of BJT as differential amplifier and hence derive the 	expression for CMRR.									    					  (10)

b.	What do you understand by the term Darlington pair?  What are the advantages?	    (5)

(OR)

21.	a.	Design a JFET source – follower circuit with a specified small – signal voltage gain.  For 	the source follower circuit shown, the transistor parameters are : IDSS = 12mA, Vp = - 4V, 	and λ = 0.01V-1.  Determine Rs and IDQ such that the small signal voltage gain is a least

Av = 0.90										   							  (12)

b.	Brief the working principle of chopper amplifier.									    (3)

22.	a.	Explain the operation of Class A power Amplifier with a neat circuit diagram.  Also 	enumerate its advantages and disadvantages.					   					  (12)

b.	Briefly explain how amplifiers are classified.                                                   		    (3)

(OR)

23.	a.	Explain the Class B Push pull amplifier circuit and derive an expression for Output power 	and efficiency.                                                                                         				  (12)

b.	When two signals V1 and V2 are connected to the two inputs of a difference amplifier, define a difference signal Vd and common-mode signal Vc.						    (3)

24.	a.	With a neat circuit explain the operation of Wien bridge oscillator.      				  (10)

b.	Explain crystal oscillation in detail.					 							  (5)

(OR)

25.	a.	Explain the working of  RC phase shift oscillators with a neat circuit.         			  (10)

b.	What is a multivibrator? List the different types of multivibrators.                    		    (5)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title : INDUCTION AND SYNCHRONOUS MACHINES			      Time : 3 hours

Subject Code: 12EE209			 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the condition for maximum starting torque in a three-phase induction motor?

2.	Define cogging.

3.	A hysteresis motor works on the principle of eddy current loss – State Yes or No?

4.	In a capacitor start motor, the phase displacement between starting and running winding can be nearly  - Yes or No?

5.	How are alternators classified?

6.	What is a fractional pitch winding?

7.	What is the necessity for parallel operation of alternators?

8.	Salient pole machines have smaller number of poles – State True or False?

9.	An over-excited synchronous motor operates at unity p.f. – State True or False?

10.	Draw the inverted V curve of a synchronous motor.

PART – B  (5 x 3 = 15 MARKS)

11.	The rotor resistance and standstill reactance per phase of 6 –pole, 50 Hz, 3-phase induction motor are 0.001 Ω and 0.005 Ω respectively.   Determine the speed at which maximum torque is obtained.

12.	Write a note on synchronous reluctance motor.

13.	Calculate the rms value of the induced emf/ph of a 10 pole, 3-phase, 50 Hz alternator with 2 slots/pole/phase and 4 conductors per slot in two layers. The coil span is 150°. The flux/pole is 0.12 Wb.

14.	Two alternators operating in parallel supply a common load of (300 + j 400) kVA. One alternator is loaded to 200 kW at 0.8 p.f lagging. What is the load shared by the other alternator?

15.	Discuss the purpose of using damper windings in synchronous machines.

PART – C  (5 x 15 = 75 MARKS)

16.	Develop an approximate equivalent circuit for a 3-phase induction motor.

(OR)

17.	Explain the operation of star-delta starter with a neat sketch.

18.	Describe the operation of stepper motor with a neat sketch.

(OR)

19.	With a neat sketch, explain the operation of universal motor. Also mention its applications.

20.	 Explain MMF method of finding the regulation of alternator.

(OR)

21.	A 3-phase star-connected synchronous generator is rated at 1.5 MVA,   11 kV.  The armature effective resistance and synchronous reactance per phase are 1.2 Ω and 25 Ω respectively. Calculate the % voltage regulation for a load of 1.4 MVA at 0.8 p.f lagging and leading.

[P.T.O.]

22.	What do you mean by synchronizing of alternators? State the conditions and then explain any one method of synchronizing.

(OR)

23.	The slip test was performed on a 3-phase, 415 V, star-connected alternator. The ammeter fluctuates between 4.5 A and 7 A and the voltmeter connected across the lines fluctuates between 87 V and 98 V. Estimate the direct-axis and quadrature-axis synchronous reactances. The effective resistance per phase is 0.8 Ω.

24.	 Discuss briefly why synchronous motors are inherently not self starting. Mention different starting methods and describe any one method.

(OR)

25.	Obtain expressions for the power input and power developed of a synchronous motor in terms of load angle.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTRICAL MACHINE DESIGN						      Time : 3 hours

Subject Code:	12EE211				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define gap contraction factor for ducts.

2.	What is fringing?

3.	Define commutator pitch.

4.	Write down the pitchelmayer’s equation.

5.	Mention clearly the condition for maximum efficiency for a transformer.

6.	Write down the core area factor kc for a square core.

7.	What is the advantage of having wound rotor construction?

8.    Where does the mush winding used?

9.	What is run-away speed?

10.	In alternators, when length of air gap is increased then stability _________.

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss the parameters governing the selection of conductor dimensions.

12.	What are the effects of armature reaction?

13.	Draw the cross section of different types of transformer cores.

14.	What are the factors which influence the power factor of an induction motor?

15.	Mention the uses of damper windings in synchronous machines.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the ampere turns/pole required to drive the flux through the teeth using Simpson’s rule with the following data: flux/pole = 0.07Wb, core-length = 35cm, number of ducts = 4, width of each duct = 1.0cm, slot pitch at the gap surface = 2.5cm, slot pitch at the root of the tooth = 2.3cm, dimensions of the slot = 1.2cm x 5cm, slots/pole-pitch = 12.

(OR)

17.	a.	Discuss in detail about the unbalance magnetic pull.                                                          (8)

b.	Calculate the apparent flux-density at a particular section of a tooth from the following 	data. Tooth width = 12mm, slot width = 10mm, gross core length = 0.32mm, number of 	ventilating ducts = 4, width of the duct each = 10mm, real flux density = 2.2T, 	permeability of teeth corresponding to real flux density = 31.4x10-6H/m. Stacking factor = 	0.9.                                                                            								    (7)

18.	a.	List out the procedure involved in design of shunt field winding and series field winding.

(10)

b.	Discuss the choice of poles and speed in DC machine.                                                       (5)

(OR)

19.	A 20 Hp, 4 pole, 250V, 1000 rpm wave wound D.C. machine has the following design data. Diameter of the armature = 25 cm, number of slots = 41, number of coil sides / slot = 4, turns / coil = 2. Calculate the number of segments, outside width of one segment and mica, brush thickness, length of the commutator and brush contact loss.

[P.T.O.]

20.	a.	Derive the voltage per turn equation of a transformer.                                                        (7)

b.	Explain how to estimate the no-load current of a three phase transformer.                        (8)

(OR)

21.	Calculate the no-load current of a 220/110V, 1kVA, 50Hz, Single phase transformer with the following data uniform cross-sectional area of the core = 25 cm2, effective magnetic core length = 0.4m, core weight = 8 kg, maximum flux density = 1.2 T, magnetizing force = 200 AT/m, specific core loss = 1.0 W/kg.

22.	A 15 kW, 440 volts, 4 pole, 50 Hz, 3 phase induction motor is built with a stator bore of 0.25 m and a core length of 0.16 m. The specific electric loading is 23000 ac/m. Using data of this machine determine the core dimensions, number of slots and number of stator conductors for a 11kW, 460 volts, 6 pole, 50 Hz motor. Assume full load efficiency of 84 % and power factor of 0.82. The winding factor is 0.955.

(OR)

23.	Following design data have been obtained during the preliminary design of a 3 phase, 850 kW, 6.6 kV, 50 Hz, 12 pole slip ring induction motor. Gross length of stator core = 45 cm, internal diameter of the stator core = 122 cm, number of stator slots = 144, Number of conductors per slot = 10. For the above stator data design a wound rotor for the motor.

24.	Determine the main dimensions of a 75 MVA, 13.8 kV, 50 Hz, 62.5 rpm, 3 phase star connected alternator. Also find the number of stator slots, conductors per slot, conductor area and work out the winding details. The peripheral speed should be less than 40 m/s. Assume average gap density as 0.65 wb/m2, Specific electric loading as 40,000 AC/m and current density as 4 amp/ mm2.

(OR)

25.	Design the stator frame of a 500 kVA, 6.6 kV, 50 Hz, 3 phase, 12 pole, star connected salient

pole alternator, giving the following informations.

(a) Internal diameter and gross length of the frame

(b) Number of stator conductors

(c) Number of stator slots and conductors per slot

Specific magnetic and electric loadings may be assumed as 0.56 Tesla and 26000 Ac/m respectively.

Peripheral speed must be less than 40 m/s and slot must be less than 1200.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTRIC DRIVES AND CONTROL					      Time : 3 hours

Subject Code:	12EE214				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In centrifugal pumps the load torque varies _______ .

a.	proportional to speed 				b.	square of the speed

c.	cube of the speed 				d.	all of the above

2.	The operation of  II quadrant is _________.

a.	Forward motoring 				b.	forward braking

c.	Reverse motoring 				d.	reverse braking

3.	Fan is the example for _________.

a.	intermittent duty 					b.	short time duty

c.	continuous duty 					d.	both a and b

4.	The method of determining motor rating from the load torque is called ____

a.	method of equivalent power 		b.	method of equivalent torque

c.	method of equivalent  current 		d.	all of the above

5.	Dynamic braking also called

a.	Regenerative braking 				b.	rheostatic braking

c.	reverse voltage braking			d.	mechanical braking

6.	Four quadrant operation of DC motors is obtained ____fed drives.

a.	half wave converter 				b.	dual converter

c.	full converter 					d.     semi converter

7.	The star delta method of speed control allows the reduction of torque by

a.	1/3    		b.	1/2    		c.	(1/3)1/2   d.	(1/2)1/2

8.	Rotor resistance control is applicable for

a.	squirrel cage induction motor 	b.	wound rotor induction motor

c.	single phase induction motor 	d.	All of the above

9.	Synchronous motor drive can be fed from

a.	voltage source inverter 			b.	current source inverter

c.	cycloconverter 					d.	All of the above

10.	Brushless DC Motor  is also known as _____

a.	Permanent magnet AC motor 		b.	commutator less DC motor

c.	DC motor 						d.	both  b and c.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the load torque equation in terms of their components.

12.	How to control speed of multi motor drive?

13.	Derive the speed torque equation of DC machine.

14. 	List the different methods of speed control of three phase induction motor.

15.	Write the different types of stepper motor.

[P.T.O.]

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Explain the speed torque characteristics for four quadrant operation of drives.		    (8)

b.	Determine the moment of inertia of flywheel for load equalization.                    		    (7)

(OR)

17.	a.	Explain the choice of electric drives for given load.                                                	    (5)

b.	How to find steady state stability of system? Explain with one example.            		  (10)

18.	Obtain the heating and cooling curve with necessary derivation.

(OR)

19.	a.	What is current limit control for electric drive? Explain.                                        		    (5)

b.	Write a brief note on classes of duty for an electric motor.                                   		  (10)

20.	A 230V, 1500rpm, 11.6 A separately excited motor is controlled by a 1-phase fully  controlled rectifier with an ac source of 230V,50HZ. Enough filter inductance is added to ensure continuous conduction for any torque greater than 25 percent of rated torque, Ra=2 ohms.  What should be the value of firing angle to get the rated torque at 1000rpm and also Calculate the firing angle for the rated braking torque and -1500rpm.

(OR)

21.	Explain the operation of chopper fed  separately excited motor with waveforms.

22. 	Explain the working of following methods with neat circuit diagram. (a) static Kramer system (b) static Scherbius system.

(OR)

23.	What is stator voltage control of induction motor? Explain the operation of three phase voltage source inverter fed induction motor drive.

24.	Explain the operation of self controlled synchronous motor drive fed from voltage source inverter with neat circuit diagram.

(OR)

25.	Write short notes on (a)solar powered pump drives   (b)battery powered vehicles.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	POWER SYSTEM ANALYSIS							      Time : 3 hours

Subject Code:	12EE216				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define unit system.

2. 	What is one line diagram?

3. 	List the types of faults.

4. 	What is sequence impedance matrix?

5. 	Define bus.

6. 	Give the importance of slack power bus.

7. 	What is transmission loss?

8. 	Give the importance of Economic scheduling.

9. 	What is steady state?

10.	Define Transient stability.

PART – B  (5 x 3 = 15 MARKS)

11. 	What is a symmetrical component?

12. 	What is Primitive network?

13. 	Define line flow.

14. 	What is optimal load scheduling?

15. 	What is voltage sag?

PART – C  (5 x 15 = 75 MARKS)

16.  Explain the Need for System analysis in planning and operation of power system.

(OR)

17.  	Explain the Short circuits analysis for fault on machine terminals.

18.  Give the   Algorithms for fault calculations   in detail.

(OR)

19.  Discuss the Unsymmetrical fault analysis using Zbus in detail.

20.  Discuss the bus classification in detail.

(OR)

21.  Give the Solution by Gauss – Seidal with an example.

22.  Give the Loss formula derivation in detail.

(OR)

23. 	Discuss the Economical scheduling of thermal plant with loss in detail.

24.  Give the importance of sags and swells in detail.

(OR)

25.  	Give the International standards of power quality.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	POWER SYSTEM PROTECTION AND SWITCHGEARS

Time : 3 hours

Subject Code:	12EE218						 			              Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.  	A wave will not experience any reflection when impedance of line is equal to ________ Impedance.

2.  	Relay contacts are normally made up of ________Contacts.

3. 	Breaking capacity of the circuit breaker is usually expressed in ________.

4. 	Arc chutes are used in a. a). Oil circuit breaker. b) Vacuum Circuit breaker c) Air blast circuit breakers.

5. 	Which curve in the figure represents inverse time characteristics?

6. 	Ionization in a circuit breaker is not facilitated by

a) High temperature of surrounding medium b) material of contacts c) increase of field strength

d) Increase of mean free path

7. 	Which circuit breaker is preferred for extra high voltage lines?

8. 	In a circuit breaker the current which exists at the instant of contact separation is known as

a) Re striking current b) surge current c) breaking current

9. 	Over-current protection for motor is provided by

a) Cartridge fuses b) high resistance fuses c) over-current relay

10.	 Which of the following are the voltage waves of magnitude higher than the desirable value?

a) Over-voltages b) Surges c) Transients d) All of the above.

PART – B  (5 x 3 = 15 MARKS)

11. 	What is a surge absorber? How do they differ from surge diverter?

12. 	Define the term ‘Insulation Coordination.

13. 	Give the advantage of SF6 circuit breaker over Air blast circuit breaker.

14. 	Compare Static and Electromagnetic relay

15. 	What is meant by time-graded system protection?

PART – C  (5 x 15 = 75 MARKS)

16. 	Describe the construction and operation of an inductive type directional over current relay with

a neat diagram. Also explain its operational characteristics.

[P.T.O.]

(OR)

17. 	Explain the principle of distance relays stating clearly the difference between impedance relay,

Reactance and mho relay. Indicate the difference on R-X diagrams and show where each type is suitable.

18. 	a. 	Explain the factors which cause difficulty in applying Merz-Price circulating current 				principle to a power transformers.                                                                                       (7)

b.	A three phase transformer of 220/11000 Line Volts is Connected in star / delta. The 				protective transformers on 220 V side have a current ratio of 600/5. What should be the 			current transformer ratio on 11000 V side?                                                                    	    (8)

(OR)

19.     Describe the differential pilot wire method of protection of feeder.

20. 	a. 	Explain how arc is initiated and sustained when the circuit breaker contacts break. 	    (5)

b. 	Explain in detail the various methods of arc extinction in a circuit breaker.       		  (10)

(OR)

21. 	a. 	What is current Chopping? Explain how can the effect of current chopping be minimized?

(8)

b. 	Derive an expression for the Rate of Rise of Restriking Voltage (RRRV) in a Circuit 	breaker.  																	    (7)

22.	Describe Protection against lighting and briefly describe about surge absorbers.

(OR)

23.	Write short notes on Overvoltage due to lightning.

24.	Discuss and compare the various methods of neutral earthing.

(OR)

25. 	a.  	Describe the essential qualities of a protective relaying.                                         	    (7)

b.  	Briefly explain the various methods of overvoltage protection of overhead transmission   			line.                           															    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	CONTROL SYSTEMS									      Time : 3 hours

Subject Code:	12EE219				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A ____________ is a control system in which the output is a mechanical position or time derivatives of position.

2.	What are the elements used to model an electrical system?

3.	Give the Laplace transform for unit step signal.

4.	Define steady state error.

5.	Define cutoff rate.

6.	The frequency at which the gain Gjω is equal to one is called ____________.

7.	 Define relative stability.

8.	What is an analytic function?

9.	Define state variables.

10.	How will you check the controllability of a system by Kalman’s test?

PART – B  (5 x 3 = 15 MARKS)

11.	Write the differential equations governing the mechanical system shown in figure.

B1

K2

K122

B2

B12

M1

M2

X2

X1

f

0)

K1

12.	The open-loop transfer function of a unity feedback servo system is given by

Gs=10s0.1s+1

Evaluate the static error constants for the system.

13.	Find the resonant peak Mr and resonant frequency ωrof a unity feedback system with

Gs=100ss+10

14.	State the principle of argument.

[P.T.O.]

15.	Obtain the characteristic equation of the matrix

A=010302−12−7−6

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the transfer function of a DC Generator.

(OR)

17.	For the signal flow graph shown in figure, obtain CsRsusing Mason’s gain formula.

G1

G2

G3

-H2

-H1

-H3

G4

G6

G5

18.	Determine time response specifications for a unit step input to a unity feedback system having

Gs=144ss+2

(OR)

19.	Draw the block diagram of a control system with proportional plus derivative controller and discuss its effects on the time response.

20.	Construct Bode plot for a unity feedback system having

Gs=2000ss+1s+100

Determine GM, PM, ωgcand  ωpc. Comment on the stability of the system.

(OR)

21.	Draw the polar plot of the function

Gs=8s+1s+2

22.	The characteristic equation of a system is given by s5+5s4+11s3+23s2+28s+12=0.    Determine the stability using Routh-Hurwitz criteria.

(OR)

23.	Sketch the root locus of the unity feedback system whose open loop transfer function is given by

Gs=Kss2+6s+25; and K varies from 0 to ∞.

24.	A feedback system has a closed-loop transfer function

YsUs=2s+5s+2s+3s+4

Construct the state model in phase variable form and canonical form.

(OR)

25.	Determine the transfer function of the system represented by the state model

x=−3−120x+10u

y=10x

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	LINEAR INTEGRATED CIRCUITS						      Time : 3 hours

Subject Code:	12EE221				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The unit for slew rate is ___________.

2.	CMRR stands for ___________.

3.	___________ is used in ramp generators.

4.	The differentiator circuit produces ___________ output for a square wave input.

5.	___________ is also called as regenerative comparator.

6.	A circuit used for rectifying signals whose peak to peak amplitude is below the cut in voltage of a diode is called as ___________.

7.	78XX series voltage regulators are ___________ three terminal regulators.

8.	In a VCO, the frequency can be varied by changing the ___________ voltage.

9.	___________ is the fastest ADC.

10.	___________ is used for frequency synthesis.

PART – B  (5 x 3 = 15 MARKS)

11.	List out the characteristics of an ideal operational amplifier.

12.	Mention the advantages of active filters over passive filters.

13.	Draw the circuit of a logarithmic amplifier.

14.	Mention the applications of IC 555 timer in Astable mode of operation.

15.	Define lock in range and capture range.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail about Dominant Pole compensation in an operational amplifier.

(OR)

17.	Explain in detail about the various DC and AC characteristics of an operational amplifier.

18.	Explain the working of a Second order Butterworth low pass filter with a neat circuit diagram. Derive its transfer function.

(OR)

19.	Explain in detail about the working of an instrumentation amplifier with a neat circuit diagram. Mention its applications.

20.	Explain the working of a Schmitt Trigger with a neat circuit diagram.

(OR)

21.	a.	Draw and explain the working of a Sample and Hold circuit.                 				    (7)

b.	Explain the working of analog multiplier with a neat block diagram.      				    (8)

22.	Explain in detail about the working of IC 723 adjustable voltage regulator with a neat block diagram.

(OR)

23.	Explain the functional block in IC 555 timer. Discuss the working of IC 555 in mono-stable mode of operation.

24.	Explain the working principle of a successive approximation type A/D converter with a neat block diagram.

(OR)

25.	Explain the working of a Phase Locked Loop (PLL) with a neat block diagram. Mention the applications of PLL.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	MICROPROCESSORS AND MICROCONTROLLERS	    Time : 3 hours

Subject Code:	12EE223		 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Why is the data bus bi-directional?

2.	What are the Control signals used for DMA operation?

3.	Compare the Microcontroller with Microprocessor.

4.	Define instruction cycle and Machine cycle.

5.	Specify the single instruction, which clears the most significant bit of B register of 8051, 	without affecting the remaining bits.

6.	Write a program to load accumulator DPH and DPL with 30H.

7.	How the RS-232 serial bus is interrupt to 1TL logic device?

8.	Explain the operating mode0 of 8051 serial ports.

9.	The 8279 is a programmable ____________interface.

10.	What is setting or conversion time in DAC?

PART – B  (5 x 3 = 15 MARKS)

11.	What is the use of addressing modes?  Mention the different types.

12.	Name the special functions registers available in 8051.

13.	Write about the jump instruction in 8051.

14.	Name the five interrupt sources of 8051.

15.	What are the basic modes of operation of 8255?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Draw the signal  Pin configuration of 8085 and explain the purpose of each signals.

(OR)

17.	Draw the architecture of 8085 and explain the each functional blocks.

18.	Discuss the internal architecture of 8051Microcontroller.

(OR)

19.	Describe the different types of Memory mapped in 8051.

20.	Explain the different operand addressing modes in 8051 with examples.

(OR)

21.	Discuss about the following instructions in 8051.

a.	Logical 		b.	Compare  	c.	Rotate 		d.	Call

22.	Explain the serial port operation in 8051 microcontroller.

(OR)

23.	Describe the different types of interrupt structure in 8051.

24.	Draw the block diagram of 8255programmable peripheral interface and explain each block.

(OR)

25.	Explain the internal blocks of 8279 Programmable Keyboard/display interfacing device.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	DIGITAL SIGNAL PROCESSING						      Time : 3 hours

Subject Code:	12EE225				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Dynamic systems.

2.	What is an Energy and Power signal?

3.	Define DFT.

4.	What is  Zero padding?

5.	What are the various methods to design IIR filters?

6.	What is prewarping?

7.	What is the reason that FIR filter is always stable?

8.	What is Gibb’s phenomenon?

9.	What is decimation and interpolation?

10.	Write notes on special purpose DSP processors.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the steps involved in digital signal processing?

12.	List the properties of DFT and explain.

13.	What is bilinear transformation?

14.	Write the procedure for designing FIR filter using frequency-sampling method.

15.	Write the applications of Adaptive filter.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail about the classification of discrete time systems.

(OR)

17.	Give the classification of signals and explain in details.

18.	An 8-point discrete time sequence is given by x(n) = {2,2,2,2,1,1,1,1}. Compute the 8-point DFT of x(n) using radix-2 FFT algorithm.

(OR)

19.	Explain about the types of Wavelet transform with neat diagram.

20.	Describe the bilinear transformation methods used for designing digital IIR filters.

(OR)

21.	An analog filter has a transfer function H(s)=10s2+7s+10. Design a digital filter equivalent to this impulse invariant method.

22.	Explain the design of FIR filters using  the Hamming, Bartlett and Kaiser windows.

(OR)

23.	Design an ideal high pass filter with

Hd(ejw) = 1 for π/4 ≤ ω ≤ π

= 0 for ω ≤ π/4    Using Hamming window for N=11.

24.	Describe in detail the architecture of TMS320C54 digital signal processor with block diagram.

(OR)

25.	Discuss in detail about the speech compression and image enhancement techniques.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	C++ AND DATA STRUCTURES							      Time : 3 hours

Subject Code:	12EE227				 					       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is data structures? Give example.

2.	Give example of self-referential structure used for node creation.

3.	What is the time complexity of bubble sort?

4.	List the advantages of binary search over linear search.

5.	Write declaration in C++ for creating a string array or array of strings.

6.	What is the advantage of copy constructor?

7.	List the operators that cannot be overloaded.

8.	List different access specefier that are available in C++.

9.	What is template?

10.	List types of files that C++ functions can handle.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the advantages of doubly linked list over singly linked list?

12.	What is Min Heap and Max Heap?

13.	Write the significance of ‘this’ pointer.

14.	Give example for run time polymorphism.

15.	Compare errors and exceptions.

PART – C  (5 x 15 = 75 MARKS)

16.	Implement stack operations using singly linked list.

(OR)

17.	Write a C++ program to implement circular linked list and perform insert and delete operations. 18.	Write a C++ program to implement quick sort algorithm? And explain it with example.

(OR)

19.	Write C++ program to implement Binary search algorithm.

20.	Briefly explain the concept of object oriented programming.

(OR)

21.	Write a C++ program to add two single dimensional array which contains ‘n’ real numbers.

22.	Explain different types of inheritance available in C++ programming language.

(OR)

23.	Write a C++ program to perform complex number addition using operator overloading.

24.	Write the need for friend function? With an example explain friend function in detail.

(OR)

25.	Write a C++ program to copy contents from one text file to other text file.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	MEASUREMENT AND INSTRUMENTATION			      Time : 3 hours

Subject Code:	12EE229				 						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is primary sensing element?

2.	Define static error.

3.	State calibration.

4.	Mention the different types of wattmeters used for power measurement.

5.	Which are used as detectors in the a.c. bridges?

6.	What is the range of low resistance?

7.	List any two applications of potentiometer.

8.	Define gauge factor of strain gauge.

9.	List any two advantages of digital voltmeter.

10.	Point out the triggering methods of CRO.

PART – B  (5 x 3 = 15 MARKS)

11.	Compare moving coil and moving iron instruments.

12.	How the voltage and current coils are connected in induction type energy meter with the load?

13.	List the errors in an AC bridge.

14.	Define transducer.

15.	Point out the advantages of digital voltmeter.

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat diagram explain the principle and construction of PMMC instruments.

(OR)

17.	Illustrate the principle of operation of A.C. galvanometer.

18.	Explain with neat sketch the construction and working principle of Electrodynamometer type wattmeter.

(OR)

19.	With a neat figure explain the principle of operation of a single phase energy meter.

20.	Illustrate the construction and working of Wheatstone bridge.

(OR)

21.	Explain with phasor diagram the construction and working of Schering’s bridge.

22.	Elucidate the working principle and operation of a typical AC potentiometer with a neat diagram.

(OR)

23.	Explain the construction and working of a Linear Variable Differential Transformer with a neat diagram.

24.	Illustrate the construction of a storage type oscilloscope. What are the accessories for a CRO?

(OR)

25.	a. 	Analyze the operation of RF spectrum analyzer.									    (7)

b. 	Compare Parallel filter bank analyzer and spectrum analyzer.						    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	BASICS OF ELECTRIC AND HYBRID VEHICLE	      	     Time : 3 hours

Subject Code:	12EE252						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Fuel cell vehicles use hydrogen gas to power a _________.

2.	Converting hydrogen gas into electricity produces only water and _________as a by product.

3.	Unit of electro - chemical equivalent of the substance of electrolyte is  _________.

4.	Negative electrode or anode of simple voltaic cell is made of Zinc. Yes/No

5.	E.C.E stands for Electro chemical equivalent. Yes/No.

6.	Name any one type of single phase Induction motor.

7.	Give any one advantage of fuel cell.

8.	BLDC motor has a Brush in its rotor. True/False

9.	In radial tyres all plies run _________.to one another and vertical to tyre bead

10.	Give one advantage of power steering.

PART – B  (5 x 3 = 15 MARKS)

11.	Write about the Principle of operation of flywheel.

12.	What is a hybrid vehicle?

13.	Write about hydrogen storage.

14.	Name the types of DC motor.

15.	Write any two factors taken into consideration while choosing tires.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain about solar powered vehicles.

(OR)

17.	Write short notes on

a.	Hybrid vehicle. 																(7)

b.	 Super capacitors. 															(8)

18.	Discuss in details about various batteries used in Vehicles.

(OR)

19.	State the cooling and heating needs of  batteries.

20.  Write short notes 	a.	Solar Photovoltaic sources.                                                   		(8)

b.	Wind power                                                                            		(7)

(OR)

21.  Discuss in details about hydrogen storage methods.

22.  Describe the construction and operation of Induction motor.

(OR)

23.  Explain the construction and principle of Operation of switched reluctance motor.

24   Describe about controls in power steering and choice of tires.

(OR)

25.  Illustrate how charging and refuelling is done in Electric vehicles.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTRIC CIRCUITS AND NETWORKS			      	     Time : 3 hours

Subject Code:	10EE201						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How are the circuit elements classified?

2.	State kirchoff’s current law.

3.	Write the condition for maximum power transfer.

4.	What is meant by non-linear performance?

5.	What is forced response?

6.	Write the significance of reactive power.

7.	What is tuned circuit?

8.	Define Quality factor.

9.	Write the necessity of two port network.

10.	Write the function of filter.

PART – B  (5 x 3 = 15 MARKS)

11.	Describe the importance of dual network.

12.	State and justify reciprocity theorem.

13.	Enumerate the significance of poles and zeros of the network function.

14.	Write the procedure to solve the unbalanced circuit.

15.	Describe the concept of impedance matching.

PART – C  (5 x 15 = 75 MARKS)

16. 	Calculate a) the equivalent resistances across the terminals of the supply, b) total current supplied by the source and c) power delivered to 16 ohm resistor in the circuit shown in the figure.

(OR)

17.  Describe the step by step procedure to formulate Cut set matrix for a sample network.

| 18. For the circuit shown, use superposition theorem to compute current I. |  |
| --- | --- |

[P.T.O.]

(OR)

19.  Find the value of RL so that maximum power is delivered to the load resistance shown in 	figure.

20. 	A Series RLC circuits has R=50 ohm, L= 0.2H, and C = 50 microfarad. Constant voltage of 100V is impressed upon the circuit at t=0. Find the expression for the transient current assuming initially relaxed conditions.

(OR)

21. In the circuit shown in figure, switch S is in position 1 for a long time and brought to	position 2 at time t=0. Determine the circuit current.

www.vidyarthiplus.com	EE 1151 –CIRCUIT THEORY

22.	a.	A balanced three phase star connected load with impedance 8+j6 ohm per phase is 	connected across a symmetrical 400V three phase 50Hz supply. Determine the line current, 	power factor of the load and total power.

b.	An alternating current is expressed as i=14.14 sin 314t. Determine rms current, frequency and instantaneous current when  t =0.02ms.

(OR)

23.	With a neat circuit and phasor diagram explain the three phase power measurement by two wattmeter method.

24.  For the two port network shown in Figure, determine the driving point impedance Z11(s) and 	the driving point admittance Y11(s). Also find the transfer impedance Z21(s).

0

0

2

1
0

(OR)

25. 	Design the m-derived T and π section high pass filter with a cut off frequency of 10 KHz with 	design impedance of 500 ohms and m = 0.4.