Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTRICAL MACHINES								      Time : 3 hours

Subject Code:	09EI220				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the torque equation of a DC motor.

2.	List out the various types of losses occurs in a DC machine.

3.	Define a transformer.

4.	Define voltage regulation in a transformer.

5.	Draw the torque-slip characteristics of induction motor.

6.	What is the usage of V curves in synchronous motor?

7.	State double field revolving theory.

8.	Mention the usage of tachogenerator.

9.	What is voltage level to be used in Indian electric traction?

10.	High frequency induction heating is used for ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	How back EMF occurs in a DC motor?

12.	Draw the equivalent circuit in a transformer.

13.	Why synchronous motor is not a self starting device?

14.	Mention the types of single phase induction motors.

15.	Write the operating principle of welding generator.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw and explain the various characteristics of DC generators.

(OR)

17.	Sketch and explain the various speed control methods adapted to DC shunt motor.

18.	Explain the open and short circuit test of a transformer with relevant experimental circuits.

(OR)

19.	a.	Derive the EMF equation of a transformer.										    (8)

b.	Describe the construction details about transformer.								    (7)

20.	Explain the various speed control schemes available for induction motors.

(OR)

21.	Explain in detail the construction and operating principle of a synchronous motor.

22.	Explain the construction and working of AC and DC servo motors.

(OR)

23.	a.	Discuss the working of stepper motor. 											    (8)

b.	Describe about linear induction motor.    		 								    (7)

24.	Draw and explain the various types of electric heating.

(OR)

25.	Discuss about the traction motor control and current scenario in electric traction.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTRONIC CIRCUITS							      	     Time : 3 hours

Subject Code:	10EI202											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Draw the symbol of Diode.

2.	Write the equation of rectification efficiency.

3.	List the specifications of the transistors.

4.	Give the typical value of hie.

5.	What is a Push-Pull amplifier?

6.	Write the voltage gain of the RC Coupled amplifier.

7.	Write the expression for Vgs.

8.	Write the expression for iD.

9.	Draw the waveform for Monostable output.

10.	Negative feedback increases the circuit gain. State True or False.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the Ripple Factor in waveform and give its analysis.

12.	Draw the circuit and explain Emitter follower configuration.

13.	Draw the circuit diagram of Class AB amplifier

14.	Write a note on Biasing FET amplifiers.

15.	State and explain Barkhausen criteria.

PART – C  (5 x 15 = 75 MARKS)

16.	Design the half and full wave rectifier. Specify the equation and draw the waveforms. Why half wave is different from full wave?

(OR)

17.	a.	Analyze the implementation of Positive and Negative Clipper with respective waveforms.

(10)

b.	What is the Average Value and RMS value? Explain the terms.					    (5)

18.	a.	State and Explain Millers Theorem.												  (10)

b.	Draw the block diagram and explain Cascading transistor amplifiers.				    (5)

(OR)

19.	Do the elaborative Analysis of transistor amplifier using h parameters.

20.	Design the single and two stage RC amplifier. Derive the equations for the same.

(OR)

21.	Explain the class A, B and C amplifiers.

22.	Derive the FET small signal model with circuit diagrams.

(OR)

23.	Explain the terminologies of Low FET (NMOS) and BIFET Amplifiers in detail.

24.	Analyze the RC and LC oscillators in detail.

(OR)

25.	What is the difference between Mono-stable and astable operation? Give detailed explanation with circuits diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	SENSORS AND TRANSDUCERS						      Time : 3 hours

Subject Code:	10EI203				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define calibration.

2.	Classify the instruments.

3.	Point out the primary sensing elements.

4.	Define transducer.

5.	List the merits and demerits of compensation circuits.

6.	State Linearity.

7.	List any two properties of piezoelectric crystals.

8.	Point out the applications of capacitance transducer.

9.	Define smart sensors.

10.	Point out the applications of semiconductor IC sensors.

PART – B  (5 x 3 = 15 MARKS)

11.	Compare the direct and indirect measurement methods.

12.	List the factors influencing choice of transducer.

13.	Outline the salient features of bonded and unbounded strain gauges.

14.	State the principle of capacitance transducer.

15.	Describe the working of pH sensor.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the static and dynamic characteristics of Instruments.

(OR)

17.	Outline the various types of errors possible in an instrument.

18.	Analyze the I order mathematical model of transducer with response to step, ramp and sinusoidal inputs.

(OR)

19.	Analyze the II order mathematical model of transducer with response to impulse, ramp and sinusoidal inputs.

20.	Illustrate the construction and working characteristics of thermistor and thermocouple.

(OR)

21.	a.	Explain the working of Linear variable differential transformer.					    (8)

b.	Outline the torque measurement on rotating shafts.								    (7)

22.	Explain the working principle and operation of a capacitance transducer with a neat diagram.

(OR)

23.	With a neat sketch, explain the construction and working of Magnetostrictive Transducer and Hall effect transducer.

24.	Explain the working principle and operation of digital transducers in detail.

(OR)

25.	Analyze the working of the following sensors

a.	Smart sensors.		b. 	Ultra sonic sensors.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	SIGNALS AND SYSTEMS							      	     Time : 3 hours

Subject Code:	10EI204						 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define System.

2.	What are the two types of signals?

3.	Write the second order difference equation.

4.	Give an example of a discrete system.

5.	What is a LTI system?

6.	What is Fourier transform?

7.	Check whether the given system y = 2x is periodic.

8.	What is power density of a signal?

9.	What is the Laplace transform of ‘sin(at)’ ?

10.	What is Z transform of ‘exp( at)’?

PART – B  (5 x 3 = 15 MARKS)

11.	Define even and odd signals.

12.	What is continuous time real exponential signal?

13.	Define Fourier transform pair.

14.	Find the Laplace transform of f( t) = t2  e -2x cos(3t )?

15.	Give the expression for Inverse Z transforms.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the types of signals with examples.

(OR)

17.	a.	Write any three properties of the system.										    (5)

b.	For the following signals, determine analytically which are periodic

(if periodic, give the period   	i.	x(t) = 4 cos(5πt)

ii.	x(t) = 4 cos(5πt–π/4)							  (10)

18.	a.	Explain a continuous system with example and sketch it. 							    (8)

b.	Explain the time domain representation of a discrete system. 						    (7)

(OR)

19.	a.	Explain the discrete time representation of a left sided sequence.					    (7)

b.	The continuous time system consists of two integrators and two scalar multipliers. Write a 	differential equation that relates the output y(t) and the input x( t ).					    (8)

20.	Derive the Convolution theorem for two signals u(t) and v(t).

(OR)

21.	 Derive Parsevals theorem for x(f) = ∂(f –g).

22. 	Derive the  2 point DFT Algorithm.

(OR)

23.	For the given long series of data u(t) how to detect the periodic signal by using DFT? Derive.

24.	Consider two functions f(t) =h(t)  h(3–t) and g(t) = h(t) – h(t–3). Are these two functions are identical? Show that Laplace transform of f(t) =Laplace transform of  g(t).

(OR)

25.	Find the Z transform of a signal −anu−n−1 and the ROC.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	SIGNAL CONDITIONING CIRCUITS				      	     Time : 3 hours

Subject Code:	10EI206						 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define an ideal op-amp.

2.	Give one application of Inverting Amplifier.

3.	What are the types of comparator?

4.	Give one important feature of Instrumentation amplifier.

5.	What is Schmitt trigger?

6.	Give the expression for transconductance.

7.	What are the two types of filters?

8.	What is frequency doubling?

9.	Suggest two demodulation techniques.

10.	What is PLL?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the circuit of an Integrator.

12.	Give three applications of a comparator.

13.	What is a lead lag filter? Draw the circuit diagram.

14.	Define resolution of a data converter.

15.	Draw the frequency multiplier circuit.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What are the characteristics of an ideal op amp? 									    (7)

b.	Define Slew Rate. Explain the cause of slew rate and derive an expression for Slew rate for an op-amp voltage follower.													    (8)

(OR)

17.	Draw the differential amplifier configuration. In that if V1 = 1V, find output voltage V0 if the same circuit is converted as an inverting and non-inverting amplifier configurations.

18.	Explain the op amp as half and full wave rectifiers.

(OR)

19.	Design the op-amp circuitry to provide the output voltage in the range of 0-5v

For the input range of 0-20mA.

20.	a.	Design a first order Low-pass filter for cut-off frequency of 2 KHz and pass-band gain of2. 																			   	    (8)

b.	Explain the Instrumentation Amplifier. 											    (7)

(OR)

21. 	Explain state variable filter and switched capacitor filter.

[P.T.O.]

22.	Refer to the given figure. If the zener diode had a rating of 1.7 V, calculate the output voltage

(OR)

23.	Explain the four quadrant multiplier.

24. 	a.	Explain the working of monolithic PLL.											    (8)

b.	Explain any one application of PLL. 				 							    (7)

(OR)

25.	a.	How is voltage regulators classified? Explain a series voltage regulator?			    (8)

b.	What is a sample and hold circuit? Briefly explain its construction and application. 	    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	MICROPROCESSORS AND MICROCONTROLLERS    	     Time : 3 hours

Subject Code:	10EI207						 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The demultiplexing of data lines from low order address lines in 8085 is due to  _________.

2.	Which of the flag I 8085 is responsible for BCD operation?

3.	Name the register that deals with the sequencing the execution of instructions in 8085.

4.	Specify the output at Port A if the following program is executed.

MVI B, 28H

MOV A, B

MOV C, A

MVI D, 37H

OUT PORT1

HLT

5.	What is the control word for setting Bit 6 of Port C in 8255?

6.	Calculate the value of full scale output for an 8 bit DAC for the 0v to 10v range.

7.	________ and ________ bits of PSW are used to select the register bank in 8051.

8.	The signal that reads external ROM in 8051 is ________.

9.	What would be the typical conversion time of an 8 bit flash ADC?

10.	Give an example for intelligent alphanumeric display.

PART – B  (5 x 3 = 15 MARKS)

11.	List the sequence of events that occur when the 8085 CPU reads from memory.

12.	What is the significance of RST instructions in 8085?

13.	Give the flowchart of A to D conversion process.

14.	Draw the internal RAM architecture of 8051.

15.	Write down three main features of D to A converters used with 8051.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the internal architectural diagram of 8085 CPU and explain the function of each block in detail.

(OR)

17.	Draw the timing diagram for execution of the instruction MVI A, 32H

18.	a.	What are the addressing modes supported in 8085? Give atleast two examples for each 	category.																	  (10)

b.	List the needs for DMA.														    (5)

(OR)

19.	a.	Write an ALP in 8085 to sort out the odd and even numbers separately in the given 	memory starting at 8000H address.												    (9)

b.	Draw the schematic that represents the 8085 interrupts with their priority, mask, triggering and vector locations.															    (6)

[P.T.O.]

20.	a.	Write an ALP in 8085 to generate the square wave of 1.5 KHZ at port A of 8255. Show the 	delay calculation for 3 MHZ operating frequency. Support your answer with a flow chart.																				           (8)

b.	Explain the different modes of operation of 8255.								    (7)

(OR)

21.	Design a stepper motor interface and illustrate the hardware diagram with its operating principle and the corresponding ALP / C code.

22.	Discuss the internal architecture of ports in 8051 and elaborate their operation.

(OR)

23.	a.	How do the timers of 8051 operate in different modes in 8051? 					  (10)

b.	Write an ALP in 8051 to convert grey code into equivalent Binary code.			    (5)

24.	Design a 4x 4 matrix key board interface for 8051. Give out the hardware interface diagram and the code that runs on the hardware.

(OR)

25.	Measure the pulse duration of the unknown input pulses received, and find out the pulse frequency. Obtain the 8051 based hardware design to perform the task and write the supporting software to complete the task.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	INDUSTRIAL INSTRUMENTATION					      Time : 3 hours

Subject Code:	10EI209				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List out any two temperature scales.

2.	Mention any one advantage and limitation of RTDs.

3.	Define manometers.

4.	Are Bourdon tubes also differential pressure gauges? Are they so used?

5.	Where is a thermal level gauge suitable?

6.	Give one practical example of ultrasonic level measurement.

7.	Define mass flow rate

8.	What is vortex?

9.	Define gas densitometers.

10.	What are the usual units for expressing viscosity?

PART – B  (5 x 3 = 15 MARKS)

11. 	Why are pressure measurements so important in Industrial Instrumentation?

12. 	List the different units of flow.

13. 	What are the steps involved in calibrating a thermometer?

14.  	How is a liquid level in a boiler drum measured?

15. 	Differentiate ‘viscosity’ and ‘consistency’ with reference to a flowing fluid.

PART – C  (5 x 15 = 75 MARKS)

16. 	Describe ionization gauge. How does it differ from the pirani gauge? What disadvantages does it have?

(OR)

17. 	Explain the following 	(a) Dead weight tester (b) Manometers.

18. 	Explain the principle of operations of an electromagnetic flow meter. Discuss its merits.

(OR)

19. 	Explain the two types of hot wire anemometers.

20.	 With the help of neat sketch, explain the sectional view of a radiation pyrometer using concave mirror.

(OR)

21. 	Discuss the various types of  errors encountered in radiation pyrometers

22. 	Describe the purpose and function of each block in the ultrasonic measurement system.

(OR)

23. A capacitance probe with a 5 pF/m characteristic is used in a level application. The bridge

component values for a particular application are :

C2 = 100 microfarad

R1 = 20 kῼ

R2 = 150 kῼ

R3 = 50 kῼ

Find the level in meters.

24. 	Describe the working of differential pressure continuous capillary viscometer.

(OR)

25. 	Briefly explain the operation of the following float viscometers (Rotameter type viscometers)

(a) Single float viscometer   (b) Two-float viscometer  (c) Concentric float viscometer

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title : BIOMEDICAL INSTRUMENTATION						      Time : 3 hours

Subject Code: 10EI211			 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are resting and action potentials?

2.	What is the frequency range of ECG, EEG and EMG waves?

3.	How does the blood circulate throughout the body?

4.	What is a synchronized d.c defibrillators?

5.	Why do we require heart-lung machine?

6.	Define pH meter.

7.	List out one difference between radiography and fluoroscopy.

8.	What is the radio frequency used in tunnel diode FM transmitter?

9.	What are microshocks and macroshocks?

10.	Justify the following statements: MRI is superior than other imaging statements.

PART – B  (5 x 3 = 15 MARKS)

11.	Give the names of the different systems in our body.

12. 	Briefly mention the uses of ECG.

13. 	Mention the limitations of CT scan.

14.	What is a ventilator? Give its importance in respiratory failure

15.	List out any four physical principles of blood flow meters based on research applications.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the neat sketch of central nervous system and explain the function of each parts.

(OR)

17.	With a neat block diagram, explain the working of an ECG machine.

18.	 Describe an ultrasonic blood flowmeter used in the measurement of velocity of blood flowing in the blood vessels.

(OR)

19.	What are oxymeter? Describe any one of the oxymeter with a suitable diagram..

20.	Describe any one of the flowmeters used to measure the flow rate of gases used in anesthesia machines.

(OR)

21.	Discuss the different modes of operation of cardiac pacemakers.

22.	Explain the principle of chromatography and its application in medicine.

(OR)

23. Draw the block diagram of an automatic blood cell counter and explain its functioning.

24. What is biotelemetry? Explain the working of single channel ECG telemetry system.

(OR)

25. Draw the block diagram of a CT and explain the different blocks in it.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	DIGITAL CONTROL SYSTEMS						      Time : 3 hours

Subject Code:	10EI213				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by sampling in discrete time system?

2.	Write the transfer function of ZOH.

3.	Obtain the Z transform of unit step input.

4.	Write the steady state error of a type-1 system with unit step input.

5.	What is meant by cascade realization?

6.	Write the control law for state feedback.

7.	How is left half of S plane mapped into Z plane?

8.	What is BIBO stability?

9.	Write the transfer function of Dahlin’s algorithm.

10.	What is ringing with respect to controllers?

PART – B  (5 x 3 = 15 MARKS)

11.	Write the significance of DAC in digital control system and list its types.

12.	State the heuristic rules to obtain Z domain equivalent of an S domain transfer function using pole- zero mapping method.

13.	Draw the state diagram of the following transfer function by direct realization.

H(z) = β1 z2 +β2z +β3z3+α1z2+α2z+α3

14.	Explain the principles of digital quantization.

15.	Write the advantages of block diagram study of digital implementation.

PART – C  (5 x 15 = 75 MARKS)

16.	State Shannon’s sampling theorem.  Explain the aliasing effect on sampling a band limited signal using sampling theorem.

(OR)

17.	a.	Draw the block diagram of a sampled data control system.  Discuss significance of various 	blocks involved.                                                                                               			    (7)

b.	Draw the functional block diagram of successive approximation ADC and explain how the 	conversion takes place.                                                                          				    (8)

18.	Obtain the response of the following sampled data system when subjected to step input. Assume T= 1 sec.

T

(OR)

[P.T.O.]

19.	a.	Find the inverse Z-transform of the function F(z)=4z2−2zz3−5z2+8z−4 .                      	(8)

b.	Determine the Modified Z transform of e−at.                                                              	(7)

20.	Check the state controllability and observability of the following system.                          (15)

x1(k+1)x2(k+1)x3(k+1) = 0100110−2−2 x1(k)x2(k)x3(k) + 001 u(k)

y(k) = 341  x1(k)x2(k)x3(k)

(OR)

21.	a.	Obtain parallel form realization for the system 	                                                 		    (8)

b.	Explain the design steps of output feedback control.                                                   	    (7)

22.	Consider the following characteristic equation of a discrete data system

Δ(z) = z4 − 1.2 z3 + 0.22 z2+0.066z − 0.008 = 0.

Check whether the system is stable or not using Jury’s stability test.

(OR)

23.	a.	Explain how bilinear transformation technique produces a stable discrete-time system for a

stable continuous-time system.                                                                                           (8)

b.	Discuss the theory of state sequences for sampled data systems                                       (7)

24.	a.	Explain in detail, positional  PID algorithm of   digital PID controller.                        	    (8)

b.	Explain the dead-beat algorithm for digital control implementation.                            	    (7)

(OR)

25.	Draw the interfacing circuit diagram of a stepper motor to microcontroller and explain the algorithmic steps to control the speed of the motor.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title : COMMUNICATION ENGINEERING						      Time : 3 hours

Subject Code: 10EI214			 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the advantage of modulation?

2.	What are the types of AM?

3.	What is the purpose of super heterodyne receiver?

4.	What is the use of low noise amplifier in receiver front end?

5.	What is the merit of differential PCM?

6.	Define ASK.

7.	What is the frequency range of operation of twisted pair cable?

8.	What are the sources of error?

9.	List the applications of Radar.

10.	What is chrominance signal?

PART – B  (5 x 3 = 15 MARKS)

11.	Derive the expression of modulation index of AM signal.

12.	Explain the effects of noise in FM system.

13.	Explain the process of getting PPM modulation.

14.	Explain the LASER operation.

15.	Narrate the operation of colour TV system.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive the expression of frequency spectrum of AM signal. 						  (10)

b.	Calculate the total power of AM signal when carrier power is1kW and modulation index is 0.23. 																  		    (5)

(OR)

17.	Discuss on vestigial side band AM concept, generation and its application.

18.	Discuss on the Industrial noise and various internal noises added to the communication   signals.

(OR)

19.	Discuss on the operation of any one of FM modulator and demodulator.

20.	a.	Compare the performance of PAM, PWM and PCM.  					     		  (10)

b.	Explain the features of FSK. 													    (5)

(OR)

21.	a.	Explain the generation of delta modulated signal. 								  (10)

b.	Explain PSK modulation?    													    (5)

22.	Discuss on co axial cable structure and explain how it carries electro magnetic signal .

(OR)

23.	Describe the operation of fiber optic communiation system with neat block diagram and its significances.

24.	Discuss on satellite communication system and its applications in detail.

(OR)

25.	Describe the operation and specifications of MODEM along with its applications.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	MODERN CONTROL SYSTEMS						      Time : 3 hours

Subject Code:	10EI218				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Find the equivalent transfer function of three parallel blocks G1(s) = 𝟏/(𝑺+𝟏),

G2(s) = 𝟏/(𝑺+𝟒) and G3(s) = (𝑺+𝟑)/(𝑺+𝟓)

2.	Compute the sum of  gains of the feedback paths in the signal flow graph shown in figure 1.

3.	What is the status of a system with gain margin close to unity or a phase margin close to zero?

4.	What kind of damping a system has, if its damping ratio is 1.25, a natural frequency of

200 rad/s and DC gain of 1?

5.	Find the gain margin in the Bode-plot of a unity feedback control system, where the value of magnitude of G(jω) at the phase crossover frequency is 1/2.

6.	Define the term, whose frequency at which the magnitude of the open loop transfer function is unity.

7.	Define the term whose path taken by the roots of the open loop transfer function, when the loop gain is varied from 0 to infinity.

8.	What is the necessary condition for stability of a system.?

9.	While designing controller, what is the advantage of pole–zero cancellation?

10.	Write the solution of non- homogeneous state equations?

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish between open loop and closed loop systems.

12.	Define any three time domain specifications.

13.	What are M and N circles?

14.	What is Routh stability criterion?

15.	What are the advantages of state space analysis?

PART – C  (5 x 15 = 75 MARKS)

16.	Write the differential equations governing the Mechanical rotational system shown in

figure. Draw the Torque-voltage and Torque-current electrical analogous circuits.

(OR)

[P.T.O.]

17.	Find the overall gain of the system whose signal flow graph is shown in figure.

18.	For a unity feedback control system the open loop transfer function

G(S) = 10(S+2)/ S2 (S+1). Find

a.	position, velocity and acceleration error constants.

b.	The steady state error when the input is R(S) where R(S) =3/S –2/S2 +1/3S3

(OR)

19.	The unity feedback system is characterized by an open loop transfer function is

G(S) = K / S(S+10). Determine the gain K, so that the system will have a damping ratio of 0.5. For this value of K, determine settling time, Peak overshoot and time to Peak overshoot for a unit-step input.

20.	Construct the polar plot for the function GH(S) =2(S+1)/ S2. Find Gain cross over frequency, Phase cross over frequency, Gain margin and Phase margin.

(OR)

21.	Construct Nyquist plot for a feedback control system whose open loop transfer function is given by G(S) H(S) =5/ S(1-S).Comment on the stability of open loop and closed loop transfer function.

22.	Sketch the root locus of the system whose open loop transfer function is

G(S) = K / S (S+2) (S+4). Find the value of K so that the damping ratio of the closed loop 	System is 0.5

(OR)

23.	a.	For F(S) = S6 +S5-2S4-3S3-7S2-4S-4 =0, Find the number of roots falling in the RHS 				plane and LHS plane.

b.	Construct Routh array and determine the stability of the system represented by the 				characteristics equation S7+9S6+24S4+24S3+24S2+23S+15=0.

24.	a.	Explain the importance of controllability and observability of the control system 				model in the design of the control system.

b.	Explain state space representation for continuous time system.	 	
                                                                          	(OR) 
25.	Test the controllability & observability of the system whose state space representation is given 	as,

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	C++ AND DATA STRUCTURES							      Time : 3 hours

Subject Code:	11EI217			 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	___________ function cannot have default parameters.

2.	___________ number of streams are automatically created when executing a program.

3.	A pointer variable stores an ___________of another variable.

4.	A string is a ___________of characters.

5.	A constructor that accepts ___________parameters is called the default constructor.

6.	Inheritance is used to ___________a class.

7.	State True or False. Linked lists are sequential data structures.

8.	The function used to remove an element from the queue is called as ___________.

9.	Quick sort is based on ___________ algorithm.

10.	Errors in a program are handled using ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Is C++ a superset of C? How?

12.	State the purpose of new and delete operators?

13.	What do you mean by this pointer?

14.	State whether Linked List is linear or Non-linear data structure.

15.	What is a bubble sort and how do you perform it?

PART – C  (5 x 15 = 75 MARKS)

16.	Write a C++ program to compute the sum of the elements of a floating point array.

(OR)

17.	Write a menu driven C++ program to perform the operations of a calculator.

18.	Explain the overloading of functions with a suitable example.

(OR)

19.	Explain the different operations that can be performed on strings with suitable examples.

20.	Describe the different types of inheritance with suitable examples.

(OR)

21.	What are friend and virtual functions? Explain with suitable examples.

22.	Explain the queue data structure with an example.

(OR)

23.	What is the need for a circular linked list? Explain with a suitable example.

24.	Explain selection sort with a suitable example application of the technique.

(OR)

25.	What is the need for a binary search tree? Explain with an example.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTRICAL MACHINES								      Time : 3 hours

Subject Code:	12EI202				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The purpose of motor starter is to limit the ___________.

2.	Write back E.M.F equation of a DC motor.

3.	Mention the different losses in a transformer.

4.	Transformer is a dynamic device.  Say True/False

5.	What is meant by slip?

6.	Mention the two types of induction motor rotor.

7.	Single phase induction motor is not a self starting motor.  Say True/ False

8.	List the application of repulsion type motor.

9.	State advantages of resistance electric heating.

10.	What type of motor is employed for traction purpose?

PART – B  (5 x 3 = 15 MARKS)

11.	List the applications of different types of D.C. Motors.

12.	What is the difference between shell and core type transformer?

13.	Draw the speed-torque slip characteristics of an Induction motor.

14.	Write short notes about tachogenerator.

15.	What is meant by Traction motor control?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in details about the working principle and theory of operation of D.C. generator with their constructional features.

(OR)

17.	Explain the followings:

a.	Methods of speed control of D.C. motor.				                				    	    (8)

b.	3-point motor starters. 							     							    	    (7)

18.	Develop the EMF equation of the transformer. Also draw the equivalent circuit of the transformer with necessary diagram.

(OR)

19.	Explain in details about the testing of transformer under no-load and load conditions with relevant diagrams.

20.	Explain the construction and working principle of operation of synchronous motor with relevant diagrams	.

(OR)

21.	Explain in details about the starting and speed control of a three phase induction motor.

22.	Explain in details about the different methods of starting of a single phase induction motor.

(OR)

23.	Write short notes about

a.	Universal motor.			    					      								    (8)

b.	Stepper motor. 								      								    (7)

24.	Explain in details about the various methods of electric heating with applications.

(OR)

25.	Explain the followings :

a.	Electric traction control system.	 			     								    (8)

b.	Recent trends in Electric traction.			                 							    (7)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTRONIC CIRCUITS								      Time : 3 hours

Subject Code:	12EI203				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A Zener diode is used as a ________.

2.	What is meant by Q-point in the load line?

3.	The purpose of ‘h’ parameters are ________.

4.	What is meant by amplifier?

5.	Mention the types of feedback amplifiers.

6.	Compare: class A and class B power amplifiers.

7.	Give an example for positive feedback device.

8.	How do generate non-sinusoidal waveforms?

9.	What is meant by small signal model?

10.	Purpose of biasing is ________.

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly explain about the clipping circuit.

12.	Draw the hybrid model for CE amplifier circuit.

13.	Difference between power amplifier and feedback amplifier.

14.	List the barkhausen criteria.

15.	Mention the different biasing methods of FET amplifier circuit.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail about the Half Wave and Full Wave Rectifiers with relevant circuit diagram.

(OR)

17.	Briefly explain about:

a.	LC filters.									     								    (8)

b.	Zener diode as voltage regulator.	 					      						    (7)

18.	Explain in detail about different biasing methods applied to BJT.

(OR)

19.	Explain in detail about the RC coupled amplifier with necessary diagram.

20.	With relevant diagram explain the characteristics of Negative feedback amplifier circuits.

(OR)

21.	Draw and explain the principle operation of Class AB amplifier with example.

22.	Briefly explain about :

a.	Bistable Multivibrator. 							     							    (8)

b.	Monostable Multivibrator.    						     							    (7)

(OR)

23.	Write short notes about :

a.	RC Phase shift oscillator.			     				      							    (8)

b.	Colpitts Oscillator.							      								    (7)

24.	Develop a small signal model of field effect transistor with relevant circuit diagram.

(OR)

25.	Explain the common source FET amplifiers with proper biasing circuit.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	SIGNALS AND SYSTEMS								      Time : 3 hours

Subject Code:	12EI209				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.   	An even signal satisfies the condition ____________.

2. 	If x(t) is a periodic signal such that  x(t) = x(t + 4), then its period is ____________.

3. 	A continuous time system is represented by ____________ equation.

4. 	The convolution integral is used for ____________.

5. 	If X(k) and Y(k) are DTFS of x(n) and y(n) respectively, then DTFS of x(n).y(n) = ____________.

6. 	If the signal x(t) is real and even, then Imaginary [X(k)]    is ____________.

7. 	The Discrete time Fourier transform of the unit impulse function is ____________.

8. 	What is Nyquist rate?

9.	Laplace transform is basically ____________on the imaginary axis in the s-plane.

10. 	The equation for two sided Z transform is ____________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Explain stability condition for a LTI system.

12. 	Mention the use of difference equation for representing a system.

13. 	Mention the conditions for convergence of Fourier series.

14. 	Define energy spectral density of a signal.

15. 	State the integration in time domain property of Laplace transform.

PART – C  (5 x 15 = 75 MARKS)

16. 	Define and plot the following  continuous time signals  (a)  unit ramp,   (b) unit step,  (c) real exponential,    (d) impulse    and   (e)  x(t) = u(t) – u(t–a).

(OR)

17. 	a. 	Distinguish between causal and noncausal systems. Give suitable examples.			    (7)

b. 	Explain linearity property of a system. Check if the system y(t) = 10x(t) +5 is linear.	    (8)

18. 	The impulse response of a LTI system is h(t) = u(t).   Determine the output of the system if the input is x(t) = e-at u(t),  a>0.

(OR)

19. 	Consider a discrete time system with impulse response h(n) = {2,2}. Using convolution sum method, find the output of the system when the input is x(n) = { 1,1,1,1}.

20. 	a. 	Explain any two properties of Continuous time Fourier Series.						    (8)

b.	Explain the Parseval's theorem.												    (7)

(OR)

21. 	Define Discrete time Fourier series. Determine the DTFS coefficients for the signal,

x(n) = Cos( nπ/3).

22.	a.	Explain the time shift property of DTFT.				             		 			    (7)

b.	Determine the DTFT of x(n) =   an  u(n).  										    (8)

(OR)

23. 	Obtain the Continuous Time Fourier Transform  of

a.	unit step function.     														    (7)

b.	x(t) = exp( j2πfct).															    (8)

24. 	Compute Laplace transform for the signal x(n)=3e−2tu(t)−2e−2tu(t) and plot the ROC.

(OR)

25. 	The difference equation of a  system is 	y(n–2) –3y(n–1) + 2 y(n)  = x(n–1) . Using Z transform  a.	Find the transfer function of the system. 								    		    (7)

b.	Find the impulse response of the system.										    (8)

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	C++ AND DATA STRUCTURES							      Time : 3 hours

Subject Code:	12EI212			 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State True of False. A Queue is a LIFO data structure.

2.	If an element is pushed inside a full stack it will ___________.

3.	Quick sort will take more time if the given array of data is ___________.

4.	___________search is faster than ___________search when the given data is small.

5.	The first function executed during the creation of an object is called its ___________.

6.	Sequence of characters with common name is called a ___________.

7.	Eating of different food by humans is an example of ___________overloading.

8.	Pointer variables store ___________of another variable.

9.	___________member functions are not attached to any particular object.

10.	State True of False. Errors in the program segments are not handled using exceptions.

PART – B  (5 x 3 = 15 MARKS)

11.	How is doubly linked list different from singly linked list?

12.	What is the purpose of the function heapify() in heap sort?

13.	What is a class in C++ programming?

14.	Why are constructors not inherited in C++ programs?

15.	What is the significance of this pointer?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the different operations that can be performed on a queue data structure with suitable 	examples.

(OR)

17.	Explain the binary search tree with an example.

18.	Describe the quick sort technique for sorting an array with an example.

(OR)

19.	Explain the binary search algorithm with suitable examples.

20.	Write a C++ program to copy the contents of one array into another in the reverse order.

(OR)

21.	Explain the different built-in functions for strings in C++ language.

22.	Describe the techniques of overloading unary and binary operators with suitable examples.

(OR)

23.	What is multiple inheritance? Explain the realization of multiple inheritance with an example.

24.	What are friend and virtual functions? Explain with suitable examples.

(OR)

25.	Explain the method of handling file streams in C++ programming language.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	SIGNAL CONDITIONING CIRCUITS					      Time : 3 hours

Subject Code:	12EI216		 								       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention the characteristics of an ideal Operational Amplifier?

2.	Define the term input offset voltage.

3.	The voltage follower has a closed loop __________.

4.	List the applications of comparator.

5.	What is the need for Instrumentation amplifier?

6.	Mention the applications of thermistor.

7.	What is a voltage regulator?

8.	Mention the advantages of IC voltage regulators.

9.	List the applications of 555 timer in Astable mode of operation

10.	What are the basic building blocks of PLL?

PART – B  (5 x 3 = 15 MARKS)

11.	State the reason for the offset currents at the input of the opamp.

12.	What is precision diode?

13.	Name the different direct types of ADC.

14.	Mention the drawbacks of linear regulators.

15.	Define lock range in a PLL.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Draw the basic block diagram of a general op-amp and explain the operation of each block 	in detail.																		    (8)

b.	Compare the ideal and practical characteristics of an op-amp.						    (7)

(OR)

17.	a.	Derive for the gain, input resistance and output resistance of an inverting op-amp. 	    (8)

b.	Explain the differential amplifier circuit using one op-amp with a neat circuit and explain 			the parameters.									   							    (7)

18.	a.	Design a non-inverting adder circuit to be used for dc applications with a gain of 10. 	    (8)

b.	Explain the principle of a basic logarithmic amplifier circuit with a neat sketch.         	    (7)

(OR)

19.	a.	With a neat circuit diagram explain the working of a current to voltage converter. 	    (8)

b.	Draw the circuit of Clampers and explain its operation in detail.			     		    (7)

20. 	a.	What is an instrumentation amplifier? Mention the desirable characteristics of an 	Instrumentation amplifier.								 						    (8)

b.	Write a brief note on Charge amplifier.											    (7)

(OR)

21. 	Design a second order low pass filter.

[P.T.O.]

22. 	a.	Compare the series and shunt voltage regulators.									    (8)

b.	What are the basic characteristics of an analog multiplier? Comment on the performance of 	a multiplier.																	    (7)

(OR)

23. 	Explain the circuit operation of a high voltage regulator using IC 723 with a neat circuit diagram. Write the equation for the output voltage and state the ranges of output voltage and current that can be obtained using the circuit.

24. 	Draw the basic block diagram of a PLL and explain in detail.

(OR)

25.  Explain the process of FSK demodulation using PLL.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ELECTRONIC INSTRUMENTATION					      Time : 3 hours

Subject Code:	12EI223				 						       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is rms value?

2.	What is aging?

3.	How beam defelection happens in crt?

4.	What is led?

5.	How accuracy of signal generator decided?

6.	What is harmonic?

7.	What is phase difference?

8.	What is parallax error?

9.	What is the advantage of vi?

10.	What type of software used to simulate vi?

PART – B  (5 x 3 = 15 MARKS)

11.	Brief about Q-meter.

12.	Explain advantages of 7 segment displays.

13.	Where spectrum analyser used?

14.	What do you mean by time interval?

15.	What is functional integration?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain electronic multimeter with block diagram.

(OR)

17.	Describe the operation of  AC voltmeter.

18.	With block diagram explain CRT.

(OR)

19.	Describe DSO.

20.	Explain  Frequency divider generator

(OR)

21.	Enumerate Signal Analyzers.

22.	Explain methods of measuring frequency, period.

(OR)

23.	Explain Digital Displacement transducer.

24.	Brief about the Programming Requirements in VI.

(OR)

25.	Describe Evolution of labview.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	DIGITAL SIGNAL PROCESSING						      Time : 3 hours

Subject Code:	12EI224			 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define continuous time signals.

2.	Name the properties of a system.

3.	State the properties of DFT.

4.	State the properties of ROC.

5.	What are the advantages of FIR filter?

6.	What is meant by linear response of a filter?

7.	Give Hamming window function.

8.	What is pre warping effect? Why is it employed?

9.	Mention the important features of Harvard architecture.

10.	What is the advantage of pipelining?

PART – B  (5 x 3 = 15 MARKS)

11.	Is a discrete time signal described by the input output relation y[n]= r x[n] time Invariant.

12.	What is the relation between DFT and Z transform?

13.	Name the basic types of analog filters.

14.	Define the condition for stability of digital filters.

15.	Draw the bit assignment diagrams of the status words ST0 and ST1.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Check whether the following is linear, time invariant, casual, Stable, recursive and Static.

(i)  	y(n ) = x(n ) + n x (n + 1) .

(ii)	 y(n) = x(1/2n)

b.	Describe in detail the process of sampling and quantization.

(OR)

17.	a.	Perform cross correlation of two sequences x1= {1, 1, 2, 1} and x2= {1, 2, 2, 1}

b.	Check whether the following are energy or power signals.

(i)	 x(n) = (1/2)n u(n)

(ii) 	x(n) = Aejw0n

18.	Compute the 8-point DFT of x(n) = {1,0,1,1,2,0,2,2} using radix-2 DIT FFT algorithm.

(OR)

19.	Determine the 4-point DFT and IDFT of the given signal

X(n) =1,0≤n≤3

20.	Design a linear phase FIR high pass filter using hamming window, with a cutoff frequency,    wC = 0.6p rad/sample and N= 5

[P.T.O]

(OR)

21.	Design a linear phase FIR bandpass filter to pass frequencies in the range 0.45p to 0.65p ra/sample by taking 7 samples of hanning window sequence.

22.	Design a low pass FIR filter of order 7 with cutoff frequencyπ3 using hanning window.

(OR)

23.	Convert the following analog filter into digital using IIM method.

H(s)=s2s2+3s+2

24.	Draw and explain the architecture of TMS320C5X processor.

(OR)

25.	Discuss about the applications of digital signal processors.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	LOGIC AND DISTRIBUTED CONTROL SYSTEMS	      	     Time : 3 hours

Subject Code:	12EI225						 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In the SCADA system computer is used as ___________.

2.	Analog output module is not used in ___________.

3.	The advantage of PLC over hardwired relay logic circuit is ___________.

4.	The numbers of registers used for timer counter instructions is___________.

5.	The LD instruction used to prevent the execution of certain rungs is ___________.

6.	The PLC based bottle filling control system in the Laboratory employs ____.

7.	Smallest collection of hardware which performs closed loop control in DCS is ___________.

8.	What is used for plant management functions?

9.	Alarm Summary display is a type of ___________.

10.	Which configuration is used in multiple loop distributed controller?

PART – B  (5 x 3 = 15 MARKS)

11.	Define SCADA.

12.	List the difference between PLC and computers.

13.	Draw the PLC symbols of flow and limit switch.

14.	List out the types of human interfacing.

15.	Mention the requirement of operator interface.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Write a short note on data loggers.												    (8)

b.	Explain in detail about Artificial Intelligent Systems.								    (7)

(OR)

17.	Draw the block diagram of SCADA system and explain each block.  Also explain the functions performed by SCADA systems.

18.	a.	Explain the different types of communication networks available for PLC.			  (10)

b.	Write the ladder diagram and draw the timing diagram using retentive timer at word location N9address:060) with an input bit at A(address : 111 /13 ) that turns on a solenoid valve at an output B(address : 101 /00 ) after a 0.5 second time delay. Use input switch 111 /01 to reset the timer.															    (5)

(OR)

19.	a.	Explain briefly about discrete I/O Modules in a PLC system.						  (10)

b.	Design a PLC program that will execute the following control circuit:				    (5)

Turns on a timer when a switch is closed.

Timer is automatically reset by an input switch.

Counter counts the number of times the timer goes to 10s.

A second input switch at a count of 5 automatically resets the counter.

Latches on a light at the count of 5.

20.	Explain the function of interlocks and alarms in PLC with a proper example.

(OR)

[P.T.O.]

21.	What are the various PLC intermediate functions? Explain in detail about sequencer function and matrix function.

22.	Illustrate the evolution of DCS and with neat sketch explain its architecture.

(OR)

23.	Explain the various LCU languages with the neat diagram.

24.	Explain the various types of operator displays with a neat diagram.

(OR)

25.	Explain the High level and Low Level Operator Interface with a neat block diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	INDUSTRIAL DATA COMMUNICATION  AND NETWORKS													      	     							Time : 3 hours

Subject Code:	12EI226						 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the basic structure of an information frame defined by protocol.

2.	List out various physical standards.

3.	Name various standards organizations involved in drawing up standards or recommendations.

4.	Name the parts of USB.

5.	Name the flow control protocols.

6.	Name an industrial application employing OSI model.

7.	List out the core functions provided by smart instruments.

8.	List out the features of industrial protocols.

9.	Classify the networks.

10.	List out various LAN standards.

PART – B  (5 x 3 = 15 MARKS)

11.	Name the communication modes available between two devices.

12.	How does current loop help in interfacing?

13.	Write down the sequence of operations for a multidrop link.

14.	Draw the protocol structure of Allen Bradley data highway protocol.

15.	Name the popular network architecture and protocols.

PART – C  (5 x 15 = 75 MARKS)

16.	How does UART help in asynchronous transmission systems? Validate through appropriate features.

(OR)

17. 	a.	Discuss about various data coding methods available to send messages in networks.	    (8)

b. 	List out the transmission characteristics and bring out salient points. 				    (7)

18.	How does RS232 aids in providing interface between DTE and DCE? Explain in detail.

(OR)

19.	Enumerate the salient features of GPIB in detail.

20.	Write short note on the following.

a.	HDLC and SDLC protocols.

b.	Binary synchronous protocol.

(OR)

21.	a. 	Name the protocols used for file transfer. Discuss various features involved in file transfer.

b. 	Draw the OSI model and discuss in detail.

22.	Discuss about MODBUS protocol in detail.

(OR)

23.	How does HART help in smart instrumentation? Explore the various possibilities available with HART to design a smart system.

24.	a. 	Name the devices used for interconnecting networks. How do they accomplish their task of 	interconnection?

b.	Discuss about token bus protocol operations.

(OR)

25.	a. 	How does token ring avoid collision? Discuss about the standards employed by to token 	rings to avoid collision.

b. 	Discuss about the performance predictions of Ethernet.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	INSTRUMENTATION AND CONTROL IN PETROCHEMICAL INDUSTRIES								                           Time: 3 hours

Subject Code:	12EI236											      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Recall the importance of desalting process in the distillation process.

2.	Mention the analyzer used in distillation column.

3.	Point out the purpose of pressure control in chemical reactors.

4.	Distinguish between batch and continuous process.

5.	State degrees of freedom.

6.	What is liquid to liquid heat exchanger?

7.	List the types of evaporators.

8.	Define conductivity.

9.	What are the chemicals used in effluent and water treatment?

10.	Define neutralization process.

PART – B  (5 x 3 = 15 MARKS)

11.	Outline the uses of cascade control in distillation process.

12.	Draw the flow sheet symbol of a chemical reactor.

13.	Describe the function of liquid to liquid heat exchangers.

14.	List the advantages and disadvantages of forced circulation evaporators.

15.	Compare chemical oxidation and chemical reduction process.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Elucidate in detail the distillation equipment and the various variables in a distillation 	column.																		  (10)

b.	List the different types of distillation column.									    (5)

(OR)

17.	Explain reboiler control and reflux control in distillation column.

18.	Illustrate the temperature and pressure control in batch reactors.

(OR)

19.	Analyze the following:

a.	Batch dryers.

b.	Continuous dryers.

20.	Describe the use of three way and two way valves in a heat exchanger.

(OR)

21.	Enlighten the feed forward control and cascade control in condensers.

22.	Outline the measurement and control of absolute pressure in an evaporator.

(OR)

23.	Describe the measurement and control of conductivity, differential pressure and flow in an evaporator with a neat sketch.

24.	Analyze the method of biological control of water treatment.

(OR)

25.	Illustrate the schematic control system of an acid waste neutralization system.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	INSTRUMENTATION AND CONTROL IN PAPER INDUSTRIES														      						      Time : 3 hours

Subject Code:	12EI237			 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write down the popular name of any two paper industries in Tamilnadu.

2.	What are the raw material of Paper Industry?

3.	Specify the units of Flow?

4.	What are all the sensors available to measure Moisture?

5. 	What is the term Digester refers to in Paper industry?

6.	What is the purpose of Dry Temperature Control?

7. 	Write down the units of Conductivity.

8.	What does the term white liquor refers in paper industry?

9.	Define Density.

10.	What is meant by Pulping?

PART – B  (5 x 3 = 15 MARKS)

11.	List the Chemical Recovery Process in paper industry.

12.	Define pH. What is its significance?

13.	Explain the function of Brown stock water level control.

14.	What are the functions and requirements of Condensate conductivity control?

15.	Write a note on input stock control.

PART – C  (5 x 15 = 75 MARKS)

16.	Sketch the outline of paper industry for Paper making process

(OR)

17.	List out and elaborate the Chemical Recovery Process.

18.	Explain any two methods of Moisture Measurements in detail with neat sketch.

(OR)

19.	Describe the temperature measurement process in paper industry with neat sketch.

20.	List the control objectives of Basic weight control and Dry temperature control.

(OR)

21.	Explain the significance of Blow tank controls and Brown stock water level control.

22.	Analyze the methodology of White liquor flow control.

(OR)

23.	Design an experiment for Dissolving tank density control in paper industry.

24.	Explain any two mechanism of   Liquid level control.

(OR)

25.	List and explain the Computer applications in pulping process control.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	ULTRASONIC INSTRUMENTATION				      	     Time : 3 hours

Subject Code:	12EI239						 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the frequency range of ultrasonic waves?

2.	Write the important parameters of ultrasonic wave medium.

3.	What is echo overlap?

4.	Write the advantage of ultrasonic wave detection using optical method?

5.	Define transit time.

6.	What is meant by pulse echo?

7.	Write an example of anisotropic material.

8.	What are the essential components inside an ultrasound device?

9.	Write the disadvantages of employing ultrasonic waves in medical field.

10.	Give an application of ultrasonics in medical therapy.

PART – B  (5 x 3 = 15 MARKS)

11.	Define reflection and transmission coefficient.

12.	List the various methods of generation and detection of ultrasonic waves.

13.  List the classification of ultrasonic test methods.

14.	What is the principle behind the measurement of thickness using ultrasonics?

15.	Differentiate between A-mode and B-mode ultrasound.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the principles of propagation of various types of waves.

(OR)

17.	Describe the characterization of ultrasonic transmission.

18.	Explain any one method of generating of ultrasonic waves.

(OR)

19.	Explain any one method of detecting ultrasonic waves.

20.	Describe with a neat diagram, how ultrasonic waves are used for density measurement.

(OR)

21.	Describe with a neat diagram, how ultrasonic sensor is used for temperature and pressure measurement.

22.	What is SONAR? Describe the application of SONAR with a neat diagram.

(OR)

23.	Describe how the condition of rail is inspected using ultrasonics.

24.	Explain the application of ultrasonics in medical imaging with a neat diagram.

(OR)

25.	Explain the application of ultrasonics in medical diagnosis with a neat diagram.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	POWER PLANT INSTRUMENTATION				      	     Time : 3 hours

Subject Code:	12EI242						 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the types of Power Generation Methods.

2.	What are the building blocks of Thermal Power Station?

3.	Define Power Factor.

4.	Why Steam temperature is important in a boiler?

5.	What is the need for analyzers?

6.	What is the importance of PH?

7.	List the control loops in a boiler.

8.	What are the two types of steam loop in boiler?

9.	List the types of turbine.

10.	What are the main parameters in turbine control?

PART – B  (5 x 3 = 15 MARKS)

11.	State the importance of instrumentation in power generation.

12.	Explain Drum level measurement.

13.	Explain chromatographs.

14.	Explain Combustion Control.

15.	Explain the Speed is important in controlling turbine.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the Renewable Energy Power generation and its importance.

(OR)

17.	Explain the piping and instrumentation diagram of boiler Co-generation.

18.	How the flow of feed water, fuel and air related with correction factor for temperature.

(OR)

19.	Write short notes on (a) Smoke density measurement (b) Dust monitor.

20.	Explain the methods of analyzing the importance of feed water and steam.

(OR)

21.	Briefly explain about pollution monitoring instruments.

22.	Write short notes on (a) Furnace draft control (b) Super heater control.

(OR)

23.	Explain about Distributed control system in power plants.

24.	Explain in detail about shell imperative monitoring and control.

(OR)

25.	Explain lubricant oil temperature control.

Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	INSTRUMENTATION AND PROCESS CONTROL FOR FOOD 						ENGINEERS															      																	     Time : 3 hours

Subject Code:	12EI252			 							       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	From which time response the steady state error can be obtained?

2.	Write the Laplace transform of unit step function.

3.	Is a feedback sensor needed in an open loop system?

4.	What is the standard operating pressure range in a pneumatic control system ?

5.	How do you call a system that obeys the superposition theorem?

6.	Give Mason’s gain formula.

7.	What is gauge factor?

8.	Based on what principle does a strain gauge work?

9.	Define pH.

10.	What is chromatography?

PART – B  (5 x 3 = 15 MARKS)

11.	Define process

12.	What is a proportional controller?

13.	When is a system said to be stable?

14.	State the three laws of thermocouple

15.	How is level sensed using capacitive sensor?

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain the Laplace transform  of the following functions

a.	sinωt	b.	u(t) = 1 ,t >=0 ,0 otherwise

(OR)

17.	With a neat sketch explain the block diagram of a process control system with a suitable example

18.	Explain the construction, principle and working of  pneumatic valves

(OR)

19.	Detail on the working of diaphragm actuator.

[P.T.O.]

20.	Using block diagram reduction rules, obtain the transfer function

G1

G2

G3

H3

H2

R

+

-

+

C

-

-

+

-

H1

(OR)

21.	Using Routh stability criterion determine the stability of  system given by the characteristic 	equation

S4+ 8S3 + 18S2 + 16S + 5 = 0.

22.	Discuss about the construction and working of  bimetallic strips. Explain in detail one application of bimetallic strip.

(OR)

23.	Explain how a capacitive sensor can be used for pressure measurement.

24.	With neat sketch explain the operation of an ultrasonic type densitometer.

(OR)

25.	Explain in detail the gas chromatographic technique.

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2015

Subject Title :	DIGITAL SIGNAL PROCESSIN	G					       	     Time : 3 hours

Subject Code:	EI262							 			              Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give some examples of discrete time signals.

2.	What are the operations performed on a signal?

3.	What are the special features of FIR filters?

4.	Define pre warping effect. Why is it employed?

5.	Write the expression for the order of Butterworth filter.

6.	Write the expression for the order of Chebyshev  filter.

7.	What are the advantages of floating point arithmetic?

8.	What do you understand by quantization error?

9.	List the buses available in TMS320C54X.

10.	List the blocks available in TMS320C54X.

PART – B  (5 x 3 = 15 MARKS)

11.	State circular shift property of DFT.

12.	Give hanning window function.

13.	What is Gibbs phenomenon?

14.	Draw the bit assignment diagrams of the status words ST1.

15.	Mention the features of DSP processor.

PART – C  (5 x 15 = 75 MARKS)

16.	Check whether the following is linear, time invariant, causal, Stable, recursive and Static.

a.	 y(n ) = x(n+1 ) + n x (n + 1) 						        b.	y(n) = x(1/2n2)

(OR)

17.	a. 	Perform cross correlation of two sequences x1= {1, 3, 2, 1} and x2= {1, 2, 2, 1}

b. 	Check whether the following are energy or power signals.

(i) x(n) = (1/2)n u(n)    							    		(ii) x(n)=Aejw0n

18.	Determine the inverse z-transform using any two methods

a.	X(Z)=(3+2z-1+z-2)/(1-3z-1+2z-2)     				b.	X(Z)=(z-0.6)/(z2+z+2)

(OR)

19.	Determine the response of LTI discrete time system governed by the difference equation

y(n)–0.3y(n–1) –0.04y(n–2)=x(n)+0.5x(n-1) for the input, x(n) = 0.3nu(n) and with initial condition y(–2) = 0, y(–1) = 1.

20.	Design a linear phase FIR high pass filter using hamming window, with a cutoff frequency,

wC = 0.7p rad/sample and N= 3.

(OR)

21.	Design a linear phase FIR bandpass filter to pass frequencies in the range 0.555p to

0.70p ra/sample by taking 5 samples of hanning window sequence.

22.	Explain in detail the architectural features of a DSP processor.

(OR)

23. 	Explain in detail about MAC unit and pipelining.

24.	Draw and explain the architecture of TMS320C54X processor.

(OR)

25.	Describe the different addressing modes of TMS320C54X processor.