Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examinations – June 2016

# Subject Title:	THERMAL ENGINEERING II						       Time : 3 hours

# Subject Code:	14ME2016								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1. 	Derive an expression for air standard efficiency of an Otto cycle. Plot the temperature entropy and pressure volume diagram.

(OR)

2. 	An ideal diesel cycle has compression ratio of 16 with lowest temperature and pressure a 27ºC and 1 bar respectively. If heat added during the cycle is 500 kJ/kg of air, find the pressure and temperature at each state point, work done per kg, thermal efficiency and mean effective pressure.

3. 	With a neat sketch explain the working principle of a four stroke petrol engine.

(OR)

4. 	Describe the battery ignition system with a schematic.

5. 	Sketch the layout of an air-conditioning system and explain the function of each component in it. What combinations are used for winter and air-conditioning?

(OR)

6. 	A conference room for seating 100 persons is to be maintained at 22ºC dry bulb temperature and 60% relative humidity. The outdoor conditions are 40ºC dry bulb temperature and 27ºC

wet bulb temperatures. The various loads in the auditorium are as follows:

Sensible and latent heat loads per person 80 W and 50 W respectively; lights and fans 15 kW; sensible heat gain through glass, walls, ceilings etc. 15000 W. The air infiltration is               20 m3/min and fresh air supply is 100 m3/min. Two-third of recirculated air and one-third of fresh air are mixed before entering the cooling coil. The by-pass factor of the coil is 0.1. Determine room sensible heat factor and apparatus dew point.

7. 	A conical diffuser has entry and exit diameters of 15 cm and 30 cm respectively. The pressure, temperature and velocity of air at entry are 0.89 bar, 340 K and 180 m/s. Determine the exit pressure, exit velocity and the force exerted on the diffuser walls.

(OR)

8. 	Air at po = 10 bar, T = 400 K is supplied to a 50 mm diameter pipe. The friction factor for the

pipe surface is 0.002. If the Mach number changes from 3.0 at the entry to 1.0 at the exit diameter the length of the pipe and the mass flow rate.

9. 	Compulsory:

Describe the working of a ram jet engine with a neat sketch. State its advantages and disadvantages.