Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	AIRCRAFT STRUCTURES							      	     Time : 3 hours

Subject Code:	09AE202 / 11AE202				 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The number of member m in frame (pin joined) is more than (2j-3), j – number of joint, so frame is called ___________.

2.	Stiffness factor (Moment distribution method) for a beam fixed at one end and simply supported at the other is ___________.

3.	Define shear center.

4.	Define Shear flow.

5.	Write Bredt – Batho formula.

6.	If a column of length is hinged at its both ends, its equivalent length will be equal to ___________.

7.	The slenderness ratio of a long column is ___________.

8.	Shortly explain Maxwell's Reciprocal theorem.

9.	Shortly explain Castigliano's theorem.

10.	Define elastic axes.

PART – B  (5 x 3 = 15 MARKS)

11.	What do you understand by ‘column’ and ‘strut’? Distinguish clearly between long columns and short columns.

12.	What is frame? Discuss its classification.

13.	Write short notes on a. Resilience b. Proof resilience c. Modulus of resilience.

14.	Draw the shear flow diagram for a Z section and indicate the shear center location (Both the flange are equal length and width).

15.	Shortly explain complete tension field beam and incomplete tension field beam.

PART – C  (5 x 15 = 75 MARKS)

16.	A truss of span 9m is loaded as shown in figure. Find the reactions and forces in the member of the truss.

[P.T.O]

(OR)

17.	A simply supported beam ABC is continuous over two spans AB and BC of 6 m and 5 m respectively. The span AB carrying a uniformly distributed load of 2 kN/m and span BC is carrying a point load of 5 kN at a distance of 2 m from B. Find the support moment and the reactions.

18.	A beam of length L is simply supported as its ends and carries a concentrated load W at its center. It has a rectangular cross-section breadth b, depth d. If C is the modulus of rigidity for the beam, find:

a.	Shear strain energy in the beam;

b.	Deflection due to shear.

(OR)

19.	Using the principle of virtual work, determine the vertical and horizontal deflection components of joint C of the truss in figure. Young’s modulus E = 200 x 106 kN/m2 and sectional area of each member A = 100 x 10-6 m2.

20.	a. 	Drive the Euler’s equation for the Column with both ends are hinged or pinned.		    (7)

b. 	Calculate the safe compressive load on a hollow cast iron column (one end fixed and other hinged) of 150 mm external diameter, 100 mm internal diameter and 10 m length. Use Euler’s formula with a factor of safety of 5 and E= 95 GN/m2. 						    (8)

(OR)

21.	Compare the crippling loads given by Rankine’s and Euler’s formula for tubular strut 2.25m long having outer and inner diameters of 37.5 mm and 32.5mm loaded through pin-joint at both ends. Take Yield stress as 315 MN/m2; a = 1/7500 and E = 200 GN/m2. If elastic limit for the material is taken as 200 MN/m2; then for what length of the strut does the Euler formula cease to apply?

22.	Determine the shear center of the C - section shown in figure. Shear force Sy, Shear center distance from web is e and take Sy / Ixx = 10 N/cm4.

(OR)

23.	Find the shear flow for the closed section shown in figure. Subjected to a vertical force V passing through the shear center. Thickness t for the entire wall.

24.	Calculate the shear flows in the web panels and direct load in the flanges and stiffeners of the beam shown in figure if the web panels resist shear stresses only.

(OR)

25.	a. 	What is rib and how does it transfer loads?										  (5)

b. 	Explain the structural idealization of wing and fuselage. 	 						(10)