Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May - 2016

Subject Title :	GAS DYNAMICS									      	     Time : 3 hours

Subject Code:	09AE207/12AE220				 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define: Speed of sound.

2.	Draw the pressure disturbance diagram for sonic and supersonic flow.

3.	Explain about the term compressibility.

4.	What is meant by Rayleigh flow?

5.	Is it possible to get subsonic to supersonic flow in a constant area duct? Give reasons.

6.	Define Mach Angle.

7.	The Aerodynamic centre for supersonic airfoils lies _________ position of its chord.

8.	Define critical Mach number.

9.	What is Drag Divergence number?

10.	List down the flow visualisation techniques used in high speed flow.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate normal shock from oblique shock.

12.	Explain about shock polar with neat sketch.

13.	Define Prandtl – Glauert rule.

14.	Write short notes on a Transonic Area rule.

15.	Explain about the working principle of Schlieren System.

PART – C  (5 x 15 = 75 MARKS)

16.	A Mach 2 C-D nozzle, run by a settling chamber with air maintained at 300 K, is discharging to an environment at atmospheric pressure. Determine the settling chamber pressure required to

run the nozzle

a.	at correctly expanded state,

b.	with maximum possible overexpansion,

c.	with normal shock at the exit.

d.	Determine the settling chamber pressure required to position a normal shock in the divergent part of the nozzle, at location where the area is 1.5 times the throat area.  What will be the flow speed behind the normal shock?

Assume isentropic flow, γ = 1.4, cp = 1.00 kJ/kg.K.

(OR)

17.	a.	What is meant by velocity of sound? Derive the expression for Velocity of sound. 	  (10)

b.	Define Mach wave and elaborate mach cone with neat sketch.						    (5)

18.	Derive the Rankine- Hugoniot equation for normal shock with neat sketch.

(OR)

19.	A Re-entry Vehicle (RV) is at an altitude of 15,000m m and has a velocity of 1850 m/s.  A bow shock envelops the RV.  Neglecting dissociation, determine the static and stagnation pressure and temperature just behind the shock on the RV centre line where the shock wave may be treated as normal shock.  Assume that the air behaves as perfect gas, with γ = 1.3, R = 0.287 kJ/kg.K.

[P.T.O]

20.	Explain about small perturbation potential theory with neat sketch.

(OR)

21.	Elaborate Prandtl-Glauert Transformation for subsonic flow as well as supersonic flow.

22.	Write short notes on

a.	Drag Divergence Number

b.	Characteristics of swept wing.

(OR)

23.	The Upper and lower surfaces of symmetrical two dimensional aerofoil are given by  z=∓εx1−xc2,where c is the chord and ε<<1. The aerofoil is at zero incidence in a steady supersonic stream of Mach number M∞in positive x-direction. (a) Find the velocity components according to the linear theory in the upper region of disturbance .(b) Show that the drag coefficient of the aerofoil is given by CD=815ε2M∞2−112.

24.	Write short notes on

a.	Critical Mach number.														    (8)

b.	Transonic area rule with neat sketch.											    (7)

(OR)

25.	Explain in detail about Schlieren flow visualization technique used in supersonic flow with neat sketch.