Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	SPACE DYNAMICS								      	     Time : 3 hours

Subject Code:	09AE208/12AE232				 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The specific impulse of solid propellants is ___ than that of liquid propellants.

2.	Static stability margin (SM) = _______.

3.	The origin of geocentric-inertial coordinate system is the centre of _______.

4.	Fins on rockets primarily generate _______.

5.	Asteroids are revolving about the Sun between the orbits of _______and Jupiter.

6.	Define ecliptic.

7.	Vis-viva equation for elliptic orbits is V2 = μ(______).

8.	What is injection epoch?

9.	The energy of elliptic orbits is _______ , and hyperbolic orbits is _______.

10.	DefineLambert Theorem in 2-body Problem.

PART – B  (5 x 3 = 15 MARKS)

11.	State the advantages of liquid fuel rockets.

12.	Using Vis-viva equation for elliptic orbits, find the radial distance when velocity is 8.2 kms-1 and semi-major axis is 8000 km. Earth’s gravitational constant (μ ) = 398600 km3s-2

13.	Use Kepler’s equation M = E - e sin E to find mean anomaly (M) in degrees if the eccentricity and eccentric anomaly (E) are 0.1 and 45 degrees, respectively.

14.	Write three perturbing forces acting on an Earth satellite.

15.	Calculate the sphere of influence of moon. The mass of moon is 1/81 that of Earth. Radius of moon’s orbit is 384400 km.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	From the first principles, derive the rocket equation

Vb = g0 Isp ln(Mi/Mf),

where Vb is the burnout velocity, g0 is the acceleration due to gravity at sea level, Isp is specific impulse and Mi/Mf is the mass ratio.  									    (9)

b.	If the Isp of a rocket is 360 s, and Vb is 8000 metres/s, calculate its mass ratio (g0=9.8 m/s2). If initial mass is 5000 kg, find the final mass. 									    (6)

(OR)

17.	a.	A two-stage rocket has the following design characteristics.  First stage: propellant mass = 	20000 kg, structural mass = 4000 kg. Second stage: propellant mass = 8000 kg, structural 	mass = 1800 kg. The payload mass is 200 kg. The Isp of the first stage is 280 seconds and 	the Isp of second stage is 360 s.  Calculate the final burnout velocity. (g0=9.8 m/s2).	    (8)

b.	With the help of a neat diagram, explain the static stability regions in terms of the 	aerodynamic moment and angle of attack. 										    (7)

18.	a.	Describe solar system.														  (10)

b.	With the help of a neat diagram, explain the six orbital parameters of an Earth satellite.  (5)

(OR)

[P.T.O]

19.	a.	If the position and velocity of a satellite are (−6045,− 3490,2500) km and (−3.457,6.618, 	2.533) km/s, respectively; find the orbital elements angular momentum (h), inclination (i), 	right ascension of ascending node (Ω), eccentricity (e), argument of  perigee (ω) and true 	anomaly (θ) of the satellite.                                                           					  (12)

b.	Calculate the radial distance r if semi-major axis, eccentricity and true anomaly are 8200 	km, 0.2 and 45 degrees, respectively.											    (3)

20.	a.	State Kepler’s laws of planetary motion. Use third law to calculate the orbital period of 	Mars if its average distance from Sun is 1.52 Astronomical Units (AU).				    (7)

b.	Explain geocentric-inertial coordinate system and heliocentric-inertial coordinate system.

(8)

(OR)

21.	a.	What are the advantages, disadvantages and applications of special and general 	perturbation methods?															    (6)

b.	Explain briefly Cowell’s method and Encke’s method.                                                	  (9)

22.	a.	Define Sun-synchronous orbits for Earth satellites. Calculate the orbital inclination for a 	circular Sun-synchronous orbit, whose semi-major axis is 8000 km. Earth’s gravitational 	constant (μ ) = 398600 km3s-2, J2 = 0.00108263 and Earth’s radius is 6378 km.		  (11)

b.	Explain one important perturbing force acting on an artificial Earth satellite. 		    (4)

(OR)

23.	a.	Explain briefly the method of the variations of orbital elements and Lagrange planetary 	equations.                                                                                                                             (8)

b.	Calculate the velocity change required to transfer a satellite from a circular orbit at an 	altitude of 500 km with an inclination of 40to an orbit of the same size at an inclination of 	30. Earth’s gravitational constant = 398600 km3s-2.								    (7)

24.	a.	Draw a neat sketch of hyperbolic trajectory. Show in the sketch, true anomaly of the 	asymptote, turn angle, periapsis, apoapsis, semi-major and semi-minor axes.                 (7)

b.	Find the additional velocity required for a Hohmann transfer from a circular earth satellite 	orbit of radius 8200 km to a circular earth satellite orbit of radius 10000 km. Earth’s 	gravitational constant = 398600 km3s-2.											    (8)

(OR)

25.	a.	Estimate the trip time T from the Earth to Mars along the Hohmann transfer orbit by 	assuming the orbits of Earth and Mars around the Sun to be circular with radii of                	149.6 x 106 and 227.9 x 106 km, respectively. The value of the Sun’s gravitational constant 	(µ) = 1.32715 x 1011 km3s-2.                                                                                		    (7)

b.	Explain the synodic period of a planet relative to the other planet. Calculate the synodic 	period of Mars relative to the Earth. The orbital periods of Earth  and Mars are  365.26 	days and 687.99 days, respectively.												    (8)