Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May - 2016

Subject Title :	AIRCRAFT PROPULSION							      	     Time : 3 hours

Subject Code:	09AE209						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List at least two functions of a convergent nozzle.

2.	Define ram pressure.

3.	What are the advantages of axial compressor over centrifugal compressor?

4.	Draw the velocity diagram for axial compressor blades.

5.	What are the two major types of turbines?

6.	What are the factors to be considered in the selection of materials for turbine blades?

7.	The percentage enhance in thrust after using afterburner is __________.

8.	The actual flow rate and geometrical flow rate is same in case of _________.

9.	What are the assumptions made in matching of components?

10.	What are the principles of control systems?

PART – B  (5 x 3 = 15 MARKS)

11.	How is air breathing engines classified?

12.	What is the secondary function of the compressor?

13.	List the three types of combustion chambers. Give the advantages and disadvantages of each type.

14.	What are the operation requirements of an intake for aircraft engine?

15.	Illustrate transient behavior of gas turbine engine.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	With a neat sketch explain the working principle of a turbo jet engine.				  (10)

b.	What are the merits and demerits of turbojet engine over ramjet engine?				    (5)

(OR)

17.	Write a short note on.

a.	Operating envelope.

b.	Standard atmosphere.

c.	Working cycle of gas turbine engine.

18.	Explain the principal of operation of centrifugal compressors with neat sketches and derive the work done and overall stagnation pressure ratio of a centrifugal compressor.

(OR)

19.	a.	What are the losses in centrifugal compressor?									    (5)

b.	A centrifugal compressor has a pressure ratio of 4:1 with an isentropic efficiency of 80% when running at 15000rpm and inducing air at 293 k. Curved vanes at inlet give the air a prewhirl of 25° to axial direction at all radii and the mean dia of eye is 250 mm. The absolute air velocity at inlet is 150 m/s. Impeller tip dia is 600 mm. calculate the slip factor.																		  (10)

20.	a.	What are the various forms of combustion system?								    (5)

b.	What are the basic requirements of a combustion chamber? 						    (5)

c.	Briefly explain the various factors affecting the combustion chamber design.		    (5)

[P.T.O]

(OR)

21.	With a neat sketch explain the combustion chamber geometry bringing out the various zones that play a part in process of combustion.

22.	a.	What are the three speed zones in which the supersonic inlet duct must operate? 		    (3)

b.	Give a brief description of the inlet duct airflow under these three conditions.		  (12)

(OR)

23.	a.	What are the requirements of a good afterburner?									    (5)

b.	Explain the operation of an afterburner.											    (5)

c.	What are the types of ignition systems used?										    (5)

24.	Describe the matching procedure for a turbofan engine.

(OR)

25.	Explain the off design operation of free turbine engine.