Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title:	UNIT OPERATIONS								      	      Time: 3 hours

Subject Code:	09BT205						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List out the mode of heat transfer operations.

2.	Steady state heat conduction is _______.

3.	Convection classified into _______and _______.

4.	Emissivity value for black body_______.

5.	Draw a temperature profile graph of parallel flow and counter flow heat exchanger.

6.	What you meant by evaporator capacity?

7.	What is the function of filter aids? Give examples.

8.	The drum for the vacuum filters turns with a speed of _______.

9.	Mention few impellor names.

10.	Size reduction meant by_______.

PART – B  (5 x 3 = 15 MARKS)

11.	State and explain the Fourier’s law of heat conduction.

12.	Write the radiation different laws.

13.	Write down the material balance for single effect evaporator.

14.	Define free settling and hindered settling.

15.	What is the need for mixing and agitation?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A steam pipe line 6.5 cm outside diameter is lagged with a 5cm layers of asbestos               	(thermal =0.12) and a 4 cm  layers of cork( thermal conductivity= 0.03) .If the temperature 	of the outer surface of the pipe is 150 ºC and temperature of the outer surface of the cork  	is 35 ºC. Calculate heat loss in calories per hour per metre.						    (8)

b.	Derive the expression for heat transfer through furnace wall made of three different materials in series. Assume k1, k2 and k3   be the thermal conductivities of materials and x1, x2 and x3 be their respective thickness. Assume hot face and cold face  temperatures as T1 and T2 respectively with neat sketch. 											    (7)

(OR)

17.	a.	A furnace is constructed with 225mm thick of fire brick, 120mm of insulating brick and     	225 mm of the building brick. The inside temperature is 1200 K and the outside 	temperature is 330K. Find the heat loss per unit area and the temperature of the junction of 	the fire brick and insulation brick. Data:  Thermal conductivity of fire brick= 1.4 (W/m K), 	Thermal conductivity of insulating brick= 0.2 (W/m K) and Thermal conductivity of 	building brick= 0.7 (W/m K).													    (8)

b.	Derive an express on hallow  cylinder with neat sketch.							    (7)

[P.T.O]

18.	Calculate the heat transfer coefficient for water at 60 ºC , flowing through a tube of i.d 0.625 cm with a velocity is 0.9 m/s. Tube  wall temperature is 40 ºC .At the mean  temperature ( ie 50ºC ),  data given: Viscosity of water is2.177 kg/ m.hr, K= 0.54 kcal/ m.hr. ºC and Cp =1 kcal/kg.K, density = 1000 kg/ m3

(OR)

19.	Derive the empirical equation for calculation of heat transfer coefficient in

a.	Laminar condition 		b.	Turbulent condition in forced convection                     (8+7)

20.	a.	Crude oil flows at the rate of 1000kg/hr through the inside pipe of a double pipe heat 	exchanger and is heated from       30 ºC to 90 ºC. The heat is supplied by kerosene initially 	at 200 ºC flow through the annular space. If the temperature of approach (minimum 	temperature difference) is 10 ºC, Determine the heat transfer area for cocurrent flow and 	kerosene flow rate.

Data : Specific heat of crude oil= 0.5 kcal/ kg ºC,

Cp of kerosene is 0.6 kcal/ kg ºC and  Uo= 400 kcal/ hr m2 ºC.						   (7)

b.	Discuss about design of 1-1 shell and tube heat exchanger with neat sketch.			   (8)

(OR)

21.	a.	Derive an express for single effect evaporator with neat Sketch.					    (5)

b.	A single effect evaporator is fed with 5000 kg/ hr of solution containing 1% solution by weight. Feed temperature is 303K and is to be concentrated to a solution of 2% solute by weight. The evaporator is at atmospheric pressure (101.325 k pa) and area of evaporator is  69m2 .Saturated steam is supplied at 143.3 k pa as a heating medium. Calculate steam economy and overall heat transfer coefficient. 									  (10)

Data:

Enthalpy of feed at 303K = 125.79 kJ/ kg,

Enthalpy of vapour at 101.325 kpa= 2676.1 kJ/ kg,

Enthalpy of saturated steam at 143.325 kpa= 2691.5 kJ/kg,

Enthalpy of product= 419.1 kJ/kg,

Enthalpy of saturated water at383K= 461.30 kJ/kg, Saturation temperature of steam= 383K

Boiling point of solution = boiling point water= 373K.

22.	a.	What is a vortex? How will you prevent the vortex? 								    (7)

b.	Derive the power calculation in mixing.											    (8)

(OR)

23.	Describe the agitation equipments? Give an account of the different types of impellers and turbines with neat sketch.

24.	Describe the top basket centrifugal filtration with neat sketch and also give its merits and demerits.

(OR)

25.	Discuss about the working principles of continuous vacuum filter press with neat sketch and give its merits and demerits.