Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	MECHANICS OF FLUIDS							      	     Time : 3 hours

Subject Code:	09CE202 / 12CE205 / CE277	 					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How are fluids classified?

2.	State Pascal’s law.

3.	Write the equation for one dimension continuity equation.

4.	Distinguish between Uniform flow and Non – Uniform flow.

5.	Write the Bernoulli’s equation for real fluid.

6.	Distinguish between Notch and weir.

7.	What is meant by Equivalent pipe?

8.	Loss of lead due to sudden expansion in the pipe is h = _________.

9.	Define Dimensional Analysis.

10.	Distinguish between Kinematic similarity & Dynamic similarity.

PART – B  (5 x 3 = 15 MARKS)

11.	State Newton’s law of viscosity and classify fluids according to it.

12.	List the assumptions of Bernoulli’s theorem

13.	Define the various Hydraulic co-efficients.

14.	Find the lead lost due to friction in a pipe of diameter 300mm & length 50m through which water is flowing at a velocity of 3 m/s using Darcy’s formula. Take γ=0.01 stoke.

15.	Define Dimensionless number and List the various Dimensionless numbers.

PART – C  (5 x 15 = 75 MARKS)

16.	The space between two square flat parallel plate is filled with oil. Each side of plate is 60cm. The thickness of oil film is 12.5mm. The upper plate, which moves at 2.5m/s requires a force of 9.81N to maintain the speed.

Determine the i) The dynamic viscosity of oil in poise. ii) Kinematic viscosity of oil in stokes if the specific gravity of oil is 0.95.

(OR)

17.	Discuss in detail about any two types of manometers.

18.	a.	Explain various types of fluid flow. 											    (7)

b.	Water flows through a pipe AB 1.2m diameter at 3 m/s and then passes through a pipe BC 1.5m diameter. At C, the pipe branches. Branch CD is 0.8m in diameter and carries one third of flow in AB. The flow velocity in branch BE is 2.5 m/s. Find the volume rate of flow in AB, the velocity BC, the velocity in CD and diameter CE.					    (8)

[

(OR)

19.	Derive continuity equation in Cartesian co-ordinates

20.	a.	What is a venturimeter? Derive the expression for rate of flow through Venturimeter.	    (8)

b.	Head of water over the centre of an orifice of diameter 20mm is 1m.The actual discharge 	through the orifice is 0.85 lit/s. Find the co-efficient of discharge.					    (7)

[P.T.O]

(OR)

21.	a.	Water flows through right angled triangular weir first and then over a rectangular weir of 	1m width. The discharge co-efficient of the triangular and rectangular weir are 0.6 and 0.7 	respectively. If the depth of water over the triangular weir is 360mm. Find the depth of 	water over the rectangular weir.												    (8)

b.	Discuss about various types of weirs & notches.									    (7)

22.	a.	 Derive Darcy’s Weisbach equation.											    (7)

b.	The rate of flow of water through a horizontal pipe is 0.25 m3/s. The diameter of the pipe which is 200mm is suddenly enlarged to 400mm. the pressure intensity in the smaller pipe is 11.772 N/cm2.Determine i) Loss of load due to sudden enlargement ii)Pressure intensity in large pipe iii) Power lost due to enlargement.									    (8)

(OR)

23.	A pipe of diameter 0.4m and a length 2000m is connected to the reservoir at one end. The other end of the pipe is connected to another reservoir, which is having level of water 10m below in water level of the above reservoir. Determine the total discharge if f =0.15, neglect  minor losses.

24.	Using Buckingham’s π - theorem, Show that the velocity through a circular orifice is given by V=√2gH Φ [ D/H, µ/ρvH] where Hishead , D is diameter of orifice , µ is co-efficient of viscosity,  ρ is density, g is acceleration due to gravity.

(OR)

25.	a.	Prove that scale ratio for discharge for a distorted model is given as

Qp/Qm=(Lr)H (Lv) V3/2								  						    (8)

b.	In the model test of a spill way the discharge and velocity of flow over the model were 2m3/s and 1.5 m3/s respectively. Calculate the velocity and discharge over the prototype which is 36 times the model size.												    (7)