Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	MECHANICS OF DEFORMABLE BODIES – I		      	     Time : 3 hours

Subject Code:	09CE205/CE240								 	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	State Hooke`s law.

2.	Define tensile stress.

3. 	Define principal stresses.

4.	What is the hoop stress in thin spherical shell?

5.	Distinguish between hinged support and roller support.

6. 	What is the maximum bending moment in the simply supported beam carrying concentrated load W at mid-span?

7.	What is the section modulus of a beam of rectangular section?

8. 	What are the uses of leaf springs?

9. 	Define torsion.

10. 	What is meant by stiffness of a spring?

PART – B  (5 x 3 = 15 MARKS)

11.	List down the two important type of stresses.

12.	A thin cylindrical shell has an internal dia of 250 mm and thickness of 6 mm. It is subjected to an internal pressure of 3 MN/m2. Estimate the circumferential and longitudinal stresses if the ends of the cylinder are closed.

13.	What are the types of beams?

14.	What are the assumptions made in the theory of simple bending?

15.	A solid shaft of 80mm diameter is transmitting 100kW power at 200 rpm. Calculate the maximum shear stress induced in the shaft.

PART – C  (5 x 15 = 75 MARKS)

16.  	A concrete column of cross-sectional area 400 mm x 400 mm is reinforced with 4 longitudinal 50 mm diameter round steel bars. The column carries an axial compressive load of 300 kN, determine (i) the loads carried (ii) the stresses in concrete and steel bars. Assume that Young`s Modulus of steel is 15 times that of concrete.

(OR)

17. 	A uniform metal bar has cross-sectional area of 700 mm2 and length of 1.50 m. The elastic limit is 160 MPa. Determine the proof resilience. Also determine the maximum value of an applied load which may be suddenly applied without exceeding the elastic limit. Determine the value of gradually applied load which would produce the same extension as that of the above suddenly applied load.

18. 	The principal stresses at a point across two perpendicular planes are 75 MPa (tensile) and        35 MPa (tensile). Determine the normal, shear stresses, the resultant stress and its obliquity on a plane at 20 with the major principal plane.

(OR)

[P.T.O]

19.	The stresses at a point of a machine component are 150 Mpa and 50 Mpa both tensile.  Find the intensities of normal, shear and resultant stresses on a plane inclined at an angle of 55° with the axis of major tensile stress.  Also find the magnitude of maximum shear stresses in the component.

20. 	A simply supported beam of span 4 m carries concentrated loads of 2kN, 4kN and 2kN at a distance of 1m, 2m and 3m from the left support. Draw shear force diagram and Bending Moment diagram.

(OR)

21. 	A beam AB 10 m long carries a uniformly distributed load of 20 kN/m over its entire length together with concentrated load of 50 kN at the left end A and 80 kN at the right end B. The beam is to be supported at two props at the same level, 6m apart so that the reaction is the same at each prop. Determine the positions of the supports and draw Shear Force Diagram and Bending Moment Diagram. Determine the value of maximum bending moment. Also locate the points of inflexion.

22.	A rectangular beam 300mm deep is simply supported over a span of 4m.  What uniformly distributed load the beam bay carry, if the bending stress is not to exceed 120 Mpa? Take I = 225x106 mm4.

(OR)

23. 	Determine the dimensions of joist of a timber for span 8 m to carry a brick wall 200 mm thick and 5 m high, if the density of brick work is 1850 kg/m3 and the maximum permissible stress is limited to 7.50 MPa. Assume that the depth of joist is twice the width.

24. 	A solid steel shaft has to transmit 100kW at 160 rpm. Taking allowable shear stress at 70 MPa, find the suitable diameter of the shaft. The max torque transmitted in each revolution exceeds the mean by 20%.

(OR)

25. 	A weight of 200 N is dropped on to a helical spring made of 15 mm wire closely coiled to a mean diameter of 120 mm with 20 coils. Determine the height of drop if the instantaneous compression is 80 mm.