Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	MECHANICS OF DEFORMABLE BODIES II/ADVANCED STRENGTH OF MATERIALS										    Time : 3 hours

Subject Code:	09CE206/12CE210/CE241						      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is Macaulay’s method?

2.	What are the two design criteria for the deflection of a cantilever or a beam?

3.	How failure of a short and long column takes place?

4.	What is slenderness ratio?

5.	What do you mean by thick compound cylinder?

6.	List some of the important theories of failure.

7.	In case of a beam having two axes of symmetry, the shear centre coincides with _________.

8.	The angle of inclination of neutral axis with respect to principal axis is α = _________.

9.	If the section is symmetrical, the principal axes are _________.

10.	Write any one reason for unsymmetrical bending.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the two important methods to find out the slope and deflection at a section?

12.	Find an expression for crippling load for a long column when one end of the column is fixed and the other end is hinged.

13.	Explain Lame’s theory and its assumptions.

14.	State and explain the Winkler Bach formula.

15.	How will you find the bending stresses due to unsymmetrical bending?

PART – C  (5 x 15 = 75 MARKS)

16.	A cantilever of length 2m carries a point load of 20kN at the free end and another load of 20kN at its centre. If E = 1x105N/mm2 and I= 1x108mm4 for the cantilever beam, then determine by moment area method the slope and deflection of the cantilever at the free end.

(OR)

17.	A simply supported beam AB of span 4m carries a point load of 100kN at its centre. The value of I for the left half (AC) is 1x108mm4 and for the right half (CB) is 2x108mm4. Find the slope at the two ends and deflection under the load.   Take E = 2x105N/mm2.

18.	A 1.5m long column has a circular cross section of 50mm diameter. One of the ends of the column is fixed in direction and position and the other end is free. Taking a factor of safety 3, calculate the safe load using  a) Rankine’s formula taking   σc = 560N/mm2 and α = 1/1600  and b) Euler’s formula taking Young’s modulus E = 1.2x105 N/mm2.

(OR)

19.	A column of circular section is subjected to a load of 120kN. The load is parallel to the axis but eccentric by an amount of 25mm. The external and internal diameter of column is 60mm and 50mm respectively. If both ends of the column are hinged and column is 2.1m long, determine the maximum stress in the column. Assume E = 2x105N/mm2.

[P.T.O]

20.	A cast iron pipe of 40 mm internal diameter and 100 mm thickness carries water under a pressure of 8 N/mm2 of 8 N/mm2.  Determine the maximum and minimum intensities of hoop stress across the section.

(OR)

21.	A pipe of 200mm internal diameter and 50mm thickness carries a fluid at a pressure of 10MPa. Calculate the maximum and minimum intensities of circumferential stress across the section. Sketch the radial stress distribution and circumferential stress distribution across the section.

22.	A channel section has flanges of 120mm x 20mm and web of 160mm x 10mm. Determine the shear centre of the channel.

(OR)

23.	A curved bar is formed of a tube of 120mm outside diameter and 7.5mm thickness. The centreline of this beam is circular arc of radius 225mm. A bending moment of 3kNm tending to increase the curvature of the beam is applied. Calculate the maximum tensile and compressive stresses setup in the bar.

24.	A beam of T section (flange 100mm x 20mm, web 150mm x 10mm) is 2.5m in length and is simply supported at its ends. It carries a load of 3.2kN inclined at 20o to the vertical and passing through the centroid of the section. If E = 2x105N/mm2, find the maximum tensile and compressive stresses, deflection due to the load and position of neutral axis.

(OR)

25.	A cantilever of I section (flanges 30x2.5mm, web 45x2mm) is subjected to a load of 200N at the free end. The inclination of the load is 20o to the vertical. The length of the cantilever is 2.4m. Determine the resulting bending stress at the corners on the fixed section of the cantilever beam.