Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2016

Subject Title :	Surveying/ Survey									      	     Time : 3 hours

Subject Code:	09CE207 / 12CE201 / CE280		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

- Define ‘Reduced level’.

- What is contour equivalent?

- What is transiting?

- What is known as omitted measurements?

- Define the term ‘staff intercept’

- Define Tacheometry.

- Draw a simple curve.

- Write any type of vertical curve.

- What are called Laplace stations?

- Define the term ‘reduction of centre’.

PART – B  (5 x 3 = 15 MARKS)

- What is permanent bench marks?

- Distinguish between trivet and tribrach.

- What is tacheometer? State the principle of tacheometry.

- What is a reverse curve?

- What is satellite station?

PART – C  (5 x 15 = 75 MARKS)

- The following observations were made during a fly leveling operation.

BS – 0.650, 2.155, 1.405, 2.655, 2.435 m

FS – 2.455, 1.305, 0.555, 2.405 m

The first backsight was taken on a BM of RL 90.500 m. From the last backsight, it is 	required to set four pegs each at a distance of 30 m on a falling gradient of 1 in 100. 	Calculate the RLs of these four pegs.

(OR)

17.	A railway embankment of formation width of 8 m and side slope 2:1 is to constructed. The 	ground level along the centre line is as follows:

Chainage	-	     0			    50	   100	   150	   200	   250

GL(m) 		-	115.75		144.35	116.80	115.20	118.50	118.25

The embankment has rising gradient 1 in 100, and the formation level at zero chainage is 115.00. Assuming the ground is level across the centre line, compute the volume of earth work.

18.	Describe the process of the temporary adjustment of a theodolite. Also explain the sensitivity of 	the bubble tube.

(OR)

[P.T.O]

19.	In order to ascertain the elevation of the top (P) of the signal on a hill, observations are made 	from two instrument stations A and B being in line with P. The angles of elevation of P at A 	and B were 28 42 and 18 16 respectively. The staff readings upon the bench mark of 	elevation 287.28 were 2.870 and 3.750 respectively when the instrument was at A and B, the 	telescope being horizontal. Determine the elevation of the foot of the signal if the height of the 	signal above its base is 2.75 m.

20.	The following observations made with a tacheometer fitted with an anallactic lens.

| Ins. Stations | Staff Point | Bearing | Vertical Angle | Staff Readings |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| P | A | 134 | +1032 | 1.360 | 1.915 | 2.470 |
|  | B | 224 | +56 | 1.065 | 1.885 | 2.705 |

Determine the gradient from a point A to a point B, if the constant of the instrument was 100 	and the staff was held vertically.

(OR)

21.	Briefly describe the advantages and disadvantages of plane table survey.

22.	Two tangents of a circular curve of radius 300 meters have a deflection angle of 90. It is 	proposed to change the position of the forward tangent by rotating it through 20, thus making 	the deflection angle equal to 110. Calculate the radius of the new curve is P.C. is unchanged. 	If the change of original P.I is 3240.8, calculate the chainage of new P.I and new P.T.

(OR)

23.	Along the alignment of a road, it is found in a particular portion that a grade of +0.5 % is 	followed by one of -0.7 %. The two end of this portion are 550.00 and 375.50 m respectively. 	Calculate the RLs of the points on the curve, taking a peg interval of 30 m, the rate of change of 	grade is 0.15 % per 30 m.

24.	Write short notes on the following:

a.	SIGNALS							b.	TOWERS

(OR)

25.	A nominal distance of 30 m was set out with a 30 m steel tape from a mark on the top of one 	peg to a mark on the top of another, the tape being in catenary under a pull of 100 N and at a 	mean temperature of 70f. the top of one peg was 0.25 m below the top of the other. The top of 	the marks on the two pegs and reduce is to mean sea level, if the tape was standardized at a 	temperature of 60 f in caternary under a pull of (a) 80 N (b) 120 N and (c) 100 N. Take radius 	of earth = 6370 km; Density of tape = 7.86 g/cm3; Section of tape = 0.08 sq.cm; Co-efficient of 	expansion = 6 x 10-6 per 1 f; Young’s modulus = 2 x 107 N/cm2.