Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	GEOTECHNICAL ENGINEERING/ SOIL MECHANICS / MECHANICS OF SOILS							      	     				      Time : 3 hours

Subject Code:	09CE212/ 12CE211/ CE247						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define degree of saturation.

2.	Define liquid limit.

3.	Define permeability.

4.	List the laboratory methods to determine permeability.

5.	Define consolidation.

6.	What are the methods adopted to compact soil in field?

7.	What is meant by effective stress?

8.	What is meant by contact pressure?

9.	Name the various methods used to find stability of slopes.

10.	Who is the father of soil mechanics?

PART – B  (5 x 3 = 15 MARKS)

11.	A compacted sample of soil with bulk unit weight 19.62kN/m3 has a water content of 15%. What is its dry density?

12.	List the factors that affect permeability.

13.	What are the factors affecting compaction?

14.	What are the various causes for settlement?

15.	What are the tests adopted to determine shear strength in laboratory?

PART – C  (5 x 15 = 75 MARKS)

16.	A wet sample of mass 1.9 kg .had a volume of 945cm3.  After drying its mass reduced to 1.7 kg. The specific gravity was found to be 2.7. Determine the moisture content, bulk density, dry density, void ratio, porosity and degree of saturation.

(OR)

17.	Elaborate briefly the laboratory procedures to determine moisture content, specific gravity and size of particles.

18.	a.	The falling head permeability test was conducted on a soil sample of 4cm diameter and     	18 cm length. The head fell from 1m to 0.4m in 20 minutes. If the cross sectional area of 	stand pipe was 1 cm2	, determine the coefficient of permeability.					    (9)

b. 	What do you understand by quick sand phenomena?	                          				    (6)

(OR)

19.	Explain in detail the IS method of classification of soil.

20.	a. 	Explain how proctor compaction test is conducted.		          					    (7)

b. 	Name the various methods of field compaction. Explain any one method.			    (8)

(OR)

21. 	Write the assumptions of Terzhaghi’s theory of one – dimensional consolidation.

[P.T.O]

22. 	Sketch the variation in total, effective and pore water pressure upto a depth of 6m below ground level given the following data. The water table is 2m below the ground level. The dry unit weight is 17kN/m3. Water content is 12% and specific gravity is 2.65. What would be the change in these stresses, if water table drops by 1m. .Assume any other data if required.

(OR)

23.	A concentrated load of 200t is applied at the ground surface. Determine the vertical stress at a point P which is 6m directly below the load. Also calculate the vertical stress at a point R which is at a depth of 6m but at a horizontal distance of 5m from the axis of the load.

24. 	Explain direct shear test in detail.

(OR)

25. 	Write the step by step procedure to determine angle of shearing resistance by Mohr’s stress circle.