Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DESIGN OF MASONRY AND TIMBER STRUCTURES/BASIC STRUCTURAL DESIGN								      Time : 3 hours

Subject Code:	09CE213 / CE235								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.  	Classify the structures on the basis of materials.

2.	Write two assumptions of working stress design of RC structures.

3. 	What is the function of load bearing walls?

4. 	Define retaining wall.

5. 	What is a short column?

6. 	What type of footing block to be provided for erecting machines?

7. 	Write any two common defects in timber.

8. 	Give the sketch of the built – up beams.

9. 	Define bolt.

10. 	Sketch any two type of welding.

PART – B  (5 x 3 = 15 MARKS)

11. 	Classify the walls under loading condition.

12. 	Name the various types of masonry structures used for construction.

13. 	Write the strength criteria of masonry structures.

14. 	Define seasoning of timber and brief any 2 methods of reasoning.

15. 	Write the advantages of high strength bolts.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Discuss the various structural elements present in the structure. 					(8)

b.	Write the requirements that govern the structural design. 							(7)

(OR)

17. 	a.	Explain in detail the drawbacks of working stress method. 						(8)

b.	Classify the main limit states in the limit state design of RC structures. 				(7)

18. 	Determine the allowable axial load per m length of a 200 mm solid wall.  The height of the wall between thicker plinth wall and slab is 3.5 m.  The wall is continuous and its length b/w cross walls is 5 m.  Take fc= 0.55 mPa.

(OR)

19. 	A concrete dam of trapezoidal section having water on vertical face is 16 m high.  The base of the dam is 8 m wide and top is 3 m wide.  Find the resultant thrust per meter length of the dam.

20. 	Determine the area and depth of foundation of a square column carrying 1000KN vertical load. The gross bearing capacity of soil is 100kN/m2, density is 17kN/m3and angle of repose is 30°.

(OR)

[P.T.O]

21.	Determine the load carrying capacity of reinforced brick Masonry composite pier of size 35 cm x 70 cm which has a concrete cone of 15 cm x 45 cm.  The pier is constructed in I class brickwork in cement mortar 1:6 with bricks having strength of 10 MPa. Take permissible bending stress of M10 concrete as 3 MPa.

22.	A teak wood column is 20 cm x 20 cm.  Determine the safe axial load if the unsupported length is (i) 2m  (ii) 4 m.  Take permissible compression stress parallel to grain, fcp as 8.63 MPa and elastic modulus of teak wood as 9420 MPa.

(OR)

23. 	A timber beam of 6m clear span carries an uniformly distributed load of 15 kN/m including the self weight of the beam.  Assume the beam to be made of deodar wood.  Design the beam.

24.	Determine the strength of a 20 mm diameter ordinary turned bolt if it is used to connect

a.	Two plates each of 20 mm thick.

b.	Three plates, the outer two plates are 8 mm thick and intermediate plate 14 mm thick.

Take τvf=100Nmm2and σpf=300Nmm2

(OR)

25.	A tie member in a truss girder is 250 mm x 14 mm in size.  It is welded to a gusset plate 10 mm thick by a fillet weld.  The overlap of the member is 300 mm.  The weld is 6 mm is size.  Find the strength of the joint if the welding is done in 3 sides as shown.  What is the increase in strength if the welding is done all around? Allowable shear stress in weld = 108 MPa.