Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	APPLIED HYDRAULICS AND FLUID MACHINES      	     Time : 3 hours

Subject Code:	09CE216/12CE209/CE246						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Distinguish between Uniform flow and Non-uniform flow.

2.	Define hydraulic jump.

3.	Write the formula for displacement thickness.

4.	Define drag force.

5.	Define Impulse momentum principle.

6.	What is a draft tube?

7.	Distinguish between a Impluse turbine and Reaction turbine.

8.	List any 4 types of pumps.

9.	What is cavitation in a pump?

10.	What is a submersible pump?

PART – B  (5 x 3 = 15 MARKS)

11. 	A 6m wide channel conveys 20m³/s of water at a depth of 1.2m. Calculate specific energy

and critical depth.

12. 	Describe the term boundary layer.

13.	A jet of water, 60mm in diameter, issues with a velocity of 15m/s impinges on a stationary flat plate which destroys its forward motion .Find the forces exerted by jet on plate and work done.

14. 	List the classification of turbines with the examples.

15. 	What is the need of air vessel in a reciprocating pump?

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Define the most economical section for an open channel. 							    (3)

b.	A trapezoidal channel having the side slope equal to 60o with the horizontal and laid on a

slope of 1 in750, carries a discharge of 10m3/s. Find dimensions of most economical channel section. Take c = 66.         												  (12)

(OR)

17. 	A 3.6m wide rectangular channel conveys 90m3/s of water with a velocity of 6m/s. (1) Is there

a condition for hydraulic jump to occur? If so, calculate the height, length and strength of jump. What is loss of energy/kg of water?

18. 	The velocity distribution in the boundary layer is given by uU=yδ , where u=velocity at a distance of y from the plate and u=U at y= δ, δ  being boundary layer thickness . Find

(a) Displacement thickness (b) Momentum thickness (c) Energy thickness

(d) Value of δθ.

[P.T.O.]

(OR)

19.	a.	What is drag and lift force.				       							    			    (3)

b.	A truck having a projected area of 6.5m2 travelling at 70km/hr has a total resistance of

2000N of this 20% is due to rolling friction and 10% is due to surface friction. The rest is

due to form drag. Make calculation for form drag. ρ=1.22kg/m3 for air.  			  (12)

20. 	A 75mm diameter jet having a velocity of 30 m/s strikes a flat plate, the normal of which is

inclined at 45o to the axis of the jet. Find the normal pressure on the plate

a.	When plate is stationary.

b. 	When plate is moving with a velocity of 15m/s, moving away from the jet, the normal force.

c. 	W.d/sec.

d.	Efficiency of the jet.

(OR)

21. 	A jet of water of 50mm diameter impinges on a curved vane and is deflected through an angle of 175o. The vane moves in the same direction of the jet with a velocity of 35m/s. If the rate of flow is 170 ltrs/sec. Determine the component of force on the vane in the direction of motion. How much would be the power developed by the vane and what would be the water efficiency Neglecting friction?

22. 	The following data relates to a pelton wheel, Head = 72m, speed of wheel = 340 R.P.M, shaft

Power of wheel 115KW, speed ratio = 0.45, co efficient of velocity = 0.98, overall efficiency

85%. Design the pelton wheel.

(OR)

23.	Give the layout of a hydro electric power plant and explain.

24.	Discuss in detail about any one type of pump with neat sketch.

(OR)

25. 	A centrifugal pump impellor having external and internal diameter 480mm and 240mm respectively is running at 1000 R.P.M. the rate of flow through the pump is 0.0576m3/s and velocity of flow is constant and is equal to 2.4m/s. the diameter of suction and delivery pipes are 180mm and 120mm respectively and suction and delivery heads are 6.2m (abs) and 30.2m of water respectively. If the power required to the pump is 23.3 kw and the outlet vane angle is 45o. Determine (a) Inlet vane angle (b) Overall efficiency of pump (c) Manometric efficiency of pump.