Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	REINFORCED CONCRETE STRUCTURES - II		       	     Time : 3 hours

Subject Code:	09CE217/CE250				 					       Maximum Marks: 100

#### Answer ALL questions

Note: IS:456-2000, IS:3370-1965, IS: 1893 – 2002, IRC, Pigeaud’s Charts,

IRC:6-2000, IRC:21-2000, SP 16 are allowed.

Assume suitable missing data.

PART – A  (10 x 1 = 10 MARKS)

1.	What are the types of staircases?

2.	Why counterfort retaining walls are recommended for larger heights?

3.	Which type of water tank is used for large storage of structures?

4.	What is the minimum recommended concrete mix for water tanks as per BIS Code?

5.	Classify R.C. bridge according to class.

6.	What is the analysis method used in T – beam girder bridge?

7.	What is the code book used for analysis of Earthquake load?

8.	Write any one type of Substitute Frame.

9.	Draw the yield line pattern for a circular slab simply supported on all sides.

10.	What are the two methods for determining the ultimate load capacity of RC slabs by yield line method?

PART – B  (5 x 3 = 15 MARKS)

11.	What is the difference between cantilever retaining wall and counter fort retaining wall?

12. 	Brief the concept of Construction Joint in water tanks.

13. 	Write the minimum reinforcement requirements in bridge slabs according to IRC:21-2000.

14. 	What are the assumptions made in the Portal method of analyzing the frames?

15. 	What are the characteristic features of yield line patterns?

PART – C  (5 x 15 = 75 MARKS)

16. 	Design the stem, toe slab for a Counterfort Retaining wall based on the following data:

Height of wall above GL = 6m

SBC of soil = 160 kN/m3

Angle of internal friction = 30o

Density of soil = 16 kN/m3

Spacing of counterforts = 3 m c/c

Show the stability calculations and show the pressure distribution diagram. Assume suitable missing data.

(OR)

17. 	Design a dog-legged staircase for an office building, given the following data:

Height between floor = 3.2m

Riser = 160mm, Tread = 270mm

Live load = 5 kN/m2

Floor finishes = 0.6 kN/m2

Assume the stairs to be supported on 230mm thick masonry walls at the outer edges of the loading, parallel to the risers. Use M20 concrete and Fe415 steel.

[P.T.O]

18. 	Design a circular water tank for a storage capacity of 3,60,000 litres. The joint between the wall and the floor of the tank is flexible. The tank is to rest at ground level. Adopt M20 grade concrete and HYSD bars.

(OR)

19.  	Design an open square RC tank to hold 60,000 litres of water. The depth of water in the tank is to be restricted to 3.5m. Good foundation for the tank is available at 0.6m below the ground level. The SBC of the soil at that depth may be taken as 5 kN/m2. Adopt M20 grade concrete and HYSD bars.

20. 	Design a RC slab culvert for a National Highway crossing to suit the following data:

a.	Carriage way – Two Lane (7.5 m wide)

b.	Foot paths – 1m on either side

c.	Clear span = 6m

d.	Wearing coat = 80mm

e.	Width of bearing = 400mm

f.	Concrete – M25 grade, Steel – Fe415 grade

g.	Loading – IRC Class AA tracked vehicle

Design the RC slab deck and sketch the details of reinforcements in the longitudinal and cross section of the slab. The design should confirm to the specifications IRC:6-2000 and IRC:21-2000 codes.

(OR)

21.	Explain Courbon’s method in detail.

22.	Write the step by step procedure of analysing a multistorey frame by substitute frame method

(OR)

23.	a.	Explain the general principle adopted from Earthquake resistant design.				    (7)

b.	Draw the reinforcement diagram for beam – column joint and footing to ensure ductility as 	per is 13920 – 1993.															    (8)

24.	A square slab 5m x 5m is simply supported at the edges. The slab is expected to carry a service live load of 3 kN/m2 and a floor finishing load of 1 kN/m2. Use M15 concrete and Fe415 steel. Design the slab. Take  = 0.75.

(OR)

25.	a.	Define yield line.																    (3)

b.	Draw the yield line patten for both simply supported and fixed conditions.

i.	Square slab

ii.	One way slab and two way slab

iii.	Circular slab																(4x3)