Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DESIGN OF STEEL STRUCTURES (LIMIT STATE METHOD)   /DESIGN OF STEEL STRUCTURES													      	     											      Time : 3 hours

Subject Code:	09CE219/12CE223/CE252						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is Column splice?

2.	Define slenderness ratio.

3.	What is bearing stress?

4.	What are the types of tension members?

5.	What are purlins?

6.	The minimum recommended rise of trusses with GI sheet is ______________.

7.	If Fe250 is used, calculate the allowable stress in axial tension as per IS 800.

8.	What are the types of roof trusses?

9.	What is the purpose of providing bearing stiffeners in a plate girder?

10.	How web crippling stress can be corrected?

PART – B  (5 x 3 = 15 MARKS)

11.	Write notes on Lug angles.

12.	What are the advantages and disadvantages of welded connections?

13.	Write about Built-up sections.

14.	What do you mean by web crippling in beams?

15.	What are the guide lines in selecting the purlin sections?

PART – C  (5 x 15 = 75 MARKS)

16.	Design a single angle section for a tension member of a roof truss to carry a factored tensile force of 225 kN. The member is subjected to the possible reversal of stress due to the action of wind. The length of the member is 3 m. Use 20 mm shop bolts of grade 4.6 for the connection.

(OR)

17.	Design a laced column with two channels back to back of length 10 m to carry an axial factored load of 1400 kN. The column may be assumed to have restrained in position but not in direction at both ends. (hinged ends).

18.	A steel is simply supported at its ends and carries an udl of 44 kN/m length excluding self-weight. The effective span of the beam is 8m. Design a suitable section for the beam. The beam is laterally prevented against deflection.

(OR)

19.	Design a simply supported beam of 10 m effective span carrying a total factored load of         60 kN/m. The depth of the beam should not exceed 500 mm. The compression flange of the beam is laterally supported by floor construction. Assume stiff end bearing is 75mm.

[P.T.O]

20.	Design a welded plate girder 24 m in span and laterally restrained throughout. It has to support a uniform load of 100 kN/m throughout the span exclusive of self-weight. Design the web, flange and flange to web connection. Also check for stresses.

(OR)

21.	Design a stiffened seat connection for an ISMB 350 @ 514 N/m transmitting an end reaction of 320 kN (due to factored loads) to a column section ISHB 300 @ 576.8 N/m. The steel is of grade Fe 410 and bolts of grade 4.6.

22.	Design an I-section purlin for the data given below

Spacing of roof truss			-	6 m

Span of the truss				-	12 m

Spacing of purlin				-	1.5 m

Weight of G.I. sheeting		-	130 kN/m2

Wind load intensity			-	2 kN/m2

Grade of Steel				-	Fe 410

(OR)

23.	Determine the design load on the purlins of an industrial building near Visakhapatnam, given

Class of Building			-	General with life of 50 years

Terrain					-	Category 2

Maximum Dimension		-	40 m

Width of building			-	15 m

Height at eve level		-	8 m

Topography				-	θ less than 30

24.	Determine the safe axial load carrying capacity in compression of the light gauge channel      (C) section shown in Fig. The effective length of the member 1s 3 m. Yield strength of the steel, fy = 250 MPa.

(OR)

25.	A Self supported steel stack is 80 m high and its diameter at top is 3m. Design the plates for the stacks. Adopt wind force as per IS 875. The location of the plate is such that the intensity of wind pressure up to 30 m high is 1.5 kN/m2.