Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	FOUNDATION ENGINEERING						      	     Time : 3 hours

Subject Code:	09CE221/12CE219/CE248						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Bearing Capacity.

2.	Write any One Requirement of a Good Foundation.

3.	Write down any two Limitations of Plate Load Test.

4.	What are the Different Types of Failures?

5.	Define Coefficient of Earth Pressure.

6.	Write two types of Sheet Pile Wall.

7.	What is the use of End bearing Pile?

8.	Can the Group Efficiency be greater than one?

9.	What do you mean by Caisson?

10.	When do you prefer Combined Footing?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the Functions of Foundation?

12.	What are the Methods to improve the Bearing Capacity of the Soil?

13.	Differentiate Active and Passive Earth Pressure.

14.	A Pile Group consisting of 9 piles with 3X3. Find the efficiency of pile group.

15.	What are the Different Components of Well Foundation?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the Geophysical Methods of Soil Exploration with neat sketches.

(OR)

17.	Explain the Different Types of Foundation with neat diagrams.

18.	A plate load test was conducted with a 30 cm square plate at a depth of 1.2 m below the ground level.  In a cohesive soil having φ=0.  The failure was observed at a load of 36 kN.  The water table was observed to be at a depth of 4.7 m below ground surface. Find the bearing capacity.

(OR)

19.	a.	Explain plate load test with suitable diagram.									  (10)

b.	Explain standard penetration Test.												   (5)

20.	a.	Define sheet pile wall.															   (5)

b.	Briefly explain the Rankine’s theory of active earth pressure.						  (10)

21.	Explain cofferdam and its types.

22.	An n- pile group has to be proportioned in a uniform pattern in soft clay with equal spacing in all directions. Assuming any value of c, determine the optimum value of spacing of piles in the group. Take n=25 and m=0.7. Neglect the end bearing effect and assume that each pile is circular in section.

[P.T.O]

(OR)

23.	a.	Explain pile load test in detail.													(8)

b.	Explain types of piles.															(7)

24.	Write short notes on

a.	Rectangular footing.

b.	Trapezoidal footing.

c.	Raft footing.

(OR)

25.	What are Caissons? Draw the section of well foundation and explain its component parts.