Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	FLUID MECHANICS AND MACHINERY  			      	     Time : 3 hours

Subject Code:	09CE242/12CE263/CE212						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.  	Distinguish between ideal fluid and real fluid.

2.  	Define Capillary rise.

3.  	Define path line.

4.  	Write the continuity equation for one dimensional flow.

5.  	List any two types of the minor losses.

6.  	What is a venturimeter?

7.   	Define slip in a reciprocating pump.

8.   	Define impulse momentum principle.

9.   	What is the basic classification of Turbine?

10. 	List the various types of pumps.

PART – B  (5 x 3 = 15 MARKS)

11. 	One litre of oil at 40˚c temperature has mass 900 gms. Calculate the mass density, specific weight and specific gravity of oil.

12.	List the types of fluids.

13.	Classify losses in pipes

14. 	Draw the performance characteristics curves for centrifugal pumps.

15. 	What is governing of turbines?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Define Newton’s law of viscosity.												    (3)

b.	A plate having area of 0.6 m2 sliding down the inclined plane at 30° to the horizontal with a velocity of 0.36 m/s. There is a cushion of fluid 1.8mm thick between the plane and the plate. Find the viscosity of the fluid if the weight of the plate is 280N.				  (12)

(OR)

17.	What is manometer? Explain briefly the various types of manometers.

18.	a.	Explain briefly the various types of flow.										    (8)

b.	In a 2 dimensional incompressible flow. The fluid velocity component are given by

u = x- 4y and v = -y - 4x. Show that velocity potential exists and find its stream function.

(7)

(OR)

19.	Derive Bernoulli’s equation and list its assumption.

20.	a.	Derive the Darcy’s equation.													    (8)

b.	A Pipeline of 600 mm diameter is 1.5 km long. To increase the discharge another line of the same diameter is introduced parallel to the first in the second-half of the length. If

f = 0.01 and head at inlet is 300mm calculate the increase in discharge.				    (7)

[P.T.O.]

(OR)

21.	a.	Briefly explain the different types of wires and notches.							    (7)

b.	Determine the height of rectangular weir of length 6m to be built across a rectangular channel. The maximum depth of water on the upstream side of the weir is 1.8m and discharge is 2000 lit/sec. Take Cd=0.6 and neglect end contraction.					    (8)

22.	a.	List the factors for selection of pump.											    (5)

b.	A double acting reciprocating pump, running at 40 rpm is discharge 1.0m3 of water/minute, the pump has a stroke of 400mm, the diameter of piston in 200mm. The delivery and suction head are 20m and 15m respectively, and the slip of the pump and lower required to derive the pump.												  (10)

(OR)

23.	a.	Explain ideal indicator diagram with a neat sketch.

b.	A centrifugal pump is to discharge 0.118m3/s at a speed of 1450rpm against a head of 25m.     The impeller diameter is 250mm, its width outlet is 50mm and Manometric efficiency is 75%.  Determine the vane angle at the outer periphery of the impeller.

24.   With a neat sketch explain any one type of turbine in detail.

(OR)

25.  A Pelton wheel is receiving water from a penstock with a cross head of 510m. One-third of 	cross head is lost in friction in the penstock. The rate of flow through the nozzle fitted at the 	end of the penstock is 2.2m3/s. The angle of deflection of the jet is 165°. Determine

i) the power given by water to the runner

ii) hydraulic efficiency of the pelton wheel.

Take cv (coefficient of velocity) = 1.0 and speed ratio = 0.45.