Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May - 2016

Subject Title :	MECHANICS OF SOLIDS							      	     Time : 3 hours

Subject Code:	09CE245/12CE202/12CE264/CE203				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Hook’s Law.

2.	Define Poisson’s ratio.

3.	What is a cantilever beam?

4.	Define point of contra flexure.

5.	Name the types of springs.

6.	Define torsion.

7.	What is a principal plane?

8.	What is principal stress?

9.	What is the critical load value for a column with both ends fixed?

10.	Define Deflection.

PART – B  (5 x 3 = 15 MARKS)

11.	Sketch the various types of beams.

12.	What is buckling?  Explain.

13.	What are the assumptions made in the theory of simple bending?

14.	Define:

a.	Cantilever beam.			b.	Simply supported beam.		c.	Overhanging beam.

15.	State the assumptions made in Euler’s theory.

PART – C  (5 x 15 = 75 MARKS)

16.	A thin cylindrical shell 100 cm diameter, 1 cm thick and 5 m long is subjected to an internal pressure of 3 N/mm2. Calculate: 	a.	the change in diameter	b.	change in length

c.	change in volume of the shell, if E= 2 x 105 N/mm2 and Poisson’s ratio is 0.3.

(OR)

17.	A brass bar having cross sectional area of 1000 mm2 is subjected to axial forces as shown 	in figure below:

Find the total elongation of the bar. Take E = 1.05x105 N/mm2

18.	A simply supported beam of length 6m carries point load of 3kN and 6kN at distances of 2m and 4m from the left end. Draw the shear force and bending moment diagram.

(OR)

[P.T.O]

19.	Draw shear force and bending moment diagram for the beam shown in figure.

30kN

20kN/m

5m

5m

4m

A

B

C

D

20.	The stiffness of a close coiled helical spring is 1.5N/mm of compression under a maximum load of 60N.The maximum shearing stress produced in the wire of the spring is 125N/mm2.The solid length of the spring is given as 5cm. Find

a.	Diameter of the wire 	       b.	number of coils required. Take C=4.5x104N/mm2.

(OR)

21.	A solid circular shaft of 10cm diameter of length 4m is transmitting 112.5kW power at 150r.p.m. Determine the maximum shear stress induced in the shaft and strain energy in the shaft. Take C=8x104N/mm2.

22.	The tensile stresses at a point across two mutually perpendicular planes are 120N/mm2 and 60N/mm2. Determine the normal, tangential and resultant stresses on a plane inclined at 30o to the axis of the minor stress.

(OR)

23.	The stresses at a point in a bar are 200 N/mm2 (tensile) and 100 N/mm2 (compressive). Using 	analytical method determine the resultant stress in magnitude and direction on a plane inclined 	at 60o to the axis of the major stress. Also determine the maximum intensity of shear stress in 	the material at the point.

24.	A beam of length 6m is simply supported at its ends and carries two point loads of 48kN and 40kN at a distance of 1m and 3m respectively from the left support. Find (a) the deflection under each load (b) maximum deflection. Given E=2x105N/mm2 and I=85x106mm4.

(OR)

25.	Derive the Euler’s equation for buckling load when both the ends of a column of length ‘ℓ’ are                                              hinged.