Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ENGINEERING MECHANICS						      	     Time : 3 hours

Subject Code:	09CE247/12CE203/CE204/CE242	 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define force.

2.	Define couple.

3.	Give an example of a vector quantity.

4.	State triangle law.

5.	List the different types of beams.

6.	What are the different types of loads?

7.	Define static friction.

8.	What is meant by inertia of a body?

9.	Define acceleration of a particle.

10.	Write the principle of conservation of mechanical energy?

PART – B  (5 x 3 = 15 MARKS)

11.	Describe about system of forces.

12.	State the conditions for equilibrium of a right body.

13.	A block weighing 40 N is just moved along a horizontal plane by applying a horizontal force of 16 N.  Find the reaction and the coefficient of friction.

14.	Differentiate between centroid and centre of gravity.

15.	Explain about the principle of work and kinetic energy.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the magnitude and direction θ of the force A so that the resultant of the system of forces shown in figure is horizontal and has a magnitude of 14.4 kN.

(OR)

17.	Four tugboats are used to move an ocean ship to its pier. Each tugboat exerts a force of kN in the direction as shown in figure. Determine the single equivalent force of the system of forces acting on the ship and its location along the longitudinal axis of the ship. Also determine the equivalent force-couple system at 0.

[P.T.O.]

18.	An overhanging beam is loaded as shown in figure.  Find the support reactions of the beam when a 35 kN.m couple acts at C as shown.

(OR)

19.	A block of weight 150 N is resting on a rough inclined plane as shown in the following figure.  The block is tied up by a horizontal string, which has a tension of 50 N.  Find (a) The frictional force on the block (b) The normal reaction of the inclined plane (c) The co-efficient of friction between the surfaces of contact.

20.	Find the moment of inertia of the section shown in figure about its centroidal axes.

(OR)

[P.T.O.]

21.	Find the moment of inertia of the section shown below.

22.	A particle moves along a straight line with variable acceleration. If the displacement is measured in ‘m’ and given by  S = 3t3 + 2t2 + 7t +3 where ‘t’ is the line. Determine (a) velocity of the particle at start and after 3 seconds.   (b) Acceleration of particle at start and after 3 seconds.

(OR)

23.	Two balls are projected from the same point in directions inclined at 60° and 30° to the horizontal. If they attain the same maximum height, what is the ratio of their velocities of projection?

24.	A body of mass 15 kg is initially at rest on a 10° inclined plane.  Then it slides down.  Calculate the distance moved by the body, on the inclined plane, when the velocity reaches to 6 m/s.  The coefficient of friction between the body and the plane is 0.1.

(OR)

25.	A body of weight 600 N, is place on a 20° inclined plane and pulled by a 500 N force, applied parallel to the plane as shown in figure.  If the initial velocity of the body is 2 m/s, calculate the final velocity of the body, when it has travelled a distance of 2m.  Take the coefficient of kinetic friction between the body and the plane is 0.2.