Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ELECTROMAGNETIC FIELDS						      	     Time : 3 hours

Subject Code:	09EC221						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Divergence of curl of a vector is ___________.

2.	Given a vector field E→=3ax→+5ay→+2az→in rectangular coordinate system. Find its direction alongE→.

3.	Give the Gauss Law for Magnetic Fields.

4.	H.dl=___________.

5.	Define Polarisation.

6.	_________ is the point form of Ohm's law.

7.	Define Magnetization.

8.	Define self inductance.

9.	What is the significance of displacement current?

10.	State Lenz's Law.

PART – B  (5 x 3 = 15 MARKS)

11.	A uniform line charge infinite in extent with ρL=20nC/m lies along Z axis. Find E→at (6, 8, 3).

12.	What do you infer from Gauss’s law for Magnetostatic fields?

13.	Determine whether the potential field V=x2−y2+z2 satisfy the Laplace equation.

14.	Calculate the inductance of a solenoid of 200 turns wound tightly on a cylindrical tube of 6cm diameter. The length of the tube is 60cm and the solenoid is in air.

15.	Explain flow of power in coaxial cable.

PART – C  (5 x 15 = 75 MARKS)

16.	A vector field A=→1Ra→R is given in spherical coordinate system. Determine A→in a cylindrical form at x=1, y=1, and z=1 unit.

(OR)

17.	Explain the following.

a.	Divergence Theorem.															    (7)

b.	Stokes Theorem.																    (8)

18.	Derive an expression for magnetic field intensity due to a linear conductor of infinite length carrying current I at a distant point P. Assume R to be the distance between conductor and point P. Use Biot Savart Law.

(OR)

[P.T.O]

19.	Apply Amperes circuital law to find the Magnetic field intensity of coaxial cable for following regions.

Region 1 : r<a

Region 2 : a<r<b

Region 3 : b<r<c

Region 4: r>c(outside the cable)

20.	Derive the boundary conditions of the normal and tangential components of electric field at the interface of two media with different dielectrics.

(OR)

21.	Derive the point form and integral form of continuity equation.

22.	a.	Derive the expressions for energy stored and energy density in the magnetic field. 	    (8)

b.	Derive an expression for inductance of a solenoid with N turns and l meter length carrying 	a current of I amperes. 														    (7)

(OR)

23.	Obtain the boundary relations which hold good for the interface of two different magnetic materials.

24.	Derive the Maxwell’s curl equation from Ampere’s law and Faraday’s law.

(OR)

25.	Discuss integral and point forms of Poynting theorem.