Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	COMMUNICATION THEORY AND SYSTEMS		      	     Time : 3 hours

Subject Code:	09EC222/11EC206								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the range of modulation index to avoid distortion?

2.	Calculate the minimum height required of an antenna to transmit a signal of frequency of 5 KHz.

3.	Define Modulation.

4.	Draw the spectrum of AM wave.

5.	What is the modulation index of FM?

6.	Draw the block diagram of high level modulation system.

7.	Give the power output range of the AM Broadcast transmitter.

8.	How many sidebands are available in WBFM signal?

9.	The error occurred during tracking (non-constant IF) is called as ___________.

10.	The signal to noise ratio is measured in ____________.

PART – B  (5 x 3 = 15 MARKS)

11.	List the types AM modulation.

12.	Give the block diagram of Communication System.

13.	Differentiate direct method and indirect method of FM generation.

14.	Define fidelity in an AM receiver.

15.	Obtain the ways to reject the image frequency signal.

PART – C  (5 x 15 = 75 MARKS)

16.	Define AM Modulation and derive the expression for AM waveform and draw the frequency spectrum of AM wave.

(OR)

17.	Explain the working of balanced law modulator and derive the expression for double side band suppressed carrier.

18.	Draw the block diagram and waveform for the working of envelope detector and estimate the RC value to avoid the  time constant.

(OR)

19.	Derive the expression for AM DSBSC detector and explain the working in detail.

20.	a.	Explain the Armstrong method of generating FM signals.							    (7)

b.	Using phasor diagrams, show how Foster Seeley discriminator is used for detecting FM signals.																		    (8)

(OR)

21.	a.	Explore the application of square law for detecting AM signals. Include mathematical 	equations wherever necessary.													    (7)

b.	How will you generate FM signals using varactor diodes?							    (7)

[P.T.O]

22.	a.	Derive the expression for additive mixer signal with necessary mathematical equations.

(8)

b.	Give the merits of the Single side band transmitter for AM radio telephony.	 	    	    (7)

(OR)

23.	a.	Explain in detail about tracking and alignment in the communication receiver and types of 	tracking used in the receiver.													    (8)

b.	Using block diagrams show how TRF receiver is used for detecting AM signals.		    (7)

24.	Explain in detail about the various types of noises in the signal.

(OR)

25.	Obtain the noise in the AM signal and draw the spectral density of the white noise.