Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL SIGNAL PROCESSING					      	     Time : 3 hours

Subject Code:	09EC224						 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Complex Conjugate property of DFT.

2.	The 16 point DFT computation using FFT algorithm requires _________numbers of stages.

3.	What is the width of the main lobe for an N-point hamming window?

4.	How to maintain the linear phase in the FIR filter?

5.	By Impulse Invariance method, obtain the digital filter transfer function of the analog filter H(s)=1(s+1).

6.	What are the properties of bilinear transformation?

7.	What do you understand by input quantization error?

8.	How can overflow limit cycles be eliminated?

9.	What is pipelining?

10.	Mention the addressing modes of TMS320C5X.

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between DIT and DIF FFT algorithms.

12.	What is Gibb’s oscillation?

13.	Give the bilinear transformation equation between s-plane and z plane.

14.	List the advantages of floating point arithmetic.

15.	Differentiate between Von Neumann and Harvard architectures.

PART – C  (5 x 15 = 75 MARKS)

16.	Using radix-2 DIT-FFT algorithm, compute 8 point-DFT of the sequence x(n) = 0 ,1, 2, 3, 4, 5, 6, 7. Draw the flow graph and show all the intermediate results.

(OR)

17.	a. 	Determine the output of a linear filter whose impulse response h(n) = 1, −3, 5 and input 	signal x(n) = −1 ,4, 7, 3, −2, 9,10, 12,−5,8using overlap add method.				  (10)

b. 	Determine the DFT of the sequence x(n) =14 , for 0 ≤ n ≤ 30 otherwise					    (5)

18.	Design a FIR linear Phase digital filter approximating the following ideal frequency response using a Hanning window with M = 7. 				     	        		         Hd(ejω) =e−j3ω for −π4≤ ω ≤ π40 for π4≤ ω ≤ π

(OR)

[P.T.O]

19.	Find the filter coefficients h(n) obtained by the frequency sampling method for N=7.			         Hd(ejω) = e−j(N−1)ω2 0 ≤ ω <π20 otherwise

20.	Design a Butterworth low pass digital IIR filter satisfying the following

0.7≤H(ejω)≤1 for 0≤ ω ≤ 0.2π  H(ejω)≤0.3 for 0.6π≤ ω ≤ π

with T=1 sec apply impulse invariance method.

(OR)

21.	Design a Butterworth digital IIR LPF satisfying the following

0.9 ≤ H(ejω) ≤ 1.0 ; 0<ω≤π2 H(ejω) < 0.2 ; 3π4≤ω<π

by taking T=1 second. Apply bilinear transformation.

22.	The coefficient of a system defined by H(z)=1(1−0.4z−1) (1−0.55z−1)are represented in a number system with a sign bit and 3 data bits using signed magnitude representation and truncation. Determine the new pole location for direct realization and for cascade realization of first order systems.

(OR)

23.	An IIR causal filter has the system function H(z)=z(z−0.97) . Assume that the input signal is zero-valued and the computed output signal values are rounded to one decimal place. Show that under these stated conditions, the filter output exhibits a dead band effect. Also determine the dead band range.

24.	a. 	Draw the flowchart for basic LMS adaptive filter and explain the computational procedure 	for the LMS algorithm.														  (10)

b. 	Distinguish between general purpose and special purpose digital signal processors.	    (5)

(OR)

25.	With suitable block diagram explain in details about TMS 320C5X DSP processor memory architecture.