Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May - 2016

Subject Title :	BASIC ELECTRICAL ENGINEERING					     Time : 3 hours

Subject Code:	09EE101/12EE101/DEE101/EE101				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Electric power is the ratio of ___________ and ___________.

2.	The equivalent resistance of two parallel connected resistors R1 and R2 is ___________.

3.	Any induced emf will circulate a current in such a direction so as to oppose the cause producing it is called as _________.

4.	Unit of magnetizing force is _____________.

5.	Form factor and peak factor for a sinusoidal wave are ___________ and ___________.

6.	The length of the short transmission line is ____________.

7.	Write the expression for voltage transformation ratio of a transformer.

8.	An alternator works on the principle of ___________.

9.	The instrument used for measuring dc quantities is __________.

10.	Induction type energy meter is an indicating instrument.  State true or false.

PART – B  (5 x 3 = 15 MARKS)

11.	State Ohm's law and mention its limitations.

12.	List down the different kinds of magnetic materials?  Give one example each.

13.	Draw the hydro power station layout.

14.	Explain the principle of a transformer.

15.	List down the different types of wiring.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Define the Kirchoff’s Laws and illustrate the use of Kirchoff’s Laws in a circuit.		  (10)

b.	Calculate the equivalent resistances across the terminals of the supply.				    (5)

(OR)

17.	Derive the expression for the equivalent star network resistances for a given delta network.

(15)

18.	a.	Compare magnetic circuits and electric circuits.									  (10)

b.	Write any five characteristics of magnetic flux lines.								    (5)

19.	The flux produced in the air gap between two electromagnetic poles is 5  102 wb. If 	the cross sectional area of the air gap is 0.2 m2 find:

a.	flux density

b.	magnetic field density

c.	reluctance

d.	Permeance of the air gap

e.	mmf dropped in the air gap given the length of the air gap to be 1.2 cm.

[P.T.O.]

20.	With a neat sketch, explain the construction and operation of a nuclear power station.

(OR)

21.	a.	Compare overhead and underground transmission system.							    (5)

b.	Explain the basic layout of a power system network with a neat diagram.			  (10)

22.	Explain the construction and working of a DC generator with a neat sketch.

(OR)

23.	With a neat diagram explain the Principle, Construction and Working of a three phase induction motor.

24.	Explain the construction and operating principle of moving iron instruments with an example.

(OR)

25.	a.	Draw a neat diagram of a staircase wiring and explain its working.					(7)

b.	What is the principle of Earthing? Explain the procedure of the same.				(8)