Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL ELECTRONICS							      	     Time : 3 hours

Subject Code:	09EE202/12EE220/EE257		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Prove that  x+x.y=x

2.	Convert (634)8 to binary.

3.	What are the types of TTL logic?

4.	For a 16-to-1 multiplexer, find the number input lines and selection lines.

5.	Define Flip-flop.

6.	Define multiplexer.

7.	Distinguish between Mealy and Moore machines.

8.	What is the classification of sequential circuits?

9.	List basic types of programmable logic devices.

10.	Define fan- in.

PART – B  (5 x 3 = 15 MARKS)

11.	Simplify the following expression

Y = (A + B) (A + C) (B + C)

12.	Identify the following gate. Write its truth table.

13.	Two negative edge-triggered T Flip-flops are connected in cascade as shown in the figure.

If the input is as given below then draw the output waveforms of both the flip-flops.

14.	Explain the flip-flop excitation tables for RS FF.

15.	Give the comparison between PROM and PLA.

PART – C  (5 x 15 = 75 MARKS)

16.	Simplify the Boolean function using tabulation method.

F = (0, 1, 2, 8, 10, 11, 14, 15).

(OR)

17.	State the postulates and theorems of Boolean algebra.

[P.T.O]

18.	Explain with neat diagram how an open collector TTL operates. Also enumerate about the advantages and disadvantages compared with the other families.

(OR)

19.	Design and implement full adder by writing Boolean expressions. How will you construct the full adder with two half adders?

20.	Reduce the following function using k-map technique.

F (A,B,C,D,E) = Σ m (1,4,8,10,11,20,22,24,25,26) + d(0,12,16,17)

(OR)

21.	a. 	Implement a Full-Adder circuit with two half-adders and an OR gate.                		(8)

b. 	Write short notes on Decoders and design a 3 to 8 decoder.                                		(7)

22.	Design a sequential circuit which has one input and one output with the state diagram shown below. Use D flip-flops.

(OR)

23.	Explain in detail with neat illustration the working principle of the serial in serial out shift register.

24.	Explain in detail about the architecture of PLA with specific examples.

(OR)

25.	Implement the following Boolean function using PLA.

F0 = A + B C

F1 = A C + A B

F2 = B C + A B

F3 = B C + A