Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	CONTROL SYSTEMS								      	     Time : 3 hours

Subject Code:	09EI219 / 10EI205 / EI203		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	True or false: Closed loop systems are accurate and reliable.

2.	Define transfer function.

3.	State ‘time constant’ of a system.

4.	What is steady state error?

5.	Define gain margin.

6.	For a system to be stable, the roots of the system should lie to the _________ half of s-plane.

7.	A system is stable if for a bounded disturbing input signal the output vanishes ultimately as ‘t’ approaches _________.

8.	True or false : PI controller reduces the steady state error.

9.	Find the ‘order of the system given below

G(s)=s+2s2s+1s+4

10.	Draw the block diagram of a closed loop control system.

PART – B  (5 x 3 = 15 MARKS)

11.	Give the basic properties of signal flow graph.

12.	List the standard test signals.

13.	List the frequency domain specifications.

14.	State Routh stability criterion

15.	Compare the features of open loop and closed loop systems.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the closed loop transfer function C(s)/R(s) using block diagram reduction technique.

(OR)

17.	Derive the expression for transfer function of field controlled dc motor.

18.	Obtain the response of first order system with unit step input by deriving the equations.

(OR)

[P.T.O]

19.	Open loop transfer function of a servo system with unity feedback if G(s)=10s0.1s+1.  Obtain the static error constants.  And also steady state error when subjected to a input given by rt=a0+a1t+a2tt2

20.	Draw the bode plot and obtain gain cross over frequency for the transfer function Gs=20s1+3s1+4s

(OR)

21.	Sketch the polar plot and determine the gain margin and phase margin of an open loop transfer function of a unity feedback system is given by Gs=1s21+s1+2s.

22.	Comment the stability of the system using Routh criterion for the characteristic equation s6+2s5+8s4+12s3+20s2+16s+16=0.  And also discuss about the location of roots on s-plane.

(OR)

23.	Sketch the locus of the roots for the unity feedback system whose open loop transfer function is

GsHs=ks+1.5ss+1s+5

24.	Design as electronic proportional controller using operational amplifier.

(OR)

25.	Find the rise time setting time for the system given below :G(s) = 10ss+4 .