Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ELECTRICAL MACHINES							      	      Time: 3 hours

Subject Code:	09EI220/12EI202				 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In a D.C generator, the generated E.M.F is directly proportional to the_________

a.	Field current		b.	Pole flux 	c.	Number of armature parallel paths

d.	Number of dummy coils.

2.	The DC series motor should never be switched on at no load because _______.

3.	How can Eddy Current Loss be minimized?

4.	A Transformer transforms

a.	Frequency 	b.	Voltage 		c.	Current 		d.	Voltage and Current

5.	Short circuit test is performed on a transformer to determine _______losses.

6.	An Electric Motor in which the both the rotor and stator fields rotate with the same speed is called _________ motor.

7.	Name the motor which may be operated either on direct or single-phase A.C. supply.

8.	The power factor of a squirrel cage induction motor is

a.	low at light loads only                     		b.	low at heavy loads only

c.	low at light and heavy loads both     	d.	low at rated load only

9.	Traction Motors are used in ___________, ___________.

10.	Give any two applications for Electric Heating __________, _________.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the necessary conditions for the Generator to be self-excited?

12.	State the concept of Ideal Transformer.

13.	List the application of an Induction motor.

14.	State the disadvantages of Shaded Pole Motor.

15.	Draw and explain the Bow Collector.

PART – C  (5 x 15 = 75 MARKS)

16.	With necessary diagrams, explain the construction and working principle of a D.C. Generator.

(OR)

17.	a.	Discuss the various methods of speed control available for DC shunt motor.                  (10)

b.	A short Shunt Compound Generator delivers a load current of 30A at 220V, and has	armature, series- field and shunt field resistances of 0.05Ω, 0.03Ω and 250Ω respectively.

Calculate the generated E.M.F and the armature current. Allow 1.0 V per brush for contact

drop.																		    (5)

18.	Describe the construction and working principle of Transformer and derive the E.M.F equation for a transformer.

(OR)

19.	A 30 KVA, 2400 / 120 V transformer has a high voltage winding resistance of 0.1 Ω and a leakage reactance of 0.22 Ω. The low voltage winding resistance is 0.035 Ω and the leakage reactance is 0.012 Ω. Find the equivalent winding resistance, reactance and impedance referred to the (i) high voltage side and (ii) the low voltage side.

[P.T.O]

20.	With a neat diagram showing all parts, explain the working of an Induction Motor. Obtain the condition for maximum torque.

(OR)

21.	a.	Explain double field revolving theory.                                                                                (5)

b.	Comment on different starting methods of Single Phase Induction Motors.                    (10)

22.	Write a detailed technical note on the following:

a.	Capacitor Start and Capacitor Run Motors                                                                  	    (8)

b.	Split-Phase Induction Motor                      		                                                              (7)

(OR)

23.	Describe the Stepper Motor operation in the following modes:

a.	Half Stepping Mode and                                                                                  			    (8)

b.	Full Stepping Mode                                                                                         			    (7)

24.	Illustrate the applications of Electric Heating with suitable example.

(OR)

25.	a.	With neat diagram explain the Recent Trends in Electric Traction.                                    (8)

b.	Discuss about Welding Generator.											            (7)