Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	COMPUTER ARCHITECTURE						      	     Time : 3 hours

Subject Code:	09IT201/12IT204/IT235							       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A fetched instruction is loaded into a register in the processor known as _________.

2.	List the four functions a computer can perform.

3.	Draw the Memory Hierarchy diagram.

4.	What is EEPROM?

5.	Write 18 using sign magnitude representation for 8 bit.

6.	An unsigned double word is used as an _________in Power PC data types.

7.	What will be the result of arithmetic right shift (3 bits) operation for 10100110.

8.	Find out the addressing mode used in the following equation: EA = A + (R).

9.	Define Micro Operation.

10.	What is micro programming?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the common Instruction state diagram for the following instruction:  SUB  A, R1.

12.	Compare Static RAM and Dynamic RAM.

13.	Perform addition operation for the following integers using 2’s complement representation. Integers are : (–4)  (+4).

14.	Draw the flow chart of instruction Pipeline.

15.	What are the advantages and disadvantages of hardwired and micro-programmed control?

PART – C  (5 x 15 = 75 MARKS)

16.	Show the changes in the CPU registers when the ADD  C, D instruction is executed. Initial values of the registers are given below: Values of C, D are stored the memory locations 940 and 941, Program counter (PC) contains 300, the address of the first instruction.

|  | Memory |  |  | CPU Registers |  |
| --- | --- | --- | --- | --- | --- |
| 300 | 1940 |  |  | 300 | PC |
| 301 | 5941 |  |  |  | AC |
| 302 | 2941 |  |  | 940 | IR |
| 940 | 0003 |  |  |  |  |
| 941 | 0002 |  |  |  |  |

(OR)

17.	a.	Explain the term Interrupts with suitable diagram.								    (7)

b.	Draw the instruction cycle state diagram with interrupts and also explain the steps in the 			state diagram.																    (8)

[P.T.O]

18.	Define Error correction code and write the necessary outcomes to be considered for 	producing better results during the design of an Error correcting code for data store and read operation.  Explain with the suitable example.

(OR)

19.	Discuss the Direct mapping and Set Associative mapping methods between cache memory and Main memory with the neat diagram.

20.	Explain the types of operations used in machine instructions.

(OR)

21.	Convert the Integers (+7) and (+3) into binary value and perform the multiplication operation with the help of Booth’s algorithm.

22.	Draw the block diagram for Indirect addressing mode and Direct addressing mode. Also explain these addressing modes with required examples.

(OR)

23.	Explain in detail the pipeline hazards.

24.	Write micro operations of Indirect cycle, Interrupt cycle and Execute cycle for the following sequence of events:

t1 : MAR  (PC)

t2 : MBR  Memory

PC  (PC) + I

t3: IR ( MBR)

Note: t1 is First time unit, t2 is Second time unit, t3 is Third time unit.

(OR)

25.	Discuss the Horizontal Microinstruction and Vertical Microinstruction types with suitable diagram.