Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DIGITAL PRINCIPLES AND SYSTEM DESIGN	      	            Time : 3 hours

Subject Code:	09IT204/ 12IT206								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Convert the following from octal to hexa-decimal: (653)8

2.	Give the circuit of odd parity generator for a 4 bit code.

3.	What is half adder?

4.	Find the complement of the function F=x+yz.

5.	Define Demultiplexer.

6.	What is PLA?

7.	What is sequential circuit?

8.	What is different between ripple and synchronous counter?

9.	Define Race Condition.

10.	What is critical race?

PART – B  (5 x 3 = 15 MARKS)

11.	State Demorgan’s theorem and absorption theorem in Boolean algebra.

12.	Design a Full Adder.

13.	Write a note on hardware description language.

14.	Draw the characteristic table of an SR flip flop.

15.	Compare synchronous and asynchronous counters.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Obtain the SOP (Sum of Product) for the following Boolean function: 				    (8)

F = AB+C

b. 	Simplify using K-map 						                        					    (7)

F (A, B, C, D) =  (4, 5, 10, 11, 14, 15)

(OR)

17.	a.	Each of the following numbers is a signed binary number. Determine the decimal value in 	each case, if they are in sign-magnitude form, 2’s complement form and 1’s complement 	form. 										          							  (12)

i. 	01101

ii. 	010111

iii. 	10111

iv. 	1101010

b.	Multiply 10112 by 1012. 				                                                                      	(3)

18.	a. 	Design and explain the operation of BCD to excess-3 converter with K-Map and logic 	diagram.								          									  (10)

b. 	What is magnitude comparator? Explain 2 bit comparator with neat circuit diagram.      (5)

(OR)

[P.T.O]

19.	Design a 4 bit binary to gray code converter using logic gates.

20.	Explain the logic diagram of 3 to 8 line decoder and 8 to 3 encoder.

(OR)

21.	Implement the following function using PLA.

F1 = AB’+AC+A’BC’

F2 = (AC+BC)’

22.	a. 	Draw the circuit diagram for SR flipflop and D flipflop.         		          			(10)

b. 	What is the function of Shift Register?  Show how it is used to convert parallel input to parallel output with clock waveforms. 				                        				(5)

(OR)

23.	Explain the following with neat circuit diagram:

a. 	Universal Shift Register.														(8)

b. 	Synchronous Binary Counter.													(7)

24.	Design a gated latch circuit with two inputs, G (gate) and D (data), and one output Q. The gated latch is a memory element that accepts the value of D when G = 1 and retains this value after G goes to 0. Once G = 0, a change in D does not change the value of the output Q.

(OR)

25.	Explain the following.			                                                                                             (8+7)

a. 	Race-free state assignment.

b. 	Hazards.