Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ENGINEERING MECHANICS						      	     Time : 3 hours

Subject Code:	09ME201/ 12ME230								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define concurrent forces in space

2.	What is a rigid body?

3.	Classify the types of forces in space.

4.	State the law of superposition

5.	The center of gravity of an equilateral triangle with each side (a) is __________ from any of the three sides.

6.	The moment of inertia of a triangle section of base (b) and height (h) about an axis through its center of gravity and parallel to the base is given by the relation ___________.

7.	If two bodies A and B are projected upwards such that the velocity of A is double the velocity of B, then the height to which the body A will rise will be ___________ times the height to which the body B will rise.

8.	Two masses of 10kg and 15 kg are connected to two ends of an inextensible rope and passing over a smooth pulley. The 10kg mass is lying over a rough plane, which is inclined at an angle of 25º with the horizontal. If this angle is made as 30º, then the tension in the string will ___________.

9.	The magnitude of the force of friction between two bodies, one lying above the other, depends upon the roughness of _____________.

(a) upper body 	(b) lower body 	(c) both the bodies

(d) body having more roughness

10.	State co-efficient of friction.

PART – B  (5 x 3 = 15 MARKS)

11.	State the theorem of transmissibility of a force.

12.	Give an outline of smooth surface.

13.	Find the center of gravity for the given T-section

14.	A ball of mass 1kg moving with a velocity of 2m/s impinges directly on a ball of mass 2kg at rest. The first ball, after impinging comes to rest. Find the velocity of the sound ball after the impact and the coefficient of restitution.

15.	Differentiate between static and dynamic friction with examples

[P.T.O]

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A horizontal line PQRS is  long, where . Forces of 	 and  act at P,Q,R and S respectively with downward 	direction. The lines of action of these forces make angle of 90º, 60º, 45º and 30º 	respectively with PS. Find the magnitude, direction and position of the resultant force.

(8)

b.	The following forces act at a point:

  - 20 N inclined at 30º towards North of east,

  - 25 N towards North,

  - 30 N towards North West, and

  - 35 N inclined at 40º towards South of West. Find the magnitude and direction of resultant force.															    (7)

(OR)

17.	a.	A tower guy wire is anchored by means of a bolt at A. The tension in the wire is 2500 N. 	Determine (a) the components   of the force acting on the bolt, (b) the angles 	 defining the direction of the force.										    (7)

b.	A wall section of precast concrete is temporarily held by the cable as shown in figure. Knowing that the tension is 840 N in cable AB and 1200 N in cable AC, determine the magnitude and direction of the resultant of the forces exerted by cables AB and AC on stake A.

(8)

18.	a.	A string ABCD, attached to fixed points A and D has two equal weights of 1000 N 	attached to it at B and C. The weights rest with the portions AB and CD inclined at angles 	as in figure. Determine the tension in the strings

(7)

b.	A light string ABCDE whose extremity A is fixed, has weights  and  attached to it at B and C. It passes round a small smooth peg at D carrying a weight of 300 N at the free end E as shown in figure, find the weights and the tension in the strings.

(8)

(OR)

19.	a.	Three cylinders weighing 100 N each and of 80 mm diameter are placed in a cannel of

180 mm width as in figure. Determine the pressure exerted by (i) the cylinder A on B at the 	point of contact (ii) the cylinder B on the base and (iii) the cylinder B on the wall.	  (10)

b.	Two equal heavy spheres of 50mm radius are in equilibrium within a smooth cup of 	150mm radius. Show that the reaction between the cups of one sphere is double than that 	between the two spheres.														    (5)

[P.T.O]

20.	a.	Find the moment of inertia about the centroidal X-X and Y-Y axes of the angle section 	shown in the figure.															    (7)

b.	A semi-circular area is removed from a trapezium as shown in figure, determine the 	centroid of the remaining area.													    (8)

(OR)

21.	a.	A semi-circle of 90mm radius is cut from a trapezium as in figure, find the position of the 	center of gravity.																    (8)

b.	A rectangular hole is made in a triangular section as in figure. Find the moment in inertia 	about X-X axis passing through its center of gravity and the base BC.				    (7)

22.	a.	A cage descends in a mine shaft with an acceleration of . After the cage has 	travelled 25 m, a stone is dropped from the top of the shaft. Determine the (a) time taken 	by the stone to hit the cage, and (b) distance travelled by the cage before impact.		    (7)

b.	An aeroplane is flying on a straight level course at 55.55m/s at a height of 1000 m above the ground. An anti-aircraft gun located on the ground fires a shell with an initial velocity of 300 m/s, at the instant when the plane is vertically above it. At what inclination, to the horizontal, should the gun be fired to hit the plane? What time after firing, the gun shell will hit the plane? What will then be the horizontal distance of the plane from the gun?

(8)

(OR)

23.	a.	Two guns are pointed at each other, one upward at an angle of 30º, and the other at the 	same angle of depression the muzzles being 30m apart. If the guns are shot with velocities 	of 350m/s upwards and 300m/s downwards respectively, find when and where they will 	meet?																		    (7)

b.	A body moves along a straight line and its acceleration (a) which varies with time (t) is given by . After 5 seconds, from start of observations, its velocity is observed to be 20m/s. After 10 seconds, from start of observation, the body was at 85m from the origin. Determine (a) its acceleration and velocity at the time of start, (b) distance from the origin at the start of observations, and (c) the time after start of observation in which the velocity becomes zero.														    (8)

24.	a.	Two blocks A and B of weights 1 kN and 2kN respectively are in equilibrium position as 	in figure. If the coefficient of friction between the two block as well as the block B and the 	floor is 0.3, find the force (P) required to move the block B.

(10)

b.	A load of 1.5kN, resting on an inclined rough plane, can be moved up the plane by a force of 2kN applied horizontally or by a force 1.25kN applied parallel to the plane. Find the inclination of the plane and the coefficient of friction.								    (5)

(OR)

25.	a.	A block (A) weighing 1 kN rests on a rough inclined plane whose inclination to the 	horizontal is . This block is connected to another block (B) weighing 3kN rests on a 	rough horizontal plane by a weightless rigid bar inclined at an angle of  to the 	horizontal as in figure. Find the horizontal force (P) required to be applied to the block (B) 	just to move the block (A) in upward direction. Assume angle of limiting friction as  at 	all surface where there is sliding.												    (8)

b.	Two loads  (equal to 1kN) and  resting on two inclined rough planes OA and OB are connected by a horizontal link PQ as shown in figure. Find the maximum and minimum values of  for which the equilibrium can exist. Take the angle of friction for both the planes as . 																    (7)