Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	ENGINEERING THERMODYNAMICS				      	     Time : 3 hours

Subject Code:	09ME202 / ME228								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the Zeroth law of thermodynamics?

2.	What is thermodynamic system?

3.	Give the clausius statement of the second law.

4.	Define the term COP.

5.	What is a pure substance?

6.	What is quality of steam?

7.	What is Avogadro's law?

8.	What is areal gas? Give example.

9.	What is air standard efficiency?

10.	Draw the P-V diagram for a diesel cycle.

PART – B  (5 x 3 = 15 MARKS)

11.	What is an ideal gas?

12.	Define the terms source, sink and heat reservoir.

13.	What is saturation temperature and saturation pressure?

14.	State the Requirements of a good fuel.

15.	Discuss the Brayton cycle.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What is the difference between closed system and open system?					    (3)

b.	Derive the steady flow energy equation (SFEE).									  (12)

(OR)

17.	Air flows steadily at the rate of 0.5 kg/s through an air compressor, entering at 7 m/s velocity, 100 kpa pressure, and 0.95 m3/kg volume, and leaving at 5 m/s. 700 kpa, and 0.19 m3/kg.  The internal energy of air leaving is 90 KJ/kg greater than that of the air entering.  Cooling water in  the compressor jacket absorbs heat from the air at the rate of 58 KW.

a.	Compute the rate of shaft work input to the air in KW.

b.	Find the ratio of the inlet pipe diameter to outlet pipe diameter.

18.	Explain Carnot heat engine cycle executed by a stationary system.

(OR)

19.	What are the causes of irreversibility of a process?

20.	Draw the phase equilibrium diagram for a pure substance on T-S plot with relevant constant property lines.

(OR)

21.	Draw the phase equilibrium diagram for a pure substance on h-s plot with relevant constant property lines.

[P.T.O]

22.	Determination of calorific value of fuel by Bomb calorimeter.

(OR)

23.	Two vessels, A and B both containing nitrogen are connected by a value which is opened to allow the contents to mix and achieve an equilibrium temperature of 27oC. Before mixing the following information is known about the gases in the two vessels

Vessel A				Vessel B

P = 1.5 MPa				P = 0.6 MPa

t=50oC					t = 20oC

contents = 0.5 kg mol		contents = 2.5 kg

Calculate the final equilibrium pressure, and the amount of heat transferred to the surroundings. If the vessel had been perfectly insulated, calculate the final temperature and pressure which would have been reached. Take =1.4

24.	An engine working on the otto cycle is supplied with air at 0.1 MPa, 35°C.  The compression ratio is 8.  Heat supplied is 2100 KJ/Kg.  Calculate the maximum pressure and temperature of the cycle, the cycle efficiency and the mean effective pressure.

(For air Cp=1.005, Cv= 0.718 and R=0.287 KJ/kgK)

(OR)

25.	A engine working on diesel engine with 200 mm cylinder diameter and 300 mm stroke. The initial pressure and temperature of air are 1 bar and 27oC. The cut off is 8% of the stroke. Determine (a) Pressure and temperature at all points. (b) Air standard efficiency (c) Mean effective pressure (d) Power of the engine, if the working cycles per minute are 380. Take            r = 15 and = 1.4.