Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	THERMAL ENGINEERING      						      	     Time : 3 hours

Subject Code:	09ME203 / ME229				 				       Maximum Marks: 100

#### Answer ALL questions

#### (Approved Steam Table, Mollier Diagram, Psychrometric Charts are Permitted)

PART – A  (10 x 1 = 10 MARKS)

1.	A carburetor is used to supply __________and_________.

2.	The thermal efficiency of a petrol engine is __________ as compared to diesel engine.

3.	What is the function on nozzle?

4.	State the necessity of governing of steam turbine.

5.	Define volumetric efficiency of an air compressor.

6.	In a high pressure compressor, the delivery pressure is 8 to 9 bar. True / false.

7.	Define the COP of refrigeration.

8.	A vapour absorption refrigerator uses __________ as a refrigerant.

9.    What is dew point temperature?

10.   Design air conditioning.

PART – B  (5 x 3 = 15 MARKS)

11.	Compare petrol and diesel engines.

12.  Shortly explain the working principle of an impulse turbine.

13.	What are the advantages of multistage air compressor?

14.	What is the basic difference between vapour compression and vapour absorption refrigeration system?

15.	What is sensible heating?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Describe the valve timing diagram of four stroke CI engine with neat sketch.		  (10)

b.	What is diesel knock? Compare knocking phenomena of the SI and CI engines.	          (5)

(OR)

17.	a.	Explain the working principle of magneto ignition system with a neat diagram.		  (10)

b.	Compare two stroke and four stroke engine.				                                              (5)

18.	Derive an expression for maximum mass flow through the nozzle.

(OR)

19.	Explain velocity compounding of Impulse turbine with a neat sketch.

20.	A single stage reciprocating air compressor is required to compress 1 kg of air from 1 bar to 4 bar.  The initial temperature is 300 K.  Compare the work requirement in the following cases.

a.	Isothermal compression

b.	Compression with pv1.2=constant and

c.	Isentropic compression.

(OR)

21.	Explain the working principle of single stage reciprocating air compressor with a neat sketch.

[P.T.O]

22.	A vapour compression machine is used to maintain a temperature of -23˚C in a refrigerated space. The ambient temperature is 37˚C.The compressor takes in dry saturated vapour of  F-12. A minimum 10˚C temperature difference is required at the evaporator as well as at condenser for effective heat transfer. There is no sub-cooling of liquid. If the refrigerant flow rate is 1 kg/min, find

a.	Tonnage of refrigeration

b.	Power requirement.

c.	Ratio of C.O.P. of this cycle to C.O.P. of Carnot cycle.

(OR)

23.	Draw a neat diagram of vapour compression refrigeration system and explain its working.

24.	Explain summer Air conditioning system with a neat sketch.

(OR)

25.	Explain the following terms:

a.	Wet-bulb temperature.                                                                                  			(5)

b.	Degree of saturation.                                                                                   			(5)

c.	Relative humidity.                                                                                      			(5)