Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	HEAT AND MASS TRANSFER  						      Time : 3 hours

Subject Code:	09ME204/12ME217/ME230						       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	According to the which law of thermodynamics the heat transfer takes place?

2. 	Which is the mostly used insulating material for the pipeline carrying steam?

3.	Define ‘Biot number’.

4. 	What is the restrictive for the application of the Heisler charts?

5.	The boundary layer thickness at a point in flow over a plate, is 8 mm. The Reynolds number is 25600. The distance from the leading edge is ___________.

6. 	The Velocity Profile obtained for fully developed laminar flow in a tube is _________.

7.	What is meant by effectiveness?

8.	Define molecular diffusion.

9.	What is meant by dropwise condensation?

10. 	Define irradiation.

PART – B  (5 x 3 = 15 MARKS)

11. 	Heat is lost at a rate of 300 W/m2 area of a 20 cm thick wall with a thermal conductivity of        	k = 1.2 W/mo C. Calculate the temperature drop across the wall.

12. 	What are Heisler charts? Give its significance.

13.	A very long fin has 25 mm diameter and has one end maintained at 1300C. The surface of the 	fin is exposed to ambient air at 300C. The thermal conductivity of the fin material is 400 	W/mK. If the convective heat transfer coefficient is 40 W/m2K, find the heat loss from the fin.

14. What are the different modes of mass transfer?

15. 	What is meant by condensation?

PART – C  (5 x 15 = 75 MARKS)

16.	Write the heat conduction equation in cylindrical coordinate system.

(OR)

17.	a. 	A composite wall is composed of 1cm thick iron(k = 60W/mK), 4cm thick fibre glass

( k = 0.08W/mK) and 0.4cm thick asbestos( k = 0.2W/mK). Determine the overall coefficient heat transfer coefficient and heat transfer rate for T = 400.		 		(8)

b. 	The wall for an industrial furnace consists of fire clay brick ( L1 = 0.2m, k1 = 1.0 W/mK)  which is covered with a layer of insulating material of L2 = 0.03m , k2 =0.05W/mK. If the t1 = 830ºC and t3 = 30º C. Find the heat transfer per unit area. 						(7)

18.	A copper plate 2 mm thick is heated  upto 400° C and then quenched in water at 25° C. Find the Biot number and verify if the lumped heat capacity analysis is applicable. Also find the         a.	time constant		b.	 time required for the plate to reach the temperature of 40° C.

The data given is: h= 100 W/m2K,   Cp= 400 J/Kg K,   k=385 W/m K, plate size = 25 m x 25 m,  ρ = 8800 kg/m3.

(OR)

19. 	How can the transient heat conduction for a plain wall be analyzed with the help of Heisler charts.

[P.T.O]

20.	State Buckingham’s  theorem. Explain the various parameters used in forced convection. Using dimensional analysis obtain an expression for Nusselt number in terms of Reynolds and Prandtl numbers.

(OR)

21. 	a.	Air at a pressure of 8kN/m2 and a temperature of 2500C flows over a flat plate 0.3m wide 	and 1 m long at a velocity of 8m/s. If the plate is to be maintained at a temperature of 	780C estimate the rate of heat to be removed continuously from the plate.                 	    (8)

b.	A 30 cm long glass plate is hung vertically in the air at 270C while its temperature is 	maintained at 770C. Calculate the boundary layer thickness at its trailing edge of the plate 	and also the average heat transfer coefficient.									    (7)

22.	 Discuss the general arrangement of parallel flow, counter flow and cross flow heat exchangers.

(OR)

23.	In a counter flow, double pipe heat exchanger, water is heated from 25oC to 65oC by oil with a specific heat of 1.45 kJ/kgK and mass flow rate of 0.9 kg/s. The oil is cooled from 230oC to 160oC. If the overall heat transfer co-efficient is 420 W/m2K, calculate the following

a.	the rate of heat transfer.

b.	the mass flow rate of water and

c.	the surface area of the heat exchanger.

24.	Discuss the various regimes of pool boiling heat transfer with a neat sketch.

(OR)

25. 	Two large parallel plates are maintained at 600 K and 900 K and emissivities are	 0.4 and 0.7 	respectively. Determine heat transfer by radiation and also calculate percentage reduction in 	heat transfer and shield temperature when another plate of emissivity 0.05 is introduced in 	between them.