Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	MECHANICS OF MACHINES - I					      	     Time : 3 hours

Subject Code:	09ME205/12ME222/ME207/ME254				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define a link.

2.	A ball and a socket joint forms a ___________.

3.	When two pulleys of different diameters are connected by means of an open belt drive, then the 	angle of contact taken into consideration should be of the ___________.

4.	When the belt is stationary, it is subjected to some tension, known as initial tension. The value 	of this tension is equal to the ___________.

5.	In a radial cam, the follower moves ___________.

6.	What kind of follower can be used, if wear rate should be reduced?

7.	In a simple gear train, if the number of idle gears is odd, then the motion of driven gear will be  	___________.

8.	A differential gear in an automobile is a ___________.

9.	A Hartnell governor is a ___________.

10.	In an engine, the work done by inertia forces in a cycle is ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the differences between a Machine and a Structure?

12.	What are the factors to be considered while selecting the belt drives?

13.	What are the different types of motion with which a follower can move?

14.	Classify the toothed wheels.

15.	What is the function of governors?

PART – C  (5 x 15 = 75 MARKS)

16.   Explain the working principle of crank and slotted lever quick returns motion Mechanism.

(OR)

17.	The crank of a slider crank mechanism rotates clockwise at a constant speed of 300 r.p.m. The 	crank is 150 mm and the connecting rod is 600 mm long. Determine

a.	Linear velocity and acceleration of the midpoint of the connecting rod.

b.	Angular velocity and angular acceleration of the connecting rod at a crank angle of 45 			from inner dead centre position.

18.	A single plate clutch transmits 7.5 kW at 900 rpm. The axial pressure is limited to 0.07 	N/mm2. If the coefficient of friction is 0.25, find

a.	Mean radius and face width of the friction 	lining assuming the ratio of the mean radius to the face width as 4, and

b.	Outer and inner radii of the clutch plate.

(OR)

[P.T.O]

19.	A full journal bearing of 50 mm diameter and 100 mm long has a bearing pressure of 1.4 	N/mm2. The speed of the journal is 900 r.p.m. and the ratio of journal diameter to the diametric 	clearance is 1000. The bearing is lubricated with oil whose absolute viscosity at the operating

temperature of 75 o C may be taken as 0.011 kg/m-s. The room temperature is 35 o C. Find: 	a.	the amount of artificial cooling required and

b.	the mass of the lubricating oil required, if 	the difference between the outlet and inlet temperature of the oil is 10 o C. Take specific heat of 	the oil as 1850 J/kg/ o C.

20.	Construct the profile of a cam to suit the following specifications:

Cam shaft diameter = 40 mm, Least radius of cam = 25 mm, Diameter of roller = 25 mm, 	Angle of lift = 1200, Angle of fall = 1500, Lift of the follower = 40 mm, Number of pauses are 	two of equal interval between motions. 	During the lift, the motion is S.H.M. During the fall 	the motion is uniform acceleration and deceleration. The speed of the cam shaft is uniform. The 	line of stroke of the follower is off-set 12.5 mm from the centre of the cam.

(OR)

21.	A cam, with a minimum radius of 50 mm, rotating clockwise at a uniform speed, is required to 	give a knife edge follower the motion as described below:  1.	 To move outwards through 40 	mm during 100o rotation of the cam; 2. To dwell for next 80o; 3. To return to its starting 	position during next 90o; and 4. To dwell for the rest period of a revolution i.e., 90o.

Draw the profile of the cam

a.	When the line of stroke of the follower passes through the centre of the cam shaft, and

b.	When the line of stroke of the follower is off-set by 15 mm.

The displacement of the follower is to take place with uniform acceleration and uniform retardation. Determine the maximum velocity and acceleration of the follower when the cam shaft rotates at 900 r.p.m. Draw the displacement, velocity and acceleration diagrams for one complete revolution of the cam.

22.	In an epicyclic gear train, an arm carries two gears A and B having 36 and 45 teeth 	respectively. If the arm rotates at 150 r.p.m in the anticlockwise direction about the centre of 	the gear A which is fixed, determine the speed of gear B. If the gear A instead of being fixed 	makes 300 r.p.m. in the clockwise direction, what will be the speed of gear B?

(OR)

23. 	With a neat diagram explain various nomenclature used in gear.

a.	Explain the working of a centrifugal governor.									  (10)

b.	Calculate the vertical height of a watt governor when it rotates at 60 r.p.m. Also find the 			change in vertical height when its speed increases to 61 r.p.m.						   (5)

24.	 A porter governor has equal arms each 250 mm long and pivoted on the axis of rotation. Each 	ball has a mass of 5 kg and the mass of the central load on the sleeve is 25 kg. The radius of 	rotation of the ball is 150 mm when the governor begins to lift and 200 mm when the governor 	is at maximum speed. Find the minimum and maximum speeds and range of speed of the 	governor.

(OR)

25.	A ship propelled by a turbine rotor which has a mass of 5 tonnes and a speed of 2100 r.p.m.

The rotor has a radius of gyration of 0.5 m and rotates in a clockwise direction when view from the stern. Find the gyroscopic effects in the following conditions:

a. 	The ship sails at a speed of 30 km/h and steers to the left in a curve having 60 m radius.

b. 	The ship pitches 6 degree above and 6 degree below the horizontal position. The bow is			descending with its maximum velocity. The motion due to pitching is simple harmonic and 		the 	periodic time is 20 seconds.

c. 	The ship rolls and at a certain instant it has an angular velocity of 0.03 rad/s clockwise when viewed from stern.

d.	Determine also the maximum angular acceleration during pitching. Explain how the direction of motion due to gyroscopic effect is determined in each case.