Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DESIGN OF MACHINE ELEMENTS              		      	      Time : 4 hours

Subject Code:	09ME206 / 12ME225 / ME257	 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the term Hardness.

2. 	Define stress concentration.

3. 	Why normal stress theory is not suitable for ductile material?

4. 	What are the types of keys?

5. 	Where flexible couplings are used?

6. 	What is the material used for flange or flange coupling?

7. 	List some types of commonly used thread forms.

8. 	What are the various parts of rivet?

9. 	What are the various types of springs?

10. 	State the use of a flywheel.

PART – B  (5 x 3 = 15 MARKS)

11. 	How is factor of safety defined for brittle and ductile materials?

12. 	What are the purposes in machinery for which couplings are used?

13. 	What is caulking and fullering?

14. 	Define the term “fluctuation of speed” and “fluctuation of energy”.

15. 	State the different types of failures involved in riveted joints.

PART – C  (5 x 15 = 75 MARKS)

16.	A shaft of 40 mm diameter is used transmit a power of 30 KW at 710 rpm and is supported in bearings 500 mm apart.  A load of 10 kN is concentrated at the centre of the shaft, acting vertically.  Find the maximum principal stress and shear stress.

(OR)

17.	A bolt is subjected to an axial force of 10,000 N with a transverse shear force of 5000 N.  Find the diameter of the bolt according to.

a.	Maximum principal stress theory.

b.	Maximum shear stress theory.

c.	Maximum strain energy theory.

It is assumed that the permissible tensile stress at elastic limit is 100 N/MM2 AND Poisson’s ratio is 0.3.

18. 	A circular bar of 500mm length is supported freely at its two ends. It is acted upon by a central concentrated cyclic load having a minimum value of 20kN and a maximum value of 50kN. Determine the diameter of bar by taking a factor of safety of 1.5, size effect of 0.85, surface finish factor of 0.9. The material properties of bar are given by: ultimate strength of 650MPa, yield strength of 500MPa and endurance strength of 350MPa.

(OR)

19.	Design a helical compression spring to carry a load of 1.5 kN with a deflection of 40 mm.  Spring index is 5.  Allowable shear stress is 400 N/mm2. Modulus of rigidity is 8X1010 N/m2.

[P.T.O]

20. A shaft supported at the ends in ball bearings carries a straight tooth spur gear at its mid span 	and is to transmit 7.5kW at 300r.p.m. The pitch circle diameter of the gear is 150mm. The 	distances between the centre line of bearings and gear are 100mm each. If the shaft is made of 	steel and the allowable shear stress is 45MPa, determine the diameter of the shaft. Show in a 	sketch how the gear will be mounted on the shaft; also indicate the ends where the bearings will 	be mounted? The pressure angle of the gear may be taken as 20°.

(OR)

21.	Design a muff coupling to transmit a power of 35 kW from a shaft running at 120 rpm.  The material for shaft and key is mild steel and material of coupling is cast iron.  Allowable shear stress of shaft is 42 N/mm2; Allowable crushing stress of key is 84 N/mm2; Allowable shear stress for coupling 14 N/mm2.

22. 	A double riveted lap joint is made between 15mm thick plates. The rivet diameter and pitch are 25mm and 75mm respectively. If the ultimate stresses are 400MPa in tension, 320MPa in shear and 640MPa in crushing, find the minimum force per pitch, which will rupture the joint.

If the above joint is subjected to a load such that the factor of safety is 4, find out the actual stresses developed in the plates and the rivets.

(OR)

23.	Design a socket and spigot colter joint to transmit a load of 50 N.  Assume σt=60Nmm2;σc=100Nmm2;τ=45 Nmm2.

24. 	Design a cast iron piston for a single acting four stroke engine for the following data:

Cylinder bore = 100mm ; Stroke = 125mm ; Maximum gas pressure =  N/mm2 ; Indicated mean effective pressure = 0.75N/mm2 ; Mechanical efficiency = 80% ; Fuel consumption = 0.15kg per brake power per hour ; Higher calorific value of fuel = 42 × 103kJ/kg ; Speed = 2000r.p.m.Any other data required for the design may be assumed.  Maximum gas pressure = 6 N/mm2.

(OR)

25. 	A single cylinder double acting steam engine develops 150kW at a mean speed of 80r.p.m. The coefficient of fluctuation of energy is 0.1 and the fluctuation of speed is ±2% of mean speed. If the mean diameter of the flywheel rim is 2metres and the hub and spokes provide 5 percent of the rotational inertia of the wheel, find the mass of the flywheel and cross-sectional area of the rim. Assume the density of the flywheel material (which is cast iron) as 7200 kg/m3.