Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	GAS DYNAMICS AND JET PROPULSION			      	     Time : 3 hours

Subject Code:	09ME207/ME217								       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by Gas Dynamics?

2.	Define Mach number.

3.	Define Adiabatic flow.

4.	What are the type of nozzles?

5.	What is fanno flow?

6.	Write the expression for the length of duct in terms of two Mach number M1 and M2 for a flow through a constant area duct with the influence of friction.

7.	What are the two types of shock?

8.	Write the Prandtl Mayer equation.

9.	Write any one type of jet engine.

10.	Write an example of bi propellant.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Crocco number.

12.	Give the expression for T/T0 and T/T* for Isentropic flow through variable area in terms of Mach number.

13.	What are Assumptions made for fanno flow?

14.	What do you mean by Rayleigh flow?

15.	Differentiate Jet propulsion and Rocket propulsion.

PART – C  (5 x 15 = 75 MARKS)

16.	An air jet (γ = 1.4, R=287 J/KgK) at 400k has sonic velocity. Determine:

(a) Velocity of sound at 400 K

(b) Velocity of sound at the stagnation condition

(c) Maximum Velocity of the Jet

(d) Stagnation enthalpy

(e) Crocco number

(OR)

17. 	Derive the expression for the effect of Mach number on compressibility.

18.  A supersonic nozzles air expands from P0=24 bar and T0= 1000 k to an exit pressure of 4.3 bar. If the exit of the nozzle is 110 cm2, calculate the following:

(a) throat area

(b) pressure and temperature at the throat

(c) temperature at exit

(d) Mass flow rate

(e) Exit velocity as fraction of the Maximum attainable velocity.

[P.T.O]

(OR)

19.	Derive an expression for area ratio as a function of Mach number.

20.	Air enters a pipe of 25mm diameter, at a Mach number of 2:4 stagnation temperature of  	300K and static pressure of 0.5 bar of the co-efficient of function is 0.003.  Determine the	following for a section at which the Mach number reaches 1.2.

a.	Static pressure and temperature

b.	Stagnation pressure and temperature

c.	Velocity of air

d.	Distance of this section from the inlet

e.	Mass flow rate.

(OR)

21.  	A constant area combustion chamber receives air at 740C, 0.55 bar and 75m/s.  If the air leaves the combustion chamber at a Mach number of 0.85, determine condition gain and the amount of heat transferred and also find the change in entropy.

22. 	Derive the expression for prandl-Meyer relation.

(OR)

23. 	A convergent –divergent air nozzle has exit to throat area ratio of 3.  A normal shock appears at the divergent section where the existing area ratio 2.2.  Find the Mach number, before and        after shock.  If the inlet stagnation properties are 500 k and 450 k, find the properties of air at exit and entropy increase across the shock.

24. 	Explain the construction and working of Ramjet Engine and state its advantages and disadvantages.

(OR)

25. 	A rocket flies at a speed of 10,000 kmph with an effective exhaust Jet velocity of 1350 m/s and the heat produced by the propellant is 6600 kJ/Kg, If the propellant flow rate 4.5 kg/s.  Determine propulsive efficiency, propulsive power, Engine output, thermal efficiency and overall efficiency.