Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title :	DESIGN OF  TRANSMISSION SYSTEMS / DESIGN OF  MECHANICAL TRANSMISSION SYSTEMS			      	     			      Time : 3 hours

Subject Code:	09ME208 / 12ME224 / ME231					       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is Lubricant?

2.	List the various types of belts.

3.	What are the components used in chain drives?

4. 	What do you understand by 6X19 constructions in wire ropes?

5. 	Define AGMA.

6. 	Where bevel gears are used?

7. 	List any two methods used for changing speeds in gear boxes.

8. 	What is a speed reducer?

9. 	What are the purposes of screws?

10. 	Define pitch.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the types of journal bearings depending upon the nature of contact and nature of lubrication?

12. 	List the advantages of chain drives.

13. 	How bevel gear is classified based on the shape of teeth?

14.	What are the possible arrangements to achieve 12 speeds from a gear box?

15. 	What stresses act on screw fastenings?

PART – C  (5 x 15 = 75 MARKS)

16. 	A bearing for an axial flow compressor is used to support a radial load of 2500 N, an axial load thrust of 1500 N. The service required for the bearing is 5 years at a rate of 40 hours per week.  Speed of the shaft is 50 mm. Select the suitable ball bearing for this purpose.

(OR)

17. 	Design a Flat belt drive for a fan running at 360 rpm, which is driven by a 10 KW, 1440 rpm motor. The belt drive is open-type and space available for a centre distance of 2m approximately. The diameter of a driven pulley is 1000mm.

18. 	Design a spur gear drive required to transmit 45 kw at a pinion speed of 800 rpm. The velocity ratio is 3.5:1.The teeth are 20° full depth involute with 18 teeth on the pinion. Both the pinion and gear are made of steel with a maximum safe static stress of 180N/mm2. Assume medium shock conditions.

(OR)

19. 	Design a helical gear to transmit 15 kw at 1400 rpm to the following specifications: Speed reduction is 3, pressure angle is 200, helix angle is 150, the materials of both the gears are C45 steel. Allowable static stress 180N/mm2, surface endurance limit is 800N/mm2, young’s modulus of material = 2x105N/mm2.

[P.T.O]

20. 	Design a cast iron bevel gear drive for a pillar drilling machine to transmit 1875w at 800rpm to a spindle at 400 rpm. The gear is to work for 40 hours per week for 3 years. Pressure angle is 200.

(OR)

21. 	A steel worm running at 240 rpm receives 1.5 kw from its shaft. The speed reduction is 10:1. Design the drive so as to have and efficiency of 80%. Also determine the cooling area required if the temperature rise is restricted to 450c. Take overall heat transfer co-efficient as 10w/m2c.

22.	For a 12 speed gear box, the speeds available at the spindle are 31.5, 45, 63, 90, 125, 180, 250, 355, 500, 710, 1000, 1410 rpm.  The structure formula is 2(1). 3(2). 2(6).  Draw the speed diagram and kinematic arrangement of the gears.

(OR)

23.	Draw the kinematic arrangement and speed diagram of a head stock gear box of a turret lathe having arrangement for a 9 spindle speed ranging from 31.5 rpm to 1050 rpm.  Calculate the number of teeth on each gear.  Minimum number of teeth on each gear is 25.

24.	An offset translating roller follower with SHM, is driven by a cam rotating at 600 rpm.  The maximum lift of the follower is 30 mm during 150° of cam rotation.  The prime circle diameter of cam is 80 mm.

If the amount of offset is 5 mm, determine

a.	pressure angle at cam angle of 60°

b.	pressure angle at cam angle of 70°, if the follower is radial in nature, instead of offset type.

(OR)

25. The following data relate to a screw jack: Pitch of the threaded screw = 8mm, Diameter of the threaded screw = 40mm, Co-efficient of friction between screw and nut = 0.1,                                                                 Load = 20kw. Assuming that the load rotates with the screw determine (a) The ratio of torques required to raise and lower the load. (b) The efficiency of the machine.