Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title : FLUID POWER CONTROL / FLUID POWER CONTROL ENGINEERING							      	     											      Time : 3 hours

Subject Code: 09ME225/12ME205/ME237		 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the term fluid power.

2.	Draw the symbol for FRL unit.

3.	Classify pneumatic actuators.

4.	How direction control valves are actuated?

5.	Write any four ways of mounting the cylinders.

6.	Define mechanical efficiency of a hydraulic motor.

7.	What is the use of synchronizing circuit?

8.	Define low-cost automation.

9.	PLC refers to______________.

10.	Name few fluid sensors.

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly explain the need for automation.

12.	Name the basic components required in Pneumatic circuit.

13.	List the types of accumulators.

14.	Define Sequencing. What does A+ B- A- B+ denotes in sequencing operation?

15.	Distinguish between meter in, meter out circuits.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Define volumetric and mechanical efficiency of a hydraulic pump. 					    (5)

b.	A pump has a displacement of 81.9 cm3. It delivers 75.8x10-3 m3/min at 1000 rpm at a 67bar. If the prime mover input torque is 100 Nm. Calculate the overall and Volumetric efficiency.                                                            									  (10)

(OR)

17.	a.	Compare pneumatic and hydraulic control system.         							    (9)

b.	Explain six physical properties of hydraulic fluids?		  						    (6)

18.	Explain the constructional features and working of a vane motors with neat sketch.

(OR)

19.	Discuss in detail about design and construction of linear actuator.

20.	Explain the working of following with circuit

a.	Quick exhaust valve.		b.	Time delay valve.	       								(7+8)

(OR)

21.	Explain hydraulic sequencing circuit, with neat diagram.

22.	Discuss about various fluidic elements.

(OR)

23.	Explain the major units of a PLC with block diagram.

24.	Draw and name the components of the hydraulic press circuit.

(OR)

25.	Draw and name the components of the following hydraulic circuits:

a.	Intensifier circuit.	       	b.	Regenerative circuit.	      								(7+8)