Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title : HEAT ENGINES AND FLUID MACHINERY			      	     Time : 3 hours

Subject Code: 09ME228 / ME223					 				       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The specific volume is the reciprocal of ___________.

2.	The specific gravity of mercury is ___________.

3.	How to avoid cavitation of pump?

4.	Write down the use of surge tank.

5.	Write the first law of Thermodynamics for a cycle.

6.	State the zeroth law of thermodynamics.

7.	In diesel cycle, the compression ratio is _____________than the expansion ratio.

8.	What is the air standard efficiency of the Otto cycle?

9.	What causes conduction of heat transfer?

10.	Define Reynolds number.

PART – B  (5 x 3 = 15 MARKS)

11.	The right limb of a simple U – tube manometer containing mercury is open to the atmosphere 	while the left limb is connected to a pipe in which a fluid of specific gravity 0.9 is flowing. The 	centre of the pipe is 12 cms below the level of the mercury of the right limb. Find the pressure 	of the fluid in the pipe, if the difference of mercury level in the two limbs is 20 cms.

12.	What are the differences between Impulse and Reaction turbine?

13.	State the two equivalent statement of second law of thermodynamics.

14.	Draw the T-S and P-V diagram of a Brayton cycle.

15.	State and explain Fourier’s law of heat conduction.

PART – C  (5 x 15 = 75 MARKS)

16.	Two large fixed parallel planes are 12m apart. The space between the surfaces is filled with oil 	of viscosity 0.972 N.s/m2 . A flat thin plate 0.25 m2 area moves through the oil at a velocity of 	0.3m/s. Calculate the drag force : (i) when the plate is equidistant from both the planes and     	(ii) when the thin plate is at a distance of 4mm from one of the plane surfaces.

(OR)

17.	Describe with the aid of neat sketch the working of a reciprocating pump.

18.	Explain the Pelton wheel with a neat sketch.

(OR)

19.	Draw the schematic diagram of a Kaplan turbine and explain briefly its construction and working. Obtain an expression for work done by the runner.

20.	A reversible heat engine operates between two reservoirs at temperatures of 600o C and 40oC. The engine drives a reversible refrigerator which operates between reservoirs at temperatures of 40o C and -20oC. The heat transfer to the engine is 2000 kJ and the net work output of the combined engine refrigerator plant is 360 kJ.

[P.T.O]

a.	Evaluate the heat transfer to the refrigerant and the net heat transfer to the reservoir at 40oC.

b.	Reconsider (a) given that the efficiency of the heat engine and the COP of the refrigerator are each 40% of their maximum possible values.

(OR)

21.	A fluid undergoes a reversible adiabatic compression from 0.5 MPa, 0.2 m3 to 0.05 m3 according to the law, pv1.3 = constant. Determine the change in enthalpy, internal energy,  entropy, the heat transfer and work transfer during the process.

22.	Compare and contrast otto, diesel and dual cycle.

(OR)

23.	a.	Explain the procedure to determine the Frictional Horse Power of an IC Engine.

b.	Explain the procedure to conduct Performance test of an IC Engine.

24.	a.	State the Fourier law of heat conduction. 										(3)

b.	Derive an expression for steady heat conduction through a plane wall of thickness L

maintained at its two surfaces at temperature T1 and T2 respectively.				(12)

(OR)

25.	A Steel tube of 80mm internal diameter and 6mm wall thickness has 8 longitudinal fins of 1.5mm thickness. Each fin extends 35 mm from the pipe wall and thermal conductivity of fin material is 50 W/m-deg. The wall temperature, the ambient temperature and surface heat transfer coefficient are stated to be 150°C, 25°C and 80 W/m2 – deg. Make calculations for the percentage increase in the rate of heat transfers  for the finned tube over the plain tube.