Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May – 2016

Subject Title : 	POWER PLANT ENGINEERING					      	     Time : 3 hours

Subject Code:	09ME237/12ME216								      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	When does pre heating of steam become necessity?

2.	When will you recommend BVC?

3.	Why cooling tower is required in SPP?

4.	What is the function of electrostatic precipitator?

5.	What are fission and fusion products?

6.	What is the function of moderator in nuclear power plant?

7.	Name critical components of diesel electric power plant.

8.	Give the specific advantages, disadvantages of gas turbine power plant.

9.	How can the cost of power generation be reduced?

10.	What are the elements which contribute the cost of electricity?

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss the effect of intercooling & reheating in a gas turbine plant.

12.	Enlist the various energy losses taking place in the steam generator.

13.	Explain the characteristic features of a pressurized water reactor.

14.	Write short notes on pumped storage system.

15.	What do you understand by load factor & capacity factor, when are they numerically equal?

PART – C  (5 x 15 = 75 MARKS)

16.	A steam power plant uses steam at boiler pressure of 150bar and temperature 5500C with reheat at 40bar and 5500C at condenser pressure of 0.1bar. Find the quality of steam at turbine exhaust, cycle efficiency and the steam rate.

(OR)

17.	Discuss briefly on simple Rankine cycle and do the thermodynamic analysis for the same. 	Also explain the methods of improving cycle efficiency.

18.	Describe with a neat sketch the constructional and operational features of a surface condenser.

(OR)

19.	What is draught? What are the functions of a draught system? Describe with neat sketch the salient features of forced draught (FD) & induced draught (ID) fan. Also explain the concept of balanced draught.

20.	Discuss briefly on Liquid metal fast breeder reactor (LMFBR) with a suitable diagram.

a.	Construction, Operational features												(10)

b.	Limitations and applications 													  (5)

(OR)

[P.T.O]

21.	A gas turbine power plant consists of a two stage compressor with intercooling and a single stage turbine with a regenerator. Air enters the compressor at 1 bar, 20oC. The maximum temperature of the cycle is limited to 900o C and the maximum pressure ratio is 6. The effectiveness of the regenerator is 0.7. The rate of air flow through the plant is 210 kg/s and calorific value of fuel used is 40.8MJ/kg. The isentropic efficiency of both the compressors is 0.82, the isentropic efficiency of the turbine is 0.92, the combustion efficiency is 0.95, the mechanical efficiency is 0.96 and the generator efficiency is 0.95. Take for air cp = 1.005 kJ/kgK and = 1.4 and for gases cp=1.08 kJ/kgK and  = 1.33. Assuming perfect intercooling and neglecting pressure and heat losses , estimate (a) the air-fuel ratio (b) the cycle efficiency (c) the power supplied by the plant and (d) the specific fuel consumption of the plant and the fuel consumption per hour.

22.	a.	Which are the different types of diesel engine? 				  					    (3)

b.	What are the applications of diesel power plant? 				  					    (3)

c.	Explain the main features of supercharging with the help of p-V diagram. What do you 	mean by mechanical supercharging and turbocharging? What is the effect of intercooling 	in turbocharging?						  										   (9)

(OR)

23.	Discuss briefly with the help of a schematic layout of Hydel power plant with special mention to constructional, operational features and also its limitations.

24.	a.	Write short notes on MHD concept of EC and energy audit. 						  (10)

b.	Explain the effect of load factor on the cost of electricity generated. 				    (5)

(OR)

25.	A power station has to supply loads as follows:

Time (in hours)         0-6  		6-12		12-14		14-18		18-24

Load (MW)	          	30             90		   	   60			  100		   50

- Draw the load curve.

- Draw the load duration curve

c.	Select suitable generating units to supply the load.

d.	Calculate the load factor.

e.	Calculate the capacity of the plant and the plant capacity factor.